3GPP TSG-RAN WG3 Meeting #127-bis	R3-252175

# Wuhan, China,  7– 11 April, 2025

Title: 	Discussion on E1/F1 to support UE performance feedback

Source: 	CMCC

Agenda item: 	11.4

Document Type:	Discussion

- Introduction

In RAN3 #124 meeting, agreements as following have been achieved for split architecture:

Transferring Measured UE performance from CU-UP to CU-CP.

And in RAN3 #125 meeting,

The details of signaling measured UE performance metrics from DU to CU-CP and/or from CU-UP To CU-CP needs further discussion during normative phase. The above overrides the agreement “Transferring Measured UE performance from CU-UP to CU-CP.” from RAN3 #124.

In RAN3 #127 meeting, the following agreement achieved:

Packet delay measured for UE Performance feedback is sent from CU-UP (the delay in DU side is aggregated in the CU-UP) to the CU-CP in the case of CP-UP split architecture.

Packet delay measured for UE Performance feedback is sent from DU to CU via DDDS in the case of CU-DU split architecture.

Stage 3 details of the solution are FFS.

.2	Discussion

This paper will focus on the discussion on the procedures needed via E1 and F1 to transfer the AIML data collection report from CU-UP to CU-CP and from DU to CU.

  - E1 procedures

According to the previous discussion, in RAN3#127, it has been agreed that packet delay needs to be sent from CU-UP to CU-CP in the case of CP-UP split architecture. To support it, DATA COLLECTION procedures similar with Xn interfaces need to be introduced via E1.

Proposal 1: It is proposed to introduce DATA COLLECTION procedure similar with Xn in E1, to get the UE/slice level packet delay from CU-UP.

The current procedure to get DATA COLLECTION from one gNB to another includes messages “REQUEST”, “RESPONSE” and “UPDATE”.  When CU-CP gets the DATA COLLECTION REQUEST, it shall send DATA COLLECTION REQUEST to CU-UP with the same UE Performance Collection Configuration IE.

However, since CU-UP needs to get the information and measure it through DDDS, it can’t response to CP  immediately. But CU-CP may wait for the response from UP to response to NG-RAN node1 via Xn.

Proposal 2：It is proposed to discuss how to response the DATA COLLECTION REQUEST on UE Packet Loss through E1 from CU-UP to CU-CP.

Option 1: CU-UP always response successful before it gets the DDDS frame.

Option 2: CU-CP needs to hold a long timer to wait for CU-UP getting the first DDDS frame and response back.

It may be an easier solution as option 1 but it conflicts with the original design of the DATA COLLECTION procedures.

In TS38.300 clause 16.20.3	Data Collection and Reporting: The collection of measured UE trajectory and UE performance feedback is triggered at successful Handover. After successful Handover, UE RACH access to gNB-DU, DU will setup the data path and send “DL DATA DELIVERY STATUS” frame to CU-UP through F1-U as the current DDDS mechanism.

In TS38.423 clause 8.4.13 Data Collection Reporting Initiation, there are 3 triggers to terminate the data collection and initiate report:

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell.

When the configured Collection Time Duration timeout, CU-UP shall also stop the data collection and report the last measurement to CU-CP.

CU-CP will terminate the data collection when UE enters into RRC_INACTIVE or RRC_IDLE state. It shall ignore all the UE Performance report from CU-UP for this UE. After UE turns into RRC_CONNECTED, the U plane will be re-setup and DDDS re-actived.

And when UE is handed over to another cell, the data path to CP-UP will be re-setup, and DDDS terminate. So it is obvious to terminate the data collection.

Proposal 3: It is proposed that CU-CP shall terminate the data collection when UE enters into RRC_INACTIVE or RRC_IDLE state. It shall ignore all the UE Performance report from CU-UP for this UE until it turns into RRC_CONNECTED again.

  - F1 procedures

According to the previous agreement, Energy Cost of gNB-DU needs to be transferred from DU to CU for aggregation.

Also based on our proposal for DL/UL throughput and DL packet loss, the measurements shall be generated on gNB-DU, and sent to CU for aggregation.

Based on these information transferred, a new DATA COLLECTION initial/report procedures similar with Xn interfaces need to be introduced in F1.

Proposal 4: It is proposed to introduce DATA COLLECTION procedures similar with Xn in F1, to transfer Energy Cost from gNB-DU.

Proposal 5: It is proposed to introduce DATA COLLECTION procedures similar with Xn in F1, to transfer UE/Slice level throughput and DL Packet Loss from gNB-DU.

3	Summary

In summary, for AIML in split architecture, following proposals are made:

Proposal 1: It is proposed to introduce DATA COLLECTION procedure similar with Xn in E1, to get the UE/slice level packet delay from CU-UP.

Proposal 2：It is proposed to discuss how to response the DATA COLLECTION REQUEST on UE Packet Loss through E1 from CU-UP to CU-CP.

Proposal 3: It is proposed that CU-CP shall terminate the data collection when UE enters into RRC_INACTIVE or RRC_IDLE state. It shall ignore all the UE Performance report from CU-UP for this UE until it turns into RRC_ACTIVE again.

Proposal 4: It is proposed to introduce DATA COLLECTION procedures similar with Xn in F1, to transfer Energy Cost from gNB-DU.

Proposal 5: It is proposed to introduce DATA COLLECTION procedures similar with Xn in F1, to transfer UE/Slice level throughput and DL Packet Loss from gNB-DU.

4. Reference

- 3GPP RAN3#125bis Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM)

- 3GPP RAN3#127 Chairman’s notes (RAN3_127_agenda_20250221_1300_EOM)

- RP-242385 WID Enhancements for Artificial Intelligence (AI) Machine Learning (ML) for NG-RAN (ZTE, NEC), 3GPP RAN#105

- DRAFT R3-244848 TR38.743 of AIML for NG-RAN_v1 (Release 19)

- TS 38.423, Xn application protocol (XnAP)

- TS 38.300, NR and NG-RAN Overall Description