3GPP TSG-RAN WG3 Meeting #127-bis	R3-252177

Wuhan, China, 7th – 11th April 2025

Title: 	(TP for BLCR to TS 38.401) OAM Requirements for Energy Saving for split architecture

Source: 	CMCC, Samsung, Huawei, NEC, CATT

Agenda item: 	11.4

Document Type:	Other

- Introduction

In RAN3 #125-bis meeting, it has been agreed that:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

In RAN3#126 meeting, LS R3-247907 has been sent to SA5, to ask SA5 to take the above conclusions into account and update their specification accordingly by enabling configuration of the mapping rule to support Energy Cost Mapping for Energy Cost Reporting from the gNB-DU and provide the feedback. And the following agreement was achieved:

In split architecture, the Energy Cost of gNB is the sum of the Energy Cost of its gNB-DUs.

Mapping rule is only configured at gNB-DU and shall be unified for all the gNB-DUs within a defined area.

And SA5 replied with the LS in S5-251087 before #127-bis including the Rel-19 CR to TS 28.541 Enhancements to energy cost mapping rule for split gNB support (S5-250902).

According to the above discussion, TP for BLRCR to TS 38.401 as below was proposed to capture the agreement on OAM requirement for Energy Cost in split architecture.

2. TP for the BLCR to TS 38.401

7.11	Support of AI/ML for NG-RAN

7.11.1	General

The support of AI/ML for NG-RAN is specified in TS 38.300 [2].

In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

For a split gNB, the Energy Cost (EC) of the gNB is the sum of the Energy Cost of its gNB-DUs.

7.11.2	OAM requirements

OAM configures the following on gNB-DU:

-	the minimum and maximum energy consumption values corresponding to the minimum and maximum EC index values respectively, based on an implementation-specific mapping rule, which is unified within a defined area.

3. Reference

- 3GPP RAN3#126 Chairman’s notes (RAN3_126_agenda_20241122_1244_EOM)

- TS 38.401, Architecture description

- R3-247907, LS on energy saving for split gNB

- S5-251087, LS reply to LS on energy saving for split gNB

- S5-250902, Rel-19 CR TS 28.541 Enhancements to energy cost mapping rule for split gNB support