3GPP TSG-RAN WG3 Meeting #127bis       	                      R3-252182

Wuhan, China, 7th-11th April 2025

Agenda item:	10.3.2

Source:	CMCC

Title:	Discussion on SON/MDT for network slicing

Document for:	Discussion

- Introduction

In last meeting [1], RAN3 discussed on SON/MDT for network slicing and sent a LS [2] to RAN2 on slice-based cell reselection.

[Agreement] The UE is unable to find suitable cell in the frequency corresponding to the prioritized NSAG, UE falls back to legacy Cell reselection procedure. LS to RAN2 on the Slice-based Cell Reselection with the issue to be solved as agreed above in R3-250914

This contribution will discuss the potential SON/MDT enhancements for network slicing.

- Discussion

## Issue 1: Slice aware MRO scenarios

Figure 1. The hospital cells are lack of coverage, while the public cells are over coverage

Figure 1 shows an example of the issue that we are facing. Some slices are deployed at a specific area with strict geographical boundaries, e.g., hospital. Such kind of slices are expected to provide continuous services within the specific area. And the public cells outside don’t support the hospital slice.

However, it is observed that, due to imperfect antenna angle planning, the slice may become unavailable in some spots inside the area, especially near the boundaries. As shown in Figure 1, as UE moving close to the boundary, the connected mode UE may handover from a hospital cell to a public cell. Although the radio condition is still good and no RLF is declared, the ongoing PDU session for the hospital slice is rejected by the public cell during the handover procedure.

If such issue can be collected by MDT and reported to OAM, it will help the operator to figure out the problematic cells and adjust the corresponding antenna angle in order to guarantee the hospital slice available inside the hospital area.

However, such kind of slice unavailability within the hospital area is difficult to be visible by OAM. Based on Rel-16 MDT, one way is to configure immediate MDT whenever PDU session is rejected during handover and to let UE report location immediately. But such kind of configuration is a bit of overkill, since normally, when UE is leaving the hospital, the slice should be released or rejected just as a normal case. Such configuration causes too much unnecessary measurement reporting, and it is difficult to distinguish the error cases from large numbers of normal cases.

Observation 1: It is difficult for the current SON/MDT procedure to identify the slice unavailability issue during HO is an error case due to less of coverage or a normal case due to leaving the specific area.

In legacy logged MDT procedure, the network can receive the logged info (e.g. frequency info, cell info and location info) from idle UEs. One possible argument may be that the network can identify the potential issue on coverage from the logged info received from idle UEs. Firstly, we think the cell measurement results in idle mode and connected mode are different, since there may be no L3 filtering for idle UE to derive the cell quality. On the other hand, although it seems the lack of coverage based on the logged info from idle UEs, there may not actual handover occurring for connected mode UE. In addition, the reporting and analysis of massive data from idle UEs will also enormously increase the UE power consumption and the workload of network, but the issue cannot be identified accurately and addressed.

Observation 2: It is difficult to identify the slice unavailability issue during HO based on current logged info from idle UEs.

Figure 2. WLAN AP deployment in the hospital

It is observed that WLAN AP is always deployed inside the hospital, as shown in Figure 2. Since the WLAN name list (SSID) configuration is supported in Rel-16 MDT, the WLAN SSID can be utilized for UE to figure out whether the slice rejection happened inside the hospital area or not. For example, UE can figure out whether UE is still inside the geographical area by detecting the WLAN AP which covers the same area as hospital slice.

Therefore, one candidate solution to address the issue is to let UE report the slice unavailability after HO only if the UE is still inside the geographical area where the slice is intended to provide coverage. This solution means to introduce a new trigger event to report the slice unavailability after HO. Considering the HO will be performed successfully, the information can be reported in the existing reports, e.g. success HO report. On the other hand, a new reports can also be introduced to report the slice unavailability.

Proposal 1: Support to report the slice unavailability after HO only if the UE is still inside the geographical area where the slice is intended to provide coverage. And the information can be reported in the existing report (e.g. success HO report) or a new report.

## Issue 2: Slice request rejection due to slice unavailable in current registration area

During the registration procedure, UE reports the requested NSSAIs which have been subscribed to AMF and then receives an allowed NSSAI list and a rejected NSSAI list from AMF. The allowed list and rejected list are sub-set of the NSSAIs in the requested list. In current specification, only the slices within the allowed NSSAI list are allowed to be initiated by the UE in the registration area.

However, some slices are triggered by higher layer Application (i.e. APP layer). In this case, even if the specific slice is not allowed in the registration area, i.e., the specific slice is not within the allowed NSSAI list, the user may still try to launch the APP for this slice. Such request of APP layer will be rejected by UE NAS layer after the NAS layer checks whether the triggered NSSAI is within the allowed NSSAI list. Such kind of rejection is done inside the UE NAS layer and AS layer cannot be aware of the rejection. So, the network side has no idea how often such rejection happens.

Figure 3. Example for reporting the failure of slice initiation inside the UE

However, we find such rejection information valuable for network side to assess if there is a strong demand of a specific slice. If the operator observes a great number of UEs trying to launch a slice which is not supported in this area, the operator will deploy this slice in the near future to meet the customers’ requirements.

Observation 3: The failure of a slice initiation inside the UE (i.e. the triggered slice is rejected by NAS) is not aware by network side, but it is valuable for the operator to assess the requirements from the mobile subscribers for the slice, which can be used by the operator for network planning of slice deployment.

Proposal 2: Support to report the failure of slice initiation inside the UE, when the higher layer triggers a slice but rejected by UE’s NAS layer due to NSSAI not within the allowed NSSAI list in current registration area.

# 3	Conclusion

Here are the observations and proposals for SON/MDT enhancements for network slicing.

Issue 1: Slice aware MRO secnarios

Observation 1: It is difficult for the current SON/MDT procedure to identify the slice unavailability issue during HO is an error case due to less of coverage or a normal case due to leaving the specific area.

Observation 2: It is difficult to identify the slice unavailability issue during HO based on current logged info from idle UEs.

Proposal 1: Support to report the slice unavailability after HO only if the UE is still inside the geographical area where the slice is intended to provide coverage. And the information can be reported in the existing report (e.g. success HO report) or a new report.

Issue 2: Slice request rejection due to slice unavailable in current registration area

Observation 3: The failure of a slice initiation inside the UE (i.e. the triggered slice is rejected by NAS)  is not aware by network side, but it is valuable for the operator to assess the requirements from the mobile subscribers for the slice, which can be used by the operator for network planning of slice deployment.

Proposal 2: Support to report the failure of slice initiation inside the UE, when the higher layer triggers a slice but rejected by UE’s NAS layer due to NSSAI not within the allowed NSSAI list in current registration area.

# 4	Reference

draft_MeetingReport_RAN3#127_Feb28

R3-250914 LS on SON for Network Slicing