3GPP TSG-RAN WG3 Meeting #127bis	             R3-252185

Wuhan, CN, 7th – 11th Apr 2025

Agenda Item:	16.2

Source:	CMCC

Title:	Discussion on Command Procedure and Signaling

Document for:	Discussion

# 1	Introduction

During last meeting, we agreed the following agreements on Command Procedure and Signaling.

About Command Procedures: - WA: Introduce a NGAP Class 1 Command Request procedure - Introduce COMMAND REQUEST message, which includes Command Request Transfer IE. - Introduce COMMAND RESPONSE message, which includes Command Response Transfer IE. - Introduce COMMAND FAILURE messages, which includes Command Failure Transfer IE. - The above Transfer IEs are transparent to AMF in case of indirect communication. - FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier. - FFS: For command related messages, whether the Correlation Identifier IE if included is the same one as in the related Inventory procedure? - FFS: Command request procedure is a per device procedure or to be used for multiple devices? - FFS: The need for a Command Report procedure? - FFS: Device association between gNB and AIOTF is used in Command procedure.

In this contribution, we will provide our views on command procedure and signaling between A-IoT RAN and A-IoT CN to support command operation.

# 2	Discussion

As discussed in our A-IoT architecture contribution R3-252183, there is no problem to transfer A-IoT command information in NGAP, so we think the work assumption made in last meeting should be changed to an agreement, i.e., introducing a NGAP Class 1 Command Request procedure.

Proposal 1: Change the WA to an agreement, i.e., Introduce a NGAP Class 1 Command Request procedure.

From the SA2 TS 23xyz-020, the command procedure is depicted as follows:

Figure 6.2.3-1: Command Procedure

It can be observed that the A-IoT command operation is performed after the inventory operation, which means the "Command-only" case is not supported by SA2, so RAN3 would follow SA2’s spec and focus on the "A-IoT Inventory and Command" case.

Proposal 2: RAN3 needs to focus on the "A-IoT Inventory and Command" case.

Regarding the service type in command operation, the conclusions documented in SA2 TS 23.xyz-020 and RAN TR 38.769 are referenced as follows:

TR 23.xyz-020 ### 5.2.2 AIoT Command service Three types of Command service operations are supported: Read, Write and Disable. An AF uses the Command service Read operation to retrieve information from a specific AIoT device. An AF uses the Command service Write operation to configure information into a specific AIoT device. An AF uses the Command service Disable operation to permanently disable the capability of the AIoT device(s) to transmit RF signals. TR 38.769 #### 6.5.1.2 Command Command can be sent by the A-IoT CN for a single device. For topology 1, A-IoT RAN node should be able to differentiate between command and inventory. NOTE 1: Whether command can be performed for a group of devices depends on RAN2, SA2 and SA3.

Currently, it can be seen that both SA and RAN reach a common understanding that the command operation can only be sent by the A-IoT CN for a single device. Based on this principle, each Command Request procedure only applies to a single device. As per device procedure, there is no need to distinguish the COMMAND RESPONSE message and COMMAND REPORT message. The command results would be carried in the COMMAND RESPONSE message once the gNB retrieves the results from the AIoT device(s). To allow AF providing multiple identical command operations towards multiple AIoT device(s), RAN3 needs to support AIOTF initiates parallel Command Request procedures for same command service operations towards multiple AIoT devices to the gNB.

Proposal 3: Each Command Request procedure only applies to a single device.

Proposal 4: It is no required to introduce a solely Command Report procedure.

Proposal 5: RAN3 needs to support AIOTF initiates parallel Command Request procedures for same command operations towards multiple AIoT devices to the gNB.

Given that command operation is subsequent to inventory operation, an AIoT device identification mechanism can be established between the gNB and AIoTF which can be analogous to the AMF/RAN UE NGAP ID association in NG interface. This architecture eliminates redundant transmission of lengthy AIoT device identifiers by implementing the following protocol sequence: The gNB can allocate a RAN Device NGAP ID within the INVENTORY REPORT message, which the AIOTF subsequently pairs with a unique AMF Device NGAP ID in the COMMAND REQUEST message. To restrict this association exclusively to combined "A-IoT inventory and command" case, the AIOTF must embed a service type indicator in the INVENTORY REQUEST message, thereby enabling the gNB to conditionally trigger RAN Device NGAP ID allocation and aware a command operation after the inventory operation. Furthermore, the AMF should aware of this association in the indirect deployment.

Proposal 6: An AIoT device identification mechanism can be established between the gNB and AIoTF which can be analogous to the AMF/RAN UE NGAP ID association in NG interface.

Proposal 7: The gNB can allocate a RAN Device NGAP ID within the INVENTORY REPORT message, which the AIOTF subsequently pairs with a unique AMF Device NGAP ID in the COMMAND REQUEST message.

Proposal 8: The AIOTF must embed a service type indicator in the INVENTORY REQUEST message in the "A-IoT Inventory and Command" case.

Since each Command Request procedure only applies to a single device, the AIoT device association can be used to identify the device ID uniquely after establishment. Instead of using Correlation Identifier IE uniquely identifies an A-IoT command service operation within an AIOTF, the Device NGAP ID association can be considered as a replacement for unique identification for A-IoT command service operation to a specific A-IoT device, so there is no requirement to add Correlation Identifier IE in the Command Request procedure.

Proposal 9: There is no requirement to add Correlation Identifier IE in the Command Request procedure.

In the indirect path deployment scenario where the AMF acts as an intermediate node between the AIOTF and the gNB, the AMF may have interfaces with multiple AIOTFs. Consequently, the gNB could receive command request messages from different AIOTFs via a single AMF. Consistent with the rationale applied in the inventory procedure for AIOTF identifier introduction, the AMF shall allocate a distinct AIOTF identifier to unambiguously associate each request with its originating AIOTF. Upon successful initiation of the command operation, the gNB shall report the command result alongside the correlation identifier in the command response message.

Proposal 10: The AIOTF identifier should be included in Command Request procedure.

From the latest SA2 specifications, three command service operation types, i.e., read, write, and disable, are explicitly supported. From RAN3 standpoint, these service type indicators must be made visible to gNB through A-IoT CN exposure, as the command type enables critical gNB scheduling optimizations. With this information, gNB can implement service-specific resource allocation strategies. For read operation, gNB shall allocate uplink resources to retrieve the content from AIoT device. For write or disable operation, gNB will only need provision minimal uplink resources for success/failure acknowledgment signaling. The AIoTF can interpret service type provide by AF and send the command service type indicator to gNB via COMMAND REQUEST message.

Proposal 11: The command service type indicator (Read/Write/Disable) should be included in COMMAND REQUEST message.

## 5.4 Assistance information provided to AIoT RAN node The AIOTF provides the assistance information to the AIoT RAN together with the service operation requests. The AIOTF determines the assistance information provided to the AIoT RAN based on the information received from the AF. The Assistance information is used by the AIoT RAN for performing service operations, e.g. radio resource allocation. The following assistance information may be provided: - AIoT service type (e.g. Inventory, Command); - Optionally, approximate number of AIoT devices based on AF request; - Optionally, approximate D2R message size based on AF request.

Besides the command service operation type, the expected D2R message size plays a critical role in enabling gNB to optimize resource allocation and scheduling strategies. As specified in SA2 CR, the AIOTF provides this parameter to the gNB if this information is available in the AF. With such information, the gNB leverages the estimated D2R message size to enhance uplink resource management for command results. Hence, the COMMAND REQUEST message supports optional inclusion of this approximate message size parameter.

Proposal 12: The COMMAND REQUEST message supports optional inclusion of this approximate message size parameter.

# 3	Conclusion

Based on the discussions mentioned above, in this contribution we provide some observations and proposals on the procedure and procedure between A-IoT RAN and A-IoT CN to support command operation:

Proposal 1: Change the WA to an agreement, i.e., Introduce a NGAP Class 1 Command Request procedure.

Proposal 2: RAN3 needs to focus on the "A-IoT Inventory and Command" case.

Proposal 3: Each Command Request procedure only applies to a single device.

Proposal 4: It is no required to introduce a solely Command Report procedure.

Proposal 5: RAN3 needs to support AIOTF initiates parallel Command Request procedures for same command operations towards multiple AIoT devices to the gNB.

Proposal 6: An AIoT device identification mechanism can be established between the gNB and AIoTF which can be analogous to the AMF/RAN UE NGAP ID association in NG interface.

Proposal 7: The gNB can allocate a RAN Device NGAP ID within the INVENTORY REPORT message, which the AIOTF subsequently pairs with a unique AMF Device NGAP ID in the COMMAND REQUEST message.

Proposal 8: The AIOTF must embed a service type indicator in the INVENTORY REQUEST message in the "A-IoT Inventory and Command" case.

Proposal 9: There is no requirement to add Correlation Identifier IE in the Command Request procedure.

Proposal 10: The AIOTF identifier should be included in Command Request procedure.

Proposal 11: The command service type indicator (Read/Write/Disable) should be included in COMMAND REQUEST message.

Proposal 12: The COMMAND REQUEST message supports optional inclusion of this approximate message size parameter.

# 4	Reference

[1] 3GPP TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things;

[2] 3GPP TR 38.769 V19.0.0, Study on solutions for Ambient IoT (Internet of Things) in NR;

[3] 3GPP TS 23.xyz-020, Architecture support for Ambient power-enabled Internet of Things;