3GPP TSG-RAN WG3 Meeting #127bis	R3-252191

Wuhan, CN, 7th April– 11st April 2025

Agenda Item:       10.2

Source:     ZTE Corporation

Title: 	(TP for SON BLCR 38.401 38.473 38.300 and 38.423)Discussion on MRO

Document for:	Discussion and Approval

1	Introduction

This contribution provides  a further discussion on enhancements to MRO for LTM, CHO-CPAC, and subsequent CPAC.

2	MRO for LTM

2.1 Near failure due to beam failure recovery

RAN3 has received a LS (R2-2501378) on SON for LTM from RAN2. In this LS, RAN2 requests RAN3 to define a network-based solution to address the issue of near failure due to beam failure recovery shortly after a successful LTM. Details are provided below:

RAN2 have discussed the issues of: - BFR shortly after successful LTM cell switch to the wrong beam (“near failure”), and - LTM cell switch failure due to wrong beam On the former issue, RAN2 have acknowledged the issue but agreed not to define a UE-based solution. RAN2 respectfully asks RAN3 to consider defining a network-based solution for this. Furthermore, RAN2 asks RAN3 to see if/how the solution (if specified) could also be used for the latter issue. # Actions To RAN3 - ACTION: RAN2 respectfully asks RAN3 to consider defining a network-based solution for the issue of BFR shortly after successful LTM cell switch (“near failure”). Furthermore, RAN2 asks RAN3 to see if/how the solution (if specified) could also be used for the case of LTM cell switch failure due to wrong beam.

The BFR shortly after successful LTM cell switch to the wrong beam (“near failure”) is described in the following Figure 1.

Figure 1. A near-failure due to LTM to Right Cell and Wrong (or Sub-optimal) Beam.

What distinguishes LTM from legacy HO is that the network must determine not only the most suitable cell, but also the most appropriate beam for the UE to attach to. Let’s dive into the scenario in above figure.

-Step 1: Using RACH-less LTM, the UE is switched from Cell 1/Beam 1-1 to Cell 2/Beam 2-1.

-Step 2: The UE successfully accesses Cell 2/Beam 2-1 but experiences a beam failure shortly thereafter.

-Step 3: The UE initiates a BFR and successfully recovers the connection via Cell 2/Beam 2-2.

In this example, Cell 2/Beam 2-2 proves to be a better target than Cell 2/Beam 2-1. Consequently, although the source node chose the correct cell (i.e., Cell 2), it selected an unsuitable beam (Beam 2-1).

As for the User Plane Interruption, we have the following analysis:

Phase 1: RACH-less LTM.

Phase 2: Limited UL/DL. The UE may be able to transmit and receive data, but data transmission could still suffer from the near failure beam. Eventually, the UE may declare a beam failure and trigger the BFR procedure.

Phase 3: BFR. This phase primarily involves a RACH procedure in the target cell, with the UE selecting a beam based on its measurements.

Because this sequence of events combines a RACH-less LTM procedure with a normal RACH procedure, it can potentially lead to a longer interruption period than a standard (non-LTM) procedure—effectively nullifying the intended benefit of RACH-less LTM. Furthermore, If left it uncorrected, the LTM strategy may significantly increase the risk of future LTM failures due to potentially incorrect or sub-optimal beam selection.  That is, this issue may propagate, leading to potential LTM handover failures for other UEs attempting to switch to the same target cell in the future.

Therefore, we propose to define a new “near-failure” case where a BFR occurs shortly after a successful LTM cell switch, and the UE selects a different beam within the same target cell.

Proposal 1 : To define a new “near-failure” case where a BFR occurs shortly after a successful LTM cell switch into Stage 2.

Since RAN2 has asked RAN3 to consider defining a network-based solution for the issue of BFR shortly after successful LTM cell switch (“near failure”), from our point of view,  the detection of such an LTM near failure event may be performed by the target DU using a time threshold approach. In this approach, after the UE is switched from the source DU to the target DU, the target DU monitors uplink or downlink data transmission on the target beam. Upon detecting a successful transmission, it initiates a time threshold evaluation for Beam Failure Recovery (BFR). If, before this threshold is exceeded, the target DU detects the UE transmitting a RACH (Random Access Channel) preamble on a different beam (i.e., the recovery beam) followed by an uplink MSG3 transmission on the same beam, it determines that the UE has experienced a BFR.

If the LTM near failure event is detected and the recovery beam is among the candidate beams, it is evident that the source DU is responsible for the incorrect beam selection in the cell switch command. Otherwise,  the target DU is responsible for the preparation of candidate beams.

Proposal 2 :  The target DU detects the LTM near failure event caused by BFR. Upon detecting this event, the target DU performs an initial analysis. If the recovery beam is among the candidate beams, the source DU is responsible for the incorrect beam selection. Otherwise, the target DU is responsible for the preparation of candidate beams.

If the source DU is responsible for the near failure event, the target DU shall provide relevant information, such as the C-RNTI and recovery beam information, to the source DU. This allows the source DU to retrieve the UE context, including L1/L2 measurements that triggered the LTM switch. Based on these information, the source DU can optimize beam selection for future LTM cell switch.

Proposal 3 : If the source DU is responsible for the near failure event, the target DU shall provide relevant information, such as the C-RNTI and recovery beam information, to the source DU.

To facilitate the above information exchange, the target DU may send a message, such as a DU-CU ACCESS AND MOBILITY INDICATION message, to the CU to report relevant information of the LTM near failure event. The CU can then forward this information to the source DU via an ACCESS AND MOBILITY INDICATION message.

Proposal 4 :  The target DU sends a message, such as a DU-CU ACCESS AND MOBILITY INDICATION message, to the CU to report relevant information of the LTM near-failure event. The CU can then forward this information to the source DU via an ACCESS AND MOBILITY INDICATION message.

Based on the above proposals, we provide the corresponding Stage 2 TP for TS 38.300 and TS 38.401, as well as the Stage 3 TP for TS 38.473, in the annex for the network based solution.

Proposal 5 : RAN3 to agree the  corresponding Stage 2 TP for TS 38.300 and TS 38.401, as well as the Stage 3 TP for TS 38.473, in the annex (A.1-A.3) for the network based solution for LTM near failure.

2.2 failure due to wrong beam

In the mentioned LS in 2.1,

RAN2 have discussed the issues of:

- BFR shortly after successful LTM cell switch to the wrong beam (“near failure”), and

- LTM cell switch failure due to wrong beam

On the former issue, RAN2 have acknowledged the issue but agreed not to define a UE-based solution. RAN2 respectfully asks RAN3 to consider defining a network-based solution for this.

As RAN2 is still discussing this LTM cell switch failure due to wrong beam issue, we can wait for RAN2 to provide a UE based solution.

Proposal 6：Wait for RAN2 to provide a UE based solution for LTM cell switch failure due to wrong beam.

2.3 SHR handling

The existing T310, T312, and T304 timers can be reused as SHR triggers in LTM near-failure scenarios. Following current MRO principles, upon receiving the SHR, the CU would first analyze the cause of the near-failure and then forward the SHR to the source DU if triggered by T310 or T312 (Configured at source DU), or to the target DU if triggered by T304 (Configured at target DU). We suggest to capture this SHR handling approach  in TS 38.401 stage 2. Additionally, if a trigger cause for beam failure recovery is introduced in the SHR report, this stage 2 would need to be updated accordingly.

Proposal 7: To capture the SHR handling approach in TS 38.401 stage 2. (The corresponding TP is provided in Annex B.)

2.4 RACH-less LTM

In the last meeting, the issue on TA acquisition was discussed. we provide solution and TPs in [1]

2.5 Subsequent LTM

If subsequent LTM is configured for UE, various report might be generated, in a higher frequency. However, the failure information or the sub-optimal success report (if there are any), might be over-written very soon. If subsequent LTM goes on and on, network might fail to fetch the report in time, and won't be able to spot the issues in a subsequent LTM (and therefore to optimize it).  (Note: a similar issue also exists in SCPAC)

Proposal 8: RAN3 to discuss the issue of frequently over-written report for MRO in subsequent LTM.  (Note: a similar issue also exists in SCPAC).

In the last meeting, the issue on ping-pong optimization for subsequent LTM was discussed, we provide solution and TPs in [2].

3	MRO for CHO-CPAC

3.1 Failure cases :

3.1.1 Case 1c, 2c, 2c, 3c, 7c, 8c:

Agreements at RAN3#126 meeting:

Case 1a), 2a), 3a), 9a): Too late handover

Case 1b), 2b), 3b), 9b): Too late CPC execution

Case 7a), 8a): Too early HO or wrong cell Handover to wrong cell

Case 7b), 8b): Too Early CPC/CPA Execution or CPC/CPA Execution to wrong PSCell

According to the agreement on failure cases, no consensus was reached on cases where PCell failure and PScell failure occur simultaneously, such as Cases 1c, 2c, 3c, 7c, and 8c. However, we believe these cases are common, therefore, we propose considering Cases 1c, 2c, 3c, 7c, and 8c in Rel-19.

Proposal 9: Consider Cases 1c, 2c, 3c, 7c, and 8c in Rel-19.

3.1.2 Case 9:

Agreements at RAN3#126 meeting:

Case 1a), 2a), 3a), 9a): Too late handover

Case 1b), 2b), 3b), 9b): Too late CPC execution

Case 9 has been agreed to capture as failure cases. It means CHO execution condition and the associated CPAC execution condition cannot be met simultaneously.

| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |
| --- | --- | --- |

During the RAN3#125bis meeting, the following enhancement of RLF were agreed:

  - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

Regarding the above enhancement, if applied to Case 9, this would seems that before an RLF encountered, the UE might record a list of candidate cells that individually met their conditions, while their paired candidate cells failed to meet. Clearly, some cells in this list would likely appear multiple times. However, this approach seems unnecessary. the UE only needs to record the number of times each specific candidate cell met the conditions individually before an RLF encountered. This way, MRO can identify which candidate cell had a higher number, helping to analyze why its configured paired cell was consistently unable to meet the conditions at the same time.

Proposal 10: In order to support MRO for case 9, enhance RLF report to include:

- Identifiers of candidate PCell(s) or PSCell(s) that met execution conditions before RLF, while their paired cells did not;

- The number of times each candidate cell met the execution conditions.

3.1.3 Case 10:

During past meeting, Companies propose to consider case 10: UE meets the execution conditions of non-associated candidate PCells and PSCells before encountering an RLF or SCG Failure, which may be different from case 9.

In Case 10, the UE needs to record the identifiers of these non-associated candidate PCell and PSCell pairs that simultaneously met the execution conditions before the RLF occurred.Similar to Case 9, each pair only needs to be recorded once, along with the number of times it met the conditions.

Proposal 11: In order to support MRO for case 10, enhance RLF report to include:

- Identifiers of non-associated candidate PCell and PSCell pairs that simultaneously met the execution conditions before the RLF occurred;

- The number of times each non-associated cell pair met the execution conditions.

3.1.4 Too later CHO failure:

During the RAN3#127bis meeting, it was agreed to reuse the Handover Report message to allow the last serving MN to inform the (candidate) target MN of a failure in the case of a too-late CHO. Whether additional information should be included in the Handover Report remains FFS..

In case of too late CHO execution, reusing Handover Report message from the last serving MN to the relevant candidate MN.

FFS on whether Handover Report message is needed for case 7a.

FFS on the additional information included in handover report.

In the current XnAP handover report message, there is no definition for a "too late handover" trigger type, as shown below. This is because, in the case of legacy HO and CHO, the last serving gNB is the node responsible for too later handover failure.

9.1.3.17	HANDOVER REPORT

This message is sent by NG-RAN node1 to NG-RAN node2 to report a handover failure event, or other critical mobility problem.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter-system ping-pong. …) |  | YES | ignore |

Proposal 12: Extend the report type in the XnAP Handover Report message to include the "Too Late Handover" type for CHO with SCG. ( the corresponding TP for TS38.423 is provided in Annex C.)

For case 3a),  the too late problem may be brought by the (candidate) target MN  due to unmet CPC/CPA execution conditions configured by the (candidate) target MN. In case 9a), the too late problem may also be brought by the source MN or  the (candidate) target MN or both,  as the CHO execution condition and the associated CPAC execution condition cannot be met simultaneously before an RLF occurs.

It was agreed that the Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered, However, is it necessary to send a Handover Report to every NG-RAN node (candidate MN) associated with each recorded candidate cell?

As discussed in Section 3.1.2, if the UE records the number of times each specific candidate cell individually met the conditions in the RLF report, MRO can analyze which candidate cells were recorded more frequently and identify the nodes most likely responsible for the failure. This approach not only helps identify the most relevant candidate MNs but also minimizes unnecessary forwarding of Handover Report messages.

Proposal 13: In case of too late HO for CHO with SCG, the last serving MN may need to forward RLF via Handover report message to the candidate MN(s) which may need optimization. RAN3 should discuss whether there is an issue on this message being excessively sent to unnecessary candidate MNs and how to avoid this problem.

FFS on whether Handover Report message is needed for case 7a.

Some company thinks that in case 7a , if the CHO failure is caused by the delayed fulfillment of CPA/CPC execution conditions, then the RLF report  may also need to be forwarded to candidate target MN. This issue involves ensuring that CHO and CPA/CPC meet their respective conditions as simultaneously as possible. However, we propose that the optimization of CHO execution should primarily be handled by the source MN to address the problem, rather than requiring optimization from both the source MN and candidate MN. So, we think the handover report is not needed for case 7a.

Proposal 14: The is no need to forward RLF to candidate MN(s) for case 7a.

3.2 Near failure case:

Based on current specification, in addition to CHO-CPAC configuration, the network can provide a complementary CHO-only configuration /execution condition to the same UE.  And in case the CHO-CPAC execution conditions are not meet, instead the CHO-only configuration can be used when the execution condition is meet. In this case, no failure report will create while the  CHO-CPAC configuration seems useless. From RAN3 point view, the useless of CHO-CPAC with CHO-only can be taken as one type of near failure case.

Proposal 15: CHO only executed as  complementary for  configuration of CHO with candidate SCG(s) can be concluded as near failure case.

4	MRO for SCPAC

In past RAN3 meetings, the issue was raised of whether to support forwarding the SCG UHI to the target node during the S-CPAC procedure to avoid ping-pong immediately. However, we believe this type of real-time optimization introduces significant complexity of SCPAC reconfiguration at network side and  is not aligned with the principles of MRO, which focuses on long-term optimization. Additionally, it would require modifications to the UE's SCPAC configuration whenever a ping-pong is detected..

Proposal 16:  The real-time reconfiguration of SCPAC to avoid ping-pong is not aligned with the principles of MRO, which focus on long-term optimization. As RAN2 has lowered the priority of SCPAC, we propose that RAN3 close the discussion on this optimization

At the RAN3#125 meeting, it was agreed that "Reusing SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN.". In Rel-18 MRO, the MN may include the CPC candidate cell list and CPC execution conditions to the SN via the SCG FAILURE INFORMATION REPORT over Xn for failure analysis.To differentiate the CPC configuration over Xn, RAN3 discuss whether to introduce  SCPAC configuration into XnAP SCG FAILURE INFORMATION REPORT message.

Proposal 17:RAN3 to discuss whether to introduce the SCPAC configuration into XnAP SCG FAILURE INFORMATION REPORT message.

- 5 Conclusion:

The following proposals are provided for MRO for LTM:

Proposal 1 : To define a new “near-failure” case where a BFR occurs shortly after a successful LTM cell switch into Stage 2.

Proposal 2 :  The target DU detects the LTM near failure event caused by BFR. Upon detecting this event, the target DU performs an initial analysis. If the recovery beam is among the candidate beams, the source DU is responsible for the incorrect beam selection. Otherwise, the target DU is responsible for the preparation of candidate beams.

Proposal 3 : If the source DU is responsible for the near failure event, the target DU shall provide relevant information, such as the C-RNTI and recovery beam information, to the source DU.

Proposal 4 :  The target DU sends a message, such as a DU-CU ACCESS AND MOBILITY INDICATION message, to the CU to report relevant information of the LTM near-failure event. The CU can then forward this information to the source DU via an ACCESS AND MOBILITY INDICATION message.

Proposal 5 : RAN3 to agree the  corresponding Stage 2 TP for TS 38.300 and TS 38.401, as well as the Stage 3 TP for TS 38.473, in the annex (A.1-A.3) for the network based solution for LTM near failure.

Proposal 6：Wait for RAN2 to provide a UE based solution for LTM cell switch failure due to wrong beam.

Proposal 7: To capture the SHR handling approach in TS 38.401 stage 2. (The corresponding TP is provided in Annex B.)

Proposal 8: RAN3 to discuss the issue of frequently over-written report for MRO in subsequent LTM.  (Note: a similar issue also exists in SCPAC).

The following proposals are provided for MRO for CHO-CPAC:

Proposal 9: Consider Cases 1c, 2c, 3c, 7c, and 8c in Rel-19.

Proposal 10: In order to support MRO for case 9, enhance RLF report to include:

- Identifiers of candidate PCell(s) or PSCell(s) that met execution conditions before RLF, while their paired cells did not;

- The number of times each candidate cell met the execution conditions.

Proposal 11: In order to support MRO for case 10, enhance RLF report to include:

- Identifiers of non-associated candidate PCell and PSCell pairs that simultaneously met the execution conditions before the RLF occurred;

- The number of times each non-associated cell pair met the execution conditions.

Proposal 12: Extend the report type in the XnAP Handover Report message to include the "Too Late Handover" type for CHO with SCG. ( the corresponding TP for TS38.423 is provided in Annex C.)

Proposal 13: In case of too late HO for CHO with SCG, the last serving MN may need to forward RLF via Handover report message to the candidate MN(s) which may need optimization. RAN3 should discuss whether there is an issue on this message being excessively sent to unnecessary candidate MNs and how to avoid this problem.

Proposal 14: The is no need to forward RLF to candidate MN(s) for case 7a.

Proposal 15: CHO only executed as  complementary for  configuration of CHO with candidate SCG(s) can be concluded as near failure case.

The following proposals are provided for MRO for SCPAC:

Proposal 16:  The real-time reconfiguration of SCPAC to avoid ping-pong is not aligned with the principles of MRO, which focus on long-term optimization. As RAN2 has lowered the priority of SCPAC, we propose that RAN3 close the discussion on this optimization

Proposal 17:RAN3 to discuss whether to introduce the SCPAC configuration into XnAP SCG FAILURE INFORMATION REPORT message.

- Reference

- R3-252192 (TP for SON BLCR 38.300 and38.473)MRO for LTM- invalid TA ZTE

- R3-252193 (TP for SON BLCR 38.300 38.40138.413 and 38.473) LTM Ping-Pong Avoidance ZTE

# Annex A.1: TP for TS38.401 for LTM BFR

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell.

After a successful LTM cell switch, if the target gNB-DU detects an LTM near failure event,specifically, Beam Failure Recovery(BFR) shortly after successful LTM cell switch to the wrong beam, the target DU may send the near failure information to the source gNB-DU via the gNB-CU in the case of the source gNB-DU is responsible for the wrong beam selection.

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>

# Annex A.2: TP for TS38.300 for LTM BFR

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

### 15.5.2	Support for Mobility Robustness Optimization

#### 15.5.2.1	General

Mobility Robustness Optimisation aims at detecting and enabling correction of following problems:

-	Connection failure due to intra-system or inter-system mobility;

-	Inter-system Unnecessary HO (too early inter-system HO from NR to E-UTRAN with no radio link failure);

-	Inter-system HO ping-pong;

-	PSCell change failure;

-	Inter-system voice fallback failure;

-	Fast MCG recovery failure or a sub-optimal Fast MCG recovery.

MRO provides means to distinguish the above problems from NR coverage related problems and other problems, not related to mobility.

For detection of a sub-optimal successful handovers, MRO additionally enables observability of:

-	Successful HO due to intra-NR mobility;

-	Successful HO due to inter-RAT mobility.

For detection of a sub-optimal successful PSCell addition/change, MRO additionally enables observability of:

-	Successful PSCell addition/change.

For detection of a sub-optimal successful LTM cell switch, MRO additionally enables observability of:

-	Successful LTM cell switch.

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGES >>>>>>>>>>>>>>>>>>>>

#### 15.5.2.9	Successful LTM cell switch

One of the functions of Mobility Robustness Optimization is to detect LTM near failure that occur due to Beam Failure Recovery(BFR). This problem is defined as follows:

LTM near failure: the target gNB-DU detects that the UE has experienced BFR from a failure condition associated with the initially used target beam, shortly after the successful LTM cell switch,

Detection mechanism:

The target DU detects the LTM near failure event caused by BFR. Upon detecting this event, the target DU performs an initial analysis. If the recovery beam is among the candidate beams, the source DU is responsible for the wrong beam selection. Otherwise, the target DU is responsible for the preparation of candidate beams.

If the source gNB-DU is responsible for the LTM near failure event, target gNB-DU may send a LTM near failure information  to it by triggering:

-	DU-CU access and mobility indication procedure over F1;

-	Access and mobility indication procedure over F1.

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>

# Annex A.3: TP for TS38.473  for LTM BFR

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

## 8.11	Self Optimisation Support procedures

### 8.11.1	Access and Mobility Indication

#### 8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

#### 8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RA Report List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the LTM Near Failure Information IE the gNB-DU shall take it into account for optimisation of LTM cell switch related parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful PSCell Change Report Information List IE, the gNB-DU may take it into account for optimisation of PSCell change/addition related parameters.

#### 8.11.1.3	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGES >>>>>>>>>>>>>>>>>>>>

### 8.11.2	DU-CU Access and Mobility Indication

#### 8.11.2.1	General

This procedure is initiated by the gNB-DU to send the Access and Mobility related Information to the gNB-CU.

The procedure uses non-UE-associated signalling.

#### 8.11.2.2	Successful Operation

Figure 8.11.2.2-1: DU-CU Access and Mobility Indication procedure. Successful operation

The DU-CU Access and Mobility Indication procedure is initiated by DU-CU ACCESS AND MOBILITY INDICATION message sent from the gNB-DU to the gNB-CU.

If the DU-CU ACCESS AND MOBILITY INDICATION message contains the DL LBT Failure Information List IE, the gNB-CU shall take it into account for optimisation of mobility parameters.

If the DU-CU ACCESS AND MOBILITY INDICATION message contains the LTM Near Failure Information IE, the gNB-CU shall notify the related source gNB-DU of the LTM Near Failure Information.

#### 8.11.2.3	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGES >>>>>>>>>>>>>>>>>>>>

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| Successful PSCell Change Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful PSCell Change Report information Item |  | 1..<maxnoofSuccessfulPSCellChangeReports> |  |  | - |  |
| >>Successful PSCell Change Report Container | M |  | OCTET STRING | Includes the SuccessPSCell-Report IE as defined in TS 38.331 [8]. | - |  |
| LTM Near Failure Information |  | 0..1 |  | Includes the LTM near failure detected by the network | YES | ignore |
| >UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | - |  |
| >Recovery Beam information | M |  | FFS |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRAReports | Maximum no. of RA Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |
| maxnoofSuccessfulPSCellChangeReports | Maximum no. of Successful PSCell Change Reports. Value is 64. |

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGES >>>>>>>>>>>>>>>>>>>>

#### 9.2.10.2	DU-CU ACCESS AND MOBILITY INDICATION

This message is sent by the gNB-DU to provide access and mobility information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DL LBT Failure Information List |  | 0..1 |  |  | YES | ignore |
| > DL LBT Failure Information Item |  | 1 .. <maxnoofLBTFailureInformation> |  |  | - |  |
| >>DL LBT Failure Information | M |  | 9.3.1.327 |  | - |  |
| LTM Near Failure Information |  | 0..1 |  | Includes the LTM near failure detected by the network | YES | ignore |
| >UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >C-RNTI | O |  | 9.3.1.32 |  | - |  |
| >Recovery Beam information | M |  | FFS |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLBTFailureInformation | Maximum no. of UEs for which LBT Failure Information is provided, the maximum value is 64. |

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>

Annex B: 	TP for TS38.401 for LTM SHR handing

## 7.X	MRO support for LTM

The gNB-CU receives a RLF report associated to an LTM mobility event from the UE or via the Failure indication message over Xn. The gNB-CU performs initial analysis and in case of failure due to inappropriate cell switch triggering may forward the RLF report to the last serving gNB-DU in case of too late LTM, or to the source gNB-DU in case of too early LTM or LTM to wrong cell.

The gNB-CU receives a SHR report associated to an LTM mobility event from the UE or via the Access and Mobility message over Xn or via Downlink RAN configuration transfer procedure over NG. The gNB-CU may forward the SHR report to the source gNB-DU  in case of the SHR triggered by the T310 and T312 timer,or to the target gNB-DU in case of the SHR triggered by the T304 timer.

Annex C: 	TP for TS38.423 for too late CHO with SCG

<<<<<<<<<<<<<<<<<<<< START OF CHANGES >>>>>>>>>>>>>>>>>>>>

### 8.4.8	Handover Report

#### 8.4.8.1	General

The purpose of the Handover Report procedure is to transfer mobility related information between NG-RAN nodes.

The procedure uses non UE-associated signalling.

#### 8.4.8.2	Successful Operation

Figure 8.4.8.2-1: Handover Report, successful operation

NG-RAN node1 initiates the procedure by sending the HANDOVER REPORT message to NG-RAN node2. When receiving the message NG-RAN node2 shall assume that a mobility-related problem was detected.

If the Handover Report Type IE is set to "HO too early" or "HO to wrong cell", then NG-RAN node1 indicates to NG-RAN node2 that, following a successful handover from a cell of NG-RAN node2 to a cell of NG-RAN node1, a radio link failure occurred and the UE attempted RRC Re-establishment or re-connected either at the original cell of NG-RAN node2 (Handover Too Early), or at another cell (Handover to Wrong Cell). The detection of Handover Too Early and Handover to Wrong Cell events is made according to TS 38.300 [9].

The HANDOVER REPORT message may include:

-	the Mobility Information IE, if the Mobility Information IE was sent for this handover from NG-RAN node2 (in case the NG-RAN node2 provided it more than once, the most recent Mobility Information IE is included in the HANDOVER REPORT message);

-	the Source cell C-RNTI IE.

-	the CHO Configuration IE, if the CHO Configuration IE was sent for this handover from NG-RAN node2.

If received, NG-RAN node2 uses the above information according to TS 38.300 [9].

If the Handover Report Type IE is set to "Inter-system ping-pong", then NG-RAN node2 shall deduce that a completed handover from a cell of NG-RAN node2 to a cell in another system might have resulted in an inter-system ping-pong and the UE was successfully handed over to a cell of NG-RAN node1 (indicated with Target cell CGI IE).

If the Handover Report Type IE is set to "HO too late", then NG-RAN node1 indicates to NG-RAN node2 that, a handover execution is not initiated for the UE prior to the connection failure. The detection of Handover Too late events is made according to TS 38.300 [9].

If the Target cell C-RNTI IE and the Time Since Failure IE are received, NG-RAN node2 uses the information as specified in TS 38.300 [9].

Interaction with the Failure Indication procedure:

If NG-RAN node1 receives a UE RLF Report from an NG-RAN node via the FAILURE INDICATION message, as described in TS 38.300 [9], NG-RAN node1 may also include it in the UE RLF Report Container IE included in the HANDOVER REPORT message.

#### 8.4.8.3	Unsuccessful Operation

Not applicable.

#### 8.4.8.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGES >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.17	HANDOVER REPORT

This message is sent by NG-RAN node1 to NG-RAN node2 to report a handover failure event, or other critical mobility problem.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Handover Report Type | M |  | ENUMERATED (HO too early, HO to wrong cell, Inter-system ping-pong,HO too late. …) |  | YES | ignore |
| Handover Cause | M |  | Cause 9.2.3.2 | Indicates handover cause employed for handover from NG-RAN node 2 | YES | ignore |
| Source cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source cell for handover procedure (in NG-RAN node 2) | YES | ignore |
| Target cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of target cell for handover procedure (in NG-RAN node 1). If the Handover Report Type is set to “Inter-system ping-pong”, it contains the target cell of the inter system handover from the other system to NG-RAN node 1 cell | YES | ignore |
| Re-establishment cell CGI | C- ifHandoverReportType HoToWrongCell |  | Global Cell Identity 9.2.2.73 | CGI of cell where UE attempted re-establishment or where UE successfully re- connected after the failure | YES | ignore |
| Target cell in E-UTRAN | C- ifHandoverReportType Intersystempingpong |  | OCTET STRING | Encoded according to Global Cell ID in the Last Visited E-UTRAN Cell Information IE, as defined in TS 36.413 [31] | YES | ignore |
| Source cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the source NG-RAN node (in NG-RAN node 2) | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information provided in the HANDOVER REQUEST message or in the SN STATUS TRANSFER message from NG-RAN node 2. | YES | ignore |
| UE RLF Report Container | O |  | UE RLF Report 9.2.2.59 | The UE RLF Report Container IE received in the FAILURE INDICATION message. | YES | ignore |
| CHO Configuration | O |  | 9.2.2.76 |  | YES | ignore |
| Target cell C-RNTI | O |  | BIT STRING (SIZE (16)) | C-RNTI allocated at the target NG-RAN node. | YES | ignore |
| Time Since Failure | O |  | INTEGER (0.. 172800, ...) | Corresponds to the TimeSinceFailure IE received from the UE in RLF Report as defined in TS 36.331 [14]. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifHandoverReportType HoToWrongCell | This IE shall be present if the Handover Report Type IE is set to the value "HO to wrong cell" |
| ifHandoverReportType Intersystempingpong | This IE shall be present if the Handover Report Type IE is set to the value "Inter-system ping-pong" |

<<<<<<<<<<<<<<<<<<<< END OF CHANGES >>>>>>>>>>>>>>>>>>>>