3GPP TSG-RAN WG3 Meeting #127bis	R3-252194

Wuhan, CN, 7th April– 11st April 2025

Title:	Vendor specific trace record support

Agenda Item:	8.1

Source:	ZTE corporation

Document for:	discussion

1	Introduction

This is a general paper discussing LS from SA5 in R3-251530.

2	Discussion

SA5 found the limitations in the current 3GPP SA5 specifications regarding trace record collection and proposes enhancements to better support vendor-defined content. Trace record content (measurements, KPIs, events) is limited to standardized messages defined in TS 32.423.Reporting control is tightly coupled to specific record types (TS 28.622).While vendors can add content to existing standardized records, configuration support for this vendor-specific content is limited. SA5 would like to enhance the Measurement Data Transfer (MDT) mechanism to support vendor-specific content and has agreed to update SA5 specifications.

In the attached CR S5-250077, SA5 decided to reused the same Trace Depth. The new introduce parameters highlight in blue.

| Trace Depth | Meaning |
| --- | --- |
| Minimum | Recording of some IEs in the signalling messages plus any vendor specific extensions to this definition, in decoded format. |
| Medium | Recording of some IEs in the signalling messages together with the radio measurement IEs plus any vendor specific extensions to this definition, in decoded format. |
| Maximum | Recording entire signalling messages plus any vendor specific extensions to this definition, in encoded format. |
| MinimumWithoutVendorSpecificExtension | Recording of some IEs in the signalling messages in decoded format. |
| MediumWithoutVendorSpecificExtension | Recording of some IEs in the signalling messages together with the radio measurement IEs in decoded format. |
| MaximumWithoutVendorSpecificExtension | Recording entire signalling messages in encoded format. |
| MinimumOnlyVendorSpecificTraceRecord | Recording of vendor specific trace records only, at vendor defined minimum level |
| MediumOnlyVendorSpecificTraceRecord | Recording of vendor specific trace records only, at vendor defined medium level |
| MaximumOnlyVendorSpecificTraceRecord | Recording of vendor specific trace records only, at vendor defined maximum level |

In current specification, the Trace Depth has already defined in S1/X2/NG/XN/F1/E1.

| Trace Depth | M |  | ENUMERATED (minimum, medium, maximum, MinimumWithoutVendorSpecificExtension, MediumWithoutVendorSpecificExtension, MaximumWithoutVendorSpecificExtension, …) | Defined in TS 32.422 [23]. | – |  |
| --- | --- | --- | --- | --- | --- | --- |

And it is note in current specification, the the Trace Depth IE will be ignore by RAN node when MDT activation is used for MDT only. This will prevent Vendor specific trace used in RAN.

If the Trace Activation IE is included in the HANDOVER REQUEST message which includes - the MDT Activation IE set to "Immediate MDT and Trace", then the target NG-RAN node shall if supported, initiate the requested trace session and MDT session as described in TS 32.422 [23]. - the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", the target NG-RAN node shall, if supported, initiate the requested MDT session as described in TS 32.422 [23] and the target NG-RAN node shall ignore the Interfaces To Trace IE, and the Trace Depth IE.

Observation 1: The Trace Depth IE will be ignore by RAN node when MDT activation is used for MDT only. This will prevent Vendor specific trace used in RAN.

In order to solve the issue, the current specification should be able to differentiate the RAN behavior of legacy Trace depth parameters and new introduced Trace depth parameter.

For legacy Trace depth parameters, "minimum", "medium", "maximum", "MinimumWithoutVendorSpecificExtension", "MediumWithoutVendorSpecificExtension", "MaximumWithoutVendorSpecificExtension", the RAN node should following the legacy behavior to ignore the Trace Depth IE in case of MDT only case.

For new introduced Trace depth parameters, such as "MinimumOnlyVendorSpecificExtension", "MediumOnlyVendorSpecificExtension", "MaximumOnlyVendorSpecificExtension", in order to support the new feature, the RAN node should allow new behavior where the RAN node should not ignore the Trace Depth IE in case of MDT only case.

Observation 2: In order to support Vendor specific trace record feature, the RAN behavior in case of Trace depth parameters set to "MinimumOnlyVendorSpecificExtension", "MediumOnlyVendorSpecificExtension", "MaximumOnlyVendorSpecificExtension" need to be defined independently.

In addition, the intention of SA5 is to support Vendor specific trace record may include multiple jobtype. Based on 2.422, trace collection is configured via a common TraceJob. A subset of the parameters, and a subset of their allowed values includes Trace, MDT, RCEF, RLF, RRC.

For sure Trace can be extended for Vendor specific.

For Immediate MDT, the function can also be extended for Vendor specific.

For logged MDT(including logged MDT and MBS logged MDT), due to no trace depth in Uu interface, then the logged MDT can not extended for Vendor specific.

For these functions RCEF, RLF, RRC, due to no trace depth in Uu interface, all these functions can not extended for Vendor specific.

Observation 3: Only Trace and Immediate MDT can be extended for Vendor specific in RAN.

Based on above analysis, an example of update the specification is shown as below:

In this way to support the Vendor specific trace record in S1/X2/NG/XN/F1/E1 is feasible.

Proposal : To support Vendor specific trace record in S1/X2/NG/XN/F1/E1 in [2][3][4][5][6][7] .

3	Conclusion

This document proposes:

Observation 1: The Trace Depth IE will be ignore by RAN node when MDT activation is used for MDT only. This will prevent Vendor specific trace used in RAN.

Observation 2: In order to support Vendor specific trace record feature, the RAN behavior in case of Trace depth parameters set to "MinimumOnlyVendorSpecificExtension", "MediumOnlyVendorSpecificExtension", "MaximumOnlyVendorSpecificExtension" need to be defined independently.

Observation 3: Only Trace and Immediate MDT can be extended for Vendor specific in RAN.

Proposal 1: To support Vendor specific trace record in S1/X2/NG/XN/F1/E1 in [2][3][4][5][6][7].

Proposal 2: RAN3 to reply SA5 in LS-out in [8].

4	References

[1]  R3-251530 LS on Vendor Specific Trace Record, Ericsson

[2]  R3-252195 Introduce Vendor Specific Trace Record in Trace depth in NGAP, ZTE corporation

[3]  R3-252196 Introduce Vendor Specific Trace Record in Trace depth in XnAP, ZTE corporation

[4]  R3-252197 Introduce Vendor Specific Trace Record in Trace depth in F1AP, ZTE corporation

[5]  R3-252198 Introduce Vendor Specific Trace Record in Trace depth in E1AP, ZTE corporation

[6]  R3-252199 Introduce Vendor Specific Trace Record in Trace depth in S1AP, ZTE corporation

[7]  R3-252200 Introduce Vendor Specific Trace Record in Trace depth in X2AP, ZTE corporation

[8]  R3-252201 Reply LS on Vendor Specific Trace Record, ZTE corporation