3GPP TSG-RAN WG3 Meeting #127-bis	R3-252211

Wuhan, China, April 7th-11th, 2025

Agenda Item:	14.3

Source:	CSCN

Title:	Discussion on support of regenerative payload for NR NTN

Document for:	Discussion

1	Introduction

In the RAN3 #127 meeting, there are some discussions on OAM-based TAI list configuration and have concensus on the conclusion. From RAN3 perspective, there still some remaining open issues shown as blow need to be further discussed [1].

Issue 1: Location-based CHO

Issue 2: Support of RRC_INACTIVE

In this contribution, we will further discuss the corresponding issues on the support of regenerative payload.

2	Discussion

Issue 1: Location-based CHO

- In the previous RAN3 meeting, some companies proposed to enhance location-based CHO. From the perspective of RAN3, it is necessary to further consider how to better support the mobility of location-based CHO between gNBs in regenerative payload. Under the assumption that there is an inter-satellite link (ISL) between two satellites, we prefer to enhance location-based CHO by transferring UE coarse location information via the Xn interface between the source and candidate cells.

- In IoT NTN, it has been demonstrated that after the eNB acquires the coarse location information of the UE, it can more effectively implement critical functionalities such as radio resource management (RRM), beam management and scheduling, as well as congestion control. These aspects are particularly vital in NTN especially for limited regenerative payload.

- From our perspective, this approach is applicable in NR NTN as well.

- Due to the extensive coverage of satellites (including LEO, MEO, and GEO), a single satellite beam may span several tens of kilometers. However, the distribution of UEs within a beam may be highly uneven or sparsely distributed at different locations. When a UE initiates HO, if the gNB can obtain the coarse location of the UE or, if possible, the precise UE location, the satellite can concentrate its transmission power and schedule at least a narrow satellite beam of several kilometers diameter to serve the UE handover to the target cell, thereby ensuring the service continuity and a better user experience to the considerable extent.

- Additionally, the current spec supports the mechanism of reporting UE coarse location via the air interface, allowing the source cell to acquire this information. However, there may be concerns whether the UE coarse location is inaccurate obtained by the target cell due to UE movement.

- Regarding this point, while the interaction between the source and the target cell is the UE coarse location the UE is unlikely to move so rapidly as to exceed the coverage of the narrow beam, except in extremely rare cases. Even when the satellite switches to the narrow beam for service provision, the narrow beam still has a relatively wide coverage range compared to the movement range of the UE during CHO. Therefore, the UE coarse location information is capable of fulfilling the beam scheduling requirements of the target cell.

Proposal 1: RAN3 to discuss on the enhancement of transferring UE coarse location information between XnAP.

Issue 2: Support of RRC_INACTIVE

RNA Configuration

The gNB can assign an RNA (RAN-based Notification Area) to the RRC_INACTIVE UE via RRC Release signal (i.e. SuspendConfig - RAN-NotificationAreaInfo). The RNA in current spec can be configured in any of the following ways:

RAN-NotificationAreaInfo ::= CHOICE { cellList PLMN-RAN-AreaCellList, ran-AreaConfigList PLMN-RAN-AreaConfigList, ... } PLMN-RAN-AreaCellList ::= SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell PLMN-RAN-AreaCell ::= SEQUENCE { plmn-Identity PLMN-Identity OPTIONAL, -- Need S ran-AreaCells SEQUENCE (SIZE (1..32)) OF CellIdentity } PLMN-RAN-AreaConfigList ::= SEQUENCE (SIZE (1..maxPLMNIdentities)) OF PLMN-RAN-AreaConfig PLMN-RAN-AreaConfig ::= SEQUENCE { plmn-Identity PLMN-Identity OPTIONAL, -- Need S ran-Area SEQUENCE (SIZE (1..16)) OF RAN-AreaConfig } RAN-AreaConfig ::= SEQUENCE { trackingAreaCode TrackingAreaCode, ran-AreaCodeList SEQUENCE (SIZE (1..32)) OF RAN-AreaCode OPTIONAL -- Need R }

Option 1: Cell List

Option 2: Tracking Area List

Option 3: Tracking Area List, RAN Area List

In NTN, if option1 is configured, UEs need to trigger RNA update procedure frequently towards the gNB on board even the UE remains stationary. In order to reduce the frequency of RNA updates and reduce signaling overhead between the UEs and the gNB on board, it is better to configure a geo-fixed RNA for regenerative payload NR NTN, i.e. A list of Tracking Area Code.

Observation 1: Geo-fixed RNA configuration can reduce RNA update signaling overhead with satellite moving.

Proposal 2: RNA could be configured as a fixed geographical area (i.e. A list of Tracking Area Code) in regenerative payload NR NTN.

UE context retrieval

The UE context remains in the UE and the last serving gNB when the UE is in the RRC_INACTIVE mode. Given that it may take several hours for the gNB to reconnect with the UE, if the UE context cannot be retrieved through the satellite with ISL (Xn interface), then the NW initiates RRC Setup in legacy way. In response to the open issue of UE context retrieval in RRC_INACTIVE, several companies have proposed multiple options as follows:

Option 1, UE context is forwarded to the next satellites

Option 2, UE context is kept on a fixed node (on-ground gNB or AMF)

Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

The introduction of RRC_INACTIVE offers multiple advantages for small data transmissions, including the reduction of signalling overhead and UE power consumption, as well as the improvement of access latency in TN. Since the UE context is already saved in UE and NG-RAN, CN is not required to involve when UE state switches to RRC_CONNECTED mode. However, option2 and 3 require CN to cooperate with the gNB to retrieve UE context, which involving CN to participate in RRC_INACTIVE resume process and increases signalling overhead. In option 1, during paging the UE to the subsequent satellite, UE have to wake up when every satellite passing through, which is not conducive to UE power consumption. Pro-actively UE context transfer hop by hop in option1 and relocate the UE context to the gNB on ground in option2 have minimal impacts on existing procedures, but raises security concerns regarding UE context interactions between multiple satellites. Such optimizations may diverge from the original objectives of introducing RRC_INACTIVE mode. Therefore, we think the further enhancements for supporting RRC_INACTIVE mode is not clear and there seems to be limited benefits for regenerative payload NR NTN.

Proposal 3: Any enhancements for RRC_INACTIVE in regenerative NR NTN in R19 should be thoroughly analyzed for their benefits and costs.

3	Conclusion

In this contribution, we provide considerations on the NTN regenerative payload in RAN3, following observations and proposals are made:

Proposal 1: RAN3 to discuss on the enhancement of transferring UE coarse location information between XnAP.

Observation 1: Geo-fixed RNA configuration can reduce RNA update signaling overhead with satellite moving.

Proposal 2: RNA could be configured as a fixed geographical area (i.e. A list of Tracking Area Code) in regenerative payload NR NTN.

Proposal 3: Any enhancements for RRC_INACTIVE in regenerative NR NTN in R19 should be thoroughly analyzed for their benefits and costs.

4	Reference

- RP-250041 Status report for WID: Non-Terrestrial Networks (NTN) for NR Phase 3