3GPP TSG-RAN WG3 Meeting #127-bis	R3-252228

Wuhan, China, 7-11 April 2025

Agenda Item:	13.2

Source:	Ericsson, Nokia, LG Electronics

Title:	CSI-RS network coordination

Document for:	Discussion, Agreement

# Introduction

At the last RAN3 meeting, the network coordination for semi-persistent CSI-RS has been discussed and the following progress is made.

For both inter-CU and intra-CU cases:

For the network Semi-Persistent CSI-RS coordination, source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell.

For the activation/deactivation procedure, a class 1 procedure is needed from the source gNB-DU/source gNB. FFS for reusing existing one or a new one.

In this paper, we further investigate the signaling and the corresponding solutions.

# Discussion

To support CSI-RS network coordination, if we consider the more complex inter-CU scenario, where both source gNB and candidate gNB are split gNBs, the network coordination requires three rounds of signaling, also shown in the figure below:

- a first class 1 procedure, between the source DU and the source CU

- a second class 1 procedure, between the source CU and the candidate CU

- a third class 1 procedure, from the candidate CU to the candidate DU

Figure 1. Inter-CU inter-DU LTM - Coordination for semi-persistent CSI-RS transmission.

To achieve a proper coordination, the source CU needs to send the response message to the source DU (in step 6) after it received the response of the candidate CU (in step 5). This means that the candidate CU needs to send its response to the source CU (in step 5), after it received the response of the candidate DU (in step 4). Reusing existing procedures can be not optimal, if there are cases where the receiver needs to send its response immediately. Also, since this coordination relates to something new from RAN3 perspective, to be more future proof we propose to support it with new procedures (in F1 and in Xn).

For F1, we propose to define two new UE associated procedures:

- one procedure (e.g., DU-CU CSI-RS Coordination), initiated by the source DU, where the source DU requests the activation / deactivation of the CSI-RS transmission, and receives the response of the candidate DU / gNB

- one procedure (e.g., CU-DU CSI-RS Coordination), initiated by the source CU, to forward the request of the source DU and receive the response from the candidate DU / gNB

For Xn, we propose to define one new UE associated procedure (e.g., CSI-RS Coordination), initiated by the source CU, where the source CU requests the activation / deactivation of the CSI-RS transmission, and receives the corresponding response.

The next two figures show the high-level flow charts of how the coordination can be achieved for intra-CU LTM and for inter-CU LTM. The detailed F1 and Xn signaling are captured in the TPs in R3-25xxxx and R3-25xxxy.

Figure 2. Intra-CU inter-DU LTM - Coordination for semi-persistent CSI-RS transmission.

Figure 3. Inter-CU LTM - Coordination between source DU and candidate DU for semi-persistent CSI-RS transmission.

- Introduce a new UE associated F1AP class 1 procedure (e.g., DU-CU CSI-RS Coordination), initiated by the source DU, to request the activation/deactivation of semi-persistent CSI-RS transmission in LTM candidate cells.

- Introduce a new UE associated F1AP class 1 proc edure (e.g., CU-DU CSI-RS Coordination), initiated by the CU, to forward a request of activation/deactivation of semi-persistent CSI-RS transmission in LTM candidate cells.

- Introduce a new UE associated XnAP class 1 procedure (e.g., CSI-RS Coordination) to request the activation/deactivation of semi-persistent CSI-RS transmission in LTM candidate cells.

- Agree on the TPs for 38.423 and 38.473 respectively in R3-252229 and R3-252230 that capture the above proposals.

# Conclusions and Proposals

In this contribution we made observations and proposals for different LTM related topics, as listed below.

Proposal 1	Introduce a new UE associated F1AP class 1 procedure (e.g., DU-CU CSI-RS Coordination), initiated by the source DU, to request the activation/deactivation of semi-persistent CSI-RS transmission in LTM candidate cells.

Proposal 2	Introduce a new UE associated F1AP class 1 proc edure (e.g., CU-DU CSI-RS Coordination), initiated by the CU, to forward a request of activation/deactivation of semi-persistent CSI-RS transmission in LTM candidate cells.

Proposal 3	Introduce a new UE associated XnAP class 1 procedure (e.g., CSI-RS Coordination) to request the activation/deactivation of semi-persistent CSI-RS transmission in LTM candidate cells.

Proposal 4	Agree on the TPs for 38.423 and 38.473 respectively in R3-252229 and R3-252230 that capture the above proposals.