3GPP TSG-RAN WG3 #127-bis					 				                                    R3-252294

# Wuhan, China, 7-11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.410 | CR | 0053 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 38.410) Introduction of Ambient IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, China Telecom, Huawei, Samsung, CMCC, Nokia, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support Ambient IoT function |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support Ambient IoT function Impact Analysis: |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambient IoT function cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 4.3, 4.4, 5.xx (new), 6.xx1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR 1260 TS 38.300 CR XXX |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: Capture agreed TP: R3-250907, R3-250909 Rev1: Resubmitted Rev2: Update WI code: Ambient_IoT_Solutions-Core |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AIOTF: as defined in TS 23.xxx [xx].

A-IoT CN node: as defined in TS 38.300 [7].

gNB: as defined in TS 38.300 [7].

NB-IoT: as defined in TS36.300 [11].

ng-eNB: as defined in TS 38.300 [7].

NG-RAN node: as defined in TS 38.300 [7].

UPF: as defined in TS 23.501 [8].

3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

A-IoT	Ambient IoT

AIOTF	Ambient IoT Function

AMF	Access and Mobility Management Function

CIoT	Cellular IoT

CN	Core Network

DRX	Discontinuous Reception

IoT	Internet of Things

MBS	Multicast/Broadcast Service

MT	Mobile Terminated

NB-IoT	Narrow Band Internet of Things

NG-U	NG User plane interface

PTP	Point to Point

PTM	Point to Multipoint

QMC	QoE Measurement Collection

QoE	Quality of Experience

RIM	Remote Interference Management

SMF	Session Management Function

UP	User Plane

UPF	User Plane Function

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

4.3	NG interface specification objectives

The NG interface specification facilitates the following:

-	inter-connection of NG-RAN nodes with AMFs supplied by different manufacturers;

-	inter-connection of NG-RAN nodes with A-IoT CN nodes supplied by different manufacturers;

Editor’s Note: whether this section needs A-IoT specific content depends on which term is used for “A-IoT CN node”. If the usage of “AMF” is specified in stage-2 to cover also “A-IoT CN node”, there might be no update necessary.

-	separation of NG interface Radio Network functionality and Transport Network functionality to facilitate introduction of future technology.

## 4.4	NG interface capabilities

The NG interface supports:

-	procedures to establish, maintain and release NG-RAN part of PDU sessions;

-	procedures to perform intra-RAT handover and inter-RAT handover;

-	the separation of each UE on the protocol level for user specific signalling management;

-	the transfer of NAS signalling messages between UE and AMF;

-	mechanisms for resource reservation for packet data streams;

-	procedures to establish, maintain and release NG-RAN part of MBS sessions;

-	the transfer of A-IoT NAS signalling messages between A-IoT device and AIOTF;

-	procedures to perform A-IoT service requests and responses.

Editor’s Note: The above text is based on the working assumption that the A-IoT specific functionalities are to be carried by NGAP containers in NGAP.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

5.31	Timing Synchronisation Status Reporting function

The Timing Synchronisation Status Reporting function enables the AMF to request the NG-RAN node to report the RAN timing synchronisation status information, and for the NG-RAN node to provide the RAN timing synchronisation status information to the AMF.

5.xx	AIoT Service Management Function

The AIoT Service Management function is responsible for handling AIoT services.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

6.27	Timing Synchronisation Status Reporting procedures

The following procedures are used to report the RAN timing synchronisation status information:

-	Timing Synchronization Status;

-	Timing Synchronization Status Report.

6.xx1	AIoT procedures

The following procedures are used for AIoT services:

-	Inventory Request;

-	Inventory Report;

-	Command Request.

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

8.3	NG-RAN NG interface: NG application protocol (NGAP) (TS 38.413)

TS 38.413 [4] specifies the radio network layer signalling procedures of the control plane between the NG-RAN node and the AMF, or between the NG-RAN node and the A-IoT CN node.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>