3GPP TSG RAN WG3#127bis	R3-252305

7th April – 11th April - 2025

Wuhan, China

Agenda Item:	13.2

Source:	China Telecom

Title:	Summary of offline discussion on inter-CU LTM

Document for:	Approval

Introduction

Considering that there are only three meeting left before Rel-19 freeze; in order to achieve more progress, we plan to have an offline discussion before the online session starts in this meeting. This draft SoD summarized companies’ view towards inter-CU LTM based on the tdocs submitted in this meeting.

For the Chairman’s Notes

Propose the following:

For the initial inter-CU LTM preparation, the source gNB computes the KgNB* for each candidate gNB, and forwards the {KgNB*, NCC} pair to the candidate gNB via Handover Request message as legacy.

WA: The new source gNB needs to provide the new KgNB* to candidate gNBs. FFS on the procedure to be reused. FFS on the NCC.

WA: The new NCC value needs to be sent from gNB-CU to the gNB-DU in case of MAC CE is agreed in RAN2 to transmit the NCC to the UE. FFS on the details and pending on RAN2 progress.

Reuse the LTM configuration update procedure to transfer UE’s 5G security capabilities and/or UE’s UP security policy from the new serving gNB to the other candidate gNBs in subsequent LTM.

Introduce new UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s).

Open issues on the granularity of the CSI-RS resource/resource set, and need to be checked with RAN2 progress.

Discussion

In this offline, we plan to continue to discuss the remaining issues on support of inter-CU LTM.

Support of inter-CU LTM in Standalone scenario

Security key update for subsequent LTM

A reply LS S3-251124/ R3-251525 about security handling for inter-CU LTM in non-DC cases is received in this meeting:

1. Overall Description: SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases. Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover. 2. Actions: To RAN2 and RAN3 ACTION: SA3 kindly asks RAN2 and RAN3 to take the above information into account. To RAN3 ACTION: SA3 kindly asks RAN3 to decide and update the SA3 on the key distribution mechanism.

In the legacy Xn handover procedure, the security key used at the target gNB is derived by the source gNB during handover preparation phase, it seems this principle can be easily reused for inter-CU LTM.

Moderator wants to check whether following statement is a common understanding:

It is the common understanding that for the initial inter-CU LTM preparation, the source gNB computes the KgNB* for each candidate gNB, and forwards the {KgNB*, NCC} pair to the candidate gNB via Handover Request message as legacy.

Moderator summary:

For the initial inter-CU LTM preparation, the source gNB computes the KgNB* for each candidate gNB, and forwards the {KgNB*, NCC} pair to the candidate gNB via Handover Request message as legacy.

Similar as legacy handover procedure, the AMF should provide a newly {NH, NCC} pair to the target gNB in the Path Switch Request Acknowledge message after each cell switch. To support subsequent LTM, we need to discuss when and how the new source gNB (the target gNB in the last inter-CU LTM cell switch) sends the new key to the candidate/target gNB. There are two options:

- Option 1: The new source gNB provides the new key to the target gNB via CELL SWITCH NOTIFICATION message when LTM is triggered.

- Option 2: The new source gNB provides the new KgNB* to each candidate gNBs via the LTM CONFIGURATION UPDATE message before next LTM execution.

In this session, we can discuss which option to adopt.

Moderator summary:

Lenovo, E///, Nokia, QC: only the KgNB* is needed.

CATT, ZTE: why not to transmit NCC is unclear. FFS on the NCC.

HW, QC, Nokia, CATT, LGE, Google: option2 is better, option1 may not meet the failure case.

ZTE: Support option1.

WA: The new source gNB needs to provide the new KgNB* to candidate gNBs. FFS on the procedure to be reused. FFS on the NCC.

Based on the reply LS, SA3 confirmed that the NCC can be transmitted to the UE via MAC CE message (i.e. LTM Cell Switch MAC CE). Since the NCC value is derived by the gNB-CU, the source gNB-CU should send the NCC value to the source gNB-DU, the F1AP impact is foreseen. For the detailed F1 signaling enhancement, there are following two options:

- Option 1: Reuse the CU-initiated UE Context Modification procedure to inform the new NCC value to the source gNB-DU. [LG Electronics, CATT, NTT Docomo, Nokia]

- Option 2: Introduce a new class-2 procedure for source gNB-CU to inform the new NCC value to the source gNB-DU. [Lenovo, Huawei, China Telecom]

In this session, we can discuss which option to adopt.

Moderator summary:

WA: The new NCC value needs to be sent from gNB-CU to the gNB-DU in case of MAC CE is agreed in RAN2 to transmit the NCC to the UE. FFS on the details and pending on RAN2 progress.

QC, NTT Docomo: Concerns on the security issues using MAC CE based NCC transmission.

CATT: RAN2 already agree to use MAC CE to transmit NCC value.

According to [R3-252118, Huawei], another issue needs to be discussed is how to update the mismatched UE security context is another issue to be considered. Based on SA3 agreement, the UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) may be updated and provided in the Path-Switch Acknowledge message to the candidate gNB after LTM switch [S3-250765]:

For the case where CU is acting as MN and DC is not configured, it is agreed that the normative work is based on the following principles: - For the parameters in AS security context, which are not changing per inter-CU LTM handover (including UE’s 5G security capabilities, selected AS security algorithms, UE’s UP security policy, UE’s UP security activation status): - If the gNB receives mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) in the Path-Switch Acknowledge message after LTM switch, - it initiates an intra-cell handover procedure which includes RRC Connection Reconfiguration procedure indicating the change of selected algorithms (as defined in clause 6.7.3.1) and/or UE’s UP activation status (as defined in clause 6.6.1) to the UE. - it updates the UE’s 5G security capabilities and/or UE’s UP security policy for the active PDU session to all candidate gNBs

In the LTM preparation phase, the source gNB-CU sends the UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) to all candidate gNBs. RAN3 should consider that the new serving gNB-CU updates the UE security context (received in Path-Switch Acknowledge message) to all candidate gNBs if UE security context mismatch is identified in LTM execution phase.

In Rel-19, the LTM configuration update procedure is introduced to transfer UE configurations, set up UE associations, etc. This procedure can be re-used for the new serving gNB to deliver mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) to other candidate gNBs in subsequent LTM.

Here we can check whether the following proposal is agreeable:

Proposal: Reuse the LTM configuration update procedure to transfer mismatched UE security context (e.g. UE’s 5G security capabilities and/or UE’s UP security policy) from the new serving gNB to the other candidate gNBs in subsequent LTM.

Moderator summary:

Reuse the LTM configuration update procedure to transfer UE’s 5G security capabilities and/or UE’s UP security policy from the new serving gNB to the other candidate gNBs in subsequent LTM.

Besides, we need to discuss whether to send a reply LS to SA3 accompany with RAN3 achievement, a draft LS is provided in R3-251836. If needed, we can the check the wording in the reply LS:

1. Overall Description: RAN3 would like to thank SA3 for their LS about security handling for inter-CU LTM in non-DC cases. RAN3 has discussed the security handling and agreed the CRs as attachments. RAN3 agrees to enhance LTM CONFIGURATION UPDATE message to facilitate NCC transfer between the new source gNB and candidate gNBs, and to enhance UE CONTEXT MODIFICATION REQUEST message for NCC transfer between gNB-CU and gNB-DU. For split architecture, when inter-CU LTM is triggered, gNB-DU will trigger next inter-CU cell switch after receiving the new NCC from gNB-CU. 2. Actions: To RAN1 and RAN2: ACTION: RAN3 respectfully asks SA3 to take the above into account. 3. Date of Next TSG-RAN WG3 Meetings: TSG RAN WG3 Meeting RAN3#128 2025-05-19- 2025-05-23 Malta, MT TSG RAN WG3 Meeting RAN3#129 2025-08-25 - 2025-08-29 India (TBC), IN

HW: send the reply LS in next meeting.

CATT: We need to send the LS, check the reply LS online.

LGE: focus on the tech.

SP CSI-RS configuration

In the last RAN3 meeting, we agreed that it is the source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell, and agreed to adopt the class-1 procedure for the SP CSI-RS activation/deactivation procedure.

For both inter-CU and intra-CU cases: For the network Semi-Persistent CSI-RS coordination, source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell. For the activation/deactivation procedure, a class 1 procedure is needed from the source gNB-DU/source gNB. FFS for reusing existing one or a new one.

In this section, we can start with intra-CU case and discuss the whether to reuse the existing F1AP message or introduce a new class-1 message for the SP CSI-RS activation/deactivation:

- Option 1: Reuse the current F1AP procedure (i.e. UE Context Modification (gNB-CU initiated) and UE Context Modification Required (gNB-DU initiated)). [NEC, Lenovo]

- Option 2: Introduce the new UE-associated procedure over F1AP. [Ericsson, Nokia, LG Electronics, Huawei, CATT, China Telecom, ZTE]

Based on companies’ contributions, it seems most companies prefer to introduce new UE-associated F1AP procedures, so moderator want to check whether we can agree to adopt Option2:

Proposal: Agree to introduce new UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell.

Moderator summary:

Introduce new UE-associated procedures over F1AP to support the source gNB-DU to trigger the activation/deactivation of the SP CSI-RS transmission in the candidate cell(s).

NEC: Introduce common F1AP procedures. Avoid to introduce to many LTM related procedures.

Similarly, for inter-CU case, we can further analysis whether to reuse the existing XnAP message or introduce a new class-1 message for the SP CSI-RS activation/deactivation:

- Option 1: Reuse the current XnAP procedure (i.e. Handover Request and LTM Configuration Update procedure). [Lenovo, CATT, Huawei, NEC, China Telecom]

- Option 2: Introduce a new UE-associated procedure over XnAP. [Ericsson, Nokia, LG Electronics, ZTE]

Companies still share different views towards this issue, in this session, we can discuss which option to adopt.

Moderator summary:

Lenovo, HW: Can support option2.

CATT, NEC: Still prefer option1.

Skip this issue.

Open issues on the granularity of the CSI-RS resource/resource set, and need to be checked with RAN2 progress.

Reference configuration

In the last RAN3 meeting, we reached following agreements and left an open issue for reference configuration for inter-CU LTM case:

The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message. The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message. FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

Reference configuration is introduced in order to reduce the RRC overload. For this purpose, the reference configuration shall contain common configuration as much as possible. In R18, we agreed that the CU can either request the source DU or the candidate DU to provide a reference configuration, and some companies think similar principle can be introduced for inter-CU case, while there are few companies think that for inter-CU case, it is more difficult and complex to coordinate the reference configuration especially when there are multiple candidate gNBs from different vendors:

- Approach 1: Similar as F1AP, the source gNB can request a candidate-gNB to provide a reference configuration. [Nokia, Google, CATT, China Telecom, ZTE, Lenovo, Huawei, NTT Docomo, Samsung, CMCC]

- Approach 2: For inter-CU LTM, only the source gNB can provide the reference configuration. [Qualcomm, Ericsson]

- Approach 3: If the source CU does not provide the reference configuration, it is up to the candidate DU to provide the reference configuration to the source CU and the source DU can decide which one to adopt as a reference configuration. [LG Electronics]

Based on companies’ contributions, it seems most companies prefer to follow the principle of intra-CU case, so moderator want to check whether we can go for Approach 1, and as a compromise, we can give the feasibility to candidate gNB on whether accept or reject the reference configuration request from the source gNB:

Proposal: For inter-CU LTM, the source gNB can request a candidate gNB to provide a reference configuration, and the candidate gNB can accept or reject it.

Moderator summary:

No consensus.

TA Information Transfer

In the last RAN3 meeting, we reached a working assumption to introduce a new non-UE associated class 2 procedure on Xn, which is also aligned with the F1AP.

WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

According to companies’ contributions, many companies propose to turn this working assumption into agreements, while in [R3-251963, Ericsson], it proposed to use UE-associated class-2 procedure to transfer the list of TA values. Since we have been stuck in this issue for several meetings, and majority companies prefer to follow the principle of R18 F1AP design, moderator wants to check we can reach agreement on this issue and agree the following proposal:

Proposal: Turn the WA into agreement: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

Moderator summary:

In addition, we need to discuss what information needed to be carried in this TA information Transfer message, it is obvious that the parameters (e.g. candidate Cell ID, TA value, Preamble index, and RA-RNTI) in F1AP TA Information Transfer messages are reused by the Xn TA Information Transfer message for inter-CU LTM. Besides, according to R3-251963 [Ericsson], it proposes to include the UE XnAP IDs of the source and target gNB in the TA information transfer message. Here we can check what information to be included in the TA information transfer message, for example:

- Candidate Cell ID

- TA value

- Preamble index

- RA-RNTI

- UE XnAP IDs

And further check if any other information is needed to be included.

Moderator summary:

Besides, there companies propose to include the collected TA values in Cell Switch Notification message for subsequent LTM, which is similar as F1AP design. While there are also companies think it is not feasible for inter-CU case since distance between gNBs is larger than the distance between gNB-Dus and the UE movement is unpredictable:

- Option 1: Support to include the TA Values in Cell Switch Notification message. [Nokia, Samsung, Qualcomm]

- Option 2: Do not support to include the TA Values in Cell Switch Notification message. [LG Electronics, China Telecom, CATT]

Here we can have a brief discussion on this issue, and check if we can reach any progress on this issue.

Moderator summary:

Handling of multiple UE associations

Issue1: UE association setup during initial handover preparation

In the last RAN3 meeting, we reached a working assumption that use a single UE association for multiple LTM handover request to the same candidate gNB.

WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.

According to companies’ proposals, there are following approaches to achieve single UE association in parallel transaction:

Approach 1: Add an LTM Indicator IE and a Target NG-RAN node UE XnAP ID IE to the HANDOVER REQUEST. [R3-251963, Ericsson]

- an IE similar to the LTM Indicator IE included in F1AP, with two codepoints, e.g., “first-candidate”, “other-candidate”,

- a Target NG-RAN node UE XnAP ID IE, present if the value of the LTM Indicator is “other-candidate”.

| >LTM Indicator | M |  | ENUMERATED (first-candidate, other-candidate, …) |  | - |  |
| --- | --- | --- | --- | --- | --- | --- |
| >Target NG-RAN node UE XnAP ID | C-ifOtherCand |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | - |  |

| Condition | Explanation |
| --- | --- |
| ifOtherCand | This IE shall be present if the LTM Indicator IE is present and set to "other-candidate". |

Approach 2: Use the same {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE} pair as well as different Target cell ID to indicate multiple candidate cells within the same candidate gNB. [R3-251881, ZTE]

If the Target NG-RAN node UE XnAP ID IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the source NG-RAN node has requested the LTM information for the same UE-associated signaling connection associated with the Target NG-RAN node UE XnAP ID allocated in the previous Handover Preparation procedure.

Approach 3: Allow parallel transactions using the same source UE AP ID only, with each identified by a unique target cell ID. [R3-251742, LG Electronics]

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. If the procedure concerns an LTM, parallel transactions are allowed using the same source UE AP ID only. In this case, possible parallel requests are identified by the target cell ID, and only one UE-associated signaling connection is established and maintained between the source NG-RAN node and the target NG-RAN node. The procedure uses UE-associated signalling.

Approach 4: To use the existing mandatory NG-C UE associated signalling reference IE in the XnAP HANDOVER REQUEST as the single UE association for multiple LTM handover request to the same candidate gNB. [R3-251670, NEC]

Here we can discuss the feasibility to use only one UE association for one UE when multiple LTM candidate cells are prepared in the same gNB based on the above approaches, and further check whether to chance the WA in to agreement:

Proposal: Turn the WA into agreement: Use a single UE association for multiple LTM handover request to the same candidate gNB.

Moderator summary:

Issue2: UE association setup after each LTM cell switch (i.e. UE association between the new serving gNB and candidate gNBs)

For subsequent LTM, after each cell switch, the UE association needs to be established between the new serving gNB and candidate gNB(s). In RAN3#126 meeting, it was agreed to reuse the LTM Configuration Update procedure to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch to support subsequent LTM.

RAN3 #126 To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

For subsequent LTM, the candidate gNB should identify that the UE is the same as the previously associated between the old serving gNB and the current candidate gNB. Since there is only one UE association among source gNB and candidate gNB, the question needs to be discussed is how the candidate gNB identify that the UE is the same as the previously associated between the old serving gNB and the current candidate gNB. Based on companies’ proposals, there are following options:

- Option 1: Use the old target UE XnAP ID for each candidate cell to the corresponding candidate gNB.

- Option 2: Use the old source UE XnAP ID for each candidate cell to the corresponding candidate gNB.

- Option 3: Use the pair of <old source UE XnAP ID, old target UE XnAP ID> for each candidate cell to the corresponding candidate gNB via the LTM Configuration Update message after the LTM cell switch.

- Option 4: All the above three options are feasible.

In this session, we can discuss which option to adopt.

Moderator summary:

Data forwarding

To support data forwarding in case of inter-CU LTM, the Source gNB needs to transfer information regarding PDU Session Resource Information List of all other Candidate gNBs to the Target gNB to execute data forwarding for subsequent LTM. Similarly, RAN3 has agreed this information to be included within LTM CONFIGURATION UPDATE message with the exact timing left to implementation. However, some Stage-3 details were left open.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch. Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation.

In latest BL CR for XnAP 38.423, the existing Data Forwarding Info IE defined in section 9.2.1.16 was introduced to the LTM CONFIGURATION UPDATE message with remark of “FFS if list is needed”. Many companies propose to update this IE to contain a list of Data Forwarding Info IEs, each corresponding to a different target gNB to where data forwarding should be performed. Here, we can check whether the following proposal is agreeable:

Proposal: Data Forwarding Information IE is a list including multiple data forwarding addresses from each candidate gNB.

Moderator summary:

Rel-19 set ID configuration

In RAN2#128 meeting, following agreement were achieved for inter-CU LTM:

Agreements on inter-CU LTM - Rel-19 Set ID is configured for a candidate configuration. For DRB: - Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell, - UE performs PDCP re-establishment to all radio bearers. - Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell, - For MN terminated bearer without change of termination point, UE does not perform PDCP re-estabdlishment. PDCP recovery is needed if it is split bearer. - For SN terminated bearer without change of termination point, UE performs PDCP re-establishment. - If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

Based on RAN2 agreement, the Rel-19 Set ID is introduced for LTM to distinguish whether a PDCP re-establishment is required. In R3-251862 [China Telecom], it proposed that the source CU need to send the Rel-19 Set ID of its own to the candidate CU(s), while in R3-251963 [Ericsson], it proposed that the source gNB sends the Rel-19 Set ID assigned to the candidate gNB in the HANDOVER REQUEST message. And a list of Rel-19 Set IDs should be introduced to the LTM CONFIGURATION UPDATE message as well.

In this session, we can discuss the RAN3 impact on generating the Rel-19 Set ID configurations.

Moderator summary:

gNB-DU initiated LTM candidate cell reconfiguration

According to R3-251972[Rakuten Mobile, Qualcomm], it proposed RAN3 to discuss gNB-DU initiated candidate/target cell re-configuration in Release 19 and permits re-configuration of resources initiated by the candidate/target gNB-DU.

However, in R18, we reached following agreement about candidate gNB-DU initiated LTM configuration:

gNB-DU initiated L1/L2 handover configuration is not allowed.

In this session, we can revisit this issue on support of candidate gNB initiated LTM configuration.

Moderator summary:

Support of inter-CU LTM in NR-DC scenario.

Avoid simultaneous MCG and SCG LTM

In RAN3 #123 meeting, RAN3 agreed that inter-CU LTM is not configured in both MCG and SCG at the same time, which is also confirmed by RAN2 agreement and the updated WID.

For coexistence of MCG and SCG LTM, following agreements were reached in RAN2#127bis meeting:

- RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

Besides, in RAN3#123bis meeting, we reached following agreement related to inter-CU LTM in NR-DC:

- Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Regarding this issue, we need to design solutions to avoid simultaneous configuration for inter-CU MCG and SCG LTM.

During the online section, we mentioned following two options:

- Option 1: The MN/source SN needs to inform the other node once it decides to initiate an inter-CU LTM.

- Option 2: The MN rejects the SN-initiated inter-CU SCG LTM request with a proper cause value.

- Option 3: Up to RAN2(i.e. implicit indication via Inter-Node RRC signaling).

In this section, we can down select from the above options.

Moderator summary:

SN initiated inter-CU SCG LTM without MCG changes

The figure of SN initiated inter-CU SCG LTM procedure from the endorsed running CR R2-2410928 in RAN2 #128 is cited as below:

Figure 10.5.2-5 SN initiated inter-SN SCG LTM

## Issue1: SN Change Required message

In the last RAN3 meeting, we reach following agreement related to the inter-CU SCG preparation:

For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN. Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

In step1 of above figure, the source SN initiates the inter-CU SCG LTM preparation procedure by sending a SN Change Required message to the MN. In existing S-CPAC functionality, an S-CPAC Request IE was introduced in the SN Change Required message to indicate the SN change is for S-CPAC. A similar approach could be used to enable inter-CU SCG LTM, where an indicator is introduced in the SN Change Required message to indicate the SN change is for inter-CU SCG LTM.

Here we can check if Proposal1 is agreeable:

Proposal 1: Introduce an indicator in SN Change Required message from the source SN to the MN, to indicate that the SN change is for inter-CU SCG LTM.

Moderator summary:

According to [R3-251947, Lenovo], in addition to help the candidate SN providing the LTM candidate configurations, the source SN needs to provide a list of candidate PSCells to the candidate SN via the MN. There are two options:

- Option 1: The source SN provides the measurement results of candidate PSCells to the candidate SN via the MN, and then the candidate SN determines the LTM candidate cells based on the measurement results.

- Option 2: The source SN provides a list of LTM candidate cells to the candidate SN via the MN, and the candidate SN accepts or rejects these LTM candidate cells.

Here we can down select from the above options.

Moderator summary:

In addition, according to [R3-251837, CATT], it is necessary to have coordination between the MN and SN for transferring the LTM configuration ID since intra-CU SCG LTM and inter-CU SCG LTM may coexist. A reasonable way is to let the source SN generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM (i.e. in step6).

5. The MN sends the received L1 RS configuration, early UL/DL sync configuration of candidate cells to the source SN via SN Modification Request message. 6. The source SN generates the common CSI resource configuration for L1 measurement on candidate PSCells. The source SN sends the generated common CSI resource configuration and the updated source SCG configuration to the MN via SN modification request acknowledge message. The source SN sends the configuration ID mapping list for the accepted candidate PSCell(s) to the MN for subsequent LTM.

Here we can check if Proposal2 is agreeable:

Proposal 2: It is the source SN to generate the LTM configuration ID mapping list for the accepted candidate PSCell(s), and send it to the MN for subsequent LTM.

Moderator summary:

## Issue2: SCG Reference configuration

According to RAN2 agreement, it is the SN (source or candidate) generates the SCG part of the reference configuration:

RAN2 #127bis agreement： The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs.

In [R3-251759, Qualcomm], it proposed that similar as in Rel-18 Subsequent CPAC, the MN can request an SCG reference configuration from either the source SN or a candidate SN. Similar proposal also provided in [R3-251701, Google] and [R3-252240, ZTE].

Here we can check if Proposal3 is agreeable:

Proposal 3. For inter-CU LTM in DC, the MN can request an SCG reference configuration from either the source SN or a candidate SN.

Moderator summary:

## Issue3: TA Information Transfer

In step 13/14a, the UE performs DL/UL sync with the LTM candidate cell(s) before receiving the cell switch command. For the PDCCH order triggered early RACH. In step 14b, the candidate SN(s) needs to transfer the calculated TA value of candidate cell(s) to the source SN via the MN. The existing TA information transfer procedure via F1 interface can be extended to Xn interface to transfer the TA information between the MN and the source/candidate SN.

Here we can check if Proposal4 is agreeable:

Proposal 4: The TA information transfer message can be reused to transfer the TA information from the candidate SN to the source SN via the MN.

Moderator summary:

## Issue4: Security key updates for SCG LTM

According to RAN2 agreement, the SCPAC-similar security update configuration is introduced for inter-CU SCG LTM:

RAN2#127bis agreement: - The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

In case of subsequent inter-CU SCG LTM, the configuration of the candidate PSCell remains in effect even after the UE has connected to the target cell, allowing the UE to connect to the same SN later without requiring any additional network reconfiguration. To support it, the MN needs to provide a list of unique security keys and sk-counter values for each SN to the UE, where each of these values is used when the UE connects to the target SCG. The candidate SN also needs to sync the security key and sk-counter value with the MN over the Xn interface.

To support subsequent SCG LTM, the security keys and sk-counter values for candidate SN needs to be transfer from MN to the candidate SN/target SN, based on companies’ proposals there are following two options:

- Option 1: MN sends a list of {S-key and SK-counter} to each of candidate SN via SN addition/modification request message. [Lenovo, ZTE]

- Option 2: MN sends security key and a list of sk-counters to the target SN the SN via the CELL SWITCH NOTIFICATION message. [Ericsson]

Here we can discuss which option to adopt.

Moderator summary:

## Issue5: Cell Switch Notification

In the last RAN3 meeting, we reach following agreement related to Cell Switch Notification procedure:

The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed.

According to [R3-251862, China Telecom] and [R3-252119, Huawei], similar as inter-CU LTM in standalone scenario, the target PSCell ID and corresponding TCI State ID(s) need to be included in the Cell Switch Notification message.

Here we can check if Proposal5 is agreeable:

Proposal 5: Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message.

Moderator summary:

## Issue6: Access Success

According to [R3-251817, CATT], in current BCLR for TS37.340, the step 23 ACCESS SUCCESS message is used for target gNB to inform source gNB that UE has accessed. However, RAN3 never defined the Access success in XnAP, and usually there is no directly interaction between source S-SN and T-SN. The key issue is whether need and how to inform S-SN that the UE access to T-SN successfully. In S-CPAC case, the MN will send a SN MODIFICATION REQUEST message to S-SN after receive the RRCReconfigurationcomplete from UE the main purpose of this step is to initial late data forwarding and update the data forwarding address.

According to [R3-251963, Ericsson], it proposed that at completion of SN initiated inter-CU SCG LTM, the target SN sends a HANDOVER SUCCESS to the MN, which forwards it to the source SN, to inform the source SN the UE successfully accessed the target cell.

Based on the above status, there are possible following options to be further discussed:

- Option 1: Reuse the SN MODIFICATION REQUEST message to inform the source SN that UE successfully accessed the target PSCell.

- Option 2: Reuse the HANDOVER SUCCESS message to inform the source SN the UE successfully accessed the target PSCell.

- Option 3: No need to inform the access success information to the source SN.

Here we can down select from the above options.

Moderator summary:

TP/BLCRs

Check stage2/3 TPs if possible.

Conclusion, Recommendations [if needed]

If needed

References

| R3-251507 | LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells (RAN1(Fujitsu)) | LS in |
| --- | --- | --- |
| R3-251525 | Reply LS on security handling for inter-CU LTM in non-DC cases (SA3(CATT)) | LS in |
| R3-251533 | TP (BL CR TS 38.300) Inter-CU LTM Procedure (Nokia) | other |
| R3-251582 | Discussion on Inter-CU LTM security (Nokia) | discussion |
| R3-251629 | Data forwarding for Inter-CU LTM DC scenarios (Ofinno, LLC) | discussion |
| R3-251630 | Issues on SN initiated inter-CU SCG LTM (Ofinno, LLC) | discussion |
| R3-251633 | Discussion on Inter-CU LTM (Rakuten Mobile, Inc) | discussion |
| R3-251670 | (TP to BL CR for TS 38.423 on Inter-CU LTM) Rel-19 inter-CU LTM issues (NEC) | other |
| R3-251671 | discussion Rel-19 inter-CU LTM in DC scenario (NEC) | discussion |
| R3-251707 | (TP to BL CR 38.423) Support subsequent mobility in inter-CU LTM (Google) | other |
| R3-251741 | (TP for NR_Mob_Ph4 TS 38.473) Discussions on key distribution and NCC delivery for Inter-CU LTM (LG Electronics Inc.) | other |
| R3-251742 | Discussions on key remaining aspects of Inter-CU LTM (LG Electronics Inc.) | discussion |
| R3-251759 | Signalling enhancements for Inter-CU LTM handover (Qualcomm Incorporated) | discussion |
| R3-251835 | (TP for 38.473, 38.423) Discussion for security handling on non-DC case (CATT) | discussion |
| R3-251836 | [Draft LS] Reply LS to SA3 on the security handling for inter-CU LTM in non-DC case (CATT) | LS out To: SA3 CC: RAN2 |
| R3-251837 | (TP for 37.340) Discussion for Inter-CU LTM (CATT) | discussion |
| R3-251862 | Discussion on signaling enhancement for inter-CU LTM (China Telecom) | discussion |
| R3-251863 | (TP to TS38.401) Support of activation or deactivation of SP CSI-RS for inter-DU LTM (China Telecom) | other |
| R3-251881 | (TP for 38.423, 38.300, 38.420, 38.470) Inter-CU LTM (ZTE Corporation) | other |
| R3-251946 | [TP to BLCR for TS 38.423] Inter-CU LTM (Lenovo) | other |
| R3-251947 | [TP to BLCR for TS 38.423] Inter-CU LTM in DC (Lenovo) | other |
| R3-251963 | Further discussion on inter-CU LTM (Ericsson) | discussion |
| R3-251964 | (TP for LTM BL CR for TS 38.423, TS 38.473, TS 38.300, TS 37.340) – Support for inter-CU LTM (Ericsson) | other |
| R3-251972 | gNB-DU initiated LTM candidate cell reconfiguration (Rakuten Mobile, Qualcomm Inc) | discussion |
| R3-251986 | Issues on source gNB initiated LTM cancel and UE association setup (Ofinno, LLC) | discussion |
| R3-252088 | Support for Inter-CU L1/L2 Triggered Mobility (LTM) over Xn (Jio Platforms Ltd (JPL)) | discussion |
| R3-252089 | Xn support for inter-CU L1/L2 Triggered Mobility (Jio Platforms Ltd (JPL)) | CR1476r, TS 38.423 v18.5.0, Rel-18, Cat. C |
| R3-252118 | (TP for LTM BLCR for TS38.473, TS38.300, TS38.470): Further discussion on inter-CU LTM procedure (Huawei) | other |
| R3-252119 | (TP for LTM BLCR for TS37.340, TS38.423): Discussion on inter-CU SCG LTM (Huawei) | other |
| R3-252148 | Discussion on inter-CU LTM (NTT DOCOMO INC..) | discussion |
| R3-252152 | Discussion on inter-gNB-CU LTM (Samsung) | discussion |
| R3-252153 | (TP to BLCR for TS38.423 and TS38.473) Inter-gNB-CU LTM (Samsung) | other |
| R3-252228 | CSI-RS network coordination (Ericsson, Nokia, LG Electronics) | discussion |
| R3-252229 | (TP for TS 38.423) – Xn support for Semi-persistent CSI-RS transmission (Ericsson, Nokia, LG Electronics) | other |
| R3-252230 | (TP for TS 38.473) – F1 support for Semi-persistent CSI-RS transmission (Ericsson, Nokia, LG Electronics) | other |
| R3-252240 | (TP for 38.423, 37.340) SN initiate SCG LTM (ZTE Corporation) | other |

Appendix

Previous meeting agreements:

RAN3#123bis:

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first.

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Early data forwarding can be supported for inter-CU LTM.

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

RAN3#124:

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Introduce a new procedure on Xn to transfer the TA information.

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

FFS on the LTM modification procedures.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

RAN3#125:

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM.

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure

RAN3#125bis:

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

The LTM Configuration IDs are allocated by the source CU.

WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

Confirm the name of the new procedure as “LTM Configuration Update”.

Introduce a new procedure for candidate gNB-initiated LTM cancellation.

- Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

RAN3#126:

RAN3 move forward on Legacy framework with PDCP change/switch for inter-CU LTM in this release, not considering the PDCP anchor based solution.

The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB.

Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation?

RAN3#127:

For both inter-CU and intra-CU cases:

For the network Semi-Persistent CSI-RS coordination, source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell.

For the activation/deactivation procedure, a class 1 procedure is needed from the source gNB-DU/source gNB. FFS for reusing existing one or a new one.

The candidate gNB-CU responds the full SSB Time/Frequency Configuration (in SSB Information IE) of candidate cells to the source gNB-CU in the Handover Request ACK message.

The source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource (to be further discussed)) to the candidate gNB-CU to request early RACH configuration.

WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

RAN3 agree the following scenarios to support LTM with NR-DC:

1.SN initiated inter-CU SCG LTM without MCG changes (high priority)

2.Inter-CU MCG LTM without SN release

3.Inter-CU MCG LTM with SN release

4.Inter-CU MCG LTM with SN addition

The format of the new introduced IE is same with the gNB-DU ID.

The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message.

The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message.

FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.

Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc.

It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch.

Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation.

For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN.

The MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed.