3GPP TSG-RAN WG3 #127bis	R3-252340

Wuhan, CN, 7 – 11 Apr. 2025

Agenda Item:	18.2. Support LP-WUS Indicating Paging Monitoring

Source:	NTT DOCOMO INC.

Title:	Summary of discussion on LP-WUS

Document for:	Discussion, agreement

For the Chair’s Notes

TPs

- (Nokia) TP for BL CR for TS 38.473: R3-251665 rev in R3-252342

- (HW) TP for BL CR for TS 38.300: R3-251754 rev in R3-252362

LS for security concern on extension of UE_ID

RAN3 will not send an LS to SA3 regarding this issue at least in this meeting.

Whether Extended UE Identity Index or Hashed UE_ID is used and how many bits are needed for UE_ID is pending on RAN2.

Introduction

The maximum number of subgroups is decided to 31.

- range of LP-WUS CN Subgroup ID is from 0 to 30: Samsung, E///, Nokia, HW,

- range of LP-WUS CN Subgroup ID is from 1 to 31: ZTE,

Turn the working assumption into agreement and remove the FFS on the presence of the LP-WUS Subgrouping Support Indication IE per cell in the F1AP Paging message.

RAN3 to update the stage2 BLCR by removing the restrictions of LP-WUS in case of emergency PDU session.

CB: # LP_WUS

- check with update TP in 1665.

- discuss on the draft to SA3, and capture the agreements.

- update the TPs according to offline agreements.

(Moderator- NTT DCM)

Offline discussion summary in R3-252340

Discussion

TPs

- (Nokia) TP for BL CR for TS 38.473: R3-251665 rev in R3-252342

- (HW) TP for BL CR for TS 38.300: R3-251754 rev in R3-252362

Companies can provide comments below, or make updates to the draft TPs in the folder.

| Company | Comment |
| --- | --- |
| Ericsson | TP look good, please add E/// a co-source, thank you! |
| Qualcomm | TPs look good. |
| ZTE | TPs look good, please add ZTE as a co-source, thank you! |

LS on security concern on extension of UE_ID [R3-251644, R3-251645]

It is proposed by CATT that there may be security concern to extend Extended UE Identity Index IE as illustrated as following:

- However, the increased length of UE_ID (i.e., truncated 5G-S-TMSI) may lead to security risk. Note that delivering the entire 5G-S-TMSI over F1/Xn interface may cause some security issues, and that is the reason why the truncated 5G-S-TMSI is used from R15. Thus, if RAN3 assumes increasing the length of truncated 5G-S-TMSI for LP-WUS subgrouping is needed, it should ask SA3 whether there is any concern.

- Observation 3: Further increasing the length of UE_ID delivered over F1/Xn interfaces may have security risk.

Can we agree with following proposal?

Proposal 1: RAN3 should send an LS to SA3 to ask if there is any concern on further increasing the length of UE_ID over F1/Xn interfaces for LP-WUS subgrouping.

Companies can provide comments below:

| Company | Comment |
| --- | --- |
| Huawei | Too early to send LS since we have not decided the exact size of the extended UE ID index or even the format (part of 5GS-TMSI or hashed ID to be decided by RAN2). |
| Ericsson | Agree with Huawei. RAN2 has not reached conclusion yet. |
| CATT | RAN2 has no conclusion because some companies in RAN2 considered that RAN3 may send LS to SA3 for confirmation, so they would like to postpone the subgrouping issue in this meeting. It’s not early to send the LS, there are only two meetings left. If we send the LS, RAN2 can take the reply of SA3 into account in the next meeting, and we think the reply LS will be beneficial for RAN2 to converge on a suitable solution. |
| Qualcomm | Agree with the comments above |
| ZTE | The similar issue, exposing 5G-S-TMSI over the air, was discussed in SL Relay. SA3 had the following discussion (S3-211664), i.e. there is no security issue on exposing the 5G-S-TMSI. 5G-GUTI is a temporary identifier of the SUPI which can be used over the air. This means exposing 5G-GUTI is acceptable and there is no security issue. Besides, 5G-GUTI is exposed in the initial NAS message, and we’ve never seen there is a security issue. I-RNTI is a temporary identifier assigned by a RAN node and is sent to the UE in RRC Release message. The UE uses I-RNTI in the RRC Resume Request message which is only integrity protected. When the UE gest back to inactive states again, the RAN node will assign a new I-RNTI to the UE. This means the I-RNTI has been exposed in the RRC Resume message, but it was integrity protected, and the RAN node will assign a new I-RNTI to the UE. Observation 2: There is no security issue on exposing 5G-GUTI and I-RNTI, because 5G-GUTI and I-RNTI is designed to be used over the air. Since 5G-TMSI is a part of 5G-GUTI, so there is no security issue on exposing 5G-TMSI, neither. Proposal 1: Reply the LS to RAN2 saying there is no security issue on exposing the 5G-TMSI/ I-RNTI.. For information, in SL relay, it is specified: The L2 U2N Remote UE in RRC_IDLE provides 5G-S-TMSI and UE specific DRX cycle (if configured by upper layer) to the L2 U2N Relay UE for requesting to perform PO monitoring. The L2 U2N Remote UE in RRC_INACTIVE provides the minimum value of two UE specific DRX cycles (if configured respectively by upper layer and NG-RAN), 5G-S-TMSI and I-RNTI to the L2 U2N Relay UE for PO monitoring. The L2 U2N Relay UE in RRC_CONNECTED can notify the L2 U2N Remote UE information (i.e. 5G-S-TMSI/I-RNTI) to the gNB via the SidelinkUEInformationNR message for paging delivery purpose. The L2 U2N Relay UE receives paging messages to check the 5G-S-TMSI/I-RNTI and sends relevant paging record to the L2 U2N Remote UE accordingly.. As above, exposing 5G-S-TMSI has no security issue. In my understanding, using truncated UE Identity Index instead of the full 5G-S-TMSI is not due to security issue. Please promoter check the history for UE Identity Index/ Extended UE Identity Index and see whether it is due to security issue. |

Moderator summary after offline discussion:

RAN3 will not send an LS to SA3 regarding this issue at least in this meeting.

Whether Extended UE Identity Index or Hashed UE_ID is used and how many bits are needed for UE_ID is pending on RAN2.

Conclusions

TPs

- (Nokia) TP for BL CR for TS 38.473: R3-251665 rev in R3-252342

- (HW) TP for BL CR for TS 38.300: R3-251754 rev in R3-252362

LS for security concern on extension of UE_ID

RAN3 will not send an LS to SA3 regarding this issue at least in this meeting.

Whether Extended UE Identity Index or Hashed UE_ID is used and how many bits are needed for UE_ID is pending on RAN2.