3GPP TSG-RAN WG3#127bis                                             	                     R3-252459

Wuhan, China, 07 – 11 April 2025

Agenda item:	16.2

Source:	Nokia, Huawei, CMCC, NEC

Title: 	[TP for BL CR AiOT for TS 38.300] Update of inventory and command

Document for:	Approval

# TP for TS 38.413

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB.  The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory.

Editor’s Note 3:	content of the A-IoT Device Identification Requested is FFS.

The Inventory Request message also includes Requested Service Area Information (A-IoT area list and/or reader list) and may include Inventory Assistance Information. The Assistance Information may provide an Approximate number of Target A-IoT devices, estimate of expected D2R message size (FFS).

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

Editor’s Note 4:	Specification of further parameters of the Inventory Request message (e.g., assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report message may include reports from multiple A-IoT devices,

### 16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Figure 16.xx.x4-1: Command procedure

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure is performed involving  the concerned device and potentially other A-IoT devices.

NOTE 1: the step 2 may happen in parallel of the Inventory procedure involving other A-IoT devices.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB. The Command Request message includes the same Correlation ID and AIOTF Identifier as the ones used in its corresponding inventory procedure.

The Command Request also includes the RAN A-IoT device NGAP ID and the A-IoT NAS PDU targeting the A-IoT device and may include some command assistance information. This command assistance information may contain the estimate of expected D2R message size, command type (FFS).

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface.

4.	The gNB replies to the Command Request with the Command Response message including the AIOTF Identifier and the Correlation ID received in the Command Request and the RAN A-IoT device NGAP ID.

NOTE 2:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>