3GPP TSG-RAN WG3 Meeting #127-bis	R3-252465

Wuhan, P.R. China, April 7th – 11th 2025

Agenda Item:	12.2

Source:	Ericsson, Huawei, China Telecom, Samsung, Qualcomm, Lenovo, Nokia, Nokia Shanghai Bell, ZTE Corporation, Jio Platforms

Title:	(TP for WAB BL CR for TS 38.401): Functional Aspects of WAB-Nodes

Document for:	Agreements

# Introduction

This TP reflects the following agreements from the RAN3#127-bis meeting:

When access and backhaul links of a WAB node are operated out-of-band, there is no need for WAB resource coordination.

RAN3 assumes that WAB deployments use out-of-band operation in case access and backhaul use different PLMNs.

In this release, In-band WAB operation is only considered for intra PLMN scenarios where BH gNB is upgraded with WAB-specific enhancements.

In addition, several editorials are proposed.

# TP for WAB BL CR for TS 38.401

-------------------------------------------Start of changes-------------------------------------------

# 6	NG-RAN architecture

## 6.1	Overview

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

# X	Wireless Access Backhaul

## X.1	WAB-node integration procedure

Figure X.1-1 WAB-node integration procedure

Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling.

Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases:

Sub-phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is configured by the OAM (e.g., with the information needed to establish NG connections towards one or more AMF(s)) and the WAB-gNB is service-authorized by the SeGW or by the OAM.

Sub-phase 2-2: NG connection setup. The WAB-gNB establishes NG connection(s) toward the AMF(s). This step follows legacy procedures. After the NG is set up, the WAB-gNB can start serving UE(s).

Sub-phase 2-3: Xn connection setup. If needed, the WAB-gNB can establish Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s). If the WAB-gNB includes a WAB-MT identifier in the signalling for Xn connection setup, the BH-RAN-node and/or other NG-RAN node(s) can understand that the peer node is the WAB-gNB of the WAB-node.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

### X.2.3	TAC/RANAC (re-)configuration for a WAB-gNB’s cell

The TAC/RANAC of WAB-gNB’s cell is configured by the OAM. During the mobility of WAB-node, it can be reconfigured by the OAM, or it can remain unchanged in a certain geographical area. The TAC/RANAC of the WAB-gNB’s cell may be the same as, or different than, the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcast by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

## X.5 	User Location Information for UEs served by a WAB-gNB

For UEs served by a WAB-gNB, in addition to the User Location Information (ULI), the WAB-gNB also provides the core network with Additional ULI, which includes a TAI and a NR CGI pertinent to the WAB-gNB’s broadcasted PLMN/SNPN.

If the PLMN/SNPN broadcasted by a WAB-gNB is the same as the PLMN/SNPN serving the WAB-MT, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the Additional ULI for UEs served by the WAB-gNB includes the TAI and the NR CGI of the cell serving the WAB-MT.

If the PLMN/SNPN serving the WAB-MT is different from the WAB-gNB’s broadcasted PLMN/SNPN, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the Additional ULI for UEs served by the WAB-gNB is determined by the WAB-gNB, based on the WAB-node’s geo-location.

If the WAB-MT connects to the BH-gNB by means of a non-terrestrial link, the Additional ULI for UEs served by WAB-gNB is determined by the WAB-gNB, based on WAB-node’s geo-location. This applies regardless of whether the PLMN/SNPN serving the WAB-MT is the same as, or different than, the WAB-gNB’s broadcasted PLMN/SNPN.

In case Additional ULI for UEs served by a WAB-gNB changes, e.g., due to WAB-node movement, the WAB-gNB derives the new Additional ULI and reports it via legacy procedures, if required by the core network.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

## X.8 	Coordination of WAB-gNB and WAB-MT radio resources

Coordination of access link and backhaul link radio resources is needed when the access link and the backhaul link of a WAB-node are operated in the same band (i.e., referred to as the in-band operation). To facilitate resource coordination, the BH-gNB discovers the co-location of the WAB-MT and the WAB-gNB based on a WAB-MT’s ID which the WAB-gNB includes in an XnAP message it sends to the BH-gNB. The resource coordination procedure is conducted between the WAB-gNB and the BH-gNB serving the WAB-gNB’s co-located WAB-MT.

Editor’s NOTE: Name of class-1 Xn WAB resource coordination procedure still needs to be provided.

NOTE: In-band WAB operation is only considered in scenarios where a WAB-gNB and its co-located WAB-MT connect to the same PLMN. When a WAB-gNB and its co-located WAB-MT connect to different PLMNs, it is assumed that the access link and the backhaul link of a WAB-node are operated in different bands.

-------------------------------------------End of changes-------------------------------------------