3GPP TSG-RAN WG3 Meeting #127bis	R3-25xxxx

Wuhan, China, 7 – 11 April 2025

Agenda item:	20.2

Source:	Xiaomi, Ericsson

Title:	(TP for 38.455) Support of Sample-based measurement for LMF-side model (case 3b)

Document for:	other

# 1	Introduction

This TP provides the configuration parameters needed from LMF to gNB to support sample-based measurement as per RAN1 agreements for case 3b

Agreement (RAN1#119) For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt’ values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt’ values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt’ values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt’, k to gNB via NRPPa. Candidate set values: - Nt’<=24. FFS: Nt’ values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt’, Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified Agreement (RAN1#120) For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting: - k = {0…5} Agreement (RAN1#120) For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt’ values” in RAN1#119 agreement is resolved by supporting: - Nt’ = {8, 16, 24}

For sample-based measurement configuration from LMF to gNB, we have the following analysis.

| Configuration Parameters | Existing NRPPa | Analysis |
| --- | --- | --- |
| k represents the timing reporting granularity factor. | Timing Reporting Granularity Factor INTEGER (0..5) | Existing Timing Reporting Granularity Factor ca be reused for value k in sample-based measurement. |
| Nt represents the window size over which to select channel response values | No existing IE | New Channel Response Window Size IE should be introduced |
| Nt’ represents the number of channel responses are selected from a list of Nt consecutive channel response values | No existing IE | New Channel Response Number IE should be introduced |

Proposal 1, RAN3 agrees the following enhancements to support sample-based measurement configuration:

- introduce new Channel Response Window Size IE and Channel Response Number IE in TRP Measurement Quantities Item

# 2	TP to 38.455 (support of case 3b)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<change starts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP, UL-RSCP, Channel Response(FFS)) | The UL-RSCP measurement is applicable only when the UL-RTOA and/or gNB-RxTxTimeDiff measurement(s) is also requested. | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16]. This IE is ignored when the Timing Reporting Granularity Factor Extended IE is included. | - |  |
| >>Timing Reporting Granularity Factor Extended | O |  | INTEGER (-6..-1, …) | Value -6 corresponds to kminus6, value -5 corresponds to kminus5 and so on, see TS 38.133 [16] | - |  |
| >>Channel Response Window Size | FFS |  | ENUMERATED (32, 64, 128, …) | It represents the window size over which to select channel response values. | Yes | Ignore |
| >>Channel Response Number | FFS |  | ENUMERATED (8, 16, 24, …) | It represents the number of channel responses are selected over the channel response window size. | Yes | Ignore |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |
| Time Window Information Measurement List | O |  | 9.2.91 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change (ANS.1)>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

id-CGI-NR,

id-SFNInitialisationTime-NR,

id-GeographicalCoordinates,

id-ResultSS-RSRP,

id-ResultSS-RSRQ,

id-ResultCSI-RSRP,

id-ResultCSI-RSRQ,

id-AngleOfArrivalNR,

id-ResultNR,

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<skip the unchanged parts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

id-SCS-SpecificCarrier,

id-MeasBasedOnAggregatedResources,

id-UEReportingInterval-milliseconds,

id-ChannelResponseWindowSize,

id-ChannelResponseNumber

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<skip the unchanged parts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

CGI-NR ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

nRcellIdentifier			NRCellIdentifier,

iE-Extensions				ProtocolExtensionContainer { {CGI-NR-ExtIEs} } OPTIONAL,

...

}

CGI-NR-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ChannelResponseWindowSize ::= ENUMERATED { 32, 64, 128, ... }

ChannelResponseNumber ::= ENUMERATED { 8, 16, 24, ... }

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<skip the unchanged parts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

TRPMeasurementQuantities ::= SEQUENCE (SIZE (1..maxnoPosMeas)) OF TRPMeasurementQuantitiesList-Item

TRPMeasurementQuantitiesList-Item ::= SEQUENCE {

tRPMeasurementQuantities-Item		TRPMeasurementType,

timingReportingGranularityFactor	INTEGER (0..5) OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ TRPMeasurementQuantitiesList-Item-ExtIEs}}		OPTIONAL,

...

}

TRPMeasurementQuantitiesList-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ID id-TimingReportingGranularityFactorExtended	CRITICALITY ignore EXTENSION TimingReportingGranularityFactorExtended PRESENCE optional}|

{ID id-ChannelResponseWindowSize	CRITICALITY ignore EXTENSION ChannelResponseWindowSize PRESENCE FFS}|

{ID id-ChannelResponseNumber	CRITICALITY ignore EXTENSION ChannelResponseNumber PRESENCE FFS},

...

}

TRPMeasurementType::= ENUMERATED {

gNB-RxTxTimeDiff,

uL-SRS-RSRP,

uL-AoA,

uL-RTOA,

...,

multiple-UL-AoA,

uL-SRS-RSRPP,

ul-RSCP,

channel-Response(FFS)

}

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<Next change>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NRPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NRPPA-CommonDataTypes;

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<skip the unchanged parts>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

id-PointA														ProtocolIE-ID ::= 154

id-NR-PCI														ProtocolIE-ID ::= 155

id-SCS-SpecificCarrier											ProtocolIE-ID ::= 156

id-MeasBasedOnAggregatedResources								ProtocolIE-ID ::= 157

id-UEReportingInterval-milliseconds								ProtocolIE-ID ::= 158

id-ChannelResponseWindowSize									ProtocolIE-ID ::= XX1

id-ChannelResponseNumber										ProtocolIE-ID ::= XX2

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< change end>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>