3GPP TSG-RAN WG3 Meeting #127bis	R3-252306
Wuhan, China, 7 – 11 April, 2025

Agenda Item:	20.2

Source:	CATT (moderator)

Title:	Summary of Offline Discussion on AIPHY

Document for:	Approval

Introduction

The following CB is discussed in this document:

CB: # AIPHY

- For Case3a, start with the solution signaling flow in 1720 based on common understanding on whether LMF or gNB selects the UE

- Check the TP for Case3b in R3-251770

- Capture agreements and open issues

(moderator - CATT)

Summary of offline disc R3-252306

For the Chairman’s Notes

Propose the following:

Case 3a:

- Stage 2 TP on Data collection?

Case 3b:

- TP?

- LS out?

Discussion

Case 3a

For Case 3a, we mainly discussed how the data collection is performed during online discussion, especially for the issue “Which entity to perform the UE selection? LMF or gNB?”.

The call flow in 1720 is taken as start point to further discuss the signalling flow of data collection.

- For Case3a, start with the solution signaling flow in 1720 based on common understanding on whether LMF or gNB selects the UE.

The call flow in 1720 is copied here for reference:

Figure 7.x.2-1 Data Collection Information Transfer Procedures

1. LMF initiates the Data Collection Notification procedure towards the gNBs which holds the AI/ML positioning model to establish the NRPPa transaction.

Note: This step could be ignored if NRPPa transaction(s) has already been established between the gNB and LMF for other purposes.

2. When gNB decides to train the model for AI/ML based positioning, it may request for the training data from LMF.

3. The gNB sends the Data Collection Request towards the LMF, with some assistance information, e.g. expected amount of UE labels, expected data type.

4. LMF confirms the requirements of the gNB and replies with the response message.

5. LMF decides the UE labels and initiates proper positioning procedures to collect the training data via the Positioning Information Transfer and Measurement procedures.

6. When the training data is collected, LMF provides the training data to the gNB via the Data Collection Update.

Potential Way Forward:

Go for a unified solution to cover both “Proactive” and “Opportunistic”.

Figure x Data Collection Information Transfer Procedures

1. LMF initiates the Data Collection Notification procedure towards the gNBs which holds the AI/ML positioning model to establish the NRPPa transaction.

Note: This step could be ignored if NRPPa transaction(s) has already been established between the gNB and LMF for other purposes.

Proposal 1: Introduce a new class 2 procedure (DATA COLLECTION NOTIFICATION) to establish NRPPa Transaction.

2. When gNB decides to train the model for AI/ML based positioning, it may request for the training data from LMF.

3. The gNB sends the Data Collection Request towards the LMF, with some assistance information, e.g. expected amount of UE labels, expected data type.

4. LMF confirms the requirements of the gNB and replies with the response message.

Editor’s Note:

Step 3/4 are non-UE specific procedure for gNB to request for the data collection towards LMF. The signalling details could be further discussed, e.g. what information should be provided in the Request (TRP ID, number of UEs, etc.)

Proposal 2: Introduce a new class 1 procedure (DATA COLLECTION REQUEST/RESPONSE) to Request for data collection between gNB and LMF.

5. LMF indicates gNB the UE is selected for data collection when performing Measurement procedures for a UE.

Editor’s Note:

After step 3/4, it’s not necessary for LMF to trigger the UE label selection and corresponding positioning procedures immediately. When positioning sessions are triggered for a UE for any purpose purpose, LMF may select it as the UE label. LMF indicates the gNB the UE is selected for data collection (e.g. add an indicator in Measurement Request), the gNB could obtain the Part A internally.

Proposal 3: Introduce an indicator in the MEASUREMENT REQUEST to indicate the UE is selected for requested data collection.

6. When the training data is collected, LMF provides the training data to the gNB via the Data Collection Update. gNB associate Part A and Part B and use them as the collected data for AI/ML model training.

Editor’s Note:

FFS on how to associate Part A and Part B, e.g. LMF Measurement ID could be used to associate the Part A and B.

9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |

Proposal 4: Introduce a new class 2 procedure (DATA COLLECTION UPDATE) to provide Part B of a UE label.

Proposal 5: WA: LMF Measurement ID could be used to associate Part A and Part B.

Proposal 6: Discuss and agree the call flows and texts for stage 2 BL CR.

Case 3b

Check stage 3 TPs for 38.455 and 38.473 in R3-251770 to make support of Sample-Based Measurement as agreed by RAN1.

Proposal 7: Discuss and agree the TPs to make support of Sample-Based Measurement.

Discuss the potential LS out on behalf of R3-252026, if the TP(s) are agreed.

Proposal 8: Reply the LS to RAN1 to indicate RAN3 progress on LMF-based AI Positioning.

Conclusion, Recommendations [if needed]

Proposal 1: Introduce a new class 2 procedure (DATA COLLECTION NOTIFICATION) to establish NRPPa Transaction.

Proposal 2: Introduce a new class 1 procedure (DATA COLLECTION REQUEST/RESPONSE) to Request for data collection between gNB and LMF.

Proposal 3: Introduce an indicator in the MEASUREMENT REQUEST to indicate the UE is selected for requested data collection.

Proposal 4: Introduce a new class 2 procedure (DATA COLLECTION UPDATE) to provide Part B of a UE label.

Proposal 5: WA: LMF Measurement ID could be used to associate Part A and Part B.

Proposal 6: Discuss and agree the call flows and texts for stage 2 BL CR.

Proposal 7: Discuss and agree the TPs to make support of Sample-Based Measurement.

Proposal 8: Reply the LS to RAN1 to indicate RAN3 progress on LMF-based AI Positioning.

References

R3-251509 Reply LS on LMF-based AI/ML Positioning for Case 3b (RAN1(Ericsson))	LS in

R3-251510 LS on AI/ML Positioning Case 3b (RAN1(Ericsson))	LS in

R3-252026 [DRAFT] LS reply on AI/ML Positioning Case 3b (Ericsson)	LS out To: RAN1, RAN2 CC:

R3-251720 (TP to BL CRs) Support of gNB-based AI positioning (CATT)	other

R3-251769 (TP for TS 38.455) Support of gNB-side model (case 3a) (Xiaomi)	other

R3-252027 Discussion on data collection procedures to support gNB-sided model (case 3a) (Ericsson)	discussion

R3-251617 [TP to 38.455 & 38.401] Support of AI/ML assisted Positioning (case 3a) (ZTE Corporation)	other

R3-251618 (TP to BLCR to TS 38.455 & TS38.473) Discussion on AI/ML assisted positioning (case 3b) (ZTE Corporation)	other

R3-251692 Discussion on AIML based Positioning Accuracy Enhancements (NEC)	discussion

R3-251719 (TP to BL CR for TS38.455) Support of LMF-based AI positioning (CATT)	other

R3-251770 (TP for 38.455 and TS 38.473) Support of Sample-based measurement for LMF-side model (case 3b) (Xiaomi, Ericsson)	other

R3-251800 Discussion on Case 3a in AI/ML for positioning (Samsung, JIO Platforms)	discussion

R3-251801 Discussion on Case 3b in AI/ML for positioning (Samsung, JIO Platforms)	discussion

R3-251867 Discussion on support of direct AI/ML positioning (Case 3b) (China Telecom)	discussion

R3-251868 Discussion on support of AI/ML assisted positioning (Case 3a) (China Telecom)	discussion

R3-251953 AIML for gNB assisted positioning (Lenovo)	discussion

R3-251957 AI/ML based positioning accuracy enhancements (Qualcomm Incorporated)	discussion

R3-252001 (TP for AI/ML BLCR to TS 38.455) Discussion on RAN3 impacts for Direct AI/ML positioning (Case 3b) (Huawei)	other

R3-252002 (TP for AI/ML BLCR to TS 38.455) Discussion on RAN3 impacts for NG-RAN node-assisted AI/ML positioning (Case 3a) (Huawei)	other

R3-252028 (TPs to NRPPa and NGAP BL CRs to support case 3a) (Ericsson)	other

R3-252034 (TPs To BL CRs) Support Direct AI ML Positioning (CMCC)	other

R3-252035 Support Assisted AI ML Positioning (CMCC)	discussion

R3-252084 (TP to TS 38.305) Model training at gNB for Case 3a (Nokia)	other

R3-252085 (TP to TS 38.300) Intermediate feature reporting and general principles for case 3a (Nokia)	other

R3-252154 AI/ML-Enhanced Positioning Enhancements for NRPPa (Jio Platforms Ltd (JPL))	discussion

R3-252163 Model training/monitoring at gNB for Case 3a (CEWiT)	discussion