3GPP TSG-RAN WG3 #127-bis	R3-25xxxx

# Wuhan, China, 7-11 April, 2025

Agenda Item:	17.2

Source:	Huawei (Moderator)

Title: 	Summary of offline discussions: OndemandSSB

Document for:	Discussion and Approval

- Introduction

CB: # ES1_OndemandSSB

- Check the compromised solution as mentioned above

(moderator - HW)

- For the Chairman’s Notes

It is proposed to capture the following RAN3 agreements in the chairman’s notes:

TBD

3 Discussion on Support on-demand SSB SCell

3.1 Background

It was discussed at previous meeting with the follwing notes.

For the allowing On-demand SSB SCell, we discussed the two way-forward as follows: - WF1: No new List from the gNB-CU to the gNB-DU for “allowing OD SSB SCell” is needed, instead we can reuse, if needed, the Rel-18 existing “Cells Allowed to be Deactivated List” (CU to DU). Potential rewording of the procedure may be needed to make the “list” more general. - WF2: It is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”. But the above is not agreed, there are two concerns raised during the discussion: - One concern is that gNB-DU does not always need the “CU to DU” assistance in order to go to the OD-SSB SCell operation. - Another concern is how and whether to handle the transition from PCell normal operation to OD-SSB SCell only operation. To be continued at the next meeting on the above basis.

In the GNB-CU CONFIGURATION UPDATE message, the Cells Allowed to be Deactivated List is included.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose

9.2.1.10	GNB-CU CONFIGURATION UPDATE

| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

3.2 Discussion and conclusion

In order to indicate the allowed cells from the gNB-CU to the gNB-DU for on-demand SSB operation, following options are provided for discussion.

- Option 1:  Reuse the Cells Allowed to be Deactivated List IE, with the updating the procedure texts.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells or to enable the on-demand SSB operation for the indicated cells for network energy saving purpose.

- Option 2: Introduce new indicaor within the Cells Allowed to be Deactivated List

Cells Allowed to be Deactivated List; 0..1; YES; ignore. >Cells Allowed to be Deactivated List Item; 1 .. <maxCellingNBDU>; EACH; ignore. >>NR CGI; M; 9.3.1.12; -. >>On-demand SSB indication; O; ENUMERATED (true, ...); YES; Ignore.

- Option 3: Introduce a new cell allowed list, e.g., Cells Allowed to On-demand SSB List in the GNB-CU CONFIGURATION UPDATE message

Proposal: Take option?

Companies can provide comments in the following table.

| Company | Comment |
| --- | --- |

- Reference

| R3-251751 | (TP to BLCR for TS 38.473 and TS 38.470) Discussion on on-demand SSB SCell operation (Huawei) | other |
| --- | --- | --- |
| R3-252044 | On-demand SSB SCell operation (Ericsson) | discussion |
| R3-251623 | Aspects of on-demand SSB for NES enhancements (Qualcomm Inc.) | discussion |
| R3-251642 | (TP for BLCR to TS 38.423, 38.473) On support of OD-SSB (CATT) | other |
| R3-251650 | Open points on OD-SSB. Adaptation of common signals/channels (Nokia) | discussion |
| R3-251683 | Support on-demand SSB operation (NEC) | discussion |
| R3-251723 | Discussion on on-demand SSB SCell operation in low-carbon green network (Samsung) | discussion |
| R3-251860 | (TP to 38.473) On support on-demand SSB SCell operation (China Telecom) | discussion |
| R3-251892 | (TP to BL CR for 38.473) Further discussion on on-demand SSB Scell operation (ZTE Corporation) | other |
| R3-251951 | Discussion on On-demand SSB for SCell (Lenovo) | discussion |
| R3-252041 | Discussion on On-Demand SSB SCell Operation (CMCC) | discussion |
| R3-252117 | XnAP Enhancements for UL WUS Configuration in On-Demand SIB1 (Jio Platforms Ltd (JPL)) | discussion |