3GPP TSG-RAN WG3 #127bis	R3-252340

Wuhan, CN, 7 – 11 Apr. 2025

Agenda Item:	18.2. Support LP-WUS Indicating Paging Monitoring

Source:	NTT DOCOMO INC.

Title:	Summary of discussion on LP-WUS

Document for:	Discussion, agreement

For the Chairman’s Notes

TPs

TBD

LS on security concern on extension of UE_ID [R3-251644, R3-251645]

TBD

Introduction

The maximum number of subgroups is decided to 31.

- range of LP-WUS CN Subgroup ID is from 0 to 30: Samsung, E///, Nokia, HW,

- range of LP-WUS CN Subgroup ID is from 1 to 31: ZTE,

Turn the working assumption into agreement and remove the FFS on the presence of the LP-WUS Subgrouping Support Indication IE per cell in the F1AP Paging message.

RAN3 to update the stage2 BLCR by removing the restrictions of LP-WUS in case of emergency PDU session.

CB: # LP_WUS

- check with update TP in 1665.

- discuss on the draft to SA3, and capture the agreements.

- update the TPs according to offline agreements.

(Moderator- NTT DCM)

Offline discussion summary in R3-252340

Discussion

TPs

- R3-251665: TP for BL CR for TS 38.473 (Nokia) rev in R3-252342

- R3-251754: TP for BL CR for TS 38.300 (HW) rev in R3-25xxxx

LS on security concern on extension of UE_ID [R3-251644, R3-251645]

It is proposed by CATT that there may be security concern to extend Extended UE Identity Index IE as illustrated as following:

- However, the increased length of UE_ID (i.e., truncated 5G-S-TMSI) may lead to security risk. Note that delivering the entire 5G-S-TMSI over F1/Xn interface may cause some security issues, and that is the reason why the truncated 5G-S-TMSI is used from R15. Thus, if RAN3 assumes increasing the length of truncated 5G-S-TMSI for LP-WUS subgrouping is needed, it should ask SA3 whether there is any concern.

- Observation 3: Further increasing the length of UE_ID delivered over F1/Xn interfaces may have security risk.

Can we agree with following proposal?

Proposal 1: RAN3 should send an LS to SA3 to ask if there is any concern on further increasing the length of UE_ID over F1/Xn interfaces for LP-WUS subgrouping.

Conclusions

TBD

References

CRs

- R3-251568, TS 38.470, Qualcomm

- R3-251569, TS 38.473, Huawei

- R3-251570, TS 38.300, Vivo

- R3-251571, TS 38.423, Ericsson

- R3-251572, TS 38.413, ZTE

Contributions

- R3-251625, Qualcomm

- R3-251644, CATT

- R3-251645, CATT (LS out)

- R3-251664, Nokia

- R3-251665, Nokia (TP for 38.473)

- R3-251693, NEC

- R3-251698, ZTE

- R3-251699, ZTE (TP for 38.413)

- R3-251754, Huawei (TP for 38.413/38.423/38.300)

- R3-251755, Huawei (TP for 38.473)

- R3-251756, Huawei (LS out)

- R3-252023, Ericsson

- R3-252024, Ericsson (TP for 38.423)

- R3-252147, NTT Docomo

- R3-252181, CMCC (TP for 38.473)

- R3-252209, Samsung