3GPP TSG-RAN WG3 #127bis                                                                           R3-25xxxx

Wuhan, China, April 7th – 11th, 2025

Source:	CATT, Ericsson, Huawei, Samsung, China Telecomm, Rakuten, ZTE, CMCC, Lenovo, Deutsche Telekom

Title:	(TP to BLCR for TS 38.300) Introduction of OD-SIB1

Agenda Item:	17.3

Document for:	Approval

1 Introduction

The TP to BL CR for TS38.300 for introducing on-demand SIB1 is provided based on the agreements achieved in previous RAN3 meetings as well as the agreements made in RAN3#127bis.

RAN3#127bis: In the new Class 1 message (Direction NES gNB to Cell A gNB), RAN3 has agreed that: - One Choice “Start with UL WUS Condiguration” - One Choice “Stop” meaning the Cell A gNB will remove/release/discard the UL WUS Configuration and stop the broadcasting, it also means that next time the NES gNB should use “Start with UL WUS Configuration”..

2 Reference

- Draft Report of 3GPP TSG RAN3 meeting #126, MCC

- Draft Report of 3GPP TSG RAN3 meeting #127, MCC

3 TP to BL CR for TS38.300

---------------------------------------START OF TP -------------------------------------------

#### 15.4.2.x2	On-demand SIB1

To facilitate reducing gNB downlink transmissions, the gNB can provide SIB1 on-demand, i.e., upon receiving OD-SIB1 request from the UE. On-demand SIB1 is supported for UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED when T311 is running. A request for SIB1 triggers a random access procedure, in which case MSG1 is used for indicating OD-SIB1 request and the gNB acknowledges the request in MSG2. UL-WUS configurations of one or more cells are included in SIBxx, which can be broadcasted in any cell, including cell’s own UL-WUS configuration. While the UE is camped on a cell, it can use the UL-WUS configuration of another cell from SIBxx valid in the camped cell to acquire OD-SIB1 of that cell for cell reselection or it can apply the UL-WUS configuration of the camped cell from SIBxx valid in the camped cell to acquire OD-SIB1 of the camped cell.

A gNB is allowed to send UL-WUS configuration information of the OD-SIB1 cell to neighbour gNB(s) and request the neighbour gNB(s) to broadcast the UL-WUS configuration. If the request is accepted, the gNB(s) receiving the UL-WUS configuration information start the broadcast via SIBxx in the relevant cell(s). The gNB can also inform the neighbour gNB(s) to discontinue the broadcast in case the OD-SIB1 cell reverts to the periodical SIB1 mode. If a gNB broadcasting the UL-WUS configuration information decides to stop broadcasting by itself, it informs the gNB which requests the broadcast over the Xn interface.

Editor’s Note: This paragraph may need to be refined in future.

---------------------------------------END OF TP -------------------------------------------