3GPP TSG-RAN WG3 Meeting #127bis	R3-252365

Wuhan, China, 7-11 April 2025

Agenda Item:	21.3

Source:	Huawei, Nokia, Nokia Shanghai Bell, CMCC, CATT

Title:	(TP to BL CR for TS 38.473) Support of PDU set based QoS handling enhancement

Document for:	Approval

1. Introduction

This contribution provides TP for F1AP to reflect the change for supporting of PDU set based QoS handling enhancement as listed in the updated WID in RP-250107 [1]. The corresponding objective is copied below

- Support of PDU set based QoS handling enhancement [RAN3]: - Support of DL PDU Set marking without PDU Set QoS - Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER). ……

The TP includes following changes:

- Extend the codepoint of Notification Cause to support indicating the direction of not fulfilled QoS;

- Add PSDB, PSER for UL and DL in Alternative QoS parameters.

Annex: TP for TS 38.473 on Alternative PDU Set QoS

CHANGES START

9.2.2.13	NOTIFY

This message is sent by the gNB-DU to notify the gNB-CU that the QoS for already established DRBs associated with notification control is not fulfilled any longer or it is fulfilled again.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DRB Notify List |  | 1 |  |  | YES | reject |
| >DRB Notify Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Notification Cause | M |  | ENUMERATED(Fulfilled, Not-Fulfilled, ..., not fulfilled DL, not fulfilled UL) |  | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters set Notify Index 9.3.1.124 | Index to the currently fulfilled alternative QoS parameters set. Value 0 indicates that NG-RAN cannot even fulfil the lowest alternative parameter set. | YES | ignore |
| >>TSC Traffic Characteristics Feedback | O |  | 9.3.1.302 |  | YES | ignore |

NEXT CHANGE

9.3.1.125	Alternative QoS Parameters Set List

This IE contains alternative sets of QoS parameters which the gNB can indicate to be fulfilled when notification control is enabled and it cannot fulfil the requested list of QoS parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Item |  | 1..<maxnoofQoSparaSets> |  |  | - |  |
| >Alternative QoS Parameters Set Index | M |  | 9.3.1.123 |  | - |  |
| >Guaranteed Flow Bit Rate Downlink | O |  | Bit Rate 9.3.1.22 |  | - |  |
| >Guaranteed Flow Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 |  | - |  |
| >Packet Delay Budget | O |  | 9.3.1.51 |  | - |  |
| >Packet Error Rate | O |  | 9.3.1.52 |  | - |  |
| >Maximum Data Burst Volume | O |  | 9.3.1.54 | Maximum Data Burst Volume is specified in TS 23.501 [21]. This IE is included if the Delay Critical IE is set to “delay critical” and is ignored otherwise. | YES | ignore |
| >PDU Set Delay Budget Downlink | O |  | Extended Packet Delay Budget 9.3.1.145 |  | - |  |
| >PDU Set Delay Budget Uplink | O |  | Extended Packet Delay Budget 9.3.1.145 |  | - |  |
| >PDU Set Error Rate Downlink | O |  | Packet Error Rate 9.3.1.52 |  | - |  |
| >PDU Set Error Rate Uplink | O |  | Packet Error Rate 9.3.1.52 |  | - |  |

NEXT CHANGE

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

---------------- unchanged part skipped ---------------------

id-LocalOrigin,

id-SRSPosPeriodicConfigHyperSFNIndex,

id-PduSetDelayBudgetDownlink,

id-PduSetDelayBudgetUplink,

id-PduSetErrorRateDownlink,

id-PduSetErrorRateUplink,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

#### -- A

---------------- unchanged part skipped ---------------------

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQoSParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParaSetIndex			QoSParaSetIndex,

guaranteedFlowBitRateDL				BitRate					OPTIONAL,

guaranteedFlowBitRateUL				BitRate					OPTIONAL,

packetDelayBudget					PacketDelayBudget		OPTIONAL,

packetErrorRate						PacketErrorRate			OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-MaxDataBurstVolume 	CRITICALITY ignore	EXTENSION MaxDataBurstVolume		PRESENCE optional		}|

{ ID id-PduSetDelayBudgetDownlink 	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-PduSetDelayBudgetUplink		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-PduSetErrorRateDownlink		CRITICALITY ignore	EXTENSION PacketErrorRate				PRESENCE optional		}|

{ ID id-PduSetErrorRateUplink		CRITICALITY ignore	EXTENSION PacketErrorRate				PRESENCE optional		},

...

}

---------------- unchanged part skipped ---------------------

#### -- N

---------------- unchanged part skipped ---------------------

NonUPTrafficType ::=	ENUMERATED {ue-associated, non-ue-associated, non-f1, bap-control-pdu,...}

NoofDownlinkSymbols	::= INTEGER (0..14)

NoofUplinkSymbols	::= INTEGER (0..14)

Notification-Cause ::= ENUMERATED {fulfilled, not-fulfilled, ..., not-fulfilled-dl, not-fulfilled-ul}

NotificationControl ::= ENUMERATED {active, not-active, ...}

NotificationInformation ::= SEQUENCE {

message-Identifier	MessageIdentifier,

serialNumber		SerialNumber,

iE-Extensions	ProtocolExtensionContainer { { NotificationInformationExtIEs} } OPTIONAL,

...

}

NotificationInformationExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

...

}

NEXT CHANGE

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

---------------- unchanged part skipped ---------------------

#### -- IEs

--

-- **************************************************************

---------------- unchanged part skipped ---------------------

id-MeasBasedOnAggregatedResources					ProtocolIE-ID ::= 847

id-SIB23-message									ProtocolIE-ID ::= 848

id-BarringExemptionforEmerCallInfo					ProtocolIE-ID ::= 849

id-SIB17bis-message									ProtocolIE-ID ::= 850

id-ReportingIntervalIMs								ProtocolIE-ID ::= 851

id-Transmission-Bandwidth-asymmetric				ProtocolIE-ID ::= 852

id-TagIDPointer										ProtocolIE-ID ::= 853

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-PduSetDelayBudgetDownlink						ProtocolIE-ID ::= a1

id-PduSetDelayBudgetUplink							ProtocolIE-ID ::= a2

id-PduSetErrorRateDownlink							ProtocolIE-ID ::= a3

id-PduSetErrorRateUplink							ProtocolIE-ID ::= a4

END

-- ASN1STOP

CHANGES END