3GPP TSG-RAN WG3 #127bis                                                 R3-252372

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	21.3

Source:	CATT, Nokia, Nokia Shanghai Bell, CMCC, Huawei

Title:	(TP to BL CR for TS 38.423) Support of available data rate report

Document for:	Other

Introduction

This TP reflect the change for supporting available data rate report.

The TP includes following changes:

- Add Monitoring Request on Available Data Rate IE in the GBR QoS Flow Information IE.

Annex: TP for 38.423 XR BL CR to support available data rate report

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

<<<Unchanged part is omitted>>>

If the DL LBT Failure Information Request IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the DL LBT failure information of the UE in the target cell for MRO analysis, as specified in TS 38.300 [9].

For each GBR QoS flow which has been successfully established in the target NG-RAN node, if the Monitoring Request on Available Data Rate IE was included in the GBR QoS Flow Information IE in the PDU Session Resource To Be Setup List IE contained in HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information and perform Available bitrate monitoring, as specified in TS 23.501 [7].

<<<Next change >>>

### 8.2.4	Retrieve UE Context

#### 8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer, or to request for small data transmission. The procedure can also be used to transfer the authorization status information of the mobile IAB-node.

The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

<<<Unchanged part is omitted>>>

If the UE is a mobile IAB-node, the old NG-RAN node shall include the Mobile IAB Authorization Status IE in the RETRIEVE UE CONTEXT RESPONSE message. If the Mobile IAB Authorization Status IE is included in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, consider that the UE is a mobile IAB-node, then store it and use it accordingly as defined in TS 38.401 [2].

For each GBR QoS flow in the new NG-RAN node, if the Monitoring Request on Available Data Rate IE was included in the GBR QoS Flow Information IE in the PDU Session Resource To Be Setup List IE contained in RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall if supported, store this information and perform Available bitrate monitoring, as specified in TS 23.501 [7].

<<<Next change >>>

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE. Possible parallel requests are identified by the PCell ID when the initiating NG-RAN node UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

<<<Unchanged part is omitted>>>

If the S-NODE ADDITION REQUEST message contains the IAB Authorization status IE, the S-NG-RAN node shall, if supported, store it and use it as defined in TS 38.401[2].

If the Monitoring Request on Available Data Rate IE is included in the GBR QoS Flow Information IE for a QoS flow contained in the DRBs To Be Setup List IE of the PDU Session Resource Setup Info – MN terminated IE, the S-NG-RAN node shall, if supported, store this information and perform Available bitrate monitoring, as specified in TS 23.501 [7].

For each GBR QoS flow which has been successfully established in the S-NG-RAN node, if the Monitoring Request on Available Data Rate IE was included in the GBR QoS Flow Information IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall store this information, and shall, if supported, store this information and perform Available bitrate monitoring, as specified in TS 23.501 [7].

<<<Next change >>>

### 8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

#### 8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUEST message to the S-NG-RAN node.

<<<Unchanged part is omitted>>>

For QoS flow offloading from the S-NG-RAN node to the M-NG-RAN, the S-NG-RAN node may provide the data forwarding related information in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE within the Data Forwarding and offloading Info from source NG-RAN node IE, in which case the M-NG-RAN node may decide to provide data forwarding addresses to the S-NG-RAN node and trigger the Xn-U Address Indication procedure as specified in TS 37.340 [8].

If the Monitoring Request on Available Data Rate IE is included in the GBR QoS Flow Information IE for a QoS flow contained in the DRBs To Be Setup List IE or the DRBs To Be Modified List IE within the PDU Session Resource Setup Info – MN terminated IE or the PDU Session Resource Modification Info – MN terminated IE, the S-NG-RAN node shall, if supported, store this information and perform Available bitrate monitoring, as specified in TS 23.501 [7]

For each GBR QoS flow which has been successfully added or modified in the S-NG-RAN node, if the Monitoring Request on Available Data Rate IE was included in the GBR QoS Flow Information IE contained in the PDU Session Resource Setup Info – SN terminated IE or the PDU Session Resource Modification Info – SN terminated IE, the S-NG-RAN node shall, if supported, store this information and perform Available bitrate monitoring, as specified in TS 23.501 [7].

<<<Next change >>>

9.2.3.6	GBR QoS Flow Information

This IE indicates QoS Parameters for a GBR QoS Flow for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum Flow Bit Rate Downlink | M |  | Bit Rate 9.2.3.4 | Maximum Bit Rate in DL. Flow Bit Rates are specified in TS 23.501 [7]. | – |  |
| Maximum Flow Bit Rate Uplink | M |  | Bit Rate 9.2.3.4 | Maximum Bit Rate in UL. Flow Bit Rates are specified in TS 23.501 [7]. | – |  |
| Guaranteed Flow Bit Rate Downlink | M |  | Bit Rate 9.2.3.4 | Guaranteed Bit Rate (provided that there is data to deliver) in DL. Flow Bit Rates are specified in TS 23.501 [7]. | – |  |
| Guaranteed Flow Bit Rate Uplink | M |  | Bit Rate 9.2.3.4 | Guaranteed Bit Rate (provided that there is data to deliver). Flow Bit Rates are specified in TS 23.501 [7]. | – |  |
| Notification Control | O |  | ENUMERATED (notification requested, ...) | Notification control is specified in TS 23.501 [7] | – |  |
| Maximum Packet Loss Rate Downlink | O |  | Packet Loss Rate 9.2.3.11 | Indicates the maximum rate for lost packets that can be tolerated in the downlink direction. Maximum Packet Loss Rate is specified in TS 23.501 [7]. | – |  |
| Maximum Packet Loss Rate Uplink | O |  | Packet Loss Rate 9.2.3.11 | Indicates the maximum rate for lost packets that can be tolerated in the uplink direction. Maximum Packet Loss Rate is specified in TS 23.501 [7]. | – |  |
| Alternative QoS Parameters Set List | O |  | 9.2.3.102 | Indicates alternative sets of QoS Parameters for the QoS flow. | YES | ignore |
| Monitoring Request on Available Data Rate |  | 0..1 |  |  | YES | ignore |
| >Monitoring Request | M |  | ENUMERATED (ul, dl, both, stop, …) | Indicates to monitor and report UL, or DL, or both UL/DL available data rate for the associated QoS flow as specified in TS 23.501 [7], or stop the corresponding QoS monitoring. | - | ignore |
| >DL Available Data Rate Report Thresholds | C-ifReportDL |  | Available Data Rate Report Threshold List 9.2.3.x |  | - | ignore |
| >UL Available Data Rate Report Thresholds | C-ifReportUL |  | Available Data Rate Report Threshold List 9.2.3.x |  | - | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportDL | This IE shall be present if the Monitoring Request IE is set to the value “dl” or “both”. |
| ifReportUL | This IE shall be present if the Monitoring Request IE is set to the value “ul” or “both”. |

<<<Next change>>>

9.2.3.x 	Available Data Rate Report Threshold List

This IE contains a list of available data rate report thresholds. It is used for available data rate report for UL and DL as specified in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Available Data Rate Report Threshold Item |  | 1..<maxnoofThresholds> |  |  |
| >Reporting Threshold | M |  | INTEGER (0..FFS) | FFS |

| Range bound | Explanation |
| --- | --- |
| maxnoofThresholds | Maximum no. of thresholds allowed to be provided by the SMF. Value is FFS. |

<<<Next change>>>

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<<<Unchanged part is omitted>>>

id-Transmission-Bandwidth-asymmetric,

id-NRPPaPositioningInformation,

id-MonitoringRequestonAvailableDataRate,

maxEARFCN,

<<<Unchanged part is omitted>>>

maxnoofSecurityConfigurations,

maxnoofRSPPQoSFlows,

maxnoofThresholds

<<<Unchanged part is omitted>>>

#### -- A

A2XPC5FlowBitRates ::= SEQUENCE {

a2XguaranteedFlowBitRate		BitRate,

a2XmaximumFlowBitRate			BitRate,

iE-Extensions		ProtocolExtensionContainer { { A2XPC5FlowBitRates-ExtIEs} }		OPTIONAL,

...

}

A2XPC5FlowBitRates-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AvailableDataRateReportThresholdList	::= SEQUENCE (SIZE(1..maxnoofThresholds)) OF AvailableDataRateReportThresholdItem

AvailableDataRateReportThresholdItem	::= SEQUENCE {

reportingThreshold				ReportingThreshold,

iE-Extensions					ProtocolExtensionContainer { { AvailableDataRateReportThresholdItem-ExtIEs} }	OPTIONAL,

...

}

AvailableDataRateReportThresholdItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated ::= SEQUENCE (SIZE(1..maxnoofTargetSNsMinusOne)) OF AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated-Item

AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated-Item ::= SEQUENCE {

pDUSessionResourceChangeConfirmInfo-SNterminated				PDUSessionResourceChangeConfirmInfo-SNterminated,

iE-Extensions		ProtocolExtensionContainer { { AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated-Item-ExtIEs} } OPTIONAL,

...

}

<<<Unchanged part is omitted>>>

#### -- G

GBRQoSFlowInfo ::= SEQUENCE {

maxFlowBitRateDL			BitRate,

maxFlowBitRateUL			BitRate,

guaranteedFlowBitRateDL		BitRate,

guaranteedFlowBitRateUL		BitRate,

notificationControl			ENUMERATED {notification-requested, ...}				OPTIONAL,

maxPacketLossRateDL			PacketLossRate											OPTIONAL,

maxPacketLossRateUL			PacketLossRate											OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {GBRQoSFlowInfo-ExtIEs} }	OPTIONAL,

...

}

GBRQoSFlowInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-AlternativeQoSParaSetList	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetList	PRESENCE optional	}|

{ ID id-MonitoringRequestonAvailableDataRate			CRITICALITY ignore	EXTENSION MonitoringRequestonAvailableDataRate		PRESENCE optional },

...

}

<<<Unchanged part is omitted>>>

#### -- M

M1ReportingTrigger ::= ENUMERATED{

periodic,

a2eventtriggered,

a2eventtriggered-periodic,

...

}

MonitoringRequestonAvailableDataRate ::= SEQUENCE {

monitoringRequest						MonitoringRequest,

dlAvailableDataRateReportThresholds		AvailableDataRateReportThresholdList			OPTIONAL,

-- The above IE shall be present if the Monitoring Request IE is set to the value “dl” or “both”

ulAvailableDataRateReportThresholds		AvailableDataRateReportThresholdList			OPTIONAL,

-- The above IE shall be present if the Monitoring Request IE is set to the value “ul” or “both”

iE-Extensions							ProtocolExtensionContainer { { MonitoringRequestonAvailableDataRate-ExtIEs} } OPTIONAL,

...

}

MonitoringRequestonAvailableDataRate-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MonitoringRequest ::= ENUMERATED {ul, dl, both, stop,...}

M1ThresholdEventA2 ::= SEQUENCE {

measurementThreshold	MeasurementThresholdA2,

iE-Extensions			ProtocolExtensionContainer { { M1ThresholdEventA2-ExtIEs} } OPTIONAL,

...

}

M1ThresholdEventA2-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<<Unchanged part is omitted>>>

#### -- R

RadioResourceStatusNR-U	::= SEQUENCE {

dL-Total-PRB-usage			INTEGER (0..100),

uL-Total-PRB-usage			INTEGER (0..100),

iE-Extensions		ProtocolExtensionContainer {{ RadioResourceStatusNR-U-ExtIEs}} OPTIONAL,

...

}

RadioResourceStatusNR-U-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ReportingThreshold	::= INTEGER (0..FFS)

<<<Unchanged part is omitted>>>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

<<<Unchanged part is omitted>>>

maxnoofSecurityConfigurations				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofThresholds							INTEGER ::= FFS

<<<Unchanged part is omitted>>>

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-MonitoringRequestonAvailableDataRate															ProtocolIE-ID ::= yy1

END

-- ASN1STOP