3GPP TSG-RAN WG3 Meeting #127-bis	R3-252314

Wuhan, China, 7-11 April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0152 | rev | 2 | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of TSC Traffic Characteristics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, China Telecom, Ericsson, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2025-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Periodicity IE included in the TSC Traffic Characteristics IE starts from zero in the Tabular while in the ASN.1 it starts from one. Note that the periodicity starts from zero in other specifications (e.g., NGAP) though value zero does not make much sense. In addition, the TSC Traffic Characteristics IE includes the TSC Assistance Information Downlink and TSC Assistance Information Uplink, but in the ASN.1, it includes both in the different order. This may lead to incorrect interpretations when delivering the different values for UL and DL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the periodicity IE starting from zero in the ASN.1. - Switch the TSC Assistance Information Downlink and TSC Assistance Information Uplink in the TSC Traffic Characteristics IE in the Tabular. This CR is NBC. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it impacts the TSC Traffic Characteristics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The value range for the Periodicity IE is not consistent between the Tabular and the ASN.1. There may have incorrect interpretations of the TSC Traffic Characteristics IE when delivering different values for UL and DL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.75, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial version: R3-250323 Rev1: R3-251820 Revert the changes for the periodicity IE by correcting the ASN.1. Update the cover page including the Category, work item code etc. Rev2: R3-252314 Add co-sources. |  |  |  |  |  |  |  |  |

Change Begins

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.3.1.75	TSC Traffic Characteristics

This IE provides the traffic characteristics of TSC QoS flows.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TSC Assistance Information Uplink | O |  | TSC Assistance Information 9.3.1.76 |  |
| TSC Assistance Information Downlink | O |  | TSC Assistance Information 9.3.1.76 |  |

<<<<<<<<<<<<<<<<<<<< For Information Only >>>>>>>>>>>>>>>>>>>>

9.3.1.77	Periodicity

This IE indicates the Periodicity of the TSC QoS flow as defined in TS 23.501 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | INTEGER (0..640000, …) | Periodicity expressed in units of 1 us. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

TSCTrafficCharacteristics		::= SEQUENCE {

tSCTrafficCharacteristicsUL				TSCAssistanceInformation			OPTIONAL,

tSCTrafficCharacteristicsDL				TSCAssistanceInformation			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { TSCTrafficCharacteristics-ExtIEs } }	OPTIONAL

}

TSCTrafficCharacteristics-ExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

TSCAssistanceInformation			::= SEQUENCE {

periodicity						Periodicity,

burstArrivalTime						BurstArrivalTime				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { TSCAssistanceInformation-ExtIEs } }	OPTIONAL

}

TSCAssistanceInformation-ExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

{ID id-SurvivalTime				CRITICALITY ignore		EXTENSION SurvivalTime				PRESENCE optional}|

{ID id-N6JitterInformation		CRITICALITY ignore		EXTENSION N6JitterInformation		PRESENCE optional},

...

}

Periodicity					::= INTEGER	(0..640000,	...)

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Change Ends