3GPP TSG-RAN WG3 Meeting #127-bis                           	  R3-252330

Wuhan, CN, 7th – 11th April, 2025

Source:	CATT

Title:	(TP to BL CR for TS38.300) A-IoT protocol stack

Agenda Item:	16.2

Document for:	Discussion and Decision

1.	TP to BL CR for TS 38.300

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>>>>>

16.xx	Support of Ambient IoT

16.xx.x1	Introduction

16.xx.x2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or more readers.

-	As specified in <reference to SA2 TS to be added> the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct communication with the AIOTF) or the AMF (in case of indirect connectivity with the AIOTF).

NOTE 1:	It is not expected a deployment will deploy both direct connectivity and indirect connectivity.

-	In this version of the specification A-IoT specific information is transported between the gNB and the A-IoT CN node via the NG-C interface, the protocol stack in section 4.3.1.2 applies. In case of indirect connectivity with the AIOTF, A-IoT specific information is transferred transparently via the AMF.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>