3GPP TSG-RAN WG3 Meeting #127bis	R3-252350

Wuhan, China, 7 - 11 April 2025

Agenda Item:	21.3

Source:	Huawei, CMCC, CATT, Nokia, Nokia Shanghai Bell, BT, Vodafone, ZTE, Lenovo, Ericsson

Title:	(TP for XR BL CR for TS 37.340) Support for ECN marking status change

Document for:	Approval

1	Introduction

This paper provides the TP for capturing the stage 2 description for the SN initiated ECN marking/congestion reporting status change procedure.

Annex ——TP for BL CR for TS 37.340

CHANGES START

### 13.y	ECN marking for L4S and congestion information exposure

The ECN marking for L4S and congestion information exposure as described in TS 38.300 [3] is extended to address the NR-DC operation.

For ECN marking for L4S at gNB or UPF, during the Xn SN Addition Preparation procedure or MN initiated SN Modification Preparation procedure, the MN provides the ECN marking request at gNB or ECN marking request at UPF to SN and SN reports the status indication back to the MN. The SN may change the status and report the new status indication to the MN during the SN initiated SN Modification procedure.

For congestion reporting from gNB to UPF, during the Xn SN Addition Preparation procedure or MN initiated SN Modification Preparation procedure, the MN provides the congestion information request to SN and SN reports the status indication back to the MN. The SN may change the status and report the new status indication to the MN during the SN initiated SN Modification procedure.

CHANGES END