3GPP TSG-RAN WG3 Meeting #127-bis	R3-252430

Wuhan, China, 7-11 April, 2025

Agenda item:	19.2

Source: 	ZTE Corporation, Ericsson, Samsung

Title: 	(TP to BL CR for 38.401) introduction of Evolution of NR duplex operation: Sub-band full duplex (SBFD)

Document for:	Discussion & Approval

Introduction

This TP tries to capture the agreements achieved for SBFD.

- Annex:	TP to BL CR for TS 38.401

/*********************Start of changes***********************/

7.3	Cross-Link Interference Management

The Cross-Link Interference Management function in non-split gNB case is specified in TS 38.300 [2].

In case of split gNB architecture, the gNB-CU forwards the TDD DL/UL patterns received from neighboring nodes to each concerned gNB-DU. The gNB-DU reports the TDD DL/UL patterns of its serving cells to the gNB-CU if Cross-Link Interference is detected.

In Sub-band full duplex (SBFD) operations Cross Link Interference (CLI) may also be present. In case of split gNB architecture, the gNB-DU reports CLI related information of its serving cells to the gNB-CU if CLI is detected. The gNB-CU forwards CLI related information received from served gNB-DUs and from neighboring gNBs to each concerned gNB-DU.

/*********************End of changes***********************/