3GPP TSG-RAN WG3 Meeting #127bis	R3-252486

Wuhan, China, 7 – 11 April, 2025

Agenda item:	20.2

Source:	CATT, Ericsson, Nokia, Huawei, Xiaomi, ZTE, CMCC, Samsung, CEWiT

Title:	(TP to TS 38.305) Model training at gNB for Case 3a

Document for:	Decision

1	Introduction

In this contribution, we provide a TP to BL CR for TS 38.305 related to AI/ML Positioning case 3a.

TP to TS 38.305

<<< start of changes >>>

7.x	AI/ML assisted positioning with gNB side model

7.x.1	General

NG-RAN support for AI/ML positioning requires inputs of ground truth and related data from LMF and measurements by the gNB for the purpose of management of the corresponding gNB side AI/ML.

7.x.2	Positioning Data Collection Procedure

Figure 7.x.2-1 shows the Positioning Data Collection procedure used to retrieve positioning information for a UE that is undergoing a positioning process.

Figure 7.x.2-1: Positioning Data Collection Procedure

1.	The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to positioning procedures.

2.	The gNB(s) determines that data collection is needed for the UE being positioned.

3.	The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and indicates that data collection is needed for the UE being positioned.

Note:	Steps 1 to 3 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [35].

4.	The LMF sends the NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated that positioning data collection is needed for the UE being positioned. The message includes information related to UE location and correlation information that enables the gNB to correlate the information related to UE location with information related to UL measurements (e.g., LMF Measurement ID and RAN Measurement ID).

Editor’s Note: The texts above could be further refined, if needed.

Editor’s Note: FFS on details of the signallings and IEs.

<<< end of changes >>>