3GPP TSG-RAN WG3 Meeting #127-bis	R3-252496

Wuhan, China, 7-11 April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1260 | rev | 3 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 38.413) Introduction of Ambient IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-04-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To support Ambient IoT, it is needed to introduce related NGAP procedures, messages and IEs based on the discussion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the following procedures with corresponding messages/IEs: - Inventory Request - Inventory Report - Inventory Report |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support Ambient IoT related RAN CN signalling exchanging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 8.1, 8.7.1, 8.7.2, 8.7.4, 8.7.5, 8.xx.1 (new), 8.xx.2 (new), 8.xx.3 (new), 9.2.x (new) , 9.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 38.300 CR ... TS 38.410 CR 0053 TS 38.401 CR 0469 TS 38.412 CR 0025 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev- : initial version, merge agreed TP in R3-250910. Rev-1: resubmission, change the Command Failure Transfer IE from reject to ignore, add asn.1 part. Rev-2: fix tittle and WI code. Rev-3: merge agreed TPs R3-252480, R3-252336. |  |  |  |  |  |  |  |  |

-----------------Start of the First Change-----------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture description".

[3]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[4]	ITU-T Recommendation X.691 (07/2002): "Information technology – ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[5]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[6]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[7]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[8]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[9]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[11]	3GPP TS 32.422: "Trace control and configuration management".

[12]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode and in RRC inactive state".

[13]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[14]	3GPP TS 38.414: "NG-RAN; NG data transport".

[15]	3GPP TS 29.281: "General Packet Radio System (GPRS); Tunnelling Protocol User Plane (GTPv1-U)".

[16]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[17]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[18]	3GPP TS 38.331: "NG-RAN; Radio Resource Control (RRC) Protocol Specification".

[19]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[20]	Void

[21]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC); Protocol specification".

[22]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[23]	3GPP TS 23.003: "Numbering, addressing and identification".

[24]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[25]	IETF RFC 5905 (2010-06): "Network Time Protocol Version 4: Protocol and Algorithms Specification".

[26]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[27]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[28]	3GPP TS 25.413: "UTRAN Iu interface RANAP signalling".

[29]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[30]	3GPP TS 29.531: "5G System; Network Slice Selection Services; Stage 3".

[31]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[32]	3GPP TS 37.340: " Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[33]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[34]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[35]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[36]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[37]	CableLabs WR-TR-5WWC-ARCH: "5G Wireless Wireline Converged Core Architecture".

[38]	3GPP TS 36.401: "E-UTRAN Architecture Description".

[39]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[40]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP) ".

[41]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA), Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".

[42]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities".

[43]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[44]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[45]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration".

[46]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[47]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[48]	3GPP TS 38.314: "NR; Layer 2 Measurements".

[49]	3GPP TS 36.314: "Evolved Universal Terrestrial Radio Access (E-UTRA); Layer 2 - Measurements".

[50]	3GPP TS 23.203: "Policy and charging control architecture".

[51]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[52]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[53]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[54]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[55]	IEEE Std 1588: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control Systems", Edition 2019.

[56]	3GPP TS 29.585: "5G System (5GS); Session Management Function (SMF) / Centralized User Configuration (CUC) to Access Network Talker Listener (AN-TL) and Core Network Talker Listener (CN-TL) protocol aspects; Stage 3".

[57]	3GPP TS 23.527: "5G System; Restoration procedures".

[58]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[z]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things; Stage 2".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

ACL functionality: as defined in TS 36.413 [16].

A-IoT CN node: as defined in TS 38.300 [8].

CAG cell: as defined in TS 38.300 [8].

DAPS Handover: as defined in TS 38.300 [8].

Elementary Procedure: NGAP consists of Elementary Procedures (EPs). An Elementary Procedure is a unit of interaction between the NG-RAN node and the AMF. These Elementary Procedures are defined separately and are intended to be used to build up complete sequences in a flexible manner. If the independence between some EPs is restricted, it is described under the relevant EP description. Unless otherwise stated by the restrictions, the EPs may be invoked independently of each other as standalone procedures, which can be active in parallel. The usage of several NGAP EPs together or together with EPs from other interfaces is specified in stage 2 specifications (e.g., TS 38.401 [2], TS 38.410 [3] and TS 38.300 [8]).

An EP consists of an initiating message and possibly a response message. Two kinds of EPs are used:

-	Class 1: Elementary Procedures with response (success and/or failure).

-	Class 2: Elementary Procedures without response.

For Class 1 EPs, the types of responses can be as follows:

Successful:

-	A signalling message explicitly indicates that the elementary procedure successfully completed with the receipt of the response.

Unsuccessful:

-	A signalling message explicitly indicates that the EP failed.

-	On time supervision expiry (i.e., absence of expected response).

Successful and Unsuccessful:

-	One signalling message reports both successful and unsuccessful outcome for the different included requests. The response message used is the one defined for successful outcome.

Class 2 EPs are considered always successful.

en-gNB: as defined in TS 37.340 [32].

gNB: as defined in TS 38.300 [8].

MBS session resource: as defined in TS 38.401 [2].

Mobile IAB-MT: as defined in TS 38.300 [8].

Mobile IAB-node: as defined in TS 38.300 [8].

NB-IoT: as defined in TS 36.300 [17].

ng-eNB: as defined in TS 38.300 [8].

NG-RAN node: as defined in TS 38.300 [8].

Non-CAG cell: as defined in TS 38.300 [8].

PDU session resource: as defined in TS 38.401 [2].

Public Network Integrated NPN: as defined in TS 23.501 [9].

Stand-alone Non-Public Network: as defined in TS 23.501 [9].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

ACL	Access Control List

A-IoT	Ambient IoT

AIOTF	Ambient IoT Function

AMF	Access and Mobility Management Function

AUN3	Authenticable Non-3GPP

CAG	Closed Access Group

CGI	Cell Global Identifier

CP	Control Plane

DAPS	Dual Active Protocol Stacks

DC	Dual Connectivity

DL	Downlink

EPC	Evolved Packet Core

FN-RG	Fixed Network Residential Gateway

GUAMI	Globally Unique AMF Identifier

HFC	Hybrid Fiber-Coax

IAB	Integrated Access and Backhaul

IoT	Internet of Things

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

MBS	Multicast/Broadcast Service

MT	Mobile Terminated

N3IWF	Non 3GPP InterWorking Function

NB-IoT	Narrow Band Internet of Things

NGAP	NG Application Protocol

NID	Network Identifier

NPN	Non-Public Network

NRPPa	NR Positioning Protocol Annex

NSAG	Network Slice AS Group

NSCI	New Security Context Indicator

NSSAI	Network Slice Selection Assistance Information

OTDOA	Observed Time Difference of Arrival

PEIPS	Paging Early Indication with Paging Subgrouping

PNI-NPN	Public Network Integrated Non-Public Network

ProSe	Proximity Services

PSCell	Primary SCG Cell

PTM	Point to Multipoint

PTP	Point to Point

QMC	QoE Measurement Collection

QoE	Quality of Experience

RedCap	Reduced Capability

RIM	Remote Interference Management

RIM-RS	RIM Reference Signal

RSN	Redundancy Sequence Number

RSPP	Ranging/SL Positioning Protocol

SCG	Secondary Cell Group

SCTP	Stream Control Transmission Protocol

SgNB	Secondary gNB

SMF	Session Management Function

S-NG-RAN node	Secondary NG-RAN node

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

TAC	Tracking Area Code

TAI	Tracking Area Identity

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNLA	Transport Network Layer Association

TSS	Timing Synchronisation Status

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

UL	Uplink

UP	User Plane

UPF	User Plane Function

V2X	Vehicle-to-Everything

W-AGF	Wireline Access Gateway Function

WUS	Wake Up Signal

-----------------Start of the Next Change-----------------

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Broadcast Session Setup | BROADCAST SESSION SETUP REQUEST | BROADCAST SESSION SETUP RESPONSE | BROADCAST SESSION SETUP FAILURE |
| Broadcast Session Modification | BROADCAST SESSION MODIFICATION REQUEST | BROADCAST SESSION MODIFICATION RESPONSE | BROADCAST SESSION MODIFICATION FAILURE |
| Broadcast Session Release | BROADCAST SESSION RELEASE REQUEST | BROADCAST SESSION RELEASE RESPONSE |  |
| Distribution Setup | DISTRIBUTION SETUP REQUEST | DISTRIBUTION SETUP RESPONSE | DISTRIBUTION SETUP FAILURE |
| Distribution Release | DISTRIBUTION RELEASE REQUEST | DISTRIBUTION RELEASE RESPONSE |  |
| Multicast Session Activation | MULTICAST SESSION ACTIVATION REQUEST | MULTICAST SESSION ACTIVATION RESPONSE | MULTICAST SESSION ACTIVATION FAILURE |
| Multicast Session Deactivation | MULTICAST SESSION DEACTIVATION REQUEST | MULTICAST SESSION DEACTIVATION RESPONSE |  |
| Multicast Session Update | MULTICAST SESSION UPDATE REQUEST | MULTICAST SESSION UPDATE RESPONSE | MULTICAST SESSION UPDATE FAILURE |
| Timing Synchronisation Status | TIMING SYNCHRONISATION STATUS REQUEST | TIMING SYNCHRONISATION STATUS RESPONSE | TIMING SYNCHRONISATION STATUS FAILURE |
| MT Communication Handling | MT COMMUNICATION HANDLING REQUEST | MT COMMUNICATION HANDLING RESPONSE | MT COMMUNICATION HANDLING FAILURE |
| Broadcast Session Transport | BROADCAST SESSION TRANSPORT REQUEST | BROADCAST SESSION TRANSPORT RESPONSE | BROADCAST SESSION TRANSPORT FAILURE |
| Inventory Request | INVENTORY REQUEST | INVENTORY RESPONSE | INVENTORY FAILURE |
| Command Request | COMMAND REQUEST | COMMAND RESPONSE | COMMAND FAILURE |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Session Release Required | BROADCAST SESSION RELEASE REQUIRED |
| Timing Synchronisation Status Report | TIMING SYNCHRONISATION STATUS REPORT |
| RAN Paging Request | RAN PAGING REQUEST |
| Inventory Report | INVENTORY REPORT |

-----------------Start of the Next Change-----------------

## 8.7	Interface Management Procedures

### 8.7.1	NG Setup

#### 8.7.1.1	General

The purpose of the NG Setup procedure is to exchange application level data needed for the NG-RAN node and the AMF to correctly interoperate on the NG-C interface. This procedure shall be the first NGAP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes, replaces it by the one received and clears AMF overload state information at the NG-RAN node. If the NG-RAN node and AMF do not agree on retaining the UE contexts this procedure also re-initialises the NGAP UE-related contexts (if any) and erases all related signalling connections in the two nodes like an NG Reset procedure would do.

If the NG-RAN node supports A-IoT and is communicating directly with the AIOTF, as specified in TS 23.369 [z], the NG Setup procedure, as depicted in Figures 8.7.1.2-x and 8.7.1.3-x and specified in the respective sections, is executed between the NG-RAN node and the AIOTF.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation with the AMF

Figure 8.7.1.2-x: NG setup: successful operation with the AIOTF

If the NG Setup procedure is executed between the NG-RAN node and the AMF:

- The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data.

- If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the NG SETUP REQUEST message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

- If the UE Retention Information IE set to “ues-retained“ is included in the NG SETUP REQUEST message, the AMF may accept the proposal to retain the existing UE related contexts and signalling connections by including the UE Retention Information IE set to “ues-retained“ in the NG SETUP RESPONSE message.

- If the AMF supports IAB, the AMF shall include the IAB Supported IE in the NG SETUP RESPONSE message. If the IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and use it for further AMF selection for the IAB-MT.

- The AMF shall include the Backup AMF Name IE, if available, in the Served GUAMI List IE in the NG SETUP RESPONSE message. The NG-RAN node shall, if supported, consider the AMF as indicated by the Backup AMF Name IE when performing AMF reselection, as specified in TS 23.501 [9].

-	If the GUAMI Type IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall store the received value and use it for further AMF selection as defined in TS 23.501 [9].

-	If the RAN Node Name IE is included in the NG SETUP REQUEST message, the AMF may use this IE as a human readable name of the NG-RAN node. If the Extended RAN Node Name IE is included in the NG SETUP REQUEST message, the AMF may use this IE as a human readable name of the NG-RAN node and shall ignore the RAN Node Name IE if also included.

-	If the Extended AMF Name IE is included in the NG SETUP RESPONSE message, the NG-RAN node may use this IE as a human readable name of the AMF and shall ignore the AMF Name IE.

-	If the NB-IoT Default Paging DRX IE is included in the NG SETUP REQUEST message, the AMF shall take it into account for paging.

-	If the RAT Information IE is included in the NG SETUP REQUEST message, the AMF shall handle this information as specified in TS 23.502 [10].

-	If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

-	If the NID IE within the NPN Support IE is included within a PLMN Support Item IE in the NG SETUP RESPONSE message, the NG-RAN node shall consider that the AMF supports the SNPN identified by the PLMN Identity IE and the NID IE.

-	If the Onboarding Support IE is also included within the same PLMN Support Item IE, the NG-RAN node shall, if supported, consider that the AMF supports UE onboarding for the identified SNPN, as specified in TS 23.501 [9].

-	If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

-	If the AMF supports mobile IAB, the AMF shall include the Mobile IAB Supported IE in the NG SETUP RESPONSE message. If the Mobile IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and further use it for AMF selection for the mobile IAB-MT.

If the NG Setup procedure is executed between the NG-RAN node and the AIOTF:

-	The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AIOTF. The AIOTF responds with an NG SETUP RESPONSE message including the appropriate data.

If the NG Setup procedure is triggered by an NG-RAN node supporting A-IoT:

Editor’s Note:	content of NG Setup related messages for A-IoT is FFS.

#### 8.7.1.3	Unsuccessful Operation

Figure 8.7.1.3-1: NG setup: unsuccessful operation with the AMF

Figure 8.7.1.3-x: NG setup: unsuccessful operation with the AIOTF

If the AMF cannot accept the setup, it should respond with an NG SETUP FAILURE message and appropriate cause value.

If the NG SETUP FAILURE message includes the Time to Wait IE, the NG-RAN node shall wait at least for the indicated time before reinitiating the NG Setup procedure towards the same AMF.

If the NG Setup procedure is executed between the NG-RAN node and the AIOTF, the specification text above concerning the AMF applies for the AIOTF.

#### 8.7.1.4	Abnormal Conditions

If the AMF does not identify any of the PLMNs/SNPNs indicated in the NG SETUP REQUEST message, it shall reject the NG Setup procedure with an appropriate cause value.

If none of the RATs indicated by the NG-RAN node in the NG SETUP REQUEST message is supported by the AMF, then the AMF shall fail the NG Setup procedure with an appropriate cause value.

If the NG Setup procedure is executed between the NG-RAN node and the AIOTF, the specification text above concerning the AMF applies for the AIOTF.

### 8.7.2	RAN Configuration Update

#### 8.7.2.1	General

The purpose of the RAN Configuration Update procedure is to update application level configuration data needed for the NG-RAN node and the AMF to interoperate correctly on the NG-C interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non UE-associated signalling.

If the NG-RAN node supports A-IoT and is communicating directly with an AIOTF, as specified in TS 23.369 [z], the RAN Configuration Update procedure, as depicted in Figures 8.7.2.2-x and 8.7.2.3-x and specified in the respective sections, is executed between the NG-RAN node and the AIOTF.

#### 8.7.2.2	Successful Operation

Figure 8.7.2.2-1: RAN configuration update: successful operation with the AMF

Figure 8.7.2.2-x: RAN configuration update: successful operation with the AIOTF

If the RAN Configuration Update procedure is executed between the NG-RAN node and the AMF:

- The NG-RAN node initiates the procedure by sending a RAN CONFIGURATION UPDATE message to the AMF including an appropriate set of updated configuration data that it has just taken into operational use. The AMF responds with a RAN CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the RAN CONFIGURATION UPDATE message, the AMF shall interpret that the corresponding configuration data is not changed and shall continue to operate the NG-C interface with the existing related configuration data.

- If the Supported TA List IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite the whole list of supported TAs and the corresponding list of supported slices for each TA, and use them for subsequent registration area management of the UE.

- If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the RAN CONFIGURATION UPDATE message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

- If the Global RAN Node ID IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall associate the TNLA to the NG-C interface instance using the Global RAN Node ID.

-	If the RAN CONFIGURATION UPDATE message includes the NG-RAN TNL Association to Remove List IE, the AMF shall, if supported, initiate removal of the TNL association(s) indicated by NG-RAN TNL endpoint(s) and AMF TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is present, or the TNL association(s) indicated by NG-RAN TNL endpoint(s) if the TNL Association Transport Layer Address at AMF IE is absent:

-	if the received TNL Association Transport Layer Address IE includes the Port Number IE, the NG-RAN TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the NG-RAN TNL endpoints correspond to all NG-RAN TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

-	if the received TNL Association Transport Layer Address at AMF IE includes the Port Number IE, the AMF TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the AMF TNL endpoints correspond to all AMF TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

-	If the RAN CONFIGURATION UPDATE message includes the RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node. If the RAN CONFIGURATION UPDATE message includes the Extended RAN Node Name IE, the AMF may store it or update this IE value if already stored, and use it as a human readable name of the NG-RAN node and shall ignore the RAN Node Name IE if also included.

-	If the NB-IoT Default Paging DRX IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite any previously stored NB-IoT default paging DRX value for the NG-RAN node.

-	If the RAT Information IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall handle this information as specified in TS 23.502 [10].

-	If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

-	If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the RAN Configuration Update procedure is executed between the NG-RAN node and the AIOTF:

-	The NG-RAN node initiates the procedure by sending a RAN CONFIGURATION UPDATE message to the AIOTF including an appropriate set of updated configuration data that it has just taken into operational use. The AIOTF responds with a RAN CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the RAN CONFIGURATION UPDATE message, the AIOTF shall interpret that the corresponding configuration data is not changed and shall continue to operate the NG-C interface with the existing related configuration data.

If the RAN Configuration Update procedure is triggered by an NG-RAN node supporting A-IoT:

Editor’s Note:	content of RAN Configuration Update related messages for A-IoT FFS.

#### 8.7.2.3	Unsuccessful Operation

Figure 8.7.2.3-1: RAN configuration update: unsuccessful operation with the AMF

Figure 8.7.2.3-x: RAN configuration update: unsuccessful operation with the AIOTF

If the AMF cannot accept the update, it shall respond with a RAN CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the Time to Wait IE is included in the RAN CONFIGURATION UPDATE FAILURE message, the NG-RAN node shall wait at least for the indicated time before reinitiating the RAN Configuration Update procedure towards the same AMF.

If the RAN Configuration Update procedure is executed between the NG-RAN node and the AIOTF, the specification text above concerning the AMF applies for the AIOTF.

#### 8.7.2.4	Abnormal Conditions

If the NG-RAN node, after initiating the RAN Configuration Update procedure, receives neither a RAN CONFIGURATION UPDATE ACKOWLEDGE nor a RAN CONFIGURATION UPDATE FAILURE message, the NG-RAN node may reinitiate a further RAN Configuration Update procedure towards the same AMF, provided that the content of the new RAN CONFIGURATION UPDATE message is identical to the content of the previously unacknowledged RAN CONFIGURATION UPDATE message.

If the RAN Configuration Update procedure is executed between the NG-RAN node and the AIOTF, the specification text above concerning the AMF applies for the AIOTF.

-----------------Start of the Next Change-----------------

### 8.7.4	NG Reset

#### 8.7.4.1	General

The purpose of the NG Reset procedure is to initialise or re-initialise the RAN, or part of RAN NGAP UE-related contexts, in the event of a failure in the 5GC or vice versa. This procedure does not affect the application level configuration data exchanged during, e.g., the NG Setup procedure. The procedure uses non-UE associated signalling.

If the NG-RAN node supports A-IoT and is communicating directly with an AIOTF, as specified in TS 23.369 [z], the NG Reset procedure, as depicted in Figures 8.7.4.2.1-x and 8.7.4.2.2-x and specified in the respective sections, is executed between the NG-RAN node and the AIOTF.

#### 8.7.4.2	Successful Operation

##### 8.7.4.2.1	NG Reset initiated by the AMF

Figure 8.7.4.2.1-1: NG reset initiated by the AMF: successful operation

Figure 8.7.4.2.1-x: NG reset initiated by the AIOTF: successful operation

If the NG Reset procedure is executed between the NG-RAN node and the AMF:

- In the event of a failure at the AMF which has resulted in the loss of some or all transaction reference information, an NG RESET message shall be sent to the NG-RAN node.

If the NG Reset procedure is executed between the NG-RAN node and the AIOTF:

-	In the event of a failure at the AIOTF which has resulted in the loss of some or all transaction reference information, an NG RESET message shall be sent to the NG-RAN node.

At reception of the NG RESET message the NG-RAN node shall release all allocated resources on NG and Uu related to the UE association(s) indicated explicitly or implicitly in the NG RESET message and remove the indicated UE contexts including NGAP ID.

After the NG-RAN node has released all assigned NG resources and the UE NGAP IDs for all indicated UE associations which can be used for new UE-associated logical NG-connections over the NG interface, the NG-RAN node shall respond with the NG RESET ACKNOWLEDGE message. The NG-RAN node does not need to wait for the release of radio resources to be completed before returning the NG RESET ACKNOWLEDGE message.

If the NG RESET message contains the UE-associated Logical NG-connection List IE, then:

-	The NG-RAN node shall use the AMF UE NGAP ID IE and/or the RAN UE NGAP ID IE to explicitly identify the UE association(s) to be reset.

-	The NG-RAN node shall include in the NG RESET ACKNOWLEDGE message, for each UE association to be reset, the UE-associated Logical NG-connection Item IE in the UE-associated Logical NG-connection List IE. The UE-associated Logical NG-connection Item IEs shall be in the same order as received in the NG RESET message and shall include also unknown UE-associated logical NG-connections. Empty UE-associated Logical NG-connection Item IEs, received in the NG RESET message, may be omitted in the NG RESET ACKNOWLEDGE message.

-	If the AMF UE NGAP ID IE is included in the UE-associated Logical NG-connection Item IE for a UE association, the NG-RAN node shall include the AMF UE NGAP ID IE in the corresponding UE-associated Logical NG-connection Item IE in the NG RESET ACKNOWLEDGE message.

-	If the RAN UE NGAP ID IE is included in the UE-associated Logical NG-connection Item IE for a UE association, the NG-RAN node shall include the RAN UE NGAP ID IE in the corresponding UE-associated Logical NG-connection Item IE in the NG RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the NG RESET message is received, any other ongoing procedure (except for another NG Reset procedure) on the same NG interface related to a UE association, indicated explicitly or implicitly in the NG RESET message, shall be aborted.

##### 8.7.4.2.2	NG Reset initiated by the NG-RAN node

Figure 8.7.4.2.2-1: NG reset initiated by the NG-RAN node: successful operation with the AMF

Figure 8.7.4.2.2-x: NG reset initiated by the NG-RAN node: successful operation with the AIOTF

If the NG Reset procedure is executed between the NG-RAN node and the AMF:

- In the event of a failure at the NG-RAN node which has resulted in the loss of some or all transaction reference information, an NG RESET message shall be sent to the AMF.

- At reception of the NG RESET message the AMF shall release all allocated resources on NG related to the UE association(s) indicated explicitly or implicitly in the NG RESET message and remove the NGAP ID for the indicated UE associations.

If the NG Reset procedure is executed between the NG-RAN node and the AIOTF:

-	At reception of the NG RESET message the AIOTF shall release all allocated NG resources.

After the AMF has released all assigned NG resources and the UE NGAP IDs for all indicated UE associations which can be used for new UE-associated logical NG-connections over the NG interface, the AMF shall respond with the NG RESET ACKNOWLEDGE message.

If the NG RESET message contains the UE-associated Logical NG-connection List IE, then:

-	The AMF shall use the AMF UE NGAP ID IE and/or the RAN UE NGAP ID IE to explicitly identify the UE association(s) to be reset.

-	The AMF shall include in the NG RESET ACKNOWLEDGE message, for each UE association to be reset, the UE-associated Logical NG-connection Item IE in the UE-associated Logical NG-connection List IE. The UE-associated Logical NG-connection Item IEs shall be in the same order as received in the NG RESET message and shall include also unknown UE-associated logical NG-connections. Empty UE-associated Logical NG-connection Item IEs, received in the NG RESET message, may be omitted in the NG RESET ACKNOWLEDGE message.

-	If the AMF UE NGAP ID IE is included in the UE-associated Logical NG-connection Item IE for a UE association, the AMF shall include the AMF UE NGAP ID IE in the corresponding UE-associated Logical NG-connection Item IE in the NG RESET ACKNOWLEDGE message.

-	If the RAN UE NGAP ID IE is included in a UE-associated Logical NG-connection Item IE for a UE association, the AMF shall include the RAN UE NGAP ID IE in the corresponding UE-associated Logical NG-connection Item IE in the NG RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the NG RESET message is received, any other ongoing procedure (except for another NG Reset procedure) on the same NG interface related to a UE association, indicated explicitly or implicitly in the NG RESET message, shall be aborted.

#### 8.7.4.3	Unsuccessful Operation

Not applicable.

#### 8.7.4.4	Abnormal Conditions

##### 8.7.4.4.1	Abnormal Condition at the 5GC

If the NG RESET message includes the UE-associated Logical NG-connection List IE, but neither the AMF UE NGAP ID IE nor the RAN UE NGAP ID IE is present for a UE-associated Logical NG-connection Item IE, then the AMF shall ignore the UE-associated Logical NG-connection Item IE. The AMF may return the empty UE-associated Logical NG-connection Item IE in the UE-associated Logical NG-connection List IE in the NG RESET ACKNOWLEDGE message.

##### 8.7.4.4.2	Abnormal Condition at the NG-RAN

If the NG RESET message includes the UE-associated Logical NG-connection List IE, but neither the AMF UE NGAP ID IE nor the RAN UE NGAP ID IE is present for a UE-associated Logical NG-connection Item IE, then the NG-RAN node shall ignore the UE-associated Logical NG-connection Item IE. The NG-RAN node may return the empty UE-associated Logical NG-connection Item IE in the UE-associated Logical NG-connection List IE in the NG RESET ACKNOWLEDGE message.

##### 8.7.4.4.3	Crossing of NG RESET Messages

If an NG Reset procedure is ongoing in the NG-RAN node and the NG-RAN node receives an NG RESET message from the peer entity on the same NG interface related to one or several UE associations previously requested to be reset, indicated explicitly or implicitly in the received NG RESET message, the NG-RAN node shall respond with the NG RESET ACKNOWLEDGE message as described in 8.7.4.2.1.

If an NG Reset procedure is ongoing in the AMF and the AMF receives an NG RESET message from the peer entity on the same NG interface related to one or several UE associations previously requested to be reset, indicated explicitly or implicitly in the received NG RESET message, the AMF shall respond with the NG RESET ACKNOWLEDGE message as described in 8.7.4.2.2.

If the NG-RAN node supports A-IoT and is communicating directly with an AIOTF and if an NG Reset procedure is ongoing in the AIOTF and the AIOTF receives an NG RESET message from the NG-RAN node on the same NG interface, the AIOTF shall respond with the NG RESET ACKNOWLEDGE message as described in 8.7.4.2.2.

### 8.7.5	Error Indication

#### 8.7.5.1	General

The Error Indication procedure is initiated by a node in order to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

If the error situation arises due to reception of a message utilising UE-associated signalling, then the Error Indication procedure uses UE-associated signalling. Otherwise the procedure uses non-UE associated signalling.

#### 8.7.5.2	Successful Operation

Figure 8.7.5.2-1: Error indication initiated by the AMF

Figure 8.7.5.2-2: Error indication initiated by the NG-RAN node to the AMF

Figure 8.7.5.2-x: Error indication initiated by the AIOTF

Figure 8.7.5.2-y: Error indication initiated by the NG-RAN node to the AIOTF

When the conditions defined in clause 10 are fulfilled, the Error Indication procedure is initiated by an ERROR INDICATION message sent from the receiving node.

The ERROR INDICATION message shall contain at least either the Cause IE or the Criticality Diagnostics IE. In case the Error Indication procedure is triggered by utilising UE-associated signalling the AMF UE NGAP ID IE and the RAN UE NGAP ID IE shall be included in the ERROR INDICATION message. If one or both of the AMF UE NGAP ID IE and the RAN UE NGAP ID IE are not correct, the cause shall be set to an appropriate value, e.g., "Unknown local UE NGAP ID" or "Inconsistent remote UE NGAP ID".

#### 8.7.5.3	Abnormal Conditions

Void.

-----------------Start of the Next Change-----------------

### 8.xx.1	Inventory Request

#### 8.xx.1.1	General

The purpose of the Inventory Request procedure is to request the NG-RAN node to perform inventory over the A-IoT radio. This procedure applies only if the NG-RAN node is a gNB.

#### 8.xx.1.2	Successful Operation

Figure 8.xx.1.2-1: Inventory Request, successful operation.

The A-IoT CN node triggers the procedure by sending an INVENTORY REQUEST message to the NG-RAN node. If the NG-RAN node accepts the request, the NG-RAN node responds with the INVENTORY RESPONSE message.

The NG-RAN node shall, include the same values of the AIOTF Identifier IE and A-IoT Correlation Identifier IE in the INVENTORY RESPONSE message as received from the INVENTORY REQUEST message.

If the Inventory Assistance Information IE is included in the INVENTORY REQUEST message, the NG-RAN node may take the information into account as defined in TS 38.300 [8].

Interaction with Inventory Report procedure

When starting the requested inventory, the NG-RAN node shall, include the same values of the AIOTF Identifier IE and A-IoT Correlation Identifier IE in the INVENTORY REPORT message(s) as received from the INVENTORY REQUEST message

If the Follow on Command Indication IE set to "true" is included in the Inventory Request Transfer IE in the INVENTORY REQUEST message, the NG-RAN node shall consider that there will be command to be transmitted to the device(s), and shall allocate and include a RAN A-IoT Device NGAP ID IE in each Device Report Item IE within the Inventory Report Transfer IE in the INVENTORY REPORT message(s).

#### 8.xx.1.3	Unsuccessful Operation

Figure 8.xx.1.3-1: Inventory Request, unsuccessful operation.

If the NG-RAN node is not able to perform the inventory operation, it shall send the INVENTORY FAILURE messagewith an appropriate cause value.

#### 8.xx.1.4		Abnormal Conditions

Void.

### 8.xx.2	Inventory Report

#### 8.xx.2.1	General

The purpose of the Inventory Report procedure is to enable the NG-RAN node to provide Inventory Report to the A-IoT CN node following a successful Inventory Request procedure. This procedure applies only if the NG-RAN node is a gNB.

#### 8.xx.2.2	Successful Operation

Figure 8.xx.2.2-1: Inventory Report

The NG-RAN node initiates the procedure by sending an INVENTORY REPORT message for the Inventory service.

#### 8.xx.2.3	Abnormal Conditions

Void.

### 8.xx.3	Command Request

#### 8.xx.3.1	General

The purpose of the Command Request procedure is to request the NG-RAN node to transfer the command to the A-IoT device over the A-IoT radio. This procedure applies only if the NG-RAN node is a gNB.

#### 8.xx.3.2	Successful Operation

Figure 8.xx.3.2-1: Command Request, successful operation.

The A-IoT CN node triggers the procedure by sending a COMMAND REQUEST message to the NG-RAN node. In case of successful operation, the NG-RAN node responds with the COMMAND RESPONSE message.

If the Command Assistance Information IE is included in the COMMAND REQUEST message, the NG-RAN node, the NG-RAN node may take the information into account as defined in TS 38.300 [8].

#### 8.xx.3.3	Unsuccessful Operation

Figure 8.xx.3.3-1: Command Request, unsuccessful operation.

If the NG-RAN node is not able to transmit the received command, it shall send the COMMAND FAILURE message with an appropriate cause value.

#### 8.xx.3.4		Abnormal Conditions

Void.

-----------------Start of the Next Change-----------------

### 9.2.x		A-IoT Messages

Editor’s Note: The A-IoT Correlation Identifier outside of the Transfer IEs in the following A-IoT messages are included based on RAN3 working assumption.

#### 9.2.x.1	INVENTORY REQUEST

This message is sent by the A-IoT CN node to trigger the NG-RAN node to perform inventory over the A-IoT radio.

Direction: A-IoT CN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Inventory Request Transfer | M |  | OCTET STRING | Containing the Inventory Request Transfer IE specified in subclause 9.3.x.1 | YES | reject |

#### 9.2.x.2	INVENTORY RESPONSE

This message is sent by the NG-RAN node to report the successful outcome of the request from the INVENTORY REQUEST message.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Inventory Response Transfer | M |  | OCTET STRING | Containing the Inventory Response Transfer IE specified in subclause 9.3.x.2 | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.3	INVENTORY FAILURE

This message is sent by the NG-RAN node to report the unsuccessful outcome of the request from the INVENTORY REQUEST message.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Inventory Failure Transfer | M |  | OCTET STRING | Containing the Inventory Failure Transfer IE specified in subclause 9.3.x.3 | YES | ignore |
| Cause | M[FFS] |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.4	INVENTORY REPORT

This message is sent by the NG-RAN node to provide the inventory report to the A-IoT CN node.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Inventory Report Transfer | M |  | OCTET STRING | Containing the Inventory Report Transfer IE specified in subclause 9.3.x.4 | YES | reject |

#### 9.2.x.5	COMMAND REQUEST

This message is sent by the A-IoT CN node to the gNB to trigger the command transmission over the A-IoT radio.

Direction: A-IoT CN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Command Request Transfer | M |  | OCTET STRING | Containing the Command Request Transfer IE specified in subclause 9.3.6.5. | YES | reject |

Editor’s note: FFS whether the Command Request also contains outside of the Command Request Transfer container, the RAN A-IoT device NGAP ID (FFS for CN NGAP device ID) which was received in the Inventory Report message or some additional transaction ID.

#### 9.2.x.6	COMMAND RESPONSE

This message is sent by the NG-RAN node to provide the command response to the A-IoT CN node.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Command Response Transfer | M |  | OCTET STRING | Containing the Command Response Transfer IE specified in subclause 9.3.6.5. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

Editor’s note: FFS whether the Command Response message also contains outside of the Command Request Transfer container, the RAN NGAP device ID (FFS for CN NGAP device ID) which was received in the Inventory Report message or some additional transaction ID.

#### 9.2.x.7	COMMAND FAILURE

This message is sent by the NG-RAN node to report the unsuccessful transmission of the command to the A-IoT CN node.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AIOTF Identifier | M |  | 9.3.3.xx2 |  | YES | reject |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| Command Failure Transfer | M |  | OCTET STRING | Containing the Command Failure Transfer IE specified in subclause 9.3.6.5. | YES | ignore |
| Cause | M[FFS] |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

-----------------Start of the Next Change-----------------

#### 9.3.1.aa1	Reader Index

This IE represents identifier of a Reader within a gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reader Index | M |  | INTEGER (1..65536, …) |  |

-----------------Start of the Next Change-----------------

#### 9.3.3.xx1	A-IoT Correlation Identifier

This IE uniquely identifies an A-IoT session within an AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | FFS | . |

#### 9.3.3.xx2	AIOTF Identifier

This IE uniquely identifies the AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AIOTF Identifier | M |  | FFS | . |

#### 9.3.3.xx3	Requested Service Area Information

This IE includes the Requested Service Area Information for the A-IoT inventory.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Requested Reader List |  | 0..1 |  |  |
| >Requested Reader Item |  | 1..<maxnoofReaders> |  |  |
| >>Global gNB ID | M |  | 9.3.1.6 |  |
| >>Reader Index | M |  | 9.3.1.aa1 |  |
| Requested A-IoT Area List |  | 0..1 |  |  |
| >Requested A-IoT Area Item |  | 1..<maxnoofAIoTAreas> |  |  |
| >>A-IoT Area | M |  | FFS |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers. Value is 65535. |
| maxnoofAIoTAreas | Maximum no. of A-IoT Areas. Value is 256. |

#### 9.3.3.xx4	Inventory Assistance Information

This IE includes the assistance information for the A-IoT Inventory Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Approximate Number of Target A-IoT Devices | O |  | INTEGER (1..2048, …) |  |

#### 9.3.3.xx5	A-IoT Device Identification Requested

This IE represents the Requested Identity of the A-IoT device(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| FFS |  |  |  |  |

Editor’s Note: this IE enables identification of a single A-IoT device, a group of A-IoT devices, all A-IoT devices, detailed coding FFS.

#### 9.3.3.yy1	RAN A-IoT Device NGAP ID

This IE uniquely identifies the A-IoT Device over the NG interface within the NG-RAN node within a A-IoT Session, as described in TS 38.401 [2].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAN A-IoT Device NGAP ID | M |  | INTEGER (0..232 -1) |  |

#### 9.3.3.yy3	Command Assistance Information

This IE includes the assistance information for the A-IoT Command Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Estimate of Expected D2R Message Size | O |  | FFS |  |

-----------------Start of the Next Change-----------------

### 9.3.x	AIOTF Related IEs

#### 9.3.x.1	Inventory Request Transfer

This IE provides the inventory request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| A-IoT Device Identification Requested | M |  | 9.3.3.xx5 |  | YES | reject |
| Requested Service Area Information | M |  | 9.3.3.xx3 |  | YES | reject |
| Inventory Assistance Information | O |  | 9.3.3.xx4 |  | YES | reject |
| Follow on Command Indication | O |  | ENUMERATED (true, ...) | This IE indicates that there is command(s) to be transmitted. | YES | reject |

#### 9.3.x.2	Inventory Response Transfer

This IE provides the inventory response related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

#### 9.3.x.3	Inventory Failure Transfer

This IE provides the inventory failure related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

#### 9.3.x.4	Inventory Report Transfer

This IE provides the inventory report related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  |
| Global gNB ID | M |  | 9.3.1.6 |  |
| Reader Report List |  | 1 |  |  |
| >Reader Report Item |  | 1..< maxnoofReaders> |  |  |
| >>Reader Index | M |  | 9.3.1.aa1 |  |
| >>Device Report List |  | 1 |  |  |
| >>>Device Report Item |  | 1..<maxnoofDevices> |  |  |
| >>>>A-IoT NAS PDU | M |  | OCTET STRING |  |
| >>>>RAN A-IoT Device NGAP ID | O |  | 9.3.3.yy1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers served by a gNB. Value is 65535. |
| maxnoofDevices | Maximum no. of Devices within a reader. Value is 65535. |

Editor’s Note 1: The A-IoT Correlation Identifier inside of the Transfer IEs in the following Command related messages are included based on RAN3 working assumptions.

Editor’s Note 2: The inclusion of the CN A-IoT Device NGAP ID IE inside of the Transfer IEs in the following Command related messages are FFS.

#### 9.3.x.5	Command Request Transfer

This IE provides the command request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| RAN A-IoT Device NGAP ID | M |  | 9.3.3.yy1 |  | YES | reject |
| A-IoT NAS PDU | M |  | OCTET STRING |  | YES | reject |
| Command Assistance Information | O |  | 9.3.3.yy3 |  | YES | reject |

#### 9.3.x.6	Command Response Transfer

This IE provides the command response related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  |
| RAN A-IoT Device NGAP ID | M |  | 9.3.3.yy1 |  |
| A-IoT NAS PDU | M |  | OCTET STRING |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

#### 9.3.x.7	Command Failure Transfer

This IE provides the command failure related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  |
| RAN A-IoT Device NGAP ID | M |  | 9.3.3.yy1 |  |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

-----------------Start of the Next Change-----------------

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

NGAP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM NGAP-CommonDataTypes

AMFConfigurationUpdate,

AMFConfigurationUpdateAcknowledge,

AMFConfigurationUpdateFailure,

AMFCPRelocationIndication,

AMFStatusIndication,

BroadcastSessionModificationFailure,

BroadcastSessionModificationRequest,

BroadcastSessionModificationResponse,

BroadcastSessionReleaseRequest,

BroadcastSessionReleaseRequired,

BroadcastSessionReleaseResponse,

BroadcastSessionSetupFailure,

BroadcastSessionSetupRequest,

BroadcastSessionSetupResponse,

BroadcastSessionTransportFailure,

BroadcastSessionTransportRequest,

BroadcastSessionTransportResponse,

CellTrafficTrace,

ConnectionEstablishmentIndication,

DeactivateTrace,

DistributionReleaseRequest,

DistributionReleaseResponse,

DistributionSetupFailure,

DistributionSetupRequest,

DistributionSetupResponse,

DownlinkNASTransport,

DownlinkNonUEAssociatedNRPPaTransport,

DownlinkRANConfigurationTransfer,

DownlinkRANEarlyStatusTransfer,

DownlinkRANStatusTransfer,

DownlinkUEAssociatedNRPPaTransport,

ErrorIndication,

HandoverCancel,

HandoverCancelAcknowledge,

HandoverCommand,

HandoverFailure,

HandoverNotify,

HandoverPreparationFailure,

HandoverRequest,

HandoverRequestAcknowledge,

HandoverRequired,

HandoverSuccess,

InitialContextSetupFailure,

InitialContextSetupRequest,

InitialContextSetupResponse,

InitialUEMessage,

LocationReport,

LocationReportingControl,

LocationReportingFailureIndication,

MTCommunicationHandlingRequest,

MTCommunicationHandlingResponse,

MTCommunicationHandlingFailure,

MulticastSessionActivationFailure,

MulticastSessionActivationRequest,

MulticastSessionActivationResponse,

MulticastSessionDeactivationRequest,

MulticastSessionDeactivationResponse,

MulticastSessionUpdateFailure,

MulticastSessionUpdateRequest,

MulticastSessionUpdateResponse,

MulticastGroupPaging,

NASNonDeliveryIndication,

NGReset,

NGResetAcknowledge,

NGSetupFailure,

NGSetupRequest,

NGSetupResponse,

OverloadStart,

OverloadStop,

Paging,

PathSwitchRequest,

PathSwitchRequestAcknowledge,

PathSwitchRequestFailure,

PDUSessionResourceModifyConfirm,

PDUSessionResourceModifyIndication,

PDUSessionResourceModifyRequest,

PDUSessionResourceModifyResponse,

PDUSessionResourceNotify,

PDUSessionResourceReleaseCommand,

PDUSessionResourceReleaseResponse,

PDUSessionResourceSetupRequest,

PDUSessionResourceSetupResponse,

PrivateMessage,

PWSCancelRequest,

PWSCancelResponse,

PWSFailureIndication,

PWSRestartIndication,

RANConfigurationUpdate,

RANConfigurationUpdateAcknowledge,

RANConfigurationUpdateFailure,

RANCPRelocationIndication,

RANPagingRequest,

RerouteNASRequest,

RetrieveUEInformation,

RRCInactiveTransitionReport,

SecondaryRATDataUsageReport,

TimingSynchronisationStatusRequest,

TimingSynchronisationStatusResponse,

TimingSynchronisationStatusFailure,

TimingSynchronisationStatusReport,

TraceFailureIndication,

TraceStart,

UEContextModificationFailure,

UEContextModificationRequest,

UEContextModificationResponse,

UEContextReleaseCommand,

UEContextReleaseComplete,

UEContextReleaseRequest,

UEContextResumeRequest,

UEContextResumeResponse,

UEContextResumeFailure,

UEContextSuspendRequest,

UEContextSuspendResponse,

UEContextSuspendFailure,

UEInformationTransfer,

UERadioCapabilityCheckRequest,

UERadioCapabilityCheckResponse,

UERadioCapabilityIDMappingRequest,

UERadioCapabilityIDMappingResponse,

UERadioCapabilityInfoIndication,

UETNLABindingReleaseRequest,

UplinkNASTransport,

UplinkNonUEAssociatedNRPPaTransport,

UplinkRANConfigurationTransfer,

UplinkRANEarlyStatusTransfer,

UplinkRANStatusTransfer,

UplinkUEAssociatedNRPPaTransport,

WriteReplaceWarningRequest,

WriteReplaceWarningResponse,

UplinkRIMInformationTransfer,

DownlinkRIMInformationTransfer,

InventoryRequest,

InventoryResponse,

InventoryFailure,

InventoryReport,

CommandRequest,

CommandResponse,

CommandFailure

FROM NGAP-PDU-Contents

id-AMFConfigurationUpdate,

id-AMFCPRelocationIndication,

id-AMFStatusIndication,

id-BroadcastSessionModification,

id-BroadcastSessionRelease,

id-BroadcastSessionReleaseRequired,

id-BroadcastSessionSetup,

id-BroadcastSessionTransport,

id-CellTrafficTrace,

id-ConnectionEstablishmentIndication,

id-DeactivateTrace,

id-DistributionRelease,

id-DistributionSetup,

id-DownlinkNASTransport,

id-DownlinkNonUEAssociatedNRPPaTransport,

id-DownlinkRANConfigurationTransfer,

id-DownlinkRANEarlyStatusTransfer,

id-DownlinkRANStatusTransfer,

id-DownlinkRIMInformationTransfer,

id-DownlinkUEAssociatedNRPPaTransport,

id-ErrorIndication,

id-HandoverCancel,

id-HandoverNotification,

id-HandoverPreparation,

id-HandoverResourceAllocation,

id-HandoverSuccess,

id-InitialContextSetup,

id-InitialUEMessage,

id-LocationReport,

id-LocationReportingControl,

id-LocationReportingFailureIndication,

id-MTCommunicationHandling,

id-MulticastGroupPaging,

id-MulticastSessionActivation,

id-MulticastSessionDeactivation,

id-MulticastSessionUpdate,

id-NASNonDeliveryIndication,

id-NGReset,

id-NGSetup,

id-OverloadStart,

id-OverloadStop,

id-Paging,

id-PathSwitchRequest,

id-PDUSessionResourceModify,

id-PDUSessionResourceModifyIndication,

id-PDUSessionResourceNotify,

id-PDUSessionResourceRelease,

id-PDUSessionResourceSetup,

id-PrivateMessage,

id-PWSCancel,

id-PWSFailureIndication,

id-PWSRestartIndication,

id-RANConfigurationUpdate,

id-RANCPRelocationIndication,

id-RANPagingRequest,

id-RerouteNASRequest,

id-RetrieveUEInformation,

id-RRCInactiveTransitionReport,

id-SecondaryRATDataUsageReport,

id-TimingSynchronisationStatus,

id-TimingSynchronisationStatusReport,

id-TraceFailureIndication,

id-TraceStart,

id-UEContextModification,

id-UEContextRelease,

id-UEContextReleaseRequest,

id-UEContextResume,

id-UEContextSuspend,

id-UEInformationTransfer,

id-UERadioCapabilityCheck,

id-UERadioCapabilityIDMapping,

id-UERadioCapabilityInfoIndication,

id-UETNLABindingRelease,

id-UplinkNASTransport,

id-UplinkNonUEAssociatedNRPPaTransport,

id-UplinkRANConfigurationTransfer,

id-UplinkRANEarlyStatusTransfer,

id-UplinkRANStatusTransfer,

id-UplinkRIMInformationTransfer,

id-UplinkUEAssociatedNRPPaTransport,

id-WriteReplaceWarning,

id-InventoryRequest,

id-InventoryReport,

id-CommandRequest

FROM NGAP-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

NGAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode	UNIQUE,

&criticality				Criticality	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

#### -- Interface PDU Definition

--

-- **************************************************************

NGAP-PDU ::= CHOICE {

initiatingMessage			InitiatingMessage,

successfulOutcome			SuccessfulOutcome,

unsuccessfulOutcome			UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	NGAP-ELEMENTARY-PROCEDURE.&procedureCode		({NGAP-ELEMENTARY-PROCEDURES}),

criticality		NGAP-ELEMENTARY-PROCEDURE.&criticality			({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			NGAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	NGAP-ELEMENTARY-PROCEDURE.&procedureCode		({NGAP-ELEMENTARY-PROCEDURES}),

criticality		NGAP-ELEMENTARY-PROCEDURE.&criticality			({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			NGAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	NGAP-ELEMENTARY-PROCEDURE.&procedureCode		({NGAP-ELEMENTARY-PROCEDURES}),

criticality		NGAP-ELEMENTARY-PROCEDURE.&criticality			({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			NGAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

NGAP-ELEMENTARY-PROCEDURES NGAP-ELEMENTARY-PROCEDURE ::= {

NGAP-ELEMENTARY-PROCEDURES-CLASS-1			|

NGAP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

NGAP-ELEMENTARY-PROCEDURES-CLASS-1 NGAP-ELEMENTARY-PROCEDURE ::= {

aMFConfigurationUpdate			|

broadcastSessionModification	|

broadcastSessionRelease			|

broadcastSessionSetup			|

broadcastSessionTransport					|

distributionSetup				|

distributionRelease				|

handoverCancel					|

handoverPreparation				|

handoverResourceAllocation		|

initialContextSetup				|

mTCommunicationHandling			|

multicastSessionActivation					|

multicastSessionDeactivation				|

multicastSessionUpdate			|

nGReset							|

nGSetup							|

pathSwitchRequest				|

pDUSessionResourceModify		|

pDUSessionResourceModifyIndication			|

pDUSessionResourceRelease		|

pDUSessionResourceSetup			|

pWSCancel						|

rANConfigurationUpdate			|

timingSynchronisationStatus		|

uEContextModification			|

uEContextRelease				|

uEContextResume								|

uEContextSuspend							|

uERadioCapabilityCheck			|

uERadioCapabilityIDMapping		|

writeReplaceWarning							|

inventoryRequest							|

commandRequest								,

...

}

NGAP-ELEMENTARY-PROCEDURES-CLASS-2 NGAP-ELEMENTARY-PROCEDURE ::= {

aMFCPRelocationIndication		|

aMFStatusIndication				|

broadcastSessionReleaseRequired	|

cellTrafficTrace				|

connectionEstablishmentIndication	|

deactivateTrace					|

downlinkNASTransport						|

downlinkNonUEAssociatedNRPPaTransport		|

downlinkRANConfigurationTransfer	|

downlinkRANEarlyStatusTransfer	|

downlinkRANStatusTransfer		|

downlinkRIMInformationTransfer				|

downlinkUEAssociatedNRPPaTransport			|

errorIndication					|

handoverNotification			|

handoverSuccess								|

initialUEMessage							|

locationReport					|

locationReportingControl		|

locationReportingFailureIndication	|

multicastGroupPaging			|

nASNonDeliveryIndication		|

overloadStart					|

overloadStop					|

paging										|

pDUSessionResourceNotify		|

privateMessage					|

pWSFailureIndication						|

pWSRestartIndication			|

rANCPRelocationIndication		|

rANPagingRequest				|

rerouteNASRequest				|

retrieveUEInformation						|

rRCInactiveTransitionReport					|

secondaryRATDataUsageReport		|

timingSynchronisationStatusReport	|

traceFailureIndication			|

traceStart						|

uEContextReleaseRequest			|

uEInformationTransfer						|

uERadioCapabilityInfoIndication	|

uETNLABindingRelease			|

uplinkNASTransport				|

uplinkNonUEAssociatedNRPPaTransport	|

uplinkRANConfigurationTransfer	|

uplinkRANEarlyStatusTransfer				|

uplinkRANStatusTransfer			|

uplinkRIMInformationTransfer				|

uplinkUEAssociatedNRPPaTransport			|

inventoryReport,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

aMFConfigurationUpdate NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AMFConfigurationUpdate

SUCCESSFUL OUTCOME		AMFConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	AMFConfigurationUpdateFailure

PROCEDURE CODE			id-AMFConfigurationUpdate

CRITICALITY				reject

}

aMFCPRelocationIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AMFCPRelocationIndication

PROCEDURE CODE			id-AMFCPRelocationIndication

CRITICALITY				reject

}

aMFStatusIndication NGAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		AMFStatusIndication

PROCEDURE CODE			id-AMFStatusIndication

CRITICALITY				ignore

}

broadcastSessionModification NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionModificationRequest

SUCCESSFUL OUTCOME		BroadcastSessionModificationResponse

UNSUCCESSFUL OUTCOME	BroadcastSessionModificationFailure

PROCEDURE CODE			id-BroadcastSessionModification

CRITICALITY				reject

}

broadcastSessionRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionReleaseRequest

SUCCESSFUL OUTCOME		BroadcastSessionReleaseResponse

PROCEDURE CODE			id-BroadcastSessionRelease

CRITICALITY				reject

}

broadcastSessionReleaseRequired NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionReleaseRequired

PROCEDURE CODE			id-BroadcastSessionReleaseRequired

CRITICALITY				reject

}

broadcastSessionSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionSetupRequest

SUCCESSFUL OUTCOME		BroadcastSessionSetupResponse

UNSUCCESSFUL OUTCOME	BroadcastSessionSetupFailure

PROCEDURE CODE			id-BroadcastSessionSetup

CRITICALITY				reject

}

broadcastSessionTransport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionTransportRequest

SUCCESSFUL OUTCOME		BroadcastSessionTransportResponse

UNSUCCESSFUL OUTCOME	BroadcastSessionTransportFailure

PROCEDURE CODE			id-BroadcastSessionTransport

CRITICALITY				reject

}

cellTrafficTrace NGAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		CellTrafficTrace

PROCEDURE CODE			id-CellTrafficTrace

CRITICALITY				ignore

}

connectionEstablishmentIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ConnectionEstablishmentIndication

PROCEDURE CODE			id-ConnectionEstablishmentIndication

CRITICALITY				reject

}

deactivateTrace NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-DeactivateTrace

CRITICALITY				ignore

}

distributionSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DistributionSetupRequest

SUCCESSFUL OUTCOME		DistributionSetupResponse

UNSUCCESSFUL OUTCOME	DistributionSetupFailure

PROCEDURE CODE			id-DistributionSetup

CRITICALITY				reject

}

distributionRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DistributionReleaseRequest

SUCCESSFUL OUTCOME		DistributionReleaseResponse

PROCEDURE CODE			id-DistributionRelease

CRITICALITY				reject

}

downlinkNASTransport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkNASTransport

PROCEDURE CODE			id-DownlinkNASTransport

CRITICALITY				ignore

}

downlinkNonUEAssociatedNRPPaTransport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkNonUEAssociatedNRPPaTransport

PROCEDURE CODE			id-DownlinkNonUEAssociatedNRPPaTransport

CRITICALITY				ignore

}

downlinkRANConfigurationTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkRANConfigurationTransfer

PROCEDURE CODE			id-DownlinkRANConfigurationTransfer

CRITICALITY				ignore

}

downlinkRANEarlyStatusTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkRANEarlyStatusTransfer

PROCEDURE CODE			id-DownlinkRANEarlyStatusTransfer

CRITICALITY				ignore

}

downlinkRANStatusTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkRANStatusTransfer

PROCEDURE CODE			id-DownlinkRANStatusTransfer

CRITICALITY				ignore

}

downlinkUEAssociatedNRPPaTransport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkUEAssociatedNRPPaTransport

PROCEDURE CODE			id-DownlinkUEAssociatedNRPPaTransport

CRITICALITY				ignore

}

errorIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-ErrorIndication

CRITICALITY				ignore

}

handoverCancel NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverCancel

SUCCESSFUL OUTCOME		HandoverCancelAcknowledge

PROCEDURE CODE			id-HandoverCancel

CRITICALITY				reject

}

handoverNotification NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverNotify

PROCEDURE CODE			id-HandoverNotification

CRITICALITY				ignore

}

handoverPreparation NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequired

SUCCESSFUL OUTCOME		HandoverCommand

UNSUCCESSFUL OUTCOME	HandoverPreparationFailure

PROCEDURE CODE			id-HandoverPreparation

CRITICALITY				reject

}

handoverResourceAllocation NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverRequest

SUCCESSFUL OUTCOME		HandoverRequestAcknowledge

UNSUCCESSFUL OUTCOME	HandoverFailure

PROCEDURE CODE			id-HandoverResourceAllocation

CRITICALITY				reject

}

handoverSuccess NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		HandoverSuccess

PROCEDURE CODE			id-HandoverSuccess

CRITICALITY				ignore

}

initialContextSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		InitialContextSetupRequest

SUCCESSFUL OUTCOME		InitialContextSetupResponse

UNSUCCESSFUL OUTCOME	InitialContextSetupFailure

PROCEDURE CODE			id-InitialContextSetup

CRITICALITY				reject

}

initialUEMessage NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		InitialUEMessage

PROCEDURE CODE			id-InitialUEMessage

CRITICALITY				ignore

}

locationReport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		LocationReport

PROCEDURE CODE			id-LocationReport

CRITICALITY				ignore

}

locationReportingControl NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		LocationReportingControl

PROCEDURE CODE			id-LocationReportingControl

CRITICALITY				ignore

}

locationReportingFailureIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		LocationReportingFailureIndication

PROCEDURE CODE			id-LocationReportingFailureIndication

CRITICALITY				ignore

}

mTCommunicationHandling NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MTCommunicationHandlingRequest

SUCCESSFUL OUTCOME		MTCommunicationHandlingResponse

UNSUCCESSFUL OUTCOME	MTCommunicationHandlingFailure

PROCEDURE CODE			id-MTCommunicationHandling

CRITICALITY				reject

}

multicastSessionActivation NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastSessionActivationRequest

SUCCESSFUL OUTCOME		MulticastSessionActivationResponse

UNSUCCESSFUL OUTCOME	MulticastSessionActivationFailure

PROCEDURE CODE			id-MulticastSessionActivation

CRITICALITY				reject

}

multicastSessionDeactivation NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastSessionDeactivationRequest

SUCCESSFUL OUTCOME		MulticastSessionDeactivationResponse

PROCEDURE CODE			id-MulticastSessionDeactivation

CRITICALITY				reject

}

multicastSessionUpdate NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastSessionUpdateRequest

SUCCESSFUL OUTCOME		MulticastSessionUpdateResponse

UNSUCCESSFUL OUTCOME	MulticastSessionUpdateFailure

PROCEDURE CODE			id-MulticastSessionUpdate

CRITICALITY				reject

}

multicastGroupPaging 		NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastGroupPaging

PROCEDURE CODE			id-MulticastGroupPaging

CRITICALITY				ignore

}

nASNonDeliveryIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NASNonDeliveryIndication

PROCEDURE CODE			id-NASNonDeliveryIndication

CRITICALITY				ignore

}

nGReset NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGReset

SUCCESSFUL OUTCOME		NGResetAcknowledge

PROCEDURE CODE			id-NGReset

CRITICALITY				reject

}

nGSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGSetupRequest

SUCCESSFUL OUTCOME		NGSetupResponse

UNSUCCESSFUL OUTCOME	NGSetupFailure

PROCEDURE CODE			id-NGSetup

CRITICALITY				reject

}

overloadStart NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		OverloadStart

PROCEDURE CODE			id-OverloadStart

CRITICALITY				ignore

}

overloadStop NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		OverloadStop

PROCEDURE CODE			id-OverloadStop

CRITICALITY				reject

}

paging NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Paging

PROCEDURE CODE			id-Paging

CRITICALITY				ignore

}

pathSwitchRequest NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PathSwitchRequest

SUCCESSFUL OUTCOME		PathSwitchRequestAcknowledge

UNSUCCESSFUL OUTCOME	PathSwitchRequestFailure

PROCEDURE CODE			id-PathSwitchRequest

CRITICALITY				reject

}

pDUSessionResourceModify NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDUSessionResourceModifyRequest

SUCCESSFUL OUTCOME		PDUSessionResourceModifyResponse

PROCEDURE CODE			id-PDUSessionResourceModify

CRITICALITY				reject

}

pDUSessionResourceModifyIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDUSessionResourceModifyIndication

SUCCESSFUL OUTCOME		PDUSessionResourceModifyConfirm

PROCEDURE CODE			id-PDUSessionResourceModifyIndication

CRITICALITY				reject

}

pDUSessionResourceNotify NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDUSessionResourceNotify

PROCEDURE CODE			id-PDUSessionResourceNotify

CRITICALITY				ignore

}

pDUSessionResourceRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDUSessionResourceReleaseCommand

SUCCESSFUL OUTCOME		PDUSessionResourceReleaseResponse

PROCEDURE CODE			id-PDUSessionResourceRelease

CRITICALITY				reject

}

pDUSessionResourceSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDUSessionResourceSetupRequest

SUCCESSFUL OUTCOME		PDUSessionResourceSetupResponse

PROCEDURE CODE			id-PDUSessionResourceSetup

CRITICALITY				reject

}

privateMessage NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-PrivateMessage

CRITICALITY				ignore

}

pWSCancel NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSCancelRequest

SUCCESSFUL OUTCOME		PWSCancelResponse

PROCEDURE CODE			id-PWSCancel

CRITICALITY				reject

}

pWSFailureIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSFailureIndication

PROCEDURE CODE			id-PWSFailureIndication

CRITICALITY				ignore

}

pWSRestartIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSRestartIndication

PROCEDURE CODE			id-PWSRestartIndication

CRITICALITY				ignore

}

rANConfigurationUpdate NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RANConfigurationUpdate

SUCCESSFUL OUTCOME		RANConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	RANConfigurationUpdateFailure

PROCEDURE CODE			id-RANConfigurationUpdate

CRITICALITY				reject

}

rANCPRelocationIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RANCPRelocationIndication

PROCEDURE CODE			id-RANCPRelocationIndication

CRITICALITY				reject

}

rANPagingRequest NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RANPagingRequest

PROCEDURE CODE			id-RANPagingRequest

CRITICALITY				ignore

}

rerouteNASRequest NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RerouteNASRequest

PROCEDURE CODE			id-RerouteNASRequest

CRITICALITY				reject

}

retrieveUEInformation NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RetrieveUEInformation

PROCEDURE CODE			id-RetrieveUEInformation

CRITICALITY				reject

}

rRCInactiveTransitionReport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RRCInactiveTransitionReport

PROCEDURE CODE			id-RRCInactiveTransitionReport

CRITICALITY				ignore

}

secondaryRATDataUsageReport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SecondaryRATDataUsageReport

PROCEDURE CODE			id-SecondaryRATDataUsageReport

CRITICALITY				ignore

}

timingSynchronisationStatus NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TimingSynchronisationStatusRequest

SUCCESSFUL OUTCOME		TimingSynchronisationStatusResponse

UNSUCCESSFUL OUTCOME	TimingSynchronisationStatusFailure

PROCEDURE CODE			id-TimingSynchronisationStatus

CRITICALITY				reject

}

timingSynchronisationStatusReport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TimingSynchronisationStatusReport

PROCEDURE CODE			id-TimingSynchronisationStatusReport

CRITICALITY				ignore

}

traceFailureIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceFailureIndication

PROCEDURE CODE			id-TraceFailureIndication

CRITICALITY				ignore

}

traceStart NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-TraceStart

CRITICALITY				ignore

}

uEContextModification NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextModificationRequest

SUCCESSFUL OUTCOME		UEContextModificationResponse

UNSUCCESSFUL OUTCOME	UEContextModificationFailure

PROCEDURE CODE			id-UEContextModification

CRITICALITY				reject

}

uEContextRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseCommand

SUCCESSFUL OUTCOME		UEContextReleaseComplete

PROCEDURE CODE			id-UEContextRelease

CRITICALITY				reject

}

uEContextReleaseRequest NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseRequest

PROCEDURE CODE			id-UEContextReleaseRequest

CRITICALITY				ignore

}

uEContextResume NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextResumeRequest

SUCCESSFUL OUTCOME		UEContextResumeResponse

UNSUCCESSFUL OUTCOME	UEContextResumeFailure

PROCEDURE CODE			id-UEContextResume

CRITICALITY				reject

}

uEContextSuspend NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextSuspendRequest

SUCCESSFUL OUTCOME		UEContextSuspendResponse

UNSUCCESSFUL OUTCOME	UEContextSuspendFailure

PROCEDURE CODE			id-UEContextSuspend

CRITICALITY				reject

}

uEInformationTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEInformationTransfer

PROCEDURE CODE			id-UEInformationTransfer

CRITICALITY				reject

}

uERadioCapabilityCheck NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UERadioCapabilityCheckRequest

SUCCESSFUL OUTCOME		UERadioCapabilityCheckResponse

PROCEDURE CODE			id-UERadioCapabilityCheck

CRITICALITY				reject

}

uERadioCapabilityIDMapping NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UERadioCapabilityIDMappingRequest

SUCCESSFUL OUTCOME		UERadioCapabilityIDMappingResponse

PROCEDURE CODE			id-UERadioCapabilityIDMapping

CRITICALITY				reject

}

uERadioCapabilityInfoIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UERadioCapabilityInfoIndication

PROCEDURE CODE			id-UERadioCapabilityInfoIndication

CRITICALITY				ignore

}

uETNLABindingRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UETNLABindingReleaseRequest

PROCEDURE CODE			id-UETNLABindingRelease

CRITICALITY				ignore

}

uplinkNASTransport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkNASTransport

PROCEDURE CODE			id-UplinkNASTransport

CRITICALITY				ignore

}

uplinkNonUEAssociatedNRPPaTransport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkNonUEAssociatedNRPPaTransport

PROCEDURE CODE			id-UplinkNonUEAssociatedNRPPaTransport

CRITICALITY				ignore

}

uplinkRANConfigurationTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkRANConfigurationTransfer

PROCEDURE CODE			id-UplinkRANConfigurationTransfer

CRITICALITY				ignore

}

uplinkRANEarlyStatusTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkRANEarlyStatusTransfer

PROCEDURE CODE			id-UplinkRANEarlyStatusTransfer

CRITICALITY				reject

}

uplinkRANStatusTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkRANStatusTransfer

PROCEDURE CODE			id-UplinkRANStatusTransfer

CRITICALITY				ignore

}

uplinkUEAssociatedNRPPaTransport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkUEAssociatedNRPPaTransport

PROCEDURE CODE			id-UplinkUEAssociatedNRPPaTransport

CRITICALITY				ignore

}

writeReplaceWarning NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		WriteReplaceWarningRequest

SUCCESSFUL OUTCOME		WriteReplaceWarningResponse

PROCEDURE CODE			id-WriteReplaceWarning

CRITICALITY				reject

}

uplinkRIMInformationTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UplinkRIMInformationTransfer

PROCEDURE CODE			id-UplinkRIMInformationTransfer

CRITICALITY				ignore

}

downlinkRIMInformationTransfer NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DownlinkRIMInformationTransfer

PROCEDURE CODE			id-DownlinkRIMInformationTransfer

CRITICALITY				ignore

}

inventoryRequest NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		InventoryRequest

SUCCESSFUL OUTCOME		InventoryResponse

UNSUCCESSFUL OUTCOME	InventoryFailure

PROCEDURE CODE			id-InventoryRequest

CRITICALITY				reject

}

inventoryReport NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		InventoryReport

PROCEDURE CODE			id-InventoryReport

CRITICALITY				reject

}

commandRequest NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CommandRequest

SUCCESSFUL OUTCOME		CommandResponse

UNSUCCESSFUL OUTCOME	CommandFailure

PROCEDURE CODE			id-CommandRequest

CRITICALITY				reject

}

END

-- ASN1STOP

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

A2X-PC5-QoS-Parameters,

AerialUEsubscriptionInformation,

AIoT-CorrelationIdentifier,

AIOTFIdentifier,

AllowedNSSAI,

AMFName,

AMFSetID,

AMF-TNLAssociationSetupList,

AMF-TNLAssociationToAddList,

AMF-TNLAssociationToRemoveList,

AMF-TNLAssociationToUpdateList,

AMF-UE-NGAP-ID,

AssistanceDataForPaging,

AssociatedSessionID,

AUN3DeviceAccessInfo,

AuthenticatedIndication,

BroadcastCancelledAreaList,

BroadcastCompletedAreaList,

BroadcastTransportFailureTransfer,

BroadcastTransportRequestTransfer,

BroadcastTransportResponseTransfer,

CancelAllWarningMessages,

Cause,

CellIDListForRestart,

CEmodeBrestricted,

CEmodeBSupport-Indicator,

CNAssistedRANTuning,

CommandRequestTransfer,

CommandResponseTransfer,

CommandFailureTransfer,

ConcurrentWarningMessageInd,

CoreNetworkAssistanceInformationForInactive,

CPTransportLayerInformation,

CriticalityDiagnostics,

DataCodingScheme,

DirectForwardingPathAvailability,

DL-CP-SecurityInformation,

DL-Signalling,

EarlyStatusTransfer-TransparentContainer,

EDT-Session,

EmergencyAreaIDListForRestart,

EmergencyFallbackIndicator,

EN-DCSONConfigurationTransfer,

EndIndication,

Enhanced-CoverageRestriction,

ERedCapIndication,

EUTRA-CGI,

EUTRA-PagingeDRXInformation,

Extended-AMFName,

Extended-ConnectedTime,

Extended-RANNodeName,

FiveGCAction,

FiveG-ProSeAuthorized,

FiveG-ProSePC5QoSParameters,

FiveG-S-TMSI,

GlobalRANNodeID,

GUAMI,

HandoverFlag,

HandoverType,

IAB-Authorized,

IABNodeIndication,

IAB-Supported,

IMSVoiceSupportIndicator,

IndexToRFSP,

InfoOnRecommendedCellsAndRANNodesForPaging,

IntersystemSONConfigurationTransfer,

InventoryRequestTransfer,

InventoryResponseTransfer,

InventoryFailureTransfer,

InventoryReportTransfer,

LAI,

LocationReportingRequestType,

LTE-A2X-ServicesAuthorized,

LTEM-Indication,

LTEUESidelinkAggregateMaximumBitrate,

LTEV2XServicesAuthorized,

MaskedIMEISV,

MBS-AreaSessionID,

MBS-DistributionReleaseRequestTransfer,

MBS-DistributionSetupRequestTransfer,

MBS-DistributionSetupResponseTransfer,

MBS-DistributionSetupUnsuccessfulTransfer,

MBS-ServiceArea,

MBS-SessionID,

MBSSessionReleaseResponseTransfer,

MBSSessionSetupOrModFailureTransfer,

MBSSessionSetupOrModRequestTransfer,

MBSSessionSetupOrModResponseTransfer,

MDTPLMNList,

MDTPLMNModificationList,

MessageIdentifier,

MobileIAB-Authorized,

MobileIABNodeIndication,

MobileIAB-Supported,

MobilityRestrictionList,

MulticastGroupPagingAreaList,

MulticastSessionActivationRequestTransfer,

MulticastSessionDeactivationRequestTransfer,

MulticastSessionUpdateRequestTransfer,

NAS-PDU,

NASSecurityParametersFromNGRAN,

NB-IoT-DefaultPagingDRX,

NB-IoT-PagingDRX,

NB-IoT-Paging-eDRXInfo,

NB-IoT-UEPriority,

NetworkControlledRepeaterAuthorized,

NewSecurityContextInd,

NGRAN-CGI,

NGRAN-TNLAssociationToRemoveList,

NGRANTraceID,

NID,

NoPDUSessionIndication,

NotifySourceNGRANNode,

NPN-AccessInformation,

NR-A2X-ServicesAuthorized,

NR-CGI,

NR-PagingeDRXInformation,

NRPPa-PDU,

NRUESidelinkAggregateMaximumBitrate,

NRV2XServicesAuthorized,

NumberOfBroadcastsRequested,

OverloadResponse,

OverloadStartNSSAIList,

PagingAssisDataforCEcapabUE,

PagingCause,

PagingDRX,

PagingOrigin,

PagingPolicyDifferentiation,

PagingPriority,

Partially-Allowed-NSSAI,

PC5QoSParameters,

PDUSessionAggregateMaximumBitRate,

PDUSessionListMTCommHReq,

PDUSessionResourceAdmittedList,

PDUSessionResourceFailedToModifyListModCfm,

PDUSessionResourceFailedToModifyListModRes,

PDUSessionResourceFailedToResumeListRESReq,

PDUSessionResourceFailedToResumeListRESRes,

PDUSessionResourceFailedToSetupListCxtFail,

PDUSessionResourceFailedToSetupListCxtRes,

PDUSessionResourceFailedToSetupListHOAck,

PDUSessionResourceFailedToSetupListPSReq,

PDUSessionResourceFailedToSetupListSURes,

PDUSessionResourceHandoverList,

PDUSessionResourceListCxtRelCpl,

PDUSessionResourceListCxtRelReq,

PDUSessionResourceListHORqd,

PDUSessionResourceModifyListModCfm,

PDUSessionResourceModifyListModInd,

PDUSessionResourceModifyListModReq,

PDUSessionResourceModifyListModRes,

PDUSessionResourceNotifyList,

PDUSessionResourceReleasedListNot,

PDUSessionResourceReleasedListPSAck,

PDUSessionResourceReleasedListPSFail,

PDUSessionResourceReleasedListRelRes,

PDUSessionResourceResumeListRESReq,

PDUSessionResourceResumeListRESRes,

PDUSessionResourceSecondaryRATUsageList,

PDUSessionResourceSetupListCxtReq,

PDUSessionResourceSetupListCxtRes,

PDUSessionResourceSetupListHOReq,

PDUSessionResourceSetupListSUReq,

PDUSessionResourceSetupListSURes,

PDUSessionResourceSuspendListSUSReq,

PDUSessionResourceSwitchedList,

PDUSessionResourceToBeSwitchedDLList,

PDUSessionResourceToReleaseListHOCmd,

PDUSessionResourceToReleaseListRelCmd,

PEIPSassistanceInformation,

PLMNIdentity,

PLMNSupportList,

PrivacyIndicator,

PWSFailedCellIDList,

QMCConfigInfo,

QMCDeactivation,

RANNodeName,

RANPagingPriority,

RANStatusTransfer-TransparentContainer,

RANTimingSynchronisationStatusInfo,

RAN-TSSRequestType,

RAN-TSSScope,

RAN-UE-NGAP-ID,

RedCapIndication,

RedirectionVoiceFallback,

RelativeAMFCapacity,

RepetitionPeriod,

ResetType,

RGLevelWirelineAccessCharacteristics,

RIMInformationTransfer,

RoutingID,

RRCEstablishmentCause,

RRCInactiveTransitionReportRequest,

RRCState,

SecurityContext,

SecurityKey,

SerialNumber,

ServedGUAMIList,

SliceSupportList,

SLPositioningRangingServiceInfo,

S-NSSAI,

SONConfigurationTransfer,

SourceToTarget-AMFInformationReroute,

SourceToTarget-TransparentContainer,

SRVCCOperationPossible,

SupportedTAList,

Suspend-Request-Indication,

Suspend-Response-Indication,

TAI,

TAIListForPaging,

TAIListForRestart,

TargetID,

TargetNSSAIInformation,

TargettoSource-Failure-TransparentContainer,

TargetToSource-TransparentContainer,

TimeSyncAssistanceInfo,

TimeToWait,

TNLAssociationList,

TraceActivation,

TrafficLoadReductionIndication,

TransportLayerAddress,

UEAggregateMaximumBitRate,

UE-associatedLogicalNG-connectionList,

UECapabilityInfoRequest,

UEContextRequest,

UE-DifferentiationInfo,

UE-NGAP-IDs,

UEPagingIdentity,

UEPresenceInAreaOfInterestList,

UERadioCapability,

UERadioCapabilityForPaging,

UERadioCapabilityID,

UERetentionInformation,

UESecurityCapabilities,

UESliceMaximumBitRateList,

UE-UP-CIoT-Support,

UL-CP-SecurityInformation,

UnavailableGUAMIList,

URI-address,

UserLocationInformation,

WarningAreaCoordinates,

WarningAreaList,

WarningMessageContents,

WarningSecurityInfo,

WarningType,

WUS-Assistance-Information,

XrDeviceWith2Rx

FROM NGAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

NGAP-PRIVATE-IES,

NGAP-PROTOCOL-EXTENSION,

NGAP-PROTOCOL-IES,

NGAP-PROTOCOL-IES-PAIR

FROM NGAP-Containers

id-A2X-PC5-QoS-Parameters,

id-AerialUEsubscriptionInformation,

id-AIoT-CorrelationIdentifier,

id-AIOTFIdentifier,

id-AllowedNSSAI,

id-AMFName,

id-AMFOverloadResponse,

id-AMFSetID,

id-AMF-TNLAssociationFailedToSetupList,

id-AMF-TNLAssociationSetupList,

id-AMF-TNLAssociationToAddList,

id-AMF-TNLAssociationToRemoveList,

id-AMF-TNLAssociationToUpdateList,

id-AMFTrafficLoadReductionIndication,

id-AMF-UE-NGAP-ID,

id-AssistanceDataForPaging,

id-AssociatedSessionID,

id-AUN3DeviceAccessInfo,

id-AuthenticatedIndication,

id-BroadcastCancelledAreaList,

id-BroadcastCompletedAreaList,

id-BroadcastTransportFailureTransfer,

id-BroadcastTransportRequestTransfer,

id-BroadcastTransportResponseTransfer,

id-CancelAllWarningMessages,

id-Cause,

id-CellIDListForRestart,

id-CEmodeBrestricted,

id-CEmodeBSupport-Indicator,

id-CNAssistedRANTuning,

id-CommandRequestTransfer,

id-CommandResponseTransfer,

id-CommandFailureTransfer,

id-ConcurrentWarningMessageInd,

id-CoreNetworkAssistanceInformationForInactive,

id-CriticalityDiagnostics,

id-DataCodingScheme,

id-DefaultPagingDRX,

id-DirectForwardingPathAvailability,

id-DL-CP-SecurityInformation,

id-DL-Signalling,

id-EarlyStatusTransfer-TransparentContainer,

id-EDT-Session,

id-EmergencyAreaIDListForRestart,

id-EmergencyFallbackIndicator,

id-ENDC-SONConfigurationTransferDL,

id-ENDC-SONConfigurationTransferUL,

id-EndIndication,

id-Enhanced-CoverageRestriction,

id-ERedCapIndication,

id-EUTRA-CGI,

id-EUTRA-PagingeDRXInformation,

id-Extended-AMFName,

id-Extended-ConnectedTime,

id-Extended-RANNodeName,

id-FiveGCAction,

id-FiveG-ProSeAuthorized,

id-FiveG-ProSePC5QoSParameters,

id-FiveG-ProSeUEPC5AggregateMaximumBitRate,

id-FiveG-S-TMSI,

id-GlobalRANNodeID,

id-GUAMI,

id-HandoverFlag,

id-HandoverType,

id-IAB-Authorized,

id-IABNodeIndication,

id-IAB-Supported,

id-IMSVoiceSupportIndicator,

id-IndexToRFSP,

id-InfoOnRecommendedCellsAndRANNodesForPaging,

id-IntersystemSONConfigurationTransferDL,

id-IntersystemSONConfigurationTransferUL,

id-InventoryRequestTransfer,

id-InventoryResponseTransfer,

id-InventoryFailureTransfer,

id-InventoryReportTransfer,

id-LocationReportingRequestType,

id-LTE-A2X-ServicesAuthorized,

id-LTE-A2X-UE-PC5-AggregateMaximumBitRate,

id-LTEM-Indication,

id-LTEUESidelinkAggregateMaximumBitrate,

id-LTEV2XServicesAuthorized,

id-ManagementBasedMDTPLMNList,

id-ManagementBasedMDTPLMNModificationList,

id-MaskedIMEISV,

id-MBS-AreaSessionID,

id-MBS-DistributionReleaseRequestTransfer,

id-MBS-DistributionSetupRequestTransfer,

id-MBS-DistributionSetupResponseTransfer,

id-MBS-DistributionSetupUnsuccessfulTransfer,

id-MBS-ServiceArea,

id-MBS-SessionID,

id-MBSSessionModificationFailureTransfer,

id-MBSSessionModificationRequestTransfer,

id-MBSSessionModificationResponseTransfer,

id-MBSSessionReleaseResponseTransfer,

id-MBSSessionSetupFailureTransfer,

id-MBSSessionSetupRequestTransfer,

id-MBSSessionSetupResponseTransfer,

id-MessageIdentifier,

id-MobileIAB-Authorized,

id-MobileIABNodeIndication,

id-MobileIAB-Supported,

id-MobilityRestrictionList,

id-MulticastGroupPagingAreaList,

id-MulticastSessionActivationRequestTransfer,

id-MulticastSessionDeactivationRequestTransfer,

id-MulticastSessionUpdateRequestTransfer,

id-NASC,

id-NAS-PDU,

id-NASSecurityParametersFromNGRAN,

id-NB-IoT-DefaultPagingDRX,

id-NB-IoT-PagingDRX,

id-NB-IoT-Paging-eDRXInfo,

id-NB-IoT-UEPriority,

id-NetworkControlledRepeaterAuthorized,

id-NewAMF-UE-NGAP-ID,

id-NewGUAMI,

id-NewSecurityContextInd,

id-NGAP-Message,

id-NGRAN-CGI,

id-NGRAN-TNLAssociationToRemoveList,

id-NGRANTraceID,

id-NoPDUSessionIndication,

id-NotifySourceNGRANNode,

id-NPN-AccessInformation,

id-NR-A2X-ServicesAuthorized,

id-NR-A2X-UE-PC5-AggregateMaximumBitRate,

id-NR-PagingeDRXInformation,

id-NRPPa-PDU,

id-NRUESidelinkAggregateMaximumBitrate,

id-NRV2XServicesAuthorized,

id-NumberOfBroadcastsRequested,

id-OldAMF,

id-OverloadStartNSSAIList,

id-PagingAssisDataforCEcapabUE,

id-PagingCause,

id-PagingDRX,

id-PagingOrigin,

id-PagingPolicyDifferentiation,

id-PagingPriority,

id-Partially-Allowed-NSSAI,

id-PC5QoSParameters,

id-PDUSessionListMTCommHReq,

id-PDUSessionResourceAdmittedList,

id-PDUSessionResourceFailedToModifyListModCfm,

id-PDUSessionResourceFailedToModifyListModRes,

id-PDUSessionResourceFailedToResumeListRESReq,

id-PDUSessionResourceFailedToResumeListRESRes,

id-PDUSessionResourceFailedToSetupListCxtFail,

id-PDUSessionResourceFailedToSetupListCxtRes,

id-PDUSessionResourceFailedToSetupListHOAck,

id-PDUSessionResourceFailedToSetupListPSReq,

id-PDUSessionResourceFailedToSetupListSURes,

id-PDUSessionResourceHandoverList,

id-PDUSessionResourceListCxtRelCpl,

id-PDUSessionResourceListCxtRelReq,

id-PDUSessionResourceListHORqd,

id-PDUSessionResourceModifyListModCfm,

id-PDUSessionResourceModifyListModInd,

id-PDUSessionResourceModifyListModReq,

id-PDUSessionResourceModifyListModRes,

id-PDUSessionResourceNotifyList,

id-PDUSessionResourceReleasedListNot,

id-PDUSessionResourceReleasedListPSAck,

id-PDUSessionResourceReleasedListPSFail,

id-PDUSessionResourceReleasedListRelRes,

id-PDUSessionResourceResumeListRESReq,

id-PDUSessionResourceResumeListRESRes,

id-PDUSessionResourceSecondaryRATUsageList,

id-PDUSessionResourceSetupListCxtReq,

id-PDUSessionResourceSetupListCxtRes,

id-PDUSessionResourceSetupListHOReq,

id-PDUSessionResourceSetupListSUReq,

id-PDUSessionResourceSetupListSURes,

id-PDUSessionResourceSuspendListSUSReq,

id-PDUSessionResourceSwitchedList,

id-PDUSessionResourceToBeSwitchedDLList,

id-PDUSessionResourceToReleaseListHOCmd,

id-PDUSessionResourceToReleaseListRelCmd,

id-PEIPSassistanceInformation,

id-PLMNSupportList,

id-PrivacyIndicator,

id-PWSFailedCellIDList,

id-QMCConfigInfo,

id-QMCDeactivation,

id-RANNodeName,

id-RANPagingPriority,

id-RANStatusTransfer-TransparentContainer,

id-RANTimingSynchronisationStatusInfo,

id-RAN-TSSRequestType,

id-RAN-TSSScope,

id-RAN-UE-NGAP-ID,

id-RedCapIndication,

id-RedirectionVoiceFallback,

id-RelativeAMFCapacity,

id-RepetitionPeriod,

id-ResetType,

id-RGLevelWirelineAccessCharacteristics,

id-RIMInformationTransfer,

id-RoutingID,

id-RRCEstablishmentCause,

id-RRCInactiveTransitionReportRequest,

id-RRC-Resume-Cause,

id-RRCState,

id-SecurityContext,

id-SecurityKey,

id-SelectedNID,

id-SelectedPLMNIdentity,

id-SerialNumber,

id-ServedGUAMIList,

id-SliceSupportList,

id-S-NSSAI,

id-SONConfigurationTransferDL,

id-SONConfigurationTransferUL,

id-SourceAMF-UE-NGAP-ID,

id-SourceToTarget-AMFInformationReroute,

id-SourceToTarget-TransparentContainer,

id-SRVCCOperationPossible,

id-SupportedTAList,

id-Suspend-Request-Indication,

id-Suspend-Response-Indication,

id-TAI,

id-TAIListForPaging,

id-TAIListForRestart,

id-TargetID,

id-TargetNSSAIInformation,

id-TargettoSource-Failure-TransparentContainer,

id-TargetToSource-TransparentContainer,

id-TimeSyncAssistanceInfo,

id-TimeToWait,

id-TNGFIdentityInformation,

id-TraceActivation,

id-TraceCollectionEntityIPAddress,

id-TraceCollectionEntityURI,

id-TWIFIdentityInformation,

id-UEAggregateMaximumBitRate,

id-UE-associatedLogicalNG-connectionList,

id-UECapabilityInfoRequest,

id-UEContextRequest,

id-UE-DifferentiationInfo,

id-UE-NGAP-IDs,

id-UEPagingIdentity,

id-UEPresenceInAreaOfInterestList,

id-UERadioCapability,

id-UERadioCapability-EUTRA-Format,

id-UERadioCapabilityForPaging,

id-UERadioCapabilityID,

id-UERetentionInformation,

id-UESecurityCapabilities,

id-UESliceMaximumBitRateList,

id-UE-UP-CIoT-Support,

id-UL-CP-SecurityInformation,

id-UnavailableGUAMIList,

id-UserLocationInformation,

id-W-AGFIdentityInformation,

id-WarningAreaCoordinates,

id-WarningAreaList,

id-WarningMessageContents,

id-WarningSecurityInfo,

id-WarningType,

id-WUS-Assistance-Information,

id-XrDeviceWith2Rx,

id-SLPositioningRangingServiceInfo

FROM NGAP-Constants;

-- **************************************************************

--

#### -- PDU SESSION MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- PDU Session Resource Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- PDU SESSION RESOURCE SETUP REQUEST

--

-- **************************************************************

PDUSessionResourceSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceSetupRequestIEs} },

...

}

PDUSessionResourceSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID								PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID								PRESENCE mandatory	}|

{ ID id-RANPagingPriority						CRITICALITY ignore	TYPE RANPagingPriority							PRESENCE optional		}|

{ ID id-NAS-PDU									CRITICALITY reject	TYPE NAS-PDU									PRESENCE optional		}|

{ ID id-PDUSessionResourceSetupListSUReq		CRITICALITY reject	TYPE PDUSessionResourceSetupListSUReq		PRESENCE mandatory	}|

{ ID id-UEAggregateMaximumBitRate				CRITICALITY ignore	TYPE UEAggregateMaximumBitRate				PRESENCE optional		}|

{ ID id-UESliceMaximumBitRateList				CRITICALITY ignore	TYPE UESliceMaximumBitRateList				PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- PDU SESSION RESOURCE SETUP RESPONSE

--

-- **************************************************************

PDUSessionResourceSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceSetupResponseIEs} },

...

}

PDUSessionResourceSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-PDUSessionResourceSetupListSURes			CRITICALITY ignore	TYPE PDUSessionResourceSetupListSURes				PRESENCE optional		}|

{ ID id-PDUSessionResourceFailedToSetupListSURes	CRITICALITY ignore	TYPE PDUSessionResourceFailedToSetupListSURes		PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		}|

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation							PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- PDU Session Resource Release Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- PDU SESSION RESOURCE RELEASE COMMAND

--

-- **************************************************************

PDUSessionResourceReleaseCommand ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceReleaseCommandIEs} },

...

}

PDUSessionResourceReleaseCommandIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY reject	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY reject	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RANPagingPriority							CRITICALITY ignore	TYPE RANPagingPriority								PRESENCE optional		}|

{ ID id-NAS-PDU										CRITICALITY ignore	TYPE NAS-PDU											PRESENCE optional		}|

{ ID id-PDUSessionResourceToReleaseListRelCmd		CRITICALITY reject	TYPE PDUSessionResourceToReleaseListRelCmd		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- PDU SESSION RESOURCE RELEASE RESPONSE

--

-- **************************************************************

PDUSessionResourceReleaseResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceReleaseResponseIEs} },

...

}

PDUSessionResourceReleaseResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-PDUSessionResourceReleasedListRelRes			CRITICALITY ignore	TYPE PDUSessionResourceReleasedListRelRes		PRESENCE mandatory	}|

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation						PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- PDU Session Resource Modify Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- PDU SESSION RESOURCE MODIFY REQUEST

--

-- **************************************************************

PDUSessionResourceModifyRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceModifyRequestIEs} },

...

}

PDUSessionResourceModifyRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID								PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID								PRESENCE mandatory	}|

{ ID id-RANPagingPriority						CRITICALITY ignore	TYPE RANPagingPriority							PRESENCE optional		}|

{ ID id-PDUSessionResourceModifyListModReq		CRITICALITY reject	TYPE PDUSessionResourceModifyListModReq	PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- PDU SESSION RESOURCE MODIFY RESPONSE

--

-- **************************************************************

PDUSessionResourceModifyResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceModifyResponseIEs} },

...

}

PDUSessionResourceModifyResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID									CRITICALITY ignore	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID									CRITICALITY ignore	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-PDUSessionResourceModifyListModRes				CRITICALITY ignore	TYPE PDUSessionResourceModifyListModRes			PRESENCE optional		}|

{ ID id-PDUSessionResourceFailedToModifyListModRes		CRITICALITY ignore	TYPE PDUSessionResourceFailedToModifyListModRes		PRESENCE optional	}|

{ ID id-UserLocationInformation							CRITICALITY ignore	TYPE UserLocationInformation							PRESENCE optional		}|

{ ID id-CriticalityDiagnostics							CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- PDU Session Resource Notify Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- PDU SESSION RESOURCE NOTIFY

--

-- **************************************************************

PDUSessionResourceNotify ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceNotifyIEs} },

...

}

PDUSessionResourceNotifyIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-PDUSessionResourceNotifyList			CRITICALITY reject	TYPE PDUSessionResourceNotifyList				PRESENCE optional		}|

{ ID id-PDUSessionResourceReleasedListNot		CRITICALITY ignore	TYPE PDUSessionResourceReleasedListNot		PRESENCE optional		}|

{ ID id-UserLocationInformation					CRITICALITY ignore	TYPE UserLocationInformation						PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- PDU Session Resource Modify Indication Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- PDU SESSION RESOURCE MODIFY INDICATION

--

-- **************************************************************

PDUSessionResourceModifyIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceModifyIndicationIEs} },

...

}

PDUSessionResourceModifyIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-PDUSessionResourceModifyListModInd		CRITICALITY reject	TYPE PDUSessionResourceModifyListModInd		PRESENCE mandatory	}|

{ ID id-UserLocationInformation					CRITICALITY ignore	TYPE UserLocationInformation						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- PDU SESSION RESOURCE MODIFY CONFIRM

--

-- **************************************************************

PDUSessionResourceModifyConfirm ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceModifyConfirmIEs} },

...

}

PDUSessionResourceModifyConfirmIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-PDUSessionResourceModifyListModCfm			CRITICALITY ignore	TYPE PDUSessionResourceModifyListModCfm		PRESENCE optional	}|

{ ID id-PDUSessionResourceFailedToModifyListModCfm	CRITICALITY ignore	TYPE PDUSessionResourceFailedToModifyListModCfm		PRESENCE optional	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- UE CONTEXT MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- Initial Context Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- INITIAL CONTEXT SETUP REQUEST

--

-- **************************************************************

InitialContextSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {InitialContextSetupRequestIEs} },

...

}

InitialContextSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-OldAMF									CRITICALITY reject	TYPE AMFName											PRESENCE optional		}|

{ ID id-UEAggregateMaximumBitRate				CRITICALITY reject	TYPE UEAggregateMaximumBitRate						PRESENCE conditional	}|

-- The above IE shall be present if the PDU Session Resource Setup List IE is present

{ ID id-CoreNetworkAssistanceInformationForInactive		CRITICALITY ignore	TYPE CoreNetworkAssistanceInformationForInactive		PRESENCE optional		}|

{ ID id-GUAMI									CRITICALITY reject	TYPE GUAMI												PRESENCE mandatory	}|

{ ID id-PDUSessionResourceSetupListCxtReq		CRITICALITY reject	TYPE PDUSessionResourceSetupListCxtReq			PRESENCE optional		}|

{ ID id-AllowedNSSAI							CRITICALITY reject	TYPE AllowedNSSAI										PRESENCE mandatory	}|

{ ID id-UESecurityCapabilities					CRITICALITY reject	TYPE UESecurityCapabilities							PRESENCE mandatory	}|

{ ID id-SecurityKey								CRITICALITY reject	TYPE SecurityKey										PRESENCE mandatory	}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation									PRESENCE optional		}|

{ ID id-MobilityRestrictionList					CRITICALITY ignore	TYPE MobilityRestrictionList							PRESENCE optional		}|

{ ID id-UERadioCapability						CRITICALITY ignore	TYPE UERadioCapability									PRESENCE optional		}|

{ ID id-IndexToRFSP								CRITICALITY ignore	TYPE IndexToRFSP										PRESENCE optional		}|

{ ID id-MaskedIMEISV							CRITICALITY ignore	TYPE MaskedIMEISV										PRESENCE optional		}|

{ ID id-NAS-PDU									CRITICALITY ignore	TYPE NAS-PDU											PRESENCE optional		}|

{ ID id-EmergencyFallbackIndicator				CRITICALITY reject	TYPE EmergencyFallbackIndicator						PRESENCE optional		}|

{ ID id-RRCInactiveTransitionReportRequest		CRITICALITY ignore	TYPE RRCInactiveTransitionReportRequest			PRESENCE optional		}|

{ ID id-UERadioCapabilityForPaging				CRITICALITY ignore	TYPE UERadioCapabilityForPaging						PRESENCE optional		}|

{ ID id-RedirectionVoiceFallback				CRITICALITY ignore	TYPE RedirectionVoiceFallback						PRESENCE optional		}|

{ ID id-LocationReportingRequestType			CRITICALITY ignore	TYPE LocationReportingRequestType					PRESENCE optional		}|

{ ID id-CNAssistedRANTuning						CRITICALITY ignore	TYPE CNAssistedRANTuning								PRESENCE optional		}|

{ ID id-SRVCCOperationPossible					CRITICALITY ignore	TYPE SRVCCOperationPossible							PRESENCE optional		}|

{ ID id-IAB-Authorized							CRITICALITY ignore	TYPE IAB-Authorized										PRESENCE optional		}|

{ ID id-Enhanced-CoverageRestriction			CRITICALITY ignore	TYPE Enhanced-CoverageRestriction					PRESENCE optional		}|

{ ID id-Extended-ConnectedTime					CRITICALITY ignore	TYPE Extended-ConnectedTime							PRESENCE optional		}|

{ ID id-UE-DifferentiationInfo					CRITICALITY ignore	TYPE UE-DifferentiationInfo							PRESENCE optional		}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional		}|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized						PRESENCE optional		}|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional		}|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-PC5QoSParameters						CRITICALITY ignore	TYPE PC5QoSParameters									PRESENCE optional 	}|

{ ID id-CEmodeBrestricted						CRITICALITY ignore	TYPE CEmodeBrestricted									PRESENCE optional		}|

{ ID id-UE-UP-CIoT-Support						CRITICALITY ignore	TYPE UE-UP-CIoT-Support									PRESENCE optional		}|

{ ID id-RGLevelWirelineAccessCharacteristics	CRITICALITY ignore	TYPE RGLevelWirelineAccessCharacteristics		PRESENCE optional		}|

{ ID id-ManagementBasedMDTPLMNList				CRITICALITY ignore	TYPE MDTPLMNList										PRESENCE optional		}|

{ ID id-UERadioCapabilityID						CRITICALITY reject	TYPE UERadioCapabilityID								PRESENCE optional		}|

{ ID id-TimeSyncAssistanceInfo					CRITICALITY ignore	TYPE TimeSyncAssistanceInfo							PRESENCE optional		}|

{ ID id-QMCConfigInfo							CRITICALITY ignore	TYPE QMCConfigInfo										PRESENCE optional		}|

{ ID id-TargetNSSAIInformation					CRITICALITY ignore	TYPE TargetNSSAIInformation							PRESENCE optional		}|

{ ID id-UESliceMaximumBitRateList				CRITICALITY ignore	TYPE UESliceMaximumBitRateList						PRESENCE optional		}|

{ ID id-FiveG-ProSeAuthorized					CRITICALITY ignore	TYPE FiveG-ProSeAuthorized							PRESENCE optional		}|

{ ID id-FiveG-ProSeUEPC5AggregateMaximumBitRate	CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional		}|

{ ID id-FiveG-ProSePC5QoSParameters				CRITICALITY ignore	TYPE FiveG-ProSePC5QoSParameters					PRESENCE optional		}|

{ ID id-NetworkControlledRepeaterAuthorized 	CRITICALITY ignore	TYPE NetworkControlledRepeaterAuthorized 		PRESENCE optional		}|

{ ID id-AerialUEsubscriptionInformation			CRITICALITY ignore	TYPE AerialUEsubscriptionInformation				PRESENCE optional		}|

{ ID id-NR-A2X-ServicesAuthorized				CRITICALITY ignore	TYPE NR-A2X-ServicesAuthorized						PRESENCE optional		}|

{ ID id-LTE-A2X-ServicesAuthorized				CRITICALITY ignore	TYPE LTE-A2X-ServicesAuthorized						PRESENCE optional		}|

{ ID id-NR-A2X-UE-PC5-AggregateMaximumBitRate	CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional		}|

{ ID id-LTE-A2X-UE-PC5-AggregateMaximumBitRate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-A2X-PC5-QoS-Parameters					CRITICALITY ignore	TYPE A2X-PC5-QoS-Parameters							PRESENCE optional		}|

{ ID id-MobileIAB-Authorized					CRITICALITY ignore	TYPE MobileIAB-Authorized								PRESENCE optional		}|

{ ID id-Partially-Allowed-NSSAI					CRITICALITY ignore	TYPE Partially-Allowed-NSSAI							PRESENCE optional		}|

{ ID id-SLPositioningRangingServiceInfo			CRITICALITY ignore	TYPE SLPositioningRangingServiceInfo				PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- INITIAL CONTEXT SETUP RESPONSE

--

-- **************************************************************

InitialContextSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {InitialContextSetupResponseIEs} },

...

}

InitialContextSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-PDUSessionResourceSetupListCxtRes			CRITICALITY ignore	TYPE PDUSessionResourceSetupListCxtRes					PRESENCE optional		}|

{ ID id-PDUSessionResourceFailedToSetupListCxtRes	CRITICALITY ignore	TYPE PDUSessionResourceFailedToSetupListCxtRes		PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics									PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- INITIAL CONTEXT SETUP FAILURE

--

-- **************************************************************

InitialContextSetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {InitialContextSetupFailureIEs} },

...

}

InitialContextSetupFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-PDUSessionResourceFailedToSetupListCxtFail	CRITICALITY ignore	TYPE PDUSessionResourceFailedToSetupListCxtFail		PRESENCE optional		}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause														PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics									PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- UE Context Release Request Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- UE CONTEXT RELEASE REQUEST

--

-- **************************************************************

UEContextReleaseRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextReleaseRequest-IEs} },

...

}

UEContextReleaseRequest-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID						CRITICALITY reject	TYPE AMF-UE-NGAP-ID								PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID						CRITICALITY reject	TYPE RAN-UE-NGAP-ID								PRESENCE mandatory	}|

{ ID id-PDUSessionResourceListCxtRelReq		CRITICALITY reject	TYPE PDUSessionResourceListCxtRelReq		PRESENCE optional		}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause										PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- UE Context Release Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- UE CONTEXT RELEASE COMMAND

--

-- **************************************************************

UEContextReleaseCommand ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextReleaseCommand-IEs} },

...

}

UEContextReleaseCommand-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-UE-NGAP-IDs				CRITICALITY reject	TYPE UE-NGAP-IDs			PRESENCE mandatory	}|

{ ID id-Cause					CRITICALITY ignore	TYPE Cause					PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- UE CONTEXT RELEASE COMPLETE

--

-- **************************************************************

UEContextReleaseComplete ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextReleaseComplete-IEs} },

...

}

UEContextReleaseComplete-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation							PRESENCE optional		}|

{ ID id-InfoOnRecommendedCellsAndRANNodesForPaging	CRITICALITY ignore	TYPE InfoOnRecommendedCellsAndRANNodesForPaging	PRESENCE optional		}|

{ ID id-PDUSessionResourceListCxtRelCpl				CRITICALITY	reject	TYPE PDUSessionResourceListCxtRelCpl					PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		}|

{ ID id-PagingAssisDataforCEcapabUE					CRITICALITY ignore	TYPE PagingAssisDataforCEcapabUE						PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- UE Context Resume Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT RESUME REQUEST

--

-- **************************************************************

UEContextResumeRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextResumeRequestIEs} },

...

}

UEContextResumeRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RRC-Resume-Cause						CRITICALITY ignore	TYPE RRCEstablishmentCause							PRESENCE mandatory	}|

{ ID id-PDUSessionResourceResumeListRESReq		CRITICALITY reject	TYPE PDUSessionResourceResumeListRESReq			PRESENCE optional		}|

{ ID id-PDUSessionResourceFailedToResumeListRESReq		CRITICALITY reject	TYPE PDUSessionResourceFailedToResumeListRESReq			PRESENCE optional		}|

{ ID id-Suspend-Request-Indication				CRITICALITY ignore	TYPE Suspend-Request-Indication						PRESENCE optional		}|

{ ID id-InfoOnRecommendedCellsAndRANNodesForPaging	CRITICALITY ignore	TYPE InfoOnRecommendedCellsAndRANNodesForPaging		PRESENCE optional		}|

{ ID id-PagingAssisDataforCEcapabUE					CRITICALITY ignore	TYPE PagingAssisDataforCEcapabUE				PRESENCE optional		}|

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation					PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- UE CONTEXT RESUME RESPONSE

--

-- **************************************************************

UEContextResumeResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextResumeResponseIEs} },

...

}

UEContextResumeResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY ignore	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY ignore	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-PDUSessionResourceResumeListRESRes		CRITICALITY reject	TYPE PDUSessionResourceResumeListRESRes			PRESENCE optional		}|

{ ID id-PDUSessionResourceFailedToResumeListRESRes		CRITICALITY reject	TYPE PDUSessionResourceFailedToResumeListRESRes			PRESENCE optional		}|

{ ID id-SecurityContext							CRITICALITY reject	TYPE SecurityContext									PRESENCE optional		}|

{ ID id-Suspend-Response-Indication				CRITICALITY ignore	TYPE Suspend-Response-Indication					PRESENCE optional		}|

{ ID id-Extended-ConnectedTime					CRITICALITY ignore	TYPE Extended-ConnectedTime							PRESENCE optional		}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- UE CONTEXT RESUME FAILURE

--

-- **************************************************************

UEContextResumeFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { UEContextResumeFailureIEs} },

...

}

UEContextResumeFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY ignore	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- UE Context Suspend Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT SUSPEND REQUEST

--

-- **************************************************************

UEContextSuspendRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextSuspendRequestIEs} },

...

}

UEContextSuspendRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY reject	TYPE AMF-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY reject	TYPE RAN-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-InfoOnRecommendedCellsAndRANNodesForPaging	CRITICALITY ignore	TYPE InfoOnRecommendedCellsAndRANNodesForPaging		PRESENCE optional		}|

{ ID id-PagingAssisDataforCEcapabUE					CRITICALITY ignore	TYPE PagingAssisDataforCEcapabUE							PRESENCE optional		}|

{ ID id-PDUSessionResourceSuspendListSUSReq			CRITICALITY reject	TYPE PDUSessionResourceSuspendListSUSReq				PRESENCE optional		}|

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation								PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- UE CONTEXT SUSPEND RESPONSE

--

-- **************************************************************

UEContextSuspendResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextSuspendResponseIEs} },

...

}

UEContextSuspendResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY ignore	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY ignore	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-SecurityContext							CRITICALITY reject	TYPE SecurityContext									PRESENCE optional		}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- UE CONTEXT SUSPEND FAILURE

--

-- **************************************************************

UEContextSuspendFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { UEContextSuspendFailureIEs} },

...

}

UEContextSuspendFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY ignore	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- UE Context Modification Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RANPagingPriority						CRITICALITY ignore	TYPE RANPagingPriority								PRESENCE optional		}|

{ ID id-SecurityKey								CRITICALITY reject	TYPE SecurityKey									PRESENCE optional		}|

{ ID id-IndexToRFSP								CRITICALITY ignore	TYPE IndexToRFSP									PRESENCE optional		}|

{ ID id-UEAggregateMaximumBitRate				CRITICALITY ignore	TYPE UEAggregateMaximumBitRate					PRESENCE optional		}|

{ ID id-UESecurityCapabilities					CRITICALITY reject	TYPE UESecurityCapabilities						PRESENCE optional		}|

{ ID id-CoreNetworkAssistanceInformationForInactive		CRITICALITY ignore	TYPE CoreNetworkAssistanceInformationForInactive		PRESENCE optional		}|

{ ID id-EmergencyFallbackIndicator				CRITICALITY reject	TYPE EmergencyFallbackIndicator					PRESENCE optional		}|

{ ID id-NewAMF-UE-NGAP-ID						CRITICALITY reject	TYPE AMF-UE-NGAP-ID									PRESENCE optional		}|

{ ID id-RRCInactiveTransitionReportRequest		CRITICALITY ignore	TYPE RRCInactiveTransitionReportRequest		PRESENCE optional		}|

{ ID id-NewGUAMI								CRITICALITY reject	TYPE GUAMI											PRESENCE optional		}|

{ ID id-CNAssistedRANTuning						CRITICALITY ignore	TYPE CNAssistedRANTuning							PRESENCE optional		}|

{ ID id-SRVCCOperationPossible					CRITICALITY ignore	TYPE SRVCCOperationPossible						PRESENCE optional		}|

{ ID id-IAB-Authorized							CRITICALITY ignore	TYPE IAB-Authorized									PRESENCE optional		}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized						PRESENCE optional		}|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized					PRESENCE optional		}|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate	PRESENCE optional		}|

{ ID id-PC5QoSParameters						CRITICALITY ignore	TYPE PC5QoSParameters								PRESENCE optional 	}|

{ ID id-UERadioCapabilityID						CRITICALITY reject	TYPE UERadioCapabilityID							PRESENCE optional		}|

{ ID id-RGLevelWirelineAccessCharacteristics	CRITICALITY ignore	TYPE RGLevelWirelineAccessCharacteristics	PRESENCE optional		}|

{ ID id-TimeSyncAssistanceInfo					CRITICALITY ignore	TYPE TimeSyncAssistanceInfo						PRESENCE optional		}|

{ ID id-QMCConfigInfo							CRITICALITY ignore	TYPE QMCConfigInfo									PRESENCE optional		}|

{ ID id-QMCDeactivation							CRITICALITY ignore	TYPE QMCDeactivation								PRESENCE optional		}|

{ ID id-UESliceMaximumBitRateList				CRITICALITY ignore	TYPE UESliceMaximumBitRateList					PRESENCE optional		}|

{ ID id-ManagementBasedMDTPLMNModificationList	CRITICALITY ignore	TYPE MDTPLMNModificationList						PRESENCE optional		}|

{ ID id-FiveG-ProSeAuthorized					CRITICALITY ignore	TYPE FiveG-ProSeAuthorized						PRESENCE optional		}|

{ ID id-FiveG-ProSeUEPC5AggregateMaximumBitRate	CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-FiveG-ProSePC5QoSParameters				CRITICALITY ignore	TYPE FiveG-ProSePC5QoSParameters				PRESENCE optional		}|

{ ID id-NetworkControlledRepeaterAuthorized 	CRITICALITY ignore	TYPE NetworkControlledRepeaterAuthorized 	PRESENCE optional		}|

{ ID id-AerialUEsubscriptionInformation			CRITICALITY ignore	TYPE AerialUEsubscriptionInformation			PRESENCE optional		}|

{ ID id-NR-A2X-ServicesAuthorized				CRITICALITY ignore	TYPE NR-A2X-ServicesAuthorized					PRESENCE optional		}|

{ ID id-LTE-A2X-ServicesAuthorized				CRITICALITY ignore	TYPE LTE-A2X-ServicesAuthorized					PRESENCE optional		}|

{ ID id-NR-A2X-UE-PC5-AggregateMaximumBitRate	CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-LTE-A2X-UE-PC5-AggregateMaximumBitRate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate	PRESENCE optional		}|

{ ID id-A2X-PC5-QoS-Parameters					CRITICALITY ignore	TYPE A2X-PC5-QoS-Parameters						PRESENCE optional		}|

{ ID id-MobileIAB-Authorized					CRITICALITY ignore	TYPE MobileIAB-Authorized							PRESENCE optional		}|

{ ID id-SLPositioningRangingServiceInfo			CRITICALITY ignore	TYPE SLPositioningRangingServiceInfo			PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

UEContextModificationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextModificationResponseIEs} },

...

}

UEContextModificationResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY ignore	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RRCState					CRITICALITY ignore	TYPE RRCState					PRESENCE optional		}|

{ ID id-UserLocationInformation		CRITICALITY ignore	TYPE UserLocationInformation	PRESENCE optional 	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION FAILURE

--

-- **************************************************************

UEContextModificationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UEContextModificationFailureIEs} },

...

}

UEContextModificationFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY ignore	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- RRC INACTIVE TRANSITION REPORT

--

-- **************************************************************

RRCInactiveTransitionReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RRCInactiveTransitionReportIEs} },

...

}

RRCInactiveTransitionReportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RRCState					CRITICALITY ignore	TYPE RRCState						PRESENCE mandatory	}|

{ ID id-UserLocationInformation		CRITICALITY ignore	TYPE UserLocationInformation		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- Retrieve UE Information

--

-- **************************************************************

RetrieveUEInformation ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { RetrieveUEInformationIEs} },

...

}

RetrieveUEInformationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-FiveG-S-TMSI					CRITICALITY reject	TYPE FiveG-S-TMSI				PRESENCE mandatory	},

...

}

-- **************************************************************

#### -- UE Information Transfer

--

-- **************************************************************

UEInformationTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {  UEInformationTransferIEs} },

...

}

UEInformationTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-FiveG-S-TMSI				CRITICALITY reject 	TYPE FiveG-S-TMSI				PRESENCE mandatory	}|

{ ID id-NB-IoT-UEPriority			CRITICALITY ignore	TYPE NB-IoT-UEPriority			PRESENCE optional		}|

{ ID id-UERadioCapability			CRITICALITY ignore	TYPE UERadioCapability			PRESENCE optional		}|

{ ID id-S-NSSAI						CRITICALITY ignore	TYPE S-NSSAI					PRESENCE optional		}|

{ ID id-AllowedNSSAI				CRITICALITY ignore	TYPE AllowedNSSAI				PRESENCE optional		}|

{ ID id-UE-DifferentiationInfo		CRITICALITY ignore	TYPE UE-DifferentiationInfo		PRESENCE optional		}|

{ ID id-MaskedIMEISV				CRITICALITY ignore	TYPE MaskedIMEISV				PRESENCE optional		}|

{ ID id-Partially-Allowed-NSSAI		CRITICALITY ignore	TYPE Partially-Allowed-NSSAI	PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- RAN CP Relocation Indication

--

-- **************************************************************

RANCPRelocationIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container { { RANCPRelocationIndicationIEs} },

...

}

RANCPRelocationIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RAN-UE-NGAP-ID					CRITICALITY reject	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-FiveG-S-TMSI					CRITICALITY reject	TYPE FiveG-S-TMSI					PRESENCE mandatory	}|

{ ID id-EUTRA-CGI						CRITICALITY ignore	TYPE EUTRA-CGI						PRESENCE mandatory	}|

{ ID id-TAI								CRITICALITY ignore	TYPE TAI							PRESENCE mandatory	}|

{ ID id-UL-CP-SecurityInformation		CRITICALITY reject	TYPE UL-CP-SecurityInformation		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- UE MOBILITY MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- Handover Preparation Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- HANDOVER REQUIRED

--

-- **************************************************************

HandoverRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {HandoverRequiredIEs} },

...

}

HandoverRequiredIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-HandoverType							CRITICALITY reject	TYPE HandoverType									PRESENCE mandatory	}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory	}|

{ ID id-TargetID								CRITICALITY reject	TYPE TargetID										PRESENCE mandatory	}|

{ ID id-DirectForwardingPathAvailability		CRITICALITY ignore	TYPE DirectForwardingPathAvailability			PRESENCE optional		}|

{ ID id-PDUSessionResourceListHORqd				CRITICALITY reject	TYPE PDUSessionResourceListHORqd				PRESENCE mandatory	}|

{ ID id-SourceToTarget-TransparentContainer		CRITICALITY reject	TYPE SourceToTarget-TransparentContainer		PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- HANDOVER COMMAND

--

-- **************************************************************

HandoverCommand ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {HandoverCommandIEs} },

...

}

HandoverCommandIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY reject	TYPE AMF-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY reject	TYPE RAN-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-HandoverType								CRITICALITY reject	TYPE HandoverType											PRESENCE mandatory	}|

{ ID id-NASSecurityParametersFromNGRAN				CRITICALITY reject	TYPE NASSecurityParametersFromNGRAN					PRESENCE conditional	}|

-- The above IE shall be present if the HandoverType IE is set to the value “5GStoEPPS” or “5GStoUTRAN”

{ ID id-PDUSessionResourceHandoverList				CRITICALITY ignore	TYPE PDUSessionResourceHandoverList					PRESENCE optional		}|

{ ID id-PDUSessionResourceToReleaseListHOCmd		CRITICALITY ignore	TYPE PDUSessionResourceToReleaseListHOCmd			PRESENCE optional		}|

{ ID id-TargetToSource-TransparentContainer			CRITICALITY reject	TYPE TargetToSource-TransparentContainer			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- HANDOVER PREPARATION FAILURE

--

-- **************************************************************

HandoverPreparationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {HandoverPreparationFailureIEs} },

...

}

HandoverPreparationFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY ignore	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		}|

{ ID id-TargettoSource-Failure-TransparentContainer		CRITICALITY ignore	TYPE TargettoSource-Failure-TransparentContainer		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Handover Resource Allocation Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- HANDOVER REQUEST

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {HandoverRequestIEs} },

...

}

HandoverRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-HandoverType							CRITICALITY reject	TYPE HandoverType									PRESENCE mandatory	}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory	}|

{ ID id-UEAggregateMaximumBitRate				CRITICALITY reject	TYPE UEAggregateMaximumBitRate					PRESENCE mandatory	}|

{ ID id-CoreNetworkAssistanceInformationForInactive		CRITICALITY ignore	TYPE CoreNetworkAssistanceInformationForInactive		PRESENCE optional		}|

{ ID id-UESecurityCapabilities					CRITICALITY reject	TYPE UESecurityCapabilities						PRESENCE mandatory	}|

{ ID id-SecurityContext							CRITICALITY reject	TYPE SecurityContext								PRESENCE mandatory	}|

{ ID id-NewSecurityContextInd					CRITICALITY reject	TYPE NewSecurityContextInd						PRESENCE optional		}|

{ ID id-NASC									CRITICALITY reject	TYPE NAS-PDU										PRESENCE optional		}|

{ ID id-PDUSessionResourceSetupListHOReq		CRITICALITY reject	TYPE PDUSessionResourceSetupListHOReq			PRESENCE mandatory	}|

{ ID id-AllowedNSSAI							CRITICALITY reject	TYPE AllowedNSSAI									PRESENCE mandatory	}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation								PRESENCE optional		}|

{ ID id-MaskedIMEISV							CRITICALITY ignore	TYPE MaskedIMEISV									PRESENCE optional		}|

{ ID id-SourceToTarget-TransparentContainer		CRITICALITY reject	TYPE SourceToTarget-TransparentContainer		PRESENCE mandatory	}|

{ ID id-MobilityRestrictionList					CRITICALITY ignore	TYPE MobilityRestrictionList						PRESENCE optional		}|

{ ID id-LocationReportingRequestType			CRITICALITY ignore	TYPE LocationReportingRequestType				PRESENCE optional		}|

{ ID id-RRCInactiveTransitionReportRequest		CRITICALITY ignore	TYPE RRCInactiveTransitionReportRequest		PRESENCE optional		}|

{ ID id-GUAMI									CRITICALITY reject	TYPE GUAMI											PRESENCE mandatory	}|

{ ID id-RedirectionVoiceFallback				CRITICALITY ignore	TYPE RedirectionVoiceFallback					PRESENCE optional		}|

{ ID id-CNAssistedRANTuning						CRITICALITY ignore	TYPE CNAssistedRANTuning							PRESENCE optional		}|

{ ID id-SRVCCOperationPossible					CRITICALITY ignore	TYPE SRVCCOperationPossible						PRESENCE optional		}|

{ ID id-IAB-Authorized							CRITICALITY reject	TYPE IAB-Authorized									PRESENCE optional		}|

{ ID id-Enhanced-CoverageRestriction			CRITICALITY ignore	TYPE Enhanced-CoverageRestriction				PRESENCE optional		}|

{ ID id-UE-DifferentiationInfo					CRITICALITY ignore	TYPE UE-DifferentiationInfo						PRESENCE optional		}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized						PRESENCE optional		}|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized					PRESENCE optional		}|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate	PRESENCE optional		}|

{ ID id-PC5QoSParameters						CRITICALITY ignore	TYPE PC5QoSParameters								PRESENCE optional 	}|

{ ID id-CEmodeBrestricted						CRITICALITY ignore	TYPE CEmodeBrestricted								PRESENCE optional		}|

{ ID id-UE-UP-CIoT-Support						CRITICALITY ignore	TYPE UE-UP-CIoT-Support								PRESENCE optional		}|

{ ID id-ManagementBasedMDTPLMNList				CRITICALITY ignore	TYPE MDTPLMNList									PRESENCE optional		}|

{ ID id-UERadioCapabilityID						CRITICALITY reject	TYPE UERadioCapabilityID							PRESENCE optional		}|

{ ID id-Extended-ConnectedTime					CRITICALITY ignore	TYPE Extended-ConnectedTime						PRESENCE optional		}|

{ ID id-TimeSyncAssistanceInfo					CRITICALITY ignore	TYPE TimeSyncAssistanceInfo						PRESENCE optional		}|

{ ID id-UESliceMaximumBitRateList				CRITICALITY ignore	TYPE UESliceMaximumBitRateList					PRESENCE optional		}|

{ ID id-FiveG-ProSeAuthorized					CRITICALITY ignore	TYPE FiveG-ProSeAuthorized						PRESENCE optional		}|

{ ID id-FiveG-ProSeUEPC5AggregateMaximumBitRate	CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-FiveG-ProSePC5QoSParameters				CRITICALITY ignore	TYPE FiveG-ProSePC5QoSParameters				PRESENCE optional		}|

{ ID id-AerialUEsubscriptionInformation			CRITICALITY ignore	TYPE AerialUEsubscriptionInformation			PRESENCE optional		}|

{ ID id-NR-A2X-ServicesAuthorized				CRITICALITY ignore	TYPE NR-A2X-ServicesAuthorized					PRESENCE optional		}|

{ ID id-LTE-A2X-ServicesAuthorized				CRITICALITY ignore	TYPE LTE-A2X-ServicesAuthorized					PRESENCE optional		}|

{ ID id-NR-A2X-UE-PC5-AggregateMaximumBitRate	CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-LTE-A2X-UE-PC5-AggregateMaximumBitRate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate	PRESENCE optional		}|

{ ID id-A2X-PC5-QoS-Parameters					CRITICALITY ignore	TYPE A2X-PC5-QoS-Parameters						PRESENCE optional		}|

{ ID id-MobileIAB-Authorized					CRITICALITY ignore	TYPE MobileIAB-Authorized							PRESENCE optional		}|

{ ID id-NoPDUSessionIndication					CRITICALITY ignore	TYPE NoPDUSessionIndication 						PRESENCE optional		}|

{ ID id-Partially-Allowed-NSSAI					CRITICALITY ignore	TYPE Partially-Allowed-NSSAI						PRESENCE optional		}|

{ ID id-SLPositioningRangingServiceInfo			CRITICALITY ignore	TYPE SLPositioningRangingServiceInfo			PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- HANDOVER REQUEST ACKNOWLEDGE

--

-- **************************************************************

HandoverRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {HandoverRequestAcknowledgeIEs} },

...

}

HandoverRequestAcknowledgeIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-PDUSessionResourceAdmittedList				CRITICALITY ignore	TYPE PDUSessionResourceAdmittedList					PRESENCE mandatory	}|

{ ID id-PDUSessionResourceFailedToSetupListHOAck	CRITICALITY ignore	TYPE PDUSessionResourceFailedToSetupListHOAck		PRESENCE optional		}|

{ ID id-TargetToSource-TransparentContainer			CRITICALITY reject	TYPE TargetToSource-TransparentContainer			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		}|

{ ID id-NPN-AccessInformation						CRITICALITY reject	TYPE NPN-AccessInformation								PRESENCE optional		}|

{ ID id-RedCapIndication							CRITICALITY ignore	TYPE RedCapIndication										PRESENCE optional		}|

{ ID id-ERedCapIndication							CRITICALITY ignore	TYPE ERedCapIndication									PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- HANDOVER FAILURE

--

-- **************************************************************

HandoverFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { HandoverFailureIEs} },

...

}

HandoverFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		}|

{ ID id-TargettoSource-Failure-TransparentContainer		CRITICALITY ignore	TYPE TargettoSource-Failure-TransparentContainer		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Handover Notification Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- HANDOVER NOTIFY

--

-- **************************************************************

HandoverNotify ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { HandoverNotifyIEs} },

...

}

HandoverNotifyIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-UserLocationInformation		CRITICALITY ignore	TYPE UserLocationInformation	PRESENCE mandatory	}|

{ ID id-NotifySourceNGRANNode		CRITICALITY ignore	TYPE NotifySourceNGRANNode		PRESENCE optional   	},

...

}

-- **************************************************************

--

##### -- Path Switch Request Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- PATH SWITCH REQUEST

--

-- **************************************************************

PathSwitchRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PathSwitchRequestIEs} },

...

}

PathSwitchRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RAN-UE-NGAP-ID								CRITICALITY reject	TYPE RAN-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-SourceAMF-UE-NGAP-ID						CRITICALITY reject	TYPE AMF-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation							PRESENCE mandatory	}|

{ ID id-UESecurityCapabilities						CRITICALITY ignore	TYPE UESecurityCapabilities								PRESENCE mandatory	}|

{ ID id-PDUSessionResourceToBeSwitchedDLList		CRITICALITY reject	TYPE PDUSessionResourceToBeSwitchedDLList			PRESENCE mandatory	}|

{ ID id-PDUSessionResourceFailedToSetupListPSReq	CRITICALITY ignore	TYPE PDUSessionResourceFailedToSetupListPSReq		PRESENCE optional		}|

{ ID id-RRC-Resume-Cause							CRITICALITY ignore	TYPE RRCEstablishmentCause								PRESENCE optional		}|

{ ID id-RedCapIndication							CRITICALITY ignore	TYPE RedCapIndication										PRESENCE optional		}|

{ ID id-ERedCapIndication							CRITICALITY ignore	TYPE ERedCapIndication									PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- PATH SWITCH REQUEST ACKNOWLEDGE

--

-- **************************************************************

PathSwitchRequestAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PathSwitchRequestAcknowledgeIEs} },

...

}

PathSwitchRequestAcknowledgeIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-UESecurityCapabilities						CRITICALITY reject	TYPE UESecurityCapabilities							PRESENCE optional		}|

{ ID id-SecurityContext								CRITICALITY reject	TYPE SecurityContext									PRESENCE mandatory	}|

{ ID id-NewSecurityContextInd						CRITICALITY reject	TYPE NewSecurityContextInd							PRESENCE optional		}|

{ ID id-PDUSessionResourceSwitchedList				CRITICALITY ignore	TYPE PDUSessionResourceSwitchedList				PRESENCE mandatory	}|

{ ID id-PDUSessionResourceReleasedListPSAck			CRITICALITY ignore	TYPE PDUSessionResourceReleasedListPSAck		PRESENCE optional		}|

{ ID id-AllowedNSSAI								CRITICALITY reject	TYPE AllowedNSSAI										PRESENCE mandatory	}|

{ ID id-CoreNetworkAssistanceInformationForInactive		CRITICALITY ignore	TYPE CoreNetworkAssistanceInformationForInactive		PRESENCE optional		}|

{ ID id-RRCInactiveTransitionReportRequest			CRITICALITY ignore	TYPE RRCInactiveTransitionReportRequest			PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional		}|

{ ID id-RedirectionVoiceFallback					CRITICALITY ignore	TYPE RedirectionVoiceFallback						PRESENCE optional		}|

{ ID id-CNAssistedRANTuning							CRITICALITY ignore	TYPE CNAssistedRANTuning								PRESENCE optional		}|

{ ID id-SRVCCOperationPossible						CRITICALITY ignore	TYPE SRVCCOperationPossible							PRESENCE optional		}|

{ ID id-Enhanced-CoverageRestriction				CRITICALITY ignore	TYPE Enhanced-CoverageRestriction					PRESENCE optional		}|

{ ID id-Extended-ConnectedTime						CRITICALITY ignore	TYPE Extended-ConnectedTime							PRESENCE optional		}|

{ ID id-UE-DifferentiationInfo						CRITICALITY ignore	TYPE UE-DifferentiationInfo							PRESENCE optional		}|

{ ID id-NRV2XServicesAuthorized						CRITICALITY ignore	TYPE NRV2XServicesAuthorized						PRESENCE optional		}|

{ ID id-LTEV2XServicesAuthorized					CRITICALITY ignore	TYPE LTEV2XServicesAuthorized						PRESENCE optional		}|

{ ID id-NRUESidelinkAggregateMaximumBitrate			CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-LTEUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-PC5QoSParameters							CRITICALITY ignore	TYPE PC5QoSParameters									PRESENCE optional 	}|

{ ID id-CEmodeBrestricted							CRITICALITY ignore	TYPE CEmodeBrestricted								PRESENCE optional		}|

{ ID id-UE-UP-CIoT-Support							CRITICALITY ignore	TYPE UE-UP-CIoT-Support								PRESENCE optional		}|

{ ID id-UERadioCapabilityID							CRITICALITY reject	TYPE UERadioCapabilityID								PRESENCE optional		}|

{ ID id-ManagementBasedMDTPLMNList					CRITICALITY ignore	TYPE MDTPLMNList										PRESENCE optional		}|

{ ID id-TimeSyncAssistanceInfo						CRITICALITY ignore	TYPE TimeSyncAssistanceInfo							PRESENCE optional		}|

{ ID id-FiveG-ProSeAuthorized						CRITICALITY ignore	TYPE FiveG-ProSeAuthorized							PRESENCE optional		}|

{ ID id-FiveG-ProSeUEPC5AggregateMaximumBitRate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-FiveG-ProSePC5QoSParameters					CRITICALITY ignore	TYPE FiveG-ProSePC5QoSParameters					PRESENCE optional		}|

{ ID id-ManagementBasedMDTPLMNModificationList		CRITICALITY ignore	TYPE MDTPLMNModificationList						PRESENCE optional		}|

{ ID id-IAB-Authorized								CRITICALITY ignore	TYPE IAB-Authorized										PRESENCE optional		}|

{ ID id-AerialUEsubscriptionInformation				CRITICALITY ignore	TYPE AerialUEsubscriptionInformation				PRESENCE optional		}|

{ ID id-NR-A2X-ServicesAuthorized					CRITICALITY ignore	TYPE NR-A2X-ServicesAuthorized						PRESENCE optional		}|

{ ID id-LTE-A2X-ServicesAuthorized					CRITICALITY ignore	TYPE LTE-A2X-ServicesAuthorized					PRESENCE optional		}|

{ ID id-NR-A2X-UE-PC5-AggregateMaximumBitRate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-LTE-A2X-UE-PC5-AggregateMaximumBitRate		CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional		}|

{ ID id-A2X-PC5-QoS-Parameters						CRITICALITY ignore	TYPE A2X-PC5-QoS-Parameters							PRESENCE optional		}|

{ ID id-MobileIAB-Authorized				   	    CRITICALITY ignore	TYPE MobileIAB-Authorized							PRESENCE optional		}|

{ ID id-Partially-Allowed-NSSAI						CRITICALITY ignore	TYPE Partially-Allowed-NSSAI						PRESENCE optional		}|

{ ID id-SLPositioningRangingServiceInfo				CRITICALITY ignore	TYPE SLPositioningRangingServiceInfo				PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- PATH SWITCH REQUEST FAILURE

--

-- **************************************************************

PathSwitchRequestFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PathSwitchRequestFailureIEs} },

...

}

PathSwitchRequestFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY ignore	TYPE AMF-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY ignore	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-PDUSessionResourceReleasedListPSFail	CRITICALITY ignore	TYPE PDUSessionResourceReleasedListPSFail	PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Handover Cancellation Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- HANDOVER CANCEL

--

-- **************************************************************

HandoverCancel ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { HandoverCancelIEs} },

...

}

HandoverCancelIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID		CRITICALITY reject	TYPE AMF-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID		CRITICALITY reject	TYPE RAN-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-Cause				CRITICALITY ignore	TYPE Cause				PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- HANDOVER CANCEL ACKNOWLEDGE

--

-- **************************************************************

HandoverCancelAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { HandoverCancelAcknowledgeIEs} },

...

}

HandoverCancelAcknowledgeIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY ignore	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- HANDOVER SUCCESS ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

###### -- HANDOVER SUCCESS

--

-- **************************************************************

HandoverSuccess ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { HandoverSuccessIEs} },

...

}

HandoverSuccessIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID		CRITICALITY reject	TYPE AMF-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID		CRITICALITY reject	TYPE RAN-UE-NGAP-ID		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- UPLINK RAN EARLY STATUS TRANSFER ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Uplink RAN Early Status Transfer

--

-- **************************************************************

UplinkRANEarlyStatusTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {UplinkRANEarlyStatusTransferIEs} },

...

}

UplinkRANEarlyStatusTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY reject	TYPE AMF-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY reject	TYPE RAN-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-EarlyStatusTransfer-TransparentContainer    CRITICALITY reject  TYPE EarlyStatusTransfer-TransparentContainer		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- DOWNLINK RAN EARLY STATUS TRANSFER ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Downlink RAN Early Status Transfer

--

-- **************************************************************

DownlinkRANEarlyStatusTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {DownlinkRANEarlyStatusTransferIEs} },

...

}

DownlinkRANEarlyStatusTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY reject	TYPE AMF-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY reject	TYPE RAN-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-EarlyStatusTransfer-TransparentContainer	CRITICALITY reject	TYPE EarlyStatusTransfer-TransparentContainer		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- Uplink RAN Status Transfer Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- UPLINK RAN STATUS TRANSFER

--

-- **************************************************************

UplinkRANStatusTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UplinkRANStatusTransferIEs} },

...

}

UplinkRANStatusTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY reject	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY reject	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RANStatusTransfer-TransparentContainer		CRITICALITY reject	TYPE RANStatusTransfer-TransparentContainer	PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- Downlink RAN Status Transfer Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- DOWNLINK RAN STATUS TRANSFER

--

-- **************************************************************

DownlinkRANStatusTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DownlinkRANStatusTransferIEs} },

...

}

DownlinkRANStatusTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY reject	TYPE AMF-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY reject	TYPE RAN-UE-NGAP-ID											PRESENCE mandatory	}|

{ ID id-RANStatusTransfer-TransparentContainer		CRITICALITY reject	TYPE RANStatusTransfer-TransparentContainer		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- PAGING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PAGING

--

-- **************************************************************

Paging ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PagingIEs} },

...

}

PagingIEs NGAP-PROTOCOL-IES ::= {

{ ID id-UEPagingIdentity				CRITICALITY ignore	TYPE UEPagingIdentity					PRESENCE mandatory	}|

{ ID id-PagingDRX						CRITICALITY ignore	TYPE PagingDRX							PRESENCE optional		}|

{ ID id-TAIListForPaging				CRITICALITY ignore	TYPE TAIListForPaging					PRESENCE mandatory	}|

{ ID id-PagingPriority					CRITICALITY ignore	TYPE PagingPriority						PRESENCE optional		}|

{ ID id-UERadioCapabilityForPaging		CRITICALITY ignore	TYPE UERadioCapabilityForPaging			PRESENCE optional		}|

{ ID id-PagingOrigin					CRITICALITY ignore	TYPE PagingOrigin						PRESENCE optional		}|

{ ID id-AssistanceDataForPaging			CRITICALITY ignore	TYPE AssistanceDataForPaging			PRESENCE optional		}|

{ ID id-NB-IoT-Paging-eDRXInfo			CRITICALITY ignore	TYPE NB-IoT-Paging-eDRXInfo				PRESENCE optional		}|

{ ID id-NB-IoT-PagingDRX				CRITICALITY ignore	TYPE NB-IoT-PagingDRX					PRESENCE optional		}|

{ ID id-Enhanced-CoverageRestriction	CRITICALITY ignore	TYPE Enhanced-CoverageRestriction		PRESENCE optional		}|

{ ID id-WUS-Assistance-Information		CRITICALITY ignore	TYPE WUS-Assistance-Information			PRESENCE optional		}|

{ ID id-EUTRA-PagingeDRXInformation		CRITICALITY ignore	TYPE EUTRA-PagingeDRXInformation		PRESENCE optional		}|

{ ID id-CEmodeBrestricted				CRITICALITY ignore	TYPE CEmodeBrestricted					PRESENCE optional		}|

{ ID id-NR-PagingeDRXInformation		CRITICALITY ignore	TYPE NR-PagingeDRXInformation			PRESENCE optional		}|

{ ID id-PagingCause						CRITICALITY ignore	TYPE PagingCause						PRESENCE optional		}|

{ ID id-PEIPSassistanceInformation		CRITICALITY ignore	TYPE PEIPSassistanceInformation			PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- NAS TRANSPORT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- INITIAL UE MESSAGE

--

-- **************************************************************

InitialUEMessage ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {InitialUEMessage-IEs} },

...

}

InitialUEMessage-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-NAS-PDU									CRITICALITY reject	TYPE NAS-PDU										PRESENCE mandatory	}|

{ ID id-UserLocationInformation					CRITICALITY reject	TYPE UserLocationInformation						PRESENCE mandatory	}|

{ ID id-RRCEstablishmentCause					CRITICALITY ignore	TYPE RRCEstablishmentCause						PRESENCE mandatory	}|

{ ID id-FiveG-S-TMSI							CRITICALITY reject	TYPE FiveG-S-TMSI									PRESENCE optional		}|

{ ID id-AMFSetID								CRITICALITY ignore	TYPE AMFSetID										PRESENCE optional		}|

{ ID id-UEContextRequest						CRITICALITY ignore	TYPE UEContextRequest								PRESENCE optional		}|

{ ID id-AllowedNSSAI							CRITICALITY reject	TYPE AllowedNSSAI									PRESENCE optional		}|

{ ID id-SourceToTarget-AMFInformationReroute	CRITICALITY ignore	TYPE SourceToTarget-AMFInformationReroute	PRESENCE optional		}|

{ ID id-SelectedPLMNIdentity					CRITICALITY ignore	TYPE PLMNIdentity									PRESENCE optional		}|

{ ID id-IABNodeIndication						CRITICALITY reject	TYPE IABNodeIndication								PRESENCE optional		}|

{ ID id-CEmodeBSupport-Indicator				CRITICALITY reject	TYPE CEmodeBSupport-Indicator					PRESENCE optional		}|

{ ID id-LTEM-Indication							CRITICALITY ignore	TYPE LTEM-Indication								PRESENCE optional		}|

{ ID id-EDT-Session								CRITICALITY ignore	TYPE EDT-Session									PRESENCE optional		}|

{ ID id-AuthenticatedIndication					CRITICALITY ignore	TYPE AuthenticatedIndication						PRESENCE optional		}|

{ ID id-NPN-AccessInformation					CRITICALITY reject	TYPE NPN-AccessInformation						PRESENCE optional		}|

{ ID id-RedCapIndication						CRITICALITY ignore	TYPE RedCapIndication								PRESENCE optional		}|

{ ID id-SelectedNID								CRITICALITY ignore	TYPE NID											PRESENCE optional		}|

{ ID id-MobileIABNodeIndication					CRITICALITY reject	TYPE MobileIABNodeIndication						PRESENCE optional		}|

{ ID id-Partially-Allowed-NSSAI					CRITICALITY ignore	TYPE Partially-Allowed-NSSAI	   				PRESENCE optional		}|

{ ID id-ERedCapIndication						CRITICALITY ignore	TYPE ERedCapIndication								PRESENCE optional		}|

{ ID id-AUN3DeviceAccessInfo					CRITICALITY ignore	TYPE AUN3DeviceAccessInfo							PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- DOWNLINK NAS TRANSPORT

--

-- **************************************************************

DownlinkNASTransport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DownlinkNASTransport-IEs} },

...

}

DownlinkNASTransport-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID					CRITICALITY reject	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID					CRITICALITY reject	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-OldAMF							CRITICALITY reject	TYPE AMFName						PRESENCE optional		}|

{ ID id-RANPagingPriority				CRITICALITY ignore	TYPE RANPagingPriority				PRESENCE optional		}|

{ ID id-NAS-PDU							CRITICALITY reject	TYPE NAS-PDU						PRESENCE mandatory	}|

{ ID id-MobilityRestrictionList			CRITICALITY ignore	TYPE MobilityRestrictionList		PRESENCE optional		}|

{ ID id-IndexToRFSP						CRITICALITY ignore	TYPE IndexToRFSP					PRESENCE optional		}|

{ ID id-UEAggregateMaximumBitRate		CRITICALITY ignore	TYPE UEAggregateMaximumBitRate		PRESENCE optional		}|

{ ID id-AllowedNSSAI					CRITICALITY reject	TYPE AllowedNSSAI					PRESENCE optional		}|

{ ID id-SRVCCOperationPossible			CRITICALITY ignore	TYPE SRVCCOperationPossible			PRESENCE optional		}|

{ ID id-Enhanced-CoverageRestriction	CRITICALITY ignore	TYPE Enhanced-CoverageRestriction	PRESENCE optional		}|

{ ID id-Extended-ConnectedTime			CRITICALITY ignore	TYPE Extended-ConnectedTime			PRESENCE optional		}|

{ ID id-UE-DifferentiationInfo			CRITICALITY ignore	TYPE UE-DifferentiationInfo			PRESENCE optional		}|

{ ID id-CEmodeBrestricted				CRITICALITY ignore	TYPE CEmodeBrestricted				PRESENCE optional		}|

{ ID id-UERadioCapability				CRITICALITY ignore	TYPE UERadioCapability				PRESENCE optional		}|

{ ID id-UECapabilityInfoRequest			CRITICALITY ignore	TYPE UECapabilityInfoRequest		PRESENCE optional		}|

{ ID id-EndIndication					CRITICALITY ignore	TYPE EndIndication					PRESENCE optional		}|

{ ID id-UERadioCapabilityID				CRITICALITY reject	TYPE UERadioCapabilityID			PRESENCE optional		}|

{ ID id-TargetNSSAIInformation			CRITICALITY ignore	TYPE TargetNSSAIInformation			PRESENCE optional		}|

{ ID id-MaskedIMEISV					CRITICALITY ignore	TYPE MaskedIMEISV					PRESENCE optional		}|

{ ID id-Partially-Allowed-NSSAI			CRITICALITY ignore	TYPE Partially-Allowed-NSSAI		PRESENCE optional		}|

{ ID id-MobileIAB-Authorized			CRITICALITY ignore	TYPE MobileIAB-Authorized			PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- UPLINK NAS TRANSPORT

--

-- **************************************************************

UplinkNASTransport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{ {UplinkNASTransport-IEs} },

...

}

UplinkNASTransport-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-NAS-PDU						CRITICALITY reject	TYPE NAS-PDU						PRESENCE mandatory	}|

{ ID id-UserLocationInformation		CRITICALITY ignore	TYPE UserLocationInformation		PRESENCE mandatory	}|

{ ID id-W-AGFIdentityInformation	CRITICALITY reject	TYPE OCTET STRING					PRESENCE optional	}|

{ ID id-TNGFIdentityInformation		CRITICALITY reject	TYPE OCTET STRING					PRESENCE optional	}|

{ ID id-TWIFIdentityInformation		CRITICALITY reject	TYPE OCTET STRING					PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- NAS NON DELIVERY INDICATION

--

-- **************************************************************

NASNonDeliveryIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NASNonDeliveryIndication-IEs} },

...

}

NASNonDeliveryIndication-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID		CRITICALITY reject	TYPE AMF-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID		CRITICALITY reject	TYPE RAN-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-NAS-PDU				CRITICALITY ignore	TYPE NAS-PDU			PRESENCE mandatory	}|

{ ID id-Cause				CRITICALITY ignore	TYPE Cause				PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- REROUTE NAS REQUEST

--

-- **************************************************************

RerouteNASRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RerouteNASRequest-IEs} },

...

}

RerouteNASRequest-IEs NGAP-PROTOCOL-IES ::= {

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID									PRESENCE mandatory	}|

{ ID id-AMF-UE-NGAP-ID							CRITICALITY ignore	TYPE AMF-UE-NGAP-ID									PRESENCE optional		}|

{ ID id-NGAP-Message							CRITICALITY reject	TYPE OCTET STRING									PRESENCE mandatory	}|

{ ID id-AMFSetID								CRITICALITY reject	TYPE AMFSetID										PRESENCE mandatory	}|

{ ID id-AllowedNSSAI							CRITICALITY reject	TYPE AllowedNSSAI									PRESENCE optional		}|

{ ID id-SourceToTarget-AMFInformationReroute	CRITICALITY ignore	TYPE SourceToTarget-AMFInformationReroute	PRESENCE optional 	}|

{ ID id-Partially-Allowed-NSSAI					CRITICALITY ignore	TYPE Partially-Allowed-NSSAI	               PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- INTERFACE MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- NG Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- NG SETUP REQUEST

--

-- **************************************************************

NGSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupRequestIEs} },

...

}

NGSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-GlobalRANNodeID				CRITICALITY reject	TYPE GlobalRANNodeID				PRESENCE mandatory	}|

{ ID id-RANNodeName					CRITICALITY ignore	TYPE RANNodeName					PRESENCE optional	}|

{ ID id-SupportedTAList				CRITICALITY reject	TYPE SupportedTAList				PRESENCE mandatory	}|

{ ID id-DefaultPagingDRX			CRITICALITY ignore	TYPE PagingDRX						PRESENCE mandatory	}|

{ ID id-UERetentionInformation		CRITICALITY ignore	TYPE UERetentionInformation			PRESENCE optional	}|

{ ID id-NB-IoT-DefaultPagingDRX		CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX		PRESENCE optional	}|

{ ID id-Extended-RANNodeName		CRITICALITY ignore	TYPE Extended-RANNodeName			PRESENCE optional	},

...

}

-- **************************************************************

--

###### -- NG SETUP RESPONSE

--

-- **************************************************************

NGSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupResponseIEs} },

...

}

NGSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMFName						CRITICALITY reject	TYPE AMFName					PRESENCE mandatory	}|

{ ID id-ServedGUAMIList				CRITICALITY reject	TYPE ServedGUAMIList			PRESENCE mandatory	}|

{ ID id-RelativeAMFCapacity			CRITICALITY ignore	TYPE RelativeAMFCapacity		PRESENCE mandatory	}|

{ ID id-PLMNSupportList				CRITICALITY reject	TYPE PLMNSupportList			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		}|

{ ID id-UERetentionInformation		CRITICALITY ignore	TYPE UERetentionInformation		PRESENCE optional		}|

{ ID id-IAB-Supported				CRITICALITY ignore	TYPE IAB-Supported				PRESENCE optional		}|

{ ID id-Extended-AMFName			CRITICALITY ignore	TYPE Extended-AMFName			PRESENCE optional		}|

{ ID id-MobileIAB-Supported			CRITICALITY ignore	TYPE MobileIAB-Supported    	PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- NG SETUP FAILURE

--

-- **************************************************************

NGSetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupFailureIEs} },

...

}

NGSetupFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional		}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- RAN Configuration Update Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- RAN CONFIGURATION UPDATE

--

-- **************************************************************

RANConfigurationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RANConfigurationUpdateIEs} },

...

}

RANConfigurationUpdateIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RANNodeName								CRITICALITY ignore	TYPE RANNodeName									PRESENCE optional	}|

{ ID id-SupportedTAList							CRITICALITY reject	TYPE SupportedTAList								PRESENCE optional	}|

{ ID id-DefaultPagingDRX						CRITICALITY ignore	TYPE PagingDRX										PRESENCE optional	}|

{ ID id-GlobalRANNodeID							CRITICALITY ignore	TYPE GlobalRANNodeID								PRESENCE optional	}|

{ ID id-NGRAN-TNLAssociationToRemoveList		CRITICALITY reject	TYPE NGRAN-TNLAssociationToRemoveList			PRESENCE optional	}|

{ ID id-NB-IoT-DefaultPagingDRX					CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX						PRESENCE optional	}|

{ ID id-Extended-RANNodeName					CRITICALITY ignore	TYPE Extended-RANNodeName							PRESENCE optional	},

...

}

-- **************************************************************

--

###### -- RAN CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

RANConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RANConfigurationUpdateAcknowledgeIEs} },

...

}

RANConfigurationUpdateAcknowledgeIEs NGAP-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- RAN CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

RANConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {RANConfigurationUpdateFailureIEs} },

...

}

RANConfigurationUpdateFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional		}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- AMF Configuration Update Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- AMF CONFIGURATION UPDATE

--

-- **************************************************************

AMFConfigurationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {AMFConfigurationUpdateIEs} },

...

}

AMFConfigurationUpdateIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMFName								CRITICALITY reject	TYPE AMFName								PRESENCE optional		}|

{ ID id-ServedGUAMIList						CRITICALITY reject	TYPE ServedGUAMIList						PRESENCE optional		}|

{ ID id-RelativeAMFCapacity					CRITICALITY ignore	TYPE RelativeAMFCapacity					PRESENCE optional		}|

{ ID id-PLMNSupportList						CRITICALITY reject	TYPE PLMNSupportList						PRESENCE optional		}|

{ ID id-AMF-TNLAssociationToAddList			CRITICALITY ignore	TYPE AMF-TNLAssociationToAddList			PRESENCE optional		}|

{ ID id-AMF-TNLAssociationToRemoveList		CRITICALITY ignore	TYPE AMF-TNLAssociationToRemoveList		PRESENCE optional		}|

{ ID id-AMF-TNLAssociationToUpdateList		CRITICALITY ignore	TYPE AMF-TNLAssociationToUpdateList		PRESENCE optional		}|

{ ID id-Extended-AMFName					CRITICALITY ignore	TYPE Extended-AMFName						PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- AMF CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

AMFConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {AMFConfigurationUpdateAcknowledgeIEs} },

...

}

AMFConfigurationUpdateAcknowledgeIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-TNLAssociationSetupList				CRITICALITY ignore	TYPE AMF-TNLAssociationSetupList			PRESENCE optional		}|

{ ID id-AMF-TNLAssociationFailedToSetupList		CRITICALITY ignore	TYPE TNLAssociationList							PRESENCE optional		}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- AMF CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

AMFConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {AMFConfigurationUpdateFailureIEs} },

...

}

AMFConfigurationUpdateFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional		}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- AMF Status Indication Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- AMF STATUS INDICATION

--

-- **************************************************************

AMFStatusIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {AMFStatusIndicationIEs} },

...

}

AMFStatusIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-UnavailableGUAMIList		CRITICALITY reject	TYPE UnavailableGUAMIList		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- NG Reset Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- NG RESET

--

-- **************************************************************

NGReset ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGResetIEs} },

...

}

NGResetIEs NGAP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-ResetType					CRITICALITY reject	TYPE ResetType					PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- NG RESET ACKNOWLEDGE

--

-- **************************************************************

NGResetAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGResetAcknowledgeIEs} },

...

}

NGResetAcknowledgeIEs NGAP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalNG-connectionList			CRITICALITY ignore	TYPE UE-associatedLogicalNG-connectionList			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics							CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- Error Indication Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- ERROR INDICATION

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {ErrorIndicationIEs} },

...

}

ErrorIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY ignore	TYPE AMF-UE-NGAP-ID				PRESENCE optional		}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY ignore	TYPE RAN-UE-NGAP-ID				PRESENCE optional		}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE optional		}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		}|

{ ID id-FiveG-S-TMSI				CRITICALITY ignore	TYPE FiveG-S-TMSI				PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- OVERLOAD START

--

-- **************************************************************

OverloadStart ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {OverloadStartIEs} },

...

}

OverloadStartIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMFOverloadResponse						CRITICALITY reject	TYPE OverloadResponse							PRESENCE optional 	}|

{ ID id-AMFTrafficLoadReductionIndication		CRITICALITY ignore	TYPE TrafficLoadReductionIndication		PRESENCE optional		}|

{ ID id-OverloadStartNSSAIList					CRITICALITY ignore	TYPE OverloadStartNSSAIList					PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- OVERLOAD STOP

--

-- **************************************************************

OverloadStop ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {OverloadStopIEs} },

...

}

OverloadStopIEs NGAP-PROTOCOL-IES ::= {

...

}

-- **************************************************************

--

#### -- CONFIGURATION TRANSFER ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- UPLINK RAN CONFIGURATION TRANSFER

--

-- **************************************************************

UplinkRANConfigurationTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UplinkRANConfigurationTransferIEs} },

...

}

UplinkRANConfigurationTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-SONConfigurationTransferUL					CRITICALITY ignore	TYPE SONConfigurationTransfer						PRESENCE optional		}|

{ ID id-ENDC-SONConfigurationTransferUL				CRITICALITY ignore	TYPE EN-DCSONConfigurationTransfer				PRESENCE optional		}|

{ ID id-IntersystemSONConfigurationTransferUL		CRITICALITY ignore	TYPE IntersystemSONConfigurationTransfer		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- DOWNLINK RAN CONFIGURATION TRANSFER

--

-- **************************************************************

DownlinkRANConfigurationTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DownlinkRANConfigurationTransferIEs} },

...

}

DownlinkRANConfigurationTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-SONConfigurationTransferDL					CRITICALITY ignore	TYPE SONConfigurationTransfer						PRESENCE optional		}|

{ ID id-ENDC-SONConfigurationTransferDL				CRITICALITY ignore	TYPE EN-DCSONConfigurationTransfer				PRESENCE optional		}|

{ ID id-IntersystemSONConfigurationTransferDL		CRITICALITY ignore	TYPE IntersystemSONConfigurationTransfer		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- WARNING MESSAGE TRANSMISSION ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- Write-Replace Warning Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- WRITE-REPLACE WARNING REQUEST

--

-- **************************************************************

WriteReplaceWarningRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {WriteReplaceWarningRequestIEs} },

...

}

WriteReplaceWarningRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MessageIdentifier				CRITICALITY reject	TYPE MessageIdentifier					PRESENCE mandatory	}|

{ ID id-SerialNumber					CRITICALITY reject	TYPE SerialNumber						PRESENCE mandatory	}|

{ ID id-WarningAreaList					CRITICALITY ignore	TYPE WarningAreaList					PRESENCE optional		}|

{ ID id-RepetitionPeriod				CRITICALITY reject	TYPE RepetitionPeriod					PRESENCE mandatory	}|

{ ID id-NumberOfBroadcastsRequested		CRITICALITY reject	TYPE NumberOfBroadcastsRequested		PRESENCE mandatory	}|

{ ID id-WarningType						CRITICALITY ignore	TYPE WarningType						PRESENCE optional		}|

{ ID id-WarningSecurityInfo				CRITICALITY ignore	TYPE WarningSecurityInfo				PRESENCE optional		}|

{ ID id-DataCodingScheme				CRITICALITY ignore	TYPE DataCodingScheme					PRESENCE optional		}|

{ ID id-WarningMessageContents			CRITICALITY ignore	TYPE WarningMessageContents				PRESENCE optional		}|

{ ID id-ConcurrentWarningMessageInd		CRITICALITY reject	TYPE ConcurrentWarningMessageInd		PRESENCE optional		}|

{ ID id-WarningAreaCoordinates			CRITICALITY ignore	TYPE WarningAreaCoordinates				PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- WRITE-REPLACE WARNING RESPONSE

--

-- **************************************************************

WriteReplaceWarningResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {WriteReplaceWarningResponseIEs} },

...

}

WriteReplaceWarningResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MessageIdentifier				CRITICALITY reject	TYPE MessageIdentifier					PRESENCE mandatory	}|

{ ID id-SerialNumber					CRITICALITY reject	TYPE SerialNumber						PRESENCE mandatory	}|

{ ID id-BroadcastCompletedAreaList		CRITICALITY ignore	TYPE BroadcastCompletedAreaList			PRESENCE optional		}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- PWS Cancel Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- PWS CANCEL REQUEST

--

-- **************************************************************

PWSCancelRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PWSCancelRequestIEs} },

...

}

PWSCancelRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MessageIdentifier			CRITICALITY reject	TYPE MessageIdentifier				PRESENCE mandatory	}|

{ ID id-SerialNumber				CRITICALITY reject	TYPE SerialNumber					PRESENCE mandatory	}|

{ ID id-WarningAreaList				CRITICALITY ignore	TYPE WarningAreaList				PRESENCE optional	}|

{ ID id-CancelAllWarningMessages	CRITICALITY reject	TYPE CancelAllWarningMessages		PRESENCE optional	},

...

}

-- **************************************************************

--

###### -- PWS CANCEL RESPONSE

--

-- **************************************************************

PWSCancelResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PWSCancelResponseIEs} },

...

}

PWSCancelResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MessageIdentifier			CRITICALITY reject	TYPE MessageIdentifier				PRESENCE mandatory	}|

{ ID id-SerialNumber				CRITICALITY reject	TYPE SerialNumber					PRESENCE mandatory	}|

{ ID id-BroadcastCancelledAreaList	CRITICALITY ignore	TYPE BroadcastCancelledAreaList		PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- PWS Restart Indication Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- PWS RESTART INDICATION

--

-- **************************************************************

PWSRestartIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PWSRestartIndicationIEs} },

...

}

PWSRestartIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-CellIDListForRestart			CRITICALITY reject	TYPE CellIDListForRestart				PRESENCE mandatory	}|

{ ID id-GlobalRANNodeID					CRITICALITY reject	TYPE GlobalRANNodeID					PRESENCE mandatory	}|

{ ID id-TAIListForRestart				CRITICALITY reject	TYPE TAIListForRestart					PRESENCE mandatory	}|

{ ID id-EmergencyAreaIDListForRestart	CRITICALITY reject	TYPE EmergencyAreaIDListForRestart	PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- PWS Failure Indication Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- PWS FAILURE INDICATION

--

-- **************************************************************

PWSFailureIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PWSFailureIndicationIEs} },

...

}

PWSFailureIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-PWSFailedCellIDList			CRITICALITY reject	TYPE PWSFailedCellIDList	PRESENCE mandatory	}|

{ ID id-GlobalRANNodeID				CRITICALITY reject	TYPE GlobalRANNodeID		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- NRPPA TRANSPORT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- DOWNLINK UE ASSOCIATED NRPPA TRANSPORT

--

-- **************************************************************

DownlinkUEAssociatedNRPPaTransport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DownlinkUEAssociatedNRPPaTransportIEs} },

...

}

DownlinkUEAssociatedNRPPaTransportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID			CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID			CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RoutingID				CRITICALITY reject	TYPE RoutingID					PRESENCE mandatory	}|

{ ID id-NRPPa-PDU				CRITICALITY reject	TYPE NRPPa-PDU					PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- UPLINK UE ASSOCIATED NRPPA TRANSPORT

--

-- **************************************************************

UplinkUEAssociatedNRPPaTransport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UplinkUEAssociatedNRPPaTransportIEs} },

...

}

UplinkUEAssociatedNRPPaTransportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID			CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID			CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RoutingID				CRITICALITY reject	TYPE RoutingID					PRESENCE mandatory	}|

{ ID id-NRPPa-PDU				CRITICALITY reject	TYPE NRPPa-PDU					PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT

--

-- **************************************************************

DownlinkNonUEAssociatedNRPPaTransport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DownlinkNonUEAssociatedNRPPaTransportIEs} },

...

}

DownlinkNonUEAssociatedNRPPaTransportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RoutingID				CRITICALITY reject	TYPE RoutingID					PRESENCE mandatory	}|

{ ID id-NRPPa-PDU				CRITICALITY reject	TYPE NRPPa-PDU					PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- UPLINK NON UE ASSOCIATED NRPPA TRANSPORT

--

-- **************************************************************

UplinkNonUEAssociatedNRPPaTransport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UplinkNonUEAssociatedNRPPaTransportIEs} },

...

}

UplinkNonUEAssociatedNRPPaTransportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RoutingID				CRITICALITY reject	TYPE RoutingID					PRESENCE mandatory	}|

{ ID id-NRPPa-PDU				CRITICALITY reject	TYPE NRPPa-PDU					PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- TRACE ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- TRACE START

--

-- **************************************************************

TraceStart ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TraceStartIEs} },

...

}

TraceStartIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-TraceActivation				CRITICALITY ignore	TYPE TraceActivation			PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- TRACE FAILURE INDICATION

--

-- **************************************************************

TraceFailureIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TraceFailureIndicationIEs} },

...

}

TraceFailureIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-NGRANTraceID				CRITICALITY ignore	TYPE NGRANTraceID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- DEACTIVATE TRACE

--

-- **************************************************************

DeactivateTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DeactivateTraceIEs} },

...

}

DeactivateTraceIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-NGRANTraceID				CRITICALITY ignore	TYPE NGRANTraceID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- CELL TRAFFIC TRACE

--

-- **************************************************************

CellTrafficTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {CellTrafficTraceIEs} },

...

}

CellTrafficTraceIEs NGAP-PROTOCOL-IES ::= {

{ID id-AMF-UE-NGAP-ID					CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ID id-RAN-UE-NGAP-ID					CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ID id-NGRANTraceID						CRITICALITY ignore	TYPE NGRANTraceID				PRESENCE mandatory	}|

{ID id-NGRAN-CGI						CRITICALITY ignore	TYPE NGRAN-CGI					PRESENCE mandatory	}|

{ID id-TraceCollectionEntityIPAddress	CRITICALITY ignore	TYPE TransportLayerAddress		PRESENCE mandatory	}|

{ID id-PrivacyIndicator					CRITICALITY ignore	TYPE PrivacyIndicator			PRESENCE optional	}|

{ID id-TraceCollectionEntityURI			CRITICALITY ignore	TYPE URI-address				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- LOCATION REPORTING ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- LOCATION REPORTING CONTROL

--

-- **************************************************************

LocationReportingControl ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {LocationReportingControlIEs} },

...

}

LocationReportingControlIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID					CRITICALITY reject	TYPE AMF-UE-NGAP-ID						PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID					CRITICALITY reject	TYPE RAN-UE-NGAP-ID						PRESENCE mandatory	}|

{ ID id-LocationReportingRequestType	CRITICALITY ignore	TYPE LocationReportingRequestType		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- LOCATION REPORTING FAILURE INDICATION

--

-- **************************************************************

LocationReportingFailureIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {LocationReportingFailureIndicationIEs} },

...

}

LocationReportingFailureIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID			CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID			CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-Cause					CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- LOCATION REPORT

--

-- **************************************************************

LocationReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {LocationReportIEs} },

...

}

LocationReportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID						CRITICALITY reject	TYPE AMF-UE-NGAP-ID							PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID						CRITICALITY reject	TYPE RAN-UE-NGAP-ID							PRESENCE mandatory	}|

{ ID id-UserLocationInformation				CRITICALITY ignore	TYPE UserLocationInformation				PRESENCE mandatory	}|

{ ID id-UEPresenceInAreaOfInterestList		CRITICALITY ignore	TYPE UEPresenceInAreaOfInterestList		PRESENCE optional		}|

{ ID id-LocationReportingRequestType		CRITICALITY ignore	TYPE LocationReportingRequestType		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- UE TNLA BINDING ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- UE TNLA BINDING RELEASE REQUEST

--

-- **************************************************************

UETNLABindingReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {UETNLABindingReleaseRequestIEs} },

...

}

UETNLABindingReleaseRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID		CRITICALITY reject	TYPE AMF-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID		CRITICALITY reject	TYPE RAN-UE-NGAP-ID		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- UE RADIO CAPABILITY MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- UE RADIO CAPABILITY INFO INDICATION

--

-- **************************************************************

UERadioCapabilityInfoIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {UERadioCapabilityInfoIndicationIEs} },

...

}

UERadioCapabilityInfoIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID					CRITICALITY reject	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID					CRITICALITY reject	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-UERadioCapability				CRITICALITY ignore	TYPE UERadioCapability				PRESENCE mandatory	}|

{ ID id-UERadioCapabilityForPaging		CRITICALITY ignore	TYPE UERadioCapabilityForPaging		PRESENCE optional		}|

{ ID id-UERadioCapability-EUTRA-Format	CRITICALITY ignore	TYPE UERadioCapability				PRESENCE optional		}|

{ ID id-XrDeviceWith2Rx					CRITICALITY ignore	TYPE XrDeviceWith2Rx				PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- UE Radio Capability Check Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- UE RADIO CAPABILITY CHECK REQUEST

--

-- **************************************************************

UERadioCapabilityCheckRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UERadioCapabilityCheckRequestIEs} },

...

}

UERadioCapabilityCheckRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID			CRITICALITY reject	TYPE AMF-UE-NGAP-ID			PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID			CRITICALITY reject	TYPE RAN-UE-NGAP-ID			PRESENCE mandatory	}|

{ ID id-UERadioCapability		CRITICALITY ignore	TYPE UERadioCapability		PRESENCE optional	}|

{ ID id-UERadioCapabilityID		CRITICALITY reject	TYPE UERadioCapabilityID	PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- UE RADIO CAPABILITY CHECK RESPONSE

--

-- **************************************************************

UERadioCapabilityCheckResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UERadioCapabilityCheckResponseIEs} },

...

}

UERadioCapabilityCheckResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID					CRITICALITY ignore	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID					CRITICALITY ignore	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-IMSVoiceSupportIndicator		CRITICALITY reject	TYPE IMSVoiceSupportIndicator		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- PRIVATE MESSAGE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container		{ { PrivateMessageIEs } },

...

}

PrivateMessageIEs NGAP-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

#### -- DATA USAGE REPORTING ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

#### -- SECONDARY RAT DATA USAGE REPORT

--

-- **************************************************************

SecondaryRATDataUsageReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {SecondaryRATDataUsageReportIEs} },

...

}

SecondaryRATDataUsageReportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID								CRITICALITY ignore	TYPE AMF-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID								CRITICALITY ignore	TYPE RAN-UE-NGAP-ID												PRESENCE mandatory	}|

{ ID id-PDUSessionResourceSecondaryRATUsageList		CRITICALITY ignore	TYPE PDUSessionResourceSecondaryRATUsageList			PRESENCE mandatory	}|

{ ID id-HandoverFlag								CRITICALITY ignore	TYPE HandoverFlag												PRESENCE optional		}|

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation								PRESENCE optional 	},

...

}

-- **************************************************************

--

#### -- RIM INFORMATION TRANSFER ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- UPLINK RIM INFORMATION TRANSFER

--

-- **************************************************************

UplinkRIMInformationTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UplinkRIMInformationTransferIEs} },

...

}

UplinkRIMInformationTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RIMInformationTransfer	CRITICALITY ignore	TYPE RIMInformationTransfer	PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- DOWNLINK RIM INFORMATION TRANSFER

--

-- **************************************************************

DownlinkRIMInformationTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DownlinkRIMInformationTransferIEs} },

...

}

DownlinkRIMInformationTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RIMInformationTransfer	CRITICALITY ignore	TYPE RIMInformationTransfer	PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- Connection Establishment Indication

--

-- **************************************************************

ConnectionEstablishmentIndication::= SEQUENCE {

protocolIEs			ProtocolIE-Container { {ConnectionEstablishmentIndicationIEs} },

...

}

ConnectionEstablishmentIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID					CRITICALITY reject	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID					CRITICALITY reject	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-UERadioCapability				CRITICALITY ignore	TYPE UERadioCapability				PRESENCE optional 	}|

{ ID id-EndIndication					CRITICALITY ignore	TYPE EndIndication					PRESENCE optional		}|

{ ID id-S-NSSAI							CRITICALITY ignore	TYPE S-NSSAI						PRESENCE optional		}|

{ ID id-AllowedNSSAI					CRITICALITY ignore	TYPE AllowedNSSAI					PRESENCE optional		}|

{ ID id-UE-DifferentiationInfo			CRITICALITY ignore	TYPE UE-DifferentiationInfo			PRESENCE optional		}|

{ ID id-DL-CP-SecurityInformation		CRITICALITY ignore	TYPE DL-CP-SecurityInformation		PRESENCE optional		}|

{ ID id-NB-IoT-UEPriority				CRITICALITY ignore	TYPE NB-IoT-UEPriority				PRESENCE optional		}|

{ ID id-Enhanced-CoverageRestriction	CRITICALITY ignore	TYPE Enhanced-CoverageRestriction	PRESENCE optional		}|

{ ID id-CEmodeBrestricted				CRITICALITY ignore	TYPE CEmodeBrestricted				PRESENCE optional		}|

{ ID id-UERadioCapabilityID				CRITICALITY reject	TYPE UERadioCapabilityID			PRESENCE optional		}|

{ ID id-MaskedIMEISV					CRITICALITY ignore	TYPE MaskedIMEISV					PRESENCE optional		}|

{ ID id-OldAMF							CRITICALITY reject	TYPE AMFName						PRESENCE optional		}|

{ ID id-Partially-Allowed-NSSAI			CRITICALITY ignore	TYPE Partially-Allowed-NSSAI		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- UE RADIO CAPABILITY ID MAPPING ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- UE RADIO CAPABILITY ID MAPPING REQUEST

--

-- **************************************************************

UERadioCapabilityIDMappingRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UERadioCapabilityIDMappingRequestIEs} },

...

}

UERadioCapabilityIDMappingRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-UERadioCapabilityID		CRITICALITY reject	TYPE UERadioCapabilityID	PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- UE RADIO CAPABILITY ID MAPPING RESPONSE

--

-- **************************************************************

UERadioCapabilityIDMappingResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {UERadioCapabilityIDMappingResponseIEs} },

...

}

UERadioCapabilityIDMappingResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-UERadioCapabilityID			CRITICALITY reject	TYPE UERadioCapabilityID		PRESENCE mandatory	}|

{ ID id-UERadioCapability			CRITICALITY ignore	TYPE UERadioCapability			PRESENCE mandatory 	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- AMF CP Relocation Indication

--

-- **************************************************************

AMFCPRelocationIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container { { AMFCPRelocationIndicationIEs} },

...

}

AMFCPRelocationIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-S-NSSAI						CRITICALITY ignore	TYPE S-NSSAI					PRESENCE optional		}| --this IE is not used and ignored

{ ID id-AllowedNSSAI				CRITICALITY ignore	TYPE AllowedNSSAI				PRESENCE optional		}| --this IE is not used and ignored

{ ID id-Partially-Allowed-NSSAI		CRITICALITY ignore	TYPE Partially-Allowed-NSSAI	PRESENCE optional		}, --this IE is not used and ignored

...

}

-- **************************************************************

--

#### -- MBS SESSION MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- Broadcast Session Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- BROADCAST SESSION SETUP REQUEST

--

-- **************************************************************

BroadcastSessionSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionSetupRequestIEs} },

...

}

BroadcastSessionSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID						CRITICALITY reject	TYPE MBS-SessionID										PRESENCE mandatory	}|

{ ID id-S-NSSAI								CRITICALITY reject	TYPE S-NSSAI											PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea						CRITICALITY reject	TYPE MBS-ServiceArea									PRESENCE mandatory	}|

{ ID id-MBSSessionSetupRequestTransfer		CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBSSessionSetupOrModRequestTransfer)		PRESENCE mandatory	}|

{ ID id-AssociatedSessionID					CRITICALITY ignore	TYPE AssociatedSessionID							PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- BROADCAST SESSION SETUP RESPONSE

--

-- **************************************************************

BroadcastSessionSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionSetupResponseIEs} },

...

}

BroadcastSessionSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID						CRITICALITY reject	TYPE MBS-SessionID										PRESENCE mandatory	}|

{ ID id-MBSSessionSetupResponseTransfer		CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBSSessionSetupOrModResponseTransfer)	PRESENCE optional		}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- BROADCAST SESSION SETUP FAILURE

--

-- **************************************************************

BroadcastSessionSetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionSetupFailureIEs} },

...

}

BroadcastSessionSetupFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID						CRITICALITY reject	TYPE MBS-SessionID										PRESENCE mandatory	}|

{ ID id-MBSSessionSetupFailureTransfer		CRITICALITY ignore	TYPE OCTET STRING (CONTAINING MBSSessionSetupOrModFailureTransfer	)	PRESENCE optional 	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause												PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Broadcast Session Modification Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- BROADCAST SESSION MODIFICATION REQUEST

--

-- **************************************************************

BroadcastSessionModificationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionModificationRequestIEs} },

...

}

BroadcastSessionModificationRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID							CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea							CRITICALITY reject	TYPE MBS-ServiceArea								PRESENCE optional		}|

{ ID id-MBSSessionModificationRequestTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBSSessionSetupOrModRequestTransfer)		PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- BROADCAST SESSION MODIFICATION RESPONSE

--

-- **************************************************************

BroadcastSessionModificationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionModificationResponseIEs} },

...

}

BroadcastSessionModificationResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID							CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-MBSSessionModificationResponseTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBSSessionSetupOrModResponseTransfer)	PRESENCE optional 	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- BROADCAST SESSION MODIFICATION  FAILURE

--

-- **************************************************************

BroadcastSessionModificationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionModificationFailureIEs} },

...

}

BroadcastSessionModificationFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID							CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-MBSSessionModificationFailureTransfer	CRITICALITY ignore	TYPE OCTET STRING (CONTAINING MBSSessionSetupOrModFailureTransfer)	PRESENCE optional	}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Broadcast Session Release Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- BROADCAST SESSION RELEASE REQUEST

--

-- **************************************************************

BroadcastSessionReleaseRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionReleaseRequestIEs} },

...

}

BroadcastSessionReleaseRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID							CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- Broadcast Session Release Required Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- BROADCAST SESSION RELEASE REQUIRED

--

-- **************************************************************

BroadcastSessionReleaseRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionReleaseRequiredIEs} },

...

}

BroadcastSessionReleaseRequiredIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID							CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-Cause									CRITICALITY ignore	TYPE Cause											PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- BROADCAST SESSION RELEASE RESPONSE

--

-- **************************************************************

BroadcastSessionReleaseResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionReleaseResponseIEs} },

...

}

BroadcastSessionReleaseResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID							CRITICALITY reject	TYPE MBS-SessionID								PRESENCE mandatory	}|

{ ID id-MBSSessionReleaseResponseTransfer		CRITICALITY ignore	TYPE OCTET STRING (CONTAINING MBSSessionReleaseResponseTransfer)		PRESENCE optional		}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Broadcast Session Transport Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- BROADCAST SESSION TRANSPORT REQUEST

--

-- **************************************************************

BroadcastSessionTransportRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionTransportRequestIEs} },

...

}

BroadcastSessionTransportRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID						CRITICALITY reject	TYPE MBS-SessionID				PRESENCE mandatory	}|

{ ID id-BroadcastTransportRequestTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING BroadcastTransportRequestTransfer)		PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- BROADCAST SESSION TRANSPORT RESPONSE

--

-- **************************************************************

BroadcastSessionTransportResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionTransportResponseIEs} },

...

}

BroadcastSessionTransportResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID						CRITICALITY reject	TYPE MBS-SessionID				PRESENCE mandatory	}|

{ ID id-BroadcastTransportResponseTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING BroadcastTransportResponseTransfer)	PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- BROADCAST SESSION TRANSPORT FAILURE

--

-- **************************************************************

BroadcastSessionTransportFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionTransportFailureIEs} },

...

}

BroadcastSessionTransportFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID						CRITICALITY reject	TYPE MBS-SessionID				PRESENCE mandatory	}|

{ ID id-BroadcastTransportFailureTransfer	CRITICALITY ignore	TYPE OCTET STRING (CONTAINING BroadcastTransportFailureTransfer)	PRESENCE 	mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Distribution Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- DISTRIBUTION SETUP REQUEST

--

-- **************************************************************

DistributionSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DistributionSetupRequestIEs} },

...

}

DistributionSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-MBS-DistributionSetupRequestTransfer		CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBS-DistributionSetupRequestTransfer)			PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- DISTRIBUTION SETUP RESPONSE

--

-- **************************************************************

DistributionSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DistributionSetupResponseIEs} },

...

}

DistributionSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-MBS-DistributionSetupResponseTransfer		CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBS-DistributionSetupResponseTransfer)			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- DISTRIBUTION SETUP FAILURE

--

-- **************************************************************

DistributionSetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DistributionSetupFailureIEs} },

...

}

DistributionSetupFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-MBS-DistributionSetupUnsuccessfulTransfer	CRITICALITY ignore	TYPE OCTET STRING (CONTAINING MBS-DistributionSetupUnsuccessfulTransfer)	PRESENCE mandatory	}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause													PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Distribution Release Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- DISTRIBUTION RELEASE REQUEST

--

-- **************************************************************

DistributionReleaseRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DistributionReleaseRequestIEs} },

...

}

DistributionReleaseRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-MBS-DistributionReleaseRequestTransfer		CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBS-DistributionReleaseRequestTransfer)		PRESENCE mandatory	}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause													PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- DISTRIBUTION RELEASE RESPONSE

--

-- **************************************************************

DistributionReleaseResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DistributionReleaseResponseIEs} },

...

}

DistributionReleaseResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Multicast Session Activation Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- MULTICAST SESSION ACTIVATION REQUEST

--

-- **************************************************************

MulticastSessionActivationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastSessionActivationRequestIEs} },

...

}

MulticastSessionActivationRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MulticastSessionActivationRequestTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING MulticastSessionActivationRequestTransfer)	PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- MULTICAST SESSION ACTIVATION RESPONSE

--

-- **************************************************************

MulticastSessionActivationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastSessionActivationResponseIEs} },

...

}

MulticastSessionActivationResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- MULTICAST SESSION ACTIVATION FAILURE

--

-- **************************************************************

MulticastSessionActivationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastSessionActivationFailureIEs} },

...

}

MulticastSessionActivationFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID									CRITICALITY reject	TYPE MBS-SessionID										PRESENCE mandatory	}|

{ ID id-Cause											CRITICALITY ignore	TYPE Cause													PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics							CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Multicast Session Deactivation Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- MULTICAST SESSION DEACTIVATION REQUEST

--

-- **************************************************************

MulticastSessionDeactivationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastSessionDeactivationRequestIEs} },

...

}

MulticastSessionDeactivationRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MulticastSessionDeactivationRequestTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING MulticastSessionDeactivationRequestTransfer)	PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- MULTICAST SESSION DEACTIVATION RESPONSE

--

-- **************************************************************

MulticastSessionDeactivationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastSessionDeactivationResponseIEs} },

...

}

MulticastSessionDeactivationResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID									CRITICALITY reject	TYPE MBS-SessionID										PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics							CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Multicast Session Update Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- MULTICAST SESSION UPDATE REQUEST

--

-- **************************************************************

MulticastSessionUpdateRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastSessionUpdateRequestIEs} },

...

}

MulticastSessionUpdateRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-MulticastSessionUpdateRequestTransfer		CRITICALITY reject	TYPE OCTET STRING (CONTAINING MulticastSessionUpdateRequestTransfer)			PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- MULTICAST SESSION UPDATE RESPONSE

--

-- **************************************************************

MulticastSessionUpdateResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastSessionUpdateResponseIEs} },

...

}

MulticastSessionUpdateResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- MULTICAST SESSION UPDATE FAILURE

--

-- **************************************************************

MulticastSessionUpdateFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastSessionUpdateFailureIEs} },

...

}

MulticastSessionUpdateFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID									CRITICALITY reject	TYPE MBS-SessionID										PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID								CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-Cause											CRITICALITY ignore	TYPE Cause													PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics							CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- MULTICAST GROUP PAGING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

###### -- MULTICAST GROUP PAGING

--

-- **************************************************************

MulticastGroupPaging ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastGroupPagingIEs} },

...

}

MulticastGroupPagingIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID							CRITICALITY ignore	TYPE MBS-SessionID						PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea							CRITICALITY ignore	TYPE MBS-ServiceArea					PRESENCE optional		}|

{ ID id-MulticastGroupPagingAreaList			CRITICALITY ignore	TYPE MulticastGroupPagingAreaList	PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- TIMING SYNCHRONISATION STATUS REPORTING ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- Timing Synchronisation Status Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- TIMING SYNCHRONISATION STATUS REQUEST

--

-- **************************************************************

TimingSynchronisationStatusRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TimingSynchronisationStatusRequestIEs} },

...

}

TimingSynchronisationStatusRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RoutingID				CRITICALITY reject	TYPE RoutingID			PRESENCE mandatory	}|

{ ID id-RAN-TSSRequestType		CRITICALITY reject	TYPE RAN-TSSRequestType	PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- TIMING SYNCHRONISATION STATUS RESPONSE

--

-- **************************************************************

TimingSynchronisationStatusResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TimingSynchronisationStatusResponseIEs} },

...

}

TimingSynchronisationStatusResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RoutingID					CRITICALITY ignore	TYPE RoutingID					PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- TIMING SYNCHRONISATION STATUS FAILURE

--

-- **************************************************************

TimingSynchronisationStatusFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { TimingSynchronisationStatusFailureIEs} },

...

}

TimingSynchronisationStatusFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RoutingID							CRITICALITY ignore	TYPE RoutingID					PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Timing Synchronisation Status Reporting Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- TIMING SYNCHRONISATION STATUS REPORT

--

-- **************************************************************

TimingSynchronisationStatusReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {TimingSynchronisationStatusReportIEs} },

...

}

TimingSynchronisationStatusReportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-RoutingID							CRITICALITY ignore	TYPE RoutingID												PRESENCE mandatory	}|

{ ID id-RANTimingSynchronisationStatusInfo	CRITICALITY ignore	TYPE RANTimingSynchronisationStatusInfo				PRESENCE mandatory	}|

{ ID id-RAN-TSSScope							CRITICALITY ignore	TYPE RAN-TSSScope											PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- MT Communication Handling Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- MT COMMUNICATION HANDLING REQUEST

--

-- **************************************************************

MTCommunicationHandlingRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { MTCommunicationHandlingRequestIEs} },

...

}

MTCommunicationHandlingRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID					CRITICALITY reject	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID					CRITICALITY reject	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-FiveGCAction					CRITICALITY reject	TYPE FiveGCAction					PRESENCE mandatory	}|

{ ID id-PDUSessionListMTCommHReq		CRITICALITY reject	TYPE PDUSessionListMTCommHReq		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- MT COMMUNICATION HANDLING RESPONSE

--

-- **************************************************************

MTCommunicationHandlingResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MTCommunicationHandlingResponseIEs} },

...

}

MTCommunicationHandlingResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID							CRITICALITY reject	TYPE AMF-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID							CRITICALITY reject	TYPE RAN-UE-NGAP-ID										PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- MT COMMUNICATION HANDLING FAILURE

--

-- **************************************************************

MTCommunicationHandlingFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { MTCommunicationHandlingFailureIEs} },

...

}

MTCommunicationHandlingFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID				CRITICALITY reject	TYPE AMF-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID				CRITICALITY reject	TYPE RAN-UE-NGAP-ID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

#### -- RAN Paging Request procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- RAN PAGING REQUEST

--

-- **************************************************************

RANPagingRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {  RANPagingRequestIEs} },

...

}

RANPagingRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID					CRITICALITY ignore	TYPE AMF-UE-NGAP-ID						PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID					CRITICALITY ignore	TYPE RAN-UE-NGAP-ID						PRESENCE mandatory	}|

{ ID id-PagingPolicyDifferentiation		CRITICALITY ignore	TYPE PagingPolicyDifferentiation		PRESENCE optional		}|

{ ID id-DL-Signalling					CRITICALITY ignore	TYPE DL-Signalling						PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- A-IoT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- Inventory Request Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- INVENTORY REQUEST

--

-- **************************************************************

InventoryRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {InventoryRequestIEs} },

...

}

InventoryRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIOTFIdentifier				CRITICALITY reject	TYPE AIOTFIdentifier												PRESENCE mandatory	}|

{ ID id-AIoT-CorrelationIdentifier	CRITICALITY reject	TYPE AIoT-CorrelationIdentifier										PRESENCE mandatory	}|

{ ID id-InventoryRequestTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING InventoryRequestTransfer)		PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- INVENTORY RESPONSE

--

-- **************************************************************

InventoryResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {InventoryResponseIEs} },

...

}

InventoryResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIOTFIdentifier				CRITICALITY reject	TYPE AIOTFIdentifier												PRESENCE mandatory	}|

{ ID id-AIoT-CorrelationIdentifier	CRITICALITY reject	TYPE AIoT-CorrelationIdentifier										PRESENCE mandatory	}|

{ ID id-InventoryResponseTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING InventoryResponseTransfer)	PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics											PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- INVENTORY FAILURE

--

-- **************************************************************

InventoryFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {InventoryFailureIEs} },

...

}

InventoryFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIOTFIdentifier				CRITICALITY reject	TYPE AIOTFIdentifier												PRESENCE mandatory	}|

{ ID id-AIoT-CorrelationIdentifier	CRITICALITY reject	TYPE AIoT-CorrelationIdentifier										PRESENCE mandatory	}|

{ ID id-InventoryFailureTransfer	CRITICALITY ignore	TYPE OCTET STRING (CONTAINING InventoryFailureTransfer	)	PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause															PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics											PRESENCE optional		},

...

}

-- **************************************************************

##### -- Inventory Report procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- INVENTORY REPORT

--

-- **************************************************************

InventoryReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {  InventoryReportIEs} },

...

}

InventoryReportIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIOTFIdentifier				CRITICALITY reject	TYPE AIOTFIdentifier												PRESENCE mandatory	}|

{ ID id-AIoT-CorrelationIdentifier	CRITICALITY reject	TYPE AIoT-CorrelationIdentifier										PRESENCE mandatory	}|

{ ID id-InventoryReportTransfer		CRITICALITY reject	TYPE OCTET STRING (CONTAINING InventoryReportTransfer	)	PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- Command Request Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

###### -- COMMAND REQUEST

--

-- **************************************************************

CommandRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {CommandRequestIEs} },

...

}

CommandRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIOTFIdentifier				CRITICALITY reject	TYPE AIOTFIdentifier												PRESENCE mandatory	}|

{ ID id-AIoT-CorrelationIdentifier	CRITICALITY reject	TYPE AIoT-CorrelationIdentifier										PRESENCE mandatory	}|

{ ID id-CommandRequestTransfer		CRITICALITY reject	TYPE OCTET STRING (CONTAINING CommandRequestTransfer)		PRESENCE mandatory	},

...

}

-- **************************************************************

--

###### -- COMMAND RESPONSE

--

-- **************************************************************

CommandResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {CommandResponseIEs} },

...

}

CommandResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIOTFIdentifier				CRITICALITY reject	TYPE AIOTFIdentifier												PRESENCE mandatory	}|

{ ID id-AIoT-CorrelationIdentifier	CRITICALITY reject	TYPE AIoT-CorrelationIdentifier										PRESENCE mandatory	}|

{ ID id-CommandResponseTransfer		CRITICALITY reject	TYPE OCTET STRING (CONTAINING CommandResponseTransfer)		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics											PRESENCE optional		},

...

}

-- **************************************************************

--

###### -- COMMAND FAILURE

--

-- **************************************************************

CommandFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {CommandFailureIEs} },

...

}

CommandFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIOTFIdentifier				CRITICALITY reject	TYPE AIOTFIdentifier												PRESENCE mandatory	}|

{ ID id-AIoT-CorrelationIdentifier	CRITICALITY reject	TYPE AIoT-CorrelationIdentifier										PRESENCE mandatory	}|

{ ID id-CommandFailureTransfer		CRITICALITY ignore	TYPE OCTET STRING (CONTAINING CommandFailureTransfer	)		PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause															PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics											PRESENCE optional		},

...

}

END

-- ASN1STOP

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AIOTFIdentifier,

id-AIoT-CorrelationIdentifier,

id-AIoT-DeviceIdentificationRequested,

id-AIoT-RequestedServiceAreaInformation,

id-AIoT-InventoryAssistanceInformation,

id-AIoT-FollowonCommandIndication,

id-RAN-AIOT-Device-NGAP-ID,

id-AIoT-CommandAssistanceInformation,

id-AIoT-NASPDU,

id-AlternativeQoSParaSetList,

id-AssistanceInformationQoE-Meas,

id-AdditionalCancelledlocationReportingReferenceIDList,

id-BurstArrivalTimeDownlink,

id-Cause,

id-CNPacketDelayBudgetDL,

id-CNPacketDelayBudgetUL,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-CommonNetworkInstance,

id-ConfiguredTACIndication,

id-CN-MT-CommunicationHandling,

id-CurrentQoSParaSetIndex,

id-DAPSRequestInfo,

id-DAPSResponseInfoList,

id-DataForwardingNotPossible,

id-DataForwardingResponseERABList,

id-DirectForwardingPathAvailability,

id-DL-NGU-UP-TNLInformation,

id-DownlinkTLContainer,

id-EndpointIPAddressAndPort,

id-EnergySavingIndication,

id-ExtendedMobilityInformation,

id-ExtendedPacketDelayBudget,

id-ExtendedRATRestrictionInformation,

id-ExtendedReportIntervalMDT,

id-ExtendedSliceSupportList,

id-ExtendedTAISliceSupportList,

id-ExtendedUEIdentityIndexValue,

id-EUTRA-PagingeDRXInformation,

id-EquivalentSNPNsList,

id-GlobalCable-ID,

id-GlobalRANNodeID,

id-GlobalTNGF-ID,

id-GlobalTWIF-ID,

id-GlobalW-AGF-ID,

id-GUAMIType,

id-HashedUEIdentityIndexValue,

id-IncludeBeamMeasurementsIndication,

id-IntersystemSONInformationRequest,

id-IntersystemSONInformationReply,

id-IntersystemResourceStatusUpdate,

id-IntersystemMobilityFailureforVoiceFallback,

id-LastEUTRAN-PLMNIdentity,

id-LastVisitedPSCellList,

id-LocationReportingAdditionalInfo,

id-M4ReportAmount,

id-M5ReportAmount,

id-M6ReportAmount,

id-ExcessPacketDelayThresholdConfiguration,

id-M7ReportAmount,

id-MaximumIntegrityProtectedDataRate-DL,

id-MBS-AreaSessionID,

id-MBS-QoSFlowsToBeSetupList,

id-MBS-QoSFlowsToBeSetupModList,

id-MBS-QoSFlowToReleaseList,

id-MBS-ServiceArea,

id-MBS-SessionFSAIDList,

id-MBS-SessionID,

id-MBS-ActiveSessionInformation-SourcetoTargetList,

id-MBS-ActiveSessionInformation-TargettoSourceList,

id-MBS-AssistanceInformation,

id-MBS-SessionTNLInfo5GC,

id-MBS-SupportIndicator,

id-MBSSessionFailedtoSetupList,

id-MBSSessionFailedtoSetuporModifyList,

id-MBSSessionSetupResponseList,

id-MBSSessionSetuporModifyResponseList,

id-MBSSessionToReleaseList,

id-MBSSessionSetupRequestList,

id-MBSSessionSetuporModifyRequestList,

id-MDTConfiguration,

id-MicoAllPLMN,

id-NetworkInstance,

id-NGAPIESupportInformationRequestList,

id-NGAPIESupportInformationResponseList,

id-NID,

id-NR-CGI,

id-NRNTNTAIInformation,

id-NPN-MobilityInformation,

id-NPN-PagingAssistanceInformation,

id-NPN-Support,

id-NR-PagingeDRXInformation,

id-OldAssociatedQosFlowList-ULendmarkerexpected,

id-OnboardingSupport,

id-PagingAssisDataforCEcapabUE,

id-PagingCauseIndicationForVoiceService,

id-PDUSessionAggregateMaximumBitRate,

id-PduSessionExpectedUEActivityBehaviour,

id-PDUSessionPairID,

id-PDUSessionResourceFailedToSetupListCxtFail,

id-PDUSessionResourceReleaseResponseTransfer,

id-PDUSessionType,

id-PEIPSassistanceInformation,

id-PSCellInformation,

id-PNI-NPNBasedMDT,

id-PNI-NPN-AreaScopeofMDT,

id-QMCConfigInfo,

id-QosFlowAdditionalInfoList,

id-QosFlowAddOrModifyRequestList,

id-QosFlowFailedToSetupList,

id-QosFlowFeedbackList,

id-QosFlowParametersList,

id-QosFlowSetupRequestList,

id-QosFlowToReleaseList,

id-QosMonitoringRequest,

id-QosMonitoringReportingFrequency,

id-SNPN-CellBasedMDT,

id-SNPN-TAIBasedMDT,

id-SNPN-BasedMDT,

id-SuccessfulHandoverReportList,

id-SupportedUETypeList,

id-UEContextReferenceAtSource,

id-RAT-Information,

id-RedundantCommonNetworkInstance,

id-RedundantDL-NGU-TNLInformationReused,

id-RedundantDL-NGU-UP-TNLInformation,

id-RedundantDLQosFlowPerTNLInformation,

id-RedundantPDUSessionInformation,

id-RedundantQosFlowIndicator,

id-RedundantUL-NGU-UP-TNLInformation,

id-SCTP-TLAs,

id-SecondaryRATUsageInformation,

id-SecurityIndication,

id-SecurityResult,

id-SgNB-UE-X2AP-ID,

id-S-NSSAI,

id-SONInformationReport,

id-SourceNodeID,

id-SourceNodeTNLAddrInfo,

id-SourceSN-to-TargetSN-QMCInfo,

id-SourceTNLAddrInfo,

id-SurvivalTime,

id-Selected-Target-SNPN-Identity,

id-TNLAssociationTransportLayerAddressNGRAN,

id-TAINSAGSupportList,

id-TargetHomeENB-ID,

id-TargetRNC-ID,

id-TimeBasedHandoverInformation,

id-TraceCollectionEntityURI,

id-TSCTrafficCharacteristics,

id-UEHistoryInformationFromTheUE,

id-UERadioCapabilityForPaging,

id-UERadioCapabilityForPagingOfNB-IoT,

id-UL-NGU-UP-TNLInformation,

id-UL-NGU-UP-TNLModifyList,

id-ULForwarding,

id-ULForwardingUP-TNLInformation,

id-UplinkTLContainer,

id-UsedRSNInformation,

id-UserLocationInformationTNGF,

id-UserLocationInformationTWIF,

id-UserLocationInformationW-AGF,

id-UserPlaneErrorIndicator,

id-EarlyMeasurement,

id-BeamMeasurementsReportConfiguration,

id-DLDiscarding,

id-TAI,

id-HFCNode-ID-new,

id-GlobalCable-ID-new,

id-FiveGProSeLayer2Multipath,

id-CandidateRelayUEInformationList,

id-FiveGProSeLayer2UEtoUERelay,

id-FiveGProSeLayer2UEtoUERemote,

id-SuccessfulPSCellChangeReportList,

id-TargetCellCRNTI,

id-TimeSinceFailure,

id-ClockQualityReportingControlInfo,

id-RANfeedbacktype,

id-QoSFlowTSCList,

id-TSCTrafficCharacteristicsFeedback,

id-ANPacketDelayBudgetUL,

id-MBSCommServiceType,

id-MobileIAB-MTUserLocationInformation,

id-PDUsetQoSParameters,

id-PDUSetbasedHandlingIndicator,

id-N6JitterInformation,

id-ECNMarkingorCongestionInformationReportingRequest,

id-ECNMarkingorCongestionInformationReportingStatus,

id-MN-only-MDT-collection,

id-XrDeviceWith2Rx,

id-MaximumDataBurstVolume,

id-MBS-NGUFailureIndication,

id-UserPlaneFailureIndication,

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-UserLocationInformationN3IWF-without-PortNumber,

maxnoofAIoTAreas,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

maxnoofAoIMinusOne,

maxnoofBluetoothName,

maxnoofBPLMNs,

maxnoofCAGforMDT,

maxnoofCAGSperCell,

maxnoofCandidateCells,

maxnoofCellIDforMDT,

maxnoofCellIDforQMC,

maxnoofCellIDforWarning,

maxnoofCellinAoI,

maxnoofCellinEAI,

maxnoofCellsforMBS,

maxnoofCellsingNB,

maxnoofCellsinngeNB,

maxnoofCellsinNGRANNode,

maxnoofCellinTAI,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDevices,

maxnoofDRBs,

maxnoofEmergencyAreaID,

maxnoofEAIforRestart,

maxnoofEPLMNs,

maxnoofEPLMNsPlusOne,

maxnoofE-RABs,

maxnoofErrors,

maxnoofExtSliceItems,

maxnoofESNPNs,

maxnoofForbTACs,

maxnoofFreqforMDT,

maxnoofMBSFSAs,

maxnoofMBSQoSFlows,

maxnoofMBSServiceAreaInformation,

maxnoofMBSAreaSessionIDs,

maxnoofMBSSessions,

maxnoofMBSSessionsofUE,

maxnoofMDTPLMNs,

maxnoofMRBs,

maxnoofMultiConnectivity,

maxnoofMultiConnectivityMinusOne,

maxnoofNeighPCIforMDT,

maxnoofNGAPIESupportInfo,

maxnoofNGConnectionsToReset,

maxNRARFCN,

maxnoofNRCellBands,

maxnoofNSAGs,

maxnoofPagingAreas,

maxnoofPC5QoSFlows,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofPLMNforQMC,

maxnoofQosFlows,

maxnoofQosParaSets,

maxnoofRANNodeinAoI,

maxnoofReaders,

maxnoofRecommendedCells,

maxnoofRecommendedRANNodes,

maxnoofAoI,

maxnoofPSCellsPerPrimaryCellinUEHistoryInfo,

maxnoofReportedCells,

maxnoofSensorName,

maxnoofServedGUAMIs,

maxnoofSliceItems,

maxnoofMDTSNPNs,

maxnoofSNSSAIforQMC,

maxnoofSuccessfulHOReports,

maxnoofTACs,

maxnoofTACsinNTN,

maxnoofTAforMDT,

maxnoofTAforQMC,

maxnoofTAIforInactive,

maxnoofTAIforMBS,

maxnoofTAIforPaging,

maxnoofTAIforRestart,

maxnoofTAIforWarning,

maxnoofTAIinAoI,

maxnoofTargetS-NSSAIs,

maxnoofTimePeriods,

maxnoofTNLAssociations,

maxnoofUEAppLayerMeas,

maxnoofUEsforPaging,

maxnoofUETypes,

maxnoofWLANName,

maxnoofXnExtTLAs,

maxnoofXnGTP-TLAs,

maxnoofXnTLAs,

maxnoofThresholdsForExcessPacketDelay,

maxnoofCandidateRelayUEs,

maxnoofSuccessfulPSCellChangeReports,

maxnoofCellsTSS,

maxnoofPeriodicities,

maxnoofPartiallyAllowedS-NSSAIs,

maxnoofRSPPQoSFlows

FROM NGAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM NGAP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Container{},

NGAP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

NGAP-PROTOCOL-IES

FROM NGAP-Containers;

#### -- A

AdditionalDLUPTNLInformationForHOList ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF AdditionalDLUPTNLInformationForHOItem

AdditionalDLUPTNLInformationForHOItem ::= SEQUENCE {

additionalDL-NGU-UP-TNLInformation				UPTransportLayerInformation,

additionalQosFlowSetupResponseList				QosFlowListWithDataForwarding,

additionalDLForwardingUPTNLInformation			UPTransportLayerInformation 							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AdditionalDLUPTNLInformationForHOItem-ExtIEs} }	OPTIONAL,

...

}

AdditionalDLUPTNLInformationForHOItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalRedundantDL-NGU-UP-TNLInformation	CRITICALITY ignore	EXTENSION UPTransportLayerInformation		PRESENCE optional		},

...

}

AdditionalQosFlowInformation ::= ENUMERATED {

more-likely,

...

}

AerialUEsubscriptionInformation ::= ENUMERATED {

allowed,

not-allowed,

...

}

A2X-PC5-FlowBitRates ::= SEQUENCE {

a2X-GuaranteedFlowBitRate		BitRate,

a2X-MaximumFlowBitRate			BitRate,

iE-Extensions		ProtocolExtensionContainer { {A2X-PC5-FlowBitRates-ExtIEs} }	OPTIONAL,

...

}

A2X-PC5-FlowBitRates-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

A2X-PC5-QoS-Flow-List ::= SEQUENCE (SIZE(1..maxnoofPC5QoSFlows)) OF A2X-PC5-QoS-Flow-Item

A2X-PC5-QoS-Flow-Item ::= SEQUENCE {

a2X-PQI								INTEGER (0..255, ...),

a2X-PC5-FlowBitRates				A2X-PC5-FlowBitRates							OPTIONAL,

a2X-Range							ENUMERATED {m50, m80, m180, m200,

m350, m400, m500, m700,

m1000, ...}							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {A2X-PC5-QoS-Flow-Item-ExtIEs} }	OPTIONAL,

...

}

A2X-PC5-QoS-Flow-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

A2X-PC5-QoS-Parameters ::= SEQUENCE {

a2X-PC5-QoS-Flow-List				A2X-PC5-QoS-Flow-List,

a2X-PC5-LinkAggregateBitRates		BitRate											OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {A2X-PC5-QoS-Parameters-ExtIEs} }	OPTIONAL,

...

}

A2X-PC5-QoS-Parameters-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalCancelledlocationReportingReferenceIDList ::= SEQUENCE (SIZE(1.. maxnoofAoIMinusOne)) OF AdditionalCancelledlocationReportingReferenceIDItem

AdditionalCancelledlocationReportingReferenceIDItem ::= SEQUENCE {

locationReportingReferenceIDToBeCancelled		LocationReportingReferenceID,

iE-Extensions		ProtocolExtensionContainer { { AdditionalCancelledlocationReportingReferenceIDItem-ExtIEs} }	OPTIONAL,

...

}

AdditionalCancelledlocationReportingReferenceIDItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoTArea ::= OCTET STRING	--FFS--

AIoT-CorrelationIdentifier ::= OCTET STRING	--FFS--

AIoT-FollowonCommandIndication ::= ENUMERATED {true, ...}

RAN-AIOT-Device-NGAP-ID ::= INTEGER (0..4294967295)

AIoT-CommandAssistanceInformation ::= SEQUENCE {

estimateofExpectedD2RMsgSize		INTEGER (1..256, ...)					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-CommandAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

AIoT-CommandAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-DeviceIdentificationRequested  ::= OCTET STRING	--FFS--

AIoT-DeviceReportList ::= SEQUENCE (SIZE(1..maxnoofDevices)) OF AIoT-DeviceReportItem

AIoT-DeviceReportItem ::= SEQUENCE {

aIoT-NASPDU					AIoT-NASPDU,

rAN-AIOT-Device-NGAP-ID		RAN-AIOT-Device-NGAP-ID									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-DeviceReportItem-ExtIEs} }	OPTIONAL,

...

}

AIoT-DeviceReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIOTFIdentifier ::= OCTET STRING	--FFS--

AIoT-InventoryAssistanceInformation ::= SEQUENCE {

approximateNoofTargetDevices				INTEGER (1..2048, ...)				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-InventoryAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

AIoT-InventoryAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-NASPDU  ::= OCTET STRING

AIoT-ReaderReportList ::= SEQUENCE (SIZE(1..maxnoofReaders)) OF AIoT-ReaderReportItem

AIoT-ReaderReportItem ::= SEQUENCE {

readerIndex					AIoT-ReaderIndex,

deviceReportList			AIoT-DeviceReportList,

iE-Extensions		ProtocolExtensionContainer { { AIoT-ReaderReportItem-ExtIEs} } OPTIONAL,

...

}

AIoT-ReaderReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-ReaderIndex ::= INTEGER (1..65535, ...)

AIoT-RequestedReaderList ::= SEQUENCE (SIZE(1..maxnoofReaders)) OF AIoT-RequestedReaderItem

AIoT-RequestedReaderItem ::= SEQUENCE {

globalgNB-ID				GlobalGNB-ID,

readerIndex					AIoT-ReaderIndex,

iE-Extensions		ProtocolExtensionContainer { { AIoT-RequestedReaderItem-ExtIEs} } OPTIONAL,

...

}

AIoT-RequestedReaderItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-RequestedAIoTAreaList ::= SEQUENCE (SIZE(1..maxnoofAIoTAreas)) OF AIoTArea

AIoT-RequestedServiceAreaInformation ::= SEQUENCE {

requestedReaderList				AIoT-RequestedReaderList					OPTIONAL,

requestedAIoTAreaList			AIoT-RequestedAIoTAreaList					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-RequestedServiceAreaInformation-ExtIEs} } OPTIONAL,

...

}

AIoT-RequestedServiceAreaInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevelARP				PriorityLevelARP,

pre-emptionCapability			Pre-emptionCapability,

pre-emptionVulnerability		Pre-emptionVulnerability,

iE-Extensions		ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationAndRetentionPriority-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

Allowed-CAG-List-per-PLMN ::= SEQUENCE (SIZE(1..maxnoofAllowedCAGsperPLMN)) OF CAG-ID

AllowedNSSAI ::= SEQUENCE (SIZE(1..maxnoofAllowedS-NSSAIs)) OF AllowedNSSAI-Item

AllowedNSSAI-Item ::= SEQUENCE {

s-NSSAI				S-NSSAI,

iE-Extensions		ProtocolExtensionContainer { {AllowedNSSAI-Item-ExtIEs} } OPTIONAL,

...

}

AllowedNSSAI-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

Allowed-PNI-NPN-List ::= SEQUENCE (SIZE(1..maxnoofEPLMNsPlusOne)) OF Allowed-PNI-NPN-Item

Allowed-PNI-NPN-Item ::= SEQUENCE {

pLMNIdentity				PLMNIdentity,

pNI-NPN-restricted			ENUMERATED {restricted, not-restricted, ...},

allowed-CAG-List-per-PLMN	Allowed-CAG-List-per-PLMN,

iE-Extensions			ProtocolExtensionContainer { {Allowed-PNI-NPN-Item-ExtIEs} } OPTIONAL,

...

}

Allowed-PNI-NPN-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AllowedTACs ::= SEQUENCE (SIZE(1..maxnoofAllowedAreas)) OF TAC

AlternativeQoSParaSetIndex ::= INTEGER (1..8, ...)

AlternativeQoSParaSetNotifyIndex ::= INTEGER (0..8, ...)

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQosParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParaSetIndex			AlternativeQoSParaSetIndex,

guaranteedFlowBitRateDL				BitRate												OPTIONAL,

guaranteedFlowBitRateUL				BitRate												OPTIONAL,

packetDelayBudget					PacketDelayBudget									OPTIONAL,

packetErrorRate						PacketErrorRate										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MaximumDataBurstVolume	CRITICALITY ignore	EXTENSION MaximumDataBurstVolume		PRESENCE optional		},

...

}

AssistanceInformationQoE-Meas ::= INTEGER (1..16, ...)

AMFName ::= PrintableString (SIZE(1..150, ...))

AMFNameVisibleString ::= VisibleString (SIZE(1..150, ...))

AMFNameUTF8String ::= UTF8String (SIZE(1..150, ...))

AMFPagingTarget ::= CHOICE {

globalRANNodeID			GlobalRANNodeID,

tAI						TAI,

choice-Extensions		ProtocolIE-SingleContainer { {AMFPagingTarget-ExtIEs} }

}

AMFPagingTarget-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

AMFPointer ::= BIT STRING (SIZE(6))

AMFRegionID ::= BIT STRING (SIZE(8))

AMFSetID ::= BIT STRING (SIZE(10))

AMF-TNLAssociationSetupList ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF AMF-TNLAssociationSetupItem

AMF-TNLAssociationSetupItem ::= SEQUENCE {

aMF-TNLAssociationAddress		CPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { {AMF-TNLAssociationSetupItem-ExtIEs} }	OPTIONAL,

...

}

AMF-TNLAssociationSetupItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AMF-TNLAssociationToAddList ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF AMF-TNLAssociationToAddItem

AMF-TNLAssociationToAddItem ::= SEQUENCE {

aMF-TNLAssociationAddress		CPTransportLayerInformation,

tNLAssociationUsage				TNLAssociationUsage										OPTIONAL,

tNLAddressWeightFactor			TNLAddressWeightFactor,

iE-Extensions		ProtocolExtensionContainer { {AMF-TNLAssociationToAddItem-ExtIEs} }	OPTIONAL,

...

}

AMF-TNLAssociationToAddItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AMF-TNLAssociationToRemoveList ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF AMF-TNLAssociationToRemoveItem

AMF-TNLAssociationToRemoveItem ::= SEQUENCE {

aMF-TNLAssociationAddress		CPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { {AMF-TNLAssociationToRemoveItem-ExtIEs} }	OPTIONAL,

...

}

AMF-TNLAssociationToRemoveItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-TNLAssociationTransportLayerAddressNGRAN	CRITICALITY reject	EXTENSION CPTransportLayerInformation	PRESENCE optional},

...

}

AMF-TNLAssociationToUpdateList ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF AMF-TNLAssociationToUpdateItem

AMF-TNLAssociationToUpdateItem ::= SEQUENCE {

aMF-TNLAssociationAddress		CPTransportLayerInformation,

tNLAssociationUsage				TNLAssociationUsage											OPTIONAL,

tNLAddressWeightFactor			TNLAddressWeightFactor										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AMF-TNLAssociationToUpdateItem-ExtIEs} }	OPTIONAL,

...

}

AMF-TNLAssociationToUpdateItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AMF-UE-NGAP-ID ::= INTEGER (0..1099511627775)

AreaOfInterest ::= SEQUENCE {

areaOfInterestTAIList			AreaOfInterestTAIList						OPTIONAL,

areaOfInterestCellList			AreaOfInterestCellList						OPTIONAL,

areaOfInterestRANNodeList		AreaOfInterestRANNodeList					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AreaOfInterest-ExtIEs} }	OPTIONAL,

...

}

AreaOfInterest-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AreaOfInterestCellList ::= SEQUENCE (SIZE(1..maxnoofCellinAoI)) OF AreaOfInterestCellItem

AreaOfInterestCellItem ::= SEQUENCE {

nGRAN-CGI			NGRAN-CGI,

iE-Extensions		ProtocolExtensionContainer { {AreaOfInterestCellItem-ExtIEs} }	OPTIONAL,

...

}

AreaOfInterestCellItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AreaOfInterestList ::= SEQUENCE (SIZE(1..maxnoofAoI)) OF AreaOfInterestItem

AreaOfInterestItem ::= SEQUENCE {

areaOfInterest						AreaOfInterest,

locationReportingReferenceID		LocationReportingReferenceID,

iE-Extensions		ProtocolExtensionContainer { {AreaOfInterestItem-ExtIEs} }	OPTIONAL,

...

}

AreaOfInterestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AreaOfInterestRANNodeList ::= SEQUENCE (SIZE(1..maxnoofRANNodeinAoI)) OF AreaOfInterestRANNodeItem

AreaOfInterestRANNodeItem ::= SEQUENCE {

globalRANNodeID		GlobalRANNodeID,

iE-Extensions		ProtocolExtensionContainer { {AreaOfInterestRANNodeItem-ExtIEs} }	OPTIONAL,

...

}

AreaOfInterestRANNodeItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AreaOfInterestTAIList ::= SEQUENCE (SIZE(1..maxnoofTAIinAoI)) OF AreaOfInterestTAIItem

AreaOfInterestTAIItem ::= SEQUENCE {

tAI					TAI,

iE-Extensions		ProtocolExtensionContainer { {AreaOfInterestTAIItem-ExtIEs} }	OPTIONAL,

...

}

AreaOfInterestTAIItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AssistanceDataForPaging ::= SEQUENCE {

assistanceDataForRecommendedCells		AssistanceDataForRecommendedCells			OPTIONAL,

pagingAttemptInformation				PagingAttemptInformation					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AssistanceDataForPaging-ExtIEs} } OPTIONAL,

...

}

AssistanceDataForPaging-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-NPN-PagingAssistanceInformation		CRITICALITY ignore	EXTENSION NPN-PagingAssistanceInformation			PRESENCE optional	}|

{ ID id-PagingAssisDataforCEcapabUE			CRITICALITY ignore	EXTENSION PagingAssisDataforCEcapabUE				PRESENCE optional	},

...

}

AssistanceDataForRecommendedCells ::= SEQUENCE {

recommendedCellsForPaging		RecommendedCellsForPaging,

iE-Extensions		ProtocolExtensionContainer { {AssistanceDataForRecommendedCells-ExtIEs} }	OPTIONAL,

...

}

AssistanceDataForRecommendedCells-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AssociatedMBSQosFlowSetupRequestList ::= SEQUENCE (SIZE(1..maxnoofMBSQoSFlows)) OF AssociatedMBSQosFlowSetupRequestItem

AssociatedMBSQosFlowSetupRequestItem ::= SEQUENCE {

mBS-QosFlowIdentifier				QosFlowIdentifier,

associatedUnicastQosFlowIdentifier	QosFlowIdentifier,

iE-Extensions						ProtocolExtensionContainer { { AssociatedMBSQosFlowSetupRequestItem-ExtIEs} }	OPTIONAL,

...

}

AssociatedMBSQosFlowSetupRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AssociatedMBSQosFlowSetuporModifyRequestList ::= SEQUENCE (SIZE(1..maxnoofMBSQoSFlows)) OF AssociatedMBSQosFlowSetuporModifyRequestItem

AssociatedMBSQosFlowSetuporModifyRequestItem ::= SEQUENCE {

mBS-QosFlowIdentifier				QosFlowIdentifier,

associatedUnicastQosFlowIdentifier	QosFlowIdentifier,

iE-Extensions						ProtocolExtensionContainer { { AssociatedMBSQosFlowSetuporModifyRequestItem-ExtIEs} }	OPTIONAL,

...

}

AssociatedMBSQosFlowSetuporModifyRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AssociatedQosFlowList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF AssociatedQosFlowItem

AssociatedQosFlowItem ::= SEQUENCE {

qosFlowIdentifier				QosFlowIdentifier,

qosFlowMappingIndication		ENUMERATED {ul, dl, ...}							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AssociatedQosFlowItem-ExtIEs} }	OPTIONAL,

...

}

AssociatedQosFlowItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetIndex	PRESENCE optional	},

...

}

AssociatedSessionID ::= OCTET STRING

AUN3DeviceAccessInfo ::= SEQUENCE {

aUN3DeviceAccess							ENUMERATED {true, ...},

existingNGConnectionwithSameLocation		ENUMERATED {true, false, ...},

iE-Extensions		ProtocolExtensionContainer { { AUN3DeviceAccessInfo-ExtIEs} }	OPTIONAL,

...

}

AUN3DeviceAccessInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AuthenticatedIndication ::= ENUMERATED {true, ...}

AveragingWindow ::= INTEGER (0..4095, ...)

AreaScopeOfMDT-NR ::= CHOICE {

cellBased					CellBasedMDT-NR,

tABased						TABasedMDT,

pLMNWide					NULL,

tAIBased					TAIBasedMDT,

choice-Extensions		ProtocolIE-SingleContainer { {AreaScopeOfMDT-NR-ExtIEs} }

}

AreaScopeOfMDT-NR-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-PNI-NPNBasedMDT		CRITICALITY ignore	TYPE PNI-NPNBasedMDT		PRESENCE mandatory }|

{ ID id-SNPN-CellBasedMDT	CRITICALITY ignore	TYPE SNPN-CellBasedMDT		PRESENCE mandatory }|

{ ID id-SNPN-TAIBasedMDT	CRITICALITY ignore	TYPE SNPN-TAIBasedMDT		PRESENCE mandatory }|

{ ID id-SNPN-BasedMDT		CRITICALITY ignore	TYPE SNPN-BasedMDT			PRESENCE mandatory },

...

}

AreaScopeOfMDT-EUTRA ::= CHOICE {

cellBased					CellBasedMDT-EUTRA,

tABased						TABasedMDT,

pLMNWide					NULL,

tAIBased					TAIBasedMDT,

choice-Extensions		ProtocolIE-SingleContainer { {AreaScopeOfMDT-EUTRA-ExtIEs} }

}

AreaScopeOfMDT-EUTRA-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

AreaScopeOfNeighCellsList ::= SEQUENCE (SIZE(1..maxnoofFreqforMDT)) OF AreaScopeOfNeighCellsItem

AreaScopeOfNeighCellsItem ::= SEQUENCE {

nrFrequencyInfo				NRFrequencyInfo,

pciListForMDT				PCIListForMDT												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AreaScopeOfNeighCellsItem-ExtIEs} }	OPTIONAL,

...

}

AreaScopeOfNeighCellsItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AreaScopeOfQMC ::= CHOICE {

cellBased				CellBasedQMC,

tABased					TABasedQMC,

tAIBased					TAIBasedQMC,

pLMNAreaBased			PLMNAreaBasedQMC,

choice-Extensions		ProtocolIE-SingleContainer { { AreaScopeOfQMC-ExtIEs} }

}

AreaScopeOfQMC-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

AvailableRANVisibleQoEMetrics ::= SEQUENCE {

applicationLayerBufferLevelList		ENUMERATED {true, ...}		OPTIONAL,

playoutDelayForMediaStartup			ENUMERATED {true, ...}		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { AvailableRANVisibleQoEMetrics-ExtIEs} }	OPTIONAL,

...

}

AvailableRANVisibleQoEMetrics-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- B

BeamMeasurementsReportConfiguration ::= SEQUENCE {

beamMeasurementsReportQuantity			BeamMeasurementsReportQuantity 	OPTIONAL,

maxNrofRS-IndexesToReport               MaxNrofRS-IndexesToReport		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { BeamMeasurementsReportConfiguration-ExtIEs} } OPTIONAL,

...

}

BeamMeasurementsReportConfiguration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BeamMeasurementsReportQuantity ::= SEQUENCE {

rSRP						ENUMERATED {true, ...},

rSRQ						ENUMERATED {true, ...},

sINR						ENUMERATED {true, ...},

iE-Extensions			ProtocolExtensionContainer { { BeamMeasurementsReportQuantity-ExtIEs} } OPTIONAL,

...

}

BeamMeasurementsReportQuantity-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BitRate	::= INTEGER (0..4000000000000, ...)

BroadcastCancelledAreaList ::= CHOICE {

cellIDCancelledEUTRA				CellIDCancelledEUTRA,

tAICancelledEUTRA					TAICancelledEUTRA,

emergencyAreaIDCancelledEUTRA		EmergencyAreaIDCancelledEUTRA,

cellIDCancelledNR					CellIDCancelledNR,

tAICancelledNR						TAICancelledNR,

emergencyAreaIDCancelledNR			EmergencyAreaIDCancelledNR,

choice-Extensions		ProtocolIE-SingleContainer { {BroadcastCancelledAreaList-ExtIEs} }

}

BroadcastCancelledAreaList-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

BroadcastCompletedAreaList ::= CHOICE {

cellIDBroadcastEUTRA				CellIDBroadcastEUTRA,

tAIBroadcastEUTRA					TAIBroadcastEUTRA,

emergencyAreaIDBroadcastEUTRA		EmergencyAreaIDBroadcastEUTRA,

cellIDBroadcastNR					CellIDBroadcastNR,

tAIBroadcastNR						TAIBroadcastNR,

emergencyAreaIDBroadcastNR			EmergencyAreaIDBroadcastNR,

choice-Extensions		ProtocolIE-SingleContainer { {BroadcastCompletedAreaList-ExtIEs} }

}

BroadcastCompletedAreaList-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

BroadcastPLMNList ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF BroadcastPLMNItem

BroadcastPLMNItem ::= SEQUENCE {

pLMNIdentity			PLMNIdentity,

tAISliceSupportList		SliceSupportList,

iE-Extensions		ProtocolExtensionContainer { {BroadcastPLMNItem-ExtIEs} } OPTIONAL,

...

}

BroadcastPLMNItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-NPN-Support					CRITICALITY reject	EXTENSION NPN-Support				PRESENCE optional}|

{ID id-ExtendedTAISliceSupportList	CRITICALITY reject	EXTENSION ExtendedSliceSupportList 	PRESENCE optional}|

{ID id-TAINSAGSupportList			CRITICALITY ignore	EXTENSION TAINSAGSupportList		PRESENCE optional},

...

}

BroadcastTransportFailureTransfer ::= SEQUENCE {

mBS-SessionID 				MBS-SessionID,

cause						Cause,

criticalityDiagnostics		CriticalityDiagnostics															OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { BroadcastTransportFailureTransfer-ExtIEs} }	OPTIONAL,

...

}

BroadcastTransportFailureTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastTransportRequestTransfer ::= SEQUENCE {

mBS-SessionID 				MBS-SessionID,

mBS-SessionTNLInfoNGRAN 	MBS-SessionTNLInfoNGRAN,

iE-Extensions				ProtocolExtensionContainer { {BroadcastTransportRequestTransfer-ExtIEs} }		OPTIONAL,

...

}

BroadcastTransportRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastTransportResponseTransfer ::= SEQUENCE {

mBS-SessionID 			MBS-SessionID,

iE-Extensions			ProtocolExtensionContainer { {BroadcastTransportResponseTransfer-ExtIEs} }		OPTIONAL,

...

}

BroadcastTransportResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BluetoothMeasurementConfiguration ::= SEQUENCE {

bluetoothMeasConfig				BluetoothMeasConfig,

bluetoothMeasConfigNameList		BluetoothMeasConfigNameList											OPTIONAL,

bt-rssi							ENUMERATED {true, ...}												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { BluetoothMeasurementConfiguration-ExtIEs } } 	OPTIONAL,

...

}

BluetoothMeasurementConfiguration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BluetoothMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofBluetoothName)) OF BluetoothMeasConfigNameItem

BluetoothMeasConfigNameItem ::= SEQUENCE {

bluetoothName		BluetoothName,

iE-Extensions		ProtocolExtensionContainer { { BluetoothMeasConfigNameItem-ExtIEs } } 	OPTIONAL,

...

}

BluetoothMeasConfigNameItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BluetoothMeasConfig::= ENUMERATED {setup,...}

BluetoothName ::= OCTET STRING (SIZE (1..248))

BurstArrivalTime ::= OCTET STRING

BurstArrivalTimeWindow ::= SEQUENCE {

burstArrivalTimeWindowStart				INTEGER (0..640000, ...),

burstArrivalTimeWindowEnd				INTEGER (0..640000, ...),

iE-Extension			ProtocolExtensionContainer { {BurstArrivalTimeWindow-ExtIEs} } OPTIONAL,

...

}

BurstArrivalTimeWindow-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

#### -- C

CAG-ID ::= BIT STRING (SIZE(32))

CandidateRelayUEInformationList ::= SEQUENCE (SIZE(1.. maxnoofCandidateRelayUEs)) OF CandidateRelayUEInformationItem

CandidateRelayUEInformationItem ::= SEQUENCE {

candidateRelayUE-Id		CandidateRelayUE-ID,

iE-Extensions		ProtocolExtensionContainer { { CandidateRelayUEInformationItem-ExtIEs} }	OPTIONAL,

...

}

CandidateRelayUEInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CandidateRelayUE-ID ::= BIT STRING (SIZE(24))

CancelAllWarningMessages ::= ENUMERATED {

true,

...

}

CancelledCellsInEAI-EUTRA ::= SEQUENCE (SIZE(1..maxnoofCellinEAI)) OF CancelledCellsInEAI-EUTRA-Item

CancelledCellsInEAI-EUTRA-Item ::= SEQUENCE {

eUTRA-CGI				EUTRA-CGI,

numberOfBroadcasts		NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { {CancelledCellsInEAI-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

CancelledCellsInEAI-EUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CancelledCellsInEAI-NR ::= SEQUENCE (SIZE(1..maxnoofCellinEAI)) OF CancelledCellsInEAI-NR-Item

CancelledCellsInEAI-NR-Item ::= SEQUENCE {

nR-CGI					NR-CGI,

numberOfBroadcasts		NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { {CancelledCellsInEAI-NR-Item-ExtIEs} } OPTIONAL,

...

}

CancelledCellsInEAI-NR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CancelledCellsInTAI-EUTRA ::= SEQUENCE (SIZE(1..maxnoofCellinTAI)) OF CancelledCellsInTAI-EUTRA-Item

CancelledCellsInTAI-EUTRA-Item ::= SEQUENCE {

eUTRA-CGI				EUTRA-CGI,

numberOfBroadcasts		NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { {CancelledCellsInTAI-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

CancelledCellsInTAI-EUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CancelledCellsInTAI-NR ::= SEQUENCE (SIZE(1..maxnoofCellinTAI)) OF CancelledCellsInTAI-NR-Item

CancelledCellsInTAI-NR-Item ::= SEQUENCE{

nR-CGI					NR-CGI,

numberOfBroadcasts		NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { {CancelledCellsInTAI-NR-Item-ExtIEs} } OPTIONAL,

...

}

CancelledCellsInTAI-NR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CandidateCellList ::= SEQUENCE (SIZE(1.. maxnoofCandidateCells)) OF CandidateCellItem

CandidateCellItem ::= SEQUENCE{

candidateCell		CandidateCell,

iE-Extensions		ProtocolExtensionContainer { {CandidateCellItem-ExtIEs} } OPTIONAL,

...

}

CandidateCellItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CandidateCell::= CHOICE {

candidateCGI			CandidateCellID,

candidatePCI			CandidatePCI,

choice-Extensions		ProtocolIE-SingleContainer { { CandidateCell-ExtIEs} }

}

CandidateCell-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

CandidateCellID::= SEQUENCE {

candidateCellID			NR-CGI,

iE-Extensions			ProtocolExtensionContainer { { CandidateCellID-ExtIEs} }			OPTIONAL,

...

}

CandidateCellID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CandidatePCI::= SEQUENCE {

candidatePCI			INTEGER (0..1007, ...),

candidateNRARFCN		INTEGER (0..maxNRARFCN),

iE-Extensions			ProtocolExtensionContainer { { CandidatePCI-ExtIEs} }			OPTIONAL,

...

}

CandidatePCI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

nas					CauseNas,

protocol			CauseProtocol,

misc				CauseMisc,

choice-Extensions		ProtocolIE-SingleContainer { {Cause-ExtIEs} }

}

Cause-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

not-enough-user-plane-processing-resources,

hardware-failure,

om-intervention,

unknown-PLMN-or-SNPN,

unspecified,

...

}

CauseNas ::= ENUMERATED {

normal-release,

authentication-failure,

deregister,

unspecified,

...,

uE-not-in-PLMN-serving-area,

mobile-IAB-not-authorized,

iAB-not-authorized

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

txnrelocoverall-expiry,

successful-handover,

release-due-to-ngran-generated-reason,

release-due-to-5gc-generated-reason,

handover-cancelled,

partial-handover,

ho-failure-in-target-5GC-ngran-node-or-target-system,

ho-target-not-allowed,

tngrelocoverall-expiry,

tngrelocprep-expiry,

cell-not-available,

unknown-targetID,

no-radio-resources-available-in-target-cell,

unknown-local-UE-NGAP-ID,

inconsistent-remote-UE-NGAP-ID,

handover-desirable-for-radio-reason,

time-critical-handover,

resource-optimisation-handover,

reduce-load-in-serving-cell,

user-inactivity,

radio-connection-with-ue-lost,

radio-resources-not-available,

invalid-qos-combination,

failure-in-radio-interface-procedure,

interaction-with-other-procedure,

unknown-PDU-session-ID,

unkown-qos-flow-ID,

multiple-PDU-session-ID-instances,

multiple-qos-flow-ID-instances,

encryption-and-or-integrity-protection-algorithms-not-supported,

ng-intra-system-handover-triggered,

ng-inter-system-handover-triggered,

xn-handover-triggered,

not-supported-5QI-value,

ue-context-transfer,

ims-voice-eps-fallback-or-rat-fallback-triggered,

up-integrity-protection-not-possible,

up-confidentiality-protection-not-possible,

slice-not-supported,

ue-in-rrc-inactive-state-not-reachable,

redirection,

resources-not-available-for-the-slice,

ue-max-integrity-protected-data-rate-reason,

release-due-to-cn-detected-mobility,

...,

n26-interface-not-available,

release-due-to-pre-emption,

multiple-location-reporting-reference-ID-instances,

rsn-not-available-for-the-up,

npn-access-denied,

cag-only-access-denied,

insufficient-ue-capabilities,

redcap-ue-not-supported,

unknown-MBS-Session-ID,

indicated-MBS-session-area-information-not-served-by-the-gNB,

inconsistent-slice-info-for-the-session,

misaligned-association-for-multicast-unicast,

eredcap-ue-not-supported,

two-rx-xr-ue-not-supported

}

CauseTransport ::= ENUMERATED {

transport-resource-unavailable,

unspecified,

...

}

Cell-CAGInformation ::= SEQUENCE {

nGRAN-CGI				NGRAN-CGI,

cellCAGList			CellCAGList,

iE-Extensions		ProtocolExtensionContainer { {Cell-CAGInformation-ExtIEs} } OPTIONAL,

...

}

Cell-CAGInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellCAGList ::= SEQUENCE (SIZE(1..maxnoofCAGSperCell)) OF CAG-ID

CellIDBroadcastEUTRA ::= SEQUENCE (SIZE(1..maxnoofCellIDforWarning)) OF CellIDBroadcastEUTRA-Item

CellIDBroadcastEUTRA-Item ::= SEQUENCE {

eUTRA-CGI			EUTRA-CGI,

iE-Extensions		ProtocolExtensionContainer { {CellIDBroadcastEUTRA-Item-ExtIEs} } OPTIONAL,

...

}

CellIDBroadcastEUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellIDBroadcastNR ::= SEQUENCE (SIZE(1..maxnoofCellIDforWarning)) OF CellIDBroadcastNR-Item

CellIDBroadcastNR-Item ::= SEQUENCE {

nR-CGI				NR-CGI,

iE-Extensions		ProtocolExtensionContainer { {CellIDBroadcastNR-Item-ExtIEs} } OPTIONAL,

...

}

CellIDBroadcastNR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellIDCancelledEUTRA ::= SEQUENCE (SIZE(1..maxnoofCellIDforWarning)) OF CellIDCancelledEUTRA-Item

CellIDCancelledEUTRA-Item ::= SEQUENCE {

eUTRA-CGI				EUTRA-CGI,

numberOfBroadcasts		NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { {CellIDCancelledEUTRA-Item-ExtIEs} } OPTIONAL,

...

}

CellIDCancelledEUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellIDCancelledNR ::= SEQUENCE (SIZE(1..maxnoofCellIDforWarning)) OF CellIDCancelledNR-Item

CellIDCancelledNR-Item ::= SEQUENCE {

nR-CGI					NR-CGI,

numberOfBroadcasts		NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { {CellIDCancelledNR-Item-ExtIEs} } OPTIONAL,

...

}

CellIDCancelledNR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellIDListForRestart ::= CHOICE {

eUTRA-CGIListforRestart		EUTRA-CGIList,

nR-CGIListforRestart		NR-CGIList,

choice-Extensions		ProtocolIE-SingleContainer { {CellIDListForRestart-ExtIEs} }

}

CellIDListForRestart-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

CellSize ::= ENUMERATED {verysmall, small, medium, large, ...}

CellType ::= SEQUENCE {

cellSize		CellSize,

iE-Extensions		ProtocolExtensionContainer { {CellType-ExtIEs} }	OPTIONAL,

...

}

CellType-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CEmodeBSupport-Indicator ::= ENUMERATED {supported,...}

CEmodeBrestricted ::= ENUMERATED {

restricted,

not-restricted,

...

}

ClockAccuracy ::= CHOICE {

clockAccuracyValue		INTEGER (1..40000000, ...),

clockAccuracyIndex		INTEGER (32..47, ...),

choice-Extensions		ProtocolIE-SingleContainer { { ClockAccuracy-ExtIEs} }

}

ClockAccuracy-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

ClockQualityAcceptanceIndication ::= SEQUENCE {

clockQualityAcceptanceCriteria		ClockQualityAcceptanceCriteria,

iE-Extensions				ProtocolExtensionContainer { {ClockQualityAcceptanceIndication-ExtIEs} } OPTIONAL,

...

}

ClockQualityAcceptanceIndication-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ClockQualityAcceptanceCriteria ::= SEQUENCE {

synchronisationState			BIT STRING (SIZE(8, ...))					OPTIONAL,

traceabletoUTC					ENUMERATED {true, ...}						OPTIONAL,

traceabletoGNSS					ENUMERATED {true, ...}						OPTIONAL,

clockFrequencyStability			BIT STRING (SIZE(16))						OPTIONAL,

clockAccuracy					INTEGER (1..40000000, ...)					OPTIONAL,

parentTImeSource				BIT STRING (SIZE(16, ...))					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ClockQualityAcceptanceCriteria-ExtIEs} }	OPTIONAL,

...

}

ClockQualityAcceptanceCriteria-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ClockQualityReportingControlInfo ::= SEQUENCE {

clockQualityDetailLevel		ClockQualityDetailLevel,

iE-Extensions				ProtocolExtensionContainer { {ClockQualityReportingControlInfo-ExtIEs} } OPTIONAL,

...

}

ClockQualityReportingControlInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ClockQualityDetailLevel ::= CHOICE {

clockQualityMetrics						NULL,

clockQualityAcceptanceIndication		ClockQualityAcceptanceIndication,

choice-extensions			ProtocolIE-SingleContainer { {ClockQualityDetailLevel-ExtIEs} }

}

ClockQualityDetailLevel-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

CNAssistedRANTuning ::= SEQUENCE {

expectedUEBehaviour					ExpectedUEBehaviour							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {CNAssistedRANTuning-ExtIEs} }	OPTIONAL,

...

}

CNAssistedRANTuning-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CNsubgroupID ::= INTEGER (0..7, ...)

CNTypeRestrictionsForEquivalent ::= SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF CNTypeRestrictionsForEquivalentItem

CNTypeRestrictionsForEquivalentItem ::= SEQUENCE {

plmnIdentity		PLMNIdentity,

cn-Type				ENUMERATED {epc-forbidden, fiveGC-forbidden, ...},

iE-Extensions		ProtocolExtensionContainer { {CNTypeRestrictionsForEquivalentItem-ExtIEs} }		OPTIONAL,

...

}

CNTypeRestrictionsForEquivalentItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::={

...

}

CNTypeRestrictionsForServing ::= ENUMERATED {

epc-forbidden,

...

}

CommandRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { CommandRequestTransferIEs} },

...

}

CommandRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIoT-CorrelationIdentifier				CRITICALITY reject	TYPE AIoT-CorrelationIdentifier				PRESENCE	mandatory	}|

{ ID id-RAN-AIOT-Device-NGAP-ID					CRITICALITY reject	TYPE RAN-AIOT-Device-NGAP-ID					PRESENCE	mandatory	}|

{ ID id-AIoT-NASPDU								CRITICALITY reject	TYPE AIoT-NASPDU								PRESENCE mandatory	}|

{ ID id-AIoT-CommandAssistanceInformation		CRITICALITY reject	TYPE AIoT-CommandAssistanceInformation	PRESENCE	optional		},

...

}

CommandResponseTransfer ::= SEQUENCE {

correlationIdentifier			AIoT-CorrelationIdentifier,

rAN-AIOT-Device-NGAP-ID			RAN-AIOT-Device-NGAP-ID,

aIoT-NASPDU						AIoT-NASPDU,

criticalityDiagnostics			CriticalityDiagnostics												OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CommandResponseTransfer-ExtIEs} }	OPTIONAL,

...

}

CommandResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CommandFailureTransfer ::= SEQUENCE {

correlationIdentifier			AIoT-CorrelationIdentifier,

rAN-AIOT-Device-NGAP-ID			RAN-AIOT-Device-NGAP-ID,

cause							Cause,

criticalityDiagnostics			CriticalityDiagnostics												OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CommandFailureTransfer-ExtIEs} }	OPTIONAL,

...

}

CommandFailureTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CommonNetworkInstance ::= OCTET STRING

CompletedCellsInEAI-EUTRA ::= SEQUENCE (SIZE(1..maxnoofCellinEAI)) OF CompletedCellsInEAI-EUTRA-Item

CompletedCellsInEAI-EUTRA-Item ::= SEQUENCE {

eUTRA-CGI			EUTRA-CGI,

iE-Extensions		ProtocolExtensionContainer { {CompletedCellsInEAI-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

CompletedCellsInEAI-EUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CompletedCellsInEAI-NR ::= SEQUENCE (SIZE(1..maxnoofCellinEAI)) OF CompletedCellsInEAI-NR-Item

CompletedCellsInEAI-NR-Item ::= SEQUENCE {

nR-CGI				NR-CGI,

iE-Extensions		ProtocolExtensionContainer { {CompletedCellsInEAI-NR-Item-ExtIEs} }	OPTIONAL,

...

}

CompletedCellsInEAI-NR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CompletedCellsInTAI-EUTRA ::= SEQUENCE (SIZE(1..maxnoofCellinTAI)) OF CompletedCellsInTAI-EUTRA-Item

CompletedCellsInTAI-EUTRA-Item ::= SEQUENCE{

eUTRA-CGI			EUTRA-CGI,

iE-Extensions		ProtocolExtensionContainer { {CompletedCellsInTAI-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

CompletedCellsInTAI-EUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CompletedCellsInTAI-NR ::= SEQUENCE (SIZE(1..maxnoofCellinTAI)) OF CompletedCellsInTAI-NR-Item

CompletedCellsInTAI-NR-Item ::= SEQUENCE{

nR-CGI				NR-CGI,

iE-Extensions		ProtocolExtensionContainer { {CompletedCellsInTAI-NR-Item-ExtIEs} } OPTIONAL,

...

}

CompletedCellsInTAI-NR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ConcurrentWarningMessageInd ::= ENUMERATED {

true,

...

}

ConfidentialityProtectionIndication ::= ENUMERATED {

required,

preferred,

not-needed,

...

}

ConfidentialityProtectionResult ::= ENUMERATED {

performed,

not-performed,

...

}

ConfiguredTACIndication ::= ENUMERATED {

true,

...

}

CoreNetworkAssistanceInformationForInactive ::= SEQUENCE {

uEIdentityIndexValue				UEIdentityIndexValue,

uESpecificDRX						PagingDRX																	OPTIONAL,

periodicRegistrationUpdateTimer		PeriodicRegistrationUpdateTimer,

mICOModeIndication					MICOModeIndication															OPTIONAL,

tAIListForInactive					TAIListForInactive,

expectedUEBehaviour					ExpectedUEBehaviour															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {CoreNetworkAssistanceInformationForInactive-ExtIEs} }	OPTIONAL,

...

}

CoreNetworkAssistanceInformationForInactive-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-EUTRA-PagingeDRXInformation					CRITICALITY ignore	EXTENSION EUTRA-PagingeDRXInformation					PRESENCE optional	}|

{ ID id-ExtendedUEIdentityIndexValue				CRITICALITY ignore	EXTENSION ExtendedUEIdentityIndexValue					PRESENCE optional	}|

{ ID id-UERadioCapabilityForPaging					CRITICALITY ignore	EXTENSION UERadioCapabilityForPaging						PRESENCE optional	}|

{ ID id-MicoAllPLMN									CRITICALITY ignore	EXTENSION MicoAllPLMN											PRESENCE optional	}|

{ ID id-NR-PagingeDRXInformation					CRITICALITY ignore	EXTENSION NR-PagingeDRXInformation						PRESENCE optional	}|

{ ID id-PagingCauseIndicationForVoiceService		CRITICALITY ignore	EXTENSION PagingCauseIndicationForVoiceService		PRESENCE optional	}|

{ ID id-PEIPSassistanceInformation					CRITICALITY ignore	EXTENSION PEIPSassistanceInformation						PRESENCE optional	}|

{ ID id-HashedUEIdentityIndexValue					CRITICALITY ignore	EXTENSION HashedUEIdentityIndexValue						PRESENCE optional	}|

{ ID id-CN-MT-CommunicationHandling					CRITICALITY ignore	EXTENSION 	 CN-MT-CommunicationHandling				PRESENCE optional	},

...

}

CN-MT-CommunicationHandling ::= ENUMERATED {supported, ...}

COUNTValueForPDCP-SN12 ::= SEQUENCE {

pDCP-SN12			INTEGER (0..4095),

hFN-PDCP-SN12		INTEGER (0..1048575),

iE-Extensions		ProtocolExtensionContainer { {COUNTValueForPDCP-SN12-ExtIEs} }	OPTIONAL,

...

}

COUNTValueForPDCP-SN12-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

COUNTValueForPDCP-SN18 ::= SEQUENCE {

pDCP-SN18			INTEGER (0..262143),

hFN-PDCP-SN18		INTEGER (0..16383),

iE-Extensions		ProtocolExtensionContainer { {COUNTValueForPDCP-SN18-ExtIEs} }	OPTIONAL,

...

}

COUNTValueForPDCP-SN18-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CoverageEnhancementLevel ::= OCTET STRING

CPTransportLayerInformation ::= CHOICE {

endpointIPAddress		TransportLayerAddress,

choice-Extensions		ProtocolIE-SingleContainer { {CPTransportLayerInformation-ExtIEs} }

}

CPTransportLayerInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-EndpointIPAddressAndPort		CRITICALITY reject	TYPE EndpointIPAddressAndPort		PRESENCE mandatory	},

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode											OPTIONAL,

triggeringMessage				TriggeringMessage										OPTIONAL,

procedureCriticality			Criticality												OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{CriticalityDiagnostics-ExtIEs}}		OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE(1..maxnoofErrors)) OF CriticalityDiagnostics-IE-Item

CriticalityDiagnostics-IE-Item ::= SEQUENCE {

iECriticality		Criticality,

iE-ID				ProtocolIE-ID,

typeOfError			TypeOfError,

iE-Extensions		ProtocolExtensionContainer {{CriticalityDiagnostics-IE-Item-ExtIEs}} OPTIONAL,

...

}

CriticalityDiagnostics-IE-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

C-RNTI ::= BIT STRING (SIZE(16))

CellBasedMDT-NR::= SEQUENCE {

cellIdListforMDT	CellIdListforMDT-NR,

iE-Extensions		ProtocolExtensionContainer { {CellBasedMDT-NR-ExtIEs} } OPTIONAL,

...

}

CellBasedMDT-NR-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellIdListforMDT-NR ::= SEQUENCE (SIZE(1..maxnoofCellIDforMDT)) OF NR-CGI

CellBasedMDT-EUTRA::= SEQUENCE {

cellIdListforMDT	CellIdListforMDT-EUTRA,

iE-Extensions		ProtocolExtensionContainer { {CellBasedMDT-EUTRA-ExtIEs} } OPTIONAL,

...

}

CellBasedMDT-EUTRA-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellBasedQMC ::= SEQUENCE {

cellIdListforQMC 	CellIdListforQMC,

iE-Extensions		ProtocolExtensionContainer { {CellBasedQMC-ExtIEs} } OPTIONAL,

...

}

CellBasedQMC-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellIdListforQMC ::= SEQUENCE (SIZE(1..maxnoofCellIDforQMC)) OF NGRAN-CGI

CellIdListforMDT-EUTRA ::= SEQUENCE (SIZE(1..maxnoofCellIDforMDT)) OF EUTRA-CGI

CAGListforMDT ::= SEQUENCE (SIZE(1.. maxnoofCAGforMDT)) OF CAGListforMDTItem

CAGListforMDTItem ::= SEQUENCE {

plmnID				PLMNIdentity,

cAGID				CAG-ID,

iE-Extensions		ProtocolExtensionContainer { {CAGListforMDTItem-ExtIEs} }	OPTIONAL,

...

}

CAGListforMDTItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::={

...

}

#### -- D

DataCodingScheme ::= BIT STRING (SIZE(8))

DataForwardingAccepted ::= ENUMERATED {

data-forwarding-accepted,

...

}

DataForwardingNotPossible ::= ENUMERATED {

data-forwarding-not-possible,

...

}

DataForwardingResponseDRBList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DataForwardingResponseDRBItem

DataForwardingResponseDRBItem ::= SEQUENCE {

dRB-ID								DRB-ID,

dLForwardingUP-TNLInformation		UPTransportLayerInformation								OPTIONAL,

uLForwardingUP-TNLInformation		UPTransportLayerInformation								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{DataForwardingResponseDRBItem-ExtIEs}}		OPTIONAL,

...

}

DataForwardingResponseDRBItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DAPSRequestInfo ::= SEQUENCE {

dAPSIndicator				ENUMERATED {daps-ho-required, ...},

iE-Extensions				ProtocolExtensionContainer { {DAPSRequestInfo-ExtIEs} } OPTIONAL,

...

}

DAPSRequestInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DAPSResponseInfoList ::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DAPSResponseInfoItem

DAPSResponseInfoItem ::= SEQUENCE {

dRB-ID				DRB-ID,

dAPSResponseInfo		DAPSResponseInfo,

iE-Extension			ProtocolExtensionContainer { {DAPSResponseInfoItem-ExtIEs} }		OPTIONAL,

...

}

DAPSResponseInfoItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DAPSResponseInfo ::= SEQUENCE {

dapsresponseindicator		ENUMERATED {daps-ho-accepted, daps-ho-not-accepted, ...},

iE-Extensions			ProtocolExtensionContainer { { DAPSResponseInfo-ExtIEs} } 	OPTIONAL,

...

}

DAPSResponseInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DataForwardingResponseERABList ::= SEQUENCE (SIZE(1..maxnoofE-RABs)) OF DataForwardingResponseERABListItem

DataForwardingResponseERABListItem ::= SEQUENCE {

e-RAB-ID							E-RAB-ID,

dLForwardingUP-TNLInformation		UPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { {DataForwardingResponseERABListItem-ExtIEs} }	OPTIONAL,

...

}

DataForwardingResponseERABListItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DelayCritical ::= ENUMERATED {

delay-critical,

non-delay-critical,

...

}

DL-CP-SecurityInformation ::= SEQUENCE {

dl-NAS-MAC				DL-NAS-MAC,

iE-Extensions			ProtocolExtensionContainer { { DL-CP-SecurityInformation-ExtIEs} }	OPTIONAL,

...

}

DL-CP-SecurityInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DL-Signalling ::= ENUMERATED {true,	...}

DL-NAS-MAC ::= BIT STRING (SIZE (16))

DLForwarding ::= ENUMERATED {

dl-forwarding-proposed,

...

}

DL-NGU-TNLInformationReused ::= ENUMERATED {

true,

...

}

DirectForwardingPathAvailability ::= ENUMERATED {

direct-path-available,

...

}

DRB-ID ::= INTEGER (1..32, ...)

DRBsSubjectToStatusTransferList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsSubjectToStatusTransferItem

DRBsSubjectToStatusTransferItem ::= SEQUENCE {

dRB-ID				DRB-ID,

dRBStatusUL			DRBStatusUL,

dRBStatusDL			DRBStatusDL,

iE-Extension		ProtocolExtensionContainer { {DRBsSubjectToStatusTransferItem-ExtIEs} }	OPTIONAL,

...

}

DRBsSubjectToStatusTransferItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-OldAssociatedQosFlowList-ULendmarkerexpected	CRITICALITY ignore EXTENSION AssociatedQosFlowList	 PRESENCE optional },

...

}

DRBStatusDL ::= CHOICE {

dRBStatusDL12			DRBStatusDL12,

dRBStatusDL18			DRBStatusDL18,

choice-Extensions		ProtocolIE-SingleContainer { {DRBStatusDL-ExtIEs} }

}

DRBStatusDL-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

DRBStatusDL12 ::= SEQUENCE {

dL-COUNTValue		COUNTValueForPDCP-SN12,

iE-Extension		ProtocolExtensionContainer { {DRBStatusDL12-ExtIEs} }		OPTIONAL,

...

}

DRBStatusDL12-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DRBStatusDL18 ::= SEQUENCE {

dL-COUNTValue		COUNTValueForPDCP-SN18,

iE-Extension		ProtocolExtensionContainer { {DRBStatusDL18-ExtIEs} }		OPTIONAL,

...

}

DRBStatusDL18-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DRBStatusUL ::= CHOICE {

dRBStatusUL12			DRBStatusUL12,

dRBStatusUL18			DRBStatusUL18,

choice-Extensions		ProtocolIE-SingleContainer { {DRBStatusUL-ExtIEs} }

}

DRBStatusUL-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

DRBStatusUL12 ::= SEQUENCE {

uL-COUNTValue					COUNTValueForPDCP-SN12,

receiveStatusOfUL-PDCP-SDUs		BIT STRING (SIZE(1..2048))						OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DRBStatusUL12-ExtIEs} }		OPTIONAL,

...

}

DRBStatusUL12-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DRBStatusUL18 ::= SEQUENCE {

uL-COUNTValue					COUNTValueForPDCP-SN18,

receiveStatusOfUL-PDCP-SDUs		BIT STRING (SIZE(1..131072))					OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {DRBStatusUL18-ExtIEs} }		OPTIONAL,

...

}

DRBStatusUL18-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsToQosFlowsMappingList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBsToQosFlowsMappingItem

DRBsToQosFlowsMappingItem ::= SEQUENCE {

dRB-ID								DRB-ID,

associatedQosFlowList				AssociatedQosFlowList,

iE-Extensions		ProtocolExtensionContainer { {DRBsToQosFlowsMappingItem-ExtIEs} }		OPTIONAL,

...

}

DRBsToQosFlowsMappingItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-DAPSRequestInfo	CRITICALITY ignore	EXTENSION DAPSRequestInfo	PRESENCE optional  },

...

}

Dynamic5QIDescriptor ::= SEQUENCE {

priorityLevelQos			PriorityLevelQos,

packetDelayBudget			PacketDelayBudget,

packetErrorRate				PacketErrorRate,

fiveQI						FiveQI													OPTIONAL,

delayCritical				DelayCritical											OPTIONAL,

-- The above IE shall be present in case of GBR QoS flow

averagingWindow				AveragingWindow											OPTIONAL,

-- The above IE shall be present in case of GBR QoS flow

maximumDataBurstVolume		MaximumDataBurstVolume									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {Dynamic5QIDescriptor-ExtIEs} }	OPTIONAL,

...

}

Dynamic5QIDescriptor-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedPacketDelayBudget	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-CNPacketDelayBudgetDL		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-CNPacketDelayBudgetUL		CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		},

...

}

#### -- E

EarlyMeasurement ::= ENUMERATED {true, ...}

EarlyStatusTransfer-TransparentContainer ::= SEQUENCE {

procedureStage			ProcedureStageChoice,

iE-Extensions			ProtocolExtensionContainer { {EarlyStatusTransfer-TransparentContainer-ExtIEs} }	OPTIONAL,

...

}

EarlyStatusTransfer-TransparentContainer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ProcedureStageChoice ::= CHOICE {

first-dl-count			FirstDLCount,

choice-Extensions		ProtocolIE-SingleContainer { {ProcedureStageChoice-ExtIEs} }

}

ProcedureStageChoice-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-DLDiscarding						CRITICALITY ignore	TYPE DLDiscarding									PRESENCE mandatory	},

...

}

DLDiscarding ::= SEQUENCE {

dRBsSubjectToDLDiscarding					DRBsSubjectToDLDiscarding-List,

iE-Extension								ProtocolExtensionContainer { {DLDiscarding-ExtIEs} }	OPTIONAL,

...

}

DLDiscarding-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsSubjectToDLDiscarding-List ::= SEQUENCE (SIZE (1..maxnoofDRBs)) OF DRBsSubjectToDLDiscarding-Item

DRBsSubjectToDLDiscarding-Item ::= SEQUENCE {

drbID				DRB-ID,

dlCount				DLCountChoice,

iE-Extension		ProtocolExtensionContainer { { DRBsSubjectToDLDiscarding-Item-ExtIEs} }	OPTIONAL,

...

}

DRBsSubjectToDLDiscarding-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DLCountChoice ::= CHOICE {

count12bits				COUNTValueForPDCP-SN12,

count18bits				COUNTValueForPDCP-SN18,

choice-extension		ProtocolIE-SingleContainer { {DLCountChoice-ExtIEs} }

}

DLCountChoice-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

FirstDLCount ::= SEQUENCE {

dRBsSubjectToEarlyStatusTransfer		DRBsSubjectToEarlyStatusTransfer-List,

iE-Extension		ProtocolExtensionContainer { {FirstDLCount-ExtIEs} }		OPTIONAL,

...

}

FirstDLCount-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DRBsSubjectToEarlyStatusTransfer-List ::= SEQUENCE (SIZE (1.. maxnoofDRBs)) OF DRBsSubjectToEarlyStatusTransfer-Item

DRBsSubjectToEarlyStatusTransfer-Item ::= SEQUENCE {

dRB-ID				DRB-ID,

firstDLCOUNT		DRBStatusDL,

iE-Extension		ProtocolExtensionContainer { { DRBsSubjectToEarlyStatusTransfer-Item-ExtIEs} }	OPTIONAL,

...

}

DRBsSubjectToEarlyStatusTransfer-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ECNMarkingorCongestionInformationReportingRequest ::= CHOICE {

eCNMarkingAtRANRequest				ECNMarkingAtRANRequest,

eCNMarkingAtUPFRequest				ECNMarkingAtUPFRequest,

congestionInformationRequest		CongestionInformationRequest,

choice-Extensions				ProtocolIE-SingleContainer { {ECNMarkingorCongestionInformationReportingRequest-ExtIEs} }

}

ECNMarkingorCongestionInformationReportingRequest-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

ECNMarkingAtRANRequest ::= ENUMERATED {ul, dl, both, stop, ...}

ECNMarkingAtUPFRequest ::= ENUMERATED {ul, dl, both, stop, ...}

CongestionInformationRequest ::= ENUMERATED {ul, dl, both, stop, ...}

ECNMarkingorCongestionInformationReportingStatus ::= SEQUENCE (SIZE (1..maxnoofQosFlows)) OF ECNMarkingorCongestionInformationReportingStatus-Item

ECNMarkingorCongestionInformationReportingStatus-Item ::= SEQUENCE {

qosFlowIdentifier				QosFlowIdentifier,

activationStatus				ActivationStatus,

iE-Extension					ProtocolExtensionContainer { { ECNMarkingorCongestionInformationReportingStatus-Item-ExtIEs} }	OPTIONAL,

...

}

ECNMarkingorCongestionInformationReportingStatus-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ActivationStatus	::= ENUMERATED {

active,

not-active,

...

}

EDT-Session ::= ENUMERATED {

true,

...

}

EmergencyAreaID ::= OCTET STRING (SIZE(3))

EmergencyAreaIDBroadcastEUTRA ::= SEQUENCE (SIZE(1..maxnoofEmergencyAreaID)) OF EmergencyAreaIDBroadcastEUTRA-Item

EmergencyAreaIDBroadcastEUTRA-Item ::= SEQUENCE {

emergencyAreaID					EmergencyAreaID,

completedCellsInEAI-EUTRA		CompletedCellsInEAI-EUTRA,

iE-Extensions		ProtocolExtensionContainer { {EmergencyAreaIDBroadcastEUTRA-Item-ExtIEs} } OPTIONAL,

...

}

EmergencyAreaIDBroadcastEUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EmergencyAreaIDBroadcastNR ::= SEQUENCE (SIZE(1..maxnoofEmergencyAreaID)) OF EmergencyAreaIDBroadcastNR-Item

EmergencyAreaIDBroadcastNR-Item ::= SEQUENCE {

emergencyAreaID				EmergencyAreaID,

completedCellsInEAI-NR		CompletedCellsInEAI-NR,

iE-Extensions		ProtocolExtensionContainer { {EmergencyAreaIDBroadcastNR-Item-ExtIEs} }	OPTIONAL,

...

}

EmergencyAreaIDBroadcastNR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EmergencyAreaIDCancelledEUTRA ::= SEQUENCE (SIZE(1..maxnoofEmergencyAreaID)) OF EmergencyAreaIDCancelledEUTRA-Item

EmergencyAreaIDCancelledEUTRA-Item ::= SEQUENCE {

emergencyAreaID					EmergencyAreaID,

cancelledCellsInEAI-EUTRA		CancelledCellsInEAI-EUTRA,

iE-Extensions		ProtocolExtensionContainer { {EmergencyAreaIDCancelledEUTRA-Item-ExtIEs} } OPTIONAL,

...

}

EmergencyAreaIDCancelledEUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EmergencyAreaIDCancelledNR ::= SEQUENCE (SIZE(1..maxnoofEmergencyAreaID)) OF EmergencyAreaIDCancelledNR-Item

EmergencyAreaIDCancelledNR-Item ::= SEQUENCE {

emergencyAreaID				EmergencyAreaID,

cancelledCellsInEAI-NR		CancelledCellsInEAI-NR,

iE-Extensions		ProtocolExtensionContainer { {EmergencyAreaIDCancelledNR-Item-ExtIEs} }	OPTIONAL,

...

}

EmergencyAreaIDCancelledNR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EmergencyAreaIDList ::= SEQUENCE (SIZE(1..maxnoofEmergencyAreaID)) OF EmergencyAreaID

EmergencyAreaIDListForRestart ::= SEQUENCE (SIZE(1..maxnoofEAIforRestart)) OF EmergencyAreaID

EmergencyFallbackIndicator ::= SEQUENCE {

emergencyFallbackRequestIndicator		EmergencyFallbackRequestIndicator,

emergencyServiceTargetCN				EmergencyServiceTargetCN						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {EmergencyFallbackIndicator-ExtIEs} }	OPTIONAL,

...

}

EmergencyFallbackIndicator-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EmergencyFallbackRequestIndicator ::= ENUMERATED {

emergency-fallback-requested,

...

}

EmergencyServiceTargetCN ::= ENUMERATED {

fiveGC,

epc,

...

}

ENB-ID ::= CHOICE {

macroENB-ID				BIT STRING (SIZE(20)),

homeENB-ID				BIT STRING (SIZE(28)),

short-macroENB-ID 		BIT STRING (SIZE(18)),

long-macroENB-ID		BIT STRING (SIZE(21)),

choice-Extensions		ProtocolIE-SingleContainer { { ENB-ID-ExtIEs} }

}

ENB-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

Enhanced-CoverageRestriction ::= ENUMERATED {restricted, ... }

Extended-ConnectedTime ::= INTEGER (0..255)

EN-DCSONConfigurationTransfer ::= OCTET STRING

EndpointIPAddressAndPort ::=SEQUENCE {

endpointIPAddress TransportLayerAddress,

portNumber			PortNumber,

iE-Extensions		ProtocolExtensionContainer { { EndpointIPAddressAndPort-ExtIEs} } OPTIONAL

}

EndIndication ::= ENUMERATED {

no-further-data,

further-data-exists,

...

}

EndpointIPAddressAndPort-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EquivalentPLMNs ::= SEQUENCE (SIZE(1..maxnoofEPLMNs)) OF PLMNIdentity

EPS-TAC ::= OCTET STRING (SIZE(2))

EPS-TAI ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

ePS-TAC				EPS-TAC,

iE-Extensions		ProtocolExtensionContainer { {EPS-TAI-ExtIEs} } OPTIONAL,

...

}

EPS-TAI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

E-RAB-ID ::= INTEGER (0..15, ...)

E-RABInformationList ::= SEQUENCE (SIZE(1..maxnoofE-RABs)) OF E-RABInformationItem

E-RABInformationItem ::= SEQUENCE {

e-RAB-ID			E-RAB-ID,

dLForwarding		DLForwarding													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {E-RABInformationItem-ExtIEs} }	OPTIONAL,

...

}

E-RABInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-SourceTNLAddrInfo		CRITICALITY ignore	EXTENSION TransportLayerAddress	PRESENCE optional}|

{ID id-SourceNodeTNLAddrInfo	CRITICALITY ignore	EXTENSION TransportLayerAddress	PRESENCE optional},

...

}

ERedCapIndication ::= ENUMERATED {

true,

...

}

EUTRACellIdentity ::= BIT STRING (SIZE(28))

EUTRA-CGI ::= SEQUENCE {

pLMNIdentity			PLMNIdentity,

eUTRACellIdentity		EUTRACellIdentity,

iE-Extensions		ProtocolExtensionContainer { {EUTRA-CGI-ExtIEs} } OPTIONAL,

...

}

EUTRA-CGI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-CGIList ::= SEQUENCE (SIZE(1..maxnoofCellsinngeNB)) OF EUTRA-CGI

EUTRA-CGIListForWarning ::= SEQUENCE (SIZE(1..maxnoofCellIDforWarning)) OF EUTRA-CGI

EUTRA-PagingeDRXInformation ::= SEQUENCE {

eUTRA-paging-eDRX-Cycle		EUTRA-Paging-eDRX-Cycle,

eUTRA-paging-Time-Window	EUTRA-Paging-Time-Window					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {EUTRA-PagingeDRXInformation-ExtIEs} }	OPTIONAL,

...

}

EUTRA-PagingeDRXInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-Paging-eDRX-Cycle ::= ENUMERATED {

hfhalf, hf1, hf2, hf4, hf6,

hf8, hf10, hf12, hf14, hf16,

hf32, hf64, hf128, hf256,

...

}

EUTRA-Paging-Time-Window ::= ENUMERATED {

s1, s2, s3, s4, s5,

s6, s7, s8, s9, s10,

s11, s12, s13, s14, s15, s16,

...

}

EUTRAencryptionAlgorithms ::= BIT STRING (SIZE(16, ...))

EUTRAintegrityProtectionAlgorithms ::= BIT STRING (SIZE(16, ...))

EventType ::= ENUMERATED {

direct,

change-of-serve-cell,

ue-presence-in-area-of-interest,

stop-change-of-serve-cell,

stop-ue-presence-in-area-of-interest,

cancel-location-reporting-for-the-ue,

...,

change-of-serving-cell-and-UE-presence-in-the-Area-of-Interest

}

ExcessPacketDelayThresholdConfiguration ::= SEQUENCE (SIZE(1..maxnoofThresholdsForExcessPacketDelay)) OF ExcessPacketDelayThresholdItem

ExcessPacketDelayThresholdItem ::= SEQUENCE {

fiveQi		FiveQI,

excessPacketDelayThresholdValue			ExcessPacketDelayThresholdValue,

iE-Extensions		ProtocolExtensionContainer { { ExcessPacketDelayThresholdItem-ExtIEs} }	OPTIONAL,

...

}

ExcessPacketDelayThresholdItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ExcessPacketDelayThresholdValue ::= ENUMERATED {

ms0dot25, ms0dot5, ms1, ms2, ms4, ms5, ms10, ms20, ms30, ms40, ms50, ms60, ms70, ms80, ms90, ms100, ms150, ms300, ms500,

...

}

ExpectedActivityPeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181, ...)

ExpectedHOInterval ::= ENUMERATED {

sec15, sec30, sec60, sec90, sec120, sec180, long-time,

...

}

ExpectedIdlePeriod ::= INTEGER (1..30|40|50|60|80|100|120|150|180|181, ...)

ExpectedUEActivityBehaviour ::= SEQUENCE {

expectedActivityPeriod						ExpectedActivityPeriod						OPTIONAL,

expectedIdlePeriod							ExpectedIdlePeriod							OPTIONAL,

sourceOfUEActivityBehaviourInformation		SourceOfUEActivityBehaviourInformation		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEActivityBehaviour-ExtIEs} }	OPTIONAL,

...

}

ExpectedUEActivityBehaviour-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUEBehaviour ::= SEQUENCE {

expectedUEActivityBehaviour		ExpectedUEActivityBehaviour 					OPTIONAL,

expectedHOInterval				ExpectedHOInterval		 						OPTIONAL,

expectedUEMobility				ExpectedUEMobility								OPTIONAL,

expectedUEMovingTrajectory		ExpectedUEMovingTrajectory						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEBehaviour-ExtIEs} }	OPTIONAL,

...

}

ExpectedUEBehaviour-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ExpectedUEMobility ::= ENUMERATED {

stationary,

mobile,

...

}

ExpectedUEMovingTrajectory ::= SEQUENCE (SIZE(1..maxnoofCellsUEMovingTrajectory)) OF ExpectedUEMovingTrajectoryItem

ExpectedUEMovingTrajectoryItem ::= SEQUENCE {

nGRAN-CGI				NGRAN-CGI,

timeStayedInCell		INTEGER (0..4095)													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ExpectedUEMovingTrajectoryItem-ExtIEs} }	OPTIONAL,

...

}

ExpectedUEMovingTrajectoryItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

Extended-AMFName	 ::= SEQUENCE {

aMFNameVisibleString		AMFNameVisibleString				OPTIONAL,

aMFNameUTF8String			AMFNameUTF8String					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Extended-AMFName-ExtIEs } } OPTIONAL,

...

}

Extended-AMFName-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedPacketDelayBudget ::= INTEGER (1..65535, ..., 65536..109999)

Extended-RANNodeName	 ::= SEQUENCE {

rANNodeNameVisibleString		RANNodeNameVisibleString					OPTIONAL,

rANNodeNameUTF8String			RANNodeNameUTF8String						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Extended-RANNodeName-ExtIEs } } OPTIONAL,

...

}

Extended-RANNodeName-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedRATRestrictionInformation ::= SEQUENCE {

primaryRATRestriction		BIT STRING (SIZE(8, ..., 16)),

secondaryRATRestriction		BIT STRING (SIZE(8, ...)),

iE-Extensions		ProtocolExtensionContainer { {ExtendedRATRestrictionInformation-ExtIEs} }	OPTIONAL,

...

}

ExtendedRATRestrictionInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedRNC-ID					::= INTEGER (4096..65535)

ExtendedSliceSupportList ::= SEQUENCE (SIZE(1..maxnoofExtSliceItems)) OF SliceSupportItem

ExtendedUEIdentityIndexValue ::= BIT STRING (SIZE(16))

EventTrigger::= CHOICE {

outOfCoverage				ENUMERATED {true, ...},

eventL1LoggedMDTConfig		EventL1LoggedMDTConfig,

choice-Extensions		ProtocolIE-SingleContainer { { EventTrigger-ExtIEs} }

}

EventTrigger-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

EventL1LoggedMDTConfig ::= SEQUENCE {

l1Threshold					MeasurementThresholdL1LoggedMDT,

hysteresis					Hysteresis,

timeToTrigger				TimeToTrigger,

iE-Extensions				ProtocolExtensionContainer { { EventL1LoggedMDTConfig-ExtIEs} } OPTIONAL,

...

}

EventL1LoggedMDTConfig-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MeasurementThresholdL1LoggedMDT ::= CHOICE {

threshold-RSRP				Threshold-RSRP,

threshold-RSRQ				Threshold-RSRQ,

choice-Extensions		ProtocolIE-SingleContainer { { MeasurementThresholdL1LoggedMDT-ExtIEs} }

}

MeasurementThresholdL1LoggedMDT-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

EquivalentSNPNsList ::= SEQUENCE (SIZE(1..maxnoofESNPNs)) OF EquivalentSNPNsItem

EquivalentSNPNsItem ::= SEQUENCE {

plmnIdentity		PLMNIdentity,

nID					NID,

iE-Extensions		ProtocolExtensionContainer { {EquivalentSNPNsItem-ExtIEs} }	OPTIONAL,

...

}

EquivalentSNPNsItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::={

...

}

#### -- F

FailureIndication ::= SEQUENCE {

uERLFReportContainer 	UERLFReportContainer,

iE-Extensions		ProtocolExtensionContainer { { FailureIndication-ExtIEs} }	OPTIONAL,

...

}

FailureIndication-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

FiveGCAction ::= CHOICE {

hLComActivate			HLComActivate,

hLComDeactivate			HLComDeactivate,

choice-Extensions		ProtocolIE-SingleContainer { {FiveGCAction-ExtIEs} }

}

FiveGCAction-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

FiveG-ProSeAuthorized ::= SEQUENCE {

fiveGProSeDirectDiscovery				FiveGProSeDirectDiscovery						OPTIONAL,

fiveGProSeDirectCommunication			FiveGProSeDirectCommunication					OPTIONAL,

fiveGProSeLayer2UEtoNetworkRelay		FiveGProSeLayer2UEtoNetworkRelay				OPTIONAL,

fiveGProSeLayer3UEtoNetworkRelay		FiveGProSeLayer3UEtoNetworkRelay				OPTIONAL,

fiveGProSeLayer2RemoteUE				FiveGProSeLayer2RemoteUE						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {FiveG-ProSeAuthorized-ExtIEs} }		OPTIONAL,

...

}

FiveG-ProSeAuthorized-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-FiveGProSeLayer2Multipath		CRITICALITY ignore	EXTENSION FiveGProSeLayer2Multipath			PRESENCE optional		}|

{ ID id-FiveGProSeLayer2UEtoUERelay		CRITICALITY ignore	EXTENSION FiveGProSeLayer2UEtoUERelay		PRESENCE optional		}|

{ ID id-FiveGProSeLayer2UEtoUERemote	CRITICALITY ignore	EXTENSION FiveGProSeLayer2UEtoUERemote		PRESENCE optional		},

...

}

FiveGProSeDirectDiscovery ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeDirectCommunication ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2UEtoNetworkRelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer3UEtoNetworkRelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2RemoteUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2Multipath ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2UEtoUERelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveGProSeLayer2UEtoUERemote ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSePC5QoSParameters ::= SEQUENCE {

fiveGProSepc5QoSFlowList					FiveGProSePC5QoSFlowList,

fiveGProSepc5LinkAggregateBitRates		BitRate												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { FiveG-ProSePC5QoSParameters-ExtIEs} }	OPTIONAL,

...

}

FiveG-ProSePC5QoSParameters-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

FiveGProSePC5QoSFlowList ::= SEQUENCE (SIZE(1..maxnoofPC5QoSFlows)) OF FiveGProSePC5QoSFlowItem

FiveGProSePC5QoSFlowItem ::= SEQUENCE {

fiveGproSepQI				FiveQI,

fiveGproSepc5FlowBitRates	FiveGProSePC5FlowBitRates				OPTIONAL,

fiveGproSerange				Range									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { FiveGProSePC5QoSFlowItem-ExtIEs} }	OPTIONAL,

...

}

FiveGProSePC5QoSFlowItem-ExtIEs  NGAP-PROTOCOL-EXTENSION ::= {

...

}

FiveGProSePC5FlowBitRates ::= SEQUENCE {

fiveGproSeguaranteedFlowBitRate		BitRate,

fiveGproSemaximumFlowBitRate		BitRate,

iE-Extensions		ProtocolExtensionContainer { { FiveGProSePC5FlowBitRates-ExtIEs} }	OPTIONAL,

...

}

FiveGProSePC5FlowBitRates-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

FiveG-S-TMSI ::= SEQUENCE {

aMFSetID			AMFSetID,

aMFPointer			AMFPointer,

fiveG-TMSI			FiveG-TMSI,

iE-Extensions		ProtocolExtensionContainer { {FiveG-S-TMSI-ExtIEs} }	OPTIONAL,

...

}

FiveG-S-TMSI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

FiveG-TMSI ::= OCTET STRING (SIZE(4))

FiveQI ::= INTEGER (0..255, ...)

ForbiddenAreaInformation ::= SEQUENCE (SIZE(1.. maxnoofEPLMNsPlusOne)) OF ForbiddenAreaInformation-Item

ForbiddenAreaInformation-Item ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

forbiddenTACs		ForbiddenTACs,

iE-Extensions		ProtocolExtensionContainer { {ForbiddenAreaInformation-Item-ExtIEs} } OPTIONAL,

...

}

ForbiddenAreaInformation-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ForbiddenTACs ::= SEQUENCE (SIZE(1..maxnoofForbTACs)) OF TAC

FromEUTRANtoNGRAN ::= SEQUENCE {

sourceeNBID				IntersystemSONeNBID,

targetNGRANnodeID		IntersystemSONNGRANnodeID,

iE-Extensions			ProtocolExtensionContainer { { FromEUTRANtoNGRAN-ExtIEs} }			OPTIONAL

}

FromEUTRANtoNGRAN-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

FromNGRANtoEUTRAN ::= SEQUENCE {

sourceNGRANnodeID		IntersystemSONNGRANnodeID,

targeteNBID				IntersystemSONeNBID,

iE-Extensions			ProtocolExtensionContainer { { FromNGRANtoEUTRAN-ExtIEs} }			OPTIONAL

}

FromNGRANtoEUTRAN-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- G

GBR-QosInformation ::= SEQUENCE {

maximumFlowBitRateDL		BitRate,

maximumFlowBitRateUL		BitRate,

guaranteedFlowBitRateDL		BitRate,

guaranteedFlowBitRateUL		BitRate,

notificationControl			NotificationControl									OPTIONAL,

maximumPacketLossRateDL		PacketLossRate										OPTIONAL,

maximumPacketLossRateUL		PacketLossRate										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {GBR-QosInformation-ExtIEs} }	OPTIONAL,

...

}

GBR-QosInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AlternativeQoSParaSetList	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetList	PRESENCE optional	},

...

}

GlobalCable-ID ::= OCTET STRING

GlobalCable-ID-new ::= SEQUENCE {

globalCable-ID			GlobalCable-ID,

tAI						TAI,

iE-Extensions			ProtocolExtensionContainer { { GlobalCable-ID-new-ExtIEs} }		OPTIONAL,

...

}

GlobalCable-ID-new-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalENB-ID ::= SEQUENCE {

pLMNidentity			PLMNIdentity,

eNB-ID					ENB-ID,

iE-Extensions			ProtocolExtensionContainer { {GlobalENB-ID-ExtIEs} }		OPTIONAL,

...

}

GlobalENB-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalGNB-ID ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

gNB-ID				GNB-ID,

iE-Extensions		ProtocolExtensionContainer { {GlobalGNB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalGNB-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalN3IWF-ID ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

n3IWF-ID			N3IWF-ID,

iE-Extensions		ProtocolExtensionContainer { {GlobalN3IWF-ID-ExtIEs} } OPTIONAL,

...

}

GlobalN3IWF-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalLine-ID ::= SEQUENCE {

globalLineIdentity		GlobalLineIdentity,

lineType			LineType									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {GlobalLine-ID-ExtIEs} } 		OPTIONAL,

...

}

GlobalLine-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-TAI				CRITICALITY ignore	EXTENSION TAI							PRESENCE optional	},

...

}

GlobalLineIdentity ::= OCTET STRING

GlobalNgENB-ID ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

ngENB-ID			NgENB-ID,

iE-Extensions		ProtocolExtensionContainer { {GlobalNgENB-ID-ExtIEs} } OPTIONAL,

...

}

GlobalNgENB-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalRANNodeID ::= CHOICE {

globalGNB-ID			GlobalGNB-ID,

globalNgENB-ID			GlobalNgENB-ID,

globalN3IWF-ID			GlobalN3IWF-ID,

choice-Extensions		ProtocolIE-SingleContainer { {GlobalRANNodeID-ExtIEs} }

}

GlobalRANNodeID-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-GlobalTNGF-ID		CRITICALITY reject	TYPE GlobalTNGF-ID			PRESENCE mandatory	}|

{ ID id-GlobalTWIF-ID		CRITICALITY reject	TYPE GlobalTWIF-ID			PRESENCE mandatory 	}|

{ ID id-GlobalW-AGF-ID		CRITICALITY reject	TYPE GlobalW-AGF-ID			PRESENCE mandatory 	},

...

}

GlobalTNGF-ID ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

tNGF-ID				TNGF-ID,

iE-Extensions		ProtocolExtensionContainer { { GlobalTNGF-ID-ExtIEs} } OPTIONAL,

...

}

GlobalTNGF-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalTWIF-ID ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

tWIF-ID				TWIF-ID,

iE-Extensions		ProtocolExtensionContainer { { GlobalTWIF-ID-ExtIEs} } OPTIONAL,

...

}

GlobalTWIF-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GlobalW-AGF-ID ::= SEQUENCE {

pLMNIdentity			PLMNIdentity,

w-AGF-ID				W-AGF-ID,

iE-Extensions			ProtocolExtensionContainer { { GlobalW-AGF-ID-ExtIEs} } OPTIONAL,

...

}

GlobalW-AGF-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GNB-ID ::= CHOICE {

gNB-ID		BIT STRING (SIZE(22..32)),

choice-Extensions		ProtocolIE-SingleContainer { {GNB-ID-ExtIEs} }

}

GNB-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

GTP-TEID ::= OCTET STRING (SIZE(4))

GTPTunnel ::= SEQUENCE {

transportLayerAddress		TransportLayerAddress,

gTP-TEID					GTP-TEID,

iE-Extensions		ProtocolExtensionContainer { {GTPTunnel-ExtIEs} } OPTIONAL,

...

}

GTPTunnel-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GUAMI ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

aMFRegionID			AMFRegionID,

aMFSetID			AMFSetID,

aMFPointer			AMFPointer,

iE-Extensions		ProtocolExtensionContainer { {GUAMI-ExtIEs} } OPTIONAL,

...

}

GUAMI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GUAMIType ::= ENUMERATED {native, mapped, ...}

-- H

HandoverCommandTransfer ::= SEQUENCE {

dLForwardingUP-TNLInformation		UPTransportLayerInformation						OPTIONAL,

qosFlowToBeForwardedList			QosFlowToBeForwardedList						OPTIONAL,

dataForwardingResponseDRBList		DataForwardingResponseDRBList					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {HandoverCommandTransfer-ExtIEs} } OPTIONAL,

...

}

HandoverCommandTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalDLForwardingUPTNLInformation		CRITICALITY ignore	EXTENSION QosFlowPerTNLInformationList			PRESENCE optional		}|

{ ID id-ULForwardingUP-TNLInformation				CRITICALITY reject	EXTENSION UPTransportLayerInformation			PRESENCE optional		}|

{ ID id-AdditionalULForwardingUPTNLInformation		CRITICALITY reject	EXTENSION UPTransportLayerInformationList		PRESENCE optional		}|

{ ID id-DataForwardingResponseERABList				CRITICALITY ignore	EXTENSION DataForwardingResponseERABList		PRESENCE optional		}|

{ ID id-QosFlowFailedToSetupList					CRITICALITY ignore	EXTENSION QosFlowListWithCause						PRESENCE optional		},

...

}

HandoverFlag ::= ENUMERATED {

handover-preparation,

...

}

HandoverPreparationUnsuccessfulTransfer ::= SEQUENCE {

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { {HandoverPreparationUnsuccessfulTransfer-ExtIEs} }	OPTIONAL,

...

}

HandoverPreparationUnsuccessfulTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

HandoverRequestAcknowledgeTransfer ::= SEQUENCE {

dL-NGU-UP-TNLInformation			UPTransportLayerInformation,

dLForwardingUP-TNLInformation		UPTransportLayerInformation										OPTIONAL,

securityResult						SecurityResult													OPTIONAL,

qosFlowSetupResponseList			QosFlowListWithDataForwarding,

qosFlowFailedToSetupList			QosFlowListWithCause											OPTIONAL,

dataForwardingResponseDRBList		DataForwardingResponseDRBList									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {HandoverRequestAcknowledgeTransfer-ExtIEs} }	OPTIONAL,

...

}

HandoverRequestAcknowledgeTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalDLUPTNLInformationForHOList		CRITICALITY ignore	EXTENSION AdditionalDLUPTNLInformationForHOList		PRESENCE optional	}|

{ ID id-ULForwardingUP-TNLInformation				CRITICALITY reject	EXTENSION UPTransportLayerInformation					PRESENCE optional	}|

{ ID id-AdditionalULForwardingUPTNLInformation		CRITICALITY reject	EXTENSION UPTransportLayerInformationList				PRESENCE optional	}|

{ ID id-DataForwardingResponseERABList				CRITICALITY ignore	EXTENSION DataForwardingResponseERABList 				PRESENCE optional	}|

{ ID id-RedundantDL-NGU-UP-TNLInformation			CRITICALITY ignore	EXTENSION UPTransportLayerInformation 					PRESENCE optional	}|

{ ID id-UsedRSNInformation							CRITICALITY ignore	EXTENSION RedundantPDUSessionInformation				PRESENCE optional	}|

{ ID id-GlobalRANNodeID 							CRITICALITY ignore	EXTENSION GlobalRANNodeID									PRESENCE optional	}|

{ ID id-MBS-SupportIndicator						CRITICALITY ignore	EXTENSION MBS-SupportIndicator								PRESENCE optional	}|

{ ID id-PDUSetbasedHandlingIndicator 				CRITICALITY ignore	EXTENSION PDUSetbasedHandlingIndicator 					PRESENCE optional }|

{ ID id-ECNMarkingorCongestionInformationReportingStatus	CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingStatus		PRESENCE optional },

...

}

HandoverRequiredTransfer ::= SEQUENCE {

directForwardingPathAvailability		DirectForwardingPathAvailability				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {HandoverRequiredTransfer-ExtIEs} }	OPTIONAL,

...

}

HandoverRequiredTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

HandoverResourceAllocationUnsuccessfulTransfer ::= SEQUENCE {

cause						Cause,

criticalityDiagnostics		CriticalityDiagnostics																	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {HandoverResourceAllocationUnsuccessfulTransfer-ExtIEs} }	OPTIONAL,

...

}

HandoverResourceAllocationUnsuccessfulTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

HandoverType ::= ENUMERATED {

intra5gs,

fivegs-to-eps,

eps-to-5gs,

...,

fivegs-to-utran

}

HashedUEIdentityIndexValue ::= BIT STRING (SIZE(13, ...))

HFCNode-ID ::= OCTET STRING

HFCNode-ID-new ::= SEQUENCE {

hFCNode-ID			HFCNode-ID,

tAI					TAI,

iE-Extensions		ProtocolExtensionContainer { { HFCNode-ID-new-ExtIEs} }	OPTIONAL,

...

}

HFCNode-ID-new-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-GlobalCable-ID				CRITICALITY ignore	EXTENSION GlobalCable-ID				PRESENCE optional		},

...

}

HLComActivate ::= SEQUENCE {

nR-Paging-Long-eDRX-Information-for-RRC-INACTIVE			NR-Paging-Long-eDRX-Information-for-RRC-INACTIVE,

iE-Extensions												ProtocolExtensionContainer { { HLComActivate-ExtIEs} }	OPTIONAL,

...

}

HLComActivate-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

HLComDeactivate ::= SEQUENCE {

uEReachabilityIndication		ENUMERATED {true, ...},

iE-Extensions		ProtocolExtensionContainer { { HLComDeactivate-ExtIEs} }	OPTIONAL,

...

}

HLComDeactivate-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

HOReport::= SEQUENCE {

handoverReportType			ENUMERATED {ho-too-early, ho-to-wrong-cell, intersystem-ping-pong, ...},

handoverCause				Cause,

sourcecellCGI				NGRAN-CGI,

targetcellCGI				NGRAN-CGI,

reestablishmentcellCGI		NGRAN-CGI									OPTIONAL,

-- The above IE shall be present if the Handover Report Type IE is set to the value “HO to wrong cell”

sourcecellC-RNTI			BIT STRING (SIZE(16))						OPTIONAL,

targetcellinE-UTRAN			EUTRA-CGI									OPTIONAL,

-- The above IE shall be present if the Handover Report Type IE is set to the value “Inter System ping-pong”

mobilityInformation			MobilityInformation							OPTIONAL,

uERLFReportContainer		UERLFReportContainer						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { HOReport-ExtIEs} }	OPTIONAL,

...

}

HOReport-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedMobilityInformation				CRITICALITY ignore	EXTENSION ExtendedMobilityInformation				PRESENCE optional		}|

{ ID id-TargetCellCRNTI								CRITICALITY ignore	EXTENSION C-RNTI										PRESENCE optional		}|

{ ID id-TimeSinceFailure						CRITICALITY ignore	EXTENSION TimeSinceFailure							PRESENCE optional		},

...

}

Hysteresis ::=                      INTEGER (0..30)

-- I

IAB-Authorized ::= ENUMERATED {

authorized,

not-authorized,

...

}

MobileIAB-MTUserLocationInformation ::= SEQUENCE {

nRCGI							NR-CGI,

tAI								TAI,

iE-Extensions					ProtocolExtensionContainer { { MobileIAB-MTUserLocationInformation-ExtIEs} }	OPTIONAL,

...

}

MobileIAB-MTUserLocationInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

IAB-Supported ::= ENUMERATED {

true,

...

}

IABNodeIndication ::= ENUMERATED {

true,

...

}

IMSVoiceSupportIndicator ::= ENUMERATED {

supported,

not-supported,

...

}

IndexToRFSP ::= INTEGER (1..256, ...)

InfoOnRecommendedCellsAndRANNodesForPaging ::= SEQUENCE {

recommendedCellsForPaging		RecommendedCellsForPaging,

recommendRANNodesForPaging		RecommendedRANNodesForPaging,

iE-Extensions		ProtocolExtensionContainer { {InfoOnRecommendedCellsAndRANNodesForPaging-ExtIEs} }	OPTIONAL,

...

}

InfoOnRecommendedCellsAndRANNodesForPaging-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

IntegrityProtectionIndication ::= ENUMERATED {

required,

preferred,

not-needed,

...

}

IntegrityProtectionResult ::= ENUMERATED {

performed,

not-performed,

...

}

IntendedNumberOfPagingAttempts ::= INTEGER (1..16, ...)

InterfacesToTrace ::= BIT STRING (SIZE(8))

ImmediateMDTNr ::= SEQUENCE {

measurementsToActivate					MeasurementsToActivate,

m1Configuration							M1Configuration						OPTIONAL,

-- The above IE shall be present if the Measurements to Activate IE has the first bit set to “1”

m4Configuration							M4Configuration						OPTIONAL,

-- The above IE shall be present if the Measurements to Activate IE has the third bit set to “1”

m5Configuration							M5Configuration						OPTIONAL,

-- The above IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”

m6Configuration							M6Configuration						OPTIONAL,

-- The above IE shall be present if the Measurements to Activate IE has the fifth bit set to “1”

m7Configuration							M7Configuration						OPTIONAL,

-- The above IE shall be present if the Measurements to Activate IE has the sixth bit set to “1”

bluetoothMeasurementConfiguration		BluetoothMeasurementConfiguration	OPTIONAL,

wLANMeasurementConfiguration			WLANMeasurementConfiguration 		OPTIONAL,

mDT-Location-Info 						MDT-Location-Info 					OPTIONAL,

sensorMeasurementConfiguration			SensorMeasurementConfiguration		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ImmediateMDTNr-ExtIEs} } OPTIONAL,

...

}

ImmediateMDTNr-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

InterSystemFailureIndication ::= SEQUENCE {

uERLFReportContainer		UERLFReportContainer	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { InterSystemFailureIndication-ExtIEs} }			OPTIONAL,

...

}

InterSystemFailureIndication-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

IntersystemSONConfigurationTransfer ::= SEQUENCE {

transferType				IntersystemSONTransferType,

intersystemSONInformation	IntersystemSONInformation,

iE-Extensions			ProtocolExtensionContainer { { IntersystemSONConfigurationTransfer-ExtIEs} }			OPTIONAL,

...

}

IntersystemSONConfigurationTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

IntersystemSONTransferType ::= CHOICE {

fromEUTRANtoNGRAN				FromEUTRANtoNGRAN,

fromNGRANtoEUTRAN				FromNGRANtoEUTRAN,

choice-Extensions		ProtocolIE-SingleContainer { { IntersystemSONTransferType-ExtIEs} }

}

IntersystemSONTransferType-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

IntersystemSONeNBID ::= SEQUENCE {

globaleNBID				GlobalENB-ID,

selectedEPSTAI			EPS-TAI,

iE-Extensions			ProtocolExtensionContainer { { IntersystemSONeNBID-ExtIEs} }			OPTIONAL,

...

}

IntersystemSONeNBID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

IntersystemSONNGRANnodeID ::= SEQUENCE {

globalRANNodeID		GlobalRANNodeID,

selectedTAI			TAI,

iE-Extensions		ProtocolExtensionContainer { { IntersystemSONNGRANnodeID-ExtIEs} }			OPTIONAL,

...

}

IntersystemSONNGRANnodeID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

IntersystemSONInformation ::= CHOICE {

intersystemSONInformationReport 	IntersystemSONInformationReport,

choice-Extensions		ProtocolIE-SingleContainer { { IntersystemSONInformation-ExtIEs} }

}

IntersystemSONInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-IntersystemSONInformationRequest		CRITICALITY ignore	TYPE IntersystemSONInformationRequest			PRESENCE mandatory }|

{ ID id-IntersystemSONInformationReply			CRITICALITY ignore	TYPE IntersystemSONInformationReply			PRESENCE mandatory },

...

}

-- --------------------------------------------------------------------

-- INTER SYSTEM SON INFORMATION REQUEST

-- --------------------------------------------------------------------

IntersystemSONInformationRequest ::= CHOICE {

nGRAN-CellActivation		IntersystemCellActivationRequest,

resourceStatus				IntersystemResourceStatusRequest,

choice-Extensions		ProtocolIE-SingleContainer { { IntersystemSONInformationRequest-ExtIEs} }

}

IntersystemSONInformationRequest-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

IntersystemCellActivationRequest ::= SEQUENCE {

activationID			INTEGER (0..16384, ...),

cellsToActivateList		CellsToActivateList,

iE-Extensions		ProtocolExtensionContainer { { IntersystemCellActivationRequest-ExtIEs} } OPTIONAL,

...

}

IntersystemCellActivationRequest-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CellsToActivateList ::= SEQUENCE (SIZE(1..maxnoofCellsinNGRANNode)) OF NGRAN-CGI

-- --------------------------------------------------------------------

-- Inter System Resource Status Request

-- --------------------------------------------------------------------

IntersystemResourceStatusRequest ::= SEQUENCE {

reportingSystem				ReportingSystem,

reportCharacteristics		ReportCharacteristics,

reportType					ReportType,

iE-Extensions		ProtocolExtensionContainer { { IntersystemResourceStatusRequest-ExtIEs} }	OPTIONAL,

...

}

IntersystemResourceStatusRequest-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ReportingSystem ::= CHOICE {

eUTRAN			EUTRAN-ReportingSystemIEs,

nGRAN			NGRAN-ReportingSystemIEs,

noReporting		NULL,

choice-Extensions		ProtocolIE-SingleContainer { { ReportingSystem-ExtIEs}}

}

ReportingSystem-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

EUTRAN-ReportingSystemIEs::= SEQUENCE {

eUTRAN-CellToReportList				EUTRAN-CellToReportList,

iE-Extensions		ProtocolExtensionContainer { {EUTRAN-ReportingSystemIEs-ExtIEs} } OPTIONAL,

...

}

EUTRAN-ReportingSystemIEs-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NGRAN-ReportingSystemIEs ::= SEQUENCE {

nGRAN-CellToReportList				NGRAN-CellToReportList,

iE-Extensions		ProtocolExtensionContainer { {NGRAN-ReportingSystemIEs-ExtIEs} } OPTIONAL,

...

}

NGRAN-ReportingSystemIEs-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EUTRAN-CellToReportList ::= SEQUENCE (SIZE(1..maxnoofReportedCells)) OF EUTRAN-CellToReportItem

EUTRAN-CellToReportItem::= SEQUENCE {

eCGI				EUTRA-CGI,

iE-Extensions		ProtocolExtensionContainer { {EUTRAN-CellToReportItem-ExtIEs} } OPTIONAL,

...

}

EUTRAN-CellToReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NGRAN-CellToReportList ::= SEQUENCE (SIZE(1.. maxnoofReportedCells)) OF NGRAN-CellToReportItem

NGRAN-CellToReportItem::= SEQUENCE {

nGRAN-CGI				NGRAN-CGI,

iE-Extensions		ProtocolExtensionContainer { {NGRAN-CellToReportItem-ExtIEs} } OPTIONAL,

...

}

NGRAN-CellToReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ReportCharacteristics ::=  BIT STRING(SIZE(32))

ReportType ::= CHOICE {

eventBasedReporting		EventBasedReportingIEs,

periodicReporting		PeriodicReportingIEs,

choice-Extensions		ProtocolIE-SingleContainer { { ReportType-ExtIEs}}

}

ReportType-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

EventBasedReportingIEs ::= SEQUENCE {

intersystemResourceThresholdLow				IntersystemResourceThreshold,

intersystemResourceThresholdHigh				IntersystemResourceThreshold,

numberOfMeasurementReportingLevels				NumberOfMeasurementReportingLevels,

iE-Extensions		ProtocolExtensionContainer { {EventBasedReportingIEs-ExtIEs} } OPTIONAL,

...

}

EventBasedReportingIEs-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

IntersystemResourceThreshold ::= INTEGER(0..100)

NumberOfMeasurementReportingLevels ::= ENUMERATED {n2, n3, n4, n5, n10, ..., n0}

PeriodicReportingIEs ::= SEQUENCE {

reportingPeriodicity			ReportingPeriodicity,

iE-Extensions		ProtocolExtensionContainer { {PeriodicReportingIEs-ExtIEs} } OPTIONAL,

...

}

PeriodicReportingIEs-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ReportingPeriodicity ::= ENUMERATED {

stop,

single,

ms1000,

ms2000,

ms5000,

ms10000,

...

}

-- --------------------------------------------------------------------

-- INTER SYSTEM SON INFORMATION REPLY

-- --------------------------------------------------------------------

IntersystemSONInformationReply ::= CHOICE {

nGRAN-CellActivation		IntersystemCellActivationReply,

resourceStatus				IntersystemResourceStatusReply,

choice-Extensions		ProtocolIE-SingleContainer { { IntersystemSONInformationReply-ExtIEs} }

}

IntersystemSONInformationReply-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

IntersystemCellActivationReply ::= SEQUENCE {

activatedCellList		ActivatedCellList,

activation-ID			INTEGER(0..16384, ...),

iE-Extensions			ProtocolExtensionContainer { { IntersystemCellActivationReply-ExtIEs} } OPTIONAL,

...

}

IntersystemCellActivationReply-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ActivatedCellList ::= SEQUENCE (SIZE(1..maxnoofCellsinNGRANNode)) OF NGRAN-CGI

-- --------------------------------------------------------------------

-- Inter System Resource Status Reply

-- --------------------------------------------------------------------

IntersystemResourceStatusReply ::= SEQUENCE {

reportingsystem				ReportingSystem,

iE-Extensions		ProtocolExtensionContainer { { IntersystemResourceStatusReply-ExtIEs} }		OPTIONAL,

...

}

IntersystemResourceStatusReply-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- --------------------------------------------------------------------

-- INTER SYSTEM SON INFORMATION REPORT

-- --------------------------------------------------------------------

IntersystemSONInformationReport::= CHOICE {

hOReportInformation				InterSystemHOReport,

failureIndicationInformation	InterSystemFailureIndication,

choice-Extensions		ProtocolIE-SingleContainer { { IntersystemSONInformationReport-ExtIEs} }

}

IntersystemSONInformationReport-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-EnergySavingIndication				CRITICALITY ignore	TYPE IntersystemCellStateIndication						PRESENCE mandatory }|

{ ID id-IntersystemResourceStatusUpdate		CRITICALITY ignore	TYPE IntersystemResourceStatusReport					PRESENCE mandatory },

...

}

IntersystemCellStateIndication ::= SEQUENCE {

notificationCellList	NotificationCellList,

iE-Extensions		ProtocolExtensionContainer { { IntersystemCellStateIndication-ExtIEs} } OPTIONAL,

...

}

IntersystemCellStateIndication-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NotificationCellList ::= SEQUENCE (SIZE(1.. maxnoofCellsinNGRANNode)) OF NotificationCell-Item

NotificationCell-Item ::= SEQUENCE {

nGRAN-CGI				NGRAN-CGI,

notifyFlag				ENUMERATED {activated, deactivated, ...},

iE-Extensions		ProtocolExtensionContainer { { NotificationCell-Item-ExtIEs} } OPTIONAL,

...

}

NotificationCell-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- --------------------------------------------------------------------

-- Inter System Resource Status Report

-- --------------------------------------------------------------------

IntersystemResourceStatusReport ::= SEQUENCE {

reportingSystem		ResourceStatusReportingSystem,

iE-Extensions					ProtocolExtensionContainer { { IntersystemResourceStatusReport-ExtIEs} }		OPTIONAL,

...

}

IntersystemResourceStatusReport-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ResourceStatusReportingSystem ::= CHOICE {

eUTRAN-ReportingStatus		EUTRAN-ReportingStatusIEs,

nGRAN-ReportingStatus		NGRAN-ReportingStatusIEs,

choice-Extensions			ProtocolIE-SingleContainer { { ResourceStatusReportingSystem-ExtIEs}}

}

ResourceStatusReportingSystem-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

EUTRAN-ReportingStatusIEs::= SEQUENCE {

eUTRAN-CellReportList						EUTRAN-CellReportList,

iE-Extensions		ProtocolExtensionContainer { {EUTRAN-ReportingStatusIEs-ExtIEs} }	OPTIONAL,

...

}

EUTRAN-ReportingStatusIEs-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EUTRAN-CellReportList ::= SEQUENCE (SIZE(1..maxnoofReportedCells)) OF EUTRAN-CellReportItem

EUTRAN-CellReportItem ::= SEQUENCE {

eCGI										EUTRA-CGI,

eUTRAN-CompositeAvailableCapacityGroup		EUTRAN-CompositeAvailableCapacityGroup,

eUTRAN-NumberOfActiveUEs					EUTRAN-NumberOfActiveUEs					OPTIONAL,

eUTRAN-NoofRRCConnections					NGRAN-NoofRRCConnections					OPTIONAL,

eUTRAN-RadioResourceStatus					EUTRAN-RadioResourceStatus					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {EUTRAN-CellReportItem-ExtIEs} }		OPTIONAL,

...

}

EUTRAN-CellReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EUTRAN-CompositeAvailableCapacityGroup	::= SEQUENCE {

dL-CompositeAvailableCapacity					CompositeAvailableCapacity,

uL-CompositeAvailableCapacity					CompositeAvailableCapacity,

iE-Extensions		ProtocolExtensionContainer { { EUTRAN-CompositeAvailableCapacityGroup-ExtIEs} }		OPTIONAL,

...

}

EUTRAN-CompositeAvailableCapacityGroup-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

CompositeAvailableCapacity ::= SEQUENCE {

cellCapacityClassValue		INTEGER (1..100, ...)				OPTIONAL,

capacityValue				INTEGER (0..100),

iE-Extensions		ProtocolExtensionContainer { {CompositeAvailableCapacity-ExtIEs} } OPTIONAL,

...

}

CompositeAvailableCapacity-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

EUTRAN-NumberOfActiveUEs ::= INTEGER (0..16777215, ...)

EUTRAN-RadioResourceStatus ::= SEQUENCE {

dL-GBR-PRB-usage							INTEGER (0..100),

uL-GBR-PRB-usage							INTEGER (0..100),

dL-non-GBR-PRB-usage						INTEGER (0..100),

uL-non-GBR-PRB-usage						INTEGER (0..100),

dL-Total-PRB-usage							INTEGER (0..100),

uL-Total-PRB-usage							INTEGER (0..100),

dL-scheduling-PDCCH-CCE-usage				INTEGER (0..100)	OPTIONAL,

uL-scheduling-PDCCH-CCE-usage				INTEGER (0..100)	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { {EUTRAN-RadioResourceStatus-ExtIEs} } OPTIONAL,

...

}

EUTRAN-RadioResourceStatus-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NGRAN-ReportingStatusIEs ::= SEQUENCE {

nGRAN-CellReportList						NGRAN-CellReportList,

iE-Extensions		ProtocolExtensionContainer { {NGRAN-ReportingStatusIEs-ExtIEs} }	OPTIONAL,

...

}

NGRAN-ReportingStatusIEs-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NGRAN-CellReportList ::= SEQUENCE (SIZE(1..maxnoofReportedCells)) OF NGRAN-CellReportItem

NGRAN-CellReportItem ::= SEQUENCE {

nGRAN-CGI									NGRAN-CGI,

nGRAN-CompositeAvailableCapacityGroup		EUTRAN-CompositeAvailableCapacityGroup,

nGRAN-NumberOfActiveUEs						NGRAN-NumberOfActiveUEs						OPTIONAL,

nGRAN-NoofRRCConnections					NGRAN-NoofRRCConnections					OPTIONAL,

nGRAN-RadioResourceStatus					NGRAN-RadioResourceStatus					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NGRAN-CellReportItem-ExtIEs} }		OPTIONAL,

...

}

NGRAN-CellReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NGRAN-NumberOfActiveUEs ::= INTEGER (0..16777215, ...)

NGRAN-NoofRRCConnections ::= INTEGER (1..65536, ...)

NGRAN-RadioResourceStatus	::= SEQUENCE {

dL-GBR-PRB-usage-for-MIMO				INTEGER (0..100),

uL-GBR-PRB-usage-for-MIMO				INTEGER (0..100),

dL-non-GBR-PRB-usage-for-MIMO			INTEGER (0..100),

uL-non-GBR-PRB-usage-for-MIMO			INTEGER (0..100),

dL-Total-PRB-usage-for-MIMO				INTEGER (0..100),

uL-Total-PRB-usage-for-MIMO				INTEGER (0..100),

iE-Extensions							ProtocolExtensionContainer { { NGRAN-RadioResourceStatus-ExtIEs} }	OPTIONAL,

...

}

NGRAN-RadioResourceStatus-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

InterSystemHOReport ::= SEQUENCE {

handoverReportType		InterSystemHandoverReportType,

iE-Extensions			ProtocolExtensionContainer { { InterSystemHOReport-ExtIEs} }			OPTIONAL,

...

}

InterSystemHOReport-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

InterSystemHandoverReportType ::= CHOICE {

tooearlyIntersystemHO				TooearlyIntersystemHO,

intersystemUnnecessaryHO			IntersystemUnnecessaryHO,

choice-Extensions		ProtocolIE-SingleContainer { { InterSystemHandoverReportType-ExtIEs} }

}

InterSystemHandoverReportType-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-IntersystemMobilityFailureforVoiceFallback  CRITICALITY ignore	TYPE  IntersystemMobilityFailureforVoiceFallback 	PRESENCE mandatory },

...

}

IntersystemMobilityFailureforVoiceFallback ::= SEQUENCE {

sourcecellID				NGRAN-CGI,

targetcellID				EUTRA-CGI,

reconnectCellID			EUTRA-CGI						OPTIONAL,

uERLFReportContainer		UERLFReportContainer				OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { IntersystemMobilityFailureforVoiceFallback-ExtIEs} }			OPTIONAL,

...

}

IntersystemMobilityFailureforVoiceFallback-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

IntersystemUnnecessaryHO ::= SEQUENCE {

sourcecellID			NGRAN-CGI,

targetcellID			EUTRA-CGI,

earlyIRATHO				ENUMERATED {true, false, ...},

candidateCellList		CandidateCellList,

iE-Extensions			ProtocolExtensionContainer { { IntersystemUnnecessaryHO-ExtIEs} }			OPTIONAL,

...

}

IntersystemUnnecessaryHO-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

InventoryRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { InventoryRequestTransferIEs} },

...

}

InventoryRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIoT-CorrelationIdentifier				CRITICALITY reject	TYPE AIoT-CorrelationIdentifier							PRESENCE	mandatory	}|

{ ID id-AIoT-DeviceIdentificationRequested		CRITICALITY reject	TYPE AIoT-DeviceIdentificationRequested				PRESENCE	mandatory	}|

{ ID id-AIoT-RequestedServiceAreaInformation	CRITICALITY reject	TYPE AIoT-RequestedServiceAreaInformation			PRESENCE mandatory	}|

{ ID id-AIoT-InventoryAssistanceInformation		CRITICALITY reject	TYPE AIoT-InventoryAssistanceInformation				PRESENCE	optional		}|

{ ID id-AIoT-FollowonCommandIndication			CRITICALITY reject	TYPE AIoT-FollowonCommandIndication					PRESENCE	optional		},

...

}

InventoryResponseTransfer ::= SEQUENCE {

correlationIdentifier		AIoT-CorrelationIdentifier,

criticalityDiagnostics		CriticalityDiagnostics													OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { InventoryResponseTransfer-ExtIEs} }	OPTIONAL,

...

}

InventoryResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

InventoryFailureTransfer ::= SEQUENCE {

correlationIdentifier		AIoT-CorrelationIdentifier,

cause						Cause,

criticalityDiagnostics		CriticalityDiagnostics													OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { InventoryFailureTransfer-ExtIEs} }		OPTIONAL,

...

}

InventoryFailureTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

InventoryReportTransfer ::= SEQUENCE {

correlationIdentifier		AIoT-CorrelationIdentifier,

globalgNB-ID				GlobalGNB-ID,

readerReportList			AIoT-ReaderReportList,

iE-Extensions				ProtocolExtensionContainer { { InventoryReportTransfer-ExtIEs} }		OPTIONAL,

...

}

InventoryReportTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- J

-- K

-- L

LAC	::= OCTET STRING (SIZE (2))

LAI ::= SEQUENCE {

pLMNidentity		PLMNIdentity,

lAC					LAC,

iE-Extensions		ProtocolExtensionContainer { {LAI-ExtIEs} } OPTIONAL,

...

}

LAI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LastVisitedCellInformation ::= CHOICE {

nGRANCell		LastVisitedNGRANCellInformation,

eUTRANCell		LastVisitedEUTRANCellInformation,

uTRANCell		LastVisitedUTRANCellInformation,

gERANCell		LastVisitedGERANCellInformation,

choice-Extensions		ProtocolIE-SingleContainer { {LastVisitedCellInformation-ExtIEs} }

}

LastVisitedCellInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

LastVisitedCellItem ::= SEQUENCE {

lastVisitedCellInformation		LastVisitedCellInformation,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedCellItem-ExtIEs} }	OPTIONAL,

...

}

LastVisitedCellItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LastVisitedEUTRANCellInformation ::= OCTET STRING

LastVisitedGERANCellInformation ::= OCTET STRING

LastVisitedNGRANCellInformation::= SEQUENCE {

globalCellID								NGRAN-CGI,

cellType									CellType,

timeUEStayedInCell							TimeUEStayedInCell,

timeUEStayedInCellEnhancedGranularity		TimeUEStayedInCellEnhancedGranularity 			OPTIONAL,

hOCauseValue								Cause		 									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedNGRANCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedNGRANCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-LastVisitedPSCellList	CRITICALITY ignore	EXTENSION LastVisitedPSCellList	PRESENCE optional },

...

}

LastVisitedPSCellList ::= SEQUENCE (SIZE(1..maxnoofPSCellsPerPrimaryCellinUEHistoryInfo)) OF LastVisitedPSCellInformation

LastVisitedPSCellInformation ::= SEQUENCE {

pSCellID			NGRAN-CGI	 			OPTIONAL,

timeStay			INTEGER (0..40950),

iE-Extensions		ProtocolExtensionContainer { {LastVisitedPSCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedPSCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LastVisitedUTRANCellInformation ::= OCTET STRING

LineType ::= ENUMERATED {

dsl,

pon,

...

}

LocationReportingAdditionalInfo ::= ENUMERATED {

includePSCell,

...

}

LocationReportingReferenceID ::= INTEGER (1..64, ...)

LocationReportingRequestType ::= SEQUENCE {

eventType										EventType,

reportArea										ReportArea,

areaOfInterestList								AreaOfInterestList								OPTIONAL,

locationReportingReferenceIDToBeCancelled		LocationReportingReferenceID					OPTIONAL,

-- The above IE shall be present if the Event Type IE is set to the value “stop UE presence in the area of interest”

iE-Extensions		ProtocolExtensionContainer { {LocationReportingRequestType-ExtIEs} }		OPTIONAL,

...

}

LocationReportingRequestType-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-LocationReportingAdditionalInfo	CRITICALITY ignore	EXTENSION LocationReportingAdditionalInfo		PRESENCE optional }|

{ ID id-AdditionalCancelledlocationReportingReferenceIDList	CRITICALITY reject	EXTENSION AdditionalCancelledlocationReportingReferenceIDList  PRESENCE optional },

...

}

LoggedMDTNr ::= SEQUENCE {

loggingInterval						LoggingInterval,

loggingDuration						LoggingDuration,

loggedMDTTrigger					LoggedMDTTrigger,

bluetoothMeasurementConfiguration	BluetoothMeasurementConfiguration	OPTIONAL,

wLANMeasurementConfiguration		WLANMeasurementConfiguration		OPTIONAL,

sensorMeasurementConfiguration		SensorMeasurementConfiguration		OPTIONAL,

areaScopeOfNeighCellsList			AreaScopeOfNeighCellsList			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LoggedMDTNr-ExtIEs} } OPTIONAL,

...

}

LoggedMDTNr-ExtIEs	NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-EarlyMeasurement		CRITICALITY ignore	EXTENSION EarlyMeasurement		PRESENCE optional	},

...

}

LoggingInterval ::= ENUMERATED {

ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960, ms61440,

infinity,

...

}

LoggingDuration ::= ENUMERATED {m10, m20, m40, m60, m90, m120, ...}

Links-to-log ::= ENUMERATED {

uplink,

downlink,

both-uplink-and-downlink,

...

}

LoggedMDTTrigger ::= CHOICE{

periodical			NULL,

eventTrigger			EventTrigger,

choice-Extensions		ProtocolIE-SingleContainer { {LoggedMDTTrigger-ExtIEs} }

}

LoggedMDTTrigger-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

LTEM-Indication	::= ENUMERATED {lte-m,...}

LTEUERLFReportContainer ::= OCTET STRING

LTEV2XServicesAuthorized ::= SEQUENCE {

vehicleUE			VehicleUE															OPTIONAL,

pedestrianUE 		PedestrianUE														OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LTEV2XServicesAuthorized-ExtIEs} }	OPTIONAL,

...

}

LTEV2XServicesAuthorized-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LTEUESidelinkAggregateMaximumBitrate ::= SEQUENCE {

uESidelinkAggregateMaximumBitRate		BitRate,

iE-Extensions		ProtocolExtensionContainer { {LTEUE-Sidelink-Aggregate-MaximumBitrates-ExtIEs} } OPTIONAL,

...

}

LTEUE-Sidelink-Aggregate-MaximumBitrates-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LTE-A2X-ServicesAuthorized ::= SEQUENCE {

aerialUE								ENUMERATED {authorized, not-authorized, ...}		OPTIONAL,

aerialControllerUE						ENUMERATED {authorized, not-authorized, ...}		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LTE-A2X-ServicesAuthorized-ExtIEs} }		OPTIONAL,

...

}

LTE-A2X-ServicesAuthorized-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- M

MaskedIMEISV ::= BIT STRING (SIZE(64))

MaximumDataBurstVolume ::= INTEGER (0..4095, ..., 4096.. 2000000)

MessageIdentifier ::= BIT STRING (SIZE(16))

MaximumIntegrityProtectedDataRate ::= ENUMERATED {

bitrate64kbs,

maximum-UE-rate,

...

}

MBS-AreaSessionID ::= INTEGER (0..65535, ...)

MBSCommServiceType ::= ENUMERATED {broadcast, multicast, ...}

MBS-DataForwardingResponseMRBList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MBS-DataForwardingResponseMRBItem

MBS-DataForwardingResponseMRBItem ::= SEQUENCE {

mRB-ID 								MRB-ID,

dL-Forwarding-UPTNLInformation		UPTransportLayerInformation,

mRB-ProgressInformation				MRB-ProgressInformation			OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { MBS-DataForwardingResponseMRBItem-ExtIEs} }	OPTIONAL,

...

}

MBS-DataForwardingResponseMRBItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-MappingandDataForwardingRequestList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MBS-MappingandDataForwardingRequestItem

MBS-MappingandDataForwardingRequestItem ::= SEQUENCE {

mRB-ID 								MRB-ID,

mBS-QoSFlowList						MBS-QoSFlowList,

mRB-ProgressInformation				MRB-ProgressInformation				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MBS-MappingandDataForwardingRequestItem-ExtIEs} }	OPTIONAL,

...

}

MBS-MappingandDataForwardingRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-QoSFlowList ::= SEQUENCE (SIZE(1..maxnoofMBSQoSFlows)) OF QosFlowIdentifier

MRB-ProgressInformation ::= CHOICE {

pDCP-SN-Length12				INTEGER (0..4095),

pDCP-SN-Length18				INTEGER (0..262143),

choice-Extensions		ProtocolIE-SingleContainer { { MRB-ProgressInformation-ExtIEs} }

}

MRB-ProgressInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

MBS-QoSFlowsToBeSetupList ::= SEQUENCE (SIZE(1.. maxnoofMBSQoSFlows)) OF MBS-QoSFlowsToBeSetupItem

MBS-QoSFlowsToBeSetupItem ::= SEQUENCE {

mBSqosFlowIdentifier				QosFlowIdentifier,

mBSqosFlowLevelQosParameters		QosFlowLevelQosParameters,

iE-Extensions						ProtocolExtensionContainer { {MBS-QoSFlowsToBeSetupItem-ExtIEs} }	OPTIONAL,

...

}

MBS-QoSFlowsToBeSetupItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceArea ::= CHOICE {

locationindependent		MBS-ServiceAreaInformation,

locationdependent		MBS-ServiceAreaInformationList,

choice-Extensions		ProtocolIE-SingleContainer { {MBS-ServiceArea-ExtIEs} }

}

MBS-ServiceArea-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

MBS-ServiceAreaInformationList ::= SEQUENCE (SIZE(1..maxnoofMBSServiceAreaInformation)) OF MBS-ServiceAreaInformationItem

MBS-ServiceAreaInformationItem ::= SEQUENCE {

mBS-AreaSessionID				MBS-AreaSessionID,

mBS-ServiceAreaInformation		MBS-ServiceAreaInformation,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformationItem-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaInformation ::= SEQUENCE {

mBS-ServiceAreaCellList		MBS-ServiceAreaCellList																OPTIONAL,

mBS-ServiceAreaTAIList		MBS-ServiceAreaTAIList																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformation-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaCellList ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBS)) OF NR-CGI

MBS-ServiceAreaTAIList ::= SEQUENCE (SIZE(1.. maxnoofTAIforMBS)) OF TAI

MBS-SessionID ::= SEQUENCE {

tMGI								TMGI,

nID								NID																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-SessionID-ExtIEs} } 		OPTIONAL,

...

}

MBS-SessionID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionFailedtoSetupList ::= SEQUENCE (SIZE(1.. maxnoofMBSSessions)) OF MBSSessionFailedtoSetupItem

MBSSessionFailedtoSetupItem ::= SEQUENCE {

mBS-SessionID			MBS-SessionID,

mBS-AreaSessionID		MBS-AreaSessionID															OPTIONAL,

cause					Cause,

iE-Extensions			ProtocolExtensionContainer { { MBSSessionFailedtoSetupItem-ExtIEs} }		OPTIONAL,

...

}

MBSSessionFailedtoSetupItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ActiveSessionInformation-SourcetoTargetList ::= SEQUENCE (SIZE(1..maxnoofMBSSessionsofUE)) OF MBS-ActiveSessionInformation-SourcetoTargetItem

MBS-ActiveSessionInformation-SourcetoTargetItem ::= SEQUENCE {

mBS-SessionID 								MBS-SessionID,

mBS-AreaSessionID							MBS-AreaSessionID							OPTIONAL,

mBS-ServiceArea								MBS-ServiceArea								OPTIONAL,

mBS-QoSFlowsToBeSetupList 					MBS-QoSFlowsToBeSetupList,

mBS-MappingandDataForwardingRequestList 	MBS-MappingandDataForwardingRequestList		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MBS-ActiveSessionInformation-SourcetoTargetItem-ExtIEs} }	OPTIONAL,

...

}

MBS-ActiveSessionInformation-SourcetoTargetItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ActiveSessionInformation-TargettoSourceList ::= SEQUENCE (SIZE(1..maxnoofMBSSessionsofUE)) OF MBS-ActiveSessionInformation-TargettoSourceItem

MBS-ActiveSessionInformation-TargettoSourceItem ::= SEQUENCE {

mBS-SessionID 						MBS-SessionID,

mBS-DataForwardingResponseMRBList	MBS-DataForwardingResponseMRBList													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MBS-ActiveSessionInformation-TargettoSourceItem-ExtIEs} }		OPTIONAL,

...

}

MBS-ActiveSessionInformation-TargettoSourceItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-AssistanceInformation ::= ENUMERATED {true, ...}

MBSSessionSetupOrModFailureTransfer ::= SEQUENCE {

cause						Cause,

criticalityDiagnostics		CriticalityDiagnostics		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MBSSessionSetupOrModFailureTransfer-ExtIEs} }	OPTIONAL,

...

}

MBSSessionSetupOrModFailureTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionSetupResponseList ::= SEQUENCE (SIZE(1.. maxnoofMBSSessions)) OF MBSSessionSetupResponseItem

MBSSessionSetupResponseItem ::= SEQUENCE {

mBS-SessionID			MBS-SessionID,

mBS-AreaSessionID		MBS-AreaSessionID													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MBSSessionSetupResponseItem-ExtIEs} }	OPTIONAL,

...

}

MBSSessionSetupResponseItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionSetupOrModRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MBSSessionSetupOrModRequestTransferIEs} },

...

}

MBSSessionSetupOrModRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionTNLInfo5GC				CRITICALITY reject	TYPE MBS-SessionTNLInfo5GC				PRESENCE	optional		}|

{ ID id-MBS-QoSFlowsToBeSetupModList		CRITICALITY reject	TYPE MBS-QoSFlowsToBeSetupList		PRESENCE	mandatory	}|

{ ID id-MBS-SessionFSAIDList				CRITICALITY ignore	TYPE MBS-SessionFSAIDList				PRESENCE	optional		}|

{ ID id-SupportedUETypeList					CRITICALITY ignore	TYPE SupportedUETypeList				PRESENCE	optional		}|

{ ID id-MBS-NGUFailureIndication			CRITICALITY reject	TYPE MBS-NGUFailureIndication			PRESENCE	optional		},

...

}

SupportedUETypeList ::= SEQUENCE (SIZE(1.. maxnoofUETypes)) OF SupportedUEType

SupportedUEType ::= ENUMERATED {non-RedCap-eRedCap-UE, redCap-eRedCap-UE, ...}

MBS-SessionFSAIDList ::= SEQUENCE (SIZE(1.. maxnoofMBSFSAs)) OF MBS-SessionFSAID

MBS-SessionFSAID ::= OCTET STRING (SIZE(3))

MBSSessionReleaseResponseTransfer ::= SEQUENCE {

mBS-SessionTNLInfoNGRAN		MBS-SessionTNLInfoNGRAN				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MBSSessionReleaseResponseTransfer-ExtIEs} }	OPTIONAL,

...

}

MBSSessionReleaseResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionSetupOrModResponseTransfer ::= SEQUENCE {

mBS-SessionTNLInfoNGRAN 	MBS-SessionTNLInfoNGRAN														OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MBSSessionSetupOrModResponseTransfer-ExtIEs} }		OPTIONAL,

...

}

MBSSessionSetupOrModResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-SupportIndicator ::= ENUMERATED {

multicast-supported,

...,

multicast-supported-with-reception-in-RRC-inactive

}

MBS-SessionTNLInfo5GC ::= CHOICE {

locationindependent			SharedNGU-MulticastTNLInformation,

locationdependent			MBS-SessionTNLInfo5GCList,

choice-Extensions		ProtocolIE-SingleContainer { {MBS-SessionTNLInfo5GC-ExtIEs} }

}

MBS-SessionTNLInfo5GC-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

MBS-SessionTNLInfo5GCList ::= SEQUENCE (SIZE(1..maxnoofMBSServiceAreaInformation)) OF MBS-SessionTNLInfo5GCItem

MBS-SessionTNLInfo5GCItem ::= SEQUENCE {

mBS-AreaSessionID						MBS-AreaSessionID,

sharedNGU-MulticastTNLInformation		SharedNGU-MulticastTNLInformation,

iE-Extensions		ProtocolExtensionContainer { {MBS-SessionTNLInfo5GCItem-ExtIEs} } 	OPTIONAL,

...

}

MBS-SessionTNLInfo5GCItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-SessionTNLInfoNGRAN ::= CHOICE {

locationindependent			UPTransportLayerInformation,

locationdependent			MBS-SessionTNLInfoNGRANList,

choice-Extensions		ProtocolIE-SingleContainer { {MBS-SessionTNLInfoNGRAN-ExtIEs} }

}

MBS-SessionTNLInfoNGRAN-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

MBS-SessionTNLInfoNGRANList ::= SEQUENCE (SIZE(1..maxnoofMBSServiceAreaInformation)) OF MBS-SessionTNLInfoNGRANItem

MBS-SessionTNLInfoNGRANItem ::= SEQUENCE {

mBS-AreaSessionID					MBS-AreaSessionID,

sharedNGU-UnicastTNLInformation		UPTransportLayerInformation								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MBS-SessionTNLInfoNGRANItem-ExtIEs} } 	OPTIONAL,

...

}

MBS-SessionTNLInfoNGRANItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-DistributionReleaseRequestTransfer ::= SEQUENCE {

mBS-SessionID							MBS-SessionID,

mBS-AreaSessionID						MBS-AreaSessionID												OPTIONAL,

sharedNGU-UnicastTNLInformation			UPTransportLayerInformation										OPTIONAL,

cause									Cause,

iE-Extensions		ProtocolExtensionContainer { { MBS-DistributionReleaseRequesTransfer-ExtIEs} } 	OPTIONAL,

...

}

MBS-DistributionReleaseRequesTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-DistributionSetupRequestTransfer ::= SEQUENCE {

mBS-SessionID							MBS-SessionID,

mBS-AreaSessionID						MBS-AreaSessionID												OPTIONAL,

sharedNGU-UnicastTNLInformation			UPTransportLayerInformation										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MBS-DistributionSetupRequestTransfer-ExtIEs} } 	OPTIONAL,

...

}

MBS-DistributionSetupRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-DistributionSetupResponseTransfer ::= SEQUENCE {

mBS-SessionID								MBS-SessionID,

mBS-AreaSessionID							MBS-AreaSessionID						OPTIONAL,

sharedNGU-MulticastTNLInformation			SharedNGU-MulticastTNLInformation		OPTIONAL,

mBS-QoSFlowsToBeSetupList					MBS-QoSFlowsToBeSetupList,

mBSSessionStatus							MBSSessionStatus,

mBS-ServiceArea								MBS-ServiceArea							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MBS-DistributionSetupResponseTransfer-ExtIEs} }	OPTIONAL,

...

}

MBS-DistributionSetupResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-DistributionSetupUnsuccessfulTransfer ::= SEQUENCE {

mBS-SessionID				MBS-SessionID,

mBS-AreaSessionID			MBS-AreaSessionID																	OPTIONAL,

cause						Cause,

criticalityDiagnostics		CriticalityDiagnostics																OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MBS-DistributionSetupUnsuccessfulTransfer-ExtIEs} } 	OPTIONAL,

...

}

MBS-DistributionSetupUnsuccessfulTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-NGUFailureIndication ::= CHOICE {

locationindependent			MBS-UP-FailureIndication,

locationdependent			MBS-UP-FailureIndicationList,

choice-Extensions		ProtocolIE-SingleContainer { {MBS-NGUFailureIndication-ExtIEs} }

}

MBS-NGUFailureIndication-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

MBS-UP-FailureIndicationList ::= SEQUENCE (SIZE(1..maxnoofMBSServiceAreaInformation)) OF MBS-UP-FailureIndicationItem

MBS-UP-FailureIndicationItem ::= SEQUENCE {

mBS-AreaSessionID					MBS-AreaSessionID,

mBS-UP-FailureIndication			MBS-UP-FailureIndication,

iE-Extensions		ProtocolExtensionContainer { {MBS-UP-FailureIndicationItem-ExtIEs} } 	OPTIONAL,

...

}

MBS-UP-FailureIndicationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-UP-FailureIndication ::= ENUMERATED {ng-u-path-failure-detected, ...}

MBSSessionSetupRequestList ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBSSessionSetupRequestItem

MBSSessionSetupRequestItem ::= SEQUENCE {

mBS-SessionID									MBS-SessionID,

mBS-AreaSessionID								MBS-AreaSessionID																	OPTIONAL,

associatedMBSQosFlowSetupRequestList			AssociatedMBSQosFlowSetupRequestList												OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { MBSSessionSetupRequestItem-ExtIEs} }		OPTIONAL,

...

}

MBSSessionSetupRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-AssistanceInformation			CRITICALITY ignore	EXTENSION MBS-AssistanceInformation		PRESENCE	optional		},

...

}

MBSSessionSetuporModifyRequestList ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBSSessionSetuporModifyRequestItem

MBSSessionSetuporModifyRequestItem ::= SEQUENCE {

mBS-SessionID										MBS-SessionID,

mBS-AreaSessionID									MBS-AreaSessionID									OPTIONAL,

associatedMBSQosFlowSetuporModifyRequestList		AssociatedMBSQosFlowSetuporModifyRequestList		OPTIONAL,

mBS-QosFlowToReleaseList 							QosFlowListWithCause								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{MBSSessionSetuporModifyRequestItem-ExtIEs}}			OPTIONAL,

...

}

MBSSessionSetuporModifyRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-AssistanceInformation			CRITICALITY ignore	EXTENSION MBS-AssistanceInformation		PRESENCE	optional		},

...

}

MBSSessionToReleaseList ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBSSessionToReleaseItem

MBSSessionToReleaseItem ::= SEQUENCE {

mBS-SessionID		MBS-SessionID,

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { { MBSSessionToReleaseItem-ExtIEs} }	OPTIONAL,

...

}

MBSSessionToReleaseItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionStatus ::= ENUMERATED {

activated,

deactivated,

...

}

MicoAllPLMN ::= ENUMERATED {

true,

...

}

MICOModeIndication ::= ENUMERATED {

true,

...

}

MobilityInformation ::= BIT STRING (SIZE(16))

ExtendedMobilityInformation ::= BIT STRING (SIZE(32))

MobilityRestrictionList ::= SEQUENCE {

servingPLMN					PLMNIdentity,

equivalentPLMNs				EquivalentPLMNs											OPTIONAL,

rATRestrictions				RATRestrictions											OPTIONAL,

forbiddenAreaInformation	ForbiddenAreaInformation								OPTIONAL,

serviceAreaInformation		ServiceAreaInformation									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MobilityRestrictionList-ExtIEs} }	OPTIONAL,

...

}

MobilityRestrictionList-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-LastEUTRAN-PLMNIdentity				CRITICALITY ignore	EXTENSION PLMNIdentity								PRESENCE optional		}|

{ ID id-CNTypeRestrictionsForServing		CRITICALITY ignore	EXTENSION CNTypeRestrictionsForServing			PRESENCE optional		}|

{ ID id-CNTypeRestrictionsForEquivalent		CRITICALITY ignore	EXTENSION CNTypeRestrictionsForEquivalent		PRESENCE optional		}|

{ ID id-NPN-MobilityInformation				CRITICALITY reject	EXTENSION NPN-MobilityInformation				PRESENCE optional		},

...

}

MDT-AlignmentInfo ::= CHOICE {

s-basedMDT				NGRANTraceID,

choice-Extensions		ProtocolIE-SingleContainer { { MDT-AlignmentInfo-ExtIEs} }

}

MDT-AlignmentInfo-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

MDTPLMNList ::= SEQUENCE (SIZE(1..maxnoofMDTPLMNs)) OF PLMNIdentity

MDTPLMNModificationList ::= SEQUENCE (SIZE(0..maxnoofMDTPLMNs)) OF PLMNIdentity

MDT-Configuration ::= SEQUENCE {

mdt-Config-NR		MDT-Configuration-NR		OPTIONAL,

mdt-Config-EUTRA	MDT-Configuration-EUTRA		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MDT-Configuration-ExtIEs} } OPTIONAL,

...

}

MDT-Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MN-only-MDT-collection		CRITICALITY ignore	EXTENSION MN-only-MDT-collection		PRESENCE optional		},

...

}

MN-only-MDT-collection ::= ENUMERATED {mn-only, ...}

MDT-Configuration-NR ::= SEQUENCE {

mdt-Activation				MDT-Activation,

areaScopeOfMDT				AreaScopeOfMDT-NR,

mDTModeNr					MDTModeNr,

signallingBasedMDTPLMNList 	MDTPLMNList												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MDT-Configuration-NR-ExtIEs} } 	OPTIONAL,

...

}

MDT-Configuration-NR-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PNI-NPN-AreaScopeofMDT		CRITICALITY ignore	EXTENSION PNI-NPN-AreaScopeofMDT		PRESENCE optional		},

...

}

MDT-Configuration-EUTRA ::= SEQUENCE {

mdt-Activation				MDT-Activation,

areaScopeOfMDT				AreaScopeOfMDT-EUTRA,

mDTMode						MDTModeEutra,

signallingBasedMDTPLMNList 	MDTPLMNList													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MDT-Configuration-EUTRA-ExtIEs} } 	OPTIONAL,

...

}

MDT-Configuration-EUTRA-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MDT-Activation 	::= ENUMERATED {

immediate-MDT-only,

logged-MDT-only,

immediate-MDT-and-Trace,

...

}

MDTModeNr ::= CHOICE {

immediateMDTNr				ImmediateMDTNr,

loggedMDTNr					LoggedMDTNr,

choice-Extensions		ProtocolIE-SingleContainer { {MDTModeNr-ExtIEs} }

}

MDTModeNr-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

MDTModeEutra ::= OCTET STRING

MeasurementsToActivate ::= BIT STRING(SIZE(8))

MobileIAB-Authorized ::= ENUMERATED {

authorized,

not-authorized,

...

}

MobileIABNodeIndication ::= ENUMERATED {

true,

...

}

MobileIAB-Supported ::= ENUMERATED {

true,

...

}

MRB-ID ::= INTEGER (1..512, ...)

MulticastSessionActivationRequestTransfer ::= SEQUENCE {

mBS-SessionID				MBS-SessionID,

iE-Extensions				ProtocolExtensionContainer { { MulticastSessionActivationRequestTransfer-ExtIEs} } 	OPTIONAL,

...

}

MulticastSessionActivationRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MulticastSessionDeactivationRequestTransfer ::= SEQUENCE {

mBS-SessionID				MBS-SessionID,

iE-Extensions				ProtocolExtensionContainer { { MulticastSessionDeactivationRequestTransfer-ExtIEs} } 	OPTIONAL,

...

}

MulticastSessionDeactivationRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MulticastSessionUpdateRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastSessionUpdateRequestTransferIEs} },

...

}

MulticastSessionUpdateRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID						CRITICALITY reject	TYPE MBS-SessionID						PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea						CRITICALITY reject	TYPE MBS-ServiceArea					PRESENCE	optional		}|

{ ID id-MBS-QoSFlowsToBeSetupModList		CRITICALITY reject	TYPE MBS-QoSFlowsToBeSetupList		PRESENCE	optional		}|

{ ID id-MBS-QoSFlowToReleaseList			CRITICALITY reject	TYPE QosFlowListWithCause				PRESENCE	optional		}|

{ ID id-MBS-SessionTNLInfo5GC				CRITICALITY reject	TYPE MBS-SessionTNLInfo5GC				PRESENCE	optional		},

...

}

MulticastGroupPagingAreaList ::= SEQUENCE (SIZE(1..maxnoofPagingAreas)) OF MulticastGroupPagingAreaItem

MulticastGroupPagingAreaItem ::= SEQUENCE {

multicastGroupPagingArea		MulticastGroupPagingArea,

uE-PagingList					UE-PagingList 												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MulticastGroupPagingAreaItem-ExtIEs} }	OPTIONAL,

...

}

MulticastGroupPagingAreaItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-AreaTAIList ::= SEQUENCE (SIZE(1..maxnoofTAIforPaging)) OF TAI

MulticastGroupPagingArea ::= SEQUENCE {

mBS-AreaTAIList		MBS-AreaTAIList,

iE-Extensions		ProtocolExtensionContainer { { MulticastGroupPagingArea-ExtIEs} } OPTIONAL,

...

}

MulticastGroupPagingArea-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UE-PagingList ::= SEQUENCE (SIZE(1..maxnoofUEsforPaging)) OF UE-PagingItem

UE-PagingItem ::= SEQUENCE {

uEIdentityIndexValue		UEIdentityIndexValue,

pagingDRX					PagingDRX 										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { UE-PagingItem-ExtIEs} }	OPTIONAL,

...

}

UE-PagingItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

M1Configuration ::= SEQUENCE {

m1reportingTrigger			M1ReportingTrigger,

m1thresholdEventA2			M1ThresholdEventA2									OPTIONAL,

--	The above IE shall be present if the M1 Reporting Trigger IE is set to the value “A2event-triggered” or “A2event-triggered periodic”

m1periodicReporting			M1PeriodicReporting									OPTIONAL,

--	The above IE shall be present if the M1 Reporting Trigger IE is set to the value “periodic” or “A2event-triggered periodic”

iE-Extensions		ProtocolExtensionContainer { { M1Configuration-ExtIEs} } 	OPTIONAL,

...

}

M1Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-IncludeBeamMeasurementsIndication		CRITICALITY ignore	EXTENSION IncludeBeamMeasurementsIndication		PRESENCE optional			}|

{ ID id-BeamMeasurementsReportConfiguration		CRITICALITY ignore	EXTENSION BeamMeasurementsReportConfiguration	PRESENCE conditional		},

-- The above IE shall be present if the Include Beam Measurements Indication IE is set to the value “true”

...

}

IncludeBeamMeasurementsIndication ::= ENUMERATED {

true,

...

}

MaxNrofRS-IndexesToReport ::= INTEGER (1..64, ...)

M1ReportingTrigger ::= ENUMERATED {

periodic,

a2eventtriggered,

a2eventtriggered-periodic,

...

}

M1ThresholdEventA2 ::= SEQUENCE {

m1ThresholdType		M1ThresholdType,

iE-Extensions		ProtocolExtensionContainer { { M1ThresholdEventA2-ExtIEs} } OPTIONAL,

...

}

M1ThresholdEventA2-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

M1ThresholdType ::= CHOICE {

threshold-RSRP				Threshold-RSRP,

threshold-RSRQ				Threshold-RSRQ,

threshold-SINR				Threshold-SINR,

choice-Extensions		ProtocolIE-SingleContainer { {M1ThresholdType-ExtIEs} }

}

M1ThresholdType-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

M1PeriodicReporting ::= SEQUENCE {

reportInterval				ReportIntervalMDT,

reportAmount				ReportAmountMDT,

iE-Extensions		ProtocolExtensionContainer { { M1PeriodicReporting-ExtIEs} } OPTIONAL,

...

}

M1PeriodicReporting-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-ExtendedReportIntervalMDT		CRITICALITY ignore	EXTENSION ExtendedReportIntervalMDT		PRESENCE optional},

...

}

M4Configuration ::= SEQUENCE {

m4period			M4period,

m4-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M4Configuration-ExtIEs} } OPTIONAL,

...

}

M4Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-M4ReportAmount		CRITICALITY ignore	EXTENSION M4ReportAmountMDT		PRESENCE optional		},

...

}

M4ReportAmountMDT ::= ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity, ...}

M4period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M5Configuration ::= SEQUENCE {

m5period			M5period,

m5-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M5Configuration-ExtIEs} } OPTIONAL,

...

}

M5Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-M5ReportAmount		CRITICALITY ignore	EXTENSION M5ReportAmountMDT		PRESENCE optional		},

...

}

M5ReportAmountMDT ::= ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity, ...}

M5period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M6Configuration ::= SEQUENCE {

m6report-Interval	M6report-Interval,

m6-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-M6ReportAmount		CRITICALITY ignore	EXTENSION M6ReportAmountMDT		PRESENCE optional		}|

{ ID id-ExcessPacketDelayThresholdConfiguration	CRITICALITY ignore	EXTENSION ExcessPacketDelayThresholdConfiguration		PRESENCE optional		},

...

}

M6ReportAmountMDT ::= ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity, ...}

M6report-Interval ::= ENUMERATED {

ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30,

...

}

M7Configuration ::= SEQUENCE {

m7period			M7period,

m7-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M7Configuration-ExtIEs} } OPTIONAL,

...

}

M7Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-M7ReportAmount		CRITICALITY ignore	EXTENSION M7ReportAmountMDT		PRESENCE optional		},

...

}

M7ReportAmountMDT ::= ENUMERATED {r1, r2, r4, r8, r16, r32, r64, infinity, ...}

M7period ::= INTEGER(1..60, ...)

MDT-Location-Info ::= SEQUENCE {

mDT-Location-Information	MDT-Location-Information,

iE-Extensions		ProtocolExtensionContainer { { MDT-Location-Info-ExtIEs} } OPTIONAL,

...

}

MDT-Location-Info-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MDT-Location-Information::= BIT STRING (SIZE (8))

-- N

N3IWF-ID ::= CHOICE {

n3IWF-ID				BIT STRING (SIZE(16)),

choice-Extensions		ProtocolIE-SingleContainer { {N3IWF-ID-ExtIEs} }

}

N3IWF-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

N6JitterInformation ::= SEQUENCE {

n6JitterLowerBound		INTEGER (-127..127),

n6JitterUpperBound		INTEGER (-127..127),

iE-Extensions			ProtocolExtensionContainer { { N6JitterInformation-ExtIEs} }	OPTIONAL,

...

}

N6JitterInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NAS-PDU ::= OCTET STRING

NASSecurityParametersFromNGRAN ::= OCTET STRING

NB-IoT-DefaultPagingDRX ::= ENUMERATED {

rf128, rf256, rf512, rf1024,

...

}

NB-IoT-PagingDRX ::= ENUMERATED {

rf32, rf64, rf128, rf256, rf512, rf1024,

...

}

NB-IoT-Paging-eDRXCycle ::= ENUMERATED {

hf2, hf4, hf6, hf8, hf10, hf12, hf14, hf16, hf32, hf64, hf128, hf256, hf512, hf1024,

...

}

NB-IoT-Paging-TimeWindow ::= ENUMERATED {

s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16,

...

}

NB-IoT-Paging-eDRXInfo ::= SEQUENCE {

nB-IoT-Paging-eDRXCycle 	NB-IoT-Paging-eDRXCycle,

nB-IoT-Paging-TimeWindow 	NB-IoT-Paging-TimeWindow 								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { NB-IoT-Paging-eDRXInfo-ExtIEs} } OPTIONAL,

...

}

NB-IoT-Paging-eDRXInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NB-IoT-UEPriority ::= INTEGER (0..255, ...)

NetworkControlledRepeaterAuthorized ::= ENUMERATED {authorized, not-authorized, ...}

NetworkInstance ::= INTEGER (1..256, ...)

NewSecurityContextInd ::= ENUMERATED {

true,

...

}

NextHopChainingCount ::= INTEGER (0..7)

NextPagingAreaScope ::= ENUMERATED {

same,

changed,

...

}

NGAPIESupportInformationRequestList ::= SEQUENCE (SIZE(1.. maxnoofNGAPIESupportInfo)) OF NGAPIESupportInformationRequestItem

NGAPIESupportInformationRequestItem ::= SEQUENCE {

ngap-ProtocolIE-Id		ProtocolIE-ID,

iE-Extensions		ProtocolExtensionContainer { { NGAPIESupportInformationRequestItem-ExtIEs} }	OPTIONAL,

...

}

NGAPIESupportInformationRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NGAPIESupportInformationResponseList ::= SEQUENCE (SIZE(1.. maxnoofNGAPIESupportInfo)) OF NGAPIESupportInformationResponseItem

NGAPIESupportInformationResponseItem ::= SEQUENCE {

ngap-ProtocolIE-Id				ProtocolIE-ID,

ngap-ProtocolIESupportInfo		ENUMERATED {supported, not-supported, ...},

ngap-ProtocolIEPresenceInfo		ENUMERATED {present, not-present, ...},

iE-Extensions		ProtocolExtensionContainer { { NGAPIESupportInformationResponseItem-ExtIEs} }	OPTIONAL,

...

}

NGAPIESupportInformationResponseItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NgENB-ID ::= CHOICE {

macroNgENB-ID			BIT STRING (SIZE(20)),

shortMacroNgENB-ID		BIT STRING (SIZE(18)),

longMacroNgENB-ID		BIT STRING (SIZE(21)),

choice-Extensions		ProtocolIE-SingleContainer { {NgENB-ID-ExtIEs} }

}

NgENB-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

NotifySourceNGRANNode ::= ENUMERATED {

notifySource,

...

}

NGRAN-CGI ::= CHOICE {

nR-CGI			NR-CGI,

eUTRA-CGI		EUTRA-CGI,

choice-Extensions		ProtocolIE-SingleContainer { {NGRAN-CGI-ExtIEs} }

}

NGRAN-CGI-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

NGRAN-TNLAssociationToRemoveList ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF NGRAN-TNLAssociationToRemoveItem

NGRAN-TNLAssociationToRemoveItem::= SEQUENCE {

tNLAssociationTransportLayerAddress			CPTransportLayerInformation,

tNLAssociationTransportLayerAddressAMF		CPTransportLayerInformation								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { NGRAN-TNLAssociationToRemoveItem-ExtIEs} } 	OPTIONAL

}

NGRAN-TNLAssociationToRemoveItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NGRANTraceID ::= OCTET STRING (SIZE(8))

NID ::= BIT STRING (SIZE(44))

NonDynamic5QIDescriptor ::= SEQUENCE {

fiveQI						FiveQI,

priorityLevelQos			PriorityLevelQos										OPTIONAL,

averagingWindow				AveragingWindow											OPTIONAL,

maximumDataBurstVolume		MaximumDataBurstVolume									OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { {NonDynamic5QIDescriptor-ExtIEs} }	OPTIONAL,

...

}

NonDynamic5QIDescriptor-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-CNPacketDelayBudgetDL	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-CNPacketDelayBudgetUL	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		},

...

}

NoPDUSessionIndication ::= ENUMERATED {true, ...}

NotAllowedTACs ::= SEQUENCE (SIZE(1..maxnoofAllowedAreas)) OF TAC

NotificationCause ::= ENUMERATED {

fulfilled,

not-fulfilled,

...

}

NotificationControl ::= ENUMERATED {

notification-requested,

...

}

NPN-AccessInformation ::= CHOICE {

pNI-NPN-Access-Information		CellCAGList,

choice-Extensions				ProtocolIE-SingleContainer { {NPN-AccessInformation-ExtIEs} }

}

NPN-AccessInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

NPN-MobilityInformation ::= CHOICE {

sNPN-MobilityInformation		SNPN-MobilityInformation,

pNI-NPN-MobilityInformation		PNI-NPN-MobilityInformation,

choice-Extensions				ProtocolIE-SingleContainer { {NPN-MobilityInformation-ExtIEs} }

}

NPN-MobilityInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

NPN-PagingAssistanceInformation ::= CHOICE {

pNI-NPN-PagingAssistance		Allowed-PNI-NPN-List,

choice-Extensions				ProtocolIE-SingleContainer { {NPN-PagingAssistanceInformation-ExtIEs} }

}

NPN-PagingAssistanceInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

NPN-Support ::= CHOICE {

sNPN					NID,

choice-Extensions		ProtocolIE-SingleContainer { {NPN-Support-ExtIEs} }

}

NPN-Support-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

NRCellIdentity ::= BIT STRING (SIZE(36))

NR-CGI ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

nRCellIdentity		NRCellIdentity,

iE-Extensions		ProtocolExtensionContainer { {NR-CGI-ExtIEs} } OPTIONAL,

...

}

NR-CGI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NR-CGIList ::= SEQUENCE (SIZE(1..maxnoofCellsingNB)) OF NR-CGI

NR-CGIListForWarning ::= SEQUENCE (SIZE(1..maxnoofCellIDforWarning)) OF NR-CGI

NR-PagingeDRXInformation ::= SEQUENCE {

nR-paging-eDRX-Cycle		NR-Paging-eDRX-Cycle,

nR-paging-Time-Window		NR-Paging-Time-Window					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NR-PagingeDRXInformation-ExtIEs} }	OPTIONAL,

...

}

NR-PagingeDRXInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NR-Paging-eDRX-Cycle ::= ENUMERATED {

hfquarter, hfhalf, hf1, hf2, hf4, hf8, hf16,

hf32, hf64, hf128, hf256, hf512, hf1024,

...

}

NR-Paging-Long-eDRX-Information-for-RRC-INACTIVE ::= SEQUENCE {

nR-paging-Long-eDRX-Cycle-for-RRC-INACTIVE		NR-Paging-Long-eDRX-Cycle-for-RRC-INACTIVE,

nR-paging-Time-Window-for-RRC-INACTIVE			NR-Paging-Time-Window-for-RRC-INACTIVE,

iE-Extensions		ProtocolExtensionContainer { {NR-Paging-Long-eDRX-Information-for-RRC-INACTIVE-ExtIEs} }	OPTIONAL,

...

}

NR-Paging-Long-eDRX-Information-for-RRC-INACTIVE-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NR-Paging-Long-eDRX-Cycle-for-RRC-INACTIVE	::= ENUMERATED {

hf2, hf4, hf8, hf16,

hf32, hf64, hf128, hf256, hf512, hf1024,

...

}

NR-Paging-Time-Window ::= ENUMERATED {

s1, s2, s3, s4, s5,

s6, s7, s8, s9, s10,

s11, s12, s13, s14, s15, s16,

...,

s17, s18, s19, s20, s21, s22, s23, s24,

s25, s26, s27, s28, s29, s30, s31, s32

}

NR-Paging-Time-Window-for-RRC-INACTIVE ::= ENUMERATED {

s1, s2, s3, s4, s5,

s6, s7, s8, s9, s10,

s11, s12, s13, s14, s15, s16,

s17, s18, s19, s20, s21, s22, s23, s24,

s25, s26, s27, s28, s29, s30, s31, s32,

...

}

NRencryptionAlgorithms ::= BIT STRING (SIZE(16, ...))

NRintegrityProtectionAlgorithms ::= BIT STRING (SIZE(16, ...))

NRMobilityHistoryReport ::= OCTET STRING

NRPPa-PDU ::= OCTET STRING

NRUERLFReportContainer ::= OCTET STRING

NRNTNTAIInformation	::= SEQUENCE {

servingPLMN							PLMNIdentity,

tACListInNRNTN						TACListInNRNTN,

uELocationDerivedTACInNRNTN			TAC							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { NRNTNTAIInformation-ExtIEs} } OPTIONAL,

...

}

NRNTNTAIInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NumberOfBroadcasts ::= INTEGER (0..65535)

NumberOfBroadcastsRequested ::= INTEGER (0..65535)

NRARFCN	::= INTEGER (0.. maxNRARFCN)

NRFrequencyBand ::= INTEGER (1..1024, ...)

NRFrequencyBand-List ::= SEQUENCE (SIZE(1..maxnoofNRCellBands)) OF NRFrequencyBandItem

NRFrequencyBandItem ::= SEQUENCE {

nr-frequency-band			NRFrequencyBand,

iE-Extension		ProtocolExtensionContainer { {NRFrequencyBandItem-ExtIEs} } 	OPTIONAL,

...

}

NRFrequencyBandItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NRFrequencyInfo ::= SEQUENCE {

nrARFCN				NRARFCN,

frequencyBand-List		NRFrequencyBand-List,

iE-Extension		ProtocolExtensionContainer { {NRFrequencyInfo-ExtIEs} }		OPTIONAL,

...

}

NRFrequencyInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NR-PCI ::= INTEGER (0..1007, ...)

NRV2XServicesAuthorized ::= SEQUENCE {

vehicleUE			VehicleUE														OPTIONAL,

pedestrianUE 		PedestrianUE													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NRV2XServicesAuthorized-ExtIEs} }	OPTIONAL,

...

}

NRV2XServicesAuthorized-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

VehicleUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

PedestrianUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

NRUESidelinkAggregateMaximumBitrate ::= SEQUENCE {

uESidelinkAggregateMaximumBitRate		BitRate,

iE-Extensions		ProtocolExtensionContainer { {NRUESidelinkAggregateMaximumBitrate-ExtIEs} } OPTIONAL,

...

}

NRUESidelinkAggregateMaximumBitrate-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

NSAG-ID ::= INTEGER (0..255, ...)

NR-A2X-ServicesAuthorized ::= SEQUENCE {

aerialUE								ENUMERATED {authorized, not-authorized, ...}		OPTIONAL,

aerialControllerUE						ENUMERATED {authorized, not-authorized, ...}		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NR-A2X-ServicesAuthorized-ExtIEs} }		OPTIONAL,

...

}

NR-A2X-ServicesAuthorized-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- O

OnboardingSupport ::= ENUMERATED {

true,

...

}

OverloadAction ::= ENUMERATED {

reject-non-emergency-mo-dt,

reject-rrc-cr-signalling,

permit-emergency-sessions-and-mobile-terminated-services-only,

permit-high-priority-sessions-and-mobile-terminated-services-only,

...

}

OverloadResponse ::= CHOICE {

overloadAction			OverloadAction,

choice-Extensions		ProtocolIE-SingleContainer { {OverloadResponse-ExtIEs} }

}

OverloadResponse-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

OverloadStartNSSAIList ::= SEQUENCE (SIZE (1..maxnoofSliceItems)) OF OverloadStartNSSAIItem

OverloadStartNSSAIItem ::= SEQUENCE {

sliceOverloadList						SliceOverloadList,

sliceOverloadResponse					OverloadResponse							OPTIONAL,

sliceTrafficLoadReductionIndication		TrafficLoadReductionIndication				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {OverloadStartNSSAIItem-ExtIEs} }	OPTIONAL,

...

}

OverloadStartNSSAIItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- P

PacketDelayBudget ::= INTEGER (0..1023, ...)

PacketErrorRate ::= SEQUENCE {

pERScalar		INTEGER (0..9, ...),

pERExponent		INTEGER (0..9, ...),

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PacketLossRate ::= INTEGER (0..1000, ...)

PagingAssisDataforCEcapabUE ::= SEQUENCE {

eUTRA-CGI							EUTRA-CGI,

coverageEnhancementLevel			CoverageEnhancementLevel,

iE-Extensions						ProtocolExtensionContainer { { PagingAssisDataforCEcapabUE-ExtIEs} } OPTIONAL,

...

}

PagingAssisDataforCEcapabUE-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PagingAttemptInformation ::= SEQUENCE {

pagingAttemptCount					PagingAttemptCount,

intendedNumberOfPagingAttempts		IntendedNumberOfPagingAttempts,

nextPagingAreaScope					NextPagingAreaScope									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PagingAttemptInformation-ExtIEs} }	OPTIONAL,

...

}

PagingAttemptInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PagingAttemptCount ::= INTEGER (1..16, ...)

PagingCause ::= ENUMERATED {

voice,

...

}

PagingCauseIndicationForVoiceService ::= ENUMERATED {

supported,

...

}

PagingDRX ::= ENUMERATED {

v32,

v64,

v128,

v256,

...

}

PagingOrigin ::= ENUMERATED {

non-3gpp,

...

}

PagingPolicyDifferentiation ::= SEQUENCE {

pDUSessionForPagingList			PDUSessionForPagingList,

iE-Extensions					ProtocolExtensionContainer { { PagingPolicyDifferentiation-ExtIEs} }	OPTIONAL,

...

}

PagingPolicyDifferentiation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionForPagingList	::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionForPagingItem

PDUSessionForPagingItem ::= SEQUENCE {

pDUSessionID						PDUSessionID,

pagingPolicyDifferentiationList		PagingPolicyDifferentiationList,

iE-Extensions						ProtocolExtensionContainer { { PDUSessionForPagingItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionForPagingItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PagingPolicyDifferentiationList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF PagingPolicyDifferentiationItem

PagingPolicyDifferentiationItem ::= SEQUENCE {

qosFlowIdentifier					QosFlowIdentifier					OPTIONAL,

pagingPolicyIndicator				PagingPolicyIndicator				OPTIONAL,

allocationAndRetentionPriority		AllocationAndRetentionPriority		OPTIONAL,

fiveQI								FiveQI								OPTIONAL,

dl-DataSize 						INTEGER (0..96000, ...)				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { PagingPolicyDifferentiationItem-ExtIEs} }	OPTIONAL,

...

}

PagingPolicyDifferentiationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PagingPolicyIndicator ::= INTEGER (0..7, ...)

PagingPriority ::= ENUMERATED {

priolevel1,

priolevel2,

priolevel3,

priolevel4,

priolevel5,

priolevel6,

priolevel7,

priolevel8,

...

}

PagingProbabilityInformation ::= ENUMERATED {

p00, p05, p10, p15, p20, p25, p30, p35, p40, p45, p50, p55, p60, p65, p70, p75, p80, p85, p90, p95, p100,

...

}

ParentTImeSource ::= ENUMERATED {synce, ptp, gnss, atomicclock, terrestrialradio, serialtimecode, ntp, handset, other, ...}

Partially-Allowed-NSSAI ::= SEQUENCE (SIZE(1.. maxnoofPartiallyAllowedS-NSSAIs)) OF PartiallyAllowedNSSAI-Item

PartiallyAllowedNSSAI-Item ::= SEQUENCE {

s-NSSAI				S-NSSAI,

iE-Extensions		ProtocolExtensionContainer { {PartiallyAllowedNSSAI-Item-ExtIEs} }	OPTIONAL,

...

}

PartiallyAllowedNSSAI-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PathSwitchRequestAcknowledgeTransfer ::= SEQUENCE {

uL-NGU-UP-TNLInformation		UPTransportLayerInformation											OPTIONAL,

securityIndication				SecurityIndication													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PathSwitchRequestAcknowledgeTransfer-ExtIEs} }	OPTIONAL,

...

}

PathSwitchRequestAcknowledgeTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalNGU-UP-TNLInformation				CRITICALITY ignore	EXTENSION UPTransportLayerInformationPairList		PRESENCE optional	}|

{ ID id-RedundantUL-NGU-UP-TNLInformation			CRITICALITY ignore	EXTENSION UPTransportLayerInformation				PRESENCE optional	}|

{ ID id-AdditionalRedundantNGU-UP-TNLInformation	CRITICALITY ignore	EXTENSION UPTransportLayerInformationPairList		PRESENCE optional	}|

{ ID id-QosFlowParametersList						CRITICALITY ignore	EXTENSION QosFlowParametersList						PRESENCE optional },

...

}

PathSwitchRequestSetupFailedTransfer ::= SEQUENCE {

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { {PathSwitchRequestSetupFailedTransfer-ExtIEs} }	OPTIONAL,

...

}

PathSwitchRequestSetupFailedTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PathSwitchRequestTransfer ::= SEQUENCE {

dL-NGU-UP-TNLInformation			UPTransportLayerInformation,

dL-NGU-TNLInformationReused			DL-NGU-TNLInformationReused							OPTIONAL,

userPlaneSecurityInformation		UserPlaneSecurityInformation						OPTIONAL,

qosFlowAcceptedList					QosFlowAcceptedList,

iE-Extensions		ProtocolExtensionContainer { {PathSwitchRequestTransfer-ExtIEs} }	OPTIONAL,

...

}

PathSwitchRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalDLQosFlowPerTNLInformation				CRITICALITY ignore	EXTENSION QosFlowPerTNLInformationList		PRESENCE optional	}|

{ ID id-RedundantDL-NGU-UP-TNLInformation					CRITICALITY ignore	EXTENSION UPTransportLayerInformation			PRESENCE optional	}|

{ ID id-RedundantDL-NGU-TNLInformationReused				CRITICALITY ignore	EXTENSION DL-NGU-TNLInformationReused			PRESENCE optional	}|

{ ID id-AdditionalRedundantDLQosFlowPerTNLInformation		CRITICALITY ignore	EXTENSION QosFlowPerTNLInformationList		PRESENCE optional	}|

{ ID id-UsedRSNInformation									CRITICALITY ignore	EXTENSION RedundantPDUSessionInformation		PRESENCE optional	}|

{ ID id-GlobalRANNodeID 									CRITICALITY ignore	EXTENSION GlobalRANNodeID							PRESENCE optional	}|

{ ID id-MBS-SupportIndicator								CRITICALITY ignore	EXTENSION MBS-SupportIndicator					PRESENCE optional	}|

{ ID id-PDUSetbasedHandlingIndicator 						CRITICALITY ignore	EXTENSION PDUSetbasedHandlingIndicator 		PRESENCE optional }|

{ ID id-ECNMarkingorCongestionInformationReportingStatus	CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingStatus		PRESENCE optional },

...

}

PathSwitchRequestUnsuccessfulTransfer ::= SEQUENCE {

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { {PathSwitchRequestUnsuccessfulTransfer-ExtIEs} }	OPTIONAL,

...

}

PathSwitchRequestUnsuccessfulTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PC5QoSParameters ::= SEQUENCE {

pc5QoSFlowList				PC5QoSFlowList,

pc5LinkAggregateBitRates	BitRate												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { PC5QoSParameters-ExtIEs} }	OPTIONAL,

...

}

PC5QoSParameters-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PC5QoSFlowList ::= SEQUENCE (SIZE(1..maxnoofPC5QoSFlows)) OF PC5QoSFlowItem

PC5QoSFlowItem::= SEQUENCE {

pQI					FiveQI,

pc5FlowBitRates		PC5FlowBitRates											OPTIONAL,

range				Range													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { PC5QoSFlowItem-ExtIEs} }	OPTIONAL,

...

}

PC5QoSFlowItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PC5FlowBitRates ::= SEQUENCE {

guaranteedFlowBitRate		BitRate,

maximumFlowBitRate			BitRate,

iE-Extensions		ProtocolExtensionContainer { { PC5FlowBitRates-ExtIEs} }	OPTIONAL,

...

}

PC5FlowBitRates-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PCIListForMDT ::= SEQUENCE (SIZE(1.. maxnoofNeighPCIforMDT)) OF NR-PCI

PrivacyIndicator ::= ENUMERATED {

immediate-MDT,

logged-MDT,

...

}

PDUSessionAggregateMaximumBitRate ::= SEQUENCE {

pDUSessionAggregateMaximumBitRateDL		BitRate,

pDUSessionAggregateMaximumBitRateUL		BitRate,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionAggregateMaximumBitRate-ExtIEs} } OPTIONAL,

...

}

PDUSessionAggregateMaximumBitRate-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionID ::= INTEGER (0..255)

PDUSessionPairID ::= INTEGER (0..255, ...)

PDUSessionResourceAdmittedList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceAdmittedItem

PDUSessionResourceAdmittedItem ::= SEQUENCE {

pDUSessionID							PDUSessionID,

handoverRequestAcknowledgeTransfer		OCTET STRING (CONTAINING HandoverRequestAcknowledgeTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceAdmittedItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceAdmittedItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToModifyListModCfm ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToModifyItemModCfm

PDUSessionResourceFailedToModifyItemModCfm ::= SEQUENCE {

pDUSessionID												PDUSessionID,

pDUSessionResourceModifyIndicationUnsuccessfulTransfer		OCTET STRING (CONTAINING PDUSessionResourceModifyIndicationUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToModifyItemModCfm-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToModifyItemModCfm-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToModifyListModRes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToModifyItemModRes

PDUSessionResourceFailedToModifyItemModRes ::= SEQUENCE {

pDUSessionID										PDUSessionID,

pDUSessionResourceModifyUnsuccessfulTransfer		OCTET STRING (CONTAINING PDUSessionResourceModifyUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToModifyItemModRes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToModifyItemModRes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToResumeListRESReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToResumeItemRESReq

PDUSessionResourceFailedToResumeItemRESReq ::= SEQUENCE {

pDUSessionID						PDUSessionID,

cause								Cause,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToResumeItemRESReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToResumeItemRESReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToResumeListRESRes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToResumeItemRESRes

PDUSessionResourceFailedToResumeItemRESRes ::= SEQUENCE {

pDUSessionID						PDUSessionID,

cause								Cause,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToResumeItemRESRes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToResumeItemRESRes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToSetupListCxtFail ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToSetupItemCxtFail

PDUSessionResourceFailedToSetupItemCxtFail ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceSetupUnsuccessfulTransfer		OCTET STRING (CONTAINING PDUSessionResourceSetupUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToSetupItemCxtFail-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToSetupItemCxtFail-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToSetupListCxtRes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToSetupItemCxtRes

PDUSessionResourceFailedToSetupItemCxtRes ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceSetupUnsuccessfulTransfer		OCTET STRING (CONTAINING PDUSessionResourceSetupUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToSetupItemCxtRes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToSetupItemCxtRes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToSetupListHOAck ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToSetupItemHOAck

PDUSessionResourceFailedToSetupItemHOAck ::= SEQUENCE {

pDUSessionID										PDUSessionID,

handoverResourceAllocationUnsuccessfulTransfer		OCTET STRING (CONTAINING HandoverResourceAllocationUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToSetupItemHOAck-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToSetupItemHOAck-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToSetupListPSReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToSetupItemPSReq

PDUSessionResourceFailedToSetupItemPSReq ::= SEQUENCE {

pDUSessionID								PDUSessionID,

pathSwitchRequestSetupFailedTransfer		OCTET STRING (CONTAINING PathSwitchRequestSetupFailedTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToSetupItemPSReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToSetupItemPSReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceFailedToSetupListSURes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceFailedToSetupItemSURes

PDUSessionResourceFailedToSetupItemSURes ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceSetupUnsuccessfulTransfer		OCTET STRING (CONTAINING PDUSessionResourceSetupUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceFailedToSetupItemSURes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceFailedToSetupItemSURes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceHandoverList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceHandoverItem

PDUSessionResourceHandoverItem ::= SEQUENCE {

pDUSessionID						PDUSessionID,

handoverCommandTransfer				OCTET STRING (CONTAINING HandoverCommandTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceHandoverItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceHandoverItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceInformationList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceInformationItem

PDUSessionResourceInformationItem ::= SEQUENCE {

pDUSessionID					PDUSessionID,

qosFlowInformationList			QosFlowInformationList,

dRBsToQosFlowsMappingList		DRBsToQosFlowsMappingList										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceInformationItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceListCxtRelCpl ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceItemCxtRelCpl

PDUSessionResourceItemCxtRelCpl ::= SEQUENCE {

pDUSessionID		PDUSessionID,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceItemCxtRelCpl-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceItemCxtRelCpl-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSessionResourceReleaseResponseTransfer	CRITICALITY ignore	EXTENSION OCTET STRING (CONTAINING PDUSessionResourceReleaseResponseTransfer)	PRESENCE optional	},

...

}

PDUSessionResourceListCxtRelReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceItemCxtRelReq

PDUSessionResourceItemCxtRelReq ::= SEQUENCE {

pDUSessionID		PDUSessionID,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceItemCxtRelReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceItemCxtRelReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceListHORqd ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceItemHORqd

PDUSessionResourceItemHORqd ::= SEQUENCE {

pDUSessionID							PDUSessionID,

handoverRequiredTransfer				OCTET STRING (CONTAINING HandoverRequiredTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceItemHORqd-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceItemHORqd-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionListMTCommHReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionListMTCommHReq-Item

PDUSessionListMTCommHReq-Item ::= SEQUENCE {

pDUSessionID							PDUSessionID,

iE-Extensions		ProtocolExtensionContainer { { PDUSessionListMTCommHReq-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionListMTCommHReq-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModifyConfirmTransfer ::= SEQUENCE {

qosFlowModifyConfirmList			QosFlowModifyConfirmList,

uLNGU-UP-TNLInformation				UPTransportLayerInformation,

additionalNG-UUPTNLInformation		UPTransportLayerInformationPairList									OPTIONAL,

qosFlowFailedToModifyList			QosFlowListWithCause												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyConfirmTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyConfirmTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-RedundantUL-NGU-UP-TNLInformation			CRITICALITY ignore	EXTENSION UPTransportLayerInformation				PRESENCE optional	}|

{ ID id-AdditionalRedundantNGU-UP-TNLInformation	CRITICALITY ignore	EXTENSION UPTransportLayerInformationPairList		PRESENCE optional	},

...

}

PDUSessionResourceModifyIndicationUnsuccessfulTransfer ::= SEQUENCE {

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyIndicationUnsuccessfulTransfer-ExtIEs} }		OPTIONAL,

...

}

PDUSessionResourceModifyIndicationUnsuccessfulTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModifyRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceModifyRequestTransferIEs} },

...

}

PDUSessionResourceModifyRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-PDUSessionAggregateMaximumBitRate			CRITICALITY reject	TYPE PDUSessionAggregateMaximumBitRate		PRESENCE	optional		}|

{ ID id-UL-NGU-UP-TNLModifyList						CRITICALITY reject	TYPE UL-NGU-UP-TNLModifyList					PRESENCE	optional		}|

{ ID id-NetworkInstance								CRITICALITY reject	TYPE NetworkInstance								PRESENCE optional		}|

{ ID id-QosFlowAddOrModifyRequestList				CRITICALITY reject	TYPE QosFlowAddOrModifyRequestList			PRESENCE optional		}|

{ ID id-QosFlowToReleaseList						CRITICALITY reject	TYPE QosFlowListWithCause						PRESENCE optional		}|

{ ID id-AdditionalUL-NGU-UP-TNLInformation			CRITICALITY reject	TYPE UPTransportLayerInformationList			PRESENCE optional		}|

{ ID id-CommonNetworkInstance						CRITICALITY ignore	TYPE CommonNetworkInstance						PRESENCE optional		}|

{ ID id-AdditionalRedundantUL-NGU-UP-TNLInformation	CRITICALITY ignore	TYPE UPTransportLayerInformationList			PRESENCE optional		}|

{ ID id-RedundantCommonNetworkInstance				CRITICALITY ignore	TYPE CommonNetworkInstance						PRESENCE optional		}|

{ ID id-RedundantUL-NGU-UP-TNLInformation			CRITICALITY ignore	TYPE UPTransportLayerInformation				PRESENCE optional	 	}|

{ ID id-SecurityIndication							CRITICALITY ignore	TYPE SecurityIndication							PRESENCE optional		}|

{ ID id-MBSSessionSetuporModifyRequestList			CRITICALITY ignore	TYPE MBSSessionSetuporModifyRequestList		PRESENCE optional		}|

{ ID id-MBSSessionToReleaseList						CRITICALITY ignore	TYPE MBSSessionToReleaseList					PRESENCE optional		}|

{ ID id-UserPlaneFailureIndication					CRITICALITY ignore	TYPE UserPlaneFailureIndication				PRESENCE optional		},

...

}

PDUSessionResourceModifyResponseTransfer ::= SEQUENCE {

dL-NGU-UP-TNLInformation				UPTransportLayerInformation											OPTIONAL,

uL-NGU-UP-TNLInformation				UPTransportLayerInformation											OPTIONAL,

qosFlowAddOrModifyResponseList			QosFlowAddOrModifyResponseList										OPTIONAL,

additionalDLQosFlowPerTNLInformation	QosFlowPerTNLInformationList										OPTIONAL,

qosFlowFailedToAddOrModifyList			QosFlowListWithCause												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyResponseTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalNGU-UP-TNLInformation					CRITICALITY ignore	EXTENSION UPTransportLayerInformationPairList	PRESENCE optional	}|

{ ID id-RedundantDL-NGU-UP-TNLInformation				CRITICALITY ignore	EXTENSION UPTransportLayerInformation				PRESENCE optional	}|

{ ID id-RedundantUL-NGU-UP-TNLInformation				CRITICALITY ignore	EXTENSION UPTransportLayerInformation				PRESENCE optional	}|

{ ID id-AdditionalRedundantDLQosFlowPerTNLInformation	CRITICALITY ignore	EXTENSION QosFlowPerTNLInformationList				PRESENCE optional	}|

{ ID id-AdditionalRedundantNGU-UP-TNLInformation		CRITICALITY ignore	EXTENSION UPTransportLayerInformationPairList	PRESENCE optional	}|

{ ID id-SecondaryRATUsageInformation					CRITICALITY ignore	EXTENSION SecondaryRATUsageInformation				PRESENCE optional	}|

{ ID id-MBS-SupportIndicator							CRITICALITY ignore	EXTENSION MBS-SupportIndicator						PRESENCE optional	}|

{ ID id-MBSSessionSetuporModifyResponseList				CRITICALITY ignore	EXTENSION MBSSessionSetupResponseList				PRESENCE optional	}|

{ ID id-MBSSessionFailedtoSetuporModifyList				CRITICALITY ignore	EXTENSION MBSSessionFailedtoSetupList				PRESENCE optional	}|

{ ID id-ECNMarkingorCongestionInformationReportingStatus	CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingStatus		PRESENCE optional }|

{ ID id-PDUSetbasedHandlingIndicator 					CRITICALITY ignore	EXTENSION PDUSetbasedHandlingIndicator 			PRESENCE optional }|

{ ID id-UserPlaneFailureIndicationReport				CRITICALITY ignore	EXTENSION UserPlaneFailureIndicationReport		PRESENCE optional	},

...

}

PDUSessionResourceModifyIndicationTransfer ::= SEQUENCE {

dLQosFlowPerTNLInformation					QosFlowPerTNLInformation,

additionalDLQosFlowPerTNLInformation		QosFlowPerTNLInformationList 									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyIndicationTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyIndicationTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecondaryRATUsageInformation					CRITICALITY ignore	EXTENSION SecondaryRATUsageInformation		PRESENCE optional	}|

{ ID id-SecurityResult									CRITICALITY ignore	EXTENSION SecurityResult						PRESENCE optional	}|

{ ID id-RedundantDLQosFlowPerTNLInformation				CRITICALITY ignore	EXTENSION QosFlowPerTNLInformation			PRESENCE optional	}|

{ ID id-AdditionalRedundantDLQosFlowPerTNLInformation	CRITICALITY ignore	EXTENSION QosFlowPerTNLInformationList		PRESENCE optional	}|

{ ID id-GlobalRANNodeID 								CRITICALITY ignore	EXTENSION GlobalRANNodeID						PRESENCE optional	}|

{ ID id-ECNMarkingorCongestionInformationReportingStatus	CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingStatus		PRESENCE optional },

...

}

PDUSessionResourceModifyListModCfm ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceModifyItemModCfm

PDUSessionResourceModifyItemModCfm ::= SEQUENCE {

pDUSessionID								PDUSessionID,

pDUSessionResourceModifyConfirmTransfer		OCTET STRING (CONTAINING PDUSessionResourceModifyConfirmTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyItemModCfm-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyItemModCfm-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModifyListModInd ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceModifyItemModInd

PDUSessionResourceModifyItemModInd ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceModifyIndicationTransfer		OCTET STRING (CONTAINING PDUSessionResourceModifyIndicationTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyItemModInd-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyItemModInd-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModifyListModReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceModifyItemModReq

PDUSessionResourceModifyItemModReq ::= SEQUENCE {

pDUSessionID								PDUSessionID,

nAS-PDU										NAS-PDU													OPTIONAL,

pDUSessionResourceModifyRequestTransfer		OCTET STRING (CONTAINING PDUSessionResourceModifyRequestTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyItemModReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyItemModReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-S-NSSAI											CRITICALITY reject	EXTENSION S-NSSAI						PRESENCE optional	}|

{ ID id-PduSessionExpectedUEActivityBehaviour			CRITICALITY ignore	EXTENSION ExpectedUEActivityBehaviour	PRESENCE optional	},

...

}

PDUSessionResourceModifyListModRes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceModifyItemModRes

PDUSessionResourceModifyItemModRes ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceModifyResponseTransfer		OCTET STRING (CONTAINING PDUSessionResourceModifyResponseTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyItemModRes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyItemModRes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceModifyUnsuccessfulTransfer ::= SEQUENCE {

cause						Cause,

criticalityDiagnostics		CriticalityDiagnostics																OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyUnsuccessfulTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyUnsuccessfulTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceNotifyList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceNotifyItem

PDUSessionResourceNotifyItem ::= SEQUENCE {

pDUSessionID						PDUSessionID,

pDUSessionResourceNotifyTransfer	OCTET STRING (CONTAINING PDUSessionResourceNotifyTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceNotifyItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceNotifyItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceNotifyReleasedTransfer ::= SEQUENCE {

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceNotifyReleasedTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceNotifyReleasedTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecondaryRATUsageInformation		CRITICALITY ignore	EXTENSION SecondaryRATUsageInformation		PRESENCE optional	}|

{ ID id-UserPlaneErrorIndicator				CRITICALITY ignore	EXTENSION UserPlaneErrorIndicator			PRESENCE optional	},

...

}

UserPlaneErrorIndicator ::= ENUMERATED {

gTP-U-error-indication-received,

...

}

PDUSessionResourceNotifyTransfer ::= SEQUENCE {

qosFlowNotifyList		QosFlowNotifyList														OPTIONAL,

qosFlowReleasedList		QosFlowListWithCause													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceNotifyTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceNotifyTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecondaryRATUsageInformation		CRITICALITY ignore	EXTENSION SecondaryRATUsageInformation		PRESENCE optional	}|

{ ID id-QosFlowFeedbackList					CRITICALITY ignore	EXTENSION QosFlowFeedbackList					PRESENCE optional	},

...

}

PDUSessionResourceReleaseCommandTransfer ::= SEQUENCE {

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceReleaseCommandTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceReleaseCommandTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-QosFlowAdditionalInfoList		CRITICALITY ignore	EXTENSION QosFlowAdditionalInfoListRelCom		PRESENCE optional	},

...

}

PDUSessionResourceReleasedListNot ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceReleasedItemNot

PDUSessionResourceReleasedItemNot ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceNotifyReleasedTransfer		OCTET STRING (CONTAINING PDUSessionResourceNotifyReleasedTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceReleasedItemNot-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceReleasedItemNot-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceReleasedListPSAck ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceReleasedItemPSAck

PDUSessionResourceReleasedItemPSAck ::= SEQUENCE {

pDUSessionID								PDUSessionID,

pathSwitchRequestUnsuccessfulTransfer		OCTET STRING (CONTAINING PathSwitchRequestUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceReleasedItemPSAck-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceReleasedItemPSAck-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceReleasedListPSFail ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceReleasedItemPSFail

PDUSessionResourceReleasedItemPSFail ::= SEQUENCE {

pDUSessionID								PDUSessionID,

pathSwitchRequestUnsuccessfulTransfer		OCTET STRING (CONTAINING PathSwitchRequestUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceReleasedItemPSFail-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceReleasedItemPSFail-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceReleasedListRelRes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceReleasedItemRelRes

PDUSessionResourceReleasedItemRelRes ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceReleaseResponseTransfer		OCTET STRING (CONTAINING PDUSessionResourceReleaseResponseTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceReleasedItemRelRes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceReleasedItemRelRes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceReleaseResponseTransfer ::= SEQUENCE {

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceReleaseResponseTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceReleaseResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecondaryRATUsageInformation		CRITICALITY ignore	EXTENSION SecondaryRATUsageInformation		PRESENCE optional	}|

{ ID id-QosFlowAdditionalInfoList		CRITICALITY ignore	EXTENSION QosFlowAdditionalInfoListRelRes		PRESENCE optional	},

...

}

PDUSessionResourceResumeListRESReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceResumeItemRESReq

PDUSessionResourceResumeItemRESReq ::= SEQUENCE {

pDUSessionID						PDUSessionID,

uEContextResumeRequestTransfer		OCTET STRING (CONTAINING UEContextResumeRequestTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceResumeItemRESReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceResumeItemRESReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceResumeListRESRes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceResumeItemRESRes

PDUSessionResourceResumeItemRESRes ::= SEQUENCE {

pDUSessionID						PDUSessionID,

uEContextResumeResponseTransfer		OCTET STRING (CONTAINING UEContextResumeResponseTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceResumeItemRESRes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceResumeItemRESRes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSecondaryRATUsageList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSecondaryRATUsageItem

PDUSessionResourceSecondaryRATUsageItem ::= SEQUENCE {

pDUSessionID							PDUSessionID,

secondaryRATDataUsageReportTransfer		OCTET STRING (CONTAINING SecondaryRATDataUsageReportTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSecondaryRATUsageItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSecondaryRATUsageItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSetupListCxtReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSetupItemCxtReq

PDUSessionResourceSetupItemCxtReq ::= SEQUENCE {

pDUSessionID								PDUSessionID,

nAS-PDU										NAS-PDU												OPTIONAL,

s-NSSAI										S-NSSAI,

pDUSessionResourceSetupRequestTransfer		OCTET STRING (CONTAINING PDUSessionResourceSetupRequestTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupItemCxtReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupItemCxtReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PduSessionExpectedUEActivityBehaviour			CRITICALITY ignore	EXTENSION ExpectedUEActivityBehaviour	PRESENCE optional	},

...

}

PDUSessionResourceSetupListCxtRes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSetupItemCxtRes

PDUSessionResourceSetupItemCxtRes ::= SEQUENCE {

pDUSessionID								PDUSessionID,

pDUSessionResourceSetupResponseTransfer		OCTET STRING (CONTAINING PDUSessionResourceSetupResponseTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupItemCxtRes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupItemCxtRes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSetupListHOReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSetupItemHOReq

PDUSessionResourceSetupItemHOReq ::= SEQUENCE {

pDUSessionID				PDUSessionID,

s-NSSAI						S-NSSAI,

handoverRequestTransfer		OCTET STRING (CONTAINING PDUSessionResourceSetupRequestTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupItemHOReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupItemHOReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PduSessionExpectedUEActivityBehaviour			CRITICALITY ignore	EXTENSION ExpectedUEActivityBehaviour	PRESENCE optional	},

...

}

PDUSessionResourceSetupListSUReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSetupItemSUReq

PDUSessionResourceSetupItemSUReq ::= SEQUENCE {

pDUSessionID								PDUSessionID,

pDUSessionNAS-PDU							NAS-PDU												OPTIONAL,

s-NSSAI										S-NSSAI,

pDUSessionResourceSetupRequestTransfer		OCTET STRING (CONTAINING PDUSessionResourceSetupRequestTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupItemSUReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupItemSUReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PduSessionExpectedUEActivityBehaviour			CRITICALITY ignore	EXTENSION ExpectedUEActivityBehaviour	PRESENCE optional	},

...

}

PDUSessionResourceSetupListSURes ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSetupItemSURes

PDUSessionResourceSetupItemSURes ::= SEQUENCE {

pDUSessionID											PDUSessionID,

pDUSessionResourceSetupResponseTransfer					OCTET STRING (CONTAINING PDUSessionResourceSetupResponseTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupItemSURes-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupItemSURes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSetupRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {PDUSessionResourceSetupRequestTransferIEs} },

...

}

PDUSessionResourceSetupRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-PDUSessionAggregateMaximumBitRate			CRITICALITY reject	TYPE PDUSessionAggregateMaximumBitRate		PRESENCE optional		}|

{ ID id-UL-NGU-UP-TNLInformation					CRITICALITY reject	TYPE UPTransportLayerInformation				PRESENCE mandatory	}|

{ ID id-AdditionalUL-NGU-UP-TNLInformation			CRITICALITY reject	TYPE UPTransportLayerInformationList			PRESENCE optional		}|

{ ID id-DataForwardingNotPossible					CRITICALITY reject	TYPE DataForwardingNotPossible					PRESENCE optional		}|

{ ID id-PDUSessionType								CRITICALITY reject	TYPE PDUSessionType									PRESENCE mandatory	}|

{ ID id-SecurityIndication							CRITICALITY reject	TYPE SecurityIndication							PRESENCE optional		}|

{ ID id-NetworkInstance								CRITICALITY reject	TYPE NetworkInstance								PRESENCE optional		}|

{ ID id-QosFlowSetupRequestList						CRITICALITY reject	TYPE QosFlowSetupRequestList					PRESENCE mandatory	}|

{ ID id-CommonNetworkInstance						CRITICALITY ignore	TYPE CommonNetworkInstance						PRESENCE optional		}|

{ ID id-DirectForwardingPathAvailability			CRITICALITY ignore	TYPE DirectForwardingPathAvailability		PRESENCE optional	 	}|

{ ID id-RedundantUL-NGU-UP-TNLInformation			CRITICALITY ignore	TYPE UPTransportLayerInformation				PRESENCE optional	 	}|

{ ID id-AdditionalRedundantUL-NGU-UP-TNLInformation	CRITICALITY ignore	TYPE UPTransportLayerInformationList			PRESENCE optional		}|

{ ID id-RedundantCommonNetworkInstance				CRITICALITY ignore	TYPE CommonNetworkInstance						PRESENCE optional		}|

{ ID id-RedundantPDUSessionInformation				CRITICALITY ignore	TYPE RedundantPDUSessionInformation 			PRESENCE optional		}|

{ ID id-MBSSessionSetupRequestList					CRITICALITY ignore	TYPE MBSSessionSetupRequestList				PRESENCE optional		}|

{ ID id-DownlinkTLContainer							CRITICALITY ignore	TYPE TLContainer									PRESENCE optional		},

...

}

PDUSessionResourceSetupResponseTransfer ::= SEQUENCE {

dLQosFlowPerTNLInformation				QosFlowPerTNLInformation,

additionalDLQosFlowPerTNLInformation	QosFlowPerTNLInformationList										OPTIONAL,

securityResult							SecurityResult														OPTIONAL,

qosFlowFailedToSetupList				QosFlowListWithCause												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupResponseTransfer-ExtIEs} }		OPTIONAL,

...

}

PDUSessionResourceSetupResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-RedundantDLQosFlowPerTNLInformation				CRITICALITY ignore	EXTENSION QosFlowPerTNLInformation			PRESENCE optional	 	}|

{ ID id-AdditionalRedundantDLQosFlowPerTNLInformation	CRITICALITY ignore	EXTENSION QosFlowPerTNLInformationList		PRESENCE optional	 	}|

{ ID id-UsedRSNInformation								CRITICALITY ignore	EXTENSION RedundantPDUSessionInformation	PRESENCE optional		}|

{ ID id-GlobalRANNodeID 								CRITICALITY ignore	EXTENSION GlobalRANNodeID						PRESENCE optional		}|

{ ID id-MBS-SupportIndicator							CRITICALITY ignore	EXTENSION MBS-SupportIndicator				PRESENCE optional		}|

{ ID id-MBSSessionSetupResponseList						CRITICALITY ignore	EXTENSION MBSSessionSetupResponseList		PRESENCE optional		}|

{ ID id-MBSSessionFailedtoSetupList						CRITICALITY ignore	EXTENSION MBSSessionFailedtoSetupList		PRESENCE optional		}|

{ ID id-QoSFlowTSCList									CRITICALITY ignore	EXTENSION QoSFlowTSCList						PRESENCE optional		}|

{ ID id-UplinkTLContainer								CRITICALITY ignore	EXTENSION TLContainer							PRESENCE optional		}|

{ ID id-ECNMarkingorCongestionInformationReportingStatus	CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingStatus		PRESENCE optional }|

{ ID id-PDUSetbasedHandlingIndicator 					CRITICALITY ignore	EXTENSION PDUSetbasedHandlingIndicator 	PRESENCE optional 	},

...

}

PDUSessionResourceSetupUnsuccessfulTransfer ::= SEQUENCE {

cause						Cause,

criticalityDiagnostics		CriticalityDiagnostics																OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupUnsuccessfulTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupUnsuccessfulTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSuspendListSUSReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSuspendItemSUSReq

PDUSessionResourceSuspendItemSUSReq ::= SEQUENCE {

pDUSessionID						PDUSessionID,

uEContextSuspendRequestTransfer		OCTET STRING (CONTAINING UEContextSuspendRequestTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSuspendItemSUSReq-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSuspendItemSUSReq-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceSwitchedList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceSwitchedItem

PDUSessionResourceSwitchedItem ::= SEQUENCE {

pDUSessionID								PDUSessionID,

pathSwitchRequestAcknowledgeTransfer		OCTET STRING (CONTAINING PathSwitchRequestAcknowledgeTransfer),

iE-Extensions		ProtocolExtensionContainer { { PDUSessionResourceSwitchedItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSwitchedItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PduSessionExpectedUEActivityBehaviour			CRITICALITY ignore	EXTENSION ExpectedUEActivityBehaviour	PRESENCE optional	},

...

}

PDUSessionResourceToBeSwitchedDLList ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceToBeSwitchedDLItem

PDUSessionResourceToBeSwitchedDLItem ::= SEQUENCE {

pDUSessionID					PDUSessionID,

pathSwitchRequestTransfer		OCTET STRING (CONTAINING PathSwitchRequestTransfer),

iE-Extensions		ProtocolExtensionContainer { { PDUSessionResourceToBeSwitchedDLItem-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceToBeSwitchedDLItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceToReleaseListHOCmd ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceToReleaseItemHOCmd

PDUSessionResourceToReleaseItemHOCmd ::= SEQUENCE {

pDUSessionID								PDUSessionID,

handoverPreparationUnsuccessfulTransfer		OCTET STRING (CONTAINING HandoverPreparationUnsuccessfulTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceToReleaseItemHOCmd-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceToReleaseItemHOCmd-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionResourceToReleaseListRelCmd ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionResourceToReleaseItemRelCmd

PDUSessionResourceToReleaseItemRelCmd ::= SEQUENCE {

pDUSessionID									PDUSessionID,

pDUSessionResourceReleaseCommandTransfer		OCTET STRING (CONTAINING PDUSessionResourceReleaseCommandTransfer),

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceToReleaseItemRelCmd-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceToReleaseItemRelCmd-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSessionType ::= ENUMERATED {

ipv4,

ipv6,

ipv4v6,

ethernet,

unstructured,

...

}

PDUSessionUsageReport ::= SEQUENCE {

rATType								ENUMERATED {nr, eutra, ..., nr-unlicensed, e-utra-unlicensed},

pDUSessionTimedReportList			VolumeTimedReportList,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionUsageReport-ExtIEs} } OPTIONAL,

...

}

PDUSessionUsageReport-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUsetQoSParameters ::= SEQUENCE {

ulPDUSetQoSInformation	PDUsetQoSInformation	OPTIONAL,

dlPDUSetQoSInformation	PDUsetQoSInformation	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { PDUsetQoSParameters-ExtIEs } }	OPTIONAL,

...

}

PDUsetQoSParameters-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUsetQoSInformation	::= SEQUENCE {

pduSetDelayBudget						ExtendedPacketDelayBudget		OPTIONAL,

pduSetErrorRate							PacketErrorRate					OPTIONAL,

pduSetIntegratedHandlingInformation		ENUMERATED {true, false, ...}	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { PDUsetQoSInformation-ExtIEs } }	OPTIONAL,

...

}

PDUsetQoSInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PDUSetbasedHandlingIndicator ::= ENUMERATED {supported, ...}

PEIPSassistanceInformation ::= SEQUENCE {

cNsubgroupID			CNsubgroupID,

iE-Extensions		ProtocolExtensionContainer { {PEIPSassistanceInformation-ExtIEs} } OPTIONAL,

...

}

PEIPSassistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

Periodicity ::= INTEGER (0..640000, ...)

PeriodicityBound ::= SEQUENCE {

periodicityLowerBound					Periodicity,

periodicityUpperBound					Periodicity,

iE-Extensions				ProtocolExtensionContainer { {PeriodicityBound-ExtIEs} } OPTIONAL,

...

}

PeriodicityBound-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PeriodicityList ::= SEQUENCE {

allowedPeriodicityList			AllowedPeriodicityList,

iE-Extensions					ProtocolExtensionContainer { {PeriodicityList-ExtIEs} } OPTIONAL,

...

}

PeriodicityList-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AllowedPeriodicityList ::= SEQUENCE (SIZE(1..maxnoofPeriodicities)) OF Periodicity

PeriodicityRange ::= CHOICE {

periodicityBound				PeriodicityBound,

periodicityList					AllowedPeriodicityList,

choice-extensions				ProtocolIE-SingleContainer { {PeriodicityRange-ExtIEs} }

}

PeriodicityRange-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

PeriodicRegistrationUpdateTimer ::= BIT STRING (SIZE(8))

PLMNIdentity ::= OCTET STRING (SIZE(3))

PLMNAreaBasedQMC ::= SEQUENCE {

plmnListforQMC		PLMNListforQMC,

iE-Extensions		ProtocolExtensionContainer { {PLMNAreaBasedQMC-ExtIEs} } OPTIONAL,

...

}

PLMNAreaBasedQMC-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PLMNListforQMC ::= SEQUENCE (SIZE(1..maxnoofPLMNforQMC)) OF PLMNIdentity

PLMNSupportList ::= SEQUENCE (SIZE(1..maxnoofPLMNs)) OF PLMNSupportItem

PLMNSupportItem ::= SEQUENCE {

pLMNIdentity			PLMNIdentity,

sliceSupportList		SliceSupportList,

iE-Extensions		ProtocolExtensionContainer { {PLMNSupportItem-ExtIEs} } OPTIONAL,

...

}

PLMNSupportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-NPN-Support						CRITICALITY reject	EXTENSION NPN-Support					PRESENCE optional	}|

{ ID id-ExtendedSliceSupportList		CRITICALITY reject	EXTENSION ExtendedSliceSupportList 	PRESENCE optional	}|

{ ID id-OnboardingSupport				CRITICALITY ignore	EXTENSION OnboardingSupport 			PRESENCE optional	},

...

}

PNI-NPN-MobilityInformation ::= SEQUENCE {

allowed-PNI-NPI-List		Allowed-PNI-NPN-List,

iE-Extensions				ProtocolExtensionContainer { {PNI-NPN-MobilityInformation-ExtIEs} }	OPTIONAL,

...

}

PNI-NPN-MobilityInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PortNumber ::= OCTET STRING (SIZE(2))

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption,

...

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable,

...

}

PriorityLevelARP ::= INTEGER (1..15)

PriorityLevelQos ::= INTEGER (1..127, ...)

PWSFailedCellIDList ::= CHOICE {

eUTRA-CGI-PWSFailedList		EUTRA-CGIList,

nR-CGI-PWSFailedList		NR-CGIList,

choice-Extensions		ProtocolIE-SingleContainer { {PWSFailedCellIDList-ExtIEs} }

}

PWSFailedCellIDList-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

PNI-NPN-AreaScopeofMDT ::= SEQUENCE {

cAGListforMDT		CAGListforMDT,

iE-Extensions		ProtocolExtensionContainer { {PNI-NPN-AreaScopeofMDT-ExtIEs} } OPTIONAL,

...

}

PNI-NPN-AreaScopeofMDT-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

PNI-NPNBasedMDT::= SEQUENCE {

cAGListforMDT		CAGListforMDT,

iE-Extensions		ProtocolExtensionContainer { {PNI-NPNBasedMDT-ExtIEs} } OPTIONAL,

...

}

PNI-NPNBasedMDT-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

#### }

-- Q

QMCConfigInfo ::= SEQUENCE {

uEAppLayerMeasInfoList				UEAppLayerMeasInfoList,

iE-Extensions		ProtocolExtensionContainer { { QMCConfigInfo-ExtIEs} } OPTIONAL,

...

}

QMCConfigInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QMCDeactivation ::= SEQUENCE {

qoEReferenceList				QoEReferenceList,

iE-Extensions		ProtocolExtensionContainer { { QMCDeactivation-ExtIEs} } OPTIONAL,

...

}

QMCDeactivation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QoEReferenceList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF QoEReference

QoEReference ::= OCTET STRING (SIZE(6))

QoERVQoEReportingPaths ::= SEQUENCE {

qoEReportingPath					ENUMERATED{srb4, srb5, ...}	OPTIONAL,

rVQoEReportingPath					ENUMERATED{srb4, srb5, ...}	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {QoERVQoEReportingPaths-ExtIEs} }		OPTIONAL,

...

}

QoERVQoEReportingPaths-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosCharacteristics ::= CHOICE {

nonDynamic5QI		NonDynamic5QIDescriptor,

dynamic5QI			Dynamic5QIDescriptor,

choice-Extensions		ProtocolIE-SingleContainer { {QosCharacteristics-ExtIEs} }

}

QosCharacteristics-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

QosFlowAcceptedList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowAcceptedItem

QosFlowAcceptedItem ::= SEQUENCE {

qosFlowIdentifier		QosFlowIdentifier,

iE-Extensions		ProtocolExtensionContainer { {QosFlowAcceptedItem-ExtIEs} } OPTIONAL,

...

}

QosFlowAcceptedItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetIndex	PRESENCE optional	},

...

}

QosFlowAdditionalInfoListRelCom ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowAdditionalInfoItemRelCom

QosFlowAdditionalInfoItemRelCom ::= SEQUENCE {

qosFlowIdentifier				QosFlowIdentifier,

downlinkTLContainer				TLContainer									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QosFlowAdditionalInfoItemRelCom-ExtIEs} }	OPTIONAL,

...

}

QosFlowAdditionalInfoItemRelCom-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowAdditionalInfoListRelRes ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowAdditionalInfoItemRelRes

QosFlowAdditionalInfoItemRelRes ::= SEQUENCE {

qosFlowIdentifier				QosFlowIdentifier,

uplinkTLContainer				TLContainer									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QosFlowAdditionalInfoItemRelRes-ExtIEs} }	OPTIONAL,

...

}

QosFlowAdditionalInfoItemRelRes-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowAddOrModifyRequestList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowAddOrModifyRequestItem

QosFlowAddOrModifyRequestItem ::= SEQUENCE {

qosFlowIdentifier				QosFlowIdentifier,

qosFlowLevelQosParameters		QosFlowLevelQosParameters									OPTIONAL,

e-RAB-ID						E-RAB-ID													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QosFlowAddOrModifyRequestItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowAddOrModifyRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-TSCTrafficCharacteristics	CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics			PRESENCE optional }|

{ID id-RedundantQosFlowIndicator	CRITICALITY ignore	EXTENSION RedundantQosFlowIndicator			PRESENCE optional }|

{ID id-UL-NGU-UP-TNLInformation		CRITICALITY ignore	EXTENSION UPTransportLayerInformation		PRESENCE optional	}|

{ID id-DownlinkTLContainer			CRITICALITY ignore	EXTENSION TLContainer						PRESENCE optional }|

{ID id-ECNMarkingorCongestionInformationReportingRequest	CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingRequest		PRESENCE optional },

...

}

QosFlowAddOrModifyResponseList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowAddOrModifyResponseItem

QosFlowAddOrModifyResponseItem ::= SEQUENCE {

qosFlowIdentifier		QosFlowIdentifier,

iE-Extensions		ProtocolExtensionContainer { {QosFlowAddOrModifyResponseItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowAddOrModifyResponseItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex				CRITICALITY ignore	EXTENSION AlternativeQoSParaSetIndex				PRESENCE optional	}|

{ ID id-TSCTrafficCharacteristicsFeedback	CRITICALITY ignore	EXTENSION TSCTrafficCharacteristicsFeedback		PRESENCE optional	}|

{ ID id-UplinkTLContainer					CRITICALITY ignore	EXTENSION TLContainer									PRESENCE optional	}|

{ ID id-DL-NGU-UP-TNLInformation			CRITICALITY ignore	EXTENSION UPTransportLayerInformation				PRESENCE optional }|

{ ID id-ANPacketDelayBudgetUL				CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget					PRESENCE optional },

...

}

QosFlowFeedbackList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowFeedbackItem

QosFlowFeedbackItem ::= SEQUENCE {

qosFlowIdentifier			QosFlowIdentifier,

updateFeedback				UpdateFeedback								OPTIONAL,

cNpacketDelayBudgetDL		ExtendedPacketDelayBudget					OPTIONAL,

cNpacketDelayBudgetUL		ExtendedPacketDelayBudget					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QosFlowFeedbackItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowFeedbackItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowIdentifier ::= INTEGER (0..63, ...)

QosFlowInformationList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowInformationItem

QosFlowInformationItem ::= SEQUENCE {

qosFlowIdentifier	QosFlowIdentifier,

dLForwarding		DLForwarding													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QosFlowInformationItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-ULForwarding				CRITICALITY ignore	EXTENSION ULForwarding			PRESENCE optional}|

{ID id-SourceTNLAddrInfo		CRITICALITY ignore	EXTENSION TransportLayerAddress	PRESENCE optional}|

{ID id-SourceNodeTNLAddrInfo	CRITICALITY ignore	EXTENSION TransportLayerAddress	PRESENCE optional},

...

}

QosFlowLevelQosParameters ::= SEQUENCE {

qosCharacteristics					QosCharacteristics,

allocationAndRetentionPriority		AllocationAndRetentionPriority,

gBR-QosInformation					GBR-QosInformation									OPTIONAL,

reflectiveQosAttribute				ReflectiveQosAttribute								OPTIONAL,

additionalQosFlowInformation		AdditionalQosFlowInformation						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QosFlowLevelQosParameters-ExtIEs} }	OPTIONAL,

...

}

QosFlowLevelQosParameters-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-QosMonitoringRequest				CRITICALITY ignore	EXTENSION QosMonitoringRequest					PRESENCE optional}|

{ID id-QosMonitoringReportingFrequency	CRITICALITY ignore	EXTENSION QosMonitoringReportingFrequency	PRESENCE optional}|

{ID id-PDUsetQoSParameters				CRITICALITY	ignore	EXTENSION PDUsetQoSParameters					PRESENCE optional},

...

}

QosMonitoringRequest ::= ENUMERATED {ul, dl, both, ..., stop}

QosMonitoringReportingFrequency ::= INTEGER (1..1800, ...)

QoSFlowList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowIdentifier

QosFlowListWithCause ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowWithCauseItem

QosFlowWithCauseItem ::= SEQUENCE {

qosFlowIdentifier		QosFlowIdentifier,

cause					Cause,

iE-Extensions		ProtocolExtensionContainer { {QosFlowWithCauseItem-ExtIEs} } OPTIONAL,

...

}

QosFlowWithCauseItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowModifyConfirmList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowModifyConfirmItem

QosFlowModifyConfirmItem ::= SEQUENCE {

qosFlowIdentifier		QosFlowIdentifier,

iE-Extensions		ProtocolExtensionContainer { {QosFlowModifyConfirmItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowModifyConfirmItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowNotifyList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowNotifyItem

QosFlowNotifyItem ::= SEQUENCE {

qosFlowIdentifier			QosFlowIdentifier,

notificationCause			NotificationCause,

iE-Extensions		ProtocolExtensionContainer { {QosFlowNotifyItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowNotifyItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex				CRITICALITY ignore	EXTENSION AlternativeQoSParaSetNotifyIndex	PRESENCE optional	}|

{ ID id-TSCTrafficCharacteristicsFeedback	CRITICALITY ignore	EXTENSION TSCTrafficCharacteristicsFeedback	PRESENCE optional	},

...

}

QosFlowParametersList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowParametersItem

QosFlowParametersItem ::= SEQUENCE {

qosFlowIdentifier				QosFlowIdentifier,

alternativeQoSParaSetList		AlternativeQoSParaSetList							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QosFlowParametersItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowParametersItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-CNPacketDelayBudgetDL			CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional	}|

{ ID id-CNPacketDelayBudgetUL			CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional	}|

{ ID id-BurstArrivalTimeDownlink		CRITICALITY ignore	EXTENSION BurstArrivalTime					PRESENCE optional	},

...

}

QosFlowPerTNLInformation ::= SEQUENCE {

uPTransportLayerInformation		UPTransportLayerInformation,

associatedQosFlowList			AssociatedQosFlowList,

iE-Extensions		ProtocolExtensionContainer { { QosFlowPerTNLInformation-ExtIEs} }	OPTIONAL,

...

}

QosFlowPerTNLInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowPerTNLInformationList ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF QosFlowPerTNLInformationItem

QosFlowPerTNLInformationItem ::= SEQUENCE {

qosFlowPerTNLInformation				QosFlowPerTNLInformation,

iE-Extensions		ProtocolExtensionContainer { { QosFlowPerTNLInformationItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowPerTNLInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QosFlowSetupRequestList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowSetupRequestItem

QosFlowSetupRequestItem ::= SEQUENCE {

qosFlowIdentifier				QosFlowIdentifier,

qosFlowLevelQosParameters		QosFlowLevelQosParameters,

e-RAB-ID						E-RAB-ID											OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QosFlowSetupRequestItem-ExtIEs} } OPTIONAL,

...

}

QosFlowSetupRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-TSCTrafficCharacteristics	CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics		PRESENCE optional }|

{ID id-RedundantQosFlowIndicator	CRITICALITY ignore	EXTENSION RedundantQosFlowIndicator		PRESENCE optional }|

{ID id-ECNMarkingorCongestionInformationReportingRequest	CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingRequest		PRESENCE optional },

...

}

QosFlowListWithDataForwarding ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowItemWithDataForwarding

QosFlowItemWithDataForwarding ::= SEQUENCE {

qosFlowIdentifier			QosFlowIdentifier,

dataForwardingAccepted		DataForwardingAccepted											OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QosFlowItemWithDataForwarding-ExtIEs} }	OPTIONAL,

...

}

QosFlowItemWithDataForwarding-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetIndex	PRESENCE optional	},

...

}

QosFlowToBeForwardedList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QosFlowToBeForwardedItem

QosFlowToBeForwardedItem ::= SEQUENCE {

qosFlowIdentifier		QosFlowIdentifier,

iE-Extensions		ProtocolExtensionContainer { {QosFlowToBeForwardedItem-ExtIEs} }	OPTIONAL,

...

}

QosFlowToBeForwardedItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowTSCList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QoSFlowTSCItem

QoSFlowTSCItem ::= SEQUENCE {

qosFlowIdentifier						QosFlowIdentifier,

tSCTrafficCharacteristicsFeedback		TSCTrafficCharacteristicsFeedback	OPTIONAL,

aNPacketDelayBudgetUL					ExtendedPacketDelayBudget			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowTSCItem-ExtIEs} }	OPTIONAL,

...

}

QoSFlowTSCItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

QoSFlowsUsageReportList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF QoSFlowsUsageReport-Item

QoSFlowsUsageReport-Item ::= SEQUENCE {

qosFlowIdentifier					QosFlowIdentifier,

rATType								ENUMERATED {nr, eutra, ..., nr-unlicensed, e-utra-unlicensed},

qoSFlowsTimedReportList				VolumeTimedReportList,

iE-Extensions		ProtocolExtensionContainer { {QoSFlowsUsageReport-Item-ExtIEs} } OPTIONAL,

...

}

QoSFlowsUsageReport-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- R

RANfeedbacktype ::= CHOICE {

proactive					RANfeedbacktype-proactive,

reactive					RANfeedbacktype-reactive,

choice-extensions			ProtocolIE-SingleContainer { {RANfeedbacktype-ExtIEs} }

}

RANfeedbacktype-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

RANfeedbacktype-proactive ::= SEQUENCE {

burstArrivalTimeWindow	BurstArrivalTimeWindow,

periodicityRange		PeriodicityRange			OPTIONAL,

iE-Extension			ProtocolExtensionContainer { {RANfeedbacktype-proactive-ExtIEs} }	OPTIONAL,

...

}

RANfeedbacktype-proactive-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RANfeedbacktype-reactive ::= SEQUENCE {

capabilityForBATAdaptation	ENUMERATED {true, ...},

iE-Extension			ProtocolExtensionContainer { {RANfeedbacktype-reactive-ExtIEs} }	OPTIONAL,

...

}

RANfeedbacktype-reactive-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

Range ::= ENUMERATED {m50, m80, m180, m200, m350, m400, m500, m700, m1000, ...}

RANNodeName ::= PrintableString (SIZE(1..150, ...))

RANNodeNameVisibleString ::= VisibleString (SIZE(1..150, ...))

RANNodeNameUTF8String ::= UTF8String (SIZE(1..150, ...))

RANPagingPriority ::= INTEGER (1..256)

RANStatusTransfer-TransparentContainer ::= SEQUENCE {

dRBsSubjectToStatusTransferList		DRBsSubjectToStatusTransferList,

iE-Extensions		ProtocolExtensionContainer { {RANStatusTransfer-TransparentContainer-ExtIEs} }	OPTIONAL,

...

}

RANStatusTransfer-TransparentContainer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RANTimingSynchronisationStatusInfo	::= SEQUENCE {

synchronisationState			ENUMERATED {locked, holdover, freerun, ...}						OPTIONAL,

traceabletoUTC					ENUMERATED {true, false, ...}									OPTIONAL,

traceabletoGNSS					ENUMERATED {true, false, ...}									OPTIONAL,

clockFrequencyStability			BIT STRING (SIZE(16))											OPTIONAL,

clockAccuracy					ClockAccuracy													OPTIONAL,

parentTImeSource				ParentTImeSource												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {RANTimingSynchronisationStatusInfo-ExtIEs} }	OPTIONAL,

...

}

RANTimingSynchronisationStatusInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RAN-TSSRequestType ::= ENUMERATED {start, stop, ...}

RAN-TSSScope	::= CHOICE {

rANNodeLevel			GlobalGNB-ID,

cellListLevel			RANTSSCellList,

choice-Extensions		ProtocolIE-SingleContainer { { RAN-TSSScope-ExtIEs} }

}

RAN-TSSScope-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

RANTSSCellList ::= SEQUENCE (SIZE(1..maxnoofCellsTSS)) OF RANTSSCellItem

RANTSSCellItem ::= SEQUENCE {

nRCGI				NR-CGI,

iE-Extensions		ProtocolExtensionContainer { {RANTSSCellItem-ExtIEs} }	OPTIONAL,

...

}

RANTSSCellItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RAN-UE-NGAP-ID ::= INTEGER (0..4294967295)

RAT-Information ::= ENUMERATED {

unlicensed,

nb-IoT,

...,

nR-LEO,

nR-MEO,

nR-GEO,

nR-OTHERSAT

}

RATRestrictions ::= SEQUENCE (SIZE(1..maxnoofEPLMNsPlusOne)) OF RATRestrictions-Item

RATRestrictions-Item ::= SEQUENCE {

pLMNIdentity					PLMNIdentity,

rATRestrictionInformation		RATRestrictionInformation,

iE-Extensions		ProtocolExtensionContainer { {RATRestrictions-Item-ExtIEs} }		OPTIONAL,

...

}

RATRestrictions-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedRATRestrictionInformation		CRITICALITY ignore	EXTENSION ExtendedRATRestrictionInformation		PRESENCE optional	},

...

}

RATRestrictionInformation ::= BIT STRING (SIZE(8, ...))

RecommendedCellsForPaging ::= SEQUENCE {

recommendedCellList			RecommendedCellList,

iE-Extensions		ProtocolExtensionContainer { {RecommendedCellsForPaging-ExtIEs} }	OPTIONAL,

...

}

RecommendedCellsForPaging-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RecommendedCellList ::= SEQUENCE (SIZE(1..maxnoofRecommendedCells)) OF RecommendedCellItem

RecommendedCellItem ::= SEQUENCE {

nGRAN-CGI				NGRAN-CGI,

timeStayedInCell		INTEGER (0..4095)		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {RecommendedCellItem-ExtIEs} }	OPTIONAL,

...

}

RecommendedCellItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RecommendedRANNodesForPaging ::= SEQUENCE {

recommendedRANNodeList		RecommendedRANNodeList,

iE-Extensions		ProtocolExtensionContainer { {RecommendedRANNodesForPaging-ExtIEs} }	OPTIONAL,

...

}

RecommendedRANNodesForPaging-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RecommendedRANNodeList::= SEQUENCE (SIZE(1..maxnoofRecommendedRANNodes)) OF RecommendedRANNodeItem

RecommendedRANNodeItem ::= SEQUENCE {

aMFPagingTarget		AMFPagingTarget,

iE-Extensions		ProtocolExtensionContainer { {RecommendedRANNodeItem-ExtIEs} }	OPTIONAL,

...

}

RecommendedRANNodeItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RedCapIndication ::= ENUMERATED {

redcap,

...

}

RedirectionVoiceFallback ::= ENUMERATED {

possible,

not-possible,

...

}

RedundantPDUSessionInformation ::= SEQUENCE {

rSN					RSN,

iE-Extensions		ProtocolExtensionContainer { {RedundantPDUSessionInformation-ExtIEs} }	OPTIONAL,

...

}

RedundantPDUSessionInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSessionPairID	CRITICALITY ignore	EXTENSION PDUSessionPairID	PRESENCE optional	},

...

}

RedundantQosFlowIndicator ::= ENUMERATED {true, false}

ReflectiveQosAttribute ::= ENUMERATED {

subject-to,

...

}

RelativeAMFCapacity ::= INTEGER (0..255)

ReportArea ::= ENUMERATED {

cell,

...

}

RepetitionPeriod ::= INTEGER (0..131071)

ResetAll ::= ENUMERATED {

reset-all,

...

}

ReportAmountMDT ::= ENUMERATED {

r1, r2, r4, r8, r16, r32, r64, rinfinity

}

ReportIntervalMDT ::= ENUMERATED {

ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, min60

}

ExtendedReportIntervalMDT ::= ENUMERATED {

ms20480, ms40960, ...

}

ResetType ::= CHOICE {

nG-Interface			ResetAll,

partOfNG-Interface		UE-associatedLogicalNG-connectionList,

choice-Extensions		ProtocolIE-SingleContainer { {ResetType-ExtIEs} }

}

ResetType-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

RGLevelWirelineAccessCharacteristics ::= OCTET STRING

RNC-ID ::= INTEGER (0..4095)

RoutingID ::= OCTET STRING

RRCContainer ::= OCTET STRING

RRCEstablishmentCause ::= ENUMERATED {

emergency,

highPriorityAccess,

mt-Access,

mo-Signalling,

mo-Data,

mo-VoiceCall,

mo-VideoCall,

mo-SMS,

mps-PriorityAccess,

mcs-PriorityAccess,

...,

notAvailable,

mo-ExceptionData

}

RRCInactiveTransitionReportRequest ::= ENUMERATED {

subsequent-state-transition-report,

single-rrc-connected-state-report,

cancel-report,

...

}

RRCState ::= ENUMERATED {

inactive,

connected,

...

}

RSN ::= ENUMERATED {v1, v2, ...}

RIMInformationTransfer ::= SEQUENCE {

targetRANNodeID-RIM			TargetRANNodeID-RIM,

sourceRANNodeID				SourceRANNodeID,

rIMInformation				RIMInformation,

iE-Extensions				ProtocolExtensionContainer { {RIMInformationTransfer-ExtIEs} }	OPTIONAL,

...

}

RIMInformationTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RIMInformation	::= SEQUENCE		{

targetgNBSetID			GNBSetID,

rIM-RSDetection			ENUMERATED	{rs-detected, rs-disappeared, ...},

iE-Extensions			ProtocolExtensionContainer { {RIMInformation-ExtIEs} }	OPTIONAL,

...

}

RIMInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

GNBSetID ::= BIT STRING (SIZE(22))

RSPPQoSFlowList ::= SEQUENCE (SIZE(1..maxnoofRSPPQoSFlows)) OF RSPPQoSFlowItem

RSPPQoSFlowItem ::= SEQUENCE {

pQI					FiveQI,

rSPPFlowBitRates	RSPPFlowBitRates											OPTIONAL,

range				Range														OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { RSPPQoSFlowItem-ExtIEs} }	OPTIONAL,

...

}

RSPPQoSFlowItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

RSPPFlowBitRates ::= SEQUENCE {

guaranteedFlowBitRate		BitRate,

maximumFlowBitRate			BitRate,

iE-Extensions		ProtocolExtensionContainer { { RSPPFlowBitRates-ExtIEs} }	OPTIONAL,

...

}

RSPPFlowBitRates-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- S

ScheduledCommunicationTime ::= SEQUENCE {

dayofWeek			BIT STRING (SIZE(7))												OPTIONAL,

timeofDayStart		INTEGER (0..86399, ...)												OPTIONAL,

timeofDayEnd		INTEGER (0..86399, ...)												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ScheduledCommunicationTime-ExtIEs}}	OPTIONAL,

...

}

ScheduledCommunicationTime-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SCTP-TLAs	::= SEQUENCE (SIZE(1..maxnoofXnTLAs)) OF TransportLayerAddress

SD ::= OCTET STRING (SIZE(3))

SecondaryRATUsageInformation ::= SEQUENCE {

pDUSessionUsageReport		PDUSessionUsageReport											OPTIONAL,

qosFlowsUsageReportList		QoSFlowsUsageReportList											OPTIONAL,

iE-Extension		ProtocolExtensionContainer { {SecondaryRATUsageInformation-ExtIEs} }	OPTIONAL,

...

}

SecondaryRATUsageInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SecondaryRATDataUsageReportTransfer ::= SEQUENCE {

secondaryRATUsageInformation		SecondaryRATUsageInformation									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SecondaryRATDataUsageReportTransfer-ExtIEs} }	OPTIONAL,

...

}

SecondaryRATDataUsageReportTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SecurityContext ::= SEQUENCE {

nextHopChainingCount		NextHopChainingCount,

nextHopNH					SecurityKey,

iE-Extensions		ProtocolExtensionContainer { {SecurityContext-ExtIEs} }	OPTIONAL,

...

}

SecurityContext-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SecurityIndication ::= SEQUENCE {

integrityProtectionIndication				IntegrityProtectionIndication,

confidentialityProtectionIndication			ConfidentialityProtectionIndication,

maximumIntegrityProtectedDataRate-UL		MaximumIntegrityProtectedDataRate		OPTIONAL,

-- The above IE shall be present if the Integrity Protection Indication IE is set to the value “required” or “preferred”

iE-Extensions		ProtocolExtensionContainer { {SecurityIndication-ExtIEs} }		OPTIONAL,

...

}

SecurityIndication-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MaximumIntegrityProtectedDataRate-DL	CRITICALITY ignore	EXTENSION MaximumIntegrityProtectedDataRate	PRESENCE optional	},

...

}

SecurityKey	::= BIT STRING (SIZE(256))

SecurityResult ::= SEQUENCE {

integrityProtectionResult			IntegrityProtectionResult,

confidentialityProtectionResult		ConfidentialityProtectionResult,

iE-Extensions		ProtocolExtensionContainer { {SecurityResult-ExtIEs} }	OPTIONAL,

...

}

SecurityResult-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SensorMeasurementConfiguration ::=	SEQUENCE {

sensorMeasConfig            SensorMeasConfig,

sensorMeasConfigNameList	SensorMeasConfigNameList            							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SensorMeasurementConfiguration-ExtIEs} } 	OPTIONAL,

...

}

SensorMeasurementConfiguration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SensorMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofSensorName)) OF SensorMeasConfigNameItem

SensorMeasConfigNameItem ::= SEQUENCE {

sensorNameConfig		SensorNameConfig,

iE-Extensions		ProtocolExtensionContainer { { SensorMeasConfigNameItem-ExtIEs } } 	OPTIONAL,

...

}

SensorMeasConfigNameItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SensorMeasConfig::= ENUMERATED {setup,...}

SensorNameConfig ::= CHOICE {

uncompensatedBarometricConfig		ENUMERATED {true, ...},

ueSpeedConfig						ENUMERATED {true, ...},

ueOrientationConfig					ENUMERATED {true, ...},

choice-Extensions		ProtocolIE-SingleContainer { {SensorNameConfig-ExtIEs} }

}

SensorNameConfig-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

SerialNumber ::= BIT STRING (SIZE(16))

ServedGUAMIList ::= SEQUENCE (SIZE(1..maxnoofServedGUAMIs)) OF ServedGUAMIItem

ServedGUAMIItem ::= SEQUENCE {

gUAMI				GUAMI,

backupAMFName		AMFName													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServedGUAMIItem-ExtIEs} }	OPTIONAL,

...

}

ServedGUAMIItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-GUAMIType		CRITICALITY ignore	EXTENSION GUAMIType		PRESENCE optional	},

...

}

ServiceAreaInformation ::= SEQUENCE (SIZE(1.. maxnoofEPLMNsPlusOne)) OF ServiceAreaInformation-Item

ServiceAreaInformation-Item ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

allowedTACs			AllowedTACs																OPTIONAL,

notAllowedTACs		NotAllowedTACs															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServiceAreaInformation-Item-ExtIEs} }		OPTIONAL,

...

}

ServiceAreaInformation-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ServiceType ::=  ENUMERATED {streaming, mTSI, vR, ...}

SgNB-UE-X2AP-ID ::= INTEGER (0..4294967295)

SharedNGU-MulticastTNLInformation ::= SEQUENCE {

iP-MulticastAddress 		TransportLayerAddress,

iP-SourceAddress			TransportLayerAddress,

gTP-TEID						GTP-TEID,

iE-Extensions				ProtocolExtensionContainer { {SharedNGU-MulticastTNLInformation-ExtIEs} } 	OPTIONAL,

...

}

SharedNGU-MulticastTNLInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SliceOverloadList ::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF SliceOverloadItem

SliceOverloadItem ::= SEQUENCE {

s-NSSAI				S-NSSAI,

iE-Extensions		ProtocolExtensionContainer { {SliceOverloadItem-ExtIEs} }	OPTIONAL,

...

}

SliceOverloadItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SliceSupportList ::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF SliceSupportItem

SliceSupportItem ::= SEQUENCE {

s-NSSAI				S-NSSAI,

iE-Extensions		ProtocolExtensionContainer { {SliceSupportItem-ExtIEs} }	OPTIONAL,

...

}

SliceSupportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SliceSupportListQMC ::= SEQUENCE (SIZE(1..maxnoofSNSSAIforQMC)) OF SliceSupportQMC-Item

SliceSupportQMC-Item ::= SEQUENCE {

s-NSSAI				S-NSSAI,

iE-Extensions		ProtocolExtensionContainer { {SliceSupportQMC-Item-ExtIEs} }		OPTIONAL,

...

}

SliceSupportQMC-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SNPN-MobilityInformation ::= SEQUENCE {

serving-NID			NID,

iE-Extensions		ProtocolExtensionContainer { {SNPN-MobilityInformation-ExtIEs} }	OPTIONAL,

...

}

SNPN-MobilityInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-EquivalentSNPNsList CRITICALITY reject EXTENSION EquivalentSNPNsList		PRESENCE optional},

...

}

S-NSSAI ::= SEQUENCE {

sST					SST,

sD					SD													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { S-NSSAI-ExtIEs} }	OPTIONAL,

...

}

S-NSSAI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SONConfigurationTransfer ::= SEQUENCE {

targetRANNodeID-SON			TargetRANNodeID-SON,

sourceRANNodeID				SourceRANNodeID,

sONInformation				SONInformation,

xnTNLConfigurationInfo		XnTNLConfigurationInfo										OPTIONAL,

-- The above IE shall be present if the SON Information IE contains the SON Information Request IE set to the value “Xn TNL Configuration Info”

iE-Extensions		ProtocolExtensionContainer { {SONConfigurationTransfer-ExtIEs} }	OPTIONAL,

...

}

SONConfigurationTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SONInformation ::= CHOICE {

sONInformationRequest		SONInformationRequest,

sONInformationReply			SONInformationReply,

choice-Extensions		ProtocolIE-SingleContainer { {SONInformation-ExtIEs} }

}

SONInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-SONInformationReport		CRITICALITY ignore	TYPE SONInformationReport		PRESENCE mandatory	},

...

}

SONInformationReply ::= SEQUENCE {

xnTNLConfigurationInfo		XnTNLConfigurationInfo									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SONInformationReply-ExtIEs} }		OPTIONAL,

...

}

SONInformationReply-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SONInformationReport::= CHOICE {

failureIndicationInformation	FailureIndication,

hOReportInformation				HOReport,

choice-Extensions		ProtocolIE-SingleContainer { { SONInformationReport-ExtIEs} }

}

SONInformationReport-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-SuccessfulHandoverReportList			    CRITICALITY ignore	TYPE SuccessfulHandoverReportList			PRESENCE mandatory }|

{ ID id-SuccessfulPSCellChangeReportList		    CRITICALITY ignore	TYPE SuccessfulPSCellChangeReportList	PRESENCE mandatory },

...

}

SNPN-CellBasedMDT::= SEQUENCE {

sNPNcellIdListforMDT	SNPNCellIdListforMDT,

iE-Extensions		ProtocolExtensionContainer { {SNPN-CellBasedMDT-ExtIEs} } OPTIONAL,

...

}

SNPN-CellBasedMDT-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SNPNCellIdListforMDT ::= SEQUENCE (SIZE(1..maxnoofCellIDforMDT)) OF SNPNCellIdListforMDTItem

SNPNCellIdListforMDTItem ::= SEQUENCE {

nR-CGI								NR-CGI,

nID									NID,

iE-Extensions		ProtocolExtensionContainer {{ SNPNCellIdListforMDTItem-ExtIEs}}		OPTIONAL,

...

}

SNPNCellIdListforMDTItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SNPN-TAIBasedMDT::= SEQUENCE {

sNPNTAIListforMDT	SNPNTAIListforMDT,

iE-Extensions		ProtocolExtensionContainer { {SNPN-TAIBasedMDT-ExtIEs} } OPTIONAL,

...

}

SNPN-TAIBasedMDT-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SNPNTAIListforMDT ::= SEQUENCE (SIZE(1.. maxnoofTAforMDT)) OF SNPNTAIListforMDTItem

SNPNTAIListforMDTItem ::= SEQUENCE {

tAI									TAI,

nID									NID,

iE-Extensions		ProtocolExtensionContainer {{ SNPNTAIListforMDTItem-ExtIEs}}		OPTIONAL,

...

}

SNPNTAIListforMDTItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SNPN-BasedMDT::= SEQUENCE {

sNPNListforMDT		SNPNListforMDT,

iE-Extensions		ProtocolExtensionContainer { {SNPN-BasedMDT-ExtIEs} } OPTIONAL,

...

}

SNPN-BasedMDT-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SNPNListforMDT ::= SEQUENCE (SIZE(1.. maxnoofMDTSNPNs)) OF SNPNListforMDTItem

SNPNListforMDTItem ::= SEQUENCE {

pLMNIdentity						PLMNIdentity,

nID									NID,

iE-Extensions		ProtocolExtensionContainer {{ SNPNListforMDTItem-ExtIEs}}		OPTIONAL,

...

}

SNPNListforMDTItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- --------------------------------------------------------------------

-- SON Information Report

-- --------------------------------------------------------------------

SuccessfulHandoverReportList ::= SEQUENCE (SIZE(1..maxnoofSuccessfulHOReports)) OF SuccessfulHandoverReport-Item

SuccessfulHandoverReport-Item ::= SEQUENCE {

successfulHOReportContainer			OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { { SuccessfulHandoverReport-Item-ExtIEs} } OPTIONAL,

...

}

SuccessfulHandoverReport-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SuccessfulPSCellChangeReportList ::= SEQUENCE (SIZE(1..maxnoofSuccessfulPSCellChangeReports)) OF SuccessfulPSCellChangeReport-Item

SuccessfulPSCellChangeReport-Item ::= SEQUENCE {

successfulPSCellChangeReportContainer			OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { { SuccessfulPSCellChangeReport-Item-ExtIEs} } OPTIONAL,

...

}

SuccessfulPSCellChangeReport-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SONInformationRequest ::= ENUMERATED {

xn-TNL-configuration-info,

...

}

SourceNGRANNode-ToTargetNGRANNode-TransparentContainer ::= SEQUENCE {

rRCContainer							RRCContainer,

pDUSessionResourceInformationList		PDUSessionResourceInformationList													OPTIONAL,

e-RABInformationList					E-RABInformationList																OPTIONAL,

targetCell-ID							NGRAN-CGI,

indexToRFSP								IndexToRFSP																			OPTIONAL,

uEHistoryInformation					UEHistoryInformation,

iE-Extensions		ProtocolExtensionContainer { {SourceNGRANNode-ToTargetNGRANNode-TransparentContainer-ExtIEs} }	OPTIONAL,

...

}

SourceNGRANNode-ToTargetNGRANNode-TransparentContainer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SgNB-UE-X2AP-ID								CRITICALITY ignore	EXTENSION SgNB-UE-X2AP-ID 							PRESENCE optional		}|

{ ID id-UEHistoryInformationFromTheUE				CRITICALITY ignore	EXTENSION UEHistoryInformationFromTheUE			PRESENCE optional		}|

{ ID id-SourceNodeID								CRITICALITY ignore	EXTENSION SourceNodeID								PRESENCE optional		}|

{ ID id-UEContextReferenceAtSource 					CRITICALITY ignore	EXTENSION RAN-UE-NGAP-ID								PRESENCE optional		}|

{ ID id-MBS-ActiveSessionInformation-SourcetoTargetList	CRITICALITY ignore	EXTENSION MBS-ActiveSessionInformation-SourcetoTargetList		PRESENCE optional		}|

{ ID id-QMCConfigInfo								CRITICALITY ignore	EXTENSION QMCConfigInfo								PRESENCE optional		}|

{ ID id-NGAPIESupportInformationRequestList			CRITICALITY ignore	EXTENSION NGAPIESupportInformationRequestList	PRESENCE optional		}|

{ ID id-CandidateRelayUEInformationList				CRITICALITY reject	EXTENSION CandidateRelayUEInformationList		PRESENCE optional		}|

{ ID id-TimeBasedHandoverInformation				CRITICALITY ignore	EXTENSION TimeBasedHandoverInformation			PRESENCE optional		}|

{ ID id-SourceSN-to-TargetSN-QMCInfo				CRITICALITY ignore	EXTENSION QMCConfigInfo								PRESENCE optional 	},

...

}

SourceNodeID ::= CHOICE {

sourceengNB-ID			GlobalGNB-ID,

choice-Extensions		ProtocolIE-SingleContainer { { SourceNodeID-ExtIEs} }

}

SourceNodeID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

SourceOfUEActivityBehaviourInformation ::= ENUMERATED {

subscription-information,

statistics,

...

}

SourceRANNodeID ::= SEQUENCE {

globalRANNodeID		GlobalRANNodeID,

selectedTAI			TAI,

iE-Extensions		ProtocolExtensionContainer { {SourceRANNodeID-ExtIEs} } OPTIONAL,

...

}

SourceRANNodeID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SourceToTarget-TransparentContainer ::= OCTET STRING

-- This IE includes a transparent container from the source RAN node to the target RAN node.

-- The octets of the OCTET STRING are encoded according to the specifications of the target system.

SourceToTarget-AMFInformationReroute ::= SEQUENCE {

configuredNSSAI					ConfiguredNSSAI														OPTIONAL,

rejectedNSSAIinPLMN				RejectedNSSAIinPLMN													OPTIONAL,

rejectedNSSAIinTA				RejectedNSSAIinTA													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SourceToTarget-AMFInformationReroute-ExtIEs} }	OPTIONAL,

...

}

SourceToTarget-AMFInformationReroute-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- This IE includes information from the source Core node to the target Core node for reroute information provide by NSSF.

-- The octets of the OCTET STRING are encoded according to the specifications of the Core network.

SRVCCOperationPossible ::= ENUMERATED {

possible,

notPossible,

...

}

ConfiguredNSSAI  ::=  OCTET STRING (SIZE(128))

RejectedNSSAIinPLMN ::= OCTET STRING (SIZE(32))

RejectedNSSAIinTA ::= OCTET STRING (SIZE(32))

SST ::= OCTET STRING (SIZE(1))

SupportedTAList ::= SEQUENCE (SIZE(1..maxnoofTACs)) OF SupportedTAItem

SupportedTAItem ::= SEQUENCE {

tAC						TAC,

broadcastPLMNList		BroadcastPLMNList,

iE-Extensions		ProtocolExtensionContainer { {SupportedTAItem-ExtIEs} } OPTIONAL,

...

}

SupportedTAItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication	PRESENCE optional	}|

{ID id-RAT-Information				CRITICALITY reject	EXTENSION RAT-Information			PRESENCE optional	},

...

}

SuspendIndicator ::= ENUMERATED {

true,

...

}

Suspend-Request-Indication ::= ENUMERATED {

suspend-requested,

...

}

Suspend-Response-Indication ::= ENUMERATED {

suspend-indicated,

...

}

SurvivalTime ::= INTEGER (0..1920000, ...)

SLPositioningRangingServiceInfo ::= SEQUENCE{

sLPositioningRangingAuthorized		SLPositioningRangingAuthorized,

sLPositioningRangingQoSParameters	SLPositioningRangingQoSParameters	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SLPositioningRangingServiceInfo-ExtIEs} }	OPTIONAL,

...

}

SLPositioningRangingServiceInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SLPositioningRangingAuthorized ::= ENUMERATED {

authorized,

not-authorized,

...

}

SLPositioningRangingQoSParameters ::= SEQUENCE {

rSPPQoSFlowList				RSPPQoSFlowList,

rSPPLinkAggregateBitRates	BitRate												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { SLPositioningRangingQoSParameters-ExtIEs} }	OPTIONAL,

...

}

SLPositioningRangingQoSParameters-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- T

TAC ::= OCTET STRING (SIZE(3))

TACListInNRNTN ::= SEQUENCE (SIZE(1..maxnoofTACsinNTN)) OF TAC

TAI ::= SEQUENCE {

pLMNIdentity		PLMNIdentity,

tAC					TAC,

iE-Extensions		ProtocolExtensionContainer { {TAI-ExtIEs} } OPTIONAL,

...

}

TAI-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIBroadcastEUTRA ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAIBroadcastEUTRA-Item

TAIBroadcastEUTRA-Item ::= SEQUENCE {

tAI								TAI,

completedCellsInTAI-EUTRA		CompletedCellsInTAI-EUTRA,

iE-Extensions		ProtocolExtensionContainer { {TAIBroadcastEUTRA-Item-ExtIEs} } OPTIONAL,

...

}

TAIBroadcastEUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIBroadcastNR ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAIBroadcastNR-Item

TAIBroadcastNR-Item ::= SEQUENCE {

tAI							TAI,

completedCellsInTAI-NR		CompletedCellsInTAI-NR,

iE-Extensions		ProtocolExtensionContainer { {TAIBroadcastNR-Item-ExtIEs} } OPTIONAL,

...

}

TAIBroadcastNR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAICancelledEUTRA ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAICancelledEUTRA-Item

TAICancelledEUTRA-Item ::= SEQUENCE {

tAI								TAI,

cancelledCellsInTAI-EUTRA		CancelledCellsInTAI-EUTRA,

iE-Extensions		ProtocolExtensionContainer { {TAICancelledEUTRA-Item-ExtIEs} } OPTIONAL,

...

}

TAICancelledEUTRA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAICancelledNR ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAICancelledNR-Item

TAICancelledNR-Item ::= SEQUENCE {

tAI							TAI,

cancelledCellsInTAI-NR		CancelledCellsInTAI-NR,

iE-Extensions		ProtocolExtensionContainer { {TAICancelledNR-Item-ExtIEs} } OPTIONAL,

...

}

TAICancelledNR-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListForInactive ::= SEQUENCE (SIZE(1..maxnoofTAIforInactive)) OF TAIListForInactiveItem

TAIListForInactiveItem ::= SEQUENCE {

tAI					TAI,

iE-Extensions		ProtocolExtensionContainer { {TAIListForInactiveItem-ExtIEs} } OPTIONAL,

...

}

TAIListForInactiveItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListForPaging ::= SEQUENCE (SIZE(1..maxnoofTAIforPaging)) OF TAIListForPagingItem

TAIListForPagingItem ::= SEQUENCE {

tAI					TAI,

iE-Extensions		ProtocolExtensionContainer { {TAIListForPagingItem-ExtIEs} } OPTIONAL,

...

}

TAIListForPagingItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListForRestart ::= SEQUENCE (SIZE(1..maxnoofTAIforRestart)) OF TAI

TAIListForWarning ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAI

TAINSAGSupportList ::= SEQUENCE (SIZE(1..maxnoofNSAGs)) OF TAINSAGSupportItem

TAINSAGSupportItem ::= SEQUENCE {

nSAG-ID						NSAG-ID,

nSAGSliceSupportList		ExtendedSliceSupportList,

iE-Extensions		ProtocolExtensionContainer { {TAINSAGSupportItem-ExtIEs} } OPTIONAL,

...

}

TAINSAGSupportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TargeteNB-ID ::= SEQUENCE {

globalENB-ID		GlobalNgENB-ID,

selected-EPS-TAI	EPS-TAI,

iE-Extensions		ProtocolExtensionContainer { {TargeteNB-ID-ExtIEs} } OPTIONAL,

...

}

TargeteNB-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TargetHomeENB-ID ::= SEQUENCE {

pLMNidentity		PLMNIdentity,

homeENB-ID			BIT STRING (SIZE(28)),

selected-EPS-TAI	EPS-TAI,

iE-Extensions		ProtocolExtensionContainer { {TargetHomeENB-ID-ExtIEs} } OPTIONAL,

...

}

TargetHomeENB-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TargetID ::= CHOICE {

targetRANNodeID			TargetRANNodeID,

targeteNB-ID			TargeteNB-ID,

choice-Extensions		ProtocolIE-SingleContainer { {TargetID-ExtIEs} }

}

TargetID-ExtIEs NGAP-PROTOCOL-IES ::= {

{ID id-TargetRNC-ID			CRITICALITY reject	TYPE TargetRNC-ID 		PRESENCE mandatory }|

{ID id-TargetHomeENB-ID		CRITICALITY reject	TYPE TargetHomeENB-ID	PRESENCE mandatory },

...

}

TargetNGRANNode-ToSourceNGRANNode-TransparentContainer ::= SEQUENCE {

rRCContainer		RRCContainer,

iE-Extensions		ProtocolExtensionContainer { {TargetNGRANNode-ToSourceNGRANNode-TransparentContainer-ExtIEs} } OPTIONAL,

...

}

TargetNGRANNode-ToSourceNGRANNode-TransparentContainer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-DAPSResponseInfoList						CRITICALITY ignore	EXTENSION DAPSResponseInfoList									PRESENCE optional }|

{ ID id-DirectForwardingPathAvailability			CRITICALITY ignore	EXTENSION DirectForwardingPathAvailability					PRESENCE optional }|

{ ID id-MBS-ActiveSessionInformation-TargettoSourceList	CRITICALITY ignore	EXTENSION MBS-ActiveSessionInformation-TargettoSourceList		PRESENCE optional }|

{ ID id-NGAPIESupportInformationResponseList		CRITICALITY ignore	EXTENSION NGAPIESupportInformationResponseList 			PRESENCE optional },

...

}

TargetNGRANNode-ToSourceNGRANNode-FailureTransparentContainer ::= SEQUENCE {

cell-CAGInformation		Cell-CAGInformation			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {TargetNGRANNode-ToSourceNGRANNode-FailureTransparentContainer-ExtIEs} } OPTIONAL,

...

}

TargetNGRANNode-ToSourceNGRANNode-FailureTransparentContainer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-NGAPIESupportInformationResponseList	CRITICALITY ignore	EXTENSION NGAPIESupportInformationResponseList 		PRESENCE optional },

...

}

TargetNSSAI ::= SEQUENCE (SIZE(1..maxnoofTargetS-NSSAIs)) OF TargetNSSAI-Item

TargetNSSAI-Item ::= SEQUENCE {

s-NSSAI				S-NSSAI,

iE-Extensions		ProtocolExtensionContainer { {TargetNSSAI-Item-ExtIEs} } OPTIONAL,

...

}

TargetNSSAI-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TargetNSSAIInformation ::= SEQUENCE {

targetNSSAI			TargetNSSAI,

indexToRFSP			IndexToRFSP,

iE-Extensions		ProtocolExtensionContainer { {TargetNSSAIInformation-Item-ExtIEs} } OPTIONAL,

...

}

TargetNSSAIInformation-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TargetRANNodeID ::= SEQUENCE {

globalRANNodeID		GlobalRANNodeID,

selectedTAI			TAI,

iE-Extensions		ProtocolExtensionContainer { {TargetRANNodeID-ExtIEs} } OPTIONAL,

...

}

TargetRANNodeID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-Selected-Target-SNPN-Identity CRITICALITY reject	EXTENSION NID PRESENCE optional },

...

}

TargetRANNodeID-RIM ::= SEQUENCE {

globalRANNodeID		GlobalRANNodeID,

selectedTAI			TAI,

iE-Extensions		ProtocolExtensionContainer { {TargetRANNodeID-RIM-ExtIEs} } OPTIONAL,

...

}

TargetRANNodeID-RIM-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TargetRANNodeID-SON ::= SEQUENCE {

globalRANNodeID		GlobalRANNodeID,

selectedTAI			TAI,

iE-Extensions		ProtocolExtensionContainer { {TargetRANNodeID-SON-ExtIEs} } OPTIONAL,

...

}

TargetRANNodeID-SON-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-NR-CGI	CRITICALITY ignore	EXTENSION NR-CGI PRESENCE optional },

...

}

TargetRNC-ID ::= SEQUENCE {

lAI					LAI,

rNC-ID				RNC-ID,

extendedRNC-ID		ExtendedRNC-ID											OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TargetRNC-ID-ExtIEs} } 	OPTIONAL,

...

}

TargetRNC-ID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TargetToSource-TransparentContainer ::= OCTET STRING

-- This IE includes a transparent container from the target RAN node to the source RAN node.

-- The octets of the OCTET STRING are encoded according to the specifications of the target system.

TargettoSource-Failure-TransparentContainer ::= OCTET STRING

-- This IE includes a transparent container from the target RAN node to the source RAN node.

-- The octets of the OCTET STRING are encoded according to the specifications of the target system (if applicable).

TimeBasedHandoverInformation ::= SEQUENCE {

hOWindowStart			HandoverWindowStart,

hOWindowDuration		HandoverWindowDuration,

iE-Extensions			ProtocolExtensionContainer { {TimeBasedHandoverInformation-ExtIEs} }	OPTIONAL,

...

}

TimeBasedHandoverInformation-ExtIEs	NGAP-PROTOCOL-EXTENSION ::= {

...

}

HandoverWindowStart ::= INTEGER (0..549755813887)

HandoverWindowDuration ::= INTEGER (1..6000)

TimerApproachForGUAMIRemoval ::= ENUMERATED {

apply-timer,

...

}

TimeSinceFailure ::= INTEGER (0..172800, ...)

TimeStamp ::= OCTET STRING (SIZE(4))

TimeSyncAssistanceInfo ::= SEQUENCE {

timeDistributionIndication		ENUMERATED {enabled, disabled, ...},

uUTimeSyncErrorBudget			INTEGER (1..1000000, ...)							OPTIONAL,

-- The above IE shall be present if the Time Distribution Indication IE is set to the value “enabled”

iE-Extensions		ProtocolExtensionContainer { {TimeSyncAssistanceInfo-ExtIEs} }	OPTIONAL,

...

}

TimeSyncAssistanceInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-ClockQualityReportingControlInfo	CRITICALITY ignore	EXTENSION ClockQualityReportingControlInfo		PRESENCE optional},

...

}

TimeToWait ::= ENUMERATED {v1s, v2s, v5s, v10s, v20s, v60s, ...}

TimeUEStayedInCell ::= INTEGER (0..4095)

TimeUEStayedInCellEnhancedGranularity ::= INTEGER (0..40950)

TMGI ::=  OCTET STRING (SIZE(6))

TNAP-ID ::= OCTET STRING

TNGF-ID ::= CHOICE {

tNGF-ID				 	BIT STRING (SIZE(32, ...)),

choice-Extensions		ProtocolIE-SingleContainer { {TNGF-ID-ExtIEs} }

}

TNGF-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

TNLAddressWeightFactor ::= INTEGER (0..255)

TNLAssociationList ::= SEQUENCE (SIZE(1..maxnoofTNLAssociations)) OF TNLAssociationItem

TNLAssociationItem ::= SEQUENCE {

tNLAssociationAddress		CPTransportLayerInformation,

cause						Cause,

iE-Extensions		ProtocolExtensionContainer { {TNLAssociationItem-ExtIEs} } OPTIONAL,

...

}

TNLAssociationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TNLAssociationUsage ::= ENUMERATED {

ue,

non-ue,

both,

...

}

TooearlyIntersystemHO::= SEQUENCE {

sourcecellID			EUTRA-CGI,

failurecellID			NGRAN-CGI,

uERLFReportContainer	UERLFReportContainer		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { TooearlyIntersystemHO-ExtIEs} }			OPTIONAL,

...

}

TooearlyIntersystemHO-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TraceActivation ::= SEQUENCE {

nGRANTraceID						NGRANTraceID,

interfacesToTrace					InterfacesToTrace,

traceDepth							TraceDepth,

traceCollectionEntityIPAddress		TransportLayerAddress,

iE-Extensions		ProtocolExtensionContainer { {TraceActivation-ExtIEs} }	OPTIONAL,

...

}

TraceActivation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MDTConfiguration			CRITICALITY ignore	EXTENSION MDT-Configuration		PRESENCE optional 	}|

{ ID id-TraceCollectionEntityURI	CRITICALITY ignore	EXTENSION URI-address			PRESENCE optional		},

...

}

TraceDepth ::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...

}

TrafficLoadReductionIndication ::= INTEGER (1..99)

TransportLayerAddress ::= BIT STRING (SIZE(1..160, ...))

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

TAIBasedMDT ::= SEQUENCE {

tAIListforMDT			TAIListforMDT,

iE-Extensions			ProtocolExtensionContainer { {TAIBasedMDT-ExtIEs} } OPTIONAL,

...

}

TAIBasedMDT-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAI

TAIBasedQMC ::= SEQUENCE {

tAIListforQMC			TAIListforQMC,

iE-Extensions			ProtocolExtensionContainer { {TAIBasedQMC-ExtIEs} } OPTIONAL,

...

}

TAIBasedQMC-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAIListforQMC ::= SEQUENCE (SIZE(1..maxnoofTAforQMC)) OF TAI

TABasedQMC ::= SEQUENCE {

tAListforQMC			TAListforQMC,

iE-Extensions		ProtocolExtensionContainer { {TABasedQMC-ExtIEs} } OPTIONAL,

...

}

TABasedQMC-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAListforQMC ::= SEQUENCE (SIZE(1..maxnoofTAforQMC)) OF TAC

TABasedMDT ::= SEQUENCE {

tAListforMDT		TAListforMDT,

iE-Extensions		ProtocolExtensionContainer { {TABasedMDT-ExtIEs} } OPTIONAL,

...

}

TABasedMDT-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TAListforMDT ::= SEQUENCE (SIZE(1..maxnoofTAforMDT)) OF TAC

Threshold-RSRP ::= INTEGER(0..127)

Threshold-RSRQ ::= INTEGER(0..127)

Threshold-SINR ::= INTEGER(0..127)

TimeToTrigger ::= ENUMERATED {ms0, ms40, ms64, ms80, ms100, ms128, ms160, ms256, ms320, ms480, ms512, ms640, ms1024, ms1280, ms2560, ms5120}

TLContainer ::= OCTET STRING

TWAP-ID ::= OCTET STRING

TWIF-ID ::= CHOICE {

tWIF-ID				 	BIT STRING (SIZE(32, ...)),

choice-Extensions		ProtocolIE-SingleContainer { {TWIF-ID-ExtIEs} }

}

TWIF-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

TSCAssistanceInformation ::= SEQUENCE {

periodicity				Periodicity,

burstArrivalTime		BurstArrivalTime												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TSCAssistanceInformation-ExtIEs} }	OPTIONAL,

...

}

TSCAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SurvivalTime		CRITICALITY ignore	EXTENSION SurvivalTime			PRESENCE optional}|

{ ID id-RANfeedbacktype		CRITICALITY ignore	EXTENSION RANfeedbacktype		PRESENCE optional}|

{ ID id-N6JitterInformation	CRITICALITY ignore	EXTENSION N6JitterInformation	PRESENCE optional},

...

}

TSCTrafficCharacteristics ::= SEQUENCE {

tSCAssistanceInformationDL		TSCAssistanceInformation								OPTIONAL,

tSCAssistanceInformationUL		TSCAssistanceInformation								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TSCTrafficCharacteristics-ExtIEs} }	OPTIONAL,

...

}

TSCTrafficCharacteristics-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TSCTrafficCharacteristicsFeedback ::= SEQUENCE {

tSCFeedbackInformationDL		TSCFeedbackInformation								OPTIONAL,

tSCFeedbackInformationUL		TSCFeedbackInformation								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { TSCTrafficCharacteristicsFeedback-ExtIEs} }	OPTIONAL,

...

}

TSCTrafficCharacteristicsFeedback-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

TSCFeedbackInformation ::= SEQUENCE {

burstArrivalTimeOffset							INTEGER (-640000..640000, ...),

adjustedPeriodicity								Periodicity							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { TSCFeedbackInformation-ExtIEs} }	OPTIONAL,

...

}

TSCFeedbackInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- U

UEAggregateMaximumBitRate ::= SEQUENCE {

uEAggregateMaximumBitRateDL		BitRate,

uEAggregateMaximumBitRateUL		BitRate,

iE-Extensions		ProtocolExtensionContainer { {UEAggregateMaximumBitRate-ExtIEs} } OPTIONAL,

...

}

UEAggregateMaximumBitRate-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UEAppLayerMeasInfoList ::= SEQUENCE (SIZE(1..maxnoofUEAppLayerMeas)) OF UEAppLayerMeasInfoItem

UEAppLayerMeasInfoItem ::= SEQUENCE {

uEAppLayerMeasConfigInfo		UEAppLayerMeasConfigInfo,

iE-Extensions		ProtocolExtensionContainer { { UEAppLayerMeasInfoItem-ExtIEs} } OPTIONAL,

...

}

UEAppLayerMeasInfoItem-ExtIEs NGAP-PROTOCOL-EXTENSION::= {

...

}

UEAppLayerMeasConfigInfo ::= SEQUENCE {

qoEReference								QoEReference,

serviceType								ServiceType,

areaScopeOfQMC							AreaScopeOfQMC								OPTIONAL,

measCollEntityIPAddress					TransportLayerAddress,

qoEMeasurementStatus					ENUMERATED {ongoing,...}					OPTIONAL,

containerForAppLayerMeasConfig			OCTET STRING (SIZE(1..8000))				OPTIONAL,

measConfigAppLayerID					INTEGER (0..15, ...) 						OPTIONAL,

sliceSupportListQMC						SliceSupportListQMC 						OPTIONAL,

mDT-AlignmentInfo						MDT-AlignmentInfo							OPTIONAL,

availableRANVisibleQoEMetrics			AvailableRANVisibleQoEMetrics				OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { UEAppLayerMeasConfigInfo-ExtIEs} }		OPTIONAL,

...

}

UEAppLayerMeasConfigInfo-ExtIEs NGAP-PROTOCOL-EXTENSION::= {

{ ID id-AssistanceInformationQoE-Meas		CRITICALITY ignore	EXTENSION AssistanceInformationQoE-Meas		PRESENCE optional	}|

{ ID id-MBSCommServiceType					CRITICALITY ignore	EXTENSION MBSCommServiceType						PRESENCE optional	}|

{ ID id-QoERVQoEReportingPaths				CRITICALITY ignore	EXTENSION QoERVQoEReportingPaths					PRESENCE optional },

...

}

UE-associatedLogicalNG-connectionList ::= SEQUENCE (SIZE(1..maxnoofNGConnectionsToReset)) OF UE-associatedLogicalNG-connectionItem

UE-associatedLogicalNG-connectionItem ::= SEQUENCE {

aMF-UE-NGAP-ID		AMF-UE-NGAP-ID																		OPTIONAL,

rAN-UE-NGAP-ID		RAN-UE-NGAP-ID																		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UE-associatedLogicalNG-connectionItem-ExtIEs} }	OPTIONAL,

...

}

UE-associatedLogicalNG-connectionItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UECapabilityInfoRequest ::= ENUMERATED {

requested,

...

}

UEContextRequest ::= ENUMERATED {requested, ...}

UEContextResumeRequestTransfer ::= SEQUENCE {

qosFlowFailedToResumeList				QosFlowListWithCause									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UEContextResumeRequestTransfer-ExtIEs} }		OPTIONAL,

...

}

UEContextResumeRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UEContextResumeResponseTransfer ::= SEQUENCE {

qosFlowFailedToResumeList				QosFlowListWithCause									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UEContextResumeResponseTransfer-ExtIEs} }		OPTIONAL,

...

}

UEContextResumeResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UEContextSuspendRequestTransfer ::= SEQUENCE {

suspendIndicator				SuspendIndicator												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UEContextSuspendRequestTransfer-ExtIEs} }		OPTIONAL,

...

}

UEContextSuspendRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UE-DifferentiationInfo ::= SEQUENCE {

periodicCommunicationIndicator	ENUMERATED {periodically, ondemand, ... }							OPTIONAL,

periodicTime					INTEGER (1..3600, ...)												OPTIONAL,

scheduledCommunicationTime		ScheduledCommunicationTime 											OPTIONAL,

stationaryIndication			ENUMERATED {stationary, mobile, ...}								OPTIONAL,

trafficProfile					ENUMERATED {single-packet, dual-packets, multiple-packets, ...}	OPTIONAL,

batteryIndication				ENUMERATED {battery-powered, battery-powered-not-rechargeable-or-replaceable, not-battery-powered, ...}		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { UE-DifferentiationInfo-ExtIEs} } OPTIONAL,

...

}

UE-DifferentiationInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UEHistoryInformation ::= SEQUENCE (SIZE(1..maxnoofCellsinUEHistoryInfo)) OF LastVisitedCellItem

UEHistoryInformationFromTheUE ::= CHOICE {

nR						NRMobilityHistoryReport,

choice-Extensions		ProtocolIE-SingleContainer { {UEHistoryInformationFromTheUE-ExtIEs} }

}

UEHistoryInformationFromTheUE-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

UEIdentityIndexValue ::= CHOICE {

indexLength10			BIT STRING (SIZE(10)),

choice-Extensions		ProtocolIE-SingleContainer { {UEIdentityIndexValue-ExtIEs} }

}

UEIdentityIndexValue-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

UE-NGAP-IDs ::= CHOICE {

uE-NGAP-ID-pair		UE-NGAP-ID-pair,

aMF-UE-NGAP-ID		AMF-UE-NGAP-ID,

choice-Extensions		ProtocolIE-SingleContainer { {UE-NGAP-IDs-ExtIEs} }

}

UE-NGAP-IDs-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

UE-NGAP-ID-pair ::= SEQUENCE{

aMF-UE-NGAP-ID		AMF-UE-NGAP-ID,

rAN-UE-NGAP-ID		RAN-UE-NGAP-ID,

iE-Extensions		ProtocolExtensionContainer { {UE-NGAP-ID-pair-ExtIEs} } OPTIONAL,

...

}

UE-NGAP-ID-pair-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UEPagingIdentity ::= CHOICE {

fiveG-S-TMSI		FiveG-S-TMSI,

choice-Extensions		ProtocolIE-SingleContainer { {UEPagingIdentity-ExtIEs} }

}

UEPagingIdentity-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

UEPresence ::= ENUMERATED {in, out, unknown, ...}

UEPresenceInAreaOfInterestList ::= SEQUENCE (SIZE(1..maxnoofAoI)) OF UEPresenceInAreaOfInterestItem

UEPresenceInAreaOfInterestItem ::= SEQUENCE {

locationReportingReferenceID		LocationReportingReferenceID,

uEPresence							UEPresence,

iE-Extensions		ProtocolExtensionContainer { {UEPresenceInAreaOfInterestItem-ExtIEs} }	OPTIONAL,

...

}

UEPresenceInAreaOfInterestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UERadioCapability ::= OCTET STRING

UERadioCapabilityForPaging ::= SEQUENCE {

uERadioCapabilityForPagingOfNR		UERadioCapabilityForPagingOfNR					OPTIONAL,

uERadioCapabilityForPagingOfEUTRA		UERadioCapabilityForPagingOfEUTRA				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UERadioCapabilityForPaging-ExtIEs} }	OPTIONAL,

...

}

UERadioCapabilityForPaging-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-UERadioCapabilityForPagingOfNB-IoT		CRITICALITY ignore	EXTENSION UERadioCapabilityForPagingOfNB-IoT		PRESENCE optional	},

...

}

UERadioCapabilityForPagingOfNB-IoT ::= OCTET STRING

UERadioCapabilityForPagingOfNR ::= OCTET STRING

UERadioCapabilityForPagingOfEUTRA ::= OCTET STRING

UERadioCapabilityID ::= OCTET STRING

UERetentionInformation ::= ENUMERATED {

ues-retained,

...

}

UERLFReportContainer ::= CHOICE {

nR			NRUERLFReportContainer,

lTE			LTEUERLFReportContainer,

choice-Extensions		ProtocolIE-SingleContainer { {UERLFReportContainer-ExtIEs} }

}

UERLFReportContainer-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

UESecurityCapabilities ::= SEQUENCE {

nRencryptionAlgorithms					NRencryptionAlgorithms,

nRintegrityProtectionAlgorithms			NRintegrityProtectionAlgorithms,

eUTRAencryptionAlgorithms				EUTRAencryptionAlgorithms,

eUTRAintegrityProtectionAlgorithms		EUTRAintegrityProtectionAlgorithms,

iE-Extensions		ProtocolExtensionContainer { {UESecurityCapabilities-ExtIEs} }	OPTIONAL,

...

}

UESecurityCapabilities-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UESliceMaximumBitRateList ::= SEQUENCE (SIZE(1..maxnoofAllowedS-NSSAIs)) OF UESliceMaximumBitRateItem

UESliceMaximumBitRateItem ::= SEQUENCE {

s-NSSAI						S-NSSAI,

uESliceMaximumBitRateDL		BitRate,

uESliceMaximumBitRateUL		BitRate,

iE-Extensions		ProtocolExtensionContainer { { UESliceMaximumBitRateItem-ExtIEs} } OPTIONAL,

...

}

UESliceMaximumBitRateItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UE-UP-CIoT-Support ::= ENUMERATED {supported, ...}

UL-CP-SecurityInformation ::= SEQUENCE {

ul-NAS-MAC				UL-NAS-MAC,

ul-NAS-Count			UL-NAS-Count,

iE-Extensions			ProtocolExtensionContainer { { UL-CP-SecurityInformation-ExtIEs} }	OPTIONAL,

...

}

UL-CP-SecurityInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UL-NAS-MAC ::= BIT STRING (SIZE (16))

UL-NAS-Count ::= BIT STRING (SIZE (5))

UL-NGU-UP-TNLModifyList ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivity)) OF UL-NGU-UP-TNLModifyItem

UL-NGU-UP-TNLModifyItem ::= SEQUENCE {

uL-NGU-UP-TNLInformation		UPTransportLayerInformation,

dL-NGU-UP-TNLInformation		UPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { {UL-NGU-UP-TNLModifyItem-ExtIEs} } OPTIONAL,

...

}

UL-NGU-UP-TNLModifyItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-RedundantUL-NGU-UP-TNLInformation	CRITICALITY ignore	EXTENSION UPTransportLayerInformation		PRESENCE optional	 	}|

{ ID id-RedundantDL-NGU-UP-TNLInformation	CRITICALITY ignore	EXTENSION UPTransportLayerInformation		PRESENCE optional	 	},

...

}

UnavailableGUAMIList ::= SEQUENCE (SIZE(1..maxnoofServedGUAMIs)) OF UnavailableGUAMIItem

UnavailableGUAMIItem ::= SEQUENCE {

gUAMI								GUAMI,

timerApproachForGUAMIRemoval		TimerApproachForGUAMIRemoval					OPTIONAL,

backupAMFName						AMFName											OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UnavailableGUAMIItem-ExtIEs} }	OPTIONAL,

...

}

UnavailableGUAMIItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

ULForwarding ::= ENUMERATED {

ul-forwarding-proposed,

...

}

UpdateFeedback ::= BIT STRING (SIZE(8, ...))

UPTransportLayerInformation ::= CHOICE {

gTPTunnel				GTPTunnel,

choice-Extensions		ProtocolIE-SingleContainer { {UPTransportLayerInformation-ExtIEs} }

}

UPTransportLayerInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

UPTransportLayerInformationList ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF UPTransportLayerInformationItem

UPTransportLayerInformationItem ::= SEQUENCE {

nGU-UP-TNLInformation		UPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { {UPTransportLayerInformationItem-ExtIEs} } OPTIONAL,

...

}

UPTransportLayerInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-CommonNetworkInstance				CRITICALITY ignore	EXTENSION  CommonNetworkInstance						PRESENCE optional		},

...

}

UPTransportLayerInformationPairList ::= SEQUENCE (SIZE(1..maxnoofMultiConnectivityMinusOne)) OF UPTransportLayerInformationPairItem

UPTransportLayerInformationPairItem ::= SEQUENCE {

uL-NGU-UP-TNLInformation		UPTransportLayerInformation,

dL-NGU-UP-TNLInformation		UPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { {UPTransportLayerInformationPairItem-ExtIEs} } OPTIONAL,

...

}

UPTransportLayerInformationPairItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

URI-address ::= VisibleString

UserLocationInformation ::= CHOICE {

userLocationInformationEUTRA	UserLocationInformationEUTRA,

userLocationInformationNR		UserLocationInformationNR,

userLocationInformationN3IWF-with-PortNumber	UserLocationInformationN3IWF-with-PortNumber,

choice-Extensions		ProtocolIE-SingleContainer { {UserLocationInformation-ExtIEs} }

}

UserLocationInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-UserLocationInformationTNGF		CRITICALITY ignore	TYPE UserLocationInformationTNGF		PRESENCE mandatory	}|

{ ID id-UserLocationInformationTWIF		CRITICALITY ignore	TYPE UserLocationInformationTWIF		PRESENCE mandatory	}|

{ ID id-UserLocationInformationW-AGF	CRITICALITY ignore	TYPE UserLocationInformationW-AGF		PRESENCE mandatory	}|

{ ID id-UserLocationInformationN3IWF-without-PortNumber	CRITICALITY ignore	TYPE UserLocationInformationN3IWF-without-PortNumber		PRESENCE mandatory	},

...

}

UserLocationInformationEUTRA ::= SEQUENCE {

eUTRA-CGI			EUTRA-CGI,

tAI					TAI,

timeStamp			TimeStamp																OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationEUTRA-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationEUTRA-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PSCellInformation	CRITICALITY ignore	EXTENSION NGRAN-CGI				PRESENCE optional},

...

}

UserLocationInformationN3IWF-with-PortNumber ::= SEQUENCE {

iPAddress			TransportLayerAddress,

portNumber			PortNumber,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationN3IWF-with-PortNumber-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationN3IWF-with-PortNumber-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-TAI			CRITICALITY ignore	EXTENSION TAI							PRESENCE optional	},

...

}

UserLocationInformationN3IWF-without-PortNumber ::= SEQUENCE {

iPAddress			TransportLayerAddress,

tAI					TAI								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { UserLocationInformationN3IWF-without-PortNumber-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationN3IWF-without-PortNumber-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UserLocationInformationTNGF ::= SEQUENCE {

tNAP-ID				TNAP-ID,

iPAddress			TransportLayerAddress,

portNumber			PortNumber															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationTNGF-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationTNGF-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-TAI			CRITICALITY ignore	EXTENSION TAI							PRESENCE optional	},

...

}

UserLocationInformationTWIF ::= SEQUENCE {

tWAP-ID				TWAP-ID,

iPAddress			TransportLayerAddress,

portNumber			PortNumber															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationTWIF-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationTWIF-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-TAI			CRITICALITY ignore	EXTENSION TAI							PRESENCE optional	},

...

}

UserLocationInformationW-AGF ::= CHOICE {

globalLine-ID	GlobalLine-ID,

hFCNode-ID		HFCNode-ID,

choice-Extensions		ProtocolIE-SingleContainer { { UserLocationInformationW-AGF-ExtIEs} }

}

UserLocationInformationW-AGF-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-GlobalCable-ID 	CRITICALITY 	ignore 	TYPE 	GlobalCable-ID 		PRESENCE 	mandatory }|

{ ID id-HFCNode-ID-new 	CRITICALITY 	ignore 	TYPE 	HFCNode-ID-new 		PRESENCE 	mandatory }|

{ ID id-GlobalCable-ID-new 	CRITICALITY ignore 	TYPE 	GlobalCable-ID-new 	PRESENCE 	mandatory },

...

}

UserLocationInformationNR ::= SEQUENCE {

nR-CGI				NR-CGI,

tAI					TAI,

timeStamp			TimeStamp															OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {UserLocationInformationNR-ExtIEs} }	OPTIONAL,

...

}

UserLocationInformationNR-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-PSCellInformation						CRITICALITY ignore	EXTENSION NGRAN-CGI											PRESENCE optional	}|

{ ID id-NID										CRITICALITY reject	EXTENSION NID												PRESENCE optional	}|

{ ID id-NRNTNTAIInformation						CRITICALITY ignore	EXTENSION NRNTNTAIInformation 							PRESENCE optional	}|

{ ID id-MobileIAB-MTUserLocationInformation		CRITICALITY ignore	EXTENSION MobileIAB-MTUserLocationInformation 		PRESENCE optional	},

...

}

UserPlaneFailureIndication ::= SEQUENCE {

userPlaneFailureType				UserPlaneFailureType,

uL-NGU-UP-TNLInformation			UPTransportLayerInformation,

dL-NGU-UP-TNLInformation			UPTransportLayerInformation,

iE-Extensions		ProtocolExtensionContainer { { UserPlaneFailureIndication-ExtIEs} }	OPTIONAL,

...

}

UserPlaneFailureIndication-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

UserPlaneFailureIndicationReport ::= ENUMERATED {

qos-flows-and-tunnel-to-be-released,

qoS-flows-to-be-moved,

new-transport-address-allocated,

...

}

UserPlaneFailureType ::= ENUMERATED {

gtp-u-error-indication-received,

up-path-failure,

...

}

UserPlaneSecurityInformation ::= SEQUENCE {

securityResult			SecurityResult,

securityIndication		SecurityIndication,

iE-Extensions		ProtocolExtensionContainer { {UserPlaneSecurityInformation-ExtIEs} }	OPTIONAL,

...

}

UserPlaneSecurityInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- V

VolumeTimedReportList ::= SEQUENCE (SIZE(1..maxnoofTimePeriods)) OF VolumeTimedReport-Item

VolumeTimedReport-Item ::= SEQUENCE {

startTimeStamp				OCTET STRING (SIZE(4)),

endTimeStamp				OCTET STRING (SIZE(4)),

usageCountUL				INTEGER (0..18446744073709551615),

usageCountDL				INTEGER (0..18446744073709551615),

iE-Extensions		ProtocolExtensionContainer { {VolumeTimedReport-Item-ExtIEs} } OPTIONAL,

...

}

VolumeTimedReport-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- W

W-AGF-ID ::= CHOICE {

w-AGF-ID				BIT STRING (SIZE(16, ...)),

choice-Extensions		ProtocolIE-SingleContainer { {W-AGF-ID-ExtIEs} }

}

W-AGF-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

WarningAreaCoordinates ::= OCTET STRING (SIZE(1..1024))

WarningAreaList ::= CHOICE {

eUTRA-CGIListForWarning			EUTRA-CGIListForWarning,

nR-CGIListForWarning			NR-CGIListForWarning,

tAIListForWarning				TAIListForWarning,

emergencyAreaIDList				EmergencyAreaIDList,

choice-Extensions		ProtocolIE-SingleContainer { {WarningAreaList-ExtIEs} }

}

WarningAreaList-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

WarningMessageContents ::= OCTET STRING (SIZE(1..9600))

WarningSecurityInfo ::= OCTET STRING (SIZE(50))

WarningType ::= OCTET STRING (SIZE(2))

WLANMeasurementConfiguration ::= SEQUENCE {

wlanMeasConfig             	WLANMeasConfig,

wlanMeasConfigNameList		WLANMeasConfigNameList            								OPTIONAL,

wlan-rssi                  	ENUMERATED {true, ...}            								OPTIONAL,

wlan-rtt                   	ENUMERATED {true, ...}            								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { WLANMeasurementConfiguration-ExtIEs } } 	OPTIONAL,

...

}

WLANMeasurementConfiguration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

WLANMeasConfigNameList ::= SEQUENCE (SIZE(1..maxnoofWLANName)) OF WLANMeasConfigNameItem

WLANMeasConfigNameItem ::= SEQUENCE {

wLANName			WLANName,

iE-Extensions		ProtocolExtensionContainer { { WLANMeasConfigNameItem-ExtIEs } } 	OPTIONAL,

...

}

WLANMeasConfigNameItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

WLANMeasConfig::= ENUMERATED {setup,...}

WLANName ::= OCTET STRING (SIZE (1..32))

WUS-Assistance-Information  ::= SEQUENCE {

pagingProbabilityInformation             PagingProbabilityInformation,

iE-Extensions		ProtocolExtensionContainer { { WUS-Assistance-Information-ExtIEs } } OPTIONAL,

...

}

WUS-Assistance-Information-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

-- X

XnExtTLAs ::= SEQUENCE (SIZE(1..maxnoofXnExtTLAs)) OF XnExtTLA-Item

XnExtTLA-Item ::= SEQUENCE {

iPsecTLA					TransportLayerAddress							OPTIONAL,

gTP-TLAs					XnGTP-TLAs										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {XnExtTLA-Item-ExtIEs} } 	OPTIONAL,

...

}

XnExtTLA-Item-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SCTP-TLAs		CRITICALITY ignore	EXTENSION SCTP-TLAs		PRESENCE optional },

...

}

XnGTP-TLAs ::= SEQUENCE (SIZE(1..maxnoofXnGTP-TLAs)) OF TransportLayerAddress

XnTLAs ::= SEQUENCE (SIZE(1..maxnoofXnTLAs)) OF TransportLayerAddress

XnTNLConfigurationInfo ::= SEQUENCE {

xnTransportLayerAddresses			XnTLAs,

xnExtendedTransportLayerAddresses	XnExtTLAs										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {XnTNLConfigurationInfo-ExtIEs} }	OPTIONAL,

...

}

XnTNLConfigurationInfo-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

XrDeviceWith2Rx ::= ENUMERATED {true, ...}

-- Y

-- Z

END

-- ASN1STOP

### 9.4.6	Common Definitions

-- ASN1START

-- **************************************************************

--

-- Common definitions

--

-- **************************************************************

NGAP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-CommonDataTypes (3) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Criticality		::= ENUMERATED { reject, ignore, notify }

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local				INTEGER (0..65535),

global				OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolExtensionID	::= INTEGER (0..65535)

ProtocolIE-ID		::= INTEGER (0..65535)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome }

END

-- ASN1STOP

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-AMFConfigurationUpdate					ProcedureCode ::= 0

id-AMFStatusIndication						ProcedureCode ::= 1

id-CellTrafficTrace							ProcedureCode ::= 2

id-DeactivateTrace							ProcedureCode ::= 3

id-DownlinkNASTransport						ProcedureCode ::= 4

id-DownlinkNonUEAssociatedNRPPaTransport	ProcedureCode ::= 5

id-DownlinkRANConfigurationTransfer			ProcedureCode ::= 6

id-DownlinkRANStatusTransfer				ProcedureCode ::= 7

id-DownlinkUEAssociatedNRPPaTransport		ProcedureCode ::= 8

id-ErrorIndication							ProcedureCode ::= 9

id-HandoverCancel							ProcedureCode ::= 10

id-HandoverNotification						ProcedureCode ::= 11

id-HandoverPreparation						ProcedureCode ::= 12

id-HandoverResourceAllocation				ProcedureCode ::= 13

id-InitialContextSetup						ProcedureCode ::= 14

id-InitialUEMessage							ProcedureCode ::= 15

id-LocationReportingControl					ProcedureCode ::= 16

id-LocationReportingFailureIndication		ProcedureCode ::= 17

id-LocationReport							ProcedureCode ::= 18

id-NASNonDeliveryIndication					ProcedureCode ::= 19

id-NGReset									ProcedureCode ::= 20

id-NGSetup									ProcedureCode ::= 21

id-OverloadStart							ProcedureCode ::= 22

id-OverloadStop								ProcedureCode ::= 23

id-Paging									ProcedureCode ::= 24

id-PathSwitchRequest						ProcedureCode ::= 25

id-PDUSessionResourceModify					ProcedureCode ::= 26

id-PDUSessionResourceModifyIndication		ProcedureCode ::= 27

id-PDUSessionResourceRelease				ProcedureCode ::= 28

id-PDUSessionResourceSetup					ProcedureCode ::= 29

id-PDUSessionResourceNotify					ProcedureCode ::= 30

id-PrivateMessage							ProcedureCode ::= 31

id-PWSCancel								ProcedureCode ::= 32

id-PWSFailureIndication						ProcedureCode ::= 33

id-PWSRestartIndication						ProcedureCode ::= 34

id-RANConfigurationUpdate					ProcedureCode ::= 35

id-RerouteNASRequest						ProcedureCode ::= 36

id-RRCInactiveTransitionReport				ProcedureCode ::= 37

id-TraceFailureIndication					ProcedureCode ::= 38

id-TraceStart								ProcedureCode ::= 39

id-UEContextModification					ProcedureCode ::= 40

id-UEContextRelease							ProcedureCode ::= 41

id-UEContextReleaseRequest					ProcedureCode ::= 42

id-UERadioCapabilityCheck					ProcedureCode ::= 43

id-UERadioCapabilityInfoIndication			ProcedureCode ::= 44

id-UETNLABindingRelease						ProcedureCode ::= 45

id-UplinkNASTransport						ProcedureCode ::= 46

id-UplinkNonUEAssociatedNRPPaTransport		ProcedureCode ::= 47

id-UplinkRANConfigurationTransfer			ProcedureCode ::= 48

id-UplinkRANStatusTransfer					ProcedureCode ::= 49

id-UplinkUEAssociatedNRPPaTransport			ProcedureCode ::= 50

id-WriteReplaceWarning						ProcedureCode ::= 51

id-SecondaryRATDataUsageReport				ProcedureCode ::= 52

id-UplinkRIMInformationTransfer				ProcedureCode ::= 53

id-DownlinkRIMInformationTransfer			ProcedureCode ::= 54

id-RetrieveUEInformation					ProcedureCode ::= 55

id-UEInformationTransfer					ProcedureCode ::= 56

id-RANCPRelocationIndication				ProcedureCode ::= 57

id-UEContextResume							ProcedureCode ::= 58

id-UEContextSuspend							ProcedureCode ::= 59

id-UERadioCapabilityIDMapping				ProcedureCode ::= 60

id-HandoverSuccess							ProcedureCode ::= 61

id-UplinkRANEarlyStatusTransfer				ProcedureCode ::= 62

id-DownlinkRANEarlyStatusTransfer			ProcedureCode ::= 63

id-AMFCPRelocationIndication				ProcedureCode ::= 64

id-ConnectionEstablishmentIndication		ProcedureCode ::= 65

id-BroadcastSessionModification				ProcedureCode ::= 66

id-BroadcastSessionRelease					ProcedureCode ::= 67

id-BroadcastSessionSetup					ProcedureCode ::= 68

id-DistributionSetup 						ProcedureCode ::= 69

id-DistributionRelease 						ProcedureCode ::= 70

id-MulticastSessionActivation 				ProcedureCode ::= 71

id-MulticastSessionDeactivation 			ProcedureCode ::= 72

id-MulticastSessionUpdate 					ProcedureCode ::= 73

id-MulticastGroupPaging						ProcedureCode ::= 74

id-BroadcastSessionReleaseRequired			ProcedureCode ::= 75

id-TimingSynchronisationStatus				ProcedureCode ::= 76

id-TimingSynchronisationStatusReport		ProcedureCode ::= 77

id-MTCommunicationHandling					ProcedureCode ::= 78

id-RANPagingRequest							ProcedureCode ::= 79

id-BroadcastSessionTransport				ProcedureCode ::= 80

id-InventoryRequest							ProcedureCode ::= 193 --to be allocated

id-InventoryReport							ProcedureCode ::= 196 --to be allocated

id-CommandRequest							ProcedureCode ::= 197 --to be allocated

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofAllowedAreas							INTEGER ::= 16

maxnoofAllowedCAGsperPLMN					INTEGER ::= 256

maxnoofAllowedS-NSSAIs						INTEGER ::= 8

maxnoofBluetoothName						INTEGER ::= 4

maxnoofBPLMNs								INTEGER ::= 12

maxnoofCAGSperCell							INTEGER ::= 64

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofCellIDforWarning						INTEGER ::= 65535

maxnoofCellinAoI							INTEGER ::= 256

maxnoofCellinEAI							INTEGER ::= 65535

maxnoofCellinTAI							INTEGER ::= 65535

maxnoofCellsforMBS							INTEGER ::= 8192

maxnoofCellsingNB							INTEGER ::= 16384

maxnoofCellsinngeNB							INTEGER ::= 256

maxnoofCellsinNGRANNode						INTEGER ::= 16384

maxnoofCellsinUEHistoryInfo					INTEGER ::= 16

maxnoofCellsUEMovingTrajectory				INTEGER ::= 16

maxnoofDRBs									INTEGER ::= 32

maxnoofEmergencyAreaID						INTEGER ::= 65535

maxnoofEAIforRestart						INTEGER ::= 256

maxnoofEPLMNs								INTEGER ::= 15

maxnoofEPLMNsPlusOne						INTEGER ::= 16

maxnoofE-RABs								INTEGER ::= 256

maxnoofErrors								INTEGER ::= 256

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofForbTACs								INTEGER ::= 4096

maxnoofFreqforMDT							INTEGER ::= 8

maxnoofMBSAreaSessionIDs					INTEGER ::= 256

maxnoofMBSFSAs								INTEGER ::= 64

maxnoofMBSQoSFlows							INTEGER ::= 64

maxnoofMBSSessions							INTEGER ::= 32

maxnoofMBSSessionsofUE						INTEGER ::= 256

maxnoofMBSServiceAreaInformation			INTEGER ::= 256

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofMRBs									INTEGER ::= 32

maxnoofMultiConnectivity					INTEGER ::= 4

maxnoofMultiConnectivityMinusOne			INTEGER ::= 3

maxnoofNeighPCIforMDT						INTEGER ::= 32

maxnoofNGAPIESupportInfo					INTEGER ::= 32

maxnoofNGConnectionsToReset					INTEGER ::= 65536

maxnoofNRCellBands							INTEGER ::= 32

maxnoofNSAGs								INTEGER ::= 256

maxnoofPagingAreas							INTEGER ::= 64

maxnoofPC5QoSFlows 							INTEGER ::= 2048

maxnoofPDUSessions							INTEGER ::= 256

maxnoofPLMNs								INTEGER ::= 12

maxnoofPSCellsPerPrimaryCellinUEHistoryInfo	INTEGER ::= 8

maxnoofQosFlows								INTEGER ::= 64

maxnoofQosParaSets							INTEGER ::= 8

maxnoofRANNodeinAoI							INTEGER ::= 64

maxnoofRecommendedCells						INTEGER ::= 16

maxnoofRecommendedRANNodes					INTEGER ::= 16

maxnoofAoI									INTEGER ::= 64

maxnoofAoIMinusOne							INTEGER ::= 63

maxnoofReportedCells						INTEGER ::= 256

maxnoofSensorName							INTEGER ::= 3

maxnoofServedGUAMIs							INTEGER ::= 256

maxnoofSliceItems							INTEGER ::= 1024

maxnoofSuccessfulHOReports					INTEGER ::= 64

maxnoofTACs									INTEGER ::= 256

maxnoofTACsinNTN							INTEGER ::= 12

maxnoofTAforMDT								INTEGER ::= 8

maxnoofTAIforInactive						INTEGER ::= 16

maxnoofTAIforMBS							INTEGER ::= 1024

maxnoofTAIforPaging							INTEGER ::= 16

maxnoofTAIforRestart						INTEGER ::= 2048

maxnoofTAIforWarning						INTEGER ::= 65535

maxnoofTAIinAoI								INTEGER ::= 16

maxnoofTimePeriods							INTEGER ::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofUEsforPaging							INTEGER ::=	4096

maxnoofUETypes								INTEGER ::=	8

maxnoofWLANName								INTEGER ::= 4

maxnoofXnExtTLAs							INTEGER ::= 16

maxnoofXnGTP-TLAs							INTEGER ::= 16

maxnoofXnTLAs								INTEGER ::= 2

maxnoofCandidateCells						INTEGER ::= 32

maxnoofTargetS-NSSAIs						INTEGER ::= 8

maxNRARFCN									INTEGER ::= 3279165

maxnoofCellIDforQMC							INTEGER ::= 32

maxnoofPLMNforQMC							INTEGER ::= 16

maxnoofUEAppLayerMeas						INTEGER ::= 16

maxnoofSNSSAIforQMC							INTEGER ::= 16

maxnoofTAforQMC								INTEGER ::= 8

maxnoofThresholdsForExcessPacketDelay		INTEGER ::= 255

maxnoofESNPNs								INTEGER ::= 15

maxnoofCandidateRelayUEs					INTEGER ::= 32

maxnoofSuccessfulPSCellChangeReports       	INTEGER ::= 64

maxnoofCellsTSS								INTEGER ::= 16384

maxnoofPeriodicities						INTEGER ::= 8

maxnoofCAGforMDT							INTEGER ::= 256

maxnoofMDTSNPNs								INTEGER ::= 16

maxnoofPartiallyAllowedS-NSSAIs				INTEGER ::= 8

maxnoofRSPPQoSFlows							INTEGER ::= 2048

maxnoofReaders								INTEGER ::= 65535

maxnoofDevices								INTEGER ::= 65535

maxnoofAIoTAreas							INTEGER ::= 256

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-AllowedNSSAI											ProtocolIE-ID ::= 0

id-AMFName												ProtocolIE-ID ::= 1

id-AMFOverloadResponse									ProtocolIE-ID ::= 2

id-AMFSetID												ProtocolIE-ID ::= 3

id-AMF-TNLAssociationFailedToSetupList					ProtocolIE-ID ::= 4

id-AMF-TNLAssociationSetupList							ProtocolIE-ID ::= 5

id-AMF-TNLAssociationToAddList							ProtocolIE-ID ::= 6

id-AMF-TNLAssociationToRemoveList						ProtocolIE-ID ::= 7

id-AMF-TNLAssociationToUpdateList						ProtocolIE-ID ::= 8

id-AMFTrafficLoadReductionIndication					ProtocolIE-ID ::= 9

id-AMF-UE-NGAP-ID										ProtocolIE-ID ::= 10

id-AssistanceDataForPaging								ProtocolIE-ID ::= 11

id-BroadcastCancelledAreaList							ProtocolIE-ID ::= 12

id-BroadcastCompletedAreaList							ProtocolIE-ID ::= 13

id-CancelAllWarningMessages								ProtocolIE-ID ::= 14

id-Cause												ProtocolIE-ID ::= 15

id-CellIDListForRestart									ProtocolIE-ID ::= 16

id-ConcurrentWarningMessageInd							ProtocolIE-ID ::= 17

id-CoreNetworkAssistanceInformationForInactive			ProtocolIE-ID ::= 18

id-CriticalityDiagnostics								ProtocolIE-ID ::= 19

id-DataCodingScheme										ProtocolIE-ID ::= 20

id-DefaultPagingDRX										ProtocolIE-ID ::= 21

id-DirectForwardingPathAvailability						ProtocolIE-ID ::= 22

id-EmergencyAreaIDListForRestart						ProtocolIE-ID ::= 23

id-EmergencyFallbackIndicator							ProtocolIE-ID ::= 24

id-EUTRA-CGI											ProtocolIE-ID ::= 25

id-FiveG-S-TMSI											ProtocolIE-ID ::= 26

id-GlobalRANNodeID										ProtocolIE-ID ::= 27

id-GUAMI												ProtocolIE-ID ::= 28

id-HandoverType											ProtocolIE-ID ::= 29

id-IMSVoiceSupportIndicator								ProtocolIE-ID ::= 30

id-IndexToRFSP											ProtocolIE-ID ::= 31

id-InfoOnRecommendedCellsAndRANNodesForPaging			ProtocolIE-ID ::= 32

id-LocationReportingRequestType							ProtocolIE-ID ::= 33

id-MaskedIMEISV											ProtocolIE-ID ::= 34

id-MessageIdentifier									ProtocolIE-ID ::= 35

id-MobilityRestrictionList								ProtocolIE-ID ::= 36

id-NASC													ProtocolIE-ID ::= 37

id-NAS-PDU												ProtocolIE-ID ::= 38

id-NASSecurityParametersFromNGRAN						ProtocolIE-ID ::= 39

id-NewAMF-UE-NGAP-ID									ProtocolIE-ID ::= 40

id-NewSecurityContextInd								ProtocolIE-ID ::= 41

id-NGAP-Message											ProtocolIE-ID ::= 42

id-NGRAN-CGI											ProtocolIE-ID ::= 43

id-NGRANTraceID											ProtocolIE-ID ::= 44

id-NR-CGI												ProtocolIE-ID ::= 45

id-NRPPa-PDU											ProtocolIE-ID ::= 46

id-NumberOfBroadcastsRequested							ProtocolIE-ID ::= 47

id-OldAMF												ProtocolIE-ID ::= 48

id-OverloadStartNSSAIList								ProtocolIE-ID ::= 49

id-PagingDRX											ProtocolIE-ID ::= 50

id-PagingOrigin											ProtocolIE-ID ::= 51

id-PagingPriority										ProtocolIE-ID ::= 52

id-PDUSessionResourceAdmittedList						ProtocolIE-ID ::= 53

id-PDUSessionResourceFailedToModifyListModRes			ProtocolIE-ID ::= 54

id-PDUSessionResourceFailedToSetupListCxtRes			ProtocolIE-ID ::= 55

id-PDUSessionResourceFailedToSetupListHOAck				ProtocolIE-ID ::= 56

id-PDUSessionResourceFailedToSetupListPSReq				ProtocolIE-ID ::= 57

id-PDUSessionResourceFailedToSetupListSURes				ProtocolIE-ID ::= 58

id-PDUSessionResourceHandoverList						ProtocolIE-ID ::= 59

id-PDUSessionResourceListCxtRelCpl						ProtocolIE-ID ::= 60

id-PDUSessionResourceListHORqd							ProtocolIE-ID ::= 61

id-PDUSessionResourceModifyListModCfm					ProtocolIE-ID ::= 62

id-PDUSessionResourceModifyListModInd					ProtocolIE-ID ::= 63

id-PDUSessionResourceModifyListModReq					ProtocolIE-ID ::= 64

id-PDUSessionResourceModifyListModRes					ProtocolIE-ID ::= 65

id-PDUSessionResourceNotifyList							ProtocolIE-ID ::= 66

id-PDUSessionResourceReleasedListNot					ProtocolIE-ID ::= 67

id-PDUSessionResourceReleasedListPSAck					ProtocolIE-ID ::= 68

id-PDUSessionResourceReleasedListPSFail					ProtocolIE-ID ::= 69

id-PDUSessionResourceReleasedListRelRes					ProtocolIE-ID ::= 70

id-PDUSessionResourceSetupListCxtReq					ProtocolIE-ID ::= 71

id-PDUSessionResourceSetupListCxtRes					ProtocolIE-ID ::= 72

id-PDUSessionResourceSetupListHOReq						ProtocolIE-ID ::= 73

id-PDUSessionResourceSetupListSUReq						ProtocolIE-ID ::= 74

id-PDUSessionResourceSetupListSURes						ProtocolIE-ID ::= 75

id-PDUSessionResourceToBeSwitchedDLList					ProtocolIE-ID ::= 76

id-PDUSessionResourceSwitchedList						ProtocolIE-ID ::= 77

id-PDUSessionResourceToReleaseListHOCmd					ProtocolIE-ID ::= 78

id-PDUSessionResourceToReleaseListRelCmd				ProtocolIE-ID ::= 79

id-PLMNSupportList										ProtocolIE-ID ::= 80

id-PWSFailedCellIDList									ProtocolIE-ID ::= 81

id-RANNodeName											ProtocolIE-ID ::= 82

id-RANPagingPriority									ProtocolIE-ID ::= 83

id-RANStatusTransfer-TransparentContainer				ProtocolIE-ID ::= 84

id-RAN-UE-NGAP-ID										ProtocolIE-ID ::= 85

id-RelativeAMFCapacity									ProtocolIE-ID ::= 86

id-RepetitionPeriod										ProtocolIE-ID ::= 87

id-ResetType											ProtocolIE-ID ::= 88

id-RoutingID											ProtocolIE-ID ::= 89

id-RRCEstablishmentCause								ProtocolIE-ID ::= 90

id-RRCInactiveTransitionReportRequest					ProtocolIE-ID ::= 91

id-RRCState												ProtocolIE-ID ::= 92

id-SecurityContext										ProtocolIE-ID ::= 93

id-SecurityKey											ProtocolIE-ID ::= 94

id-SerialNumber											ProtocolIE-ID ::= 95

id-ServedGUAMIList										ProtocolIE-ID ::= 96

id-SliceSupportList										ProtocolIE-ID ::= 97

id-SONConfigurationTransferDL							ProtocolIE-ID ::= 98

id-SONConfigurationTransferUL							ProtocolIE-ID ::= 99

id-SourceAMF-UE-NGAP-ID									ProtocolIE-ID ::= 100

id-SourceToTarget-TransparentContainer					ProtocolIE-ID ::= 101

id-SupportedTAList										ProtocolIE-ID ::= 102

id-TAIListForPaging										ProtocolIE-ID ::= 103

id-TAIListForRestart									ProtocolIE-ID ::= 104

id-TargetID												ProtocolIE-ID ::= 105

id-TargetToSource-TransparentContainer					ProtocolIE-ID ::= 106

id-TimeToWait											ProtocolIE-ID ::= 107

id-TraceActivation										ProtocolIE-ID ::= 108

id-TraceCollectionEntityIPAddress						ProtocolIE-ID ::= 109

id-UEAggregateMaximumBitRate							ProtocolIE-ID ::= 110

id-UE-associatedLogicalNG-connectionList				ProtocolIE-ID ::= 111

id-UEContextRequest										ProtocolIE-ID ::= 112

id-UE-NGAP-IDs											ProtocolIE-ID ::= 114

id-UEPagingIdentity										ProtocolIE-ID ::= 115

id-UEPresenceInAreaOfInterestList						ProtocolIE-ID ::= 116

id-UERadioCapability									ProtocolIE-ID ::= 117

id-UERadioCapabilityForPaging							ProtocolIE-ID ::= 118

id-UESecurityCapabilities								ProtocolIE-ID ::= 119

id-UnavailableGUAMIList									ProtocolIE-ID ::= 120

id-UserLocationInformation								ProtocolIE-ID ::= 121

id-WarningAreaList										ProtocolIE-ID ::= 122

id-WarningMessageContents								ProtocolIE-ID ::= 123

id-WarningSecurityInfo									ProtocolIE-ID ::= 124

id-WarningType											ProtocolIE-ID ::= 125

id-AdditionalUL-NGU-UP-TNLInformation					ProtocolIE-ID ::= 126

id-DataForwardingNotPossible							ProtocolIE-ID ::= 127

id-DL-NGU-UP-TNLInformation								ProtocolIE-ID ::= 128

id-NetworkInstance										ProtocolIE-ID ::= 129

id-PDUSessionAggregateMaximumBitRate					ProtocolIE-ID ::= 130

id-PDUSessionResourceFailedToModifyListModCfm			ProtocolIE-ID ::= 131

id-PDUSessionResourceFailedToSetupListCxtFail			ProtocolIE-ID ::= 132

id-PDUSessionResourceListCxtRelReq						ProtocolIE-ID ::= 133

id-PDUSessionType										ProtocolIE-ID ::= 134

id-QosFlowAddOrModifyRequestList						ProtocolIE-ID ::= 135

id-QosFlowSetupRequestList								ProtocolIE-ID ::= 136

id-QosFlowToReleaseList									ProtocolIE-ID ::= 137

id-SecurityIndication									ProtocolIE-ID ::= 138

id-UL-NGU-UP-TNLInformation								ProtocolIE-ID ::= 139

id-UL-NGU-UP-TNLModifyList								ProtocolIE-ID ::= 140

id-WarningAreaCoordinates								ProtocolIE-ID ::= 141

id-PDUSessionResourceSecondaryRATUsageList				ProtocolIE-ID ::= 142

id-HandoverFlag											ProtocolIE-ID ::= 143

id-SecondaryRATUsageInformation							ProtocolIE-ID ::= 144

id-PDUSessionResourceReleaseResponseTransfer			ProtocolIE-ID ::= 145

id-RedirectionVoiceFallback								ProtocolIE-ID ::= 146

id-UERetentionInformation								ProtocolIE-ID ::= 147

id-S-NSSAI												ProtocolIE-ID ::= 148

id-PSCellInformation									ProtocolIE-ID ::= 149

id-LastEUTRAN-PLMNIdentity								ProtocolIE-ID ::= 150

id-MaximumIntegrityProtectedDataRate-DL					ProtocolIE-ID ::= 151

id-AdditionalDLForwardingUPTNLInformation				ProtocolIE-ID ::= 152

id-AdditionalDLUPTNLInformationForHOList				ProtocolIE-ID ::= 153

id-AdditionalNGU-UP-TNLInformation						ProtocolIE-ID ::= 154

id-AdditionalDLQosFlowPerTNLInformation					ProtocolIE-ID ::= 155

id-SecurityResult										ProtocolIE-ID ::= 156

id-ENDC-SONConfigurationTransferDL						ProtocolIE-ID ::= 157

id-ENDC-SONConfigurationTransferUL						ProtocolIE-ID ::= 158

id-OldAssociatedQosFlowList-ULendmarkerexpected			ProtocolIE-ID ::= 159

id-CNTypeRestrictionsForEquivalent						ProtocolIE-ID ::= 160

id-CNTypeRestrictionsForServing							ProtocolIE-ID ::= 161

id-NewGUAMI												ProtocolIE-ID ::= 162

id-ULForwarding											ProtocolIE-ID ::= 163

id-ULForwardingUP-TNLInformation						ProtocolIE-ID ::= 164

id-CNAssistedRANTuning									ProtocolIE-ID ::= 165

id-CommonNetworkInstance								ProtocolIE-ID ::= 166

id-NGRAN-TNLAssociationToRemoveList						ProtocolIE-ID ::= 167

id-TNLAssociationTransportLayerAddressNGRAN				ProtocolIE-ID ::= 168

id-EndpointIPAddressAndPort								ProtocolIE-ID ::= 169

id-LocationReportingAdditionalInfo						ProtocolIE-ID ::= 170

id-SourceToTarget-AMFInformationReroute					ProtocolIE-ID ::= 171

id-AdditionalULForwardingUPTNLInformation				ProtocolIE-ID ::= 172

id-SCTP-TLAs											ProtocolIE-ID ::= 173

id-SelectedPLMNIdentity									ProtocolIE-ID ::= 174

id-RIMInformationTransfer								ProtocolIE-ID ::= 175

id-GUAMIType											ProtocolIE-ID ::= 176

id-SRVCCOperationPossible								ProtocolIE-ID ::= 177

id-TargetRNC-ID											ProtocolIE-ID ::= 178

id-RAT-Information										ProtocolIE-ID ::= 179

id-ExtendedRATRestrictionInformation					ProtocolIE-ID ::= 180

id-QosMonitoringRequest									ProtocolIE-ID ::= 181

id-SgNB-UE-X2AP-ID										ProtocolIE-ID ::= 182

id-AdditionalRedundantDL-NGU-UP-TNLInformation			ProtocolIE-ID ::= 183

id-AdditionalRedundantDLQosFlowPerTNLInformation		ProtocolIE-ID ::= 184

id-AdditionalRedundantNGU-UP-TNLInformation				ProtocolIE-ID ::= 185

id-AdditionalRedundantUL-NGU-UP-TNLInformation			ProtocolIE-ID ::= 186

id-CNPacketDelayBudgetDL								ProtocolIE-ID ::= 187

id-CNPacketDelayBudgetUL								ProtocolIE-ID ::= 188

id-ExtendedPacketDelayBudget							ProtocolIE-ID ::= 189

id-RedundantCommonNetworkInstance						ProtocolIE-ID ::= 190

id-RedundantDL-NGU-TNLInformationReused					ProtocolIE-ID ::= 191

id-RedundantDL-NGU-UP-TNLInformation					ProtocolIE-ID ::= 192

id-RedundantDLQosFlowPerTNLInformation					ProtocolIE-ID ::= 193

id-RedundantQosFlowIndicator							ProtocolIE-ID ::= 194

id-RedundantUL-NGU-UP-TNLInformation					ProtocolIE-ID ::= 195

id-TSCTrafficCharacteristics							ProtocolIE-ID ::= 196

id-RedundantPDUSessionInformation 						ProtocolIE-ID ::= 197

id-UsedRSNInformation									ProtocolIE-ID ::= 198

id-IAB-Authorized										ProtocolIE-ID ::= 199

id-IAB-Supported										ProtocolIE-ID ::= 200

id-IABNodeIndication									ProtocolIE-ID ::= 201

id-NB-IoT-PagingDRX										ProtocolIE-ID ::= 202

id-NB-IoT-Paging-eDRXInfo								ProtocolIE-ID ::= 203

id-NB-IoT-DefaultPagingDRX								ProtocolIE-ID ::= 204

id-Enhanced-CoverageRestriction							ProtocolIE-ID ::= 205

id-Extended-ConnectedTime								ProtocolIE-ID ::= 206

id-PagingAssisDataforCEcapabUE							ProtocolIE-ID ::= 207

id-WUS-Assistance-Information							ProtocolIE-ID ::= 208

id-UE-DifferentiationInfo								ProtocolIE-ID ::= 209

id-NB-IoT-UEPriority									ProtocolIE-ID ::= 210

id-UL-CP-SecurityInformation							ProtocolIE-ID ::= 211

id-DL-CP-SecurityInformation							ProtocolIE-ID ::= 212

id-TAI													ProtocolIE-ID ::= 213

id-UERadioCapabilityForPagingOfNB-IoT					ProtocolIE-ID ::= 214

id-LTEV2XServicesAuthorized								ProtocolIE-ID ::= 215

id-NRV2XServicesAuthorized								ProtocolIE-ID ::= 216

id-LTEUESidelinkAggregateMaximumBitrate					ProtocolIE-ID ::= 217

id-NRUESidelinkAggregateMaximumBitrate					ProtocolIE-ID ::= 218

id-PC5QoSParameters										ProtocolIE-ID ::= 219

id-AlternativeQoSParaSetList							ProtocolIE-ID ::= 220

id-CurrentQoSParaSetIndex								ProtocolIE-ID ::= 221

id-CEmodeBrestricted									ProtocolIE-ID ::= 222

id-EUTRA-PagingeDRXInformation							ProtocolIE-ID ::= 223

id-CEmodeBSupport-Indicator								ProtocolIE-ID ::= 224

id-LTEM-Indication										ProtocolIE-ID ::= 225

id-EndIndication										ProtocolIE-ID ::= 226

id-EDT-Session											ProtocolIE-ID ::= 227

id-UECapabilityInfoRequest								ProtocolIE-ID ::= 228

id-PDUSessionResourceFailedToResumeListRESReq			ProtocolIE-ID ::= 229

id-PDUSessionResourceFailedToResumeListRESRes			ProtocolIE-ID ::= 230

id-PDUSessionResourceSuspendListSUSReq					ProtocolIE-ID ::= 231

id-PDUSessionResourceResumeListRESReq					ProtocolIE-ID ::= 232

id-PDUSessionResourceResumeListRESRes					ProtocolIE-ID ::= 233

id-UE-UP-CIoT-Support									ProtocolIE-ID ::= 234

id-Suspend-Request-Indication							ProtocolIE-ID ::= 235

id-Suspend-Response-Indication							ProtocolIE-ID ::= 236

id-RRC-Resume-Cause										ProtocolIE-ID ::= 237

id-RGLevelWirelineAccessCharacteristics					ProtocolIE-ID ::= 238

id-W-AGFIdentityInformation								ProtocolIE-ID ::= 239

id-GlobalTNGF-ID										ProtocolIE-ID ::= 240

id-GlobalTWIF-ID										ProtocolIE-ID ::= 241

id-GlobalW-AGF-ID										ProtocolIE-ID ::= 242

id-UserLocationInformationW-AGF							ProtocolIE-ID ::= 243

id-UserLocationInformationTNGF							ProtocolIE-ID ::= 244

id-AuthenticatedIndication								ProtocolIE-ID ::= 245

id-TNGFIdentityInformation								ProtocolIE-ID ::= 246

id-TWIFIdentityInformation								ProtocolIE-ID ::= 247

id-UserLocationInformationTWIF							ProtocolIE-ID ::= 248

id-DataForwardingResponseERABList						ProtocolIE-ID ::= 249

id-IntersystemSONConfigurationTransferDL				ProtocolIE-ID ::= 250

id-IntersystemSONConfigurationTransferUL				ProtocolIE-ID ::= 251

id-SONInformationReport									ProtocolIE-ID ::= 252

id-UEHistoryInformationFromTheUE						ProtocolIE-ID ::= 253

id-ManagementBasedMDTPLMNList							ProtocolIE-ID ::= 254

id-MDTConfiguration										ProtocolIE-ID ::= 255

id-PrivacyIndicator										ProtocolIE-ID ::= 256

id-TraceCollectionEntityURI								ProtocolIE-ID ::= 257

id-NPN-Support											ProtocolIE-ID ::= 258

id-NPN-AccessInformation								ProtocolIE-ID ::= 259

id-NPN-PagingAssistanceInformation						ProtocolIE-ID ::= 260

id-NPN-MobilityInformation								ProtocolIE-ID ::= 261

id-TargettoSource-Failure-TransparentContainer			ProtocolIE-ID ::= 262

id-NID													ProtocolIE-ID ::= 263

id-UERadioCapabilityID									ProtocolIE-ID ::= 264

id-UERadioCapability-EUTRA-Format						ProtocolIE-ID ::= 265

id-DAPSRequestInfo									ProtocolIE-ID ::= 266

id-DAPSResponseInfoList								ProtocolIE-ID ::= 267

id-EarlyStatusTransfer-TransparentContainer 			ProtocolIE-ID ::= 268

id-NotifySourceNGRANNode								ProtocolIE-ID ::= 269

id-ExtendedSliceSupportList								ProtocolIE-ID ::= 270

id-ExtendedTAISliceSupportList							ProtocolIE-ID ::= 271

id-ConfiguredTACIndication								ProtocolIE-ID ::= 272

id-Extended-RANNodeName									ProtocolIE-ID ::= 273

id-Extended-AMFName										ProtocolIE-ID ::= 274

id-GlobalCable-ID										ProtocolIE-ID ::= 275

id-QosMonitoringReportingFrequency						ProtocolIE-ID ::= 276

id-QosFlowParametersList								ProtocolIE-ID ::= 277

id-QosFlowFeedbackList									ProtocolIE-ID ::= 278

id-BurstArrivalTimeDownlink								ProtocolIE-ID ::= 279

id-ExtendedUEIdentityIndexValue							ProtocolIE-ID ::= 280

id-PduSessionExpectedUEActivityBehaviour				ProtocolIE-ID ::= 281

id-MicoAllPLMN											ProtocolIE-ID ::= 282

id-QosFlowFailedToSetupList								ProtocolIE-ID ::= 283

id-SourceTNLAddrInfo									ProtocolIE-ID ::= 284

id-ExtendedReportIntervalMDT							ProtocolIE-ID ::= 285

id-SourceNodeID											ProtocolIE-ID ::= 286

id-NRNTNTAIInformation									ProtocolIE-ID ::= 287

id-UEContextReferenceAtSource							ProtocolIE-ID ::= 288

id-LastVisitedPSCellList								ProtocolIE-ID ::= 289

id-IntersystemSONInformationRequest						ProtocolIE-ID ::= 290

id-IntersystemSONInformationReply						ProtocolIE-ID ::= 291

id-EnergySavingIndication								ProtocolIE-ID ::= 292

id-IntersystemResourceStatusUpdate						ProtocolIE-ID ::= 293

id-SuccessfulHandoverReportList							ProtocolIE-ID ::= 294

id-MBS-AreaSessionID 									ProtocolIE-ID ::= 295

id-MBS-QoSFlowsToBeSetupList							ProtocolIE-ID ::= 296

id-MBS-QoSFlowsToBeSetupModList							ProtocolIE-ID ::= 297

id-MBS-ServiceArea 										ProtocolIE-ID ::= 298

id-MBS-SessionID 										ProtocolIE-ID ::= 299

id-MBS-DistributionReleaseRequestTransfer				ProtocolIE-ID ::= 300

id-MBS-DistributionSetupRequestTransfer 				ProtocolIE-ID ::= 301

id-MBS-DistributionSetupResponseTransfer				ProtocolIE-ID ::= 302

id-MBS-DistributionSetupUnsuccessfulTransfer			ProtocolIE-ID ::= 303

id-MulticastSessionActivationRequestTransfer			ProtocolIE-ID ::= 304

id-MulticastSessionDeactivationRequestTransfer			ProtocolIE-ID ::= 305

id-MulticastSessionUpdateRequestTransfer				ProtocolIE-ID ::= 306

id-MulticastGroupPagingAreaList							ProtocolIE-ID ::= 307

id-MBS-SupportIndicator									ProtocolIE-ID ::= 309

id-MBSSessionFailedtoSetupList							ProtocolIE-ID ::= 310

id-MBSSessionFailedtoSetuporModifyList					ProtocolIE-ID ::= 311

id-MBSSessionSetupResponseList							ProtocolIE-ID ::= 312

id-MBSSessionSetuporModifyResponseList					ProtocolIE-ID ::= 313

id-MBSSessionSetupFailureTransfer						ProtocolIE-ID ::= 314

id-MBSSessionSetupRequestTransfer						ProtocolIE-ID ::= 315

id-MBSSessionSetupResponseTransfer						ProtocolIE-ID ::= 316

id-MBSSessionToReleaseList								ProtocolIE-ID ::= 317

id-MBSSessionSetupRequestList 							ProtocolIE-ID ::= 318

id-MBSSessionSetuporModifyRequestList 					ProtocolIE-ID ::= 319

id-MBS-ActiveSessionInformation-SourcetoTargetList		ProtocolIE-ID ::= 323

id-MBS-ActiveSessionInformation-TargettoSourceList		ProtocolIE-ID ::= 324

id-OnboardingSupport									ProtocolIE-ID ::= 325

id-TimeSyncAssistanceInfo								ProtocolIE-ID ::= 326

id-SurvivalTime											ProtocolIE-ID ::= 327

id-QMCConfigInfo										ProtocolIE-ID ::= 328

id-QMCDeactivation										ProtocolIE-ID ::= 329

id-PDUSessionPairID										ProtocolIE-ID ::= 331

id-NR-PagingeDRXInformation								ProtocolIE-ID ::= 332

id-RedCapIndication										ProtocolIE-ID ::= 333

id-TargetNSSAIInformation								ProtocolIE-ID ::= 334

id-UESliceMaximumBitRateList							ProtocolIE-ID ::= 335

id-M4ReportAmount										ProtocolIE-ID ::= 336

id-M5ReportAmount										ProtocolIE-ID ::= 337

id-M6ReportAmount										ProtocolIE-ID ::= 338

id-M7ReportAmount										ProtocolIE-ID ::= 339

id-IncludeBeamMeasurementsIndication					ProtocolIE-ID ::= 340

id-ExcessPacketDelayThresholdConfiguration				ProtocolIE-ID ::= 341

id-PagingCause											ProtocolIE-ID ::= 342

id-PagingCauseIndicationForVoiceService					ProtocolIE-ID ::= 343

id-PEIPSassistanceInformation							ProtocolIE-ID ::= 344

id-FiveG-ProSeAuthorized								ProtocolIE-ID ::= 345

id-FiveG-ProSeUEPC5AggregateMaximumBitRate				ProtocolIE-ID ::= 346

id-FiveG-ProSePC5QoSParameters							ProtocolIE-ID ::= 347

id-MBSSessionModificationFailureTransfer				ProtocolIE-ID ::= 348

id-MBSSessionModificationRequestTransfer				ProtocolIE-ID ::= 349

id-MBSSessionModificationResponseTransfer				ProtocolIE-ID ::= 350

id-MBS-QoSFlowToReleaseList								ProtocolIE-ID ::= 351

id-MBS-SessionTNLInfo5GC								ProtocolIE-ID ::= 352

id-TAINSAGSupportList									ProtocolIE-ID ::= 353

id-SourceNodeTNLAddrInfo								ProtocolIE-ID ::= 354

id-NGAPIESupportInformationRequestList					ProtocolIE-ID ::= 355

id-NGAPIESupportInformationResponseList					ProtocolIE-ID ::= 356

id-MBS-SessionFSAIDList									ProtocolIE-ID ::= 357

id-MBSSessionReleaseResponseTransfer					ProtocolIE-ID ::= 358

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-HFCNode-ID-new										ProtocolIE-ID ::= 362

id-GlobalCable-ID-new									ProtocolIE-ID ::= 363

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-HashedUEIdentityIndexValue							ProtocolIE-ID ::= 365

id-ExtendedMobilityInformation							ProtocolIE-ID ::= 366

id-NetworkControlledRepeaterAuthorized					ProtocolIE-ID ::= 367

id-AdditionalCancelledlocationReportingReferenceIDList	ProtocolIE-ID ::= 368

id-Selected-Target-SNPN-Identity						ProtocolIE-ID ::= 369

id-EquivalentSNPNsList                                  ProtocolIE-ID ::= 370

id-SelectedNID											ProtocolIE-ID ::= 371

id-SupportedUETypeList									ProtocolIE-ID ::= 372

id-AerialUEsubscriptionInformation						ProtocolIE-ID ::= 373

id-NR-A2X-ServicesAuthorized							ProtocolIE-ID ::= 374

id-LTE-A2X-ServicesAuthorized							ProtocolIE-ID ::= 375

id-NR-A2X-UE-PC5-AggregateMaximumBitRate				ProtocolIE-ID ::= 376

id-LTE-A2X-UE-PC5-AggregateMaximumBitRate				ProtocolIE-ID ::= 377

id-A2X-PC5-QoS-Parameters								ProtocolIE-ID ::= 378

id-FiveGProSeLayer2Multipath							ProtocolIE-ID ::= 379

id-FiveGProSeLayer2UEtoUERelay							ProtocolIE-ID ::= 380

id-FiveGProSeLayer2UEtoUERemote							ProtocolIE-ID ::= 381

id-CandidateRelayUEInformationList						ProtocolIE-ID ::= 382

id-SuccessfulPSCellChangeReportList						ProtocolIE-ID ::= 383

id-IntersystemMobilityFailureforVoiceFallback			ProtocolIE-ID ::= 384

id-TargetCellCRNTI                                      ProtocolIE-ID ::= 385

id-TimeSinceFailure                                     ProtocolIE-ID ::= 386

id-RANTimingSynchronisationStatusInfo					ProtocolIE-ID ::= 387

id-RAN-TSSRequestType									ProtocolIE-ID ::= 388

id-RAN-TSSScope											ProtocolIE-ID ::= 389

id-ClockQualityReportingControlInfo						ProtocolIE-ID ::= 390

id-RANfeedbacktype										ProtocolIE-ID ::= 391

id-QoSFlowTSCList										ProtocolIE-ID ::= 392

id-TSCTrafficCharacteristicsFeedback					ProtocolIE-ID ::= 393

id-DownlinkTLContainer									ProtocolIE-ID ::= 394

id-UplinkTLContainer									ProtocolIE-ID ::= 395

id-ANPacketDelayBudgetUL								ProtocolIE-ID ::= 396

id-QosFlowAdditionalInfoList							ProtocolIE-ID ::= 397

id-AssistanceInformationQoE-Meas						ProtocolIE-ID ::= 398

id-MBSCommServiceType									ProtocolIE-ID ::= 399

id-MobileIAB-Authorized									ProtocolIE-ID ::= 400

id-MobileIAB-MTUserLocationInformation					ProtocolIE-ID ::= 401

id-MobileIABNodeIndication							    ProtocolIE-ID ::= 402

id-NoPDUSessionIndication 								ProtocolIE-ID ::= 403

id-MobileIAB-Supported							        ProtocolIE-ID ::= 404

id-CN-MT-CommunicationHandling							ProtocolIE-ID ::= 405

id-FiveGCAction											ProtocolIE-ID ::= 406

id-PagingPolicyDifferentiation							ProtocolIE-ID ::= 407

id-DL-Signalling										ProtocolIE-ID ::= 408

id-PNI-NPN-AreaScopeofMDT								ProtocolIE-ID ::= 409

id-PNI-NPNBasedMDT										ProtocolIE-ID ::= 410

id-SNPN-CellBasedMDT									ProtocolIE-ID ::= 411

id-SNPN-TAIBasedMDT										ProtocolIE-ID ::= 412

id-SNPN-BasedMDT										ProtocolIE-ID ::= 413

id-Partially-Allowed-NSSAI								ProtocolIE-ID ::= 414

id-AssociatedSessionID									ProtocolIE-ID ::= 415

id-MBS-AssistanceInformation							ProtocolIE-ID ::= 416

id-BroadcastTransportFailureTransfer					ProtocolIE-ID ::= 417

id-BroadcastTransportRequestTransfer					ProtocolIE-ID ::= 418

id-BroadcastTransportResponseTransfer					ProtocolIE-ID ::= 419

id-TimeBasedHandoverInformation							ProtocolIE-ID ::= 420

id-DLDiscarding											ProtocolIE-ID ::= 421

id-PDUsetQoSParameters									ProtocolIE-ID ::= 422

id-PDUSetbasedHandlingIndicator							ProtocolIE-ID ::= 423

id-N6JitterInformation									ProtocolIE-ID ::= 424

id-ECNMarkingorCongestionInformationReportingRequest	ProtocolIE-ID ::= 425

id-ECNMarkingorCongestionInformationReportingStatus		ProtocolIE-ID ::= 426

id-ERedCapIndication									ProtocolIE-ID ::= 427

id-XrDeviceWith2Rx										ProtocolIE-ID ::= 428

id-UserPlaneErrorIndicator								ProtocolIE-ID ::= 429

id-SLPositioningRangingServiceInfo						ProtocolIE-ID ::= 430

id-PDUSessionListMTCommHReq								ProtocolIE-ID ::= 431

id-MaximumDataBurstVolume 								ProtocolIE-ID ::= 432

id-MN-only-MDT-collection								ProtocolIE-ID ::= 433

id-MBS-NGUFailureIndication								ProtocolIE-ID ::= 434

id-UserPlaneFailureIndication							ProtocolIE-ID ::= 435

id-UserPlaneFailureIndicationReport						ProtocolIE-ID ::= 436

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-InventoryRequestTransfer							ProtocolIE-ID ::= 993 --to be allocated

id-InventoryResponseTransfer						ProtocolIE-ID ::= 994 --to be allocated

id-InventoryFailureTransfer							ProtocolIE-ID ::= 995 --to be allocated

id-InventoryReportTransfer							ProtocolIE-ID ::= 996 --to be allocated

id-CommandRequestTransfer							ProtocolIE-ID ::= 997 --to be allocated

id-CommandResponseTransfer							ProtocolIE-ID ::= 998 --to be allocated

id-CommandFailureTransfer							ProtocolIE-ID ::= 999 --to be allocated

id-AIoT-CorrelationIdentifier							ProtocolIE-ID ::= 880 --to be allocated

id-AIOTFIdentifier									ProtocolIE-ID ::= 881 --to be allocated

id-AIoT-DeviceIdentificationRequested					ProtocolIE-ID ::= 882 --to be allocated

id-AIoT-RequestedServiceAreaInformation					ProtocolIE-ID ::= 883 --to be allocated

id-AIoT-InventoryAssistanceInformation					ProtocolIE-ID ::= 884 --to be allocated

id-AIoT-FollowonCommandIndication						ProtocolIE-ID ::= 885 --to be allocated

id-RAN-AIOT-Device-NGAP-ID								ProtocolIE-ID ::= 886 --to be allocated

id-AIoT-CommandAssistanceInformation					ProtocolIE-ID ::= 887 --to be allocated

id-AIoT-NASPDU										ProtocolIE-ID ::= 888 --to be allocated

END

-- ASN1STOP

### 9.4.8	Container Definitions

-- ASN1START

-- **************************************************************

--

-- Container definitions

--

-- **************************************************************

NGAP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

Presence,

PrivateIE-ID,

ProtocolExtensionID,

ProtocolIE-ID

FROM NGAP-CommonDataTypes

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs

FROM NGAP-Constants;

-- **************************************************************

--

#### -- Class Definition for Protocol IEs

--

-- **************************************************************

NGAP-PROTOCOL-IES ::= CLASS {

&id				ProtocolIE-ID					UNIQUE,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol IEs

--

-- **************************************************************

NGAP-PROTOCOL-IES-PAIR ::= CLASS {

&id					ProtocolIE-ID				UNIQUE,

&firstCriticality	Criticality,

&FirstValue,

&secondCriticality	Criticality,

&SecondValue,

&presence			Presence

}

WITH SYNTAX {

ID						&id

FIRST CRITICALITY		&firstCriticality

FIRST TYPE				&FirstValue

SECOND CRITICALITY		&secondCriticality

SECOND TYPE				&SecondValue

PRESENCE				&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol Extensions

--

-- **************************************************************

NGAP-PROTOCOL-EXTENSION ::= CLASS {

&id				ProtocolExtensionID			UNIQUE,

&criticality	Criticality,

&Extension,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

EXTENSION		&Extension

PRESENCE		&presence

}

-- **************************************************************

--

#### -- Class Definition for Private IEs

--

-- **************************************************************

NGAP-PRIVATE-IES ::= CLASS {

&id				PrivateIE-ID,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

#### -- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container {NGAP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-SingleContainer {NGAP-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field {NGAP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				NGAP-PROTOCOL-IES.&id				({IEsSetParam}),

criticality		NGAP-PROTOCOL-IES.&criticality		({IEsSetParam}{@id}),

value			NGAP-PROTOCOL-IES.&Value			({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container for Protocol IE Pairs

--

-- **************************************************************

ProtocolIE-ContainerPair {NGAP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-FieldPair {{IEsSetParam}}

ProtocolIE-FieldPair {NGAP-PROTOCOL-IES-PAIR : IEsSetParam} ::= SEQUENCE {

id					NGAP-PROTOCOL-IES-PAIR.&id					({IEsSetParam}),

firstCriticality	NGAP-PROTOCOL-IES-PAIR.&firstCriticality	({IEsSetParam}{@id}),

firstValue			NGAP-PROTOCOL-IES-PAIR.&FirstValue			({IEsSetParam}{@id}),

secondCriticality	NGAP-PROTOCOL-IES-PAIR.&secondCriticality	({IEsSetParam}{@id}),

secondValue			NGAP-PROTOCOL-IES-PAIR.&SecondValue			({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, NGAP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-SingleContainer {{IEsSetParam}}

ProtocolIE-ContainerPairList {INTEGER : lowerBound, INTEGER : upperBound, NGAP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-ContainerPair {{IEsSetParam}}

-- **************************************************************

--

#### -- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer {NGAP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField {NGAP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					NGAP-PROTOCOL-EXTENSION.&id				({ExtensionSetParam}),

criticality			NGAP-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		NGAP-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

#### -- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container {NGAP-PRIVATE-IES : IEsSetParam } ::=

SEQUENCE (SIZE (1..maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field {NGAP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id					NGAP-PRIVATE-IES.&id				({IEsSetParam}),

criticality			NGAP-PRIVATE-IES.&criticality		({IEsSetParam}{@id}),

value				NGAP-PRIVATE-IES.&Value				({IEsSetParam}{@id})

}

END

-- ASN1STOP

-----------------End of the Changes-----------------