3GPP TSG-RAN WG3#127 	R3-250058

Athens, Greece, 17-21 February 2025

Agenda item:	13.2 Support for inter-CU LTM

Source:	Nokia

Title:	Discussion on Inter-gNB-CU LTM Procedure

Document for:	Discussion and Decision

# 1	Introduction

In regard to L1/L2 Triggered Mobility (LTM), a new Work Item has been approved (RP-242356 NR mobility enhancements Phase 4), including the following related objectives and scope impacting RAN3:

• Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] o Prioritize the case when CU is acting as MN when DC is not configured o When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:  As secondary priority, support the case where CU is acting as SN and MN is unchanged  As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released o Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM  Coordination with SA3 needed with respect to security key handling o Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support • Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] o Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM o Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] o Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] o Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. • Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] o Specify UE evaluated conditions for triggering LTM o Aim to support conditional LTM including subsequent LTM o Limit specifying the conditional LTM to the scenario where the UE is in non-DC o Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint • Specify RRM requirements related to the above objectives as necessary [RAN4]

This paper discusses the basic required mobility procedures for inter-CU L1/L2 triggered mobility (LTM) and its potential impact to the disaggregated gNB architecture signalling.

# 2	Stage-2 Inter-CU LTM for non-split gNB Architecture

In Rel-18, LTM was specified with limitation that LTM mobility could be configured and carried out only to cells within the same gNB-CU (i.e. intra-gNB LTM). In Rel-19, this feature expands to also cover inter-gNB-CU (i.e. inter-gNB LTM) mobility.

For Inter-CU LTM, we foresee the following as necessary to be considered.

- Rel-18 LTM framework and LTM handover phases for intra-gNB-CU LTM are basis also for inter-gNB-CU LTM. That is, inter-CU LTM also has the following phases:

(a) Preparation phase, (b) Early synchronization phase, (c) Execution phase, and (d) Completion phase.

The following are agreements from the previous RAN3 meeting, which are basis for proposals in this paper.

Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: As secondary priority, support the case where CU is acting as SN and MN is unchanged As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first. Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation. Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time. Early data forwarding can be supported for inter-CU LTM. Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution. Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM. Introduce a new procedure on Xn to transfer the TA information. A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU. Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell. The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs. Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation. Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages. A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM. Change the name of LTM Cell Switch Notification message to Cell Switch Notification message. Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM. Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase. WA: The source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. The LTM Configuration IDs are allocated by the source CU. Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message. Confirm the name of the new procedure as “LTM Configuration Update”. Introduce a new procedure for candidate gNB-initiated LTM cancellation. Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. RAN3#126: RAN3 move forward on Legacy framework with PDCP change/switch for inter-CU LTM in this release, not considering the PDCP anchor based solution. The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message. The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message. Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch. Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation. Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB. Late data forwarding may be initiated after the source gNB decides to trigger the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation? Continue to work on the solutions comparison and details…

## 2.1	Preparation Phase

With respect to the preparation phase, the following aspects require further consideration.

Semi-persistent CSI-RS Support for LTM

RAN3 has agreed to utilize UE-associated procedures for LTM with periodic CSI-RS measurements. This is achieved in two steps. First, a procedure to retrieve the CSI-RS configuration from the candidate LTM cells, and second, another procedure for signaling the CSI-RS resource and report configurations accordingly.

Similarly, RAN3 replied to RAN1 that it is feasible to support semi-persistent CSI-RS measurements for LTM, although pending specifying the details.

Semi-persistent CSI-RS can be activated and de-activated, which could be carried out by the Source-DU, or Candidate-DU(s). In our view, the Source-DU should be in charge of triggering the activation and de-activation of semi-persistent CSI-RS. This is mainly because the Source-DU can select the appropriate SP CSI-RS(s) based on either L1 or L3 measurements. For example, if based on L1 measurements (SSB based or periodic CSI-RS based), the Source-DU can decide whether additional CSI-RS(s) need to be activated for some candidate cells to perform beam measurements with more refined beams.

Proposal 1: Source DU triggers (de-)activation of semi-persistent CSI-RS resource measurement and reporting.

The above can be done by specifying a new procedure over F1 and Xn interface to transfer the information between gNB-DUs via the gNB-CUs. Once, the (de-)activation request is accepted by the target DU, an acknowledgement message should be sent to the source DU which in turn will trigger CSI measurement and reporting on the UE side.

Proposal 2: Introduce a new UE-associated procedure over F1AP and XnAP to (de-)activate semi-persistent CSI-RS measurements.

Xn Interface between Candidate gNBs:

In case of Subsequent inter-CU LTM, there is need for the new Source gNB to be able to exchange messages with the Candidate gNBs. That is, there is need for an Xn control interface between all Candidate gNBs configured. Likewise, RAN3 has agreement that “Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM.”. This implies that a prerequisite for inter-CU LTM shall be that an Xn control interface link exists between all Candidate gNBs. That is, a Candidate-CU that becomes the Source-CU after mobility shall not trigger additional Xn-C setup procedures with Candidate-CUs if a Xn control interface had not been established beforehand.

Proposal 3: It is a pre-requisite for Inter-CU LTM candidate configuration that Candidate gNBs have an Xn control interface established between each other. Otherwise, the Inter-CU LTM preparation shall not be accepted by the Candidate gNB.

## 2.2	Early Synchronization Phase

With respect to the early synchronization phase, the following aspects require further consideration.

TA Information Transfer

RAN3 agreed at prior meetings to “introduce a new procedure on Xn to transfer the TA information”. However, left inconclusive the details regarding procedure’s Class and whether it is non UE-associated.

The early synchronization can be supported for inter-CU LTM taking Rel-18 approach for intra-CU case as baseline in a straightforward manner. Hence, a new non-UE associated procedure is required over Xn from Target gNB to Source gNB. Likewise, in case of disaggregated architecture, existing DU-CU/CU-DU TA INFORMATION TRANSFER messages can be reused.

Proposal 4: The XnAP TA INFORMATION TRANSFER procedure is a non-UE associated procedure.

Proposal 5: Existing F1AP DU-CU TA INFORMATION TRANSFER and CU-DU TA INFORMATION TRANSFER messages can be reused in case of disaggregated gNB architecture for Inter-CU LTM scenario.

## 2.3	LTM Execution Phase

With respect to the LTM execution phase, the following aspects require further consideration.

Data Forwarding and TNL Information Update:

To support data forwarding in case of inter-CU LTM, the Source gNB needs to transfer information regarding PDU Session Resource Information List of all other Candidate gNBs to the Target gNB. Otherwise, the Target gNB (which becomes Source gNB after LTM mobility) will lack the necessary TNL information to execute data forwarding for subsequent inter-CU LTM. One possible alternative to address this issue is to signal this information after cell switch has been triggered toward the UE and include this information as part of the XnAP CELL SWITCH NOTIFICATION message. Nevertheless, the signalling design can be flexible to also accommodate providing this information at an earlier step.

Proposal 6: The Source gNB includes the PDU Session Resource Information List of all Candidate gNBs in the XnAP LTM CELL SWITCH NOTIFICATION message.

Proposal 7: The Source gNB may also provide PDU Session Resource Information List of all Candidate gNBs prior to LTM cell switch operation.

TA Forwarding:

In the Rel.18 DU-CU and CU-DU CELL SWITCH NOTIFICATION messages, the TA information list IE may be included to transfer obtained TA values to the target (i.e., new serving) cell. Similarly, the TA validity was left up to implementation in case of intra-CU LTM.

If TA forwarding is supported for Rel-19 Inter-CU LTM, we believe the TA validity cannot be left up to implementation as the Xn link delay needs to be accounted for. A possible approach which can be linked to UE signal experienced, is to provide information such as the RSRP which the UE experienced when TA was acquired. This would allow the Target-gNB/DU to better understand the signal conditions associated with a TA value being received, rather than a “time” value tied to the TA acquisition itself. Based on the received RSRP value along with TA value(s) and the new RSRP value, the new source DU can determine the validity of the TA value. Note that such scheme is already possible for SDT and should be considered for LTM.

Proposal 8: The source gNB(-DU) signals to target gNB(-DU) the RSRP measured during TA Acquisition along with the TA value within the F1AP DU-CU CELL SWITCH NOTIFICATION and XnAP CELL SWITCH NOTIFICATION. The target gNB-CU forwards the RSRP within the F1AP CU-DU CELL SWITCH NOTIFICATION message.

Figure 1: Execution Phase Considerations for Subsequent Inter-gNB-CU LTM

## 2.4	LTM Completion Phase

With respect to the LTM completion phase, the following aspects require further consideration.

Handling of Xn UE AP Identifiers:

At prior RAN3 meetings, it was agreed that “To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.”. However, some companies concerned on how a new Source-CU is aware of the other Candidate-CUs that it is to contact after the LTM cell switch to update the identifiers.

As discussed in proposals for preparation phase, if LTM operation is agreed to be carried out only between gNBs which have an existing Xn-C interface between each other, the above concern is not an issue. In other words, a Candidate-CU during preparation will be informed of all of the other LTM candidate cells. Hence, any Candidate-CU is in position to reject the LTM configuration if there is no Xn-C interface toward any of the LTM candidate cells indicated. Likewise, it is possible to introduce a new cause value to explicitly signal this situation to Source-CU in case of such rejection.

Observation 1: With pre-requisite for Inter-CU LTM candidate configuration that Candidate gNBs have an Xn control interface established between each other to accept an LTM cell configuration, a new Source-CU is able to reach all Candidate-CUs after LTM cell switch.

Proposal 9: The Xn LTM CONFIGURATION UPDATE message is used by new Source gNB to exchange Xn UE AP Identifiers and indicate to signal the change of gNB.

Proposal 10: Remove editor note on BLCR 38.300 “Editor’s Note: The new source gNB may initiate to setup the UE associated Xn singnalling connection with the candidate gNBs. Details are FFS.”

# 3	Stage 2 Inter-CU LTM for split gNB Architecture

Similar to Section 2, this section describes the stage-2 signalling flow for inter-CU LTM considering disaggregated gNB architecture.

As starting baseline, existing procedures in Rel-18 in F1AP, E1AP and XnAP should be reused and applicable also for Rel-19 inter-CU LTM.

Inclusion of source DU and source CU/gNB identity:

In Rel-18, CFRA resources to be used for random access can be shared only among UEs belonging to same DU. This allows the CFRA resource sharing among UEs without additional burden of signalling exchange for coordination among DUs. This is because, to have collision free random access only one CFRA resource (preamble) can be used at a given time.

In our view, this holds true also for inter-CU case and we should allow the possibility of CFRA resource sharing. Hence, both the source gNB-DU ID IE and source gNB ID IE are included in the HANDOVER REQUEST message.

Observation 2: CFRA resource sharing is allowed in intra-CU LTM and is beneficial for dense deployments where CFRA resources are scarce.

Proposal 11: Both source gNB ID (global ID) and source gNB-DU ID (local ID per CU) are provided to candidate target gNBs.

Proposal 12: The candidate target gNB allocates CFRA resources based on the source gNB ID and source gNB-DU ID.

The resulting stage-2 signaling flow for Inter-CU LTM preparation phase with disaggregated architecture is shown below.

Figure 2: Inter-gNB-CU LTM Preparation Phase Signaling with split architecture

# 5	Conclusions

Proposal 1: Source DU triggers (de-)activation of semi-persistent CSI-RS resource measurement and reporting.

Proposal 2: Introduce a new UE-associated procedure over F1AP and XnAP to (de-)activate semi-persistent CSI-RS measurements.

Proposal 3: It is a pre-requisite for Inter-CU LTM candidate configuration that Candidate gNBs have an Xn control interface established between each other. Otherwise, the Inter-CU LTM preparation shall not be accepted by the Candidate gNB.

Proposal 4: The XnAP TA INFORMATION TRANSFER procedure is a non-UE associated procedure.

Proposal 5: Existing F1AP DU-CU TA INFORMATION TRANSFER and CU-DU TA INFORMATION TRANSFER messages can be reused in case of disaggregated gNB architecture for Inter-CU LTM scenario.

Proposal 6: The Source gNB includes the PDU Session Resource Information List of all Candidate gNBs in the XnAP LTM CELL SWITCH NOTIFICATION message.

Proposal 7: The Source gNB may also provide PDU Session Resource Information List of all Candidate gNBs prior to LTM cell switch operation.

Proposal 8: The source gNB(-DU) signals to target gNB(-DU) the RSRP measured during TA Acquisition along with the TA value within the F1AP DU-CU CELL SWITCH NOTIFICATION and XnAP CELL SWITCH NOTIFICATION. The target gNB-CU forwards the RSRP within the F1AP CU-DU CELL SWITCH NOTIFICATION message.

Observation 1: With pre-requisite for Inter-CU LTM candidate configuration that Candidate gNBs have an Xn control interface established between each other to accept an LTM cell configuration, a new Source-CU is able to reach all Candidate-CUs after LTM cell switch.

Proposal 9: The Xn LTM CONFIGURATION UPDATE message is used by new Source gNB to exchange Xn UE AP Identifiers and indicate to signal the change of gNB.

Proposal 10: Remove editor note on BLCR 38.300 “Editor’s Note: The new source gNB may initiate to setup the UE associated Xn singnalling connection with the candidate gNBs. Details are FFS.”

Observation 2: CFRA resource sharing is allowed in intra-CU LTM and is beneficial for dense deployments where CFRA resources are scarce.

Proposal 11: Both source gNB ID (global ID) and source gNB-DU ID (local ID per CU) are provided to candidate target gNBs.

Proposal 12: The candidate target gNB allocates CFRA resources based on the source gNB ID and source gNB-DU ID.

# References

- 3GPP TS 38.401

- RP-242356 NR mobility enhancements Phase 4, Intel (moderator)

# Annex: Text Proposal for TS 38.300

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

##### 9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The source gNB may request the candidate gNB(s) to provide the CSI-RS configuration.

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell. The candidate gNB(s) may also include the CSI-RS configuration upon request.

6.	The source gNB sends an LTM CONFIGURATION UPDATE message to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI-RS resource configuration.

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI-RS report configuration.

Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. Early synchronization to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB if early TA acquisition is performed to the candidate gNB(s).

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate inter-gNB LTM.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE. The PDU session resource and the TA value(s) related information applicable for subsequent LTM may also be included.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB.

20. The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs.

21. The candidate gNB(s) responds the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB.

22. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>