3GPP TSG-RAN WG3 Meeting #127			 R3-250085

Athens, Greece, February 17th – 21st, 2025

| Agenda item: | 20.2 |
| --- | --- |
| Source: | Baicells |
| Title: | Discussion on support of AI/ML-based positioning accuracy enhancement |
| Document for: | Discussion and Decision |

Introduction

Accurate positioning is an essential and integral component of many industrial use cases and verticals. In R18, a SI Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1] investigated the necessity, feasibility, and potential enhancement of using AI/ML for positioning as one of the target use cases. Under this background, a new R19 WI [2] on AI/ML for NR Air Interface has been approved at RAN #102. This WI aims to provide normative support for general framework for AI/ML for air interface and use cases such as positioning accuracy enhancement in the preceding R18 study. In addition, some study objectives will be continued to deal with outstanding issues identified in R18 study.

The following objectives regarding AI/ML based positioning are included in the WID that was updated at RAN #105:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

Figure 1-1  Sub-use cases with different priorities

Discussion

In RAN #102, the five sub-use cases have been assigned with different priorities as shown in Figure 1-1. In our understanding, RAN3 should focus on the NW side cases with 1st priority (Case 3a and Case 3b) regarding LCM for positioning, or what have been agreed by RAN1 or RAN2.

RAN3 to focus on:

NW side cases with 1st priority (Case 3a and Case 3b)

Works have been agreed by RAN1 or RAN2

Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

In previous meeting, the following Case 3a related agreements were reached.

Case 3a agreements: - The AI/ML model (inference and training functions) is at the gNB. - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1. - RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. - Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. - WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed. FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1). FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified. Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used. FFS if LMF should be aware that the reported measurements are AI/ML generated.

Table 2-1 shows the model input and model output related information for Case 3a.

Table 2-1  model input and model output (label) related information for Case 3a

|  | Measurements for model input | CIR/PDP | By TRP |
| --- | --- | --- | --- |
|  | Ground-truth label | Intermediate measurement: -New measurement report: e.g., ToA, path phase -Existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP -Enhancement of existing measurement report: e.g., soft information/high resolution of RSTD | By network entity with known PRU location |

Data collection for model training

Previous agreement has been reached that the AI/ML model (inference and training functions) resides at the gNB, given its advantages. Firstly, compared with LMF training gNB-side model, it’s more straightforward for a gNB to train gNB-side models with assistance information from LMF or CN in Case 3a. In addition, it can greatly reduce the workload of normative phase regarding training data collection since the whole procedure can have minimal specification impact. Thirdly, NW/LMF-side training burden can also be released if a gNB or a RAN-side node can train gNB-side models by itself, which is a distributed training pattern instead of centralized training. Therefore, following the agreement, the primary focus of normative work for Case 3a data collection for model training can be the training mechanism/procedure which may involve gNB, LMF and UE/PRU. Using gNBs for positioning model training, one of the key issues during the procedure is how labels are provided to gNB by NW entity (e.g., LMF). With labels and measurements gathered at the gNB, the model training can be performed.

Regarding the signaling design, we prefer a request and response of the ground truth label. Although it seems against the NRPPa principle that LMF starts the NRPPa transaction, it’s natural that the gNB finds out it needs to collect data for training then sends the request. The detailed design of the request and response can be further discussed.

Support a request and response procedure for gNBs to obtain the ground truth label.

Whether measurements are AI/ML generated for Case 3a

LOS/NLOS indicator may be used in some AI/ML algorithms as model output. For Case 3a, we think the existing IE “LoS/NLos information” in 38.455 can be reused, and it’s unnecessary to indicate that LoS/NLoS indicator is generated by AI/ML in an explicit way. For example, when using AI positioning in Case 3a, the RTOA value indicates the time takes for a light-of-sight virtual path propagation between UE and TRP. In practice, by reusing soft LoS/NLos indicator with special values such as 0 or 1 (representing 0% or 100% respectively, which are the special values that an AI model can hardly output), the indicating of AI/ML can be achieved.

TS38.455 ### 9.2.77 LoS/NLoS Information This IE contains the LoS/NLoS information for UL measurement. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE LoS/NLoS Indicator; M. >Soft Indicator. >>LoS/NLoS Indicator Soft; M; INTEGER (0..10); Values provide the likelihood of a LOS propagation path in the range between 0 and 1 with 0.1 steps resolution. Value '0' indicates NLOS and value '1' indicates LOS.. >Hard Indicator. >>LoS/NLoS Indicator Hard; M; ENUMERATED (NLoS, LoS).

Reuse existing “LoS/NLoS information” in TS 38.455 for Case 3a. In practice, by reusing soft LoS/NLos indicator with special values such as 0 or 1 (representing 0% or 100% respectively, which are the special values that an AI model can hardly output), the indicating of AI/ML can be achieved.

Data collection for monitoring

In RAN1 #118, the following agreement regarding monitoring was reached.

Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.

The Case 3a monitoring can be either at gNB side or LMF side, and RAN3 now proceeds with Option A: NG-RAN node performs monitoring metric calculation for its own model.

If the monitoring is at gNB side, on one hand, minimum specification effort can be expected as it can be totally up to implementation. For example, using model-input-based monitoring, or other methods which do not rely on labels. On the other hand, if the gNB-side monitoring method needs label information from network entities (with known PRU position), specification impact can be expected same as model training. For example, labels or related information need to be sent from a network entity to gNB. Therefore, we think model training and monitoring can use the same procedure acquiring label-related information from NW for Case 3a.

For Case 3a model monitoring at gNB side, label-related information is sent from network entity to gNB. In such case, model training and monitoring can share similar procedure.

If LMF performs the monitoring, monitoring data or metrics can be provided to LMF via NRPPa. Then LMF performs the monitoring, detects anomalous data, and reports the anomalies to gNB. In this logic, we may discuss issues such as what’s the monitoring data and metrics, what/how to collect them, and when/what/how to report anomalies or other information.

For Case 3a model monitoring at LMF side, further discuss the data to be monitored, metrics that may result in monitoring decisions, and related procedures.

Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

Table 2-2 shows the model input and model output related information for Case 3b.

Table 2-2  model input and model output (label) related information for Case 3b

|  | Measurements for model input | -Potential new measurement: CIR/PDP -Existing measurement: e.g., RSRP/RSRPP/RSTD | By TRP |
| --- | --- | --- | --- |
|  | Ground-truth label | UE position | By LMF with known PRU location |

Data collection for model training

The existing traditional positioning framework and procedures are designed to derive UE position via UE-RAN-CN cooperation, which can be easily reused for the model inference procedure. However, for data-driven AI/ML based positioning, the training data collection procedure has become as important as inference procedure, which may need further consideration or even redesign to meet various data collection requirements. For example, when a large scale of data is required for offline model training, it may have a lower real-time requirement compared to the existing positioning procedure while the number of measurement types within one transfer may increase significantly such that data with certain relationships can be collected in certain pattern for different training purpose.

For data-driven AI/ML based positioning, the training data collection procedure may need further consideration or redesign such that it can meet various data collection requirements which is different from the existing positioning procedure.

Based on the above analysis, there may be several potential signalling enhancements to facilitate training data collection in our understanding.

1. Required/desired SRS configuration

Firstly, in existing positioning procedures, the SRS configuration is decided by the serving gNB as illustrated in step 3 of Figure 2-1, then the specific SRS configuration will be provided to LMF in step 4. However, when LMF needs to collect data for Case 3b model training, LMF is expected to directly indicate the specific SRS configuration under which the data will be collected since it is the LMF or the NW side entity which has the full knowledge/control of the data and NW-side models. By LMF indicating the required/desired SRS configuration, if successful, LMF can easily collect measurements from gNBs and train the LMF-side corresponding model subsequently. After that, when certain SRS configuration is reported from a gNB to LMF at inferencing phase, LMF can search its model repository and select the model suitable to this SRS configuration. If no suitable model found, then LMF can trigger the data collection process by indicating this SRS configuration to train specific model. As we can see, this loop ensures a more automatic and efficient operation of LMF-side model LCM and allows positioning applications to better harness the power of AI/ML.

Therefore, as illustrated in Figure 2-1, for Case 3b data collection for model training, the LMF can indicate its required/desired SRS configuration in data collection request, and the serving gNB should provide the configuration result to LMF in the response message.

Figure 2-1 Including required/desired SRS config for model training data collection

By LMF indicating the required/desired SRS configuration, a more automatic and efficient closed-loop LMF-side model LCM can be formed which allows positioning applications to better harness the power of AI/ML.

For Case 3b data collection for model training, consider LMF to indicate its required/desired SRS configuration for data collection request, and the serving gNB provides the configuration result to LMF in the response message.

2. Support of multiple measurements

Secondly, considering the number of types of the measurements in one transfer for Case 3b data collection may be large, and LMF may request multiple gNBs to transfer the data during the procedure, it would be beneficial that LMF is aware that whether all selected gNBs are able to provide all required measurements/data before the start of measurements reporting. Otherwise gNBs may provide different number of measurement types of the data that can only be aligned after further processing at NW side to form a dataset, which can cause heavy waste of the bandwidth transferring data that cannot be used, as well as extra NW-side computation resources when organizing the dataset.

For Case 3b data collection for model training, due to different gNB capabilities and other factors, gNBs may provide different number of measurement types of the data that can only be aligned after further processing at NW side to form a dataset, which can cause heavy waste of the bandwidth transferring data that cannot be used, as well as extra NW-side computation resources when organizing the dataset.

Therefore, as illustrated in Figure 2-2, we think it’s beneficial to have a measurement request indicating the desired measurements for data collection from LMF, as well as a measurement response indicating the supported measurements by gNBs.

Figure 2-2 Measurement request and response for multi-measurement support

For Case 3b data collection for model training, an exchange procedure with a measurement request indicating the desired measurements for data collection from LMF, and a measurement response indicating the supported measurements by gNBs can be beneficial to better support data collection.

Conclusions

Observation 1.	For data-driven AI/ML based positioning, the training data collection procedure may need further consideration or redesign such that it can meet various data collection requirements which is different from the existing positioning procedure.

Observation 2.	By LMF indicating the required/desired SRS configuration, a more automatic and efficient closed-loop LMF-side model LCM can be formed which allows positioning applications to better harness the power of AI/ML.

Observation 3.	For Case 3b data collection for model training, due to different gNB capabilities and other factors, gNBs may provide different number of measurement types of the data that can only be aligned after further processing at NW side to form a dataset, which can cause heavy waste of the bandwidth transferring data that cannot be used, as well as extra NW-side computation resources when organizing the dataset.

Proposal 1.	RAN3 to focus on:

	NW side cases with 1st priority (Case 3a and Case 3b)

	Works have been agreed by RAN1 or RAN2

Proposal 2.	Support a request and response procedure for gNBs to obtain the ground truth label.

Proposal 3.	Reuse existing “LoS/NLoS information” in TS 38.455 for Case 3a. In practice, by reusing soft LoS/NLos indicator with special values such as 0 or 1 (representing 0% or 100% respectively, which are the special values that an AI model can hardly output), the indicating of AI/ML can be achieved.

Proposal 4.	For Case 3a model monitoring at gNB side, label-related information is sent from network entity to gNB. In such case, model training and monitoring can share similar procedure.

Proposal 5.	For Case 3a model monitoring at LMF side, further discuss the data to be monitored, metrics that may result in monitoring decisions, and related procedures.

Proposal 6.	For Case 3b data collection for model training, consider LMF to indicate its required/desired SRS configuration for data collection request, and the serving gNB provides the configuration result to LMF in the response message.

Proposal 7.	For Case 3b data collection for model training, an exchange procedure with a measurement request indicating the desired measurements for data collection from LMF, and a measurement response indicating the supported measurements by gNBs can be beneficial to better support data collection.

- References

3GPP TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), V2.0.0, 2023.

[2] RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface RAN#102 December 2023.

- Appendix

- Agreements on inferencing

RAN2 #125bis Agreement There is no specification impact associated to gNB-side model inference, depending on further RAN1 input.

- Agreements on data collection/training

In the SI - Model Training: - For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM. - Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress. RAN2 #125bis Agreement RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management). RAN1 #116bis Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope.

- Agreements on monitoring

In the SI - Monitoring: - The UE monitors the performance of its UE-side model. - For monitoring at the gNB side, and if needed, calculated performance metrics or data required for performance metric calculation, can at least be generated by the gNB. - For monitoring at the LMF side, the gNB or UE can generate, if needed, calculated performance metrics or data required for performance metric calculation, while the termination points for these metrics is the LMF. RAN2 #125bis Agreement For POS, RAN2 assumes that NRPPa is used for the signalling between gNB and LMF for case 3a and 3b and the detailed signalling design is up to RAN3. RAN1 #118 Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved.