3GPP TSG-RAN WG3 Meeting #127	R3-250086

Athens, GR, 17th – 21st  Feb 2025

Agenda Item:		12.3

Source:		Baicells

Title:		Discussion on Slice Feature and Access Control for NR Femto

Document for:		Discussion and decision

- Supported Slice List transfer between Femto and GW

One of the most obvious difference between 5G and 4G is the support of slice in 5G system. And AMF and NW Slice Selection is designed in 38.300[1] as below:

16.3.4.2 AMF and NW Slice Selection RAN selects the AMF based on a Temp ID or NSSAI provided by the UE. Figure 16.3.4.2-1: AMF selection In case a Temp ID is not available, the NG-RAN uses the NSSAI provided by the UE at RRC connection establishment to select the appropriate AMF (the information is provided after MSG3 of the random access procedure). If such information is also not available, the NG-RAN routes the UE to one of the configured default AMF(s). The NG-RAN uses the list of supported S-NSSAI(s) previously received in the NG Setup Response message when selecting the AMF with the NSSAI. This list may be updated via the AMF Configuration Update message.

NG-RAN and AMF shall exchange each supported slice lists during NG Setup procedure, i.e. NG-RAN shall transfer its supported S-NSSAIs per TAC per PLMN to AMFs and AMF shall transfer its supported S-NSSAIs per PLMN to NG-RANs.

NG-RAN uses S-NSSAIs from AMFs and NSSAI provided by UE to choose a proper AMF for the UE.

5GC uses S-NSSAIs provided by the NG-RAN to determines the UE’s Allowed NSSAI.

Therefore the supported slices information is necessary to exchange between NG-RAN and AMF during NG Setup procedure.

However in Femto Scenario, based on the agreements achieved in RAN3 #126 meeting:

In cases of NR Femto connecting to a NR Femto GW, the NR Femto shall only connect to a single NR Femto GW at any point in time. The NR Femto GW supports NG-Flex configuration and can simultaneously connect to multiple AMFs.

We can conclude that in the case of an NR Femto connecting to an NR Femto GW, the selection of an AMF is executed by the NR Femto GW rather than the NR Femto itself. Moreover, information that assists in choosing a proper AMF, such as the supported slice list per PLMN by AMFs, is also not required by the NR Femto.

Therefore, during the NG Setup procedure between the NR Femto and GW, there is no need to include the Slice Supported List per PLMN in the NG Setup Response message from the GW to the Femto.

Proposal 1: Slice Supported List per PLMN is not necessary to be transferred from NR Femto GW to NR Femto.

Based on above analysis, we can choose one of below two methods to achieve Proposal 1’s goal.

1.1 Option 1

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

9.2.6.2	NG SETUP RESPONSE[2]

This message is sent by the AMF to transfer application layer information for an NG-C interface instance.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 |  | YES | reject |
| Served GUAMI List |  | 1 |  |  | YES | reject |
| >Served GUAMI Item |  | 1..<maxnoofServedGUAMIs> |  |  | - |  |
| >>GUAMI | M |  | 9.3.3.3 |  | - |  |
| >>Backup AMF Name | O |  | AMF Name 9.3.3.21 |  | - |  |
| >>GUAMI Type | O |  | ENUMERATED (native, mapped, …) |  | YES | ignore |
| Relative AMF Capacity | M |  | 9.3.1.32 |  | YES | ignore |
| PLMN Support List |  | 1 |  |  | YES | reject |
| >PLMN Support Item |  | 1..<maxnoofPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >>Slice Support List | C- ifnotfromFemtoGW |  | 9.3.1.17 | Supported S-NSSAIs per PLMN or per SNPN. If this message sent from NR Femto GW to NR Femto, this IE can be optional. | - |  |
| >>NPN Support | O |  | 9.3.3.44 | If NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>Extended Slice Support List | O |  | 9.3.1.191 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >>Onboarding Support | O |  | ENUMERATED (true, ...) | Indication of onboarding support. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for IAB. | YES | ignore |
| Extended AMF Name | O |  | 9.3.3.51 |  | YES | ignore |
| Mobile IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for mobile IAB. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofServedGUAMIs | Maximum no. of GUAMIs served by an AMF. Value is 256. |
| maxnoofPLMNs | Maximum no. of PLMNs per message. Value is 12. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

2.2 Option 2

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

9.3.1.17	Slice Support List[2]

This IE indicates the list of supported slices.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Support Item |  | 0..<maxnoofSliceItems> |  |  |
| >S-NSSAI | M |  | 9.3.1.24 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

Option 1 requires the illustration of a condition where the "Slice Support List" IE is optional when the message is sent from the NR Femto GW to the NR Femto. Therefore, the description related to NR Femto must be included in 38.413, which is to be avoided in 36.413.. In contrast, Option 2 only necessitates the modification of the item's number scope, where 0 signifies that the "Slice Support List" contains no items and will not involve any NR Femto description.

Proposal 2: Adopt Option 2 to 38.413.

- Minor corrections discussion

2.1 For NNSF in R3-247938[3]

In the agreed R3-247938, we have below description:

If the gNB does not support NNSF: - If the gNB has received from the radio interface the GUAMI IE, the gNB shall include it in the INITIAL UE MESSAGE message. - If the gNB has received from the radio interface the GUAMI Type IE, the gNB shall include it in the INITIAL UE MESSAGE message. - If the gNB has received from the radio interface the Requested S-NSSAI IE, the gNB shall include it in the INITIAL UE MESSAGE message.

The Requested S-NSSAI is only a part of the Requested NSSAI, which may include multiple slice details. Therefore,  just using the Requested S-NSSAI could cause confusion.

Proposal 3: Use “Requested NSSAI” instead of “Requested S-NSSAI” similar as to the Allowed NSSAI IE used in 38.413.

2.2 For Access Control in R3-247936[4]

In the agreed R3-247936, we have below description:

4.x.4 Access Control - Cells served by an NR Femto node may be deployed as part of a PNI-NPN (see clause 4.8) in order to restrict access to UEs according to the respective subscription. NOTE: The NR Femto node may use the CAG mechanism for PNI-NPN (see clause 16.7.4) as follows: - The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG. - The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell. - The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

1.In other part in the doc, we only use “NR Femto” to describe this node, so we propose unifying the name.

Proposal 4: Unify the node’s name as “NR Femto” or “NR Femto node”.

- 2.CAG-ID is only unique per PLMN, so we need to add additional “PLMN” restriction to the “UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell”. Otherwise, this may allow the UEs having the same CAG ID but supporting different PLMNs to access this cell.

Proposal 5: Add “PLMN” restriction to the “UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell”.

From TS 23.122 4.4.3.1.1[5], we have the following description:

4.4.3.1.1 Automatic Network Selection Mode Procedure The MS selects and attempts registration on other PLMN/access technology combinations, if available and, for bullets i, ii, iii, iv, v, allowable, in the following order: i) either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present); ii) each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order); iii) each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order) or stored in the ME (in priority order); iv) other PLMN/access technology combinations with received high quality signal in random order; NOTE 1: High quality signal is defined in the appropriate AS specification. v) other PLMN/access technology combinations in order of decreasing signal quality. ......(Skip Unconcerned Part) m) In i to vii, if the MS supports CAG and: 1) is provisioned with a non-empty "CAG information list", the MS shall consider a PLMN indicated by an NG-RAN cell only if: A) the cell is a CAG cell and broadcasts a CAG-ID for the PLMN such that there exists an entry with the PLMN ID of the PLMN in the "CAG information list" and the CAG-ID is authorized based on the "Allowed CAG list" of the entry; or B) the cell is not a CAG cell and: - there is no entry with the PLMN ID of the PLMN in the "CAG information list"; or - there exists an entry with the PLMN ID of the PLMN in the "CAG information list" but the "indication that the MS is only allowed to access 5GS via CAG cells" is not included in the entry; or - is provisioned with an empty "CAG information list" or is not provisioned with a "CAG information list", the MS shall consider a PLMN indicated by an NG-RAN cell only if the cell is not a CAG cell. ......(Skip Unconcerned Part)

Based on the aforementioned UE support for CAG behavior, and considering the combination of cells with CAG or the absence of access control, we can summarize as follows:

- A non-CAG cell broadcasting PLMN ID A in the PLMN list, it will allow five types UEs to access:

  - UEs not supporting CAG, with PLMN ID A; Set UE a

  - UEs supporting CAG with “CAG information list” as below: [ m) -> 1) -> B) -> the second bullet]; Set UE b

    - PLMN ID A

    - Any CAG-ID in the “Allowed CAG list”

    - Without “indication that the MS is only allowed to access 5GS via CAG cells”, that is ,without CAG-only indication.

  - UEs supporting CAG with “CAG information list” as below: [m) -> 1) -> B) -> the first bullet]; Set UE c

    - Any PLMN ID except PLMN ID A;

    - Any CAG-ID in the “Allowed CAG list”

    - Without CAG-only indication.

  - UEs supporting CAG with “CAG information list” as below: [m) -> 1) -> B) -> the first bullet]; Set UE d

    - Any PLMN ID except PLMN ID A;

    - Any CAG-ID in the “Allowed CAG list”

    - with CAG-only indication.

  - UEs supporting CAG with an empty "CAG information list" or without a "CAG information list" [m) -> 2)]; Set UE e

- A CAG-only cell broadcasting PLMN ID B with CAG ID C in the NPN list , it will allow two types UEs to access:

  - UEs supporting CAG with “CAG information list” as below: [ m) -> 1) -> A) ]; Set UE f

    - PLMN ID B

    - CAG ID C in the “Allowed CAG list”

    - With CAG-only indication

  - UEs supporting CAG with “CAG information list” as below: [ m) -> 1) -> A) ]; Set UE g

    - PLMN ID B

    - CAG ID C in the “Allowed CAG list”

    - Without CAG-only indication

- A cell shared by PLMN and NPN broadcasting a PLMN ID A in the PLMN ID list and a NPN ID with PLMN ID B and CAG ID C in the NPN identity list, it will be accessible both for five types UEs in 1 and two types UEs in 2.

By analyzing the features of the cell and the capabilities of the UE, we can have the following table:

| UE Cell | UE a | UE b | UE c | UE d | UE e | UE f | UE g |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-CAG Cell with PLMN ID A | √ | √ | √ | √ | √ |  |  |
| Cell shared by PLMN and NPN with PLMN ID A and PLMN ID B+ CAG ID C [CAG Cell without the cellReservedForOtherUse] | √ | √ | √ | √ | √ | √ | √ |
| CAG-only Cell with PLMN ID B+ CAG ID C [CAG Cell with the cellReservedForOtherUse set true] |  |  |  |  |  | √ | √ |

- Therefore, the description in the Access Control may not fully cover all situations; the analysis is presented below:

-	The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG.

We can see that the legacy UE in this bullet refers to UE a/b/c/d/e based on the analysis above. However UE a/b/c/d/e include UE without CAG capability, UE with CAG capability and CAG-only indication and UE with CAG capability and no CAG-only indication. So using “legacy” will cause ambiguous.

-	The NR Femto may activate a cell shared by both PLMN and PNI-NPN, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse. Then, this cell is accessible to UEs which have the allowed CAG list including a CAG-ID broadcasted by the cell. For the legacy UE not supporting CAG, this cell is viewed as a normal PLMN cell.

UEs covered in yellow colour in this sentence only refers to UE f and UE g. For the legacy UE not supporting CAG covered in green colour in this sentence refers to UE a. Therefore UE a/c/d/e with CAG capability have been excluded from this scenario which are allowed to access this cell when it runs as a normal PLMN cell.

-	The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

- Proposal 6: Use explicit words instead of “legacy” to describe the involved UEs.

- Proposal 7: Modify the second bullet to include all the UEs which can be covered in this scenarios.

- Proposals

In this contribution, we provide our considerations on NR Femto and Access Control, and the following proposals were drawn from the analysis.

Proposal 1: Slice Supported List per PLMN is not necessary to be transferred from NR Femto GW to NR Femto.

Proposal 2: Adopt Option 2 to 38.413.

Proposal 3:  Use “Requested NSSAI” instead of “Requested S-NSSAI” similar as to the Allowed NSSAI IE used in 38.413.

Proposal 4: Unify the node’s name as “NR Femto” or “NR Femto node”.

Proposal 5: Add “PLMN” restriction to the “UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell”.

- Proposal 6: Use explicit words instead of “legacy” to describe the involved UEs.

- Proposal 7: Modify the second bullet to include all the UEs which can be covered in this scenarios.

- Reference

- TS 38.300, NR; NR and NG-RAN Overall description; Stage-2

- TS 38.413, NG-RAN; NG Application Protocol (NGAP)

- R3-247938

- R3-247936

- TS 23.122, Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode