3GPP TSG-RAN WG3 Meeting #127	R3-250091

Athens, Greece, 17 – 21 February 2025

Agenda item:	18.2

Source:	Nokia

Title:	Conclusions on RAN3 Impacts of LP-WUS

WID/SID:	NR_LPWUS

Document for:	Discussion and Decision

# 1	Introduction

At the last RANP#102 meeting, the work item of LP-WUS was agreed in [1] including the following objective:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

The work item has progressed in previous RAN3 meetings reaching already some agreements and working assumptions while some points remain open. This paper reviews the current agreements and propose conclusions for current open points in order to prepare the baseline CRs.

# 2	Discussion

The LP-WUS feature has been designed to optimize further energy consumption of the device. It provides similar benefits as prior low power features based on wake-up signals such as PEI but with some differences. Even though the feature applies to both idle, inactive and connected state the main RAN3 impact is related to idle/inactive states.

For example, due to the presence of the new WUR (wake up receiver), some further optimization is reached due to the possibility for the UE to not turn on at all the main radio to monitor the PDCCH even for monitoring the wake-up signal. In this so-called “deep sleep” mode, the wake-up signal is monitored by the WUR instead, enabling further energy saving. Moreover, the main radio could be relaxed from doing some of the measurements taken over by WUR.

RRC Idle/Inactive state

As highlighted in the description of the objective, the feature reuses subgrouping functions in similarity with PEI subgrouping. RAN2 agreed the following at last RAN2#125bis:

  - The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

Impacts to RAN3 are therefore similar to PEI with impacts due to:

- the configuration of subgroups. NGAP impacts are foreseen for the Context setup procedure,

- paging subgroups of devices. NGAP, XnAP and F1AP impacts for the paging procedures.

A summary of the impacts with resolution of open points is listed below per interface.

NG Impacts

At RAN3#125bis meeting it was agreed:

The LP-WUS assistance information IE, which includes the CN assigned subgroup ID for LP-WUS, is introduced in the NG paging message. FFS the number of the subgroups.

The LP-WUS assistance information IE is introduced in the NGAP core Networking assistance information IE to assist anchor gNB to initiate RAN paging. FFS the number of the subgroups.

RAN1#118bis further took a working assumption that the maximum number of subgroups for LP-WUS was set to 32.

Proposal 1: add to the NGAP BLCR the LP-WUS Paging Subgroup Assistance Information IE into the NGAP Paging message and into the NGAP Core Network Inactive Assistance Information IE including the CN subgroup ID IE with range from value 1 to value 32.

The second open point concerning NG was how to configure the number of CN subgroups which is known in the core network into the NG-RAN nodes. This number is necessary in the NG-RAN nodes in order to calculate the number of UE-ID based subgroups so that the sum of CN subgroups and UE-ID based subgroups match the total number of LP-WUS subgroups.

There were two options for NG-RAN to learn the number of CN subgroups:

- option 1: via NGAP signaling

- option 2: via O&M

Because this number is expected to be relatively static RAN3#126 agreed to use option 2. This should be captured in the BL CR for TS 38.300:

Proposal 2: add to the 38.300 BL CR that the number of CN subgroups is configured in NG-RAN nodes by O&M.

F1 Impacts

RAN3#125bis agreed that:

The LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, is introduced in F1 paging message for CN paging and RAN paging. FFS the number of the subgroups.

Proposal 3: add to the F1AP BL CR the LP-WUS Paging Subgroup Assistance Information IE (which includes the CN assigned subgroup ID) into the F1 Paging message including the CN subgroup ID IE with range from value 1 to value 32.

RAN3#125bis further agreed that:

The gNB-DU is responsible to generate and broadcast the LP-WUS related configuration, including LP-SS configuration, LP-WUS configuration. FFS on entry/exit condition for LP-WUS monitoring.

RAN3#126 further concluded that it is the gNB-DU to generate the entry/exit conditions and therefore no need to send them over F1 interface:

It is gNB-DU responsibility to generate and broadcast the entry/exit condition for LP-WUS monitoring for RRC_IDLE and RRC_INACTIVE.

This can be specified in TS 38.470. Also, draft baseline CR has been provided by Nokia in [2] as per agreed plan.

Proposal 4: add to the 38.470 BL CR that the gNB-DU is in charge of determining and broadcasting the LP-WUS entry/exit conditions. Agree tdoc [2] as baseline CR for TS 38.470.

RAN3#125bis further agreed that:

Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping.

RAN3#126 also took a working assumption that the LP-WUS subgrouping capability to be sent in the F1 Paging message is per cell and that it also applies to CN based subgrouping:

WA: Introducing LP-WUS subgrouping support indication per-cell in F1 paging message. This applies for both UE_ID based subgrouping and CN based subgrouping.

In fact, these two assumptions make the LP-WUS operation similar to PEI capability which is intended by RAN1/RAN2.  Besides, RAN2 recently agreed that LP-WUS will not be supported by all bands of network and UE. We therefore further propose to turn this working assumption into an agreement in order to proceed with the F1AP baseline CR:

Proposal 5: add to the F1AP BL CR the LP-WUS Subgrouping Support indication per cell in the F1 Paging message.

Xn Impacts

RAN3#126 took the following working assumption:

WA: the anchor gNB provides the new LP-WUS paging subgrouping assistance information IE into the Xn RAN paging message including CN assigned subgroup ID.

This working assumption was taken under the assumption that the LP-WUS operation is not unnecessarily restricted to the last serving cell. RAN2 has still not discussed this point but we can assume that RAN1/RAN2 does therefore not envision such restriction and it will still be possible to revert in the very unlikely case that it would change. Therefore, we also propose to turn this working assumption into an agreement for the time being to progress the stage 3:

Proposal 6: add to the XnAP BL CR the LP-WUS Paging Subgroup Assistance Information IE (which includes the CN assigned subgroup ID) into the XnAP RAN Paging message including the CN subgroup ID IE with range from value 1 to value 32.

Emergency Call Impact

The handling of emergency call when using LP-WUS was discussed at RAN3#126 and was left unconcluded.

In was clarified at last RAN3#126 meeting that this issue only concerns the PSAP fallback case.

Therefore, in our view, the extra delay may range at maximum to one DRX cycle is acceptable and no additional standardization is necessary and we can close this open point.

Proposal 7: No extra standards work is needed for the emergency call case.

# 3	Conclusion and Proposals

This paper has provided a summary of current agreements and has proposed conclusions for current open points. It is proposed to prepare the baseline CRs along the following lines:

Proposal 1: add to the NGAP BLCR the LP-WUS Paging Subgroup Assistance Information IE into the NGAP Paging message and into the NGAP Core Network Inactive Assistance Information IE including the CN subgroup ID IE with range from value 1 to value 32.

Proposal 2: add to the 38.300 BL CR that the number of CN subgroups is configured in NG-RAN nodes by O&M.

Proposal 3: add to the F1AP BL CR the LP-WUS Paging Subgroup Assistance Information IE (which includes the CN assigned subgroup ID) into the F1 Paging message including the CN subgroup ID IE with range from value 1 to value 32.

Proposal 4: add to the 38.470 BL CR that the gNB-DU is in charge of determining and broadcasting the LP-WUS entry/exit conditions. Agree tdoc [2] as baseline CR for TS 38.470.

Proposal 5: add to the F1AP BL CR the LP-WUS Subgrouping Support indication per cell in the F1 Paging message.

Proposal 6: add to the XnAP BL CR the LP-WUS Paging Subgroup Assistance Information IE (which includes the CN assigned subgroup ID) into the XnAP RAN Paging message including the CN subgroup ID IE with range from value 1 to value 32.

Proposal 7: No extra standards work is needed for the emergency call case.

# 4	References

- RP-240801, Low Power Wake up signal and receiver for NR, Vivo, NTT Docomo, Ericsson, Mediatek, Samsung, Sony

- R3-250092, Support of LP-WUS