3GPP TSG-RAN3 Meeting #127	R3-250095

Athens, Greece, Feb 17th – 21nd, 2025

Agenda Item:	16.2

Source: 	Qualcomm Incorporated

Title: 	Architecture, Protocols and Signaling to support Topology 1 of A-IoT

Document for:	Discussion/Decision

# Background

In this paper, we provide our views on aspects related to Architecture, Protocols and signaling to support Topology 1 of AIoT Topology.

# Discussion

## A-IoT RAN node

In our view, we should continue to use the terminology of “A-IoT RAN node” we used in study item phase to describe the RAN node supporting reader functionalities in Topology 1. We should not use the terminology gNB or NG-RAN node because the legacy NG-RAN node as defined in 38.300 has a specific definition.

As per TS 38.300, NG-RAN node is either a gNB (or ng-eNB), providing NR (or E-UTRA) user plane and control plane protocol terminations towards the UE, hosts host several functions (e.g., connection setup, paging, system information broadcast, header compression, ciphering, integrity protection), interconnected with each other via Xn interface, connects to AMF/UPF via NG-C/NG-U interface.

The A-IoT RAN node is an entity which interacts with an A-IoT device (not a UE) via a new A-IoT radio interface and several of the above functionalities e.g., header compression, ciphering, integrity protection, connection setup, “legacy” paging doesn’t apply to A-IoT RAN node.

Further, SA2 also captured in their conclusions (as copied below) that there is no assumption whether the A-IoT RAN node also has any gNB functionality and an assumption that A-IoT services should be able to deploy independent from existing deployments (i.e., an A-IoT RAN node functionalities need to be part of a legacy NG-RAN node and should be possible to deploy standalone).

From 23.700-13,

NOTE 1:	It is assumed AIoT RAN supports Ambient IoT specific functionalities. There is no assumption about whether the AIoT RAN (i.e. BS Reader) also has any gNB functionality for NR-Uu or not.

NOTE 3:	It is assumed Ambient IoT services can be deployed independently from existing deployments.

SA2 concluded that there is no assumption whether A-IoT RAN node has any gNB functionality for NR-Uu or not and that the A-IoT services can be deployed independently from exisiting deployments

A-IoT RAN node in Topology 1 interacts with a A-IoT device (not a UE) via a new A-IoT radio interface (not Uu) and doesn’t host several of the legacy NG-RAN node functionalities.

We therefore propose the following:

For Topology 1, RAN3 agrees to define a new type of NG-RAN node called "A-IoT RAN node" with the following principles:

  - A-IoT RAN node is a special type of NG-RAN node that has A-IoT reader functionalities and can communicate with A-IoT device via an A-IoT air interface

  - A-IoT RAN node is not assumed to support other functionalities of an NG-RAN node (e.g., it need not communicate with UE over Uu interface)

## “AIoT Reader Control” (A-RC) between A-IoT RAN node and AIoTF

From the conclusions in SA2 TR 23.700-13,

The principles and aspects in this clause are agreed to support Topology 1:

The new core network function (AIOTF) is introduced to support Ambient IoT functionality, described in clause 8.1.1, with the following features for topology 1:

Send requests that trigger BS Reader(s) (via AIoT RAN) to perform AIoT operations, either directly or via an AMF.

To align with SA2’s conclusions, RAN3 agrees to support both the direct and indirect  options for the A-IoT RAN node to communicate with AIoTF in Topology 1

- Direct path solution: A-IoT RAN node directly communicates with the AIoTF

- Indirect path solution: A-IoT RAN node indirectly (via the AMF) communicates with the AIoTF

Direct path solution:

Indirect path solution

Figure 4.6.3.1-2: Protocol Stack between AF and AIoT Device for Indirect Path via AMF option

Editor's note:	Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be updated based on RAN WG3 decision.

Regarding the above Editor’s note, we see 2 alternatives (Alt 1 and Alt 2 as captured in the Table below). We evaluate the pros and cons of the 2 options below:

| Alternatives | Evaluation |
| --- | --- |
| Alt 1: Define a new protocol "AIoT Reader Control" (A-RC) between A-IoT RAN node and AIoTF to transport A-IoT related information A-RC is carried over NGAP in case of both direct and indirect option | Same behavior for AIoTF for direct and indirect options AIoTF uses A-RC protocol to transport A-IoT related information to A-IoT RAN node for both direct and indirect option Direct option with Alt 1 Indirect option with Alt 1 Need to define a new spec for A-RC protocol |
| Alt 2: No end-to-end "A-IoT Reader Control" (A-RC) protocol is defined between A-IoT RAN node and AIoTF. A-IoT related information is carried explicitly in NGAP | Different behavior for AIoTF depending on direct vs. indirect In case of direct, AIoTF needs to transport AIoT information in NGAP. In case of indirect, AIoTF needs to transport AIoT information in API to AMF; AMF then includes the AIoTF information in NGAP Direct option with Alt 2 Indirect option with Alt 2 No need to define a new spec. NGAP is reused. |

RAN3 to discuss and down select among the 2 alternatives for the protocol stack and interfaces for Topology 1

- Alt 1: Define a new protocol "AIoT Reader Control" (A-RC) between A-IoT RAN node and AIoTF. A-RC is carried over NGAP for both direct and indirect options

- Alt 2: No end-to-end "A-IoT Reader Control" (A-RC) protocol is defined between A-IoT RAN node and AIoTF. A-IoT related information is carried explicilty over NGAP between A-IoT RAN and AIoTF (or AMF) in case of direct (or indirect) options

Alt 2 would mean different behavior for AIoTF depending on direct vs. indirect and is a drawback compared to Alt 1 (which ensures same behavior)

- In case of direct, AIoTF would need to transport AIoT information in NGAP.

- In case of indirect, AIoTF needs to transport AIoT information in API to AMF; AMF then includes the AIoTF information in NGAP

We therefore think Alt 1 is the best approach and therefore the following is proposed:

RAN3 agrees to support a a new protocol "AIoT Reader Control" (A-RC) between A-IoT RAN node and AIoTF to carry A-IoT related information for both direct and indirect options.

## Reader awareness at AIoTF

In the study item, there was a discussion on whether/how to support BS Reader awareness at the AIoTF. There were some views that the Readers within an AIoT RAN node are “internal” and not known to the AIoTF.

However, in our view, Readers within an AIoT RAN node should be “discoverable” by the AIoTF. There is no need to introduce such a restriction to keep the Readers “internal” to the AIoT RAN node and not expose it to the AIoTF. Even for LCS, currently TRPs hosted by a NG-RAN node are made aware at the CN via TRP Information Exchange procedures.

We therefore think that the AIoTF should be able to discover the “Readers” and can obtain information about the Readers supported by an AIoT RAN node, either via signalling with AIoT RAN or via OAM. Our preference is to specify signalling between AIoT RAN node and AIoTF instead of an OAM based solution.

Also SA2 assumes that AIoTF can be configured with needed information for BS Reader selection as mentioned in the SA2 TR and in the LS sent to RAN3.

SA2 has concluded for topology 1, the AIOTF selects the AIOT RAN node(s) and optionally provides a list of the BS readers to AIOT RAN.

SA2 assumes that OAM can configure the AIOTF with needed information for this selection.

SA2 assumes that AIoTF can configured with needed information (e.g., by OAM and/or A-IoT RAN node) for BS Reader selection

In NR positioning, TRPs supported by a NG-RAN node and the TRP information can be made aware at the LMF via TRP Information Exchange procedure and there is no limitation to keep the TRPs “hidden” at the CN.

In Topology 1, BS Readers within an AIoT RAN node should be “discoverable” by the AIoTF

AIoTF obtains the information of BS Readers supported by an AIoT RAN node via signaling with AIoT RAN node.

## Reader selection

SA2 has concluded for topology 1, the AIOTF selects the AIOT RAN node(s) and optionally provides a list of the BS readers to AIOT RAN.

With the Reader awareness in AIoTF as proposed in section 2.3, AIoTF can select a list of BS Reader(s) in addition to selecting the A-IoT RAN node(s). We therefore propose to confirm the SA2 conclusion.

Confirm SA2 assumption that AIoTF can select suitable AIoT RAN node(s) and also the BS Reader(s) within the selected AIoT RAN node(s) and signal the selected BS readers to the A-IoT RAN node

Further, SA2 sent a LS to RAN3 asking whether A-IoT RAN node can further provide supported A-IOT RAN node serving area and BS reader serving area.

SA2 assumes that OAM can configure the AIOTF with needed information for this selection.

SA2 have further discussed whether the AIOT RAN may provide the AIOT RAN information to the core network. e.g., supported AIOT RAN serving area, supported BS reader ID list and corresponding BS reader serving area

SA2 kindly asks RAN3 to clarify whether RAN3 plan to support the above AIOT RAN information exchange with AIOTF.

If so, SA2 kindly ask RAN3 to provide the definition of the appropriate parameters

We think it is beneficial to define an A-IoT RAN node’s serving area and BS reader’s serving area and make this information available dynamically (i.e., not only by OAM) to the AIoTF, so that the AIoTF can select appropriate AIoT RAN node(s) and optionally also the BS Reader(s). We therefore propose the following:

AIoTF obtains information about A-IoT RAN node’s serving area and BS reader’s serving area, via signaling with A-IoT RAN node.

## NGAP Signaling to support direct and indirect path options

NG Setup Request contains some mandatory IEs (Global RAN Node ID, Supported TA List, TAI Slice Support List, Default Paging DRX) which are not applicable to A-IoT e.g., there is no concept of TA List or slices or Paging DRX for A-IoT.

NG Setup Response contains some mandatory IEs (AMF Name, GUAMI, Relative AMF Capacity) which are also not all relevant to A-IoT e.g., there is no GUAMI at A-IoT device or there is no requirement to support AMF load balancing for which Relative AMF Capacity was defined.

NG Setup Request contains mandatory IEs (Global RAN Node ID, Supported TA List, TAI Slice Support List, Default Paging DRX) and NG Setup Response also includes some mandatory IEs (AMF Name, GUAMI, Relative AMF Capacity), all of which are not applicable to A-IoT

One way would be to reuse NG Setup Request/Response for A-IoT, but we would have to define exceptions for these mandatory IEs or add clarifications in semantics, which is not a clean way of design.

Reusing NG Setup Request/Response for A-IoT would mean defining exceptions for the mandatory IEs which are not applicable to A-IoT or semantic clarifications, which is not a clean way of design.

To have a clean signalling design, we propose to define a new “AIoT NG Setup Request/Response” message for A-IoT RAN node to communicate with AIoTF (in case of direct option) or with AMF (in case of indirect option).

Define a new “AIoT NG Setup Request/Response” message for A-IoT RAN node to communicate with AIoTF (in case of direct option) or with AMF (in case of indirect option).

AIoT NG Setup Request may contain a A-RC UL container which can include several IEs such as A-IoT RAN node ID, A-IoT RAN node’s serving area, Supported BS Reader(s), BS Reader’s serving area

In response, AIoTF (in case of direct) and AMF (in case of indirect) signals an AIoT NG Setup Response containing an A-RC DL container containing IEs TBD.

A-IoT NG Setup Request may contain a A-RC UL container which can include several IEs such as A-IoT RAN node ID, A-IoT RAN node’s serving area, Supported BS Reader(s), BS Reader’s serving area

Direct option

Indirect option

# Summary

Based on the discussion above, we have following observations and proposals:

2.1 A-IoT RAN node

Observation 1:	SA2 concluded that there is no assumption whether A-IoT RAN node has any gNB functionality for NR-Uu or not and that the A-IoT services can be deployed independently from exisiting deployments

Observation 2:	A-IoT RAN node in Topology 1 interacts with a A-IoT device (not a UE) via a new A-IoT radio interface (not Uu) and doesn’t host several of the legacy NG-RAN node functionalities.

Proposal 1: For Topology 1, RAN3 agrees to define a new type of NG-RAN node called "A-IoT RAN node" with the following principles:

a. A-IoT RAN node is a special type of NG-RAN node that has A-IoT reader functionalities and can communicate with A-IoT device via an A-IoT air interface

b. A-IoT RAN node is not assumed to support other functionalities of an NG-RAN node (e.g., it need not communicate with UE over Uu interface)

2.2 “AIoT Reader Control” (A-RC) between A-IoT RAN node and AIoTF

Proposal 2: To align with SA2’s conclusions, RAN3 agrees to support both the direct and indirect  options for the A-IoT RAN node to communicate with AIoTF in Topology 1

 Direct path solution: A-IoT RAN node directly communicates with the AIoTF

 Indirect path solution: A-IoT RAN node indirectly (via the AMF) communicates with the AIoTF

Proposal 3: RAN3 to discuss and down select among the 2 alternatives for the protocol stack and interfaces for Topology 1

 Alt 1: Define a new protocol "AIoT Reader Control" (A-RC) between A-IoT RAN node and AIoTF. A-RC is carried over NGAP for both direct and indirect options

  Alt 2: No end-to-end "A-IoT Reader Control" (A-RC) protocol is defined between A-IoT RAN node and AIoTF. A-IoT related information is carried explicilty over NGAP between A-IoT RAN and AIoTF (or AMF) in case of direct (or indirect) options

Observation 3: Alt 2 would mean different behavior for AIoTF depending on direct vs. indirect and is a drawback compared to Alt 1 (which ensures same behavior)

 In case of direct, AIoTF would need to transport AIoT information in NGAP.

 In case of indirect, AIoTF needs to transport AIoT information in API to AMF; AMF then includes the AIoTF information in NGAP

Proposal 4: RAN3 agrees to support a a new protocol "AIoT Reader Control" (A-RC) between A-IoT RAN node and AIoTF to carry A-IoT related information for both direct and indirect options.

2.3 Reader awareness at AIoTF

Observation 4:	SA2 assumes that AIoTF can configured with needed information (e.g., by OAM and/or A-IoT RAN node) for BS Reader selection

Observation 5:	In NR positioning, TRPs supported by a NG-RAN node and the TRP information can be made aware at the LMF via TRP Information Exchange procedure and there is no limitation to keep the TRPs “hidden” at the CN.

Proposal 5:	In Topology 1, BS Readers within an AIoT RAN node should be “discoverable” by the AIoTF

Proposal 6:	AIoTF obtains the information of BS Readers supported by an AIoT RAN node via signaling with AIoT RAN node.

2.4 Reader selection

Proposal 7:	Confirm SA2 assumption that AIoTF can select suitable AIoT RAN node(s) and also the BS Reader(s) within the selected AIoT RAN node(s) and signal the selected BS readers to the A-IoT RAN node

Proposal 8:	AIoTF obtains information about A-IoT RAN node’s serving area and BS reader’s serving area, via signaling with A-IoT RAN node.

2.5 NGAP Signaling to support direct and indirect path options

Observation 6:	NG Setup Request contains mandatory IEs (Global RAN Node ID, Supported TA List, TAI Slice Support List, Default Paging DRX) and NG Setup Response also includes some mandatory IEs (AMF Name, GUAMI, Relative AMF Capacity), all of which are not applicable to A-IoT

Observation 7:	Reusing NG Setup Request/Response for A-IoT would mean defining exceptions for the mandatory IEs which are not applicable to A-IoT or semantic clarifications, which is not a clean way of design.

Proposal 9:	Define a new “AIoT NG Setup Request/Response” message for A-IoT RAN node to communicate with AIoTF (in case of direct option) or with AMF (in case of indirect option).

Proposal 10:	A-IoT NG Setup Request may contain a A-RC UL container which can include several IEs such as A-IoT RAN node ID, A-IoT RAN node’s serving area, Supported BS Reader(s), BS Reader’s serving area