3GPP TSG-RAN3 Meeting #127	R3-250096

Athens, Greece, Feb 17th – 21nd, 2025

Agenda Item:	16.2

Source: 	Qualcomm Incorporated

Title: 	Inventory and Command procedures for Topology 1 of AIoT

Document for:	Discussion/Decision

# Background

In this paper, we provide our views on how to support Inventory and Command procedures for Topology 1 of A-IoT.

# Discussion

## Inventory-only

The following call flow was captured in the TR 38.769:

Figure 6.5.2.1-1: Message flow for A-IoT Inventory in Topology 1

We propose to follow the similar signalling structure as in the TR and therefore propose the following:

To support Inventory for Topology 1, define a class-1 message Inventory Request/Inventory Response and a class-2 message Inventory Report message (over A-RC protocol)

In TR 38.769, it was captured that the Inventory Request from AIoTF to A-IoT RAN node may contain an A-IoT Device Identification (details upto SA2 and RAN2) and a Scope of Inventory Request (e.g., a certain area in which the inventory is to be triggered). We propose to confirm the above details to be included in the Inventory Request.

Further as per SA2 conclusions and as mentioned in R3-25aaaa, the AIoTF may further select BS Readers and signal the selected BS Readers to the A-IoT RAN node. We therefore propose to also include the Selected BS Readers in the Inventory Request.

Further, the following was captured in the Conclusions in SA2 TR:

The AIOTF may provide the following assistance information to AIoT RAN/UE Reader:

- AIoT service type (e.g. Inventory, Command);

- approximate number of AIoT devices based on AF request;

- approximate D2R message size based on AF request.

We think that the A-IoT RAN node can distinguish whether it is an inventory or command based on the type of A-RC message it receives from AIoTF (Inventory Request vs. Command Request). There is no need to explictly indicate the type of A-IoT service request (inventory or command).

The other two information (approximate number of AIoT devices and approximate D2R message size) can be provided from AIoTF to A-IoT RAN node and can be used as assistance information for planning resources for inventory.

Further, SA2 concluded that a “Correlation ID” can be useful to correlate the inventory responses to the request as shown below:

AIoTF constructs a request for an Inventory operation using the determined A-IoT Device Identification information page the AIoT Devices, and a correlation identifier for the AIOTF to correlate the inventory responses to the request. The Inventory request is routed to the Readers determined by the initial reader selection.

The Reader executes the inventory request, reporting AIoT specific NAS message responses from the AIoT Device to the AIOTF, including its Reader ID and correlation identifier from the AIOTF. The Reader may aggregate results from multiple AIoT Devices in the responding messages. The AIOTF can determine which request the results are for using the correlation identifier.

Therefore, the following is proposed:

Inventory Request sent from AIoTF to A-IoT RAN node (over A-RC protocol) may contain the following

  - Correlation ID

  - A-IoT Device Identification. Details of “A-IoT Device Identification” up to SA2 and RAN2

  - Scope of Inventory Request (e.g., a certain area in which the inventory is to be triggered)

  - Selected BS Reader(s) for Inventory

  - Approximate number of A-IoT devices

  - Approximate D2R message size

Inventory Response (class-1 response to Inventory Request) shall be sent to AIoTF acknowledging that the inventory request was successfully received and shall contain the Correlation ID.

Inventory Response sent from A-IoT RAN node to AIoTF (over A-RC protocol) may contain the following:

  - Correlation ID

Further, SA2 concluded the following:

AIoT Device NAS protocol is supported between the AIoT Device and the AIOTF.

The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response.

Please note that the Inventory Response in SA2 call flows corresponds to the Inventory Report in RAN3 call flows. Both these messages refer to the message where the Device ID(s) of the inventoried devices are sent from A-IoT RAN node to the AIoTF. Since this is carried in AIoT NAS layer as mentioned in SA2 conclusion above, we propose to provide a list of AIoT NAS containers in the Inventory Report.

Further, the Inventory Report may contain the Correlation ID, and the Reader ID associated to the A-IoT device(s) which are inventoried, so that the AIoTF can use this information for any subsequent command signaling.

Inventory Report sent from A-IoT RAN node to AIoTF (over A-RC protocol) may contain the following:

  - Correlation ID

  - A list of AIoT NAS containers, each carrying the A-IoT device ID of the inventoried A-IoT device

  - Reader ID

## Inventory followed by Command

The following call flow was captured in TR 38.769

Figure 6.5.2.2-1: Message flow for A-IoT Command in Topology 1

We propose to follow the similar signalling structure as captured in the TR 38.769 i.e., define a class-1 message (Command Request/Response)

To support Command for Topology 1, define a class-1 message (Command Request / Command Response).

As mentioned in the Inventory-only section, Command Request and Command Response are sent in AIoT NAS layer and therefore the Command Request/Response (over A-RC protocol) shall contain a AIoT NAS container.

Further, Correlation ID and other assistance information (e.g., the BS reader last associated to the A-IoT device for which the command is sent and Approximate D2R message size) can be sent in Command Request.

We therefore propose the following:

Command Request sent from AIoTF to A-IoT RAN node may contain the following:

  - Correlation ID

  - AIoT NAS container (with the Device ID for which the command is sent)

  - BS Reader last associated to the A-IoT device for which the command is to be sent

  - Approximate D2R message size

Command Response sent from A-IoT RAN node to AIoTF may contain the following:

  - Correlation ID

  - AIoT NAS container (with the Command Response)

# Summary

Based on the discussion above, we have following observations and proposals:

2.1	Inventory-only

Proposal 1:	To support Inventory for Topology 1, define a class-1 message Inventory Request/Inventory Response and a class-2 message Inventory Report message (over A-RC protocol)

Proposal 2:	Inventory Request sent from AIoTF to A-IoT RAN node (over A-RC protocol) may contain the following

a. Correlation ID

b. A-IoT Device Identification. Details of “A-IoT Device Identification” up to SA2 and RAN2

c. Scope of Inventory Request (e.g., a certain area in which the inventory is to be triggered)

d. Selected BS Reader(s) for Inventory

e. Approximate number of A-IoT devices

f. Approximate D2R message size

Proposal 3:	Inventory Response sent from A-IoT RAN node to AIoTF (over A-RC protocol) may contain the following:

a. Correlation ID

Proposal 4:	Inventory Report sent from A-IoT RAN node to AIoTF (over A-RC protocol) may contain the following:

a. Correlation ID

b. A list of AIoT NAS containers, each carrying the A-IoT device ID of the inventoried A-IoT device

c. Reader ID associated to each A-IoT device ID

2.2 Inventory followed by Command

Proposal 5:	To support Command for Topology 1, define a class-1 message (Command Request / Command Response).

Proposal 6:	Command Request sent from AIoTF to A-IoT RAN node may contain the following:

a. Correlation ID

b. AIoT NAS container (with the Device ID for which the command is sent)

c. BS Reader last associated to the A-IoT device for which the command is to be sent

d. Approximate D2R message size

Proposal 7:	Command Response sent from A-IoT RAN node to AIoTF may contain the following:

a. Correlation ID

b. AIoT NAS container (with the Command Response)