3GPP TSG-RAN3 Meeting #127	R3-250097

Athens, Greece, Feb 17th – 21nd, 2025

Agenda Item:	16.3

Source: 	Qualcomm Incorporated

Title: 	Locating Ambient IoT devices

Document for:	Discussion/Decision

# Background

In this paper, we provide our views for locating an AIoT device at “reader” granularity as concluded in the SI phase.

# Discussion

## Signalling details for A-IoT location reporting

The AIoTF can send a AIoT location report config (e.g., indicating the AIoT device(s) for which location is requested, location reporting conditions) to a Reader and the Reader can send the AIoT device’s “location” (e.g., the “Reader ID” to which the AIoT device was last associated with). An example procedure is shown below:

Step 1: A-IoT RAN node receives a message from the AIoTF requesting the location information of one or more AIoT device(s) via an AIoT location report config.

- The AIoT devices for which the location information is requested can be identified by a AIoT device ID. FFS on the format of AIoT device ID (pending SA2 progress).

- The AIoT location report config can include when/how to report location information e.g., location to be reported just “one-time” or periodically or upon some other event being met.

- AIoT location report config can be sent as part of Inventory Request/Command Request or can be sent separately in a dedicated message (Location Request)

Step 2: A-IoT RAN node stores the AIoT location report config and determines whether the condition to report the location information is satisfied.

Step 3: Upon determining that the condition to report location is satisfied, A-IoT RAN node sends an AIoT Location Report to the AIoTF with the Reader ID(s) associated to the requested AIoT device(s).

Inventory Request and/or Command Request can include a “AIoT location reporting configuration”, indicating the AIoT device(s) for which location is requested, and the location reporting conditions (e.g., periodicity).

Upon determining that the condition to report location is satisfied, the A-IoT RAN node can send the AIoT device’s “location” (e.g., the “Reader ID”) to the AIoTF in Inventory Report and/or Command Report.

The AIoTF might want to obtain the A-IoT device location independently, not necessarily along with inventory or command. Further, the AIoTF might want to obtain location with a different periodicity compared to the periodicity with which Inventory/Command Reports are sent to AIoTF. We therefore propose to support an independent procedure (“Location Request/Response) for AIoTF to request location from Reader.

AIoTF should be able to request A-IoT RAN node to provide the location of an AIoT device and obtain the requested location in a dedicated message (other than Inventory and Command) e.g., via Location Request/Response

## Reader location awareness at AIoTF

For T1, the following options were discussed in the SI phase:

In Topology 1, the AIoTF knows the location of a reader supported by an AIoT RAN node by one of the following methods:

- Option 1a: AIoTF is configured by O&M about the readers supported by an AIoT RAN node and their location.

- Option 1b: AIoTF receives the list of readers supported by an AIoT RAN node and their location in an XXAP Setup/Configuration Update message received beforehand.

In our view, Option 1b (via the A-RC signaling procedure) is preferred compared to an OAM based solution (which is usually proprietary). This also aligns with our proposal in the companion paper R3-250095 where we propose that the AIoT RAN node indicates its list of supported readers to the AIoTF in a A-RC signaling procedure. The location of the readers can also be indicated via this A-RC signaling procedure.

In case of T1, the location of the Readers supported within an AIoT RAN node can be known via a A-RC signaling procedure, wherein the AIoT RAN node indicates its list of supported readers and their location to the AIoTF.

# Summary

2.1	Signalling details for A-IoT location reporting

Proposal 1:	Inventory Request and/or Command Request can include a “AIoT location reporting configuration”, indicating the AIoT device(s) for which location is requested, and the location reporting conditions (e.g., periodicity).

Proposal 2:	Upon determining that the condition to report location is satisfied, the A-IoT RAN node can send the AIoT device’s “location” (e.g., the “Reader ID”) to the AIoTF in Inventory Report and/or Command Report.

Proposal 3:	AIoTF should be able to request A-IoT RAN node to provide the location of an AIoT device and obtain the requested location in a dedicated message (other than Inventory and Command) e.g., via Location Request/Response

2.2	Reader location awareness at AIoTF

Proposal 4:	In case of T1, the location of the Readers supported within an AIoT RAN node can be known via a A-RC signaling procedure, wherein the AIoT RAN node indicates its list of supported readers and their location to the AIoTF.