3GPP TSG-RAN3 Meeting #127	R3-250098

Athens, Greece, Feb 17th – 21nd, 2025

Agenda Item:	10.2

Source: 	Qualcomm Incorporated

Title: 	MRO enhancements for R18 mobility mechanisms

Document for:	Discussion/Decision

# Background

In this paper, we discuss potential MRO enhancements for R18 mobility mechanisms for the following objective in Rel-19 SON/MDT WI based on the open issues and agreements last meeting.

MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

- Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

# Discussion

## MRO for LTM

### Distinguishing UE-based TA vs. NW provided TA

RAN2 agreed the following:

Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined).

RAN3 has the following open issue:

FFS whether to support network-based solution or UE-based solution for the gNB to be aware of the LTM candidate cells and the TA acquisition type (UE-based or NW-provided)

During LTM preparation, source gNB provides the UE with Early UL Sync configuration (obtained from candidate cells) and whether UE-based TA measurement should be performed for each LTM candidate cell. Also, the source gNB-DU knows that it has sent a PDCCH order to trigger Early UL sync procedure and whether it has sent a LTM CSC MAC-CE with a valid TA. The network can store this information and determine whether the UE used the NW-provided TA or UE-based TA and there is no need for UE to report it in RLF Report.

For each LTM candidate cell, source gNB knows whether it has provided the UE with Early UL Sync configuration, whether it has configured the UE to perform UE-based TA measurement, whether it has sent a PDCCH order to trigger Early UL sync procedure with that candidate cell and whether it has sent an LTM CSC MAC-CE with a valid TA

Source gNB can determine whether the UE had determined the TA via UE-based TA measurement or gNB indicated TA value in CSC by retrieving the UE context upon receiving the RLF Report

Send an LS to RAN2 that RAN3 has agreed on NW-based solution and that there is no need for UE to report to the gNB in RLF Report how the UE got the TA value (gNB indicated or UE determined)

### Optimizing LTM candidate cells at gNB-CU

For failures due to wrong selection of candidate LTM cell, CU is in charge of root cause analysis and performs optimization.

Aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

During LTM preparation, gNB-CU determines the LTM candidate cells (e.g., based on L3 measurements from UE). It was also agreed in RAN3 meeting that gNB-CU is responsible for optimizing LTM candidate cells. But it is not yet clear how the gNB-CU optimizes the LTM candidate cell list (e.g., based on UE assistance in RLF Report or based on a network-based solution).

RAN2 further agreed the following:

Unless RAN3 defines a NW-based solution: Introduce an explicit indication in RLF-Report to indicate whether a neighbour cell is an LTM candidate cell.

In Rel-17, we similarly discussed how to optimize the CHO candidate cell list and there were 2 options discussed - a network-based solution and a UE based solution. After a contentious discussion, both solutions were agreed, albeit for different sub cases:

- UE based solution (for the case of CHO execution failure): UE reports the CHO candidate cell list in RLF Report, in case of CHO execution failure.

- Network based solution (for the case of RLF shortly after a successful CHO execution): Source gNB provides the CHO configuration (which includes CHO candidate cell list and CHO execution conditions) to target gNB in SN STATUS TRANSFER. Target gNB stores it and provides it back to source gNB in HANDOVER REPORT

In Rel-17, the UE based solution was agreed first, but then we realized that the UE releases the CHO configuration after a successful CHO execution and therefore UE based solution can’t work in the case that there is a RLF shortly after the successful CHO execution. Therefore, the network-based solution was also agreed.

To optimize CHO candidate cell list, Rel-17 specified two solutions - a UE based solution (UE reports the CHO candidate cell list in RLF Report) and a network-based solution (source gNB provides the CHO configuration to target gNB in SN STATUS TRANSFER which stores it and provides it back to source gNB in HANDOVER REPORT).

LTM is different from CHO in that we support subsequent LTM and therefore UE stores the LTM configuration even after a successful LTM execution. Also, Rel-18 only supports intra-CU LTM and therefore the gNB-CU can have the LTM candidate cells in its UE context even if there is inter-DU mobility within the gNB-CU.

In Rel-19 MRO for LTM, we should avoid having two solutions (i.e., both UE based and network-based solution) for optimizing LTM candidate cell list. Also, existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF.

We propose to use a network-based solution as follows:

There is no need for UE to report the LTM candidate cell list (or an indication whether a cell was LTM candidate or not) in RLF Report. Existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF. Send a LS to RAN2 informing this NW-based solution.

### UHI/MHR enhancements for LTM

The following open issue was captured as FFS in last RAN3 meeting:

On UHI and ping-pong,

Which node (CU or DU) is responsible for avoiding ping-pong of LTM cell switch?

Whether CU should forward UHI to DUs?

gNB-DU is responsible for selecting the target cell among the configured LTM candidates and sending the LTM CSC MAC-CE. Therefore, gNB-DU should be responsible for avoiding ping-pong LTM cell switches

gNB-DU is responsible for avoiding ping-pong LTM cell switches

Upon completing LTM cell switch, gNB-DU indicates ACCESS SUCCESS message to gNB-CU. Upon receiving this, gNB-CU should update the UHI to include the cell ID, time stayed in the cell and add an indicator for the “HO Type” (i.e., LTM or L3) to distinguish L3 and LTM cell switch.

Further, a UE supporting LTM might undergo both LTM and normal L3 HO scenarios depending on the gNB implementation. A gNB might do a back-to-back L3 HO and LTM due to L3 execution conditions being met in gNB-CU and LTM execution condition met in gNB-DU.

Therefore, it useful for the gNB to know whether the UE’s visited cells were due to LTM or L3 HO. Without such knowledge, the gNB might unnecessarily be trying to optimize ping-pong handovers even though they happened due to different reasons (i.e., L3 HO and LTM)

We therefore propose RAN3 to discuss how to distinguish genuine ping-pong handovers vs. back-to-back L3 HO and LTM. One solution could be to optimize the UHI to include a HO-type (i.e., LTM or L3 HO) similar to how it has been agreed by RAN2 for RLF Report.

Upon receiving ACCESS SUCCESS from gNB-DU to gNB-CU (indicating an LTM cell switch), the gNB-CU shall update UHI and add an indicator in UHI that the “last HO Type” was due to LTM cell switch.

After updating the UHI, gNB-CU shall forward the UHI to gNB-DU for gNB-DU to avoid ping-pong LTM cell switches.

gNB-CU may forward UHI to gNB-DU, so that gNB-DU can detect and avoid ping-pong LTM cell switches. Enhance F1AP to support this.

### RACH-less LTM failure scenarios

In the previous meetings, some companies proposed to discuss RACH-less LTM to RACH-based LTM “fall back” scenarios. In our understanding and as per TS 38.321, there is no “fall back” from RACH-less LTM to RACH-based LTM.

As per TS 38.321, UE considers RACH-less LTM as ongoing only if it receives a valid TA from LTM CSC or if the UE-based TA measurement is successful.

In other words, if the UE doesn’t receive a valid TA from LTM CSC or if the UE-based TA measurement is not successful, UE doesn’t even start the RACH-less LTM cell switch procedure and directly attempts a RACH-based LTM procedure.

If the UE receives a TA value “FFF” in the CSC or if the UE-based TA measurement is not successful, UE doesn’t even start the RACH-less LTM cell switch procedure and directly attempts a RACH-based LTM procedure. In other words, there is no “fall back” from RACH-less LTM to RACH-based LTM and there is only RACH-less LTM “initiation” failure.

However, there could be a scenario where the RACH-less LTM is initiated (because of UE having a valid TA either received from LTM CSC or via UE-based measurement) but the RACH-less LTM cell switch execution is not successful e.g., because UE couldn’t determine that the network has successfully received its first UL data. In such a case, T304 could expire.

RACH-less LTM cell switch execution may fail (i.e., T304 may expire) because UE couldn’t determine that the network has successfully received its first UL data after RACH-less LTM cell switch execution

Upon T304 expiry, UE performs cell selection and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after LTM execution failure (i.e., LTM recovery), then the UE attempts RACH-based LTM execution once, otherwise re-establishment is performed. We propose RAN3 to consider these the following scenarios as part of MRO for LTM:

RAN3 to consider the following scenarios:

  - RACH-less LTM “initiation” failure (e.g., due to TA = FFF received in CSC or unsuccessful UE-based TA measurement) resulting in UE triggering RACH-based LTM procedure

  - RACH-less LTM execution failure

    - followed by successful RACH-based LTM recovery

    - followed by RACH-based LTM recovery failure

### Optimizing beam information indicated in LTM CSC

From TS 38.300,

The gNB-DU decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state, or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available.

As mentioned above, the gNB-DU can indicate beam information (TCI state ID, UL TCI state ID) in LTM cell switch command MAC CE to the UE.

The UE can use this information during LTM execution e.g., considers the SSB associated to the TCI state indicated by TCI state ID field as the one used for configured uplink grant selection for the initial uplink transmission towards the candidate cell during RACH-less LTM cell switch.

It is possible that the gNB-DU might not have indicated the right TCI state ID and therefore the UE might have used the wrong one thereby resulting in LTM cell switch failure. We therefore propose the following:

RAN3 should discuss solutions on how to optimize the target beam(s) (indicated via TCI state(s)) in LTM cell switch command MAC CE to UE for LTM execution.

### LTM-CHO coexistence

Suppose CHO and LTM are both configured and there is a LTM failure (or a RLF shortly after a successful LTM). The following recovery mechanisms are possible, if configured by the network:

- UE will perform CHO recovery if the selected cell after RLF is a CHO candidate cell

- UE will perform LTM recovery if the selected cell after RLF is an LTM candidate cell

- It is up to UE on what recovery UE should perform if the selected cell after RLF is both a CHO candidate and LTM candidate cell

If no recovery is configured by the network, the UE simply performs cell selection and performs reestablishment on the selected cell

Further there could be scenarios where a UE is configured with both CHO candidate cells and LTM candidate cell. The gNB might want to avoid configuring CHO in case there is always an LTM that gets executed resulting in a “wasteful” CHO configuration.

Therefore, the following is proposed.

RAN3 should discuss scenarios when a UE is configured with both LTM and CHO and whether any enhancements are needed to enable joint MRO for L3 HO and LTM e.g., indicate to gNB that CHO was configured before LTM was executed (or vice-versa)

## 2.2 MRO for CHO with candidate SCG

### 2.2.2 Forwarding mechanism

The following was agreed in the last RAN3 meeting:

The failure types defined for CHO in TS 38.300 and for CPAC defined in TS 37.340 are used as baseline. Clarification or amendment could be made on top of that if needed

The cases identified for MRO for CHO with candidate SCG were classified as follow:

Case 1a), 2a), 3a), 9a): Too late handover (i.e., Too late CHO execution)

Case 1b), 2b), 3b), 9b): Too late CPC execution

Case 7a), 8a): Too early HO (i.e. Too early CHO execution) or Handover (i.e. CHO execution) to wrong cell

Case 7b), 8b): Too Early CPC/CPA Execution or CPC/CPA Execution to wrong PSCell

Also, the following was agreed:

In case of too late CHO execution, the last serving MN may need to send message to the candidate MN(s) which may need optimization. FFS which message is used from the last serving MN to the candidate MN(s): Handover Report or via new message

- The following stage-2 text was agreed:

In case of Too Late Handover when UE is configured with CHO with candidate SCG(s), the NG-RAN node receiving the failure indication may inform the failure event to the candidate NG-RAN node(s) controlling the candidate PSCell where the mobility configuration caused the failure.

Editor’s note: FFS on the definition of failure event.

In cases 1a, 2a, 3a, 9a (Too late CHO execution), where there is a RLF in source PCell before the CHO with candidate SCG is executed, UE collects RLF Report and sends it to a node upon request. The retrieving node can identify the type of HO failure (i.e. Too late handover) and forward the RLF Report to the source MN in the HANDOVER REPORT like it is done currently.

As per the current agreement, the “failure event” is also sent to the candidate MN(s) for optimization. It is not clear what this “failure event” means. One option would be to send the complete RLF Report to candidate MN(s) or another option would be to just send only the necessary information in RLF Report to candidate MN(s). In our view, only a subset of the information is sufficient. We therefore propose the following:

Candidate MN(s) should be only provided with a subset of information from RLF Report (not the complete RLF Report) for optimizing CHO with candidate configuration (e.g., identifier of the candidate PCells/PSCells which met the configured CHO/CPC execution conditions before the RLF is encountered). RAN3 should discuss which information is useful at the candidate MN(s)

One thing which is not clear is how the forwarding mechanism should work. We see two alternative options and propose RAN3 to clarify this:

RAN3 should discuss how the node retrieving the RLF Report should forward the “failure event” to the candidate MN(s)

  - Alt 1: Node retrieving RLF Report  source MN  candidate MN(s)

  - Alt 2: Node retrieving RLF Report  source MN and Node retrieving RLF Report  candidate MN(s)

- RAN3 also agreed the following:

SCGFailureInfomation handling for case 1b)/2b)/3b)/9b) and case 7b)/8b):

The serving MN which receives SCGFailureInformation from the UE performs the initial analysis

The serving MN forward the SCGFailureInformation to the respective MN which should perform the optimization if needed (e.g., for case 3b)

Whether the above mechanism works depends on RAN2 progress on SCGFailureInfomation.

From the above agreement, it is not clear what “respective MN” means. We assume this means other candidate MN(s) involved in the CHO with candidate SCG configuration. We raise a similar question as above – whether to forward the complete SCGFailureInformation or only need to forward a subset of the information in SCGFailureInformation. In our view, a subset of the information is sufficient.

Serving MN should forward only a subset of information from SCGFailureInformation Report (not the complete SCGFailureInformation) to candidate MN(s) for optimizing CHO with candidate configuration. RAN3 should discuss which information is useful at the Candidate MN(s) and to be forwarded.

### 2.2.1 Concurrent RLF + SCG Failure scenario

Agreements and open issues so far:

MRO for Case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed

Do not distinguish between fast MCG recovery/no-fast MCG recovery (after the CHO with candidate SCG execution failure)

FFS whether there is any specification impact for case 4/5/6.

Case 1c, 2c, 3c, 7c, 8c are FFS

FFS whether to support concurrent failure cases (RLF + SCG failure). The definition of these cases needs to be further clarified.

Concurrent MCG and SCG failures can be deprioritized as this is not specific to CHO with candidate SCG. This was also considered in R17/R18 (e.g., MCG failure before UE sends SCGFailureInformation) but no enhancements were specified.

The other scenario where fast MCG recovery via SCG could not be initiated because SCG fails immediately after MCG fails is already deprioritized as RAN3 agreed to not distinguish between fast MCG recovery/no-fast MCG recovery (after the CHO with candidate SCG execution failure). Therefore, the following is proposed:

There is no need to support concurrent failure cases (RLF + SCG failure) i.e., there is no need to support case 1c, 2c, 3c, 7c, 8c

## 2.3 MRO for Subsequent CPAC (S-CPAC)

RAN3 agreements:

Work on the scenarios of failure in S-CPAC.

The optimization of non-failure scenarios (e.g., near failure and ping-pong) is not excluded.

R19 SON/MDT solution discussion is based on R18 work.

Reuse SCG FAILURE INFORMATION REPORT message over Xn for MN to report SCG failure of SCPAC to the concerned SN.

FFS whether to capture in stage-2 the following text: “For the CPAC failures which occurred during an S-CPAC procedure, CPC Execution above refers to the initial CPC execution or the subsequent CPC execution”

FFS whether the differentiation between initial and following CPC is needed

It was agreed to reuse existing text on forwarding mechanism for CPAC is reused for S-CPAC​. Further, following stage-2 text was agreed:

​For CPA or MN initiated CPC or initial S-CPAC initiated by MN, if the suitable PSCell is one of the candidate PSCells provided by the MN at CPAC preparation, but not one of the candidate PSCells selected by the candidate or target SN, MN sends the SCG Failure Information Report message to the candidate or target SN, which perform the final MRO related optimisation. Otherwise, the MN performs the final MRO related optimisation.​

​For SN initiated CPC or a following S-CPC initiated by MN, the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.​

​Editor’s note: The forwarding mechanism in the above two paragraphs needs to be further checked.

In our view, the current stage-2 text does not consider the forwarding mechanism for SN-initiated S-CPAC. We therefore propose the following:

Enhance the currently agreed stage-2 text on forwarding mechanism for S-CPAC to also include “SN initiated S-CPAC” scenario.

# Summary

2.1	MRO for LTM

Observation 1.	For each LTM candidate cell, source gNB knows whether it has provided the UE with Early UL Sync configuration, whether it has configured the UE to perform UE-based TA measurement, whether it has sent a PDCCH order to trigger Early UL sync procedure with that candidate cell and whether it has sent a LTM CSC MAC-CE with a valid TA

Proposal 1:	Source gNB can determine whether the UE had determined the TA via UE-based TA measurement or gNB indicated TA value in CSC by retrieving the UE context upon receiving the RLF Report

Proposal 2:	Send an LS to RAN2 that RAN3 has agreed on NW-based solution and that there is no need for UE to report to the gNB in RLF Report how the UE got the TA value (gNB indicated or UE determined)

Observation 2.	To optimize CHO candidate cell list, Rel-17 specified two solutions - a UE based solution (UE reports the CHO candidate cell list in RLF Report) and a network-based solution (source gNB provides the CHO configuration to target gNB in SN STATUS TRANSFER which stores it and provides it back to source gNB in HANDOVER REPORT).

Proposal 3:	There is no need for UE to report the LTM candidate cell list (or an indication whether a cell was LTM candidate or not) in RLF Report. Existing information e.g., the timeConnFailure and C-RNTI reported in RLF Report can be used to retrieve the UE’s LTM candidate cells at the time of RLF. Send a LS to RAN2 informing this NW-based solution.

Proposal 4:	gNB-DU is responsible for avoiding ping-pong LTM cell switches

Proposal 5:	Upon receiving ACCESS SUCCESS from gNB-DU to gNB-CU (indicating an LTM cell switch), the gNB-CU shall update UHI and add an indicator in UHI that the “last HO Type” was due to LTM cell switch.

Proposal 6:	gNB-CU may forward UHI to gNB-DU, so that gNB-DU can detect and avoid ping-pong LTM cell switches. Enhance F1AP to support this.

Observation 3.	If the UE receives a TA value “FFF” in the CSC or if the UE-based TA measurement is not successful, UE doesn’t even start the RACH-less LTM cell switch procedure and directly attempts a RACH-based LTM procedure. In other words, there is no “fall back” from RACH-less LTM to RACH-based LTM and there is only RACH-less LTM “initiation” failure.

Observation 4.	RACH-less LTM cell switch execution may fail (i.e., T304 may expire) because UE couldn’t determine that the network has successfully received its first UL data after RACH-less LTM cell switch execution

Proposal 7:	RAN3 to consider the following scenarios:

a.	RACH-less LTM “initiation” failure (e.g., due to TA = FFF received in CSC or unsuccessful UE-based TA measurement) resulting in UE triggering RACH-based LTM procedure

b.	RACH-less LTM execution failure

i.	followed by successful RACH-based LTM recovery

ii.	followed by RACH-based LTM recovery failure

Proposal 8:	RAN3 should discuss solutions on how to optimize the target beam(s) (indicated via TCI state(s)) in LTM cell switch command MAC CE to UE for LTM execution.

Proposal 9:	RAN3 should discuss scenarios when a UE is configured with both LTM and CHO and whether any enhancements are needed to enable joint MRO for L3 HO and LTM e.g., indicate to gNB that CHO was configured before LTM was executed (or vice-versa)

2.2 MRO for CHO with candidate SCG

Proposal 10:	Candidate MN(s) should be only provided with a subset of information from RLF Report (not the complete RLF Report) for optimizing CHO with candidate configuration (e.g., identifier of the candidate PCells/PSCells which met the configured CHO/CPC execution conditions before the RLF is encountered). RAN3 should discuss which information is useful at the candidate MN(s)

Proposal 11:	RAN3 should discuss how the node retrieving the RLF Report should forward the “failure event” to the candidate MN(s)

a.	Alt 1: Node retrieving RLF Report  source MN  candidate MN(s)

b.	Alt 2: Node retrieving RLF Report  source MN and Node retrieving RLF Report  candidate MN(s)

Proposal 12:	Serving MN should forward only a subset of information from SCGFailureInformation Report (not the complete SCGFailureInformation) to candidate MN(s) for optimizing CHO with candidate configuration. RAN3 should discuss which information is useful at the Candidate MN(s) and to be forwarded.

Proposal 13:	There is no need to support concurrent failure cases (RLF + SCG failure) i.e., there is no need to support case 1c, 2c, 3c, 7c, 8c

2.3 MRO for Subsequent CPAC (S-CPAC)

Proposal 14:	Enhance the currently agreed stage-2 text on forwarding mechanism for S-CPAC to also include “SN initiated S-CPAC” scenario.

# 4 Annex 1: MRO for CHO with candidate SCG - Scenarios

| Cases | Figure: CHO with candidate SCG(s) is configured | Brief Description |
| --- | --- | --- |
| Case 1 |  | Failure happens before CHO and/or CPAC execution. 1a) RLF@srcPCell 1b) SCG failure 1c) MCG failure+SCG failure |
| Case 2 |  | CPC/CPA execution condition is met. Failure happens before CHO execution condition is met. 2a) RLF@srcPCell 2b) SCG failure 2c) MCG failure + SCG failure |
| Case 3 |  | CHO execution condition is met. No CHO only or CHO with SCG configuration. Failure happens before CPAC execution condition is met. 3a) RLF@srcPCell 3b) SCG failure 3c) MCG failure + SCG failure |
| Case 4 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. CHO execution to a candidate PCell fails. |
| Case 5 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. RLF shortly after CHO execution success |
| Case 6 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. SCG failure shortly after CHO execution success |
| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| Case 8 |  | Failure happens shortly after CHO and CPAC execution success 8a) RLF@targetPCell 8b) SCG failure 8c) MCG failure + SCG failure |
| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |