3GPP TSG-RAN WG3 Meeting #127	R3-250207

Athens, Greece, 17 – 21 February 2025

Agenda Item:	22.2

Source: 				LG Electronics Inc.

Title:			(TP for TS 38.401) Support of Multi-hop relay

Document for:		Discussion and Decision

1.	Introduction

New Rel-19 WID on NR sidelink multi-hop relay was agreed at RAN #104 [1]. One of the objectives in this WID is to specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay as follows:

… The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3] - Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused. …

In this contribution, we will discuss the RAN3 impacts on mechanisms to support the multi-hop relay, and provide our view on it.

2.	Discussion

In RAN2#128 meeting, the following way forward was agreed in [2]:

Agreement in RAN2#128:

Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

1. Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell.

2. Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment.

All agreements in this WI apply to both these cases unless otherwise specified.

In the control plane procedure, when the Remote UE goes to RRC_CONNECTED, all the Intermediate Relay UEs must be in RRC_CONNECTED. That is, the Intermediate Relay UE in RRC_IDLE/RRC_INACTIVE needs to enter into RRC_CONNECTED before the connection establishment for the Remote UE. This can be supported based on the existing Rel-17 U2N procedure. According to RAN2 discussion, the figure and description under P1 of R2-2410006 [3] serves as the baseline connection establishment procedure as follows:

R2-2410006 [3] Figure 1: Control plane baseline procedure in RAN2 - The U2N Remote UE, First Relay UE, Intermediate Relay UE, and Last Relay UE perform discovery procedure, and establish a PC5-RRC connection between each adjacent UE (U2N Remote UE<->First Relay UE, First Relay UE <-> Intermediate Relay UE, Intermediate Relay UE <-> Last Relay UE) using the NR sidelink PC5 unicast link establishment procedure. [FFS whether to support PC5-RRC connection establishment between some adjacent UEs after transmission of the first RRC message in step 2.] - The L2 U2N Remote UE sends the first RRC message (i.e., RRCSetupRequest) for its connection establishment with gNB via the First Relay UE, using a specified PC5 Relay RLC channel configuration. The first Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. If the First Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment via the Intermediate Relay UE (using similar actions as a U2N Remote UE) upon reception of a message from U2N Remote UE on the specified PC5 Relay RLC channel. The Intermediate Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. If the Intermediate Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment via the Last Relay UE (using similar actions as a U2N Remote UE) upon reception of a message from the First Relay UE on the specified PC5 Relay RLC channel. The Last Relay UE sends the SidelinkUEInformationNR message to request for the dedicated configurations required to support the multi-hop relay operation for the U2N Remote UE. If the Last Relay UE is not in RRC_CONNECTED, it needs to do its own Uu RRC connection establishment upon reception of a message from the Intermediate Relay UE on the specified PC5 Relay RLC channel. In each of the previous sub-steps, if a given relay UE and its parent relay UE both need to enter RRC_CONNECTED, the given relay UE cannot do so until the parent relay UE has completed its own RRC connection establishment. The Last Relay UE receives SRB0 relaying Uu Relay RLC channel configuration for the Intermediate Relay UE from gNB. The Intermediate Relay UE receives SRB0 relaying Relay RLC channel configuration for the First Relay UE from gNB. The gNB configures SRB0 (for U2N Remote UE) relaying RLC channel to the first Relay UE. The gNB responds with an RRCSetup message to U2N Remote UE. The RRCSetup message is sent to the U2N Remote UE using SRB0 relaying Last Relay RLC channel over Uu and the specified PC5 Relay RLC channels over each of the PC5 links. [FFS whether the Last Relay UE can send SUI on behalf of all other relay UEs.] - The gNB, Last Relay UE, Intermediate Relay UE and First Relay UE perform relaying channel setup procedure over Uu. According to the configuration from the gNB, the First Relay/U2N Remote UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the U2N Remote UE/First Relay UE over PC5, the Intermediate Relay/First Relay UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the First Relay UE/Intermediate Relay UE over PC5 and the Last Relay UE/ Intermediate Relay UE establishes a PC5 Relay RLC channel for relaying of SRB1 towards the Intermediate Relay UE/Last Relay UE over PC5. [FFS if each relay UE can establish RLC channel for relaying of SRB1 at the same time as its connection establishment in step 2]. - The RRCSetupComplete message is sent by the U2N Remote UE to the gNB via the First Relay UE, Intermediate Relay UE and the Last Relay UE using SRB1 relaying channels over PC5 and SRB1 relaying channel configured to the Last Relay UE over Uu. Then the U2N Remote UE is RRC_CONNECTED with the gNB. - The L2 U2N Remote UE and gNB establish security following the Uu security mode procedure and the security messages are forwarded through the First Relay UE, Intermediate Relay UE, and Last Relay UE. - The gNB sends an RRCReconfiguration message to the U2N Remote UE via the Last Relay UE, Intermediate Relay UE, and First Relay UE to setup the end-to-end SRB2/DRBs of the U2N Remote UE. The U2N Remote UE sends an RRCReconfigurationComplete message to the gNB via the First Relay UE, Intermediate Relay UE, and Last Relay UE as a response. In addition, the gNB may configure additional Uu Relay RLC channels between the gNB and Last Relay UE, and PC5 Relay RLC channels between each of the Intermediate Relay UE, First Relay UE, and U2N Remote UE for the relaying traffic.

As shown in Figure 1, the Intermediate Relay UE in RRC_IDLE/RRC_INACTIVE performs its own RRC connection establishment as another Remote UE using the Rel-17 U2N procedure in Figure 16.12.5.1-1 of TS 38.300. From RAN3’s point of view, the existing Remote UE Initial Access procedure in Figure 8.19.1-1 of TS 38.401 can be also used as a baseline to support the Multi-hop relay.

Proposal 1: The existing Remote UE Initial Access procedure in Figure 8.19.1-1 of TS 38.401 can be used as a baseline to support the Multi-hop relay.

As shown in Figure 2, suppose that the Remote UE is connected to the gNB via First Relay UE, Intermediate Relay UE, and Last Relay UE. When the First Relay UE is in RRC_IDLE/RRC_INACTIVE state, it withholds the RRC message from the Remote UE and then initiates its own connection establishment procedure. Therefore, the First Relay UE acting as another Remote UE sends its own RRCSetupRequest message to the Intermediate Relay UE. When the Intermediate Relay UE is in RRC_IDLE/RRC_INACTIVE state, it also withholds the RRC message for the First Relay UE and then initiates the existing procedure in Figure 8.19.1-1 for its state transition into RRC_CONNECTED.

Figure 2: Overall procedure for Remote UE’s initial access via multi-hop relay

Then, after Step 17, the First Relay UE in RRC_CONNECTED can send its own SUI message to request for the dedicated configurations required to support the multi-hop relay operation for the Remote UE. Upon reception of the SUI message, the gNB-CU allocates the local ID of the Remote UE. Whether the existing local ID format can be reused is pending to RAN2 decision.

Then, the gNB-CU initiates the procedure to establish the PC5/Uu Relay RLC channel(s) for the transmission of Remote UE’s SRB0 to the gNB-DU. We think that the existing F1AP UE Context Setup and UE Context Modification procedure can be used as a baseline. For example, the gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message of the Last Relay UE to gNB-DU to establish the PC5/Uu Relay RLC channel(s) for relaying of Remote UE’s SRB0. In Steps 21 and 22, the gNB-CU also sends the UE CONTEXT MODIFICATION REQUEST message of the Intermediate Relay UE and the First Relay UE to gNB-DU to establish the PC5 Relay RLC channel(s) for relaying of Remote UE’s SRB0.

Proposal 2: The existing F1AP UE Context Setup and UE Context Modification procedure can be used as a baseline to establish the PC5/Uu Relay RLC channel(s) for relaying of Remote UE’s SRB/DRB.

Proposal 2a: Whether the existing local ID format can be reused is pending to RAN2 decision.

However, when establishing the PC5 Relay RLC channel, there is the difference between Rel-17 U2N relay and Rel-19 Multi-hop relay. In Rel-17 U2N relay, the gNB-DU only configures the PC5 Relay RLC channel towards the Remote UE/Relay UE to the Relay UE/Remote UE. However, in Rel-19 Multi-hop relay, the First Relay UE establishes the PC5 Relay RLC channel towards the Remote UE and Intermediate Relay UE over PC5, respectively. Therefore, the gNB-DU needs to know the established PC5 Relay RLC Channel belongs to which PC5 connection (i.e., between Remote UE and First Relay UE, or between Intermediate Relay UE and First Relay UE). The gNB-CU is already able to provide the Peer UE ID IE to the gNB-DU for Rel-18 U2U Relay operation. We think that since the gNB-CU can have the whole path information for Remote UE as discussed in Proposal 7, the Peer UE related information needs to be provided to the gNB-DU to establish the PC5 Relay RLC channel for Rel-19 Multi-hop relay.

Proposal 3: The Peer UE related information needs to be provided to the gNB-DU to generate the PC5 Relay RLC channel for Rel-19 Multi-hop relay.

After receiving the local ID of the Remote UE and the PC5/Uu Relay RLC channel(s) configuration and bearer mapping for relaying of Remote UE’s SRB0, the First Relay UE sends the RRCSetupRequest message for the Remote UE to the gNB-DU via the Intermediate Relay UE and Last Relay UE. The gNB-DU delivers the RRC message to the gNB-CU in F1AP INITIAL UL RRC MESSAGE TRANSFER message. In Rel-17 U2N Relay, the gNB-DU also includes the local ID of the Remote UE, the gNB-DU UE F1AP ID of the Relay UE and the sidelink configuration container for at least the PC5 Relay RLC channel configuration for relaying of Remote UE’s SRB1. In the Multi-hop Relay, a similar information can be included in the INITIAL UL RRC MESSAGE TRANSFER message. For example, since it was already agreed in RAN2 that Remote UE ID and BEARER ID are included in Rel-19 SRAP header for multi-hop relay operation, the gNB-DU can include the local ID for Remote UE in Step 24. Also, since the gNB-DU at least knows that the RRC message is transferred from the Last Relay UE, the gNB-DU UE F1AP ID of the Last Relay UE can be included.

In Rel-17 U2N Relay, the sidelink configuration container in the INITIAL UL RRC MESSAGE TRANSFER message is the configuration for the PC5 Relay RLC channel between Remote UE and Relay UE for relaying of Remote UE’s SRB1. In Rel-19 Multi-hop Relay, our understanding is that for relaying of Remote UE’s SRB1, this information includes the PC5 Relay RLC channel between First Relay UE and Remote UE in Figure 2. However, in order for the gNB-DU to include this information in the INITIAL UL RRC MESSAGE TRANSFER message, it needs to know the Remote UE is connected to the gNB via multi-hop relay. Without a priori knowledge, the gNB-DU is difficult to differentiate the RRCSetupRequest message from Remote UE#1 with that from Remote UE#4 as shown in Figure 3. For example, even though the RRCSetupRequest message (for Remote UE#1) is delivered via First Relay UE, the Intermediate Relay UE, and the Last Relay UE, the gNB-DU may allocate the PC5 Relay RLC channel for single-hop relay operation, thus causing misconfiguration of the PC5 Relay RLC channel for Remote UE#1’s SRB1. If RAN2 introduces a new Remote UE local ID or SRAP header for multi-hop relay operation, it is natural for the gNB-DU to know that the Remote UE is connected to the gNB via multi-hop relay.

Figure 3: Example of coexistence of single hop relay operation and multi-hop relay operation

From the gNB-CU’s point of view, it already knows that the Intermediate Relay UE in RRC_CONNECTED is connected to the gNB via the Last Relay UE. Upon reception of the SUI message from the Intermediate Relay UE in Step 5, the gNB-CU becomes aware that the First Relay UE is connected to the gNB via the Intermediate Relay UE and the Last Relay UE. Similarly, when the First Relay UE in RRC_CONNECTED sends its own SUI message to the gNB-CU, the gNB-CU becomes aware that the Remote UE is connected to the gNB via the First Relay UE, the Intermediate Relay UE and the Last Relay UE. Therefore, alternative way to inform the gNB-DU of the multi-hop relay operation for Remote UE is to provide the Remote UE local ID to the gNB-DU in Figure 2.

Proposal 4: The gNB-DU informs the gNB-CU of the multi-hop relay related information including at least gNB-DU UE F1AP ID of the Last Relay UE and Remote UE Local ID. Whether the gNB-DU UE F1AP IDs of the First Relay UE and Intermediate Relay UE(s) are included is FFS.

Proposal 5: In Rel-19 Multi-hop Relay, the sidelink configuration container includes the PC5 Relay RLC channel between Remote UE and the First Relay UE for relaying of Remote UE’s SRB1.

Proposal 6: When receiving the RRCSetupRequest message from Remote UE, the gNB-DU needs to know that the Remote UE is connected to the gNB via multi-hop relay.

Proposal 7: The gNB-CU can realize the whole path for Remote UE when entering all the Relay UEs into RRC_CONNECTED.

In Rel-17 U2N Relay, the gNB can configure the PC5/Uu Relay RLC channel(s) for relaying of Remote UE’s SRB1 to the Relay UE before sending the RRCSetup message to the Remote UE. In [4], RAN2 has the on-going discussion on whether the principle in Rel-17 U2N Relay is also applied to the Rel-19 Multi-hop Relay. Therefore, RAN3 needs to wait for RAN2 progress on this issue.

Proposal 8: RAN3 waits for RAN2 progress on whether the gNB can configure the PC5/Uu Relay RLC channel(s) for relaying of Remote UE’s SRB1 to the Relay UE(s) before sending the RRCSetup message to the Remote UE.

In order to capture the above observation and proposals, the following proposal is also suggested to RAN3:

Proposal 9: It is proposed to agree the corresponding TPs in Appendix.

3.	Conclusion

In this contribution, we focused on the RAN3 impacts on mechanisms to support the multi-hop relay, and then provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: The existing Remote UE Initial Access procedure in Figure 8.19.1-1 of TS 38.401 can be used as a baseline to support the Multi-hop relay.

Proposal 2: The existing F1AP UE Context Setup and UE Context Modification procedure can be used as a baseline to establish the PC5/Uu Relay RLC channel(s) for relaying of Remote UE’s SRB/DRB.

Proposal 2a: Whether the existing local ID format can be reused is pending to RAN2 decision.

Proposal 3: The Peer UE related information needs to be provided to the gNB-DU to generate the PC5 Relay RLC channel for Rel-19 Multi-hop relay.

Proposal 4: The gNB-DU informs the gNB-CU of the multi-hop relay related information including at least gNB-DU UE F1AP ID of the Last Relay UE and Remote UE Local ID. Whether the gNB-DU UE F1AP IDs of the First Relay UE and Intermediate Relay UE(s) are included is FFS.

Proposal 5: In Rel-19 Multi-hop Relay, the sidelink configuration container includes the PC5 Relay RLC channel between Remote UE and the First Relay UE for relaying of Remote UE’s SRB1.

Proposal 6: When receiving the RRCSetupRequest message from Remote UE, the gNB-DU needs to know that the Remote UE is connected to the gNB via multi-hop relay.

Proposal 7: The gNB-CU can realize the whole path for Remote UE when entering all the Relay UEs into RRC_CONNECTED.

Proposal 8: RAN3 waits for RAN2 progress on whether the gNB can configure the PC5/Uu Relay RLC channel(s) for relaying of Remote UE’s SRB1 to the Relay UE(s) before sending the RRCSetup message to the Remote UE.

Proposal 9: It is proposed to agree the corresponding TPs in Appendix.

4.	Reference

- RP-242349, “Revised WID on NR sidelink multi-hop relay,” LG Electronics, InterDigital, FirstNet, September, 2024.

- R2-24xxxxx, “Report from session on positioning and sidelink relay,” Session Chair (MediaTek), November, 2024.

- R2-2410006, “Report of [POST127][402][Relay] Multi-hop relay control plane,” InterDigital, November, 2024.

- R2-25xxxxx, “Report of [POST128][401][Relay] Control Plane Baseline Solution,” InterDigital, February, 2025.

5.	Appendix: TP for TS 38.401

This appendix provides the Text proposal for TS 38.401 based on the proposals of this contribution.

----------------Start of the First Change---------------

8.xx	Overall procedures for Multi-hop Layer-2 UE-to-Network Relay

8.xx.1	Remote UE initial access

The signalling flow for Remote UE Initial access is shown in Figure 8.xx.1-1.

Figure 8.xx.1-1: Overall procedure for Remote UE’s initial access via multi-hop relay

1.	The U2N Remote UE, the U2N First Relay UE, the Intermediate Relay UE, and the U2N Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure.

2.	The U2N Remote UE sends an RRCSetupRequest message to the U2N First Relay UE via PC5 Relay RLC Channel.

3.	The U2N First Relay UE withholds the received RRC message. If the U2N First Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should send its own RRCSetupRequest message to the U2N Intermediate Relay UE via PC5 Relay RLC Channel in order to trigger the RRC establishment/resume procedure to enter RRC_CONNECTED state upon reception of the RRC message from the U2N Remote UE. If the U2N First Relay is in RRC_CONNECTED state, Steps 3-17 could be skipped.

4.	The U2N Intermediate Relay UE withholds the RRC message from the U2N First Relay UE. If the U2N Intermediate Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should trigger the RRC establishment/resume procedure in clause 8.19.1 to enter RRC_CONNECTED state upon reception of the RRC message. If the U2N Intermediate Relay is in RRC_CONNECTED state, this step could be skipped.

5.	The U2N Intermediate Relay UE in RRC_CONNECTED state sends the SidelinkUEInformationNR message to the gNB-DU via the U2N Last Relay.

6.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the U2N Intermediate Relay UE by encapsulating the SidelinkUEInformationNR message to gNB-CU, and gNB-CU allocates the local ID of U2N First Relay UE.

7.	The gNB-CU configures the U2N Last Relay UE with the local ID allocated to the U2N First Relay UE, PC5 Relay RLC channel, Uu Relay RLC channel and bearer mapping for relaying of U2N First Relay UE’s SRB0. According to the configuration from gNB-CU, the U2N Last Relay UE establishes a PC5 Relay RLC channel for relaying of U2N First Relay UE’s SRB0 over PC5 and establishes a Uu Relay RLC channel for relaying of U2N First Relay UE’s SRB0 towards gNB-DU.

Editor’s Note: FFS whether the Peer UE related information can be provided to the gNB-DU.

8.	The gNB-CU configures the U2N Intermediate Relay UE with the local ID allocated to the U2N First Relay UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N First Relay UE’s SRB0. According to the configuration from gNB-CU, the U2N Intermediate Relay UE establishes a PC5 Relay RLC channel for relaying of U2N First Relay UE’s SRB0 over PC5.

9.	After receiving the local ID of the U2N First Relay UE and the PC5 Relay RLC channel(s) configuration and bearer mapping for relaying of U2N First Relay UE’s SRB0, the U2N Intermediate Relay UE sends the RRCSetupRequest message of the U2N First Relay UE to gNB-DU via the U2N Last Relay UE. The local ID of the U2N First Relay UE and RB ID for SRB0 are conveyed in the SRAP header.

10.	The gNB-DU allocates a C-RNTI and a gNB-DU UE F1AP ID for the U2N First Relay UE and sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCSetupRequest message of the U2N First Relay UE. In addition, the local ID of the U2N First Relay UE, the gNB-DU UE F1AP ID of the U2N Last Relay UE and the sidelink configuration container for the PC5 Relay RLC channel configuration between U2N First Relay UE and U2N Intermediate Relay UE for relaying of U2N First Relay UE’s SRB1 are included in the INITIAL UL RRC MESSAGE TRANSFER message.

Editor’s Note: FFS whether the gNB-DU UE F1AP ID of the U2N Intermediate Relay UE is included in the INITIAL UL RRC MESSAGE TRANSFER message.

Editor’s Note: FFS how the gNB-DU becomes aware that the First Relay UE is connected to the gNB-DU via the U2N Intermediate Relay UE and U2N Last Relay UE.

11.	The gNB-CU allocates a gNB-CU UE F1AP ID for the U2N First Relay UE and generates a RRCSetup message towards the U2N First Relay UE. The RRC message is encapsulated in the DL RRC MESSAGE TRANSFER message, and includes the configurations of PC5 Relay RLC channel and bearer mapping at least for the transmission of U2N First Relay UE’s SRB1.

12.	The gNB-DU sends the RRCSetup message to the U2N First Relay UE via the U2N Intermediate Relay UE and U2N Last Relay UE.

13.	The gNB-CU configures the U2N Last Relay UE with PC5 Relay RLC channel, Uu Relay RLC channel and bearer mapping for relaying of U2N First Relay UE’s SRB1. According to the configuration from gNB-CU, the U2N Last Relay UE establishes a PC5 Relay RLC channel for relaying of U2N First Relay UE’s SRB1 over PC5 and establishes a Uu Relay RLC channel for relaying of U2N First Relay UE’s SRB1 towards gNB-DU if not configured yet.

14.	The gNB-CU configures the U2N Intermediate Relay UE with PC5 Relay RLC channel and bearer mapping for relaying of U2N First Relay UE’s SRB1. According to the configuration from gNB-CU, the U2N Intermediate Relay UE establishes a PC5 Relay RLC channel for relaying of U2N First Relay UE’s SRB1 over PC5 if not configured yet.

Editor’s Note: FFS whether Step 13 and 14 can be performed earlier, e.g., via Step 7 and 8.

15.	The U2N First Relay UE sends the RRCSetupComplete message to the gNB-DU via the U2N Intermediate Relay UE and U2N Last Relay UE.

16.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

17.	The remaining part of the procedure follows the steps 18-30 of the Remote UE Initial Access procedure defined in clause 8.19.1.

18-32.	The steps 5 to 17 are repeated, where the U2N First Relay UE in RRC_CONNECTED state sends the SidelinkUEInformationNR message to the gNB-DU via the U2N Intermediate Relay UE and Last Relay UE.

-----------------End of the First Change---------------