3GPP TSG-RAN WG3 Meeting #127	R3-250232

Athens, Greece, 17th -21st Feburary,2025

Agenda Item:	12.2

Source:	Huawei

Title:	(TPs for TS 38.413/38.423) Architecture, Access Control and Additional ULI for WAB

Document for:	Discussion

1	Introduction

In the last meeting, the following agreements were achieved for the user location information report and the access control during handover of Rel-19 WAB [1].

WA: Additional ULI for WAB consists of TAI and NR CGI, which are determined by the WAB-node, reflecting the WAB-node’s physical location. This solution is compliant with Opton1 and Option3. It is up to SA2 to support one of Opton1 and Option3 or both.

It is FFS whether there are issues with applying the solution to MT access via NTN.

It is FFS whether further information needs to be included in the additional ULI

For multi-hop prevention, we have the following solutions:

- For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells.

- For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling.

- For initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

Agree on Solution 2: For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling.

To be further discussed whether Solution 1 or Solution 3 (or both) need to be selected

1. For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells.

3. For initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

In this paper, we further analyse the scenario, architecture, access control and the user location information report for WAB.

2	Discussion

2.1	Scenario and architecture of WAB

2.1.1	Architecture: Management of Xn interface for WAB

The WID objective includes “Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs” [2]. So, there are two issues to be discussed.

2.1.1.1 How to manage Xn interface efficiently for WAB-gNB

The mobility of the WAB-node results in dynamic changes to the Xn interface of the WAB-gNB. The WAB-gNB may need to establish new Xn interface to the newly appeared neighbour gNBs frequently. One issue here is how can the WAB-gNB acquire the TNL address of the neighbour gNBs. In legacy scenario, the TNL address of the neighbour gNBs is either configured by OAM or obtained via the Xn-C TNL address discovery procedure. The frequent update to the Xn interface will result in large overhead for these interactions. Moreover, the delay of such interactions will be another challenging factor especially considering all the transmission should be via MT’s PDU session.

So, we need to think about a more efficient way for the WAB-gNB to establish Xn interface towards neighboring gNBs (including BH-gNB). As an alternative, the BH-gNB can provide assistant information about its own neighboring gNBs (e.g., including its neighbour gNBs' TNL information) to the WAB-node.

Proposal 1: The BH-gNB can provide the TNL information of neighbour gNBs to the WAB node.

2.1.1.2 How to avoid setting up Xn between WAB-gNBs

Another issue is how to avoid establishing Xn between two WAB-nodes. The essential thing is the WAB-node should be aware of the node type of another WAB-gNB, for either to determine not initializing Xn setup towards a WAB-gNB, or to reject the Xn setup in case the Xn setup request is received from a WAB-gNB. It is proposed:

Proposal 2: To avoid establishing Xn between two WAB-nodes, the WAB-node should be aware of the node type of another WAB-gNB.

2.1.2	Scenario: in-band or out-band deployment

During SI phase, both in-band deployment and out-band deployment were considered for WAB, and the in-band scenario was not excluded by the WI. Thus, one issue here is how to determine the in-band/out-band operational mode for the WAB node. According to TR 38.799 [3], after the WAB-MT setup, OAM provides the WAB-gNB the initial configuration including the operating frequency during the WAB-gNB initialization phase. On this stage the OAM may determine the in-band/out-band mode and configure the WAB-gNB frequency accordingly. Alternatively, the OAM may preconfigure the in-band/out-band mode and the WAB-gNB frequency before the WAB-MT setup. It is proposed:

- Proposal 3: OAM configures the inband/outband mode to WAB-node.

The in-band mode and out-band mode have different requirements for the BH-RAN.

For out-band, the collocated WAB-gNB and WAB-MT use different frequency band, so there is no mutal interference and no resource coordination is needed between the collocated WAB-gNB and WAB-MT.

But for in-band, on the contrary, the resources used by the collocated WAB-gNB and WAB-MT must be coordinated. The BH-RAN node must be an upgraded gNB to be aware of which WAB-gNB and which WAB-MT are collocated, and implement the resource multiplexing.

Therefore, either the WAB-MT needs to select a proper BH-gNB based on the WAB-gNB’s working frequency and the BH-gNB resource multiplexing capability, or the OAM configures the WAB-gNB’s working frequency and WAB operation mode considering the BH-gNB resource multiplexing capability and WAB-MT’s frequency. Specifically:

- Option 1: The WAB selects a proper BH-gNB. The WAB-MT first accesses the network using the legacy RSRP criterion. After the OAM configures the WAB-gNB frequency and the WAB node’s operation mode, the WAB node verifies whether the BH-gNB is proper. If the BH-gNB is inproper, the WAB node should detach and reselect another proper BH-gNB. In this option, if the WAB operation mode and the WAB-gNB frequency are pre-configured before the WAB-MT accessing, the WAB-MT can direcly select a proper BH-gNB. The proper BH-gNB in this option represents the gNB using different frequency than the WAB-gNB, or using the same frequency as the WAB-gNB and supporting the resource multiplexing.

- Option 2: OAM configers the WAB-gNB frequency. In this option, OAM should configure a frequency different from that of the co-located WAB-MT for the WAB-gNB if the BH-gNB does not support resource multiplexing. Conversely, if the BH-gNB supports resource multiplexing, OAM could configure the same frequency for the WAB-gNB as the co-located WAB-MT, allowing the WAB node to operate in in-band mode. In such option, the work mode of the WAB-node can be flexibly configured and the WAB-MT does not need to detach the BH-gNB.

It is proposed:

Proposal 4: To support the in-band WAB deployment, the BH-RAN must be upgraded to aware of WAB and can support the resource multiplexing. RAN3 to discuss how to ensure the in-band WAB selects a proper BH-gNB, or how can the OAM configure the WAB-gNB frequency considering the BH-gNB’s resource multiplexing capability and frequency.

2.1.3	Scenario: NTN backhaul

According to the NOTE 5 in the WID, NTN backhaul is considered between the WAB-MT and the BH-gNB. According to TS 23.501 [5] the UE’s 5QI is set to a fixed value (5QI=10) as long as the UE is served by NTN, with the PDB directly set to a really large value (1100ms).

Fig 1. Special QoS parameters for NTN UE in TS 23.501

In the legacy NTN scenario, according to the TS 38.413 [4], the gNB will provide the RAT Information which indicates the NR satellite RAT Type to AMF during the NG Setup procedure or the RAN Configuration Update procedure as follows.

-------------Excerpted from TS 38.413-------------

9.3.1.125	RAT Information

This IE provides RAT related information associated with a TAC, used as described in TS 23.501 [5].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAT Information | M |  | ENUMERATED (unlicensed, nb-IoT, ..., nR-LEO, nR-MEO, nR-GEO, nR-OTHERSAT) |  |

In the WAB scenario, if the backhaul link uses NTN, the UE’s CN is naturally unaware of that according to the current RAT information report as shown above, because the WAB-gNB itself is not an NTN gNB. But if the WAB-MT adopts NTN backhaul, the served UE’s QoS parameters cannot be normally configured but should be configured considering the bottleneck of NTN backhaul. The UE’s CN needs to know the backhaul is via NTN and configures the NTN related QoS parameters for UE’s QoS flow. Specifically, the WAB-gNB can inform the UE’s CN the backhaul link type is NTN, otherwise the WAB-MT’s CN informs that to the UE’s CN which may need interactions between the UE AMF and MT AMF. Therefore, it is proposed:

Proposal 5:	If the backhaul is NTN link, RAN3 to discuss whether the WAB-gNB or the WAB-MT’s CN informs UE’s CN that the BH link type is NTN.

2.2	Access control of WAB

2.2.1	Prevent multi-hop access

According to the agreements made in the last meeting, RAN3 reached a conclusion “the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling for handover”. A TP for 38.423 to capture such agreement is provided in Annex B. Two remaining solutions to prevent multi-hop access during initial access are still under discussion. We provide our views regarding each solution as follows.

- Solution 1: The WAB-gNB uses dedicated frequencies and/or PCIs.

Solution 1 is up to operator’s implementation, and can guarantee the WAB-MT will never camp on a WAB-gNB cell.  However, the usage of solution 1 is quite limited, because it depends on whether the operator has spare frequency bands/reserved PCI space. If not, this solution will impact the existing network because if the WAB dedicated frequency band/PCI has been used by existing gNBs. So, Solution 1 is unfriendly for the operators with resource constraint.

On the other hand, the solution 1 still requires some update to the WAB-MT, since the WAB-MT should consider cells with some special PCI/frequency/other broadcasted parameters as “barred”. In such sense, the PCI/frequency/other broadcasted parameters can be interpreted as an implicit indication of “WAB-node barred”.

Observation 1-1: In option 1, the dedicated frequencies and/or PCIs are actually some kinds of implicit “WAB-node barred” indication.

Observation 1-2:  The usage of Solution 1 is quite limited, because it depends on whether the operator has spare frequency bands/reserved PCI space.

•	Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

For solution 3, WAB-gNB broadcasts a WAB-barred or WAB-cell indication in SIB1. The current 38.331 defines variety of barring IEs for the access control, e.g., cellBarredNTN, cellBarredNES, etc. It is conventional to introduce a new barring bit for the AS access control. This solution can avoid multi-hop topology at early stage (i.e., when the WAB-MT try to access the network), and it allows the BH-gNB which is lack of enough resources for backhaul to bar the WAB-MT.

It is proposed:

Proposal 6: For multi-hop avoidance during WAB-MT initial access, RAN3 to agree that the WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT.

2.2.2	Prevent Self-access

- In the SA2’s LS S2-2407345, they mentioned the following 2 scenarios to be further discussed by RAN:

---------Excerption from S2-2407345----------

Issue-1:

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell)  for below two cases:

a)	MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

b)	MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

- In this section, we will discuss the case b, which was not included our reply LS to SA2 in R3-247909 [6]. First, since the WAB-gNB and WAB-MT should be coordinated for the transmission and receiving operation (e.g., TDM, FDM, SDM, etc), it is not possible for the WAB-MT to consider the cell served by its co-located WAB-gNB as the suitable cell when performing cell selection even if full-duplex is supported since some isolation/interference cancellation needs to be conducted. Otherwise, the transmitting of WAB-gNB and WAB-MT will cause severe interference to each other.

- Second, even if the WAB-MT incidently detects the signal of its co-located WAB-gNB, it is easy to prevent this WAB-MT from accessing this cell based on simple implementation, e.g., the implementation can set restriction that a WAB-MT never access the cell configured to this WAB-node by OAM. On the other hand, the RAN-based broadcasting “WAB-support/WAB-barred” can also avoid such issue.

- Observation 2: With proper multiplexing coordination/isolation, the WAB-MT will not detect signal of cell served by its co-located WAB-gNB. Even if the WAB-MT incidently detects the signalling, it is easy to prevent this WAB-MT from accessing this cell based on simple implementation.

Proposal 7: RAN3 to agree that “MWAB-UE accessing MWAB-gNB belonging to same MWAB” is not an issue to be solved because this can be avoided by proper setting/implementation, and send reply LS to SA2.

2.3	User location information

- In the previous meeting, RAN3 agreed a WA that the additional ULI for WAB consists of TAI and NR CGI, reflecting the WAB node’s physical location, and replied the LS to SA2 in R3-247909 [6]. This solution is compliant with both option 1 and option 3, while excluding the option 2. The 3 additional ULI options are listed here for the ease of reference.

-	Option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM.

-	Option 2: geo-location of the MWAB.

-	Option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN.

- In addition, there are some “to be continued issues” according to the Chair notes of RAN3 #126 meeting:

It is FFS whether there are issues with applying the solution to MT access via NTN.

It is FFS whether further information needs to be included in the additional ULI

- So, RAN3 can continue to discuss whether there are issues with applying this solution to MT access via NTN, due to the special format of cell ID and TAC in NTN ULI.

- As for the cell ID, according to TS 38.300 as exerpted below [7], when the UE access through NTN, the mapped cell ID should be used as part of the ULI instead of the normal cell ID. The gNB constructs the mapped cell ID based on the UE’s location information, and this capability could be preconfigured on the gNB. UE is unaware of the mapped cell ID contained in the ULI.

-------------Excerpted from TS 38.300-------------

16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

- As for the TAI, according to the definition of ULI in TS 38.413 as excerpted below [4], the NR NTN TAI information should replace the TAI IE in NTN. Due to the large coverage area of NTN cells, a single NTN cell may be associated with multiple TACs, so the NR NTN TAI information includes a list of TACs. And if the gNB can determine the accurate TAC based on the UE’s location reported by the UE itself, the UE location derived TAC can be included in the NR NTN TAI information.

-------------Excerpted from TS 38.413-------------

9.3.1.16	User Location Information

This IE is used to provide location information of the UE.

| >NR user location information |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| >>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>TAI | M |  | 9.3.3.11 | This IE is ignored if the NR NTN TAI Information IE is present. | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >>NID | O |  | 9.3.3.42 |  | YES | reject |
| >>NR NTN TAI Information | O |  | 9.3.3.53 |  | YES | ignore |

9.3.3.53	NR NTN TAI Information

This IE contains the TAI information for NR NTN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Serving PLMN | M |  | PLMN Identity 9.3.3.5 | Indicates the UE’s serving PLMN. |
| TAC List in NR NTN |  | 1..<maxnoofTACsinNTN> |  | Includes all TAC(s) broadcast in the cell, for the UE’s serving PLMN. |
| >TAC | M |  | 9.3.3.10 |  |
| UE Location Derived TAC in NR NTN | O |  | TAC 9.3.3.10 | This IE contains TAC information derived from the actual UE location, if available. |

- RAN3 sent LS to SA2 and ask SA2 to decide whether option 1 or option 3 to be adopted, and SA2 has provided their latest conclusion in the CR [9]. Based on SA2’s conclusion, the mapped ULI under the WAB-gNB’s PLMN will be used for inter-PLMN scenarios, and the WAB-MT’s ULI under the BH-PLMN will be used for intra-PLMN scenarios. In other words, option 1 applies for inter-PLMN, while option 3 applies for intra-PLMN. Based onthe SA2 conclusion, we provide our views regarding option 1 and option 3 applying to the NTN BH as follows：

- For option 1 used for inter-PLMN, the ULI of WAB-MT is mapped to the information corresponding the WAB-gNB’s PLMN. If the WAB-gNB’s PLMN has fixed TN coverage in this area, the TAC and cell ID are indicated as the normal format. If the WAB-gNB’s PLMN has NTN coverage in this area, the cell ID should be a mapped cell ID in NTN coverage, and the TAI should reuse the format for NR NTN TAI Information IE which contains a TAC list plus the UE location derived TAC value. But the NTN BH has no impact on the format of the additional ULI. The TAC/Cell ID is firstly mapped from the BH PLMN to the WAB-gNB’s PLMN. The final format of the reported additional ULI depends on whether the WAB-gNB PLMN has TN or NTN coverage here.

- For option 3 used for intra-PLMN, the additional ULI contains the Cell ID and TAC belong to the BH-PLMN. When considering the NTN BH, the additional ULI should include the mapped Cell ID and the NR NTN TAI information of the MT’s serving NTN cell. If applicable, the NR NTN TAI information could include the UE location derived TAC. In other words, the format of NR NTN TAI Information IE can be reused.  The issue here is how could the WAB-node obtain the mapped Cell ID and UE location derived TAC in the WAB-MT’s serving cell. Since the WAB-gNB and BH-gNB belongs the same PLMN, similar as the normal NTN access case captured in 38.300, the mapping may be pre-configured at the WAB-node (e.g., up to operator's policy) or up to implementation.

- Regarding the format of the additional ULI in case of NTN backhaul, we do not see any issue to reuse the mapped value of NR CGI and the format of NR NTN TAI Information, regardless option 1 or option 3 will be adopted.

- Observation 3: For the case of NTN backhauling, the additional ULI can contain the mapped NR CGI and the TAI which reuse the format of NR NTN TAI Information IE.

- Proposal 8:  For the case of NTN backhauling, RAN3 to agree the additional ULI contains the mapped NR CGI, and the TAI which reuse the format of NR NTN TAI Information IE.

- Proposal 9: It is up to pre-configuration or implementation for the WAB-gNB to derive the reported information in additional ULI.

3	Conclusion

In this contribution we discuss the architecture and access control issues in the WAB scenario.

Scenario and architecture

Proposal 1: The BH-gNB can provide the TNL information of neighbour gNBs to the WAB node.

Proposal 2: To avoid establishing Xn between two WAB-nodes, the WAB-node should be aware of the node type of another WAB-gNB.

- Proposal 3: OAM configures the inband/outband mode to WAB-node.

Proposal 4: To support the in-band WAB deployment, the BH-RAN must be upgraded to aware of WAB and can support the resource multiplexing. RAN3 to discuss how to ensure the in-band WAB selects a proper BH-gNB, or how can the OAM configure the WAB-gNB frequency considering the BH-gNB’s resource multiplexing capability and frequency.

Proposal 5: If the backhaul is NTN link, RAN3 to discuss whether the WAB-gNB or the WAB-MT’s CN informs UE’s CN that the BH link type is NTN.

Access control

Observation 1-1: In option 1, the dedicated frequencies and/or PCIs are actually some kinds of implicit “WAB-node barred” indication.

Observation 1-2:  The usage of Solution 1 is quite limited, because it depends on whether the operator has spare frequency bands/reserved PCI space.

Proposal 6: For multi-hop avoidance during WAB-MT initial access, RAN3 to agree that the WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT.

- Observation 2: With proper multiplexing coordination/isolation, the WAB-MT will not detect signal of cell served by its co-located WAB-gNB. Even if the WAB-MT incidently detects the signalling, it is easy to prevent this WAB-MT from accessing this cell based on simple implementation.

Proposal 7: RAN3 to agree that “MWAB-UE accessing MWAB-gNB belonging to same MWAB” is not an issue to be solved because this can be avoided by proper setting/implementation, and send reply LS to SA2.

User location information

- Observation 3: For the case of NTN backhauling, the additional ULI can contain the mapped NR CGI and the TAI which reuse the format of NR NTN TAI Information IE.

- Proposal 8: For the case of NTN backhauling, RAN3 to agree the additional ULI contains the mapped NR CGI, and the TAI which reuse the format of NR NTN TAI Information IE.

- Proposal 9: It is up to pre-configuration or implementation for the WAB-gNB to derive the reported information in additional ULI.

4	References

- RAN3 #126 Chairman notes.

- RP-242359, New WID on additional topological enhancements for NR

- TR 38.799, Study on additional topological enhancements for NR, V19.0.0

- TS 38.413, NG Application Protocol (NGAP), Stage2, V18.3.0

- TS 23.501, System architecture for the 5G System (5GS), Stage 2, V18.5.0

- R3-247909, Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access Control and Additional ULI), RAN3

- TS 38.300, NR and NG-RAN Overall Description, Stage 2, V18.3.0

- TS 38.423, Xn application protocol (XnAP) , V18.3.0

- S2-2501322, Solve the Editor’s Note related to the Additional ULI, SA2

Annex A:	TP for TS 38.413

Start of Change

9.3.1.16	User Location Information

This IE is used to provide location information of the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE User Location Information | M |  |  |  | - |  |
| >E-UTRA user location information |  |  |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.3.1.9 |  | - |  |
| >>TAI | M |  | 9.3.3.11 |  | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >NR user location information |  |  |  |  |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >>TAI | M |  | 9.3.3.11 | This IE is ignored if the NR NTN TAI Information IE is present. | - |  |
| >>Age of Location | O |  | Time Stamp 9.3.1.75 | Indicates the UTC time when the location information was generated. | - |  |
| >>PSCell Information | O |  | NG-RAN CGI 9.3.1.73 |  | YES | ignore |
| >>NID | O |  | 9.3.3.42 |  | YES | reject |
| >>NR NTN TAI Information | O |  | 9.3.3.53 |  | YES | ignore |
| >>Mobile IAB-MT User Location Information | O |  | 9.3.1.260 | Indicates the user location information of a mobile IAB-MT, which is co-located with the mobile IAB-DU which serves the UE. | YES | ignore |
| >>WAB-MT User Location Information | O |  | 9.3.1.X | Indicates the user location information of a WAB-MT, which is co-located with the WAB-gNB which serves the UE. | YES | ignore |
| >N3IWF user location information |  |  |  |  |  |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the N3IWF | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TNGF user location information |  |  |  |  | YES | ignore |
| >>TNAP ID | M |  | OCTET STRING | TNAP Identifier used to identify the TNAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | UE's local IP address used to reach the TNGF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >TWIF user location information |  |  |  |  | YES | ignore |
| >>TWAP ID | M |  | OCTET STRING | TWAP Identifier used to identify the TWAP. Details in TS 29.571 [35]. | - |  |
| >>IP Address | M |  | Transport Layer Address 9.3.2.4 | Non-5G-Capable over WLAN device's local IP address used to reach the TWIF. | - |  |
| >>Port Number | O |  | OCTET STRING (SIZE(2)) | UDP or TCP source port number if NAT is detected. | - |  |
| >>TAI | O |  | 9.3.3.11 |  | YES | ignore |
| >W-AGF user location information |  |  |  | Indicates the location information via wireline access as specified in TS 23.316 [34]. | YES | ignore |
| >>W-AGF user location information | M |  | 9.3.1.164 |  | - |  |

Next Change

9.3.1.X	WAB-MT User Location Information

This IE contains the NR CGI and the TAI for the co-located WAB-MT of the WAB-gNB which serves the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.7 | The NR CGI or mapped NR CGI of the cell, which is the serving cell of the WAB-MT co-located with the WAB-gNB that serves the UE. |
| CHOICE TAI information | M |  |  |  |
| >>TAI |  |  | 9.3.3.11 | The TAI or mapped TAI supported by the cell, which is the serving cell of the WAB-MT co-located with the WAB-gNB which serves the UE. |
| >>NR NTN TAI Information |  |  | 9.3.3.53 | The TAI or mapped TAI supported by the cell, which is the serving cell of the WAB-MT co-located with the WAB-gNB which serves the UE. This IE is present when the WAB-MT access via NTN. |

End of Change

Annex B:	TP for TS 38.423

Start of Change

8.2.1.3	Unsuccessful Operation

Figure 8.2.1.3-1: Handover Preparation, unsuccessful operation

If the target NG-RAN node does not admit at least one PDU session resource, or a failure occurs during the Handover Preparation, the target NG-RAN node shall send the HANDOVER PREPARATION FAILURE message to the source NG-RAN node. The message shall contain the Cause IE with an appropriate value.

If the Conditional Handover Information Request IE is contained in the HANDOVER REQUEST message and the target NG-RAN node rejects the handover or a failure occurs during the Handover Preparation, the target NG-RAN node shall include the Requested Target Cell ID IE in the HANDOVER PREPARATION FAILURE message.

If the WAB specific S-NSSAI is included in the UE Context Information in the HANDOVER REQUEST message, and the target NG-RAN node does not support serving the WAB-node, the target NG-RAN node shall send the HANDOVER PREPARATION FAILURE message to the source NG-RAN node. The message shall contain the Cause IE with the WAB not supported.

Interactions with Handover Cancel procedure:

If there is no response from the target NG-RAN node to the HANDOVER REQUEST message before timer TXnRELOCprep expires in the source NG-RAN node, the source NG-RAN node should cancel the Handover Preparation procedure towards the target NG-RAN node by initiating the Handover Cancel procedure with the appropriate value for the Cause IE. The source NG-RAN node shall ignore any HANDOVER REQUEST ACKNOWLEDGE or HANDOVER PREPARATION FAILURE message received after the initiation of the Handover Cancel procedure and remove any reference and release any resources related to the concerned Xn UE-associated signalling.

Next Change

9.2.3.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the XnAP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED ( Cell not Available, Handover Desirable for Radio Reasons, Handover Target not Allowed, Invalid AMF Set ID, No Radio Resources Available in Target Cell, Partial Handover, Reduce Load in Serving Cell, Resource Optimisation Handover, Time Critical Handover, TXnRELOCoverall Expiry, TXnRELOCprep Expiry, Unknown GUAMI ID, Unknown Local NG-RAN node UE XnAP ID, Inconsistent Remote NG-RAN node UE XnAP ID, Encryption And/Or Integrity Protection Algorithms Not Supported, Multiple PDU Session ID Instances, Unknown PDU Session ID, Unknown QoS Flow ID, Multiple QoS Flow ID Instances, Switch Off Ongoing, Not supported 5QI value, TXnDCoverall Expiry, TXnDCprep Expiry, Action Desirable for Radio Reasons, Reduce Load, Resource Optimisation, Time Critical action, Target not Allowed, No Radio Resources Available, Invalid QoS combination, Encryption Algorithms Not Supported, Procedure cancelled, RRM purpose, Improve User Bit Rate, User Inactivity, Radio Connection With UE Lost, Failure in the Radio Interface Procedure, Bearer Option not Supported, UP integrity protection not possible, UP confidentiality protection not possible, Resources not available for the slice(s), UE Maximum integrity protected data rate reason, CP Integrity Protection Failure, UP Integrity Protection Failure, Slice(s) not supported by NG-RAN, MN Mobility, SN Mobility, Count reaches max value, Unknown Old NG-RAN node UE XnAP ID, PDCP Overload, DRB ID not available, Unspecified, …, UE Context ID not known, Non-relocation of context, CHO-CPC resources to be changed, RSN not available for the UP, NPN access denied, Report Characteristics Empty, Existing Measurement ID, Measurement Temporarily not Available, Measurement not Supported For The Object, UE Power Saving, Not existing NG-RAN node2 Measurement ID, Insufficient UE Capabilities, Normal Release, Value out of allowed range, SCG activation deactivation failure, SCG deactivation failure due to data transmission, SSB not Available, LTM Triggered, No Backhaul Resource, mIAB-node not authorized, IAB not Authorized, WAB not supported) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Hardware Failure, O&M Intervention, Not enough User Plane Processing Resources, Unspecified, …) |  |

End of Change

Annex C:	Reply LS to SA2

3GPP TSG-RAN3 Meeting #127	R3-xxxxxx

Athens, Greece, 17th -21st Feburary,2025

Title:	[DRAFT] Reply LS on access restriction and additional ULI of WAB

Response to:	S2-2407345/ R3-244019

Release:	Rel-19

Work Item:	VMR_Ph2, NR_WAB_5GFemto-Core

Source:	Huawei [to be: RAN3]

To:	SA2

Cc:	RAN2

Contact Person:

Name:	Yuanping Zhu

Tel. Number:

E-mail Address:	zhuyuanping@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	 -

1. Overall Description:

RAN3 thanks SA2 for the LS on FS_VMR_Ph2 (S2-2407345/R3-244019), and for the Issue-1, RAN3 only provide partially reply for issue 1a in R3-247909.

RAN3 further discussed the RAN based approach for preventing multi-hop backhauling and self-acessing at the WAB-MT’s initial access, and provides the following further replies to SA2.

Issue-1:

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases:

a)	MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

b)	MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

RAN3 further Reply to Issue-1:

- RAN3 agree the following RAN-based solution to prevent multi-hop backhauling at the WAB-MT’s initial access: WAB-gNB cells broadcast a new indicator in SIB to bar the WAB-MT.

- For self-access prevention, RAN3 do not think the “MWAB-UE accessing MWAB-gNB belonging to same MWAB” is an issue to be solved, because this can be avoided by proper seting/implementation.

2. Actions:

To SA2:

ACTION: 	RAN3 asks SA2 to take the above feedback into account.

3. Date of Next RAN3 Meetings:

RAN3#127bis		7th Apr – 11th Apr, 2025			Wuhan, CN

RAN3#128			19th May – 23rd May, 2025		Malta, MT