3GPP TSG-RAN3 Meeting #127	R3-250244

Athens, Greece, Feb 17-21, 2025

Agenda Item:	11.3

Source: 	Qualcomm Incorporated

Title: 	Discussion on remaining issues in AI/ML enabled CCO

Document for:	Discussion/Decision

# Background

In RAN3#126 we made some good progress in AI/ML enabled CCO on stage 3 issues.

RAN3#125bis Change WA to agreement: Transferring the predicted CCO issue via the CU configuration update procedure and transferring the Future Coverage State via the DU configuration update procedure over F1. Introducing new IE for Future Coverage Modification List over Xn. Introducing new IE for Future Coverage Modification Notification over F1. Introducing new IE for predicted CCO assistance information over F1. WA: Introduce time information for predicted CCO issue and future CCO state encoded as Requested Prediction Time IE. Time information for Predicted CCO Issue is a relative time from the time of receiving the gNB-CU Configuration Update message over F1. Time information for future CCO state is a relative time from the time of receiving the gNB-DU Configuration Update message over F1 and NG-RAN node Configuration Update message over Xn. FFS on whether to send time information related to coverage modification cause over Xn. Further discussion on whether Value 0 needed for Future Cell Coverage State IE. Time interval for predicted CCO issue and future CCO state can be in the granularity of seconds. Continue to work on the stage2/3 details…

In this paper we discuss our views on remaining stage 3 issues in AI/ML enabled CCO.

# Discussion

## Stage 3 Open Issues

In the last meeting it was agreed to send Future Cell Coverage State IE over F1 from gNB-DU to gNB-CU. One company needed more time to analyse the impacts of Value “0” usage this IE. Value “0” indicates cell will be inactive. After further checking we see no issues including value “0” in the Future Coverage State IE as it is a legacy behaviour as well.

Confirm that value “0” can be included in Future Cell Coverage State IE in F1AP interface.

The associated time information for predicted CCO issue and future CCO state was agreed to be a relative time of certain duration in seconds calculated from the time of receiving the Config Update message over Xn or F1 interface. However the max value of the time information could not be agreed upon in the last meeting. Few companies preferred a shorter duration and few companies wanted more relaxed duration. We think the max duration can be around 5 mins (300 seconds) to allow the node to react and change its coverage.

The associated time information for predicted CCO issue and future CCO state can have a maximum value of 300 seconds (5 mins) to allow the node to react to the issue and modify its coverage.

In the last meeting few companies were proposing to send Time for Predicted CCO issue to neighbouring CU over Xn and neighbouring CU can pass the same information to neighbouring DU. Currently the Predicted CCO issue is passed as Predicted Coverage Modification cause. It is unclear on how the Time for Predicted CCO issue will be used in the neighbouring CU. We are open to discuss the usage of Time for Predicted CCO issue over Xn in the neighbouring CU.

It is unclear on how the Time for Predicted CCO issue will be used in the neighbouring CU. We are open to discuss the usage of “Time for Predicted CCO issue” IE in the neighbouring CU, when sent over Xn interface.

## Future CCO State Feedback

During the SI phase Predicted CCO issue and future CCO state feedback was not discussed due to time constraints. We therefore propose to discuss the feedback for Predicted CCO issue and future CCO state in normative phase.  Once the predicted coverage pattern is applied, the neighboring gNBs can send feedback on Cell performance like throughput, packet loss, packet delay, RLF reports etc to the source gNB to inform the effects of applying predicted coverage pattern. With only UE performance it is unclear which cell the UE belong to (whether it is the cell where the future CCO state was applied or a normal cell). Hence the neighbouring node can aggregate the UE performance at a cell level and send it over Xn or the neighbouring node also reports the NR-CGI where the UE belongs for an UE reporting its performance.

RAN3 to discuss the following options as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn.

- Legacy UE performance metrics (throughput, packet delay, packet loss) including Cell ID where the UE belonged to.

- SON Reports like RLF, RACH report etc

## Future CCO State Assistance Information

A Note was added in the TR 38.743 to further discuss if gNB-CU-CP can generate the future CCO state as assistance information and send it to gNB-DU. On this Note, we are unclear on what are the input parameters which gNB-CU-CP will use to generate future CCO state. Moreover OAM provides the CCO state index configuration to gNB-DU. If gNB-CU-CP needs to generate the future CCO state, OAM also needs to configure gNB-CU-CP with CCO state index values which would involve changes to the configuration model and SA5 impact. This is also not a legacy behaviour. There could also be a race condiciton of CU suggesting a different CCO state and OAM informing DU a different CCO State. Hence we propose, that gNB-DU generates the future CCO state on its own and future CCO state need not be provided as assistance information from gNB-CU-CP.

The input parameters used by gNB-CU-CP to generate future CCO state is unclear. OAM provides the CCO state index configuration to gNB-DU. If gNB-CU-CP needs to generate the future CCO state, OAM also needs to configure gNB-CU-CP with CCO state index values which would involve changes to the configuration model and SA5 impact. Race condition at gNB-DU is also a possibility if CU and OAM suggest different CCO state to gNB-DU.

Future CCO state proposed to be generated by gNB-CU-CP as assistance information to gNB-DU is not needed as it involves changes to OAM to configure CCO index value to gNB-CU-CP similar to that of gNB-DU and duplicating functionality across gNB-CU-CP and gNB-DU and causing race condition.

## Assistance Information for Prediction of CCO Issue

It was agreed the gNB-CU will predict the CCO issue and forward it to the gNB-DU. gNB-CU predicts the CCO issue based on L3 measurements. gNB-DU has L1 measurements and beam level load information. Hence gNB-DU has a better visibility of the beam level information for detecting CCO issue. We therefore propose that gNB-DU can send affected beams based on L1 measurements as assistance information to gNB-CU.

gNB-CU uses L3 measurement information as one of the inputs to predict CCO issue. However gNB-CU lacks insight into the beam level information which is available in the gNB-DU. gNB-DU has better visibility of beam level information based on L1 measurements for detecting CCO issue.

For CCO issue prediction, gNB-DU can send assistance information to gNB-CU on affected beams based on L1 measurements. RAN3 to discuss including “Assistance Information on Affected beams” over F1 as input data for CCO issue prediction.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	The input parameters used by gNB-CU-CP to generate future CCO state is unclear. OAM provides the CCO state index configuration to gNB-DU. If gNB-CU-CP needs to generate the future CCO state, OAM also needs to configure gNB-CU-CP with CCO state index values which would involve changes to the configuration model and SA5 impact. Race condition at gNB-DU is also a possibility if CU and OAM suggest different CCO state to gNB-DU.

Observation 2.	gNB-CU uses L3 measurement information as one of the inputs to predict CCO issue. However gNB-CU lacks insight into the beam level information which is available in the gNB-DU. gNB-DU has better visibility of beam level information based on L1 measurements for detecting CCO issue.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Confirm that value “0” can be included in Future Cell Coverage State IE in F1AP interface.

Proposal 2:	The associated time information for predicted CCO issue and future CCO state can have a maximum value of 300 seconds (5 mins) to allow the node to react to the issue and modify its coverage.

Proposal 3:	It is unclear on how the Time for Predicted CCO issue will be used in the neighbouring CU. We are open to discuss the usage of “Time for Predicted CCO issue” IE in the neighbouring CU, when sent over Xn interface.

Proposal 4:	RAN3 to discuss the following options as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn.

	Legacy UE performance metrics (throughput, packet delay, packet loss) including Cell ID where the UE belonged to.

	SON Reports like RLF, RACH report etc

Proposal 5:	Future CCO state proposed to be generated by gNB-CU-CP as assistance information to gNB-DU is not needed as it involves changes to OAM to configure CCO index value to gNB-CU-CP similar to that of gNB-DU and duplicating functionality across gNB-CU-CP and gNB-DU and causing race condition.

Proposal 6:	For CCO issue prediction, gNB-DU can send assistance information to gNB-CU on affected beams based on L1 measurements. RAN3 to discuss including “Assistance Information on Affected beams” over F1 as input data for CCO issue prediction.