3GPP TSG-RAN WG3 #127	R3-250276

Athens, GR, 17th – 21st Feb 2025

Agenda Item:	11.4

Source:	Lenovo

Title: 		(TP to BLCR 38.473 and 37.483) Discussion on split architecture related issues for Rel18 use cases

Document for:	Discussion and Approval

# Introduction

In this paper, we discuss UE performance collection related to split architecture concerning Rel18 use cases.

# Discussion

## 2.1	UE performance measurement

The existing UE performance IE consists of average UE throughput DL, average UE throughput UL, average packet delay, and average packet loss DL as below:

#### 9.2.3.179 UE Performance This IE indicates the UE performance measurements metrics. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. Average UE Throughput DL; O; Bit Rate 9.2.3.4. Average UE Throughput UL; O; Bit Rate 9.2.3.4. Average Packet Delay; O; 9.2.3.187. Average Packet Loss DL; O; Packet Loss Rate 9.2.3.11.

TS 38.314 #### 4.2.1.2 Packet delay ##### 4.2.1.2.1 General Packet delay includes RAN part of delay and CN part of delay. The RAN part of DL packet delay measurement comprises: - D1 (DL delay in over-the-air interface), referring to Average delay DL air-interface in TS 28.552 [2] 5.1.1.1.1. - D2 (DL delay on gNB-DU), referring to Average delay in RLC sublayer of gNB-DU in TS 28.552 [2] 5.1.3.3.3. - D3 (DL delay on F1-U), referring to Average delay on F1-U in TS 28.552 [2] 5.1.3.3.2. - D4 (DL delay in CU-UP), referring to Average delay DL in CU-UP in TS 28.552 [2] 5.1.3.3.1. The DL packet delay measurements, i.e. D1 (the DL delay in over-the-air interface ), D2 (the DL delay in gNB-DU), D3 (the DL delay on F1-U) and D4 (the DL delay in CU-UP), should be measured per DRB per UE. NOTE: The delay measurements D1, D2 and D4 are also applicable for EUTRA in case of EN-DC related DL delay measurements on the MN side. The RAN part (including UE) of UL packet delay measurement comprises: - D1 (UL PDCP packet average delay, as defined in clause 4.3.1.1). - D2.1 (average over-the-air interface packet delay, as defined in 4.2.1.2.2). - D2.2 (average RLC packet delay, as defined in 4.2.1.2.3). - D2.3 (average delay UL on F1-U, it is measured using the same metric as the average delay DL on F1-U defined in TS 28.552 [2] clause 5.1.3.3.2). - D2.4 (average PDCP re-ordering delay, as defined in 4.2.1.2.4). The UL packet delay measurements, i.e. D1(UL PDCP packet average delay), D2.1(average over-the-air interface packet delay), D2.2(average RLC packet delay), D2.3(average delay UL on F1-U) and D2.4(average PDCP re-ordering delay), should be measured per DRB per UE. The unit of D1, D2.1, D2.2, D2.3 and D2.4 is 0.1ms. NOTE: The delay measurements D1, D2.1, D2.2 and D2.4 are also applicable for EUTRA in case of EN-DC related UL delay measurements on the MN side. 4.2.1.5 Packet Loss Rate 4.2.1.5.1 Packet Uu Loss Rate in the DL per DRB per UE The objective of this measurement is to measure packets that are lost at Uu transmission, for OAM performance observability or for QoS verification of MDT. Protocol Layer: RLC Table 4.2.1.5.1-1: Definition for Packet Uu Loss Rate in the DL per DRB per UE Definition; Uu Packet Loss Rate in the DL per DRB per UE. One packet corresponds to one RLC SDU. The measurement is done separately per DRB. Detailed Definition: M(T,drbid)=Dloss(T,drbid)*1000000N(T,drbid)+Dloss(T,drbid), where explanations can be found in the table 4.2.1.5.1-2 below.. 4.2.1.5.2 Packet Uu Loss Rate with delay threshold in the DL per DRB per UE The objective of this measurement is to measure the DL packets loss including any packets not successfully transmitted or packets successfully received but delayed more than a delay threshold at Uu transmission, for OAM performance observability or for QoS verification of MDT. Protocol Layer: RLC Table 4.2.1.5.2-1: Definition for Packet Uu Loss Rate with delay threshold in the DL per DRB per UE Definition; Uu Packet Loss Rate with delay threshold in the DL per DRB per UE: One packet corresponds to one RLC SDU. The measurement is done separately per DRB. Detailed definition: M_dt(T,drbid)=[DlossT,drbid+DexdT, drbid]*1000000N_dtT,drbid+DlossT,drbid+DexdT, drbid Where explanations can be found in the table 4.2.1.5.2-2 below..

TS 28.558 6.3.1.4 UE throughput 6.3.1.4.1 Average DL UE throughput in gNB a) This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as DL M5 in TS 37.320 [9]. b) DER(N=1) c) This measurement is obtained according to the following formula based on the "ThpVolDl" and "ThpTimeDl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC architecture [16]) and for each supported S-NSSAI. If ThpTimeDl>0, ThpVolDlThpTimeDl×1000 [kbit/s] If ThpTimeDl=0, 0 [kbit/s] For small data bursts, where all buffered data is included in one initial HARQ transmission,, otherwise ThpTimeDl; The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one Data Radio Bearer (DRB) is emptied.. The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE.. The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB.. The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer).. 6.3.1.4.2 Average UL UE throughput in gNB a) This measurement provides the average UE throughput in uplink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC architecture [16]) and subcounters per supported S-NSSAI. This measurement is also referred to as UL M5 in TS 37.320[9]. B) DER(N=1) c) This measurement is obtained according to the following formula based on the "ThpVolUl" and "ThpTimeUl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC architecture [16]) and for each supported S-NSSAI. If ThpTimeUl>0, ThpVolUlThpTimeUl×1000 [kbit/s] If ThpTimeUl=0, 0 [kbit/s] For small data bursts, where all buffered data is included in one initial HARQ transmission otherwise ThpTimeUl; The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeUl" for each time the UL buffer for one Data Radio Bearer (DRB) is emptied.. The point in time when the data up until the second last piece of data in data burst has been successfully received for a particular DRB. The point in time when transmission is started for the first data in data burst for a particular DRB.. The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbit received in UL for one DRB during a sample of ThpTimeUl, (It shall exclude the volume of the last piece of data emptying the buffer)..

In CU and DU split architecture, DU measures the following packet delay per DRB: UL/DL delay in over-the-air interface, UL/DL delay in DU (e.g., RLC delay), DL packet loss, UL/DL throughput.

In CU-CP and CU-UP split architecture, CU-UP measures the UL/DL delay in CU-UP (e.g., PDCP delay)

Besides, the UE performance feedback measurement and report is triggered by the handover event. For split architecture discussion, both intra-gNB (with or without DU/CU-UP change) or inter-gNB handover should be considered.

For UE performance collection in split architecture, both intra-gNB (with or without DU/CU-UP change) and inter-gNB handover should be considered.

Over Xn interface, the UE performance feedback collection is done in two steps:

1. Source gNB configures the target gNB to measure and report required UE performance if the concerned handover happens

2. In the HANDOVER REQUEST message, source gNB includes an indicator that triggers target gNB to measure and report accordingly.

The advantage of above solution in Rel18 discussion is that source gNB does not need to configure the UE performance collection every time there is a concerned UE handed over to the target gNB, which could cause heavy signalling overhead if many UEs are handed over to the target gNB.

UE performance collection over Xn interface is designed in a way to avoid heavy signaling overhead.

In our view, the procedures over E1 and F1 interface can follow the similar design. For example,

- For E1 interface, the CU-CP can configure the CU-UP to measure and report required UE performance via a E1 Data Collection Reporting Initiation procedure if concerned handover happens. Then in the BEARER CONTEXT SETUP REQUEST message (e.g., for inter gNB handover with CU-UP change) or in the BEARER CONTEXT MODIFICATOIN REQUEST message (e.g., for intra gNB handover without CU-UP change), an indicator (e.g., Data Collection ID IE) can be added to trigger the UE performance measurement. The CU-UP will then measure and report the UE performance to CU-CP via a E1 Data Collection Reporting procedure.

- For F1 interface, the CU can configure the DU to measure and report required UE performance via a F1 Data Collection Reporting Initiation procedure if concerned handover happens. Then in the UE CONTEXT SETUP REQUEST message (e.g., for inter gNB handover with DU change) or in the UE CONTEXT MODIFICATOIN REQUEST message (e.g., for intra gNB handover without DU change), an indicator (e.g., Data Collection ID IE) can be added to trigger the UE performance measurement. The DU will then measure and report the UE performance to CU via a F1 Data Collection Reporting procedure.

For E1 interface, RAN3 uses the new DATA COLLECTION REQUEST/RESPONSE/UPDATE messages for CU-CP to configure the CU-UP to measure and report required UE performance per DRB, e.g., UL/DL delay in CU-UP.

For E1 interface, BEARER CONTEXT SETUP/MODIFICATION REQUEST messages are enhanced by including an indicator (e.g., Data Collection ID IE) to trigger the UE performance measurement and report.

For F1 interface, RAN3 uses new DATA COLLECTION REQUEST/RESPONSE/UPDATE messages for CU to configure the DU to measure and report required UE performance per DRB, e.g., UL/DL delay in over-the-air interface, UL/DL delay in DU (e.g., RLC delay), DL packet loss, UL/DL throughput.

For F1 interface, UE CONTEXT SETUP/MODIFICATION REQUEST messages are enhanced by including an indicator (e.g., Data Collection ID IE) to trigger the UE performance measurement and report.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	In CU and DU split architecture, DU measures the following packet delay per DRB: UL/DL delay in over-the-air interface, UL/DL delay in DU (e.g., RLC delay), DL packet loss, UL/DL throughput.

Observation 2	In CU-CP and CU-UP split architecture, CU-UP measures the UL/DL delay in CU-UP (e.g., PDCP delay)

Observation 3	UE performance collection over Xn interface is designed in a way to avoid heavy signaling overhead.

Based on the discussion above, we propose:

Proposal 1	For UE performance collection in split architecture, both intra-gNB (with or without DU/CU-UP change) and inter-gNB handover should be considered.

Proposal 2	For E1 interface, RAN3 uses the new DATA COLLECTION REQUEST/RESPONSE/UPDATE messages for CU-CP to configure the CU-UP to measure and report required UE performance per DRB, e.g., UL/DL delay in CU-UP.

Proposal 3	For E1 interface, BEARER CONTEXT SETUP/MODIFICATION REQUEST messages are enhanced by including an indicator (e.g., Data Collection ID IE) to trigger the UE performance measurement and report.

Proposal 4	For F1 interface, RAN3 uses new DATA COLLECTION REQUEST/RESPONSE/UPDATE messages for CU to configure the DU to measure and report required UE performance per DRB, e.g., UL/DL delay in over-the-air interface, UL/DL delay in DU (e.g., RLC delay), DL packet loss, UL/DL throughput.

Proposal 5	For F1 interface, UE CONTEXT SETUP/MODIFICATION REQUEST messages are enhanced by including an indicator (e.g., Data Collection ID IE) to trigger the UE performance measurement and report.

# 4	TP to 38.473

<<<<<<<<<<<<<<<<<<<<<<<< START OF CHANGE >>>>>>>>>>>>>>>>>>>>

### 8.x	Data Collection Procedures

### 8.x.1	Data Collection Reporting Initiation

#### 8.x.1.1	General

This procedure is used by gNB-CU to request from gNB-DU the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.x.1.2	Successful Operation

Figure 8.x.1.2-1: Data Collection Reporting Initiation, successful operation

gNB-CU initiates the procedure by sending the DATA COLLECTION REQUEST message to gNB-DU to start information reporting or to stop information reporting. Upon receipt, gNB-DU:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If gNB-DU is capable of providing all of the requested information, it shall initiate the information reporting as requested by gNB-CU and respond with the DATA COLLECTION RESPONSE message.

If gNB-DU is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The gNB-DU shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the gNB-DU shall take it into account for the configuration of UE performance collection and reporting. gNB-DU shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE context is released.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects gNB-DU performs measurements or predictions on. gNB-DU shall include in the DATA COLLECTION UPDATE message:

-	the Average Packet Delay IE, if the first bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by GNB-DU.

-	the Energy Cost IE, if the second bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by GNB-DU.

#### 8.x.1.3	Unsuccessful Operation

Figure 8.x.1.3-1: Data Collection Reporting Initiation, unsuccessful operation

If none of the requested information can be initiated, gNB-DU shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.x.1.4	Abnormal Conditions

For the same Measurement ID, if the initiating gNB-CU does not receive either the DATA COLLECTION RESPONSE message or the DATA COLLECTION FAILURE message, the gNB-CU may reinitiate the Data Collection Reporting Initiation procedure towards the same gNB-DU, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message.

If the gNB-DU receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "stop" and if the gNB-DU Measurement ID value received in the DATA COLLECTION REQUEST message is not used, the gNB-DU shall initiate DATA COLLECTION FAILURE message with an appropriate cause value.

If in the Report Characteristics for Data Collection IE bitmap all bits are set to "0" in the DATA COLLECTION REQUEST message, then gNB-DU shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the gNB-DU receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "start" and the GNB-CU Measurement ID IE corresponding to an existing on-going Data Collection reporting, then GNB-DU shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

### 8.x.2	Data Collection Reporting

#### 8.x.2.1	General

This procedure is initiated by gNB-DU to report information accepted by the gNB-DU following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.x.2.2	Successful Operation

Figure 8.4.14.2-1: Data Collection Reporting, successful operation

gNB-DU shall report the accepted information in DATA COLLECTION UPDATE message. The accepted information is the information that was successfully initiated during the preceding Data Collection Reporting Initiation procedure.

#### 8.x.2.3	Unsuccessful Operation

Not applicable.

#### 8.x.2.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| Candidate SpCell List |  | 0..1 |  |  | YES | ignore |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> |  |  | EACH | ignore |
| >>Candidate SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| DRX Cycle | O |  | 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| **** Irrelevant Text Skipped *** |  |  |  |  |  |  |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Data Collection ID | O |  | 9.3.1.z |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N or U2U relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofLTMgNBDUs | Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

<<<<<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| **** Irrelevant Text Skipped *** |  |  |  |  |  |  |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Data Collection ID | O |  | 9.3.1.z |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying or L2 N3C relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channel allowed for L2 U2N or U2U relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofSLdestinations | Maximum number of destination for NR sidelink communication, the maximum value is 32 |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

| Condition | Explanation |
| --- | --- |
| ifCHOcancel | This IE may be present if the CHO Trigger IE is present and set to "CHO-cancel". |

<<<<<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

### 9.2.x	Data Collection Messages

#### 9.2.x.1	DATA COLLECTION REQUEST

This message is sent by gNB-CU to gNB-DU to initiate the requested information reporting according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the GNB-DU is requested to report. First Bit = Average Packet Delay, Second Bit = Energy Cost. Other bits are ignored by the gNB-DU. | YES | reject |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.3.1.x |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

#### 9.2.x.2	DATA COLLECTION RESPONSE

This message is sent by gNB-DU to gNB-CU to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the GNB-DU. First Bit = Energy Cost, Second Bit = Average. Other bits are ignored by the gNB-CU. | – |  |
| >>Cause | M |  | 9.3.1.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedCellMeasObjects | Maximum number of measurement objects that can fail per cell. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

#### 9.2.x.3	DATA COLLECTION FAILURE

This message is sent by the gNB-DU to gNB-CU to indicate that for all of the requested objects the reporting cannot be initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by GNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by GNB-DU | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by gNB-DU to gNB-CU to report the requested information.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | reject |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | gNB-CU UE F1AP ID 9.3.1.4 | gNB-CU UE F1AP allocated by gNB-CU. | – |  |
| >>UE Performance | O |  | 9.3.1.y |  | – |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |

#### 9.3.1.x	UE Performance Collection Configuration

This IE indicates the configuration for UE performance measurement collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond |

#### 9.3.1.y	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average Packet Delay | O |  | 9.2.3.187 |  |
| Average Packet Loss DL | O |  | INTEGER (0.. 10000) | Defined in TS 38.314 [42]. Unit: 0.1 millisecond |

#### 9.3.1.z	Data Collection ID

This IE indicates the gNB-CU and gNB-DU Measurement IDs which identify a Data Collection Reporting context.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Together with gNB-DU Measurement ID, identifies a Data Collection Reporting context. |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Together with gNB-CU Measurement ID, identifies a Data Collection Reporting context. |

<<<<<<<<<<<<<<<<<<<<<<<< END OF CHANGE >>>>>>>>>>>>>>>>>>>>

# 5	TP to 37.483

<<<<<<<<<<<<<<<<<<<<<<<< START OF CHANGE >>>>>>>>>>>>>>>>>>>>

### 8.x	Data Collection Procedures

### 8.x.1	Data Collection Reporting Initiation

#### 8.x.1.1	General

This procedure is used by gNB-CU-CP to request from gNB-CU-UP the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.x.1.2	Successful Operation

Figure 8.X.1.2-1: Data Collection Reporting Initiation, successful operation

gNB-CU-CP initiates the procedure by sending the DATA COLLECTION REQUEST message to GNB-DU to start information reporting or to stop information reporting. Upon receipt, gNB-CU-UP:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If gNB-CU-UP is capable of providing all of the requested information, it shall initiate the information reporting as requested by GNB-CU-CP and respond with the DATA COLLECTION RESPONSE message.

If gNB-DU-UP is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The gNB-CU-UP shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the gNB-CU-UP shall take it into account for the configuration of UE performance collection and reporting. gNB-CU-UP shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over is equal to the value of the Collection Time Duration for UE Performance IE;

-	the bearer context for the UE is released.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects gNB-CU-UP performs measurements. gNB-CU-UP shall include in the DATA COLLECTION UPDATE message:

-	the Average UE Throughput DL IE, if the first bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by gNB-CU-UP.

-	the Average UE Throughput UL IE, if the second bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by gNB-CU-UP.

-	the Average Packet Delay IE, if the third bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by gNB-CU-UP.

-	the Average Packet Loss DL IE, if the fourth bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by gNB-CU-UP.

#### 8.x.1.3	Unsuccessful Operation

Figure 8.X.1.2-1: Data Collection Reporting Initiation, unsuccessful operation

If none of the requested information can be initiated, gNB-CU-UP shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.x.1.4	Abnormal Conditions

For the same Measurement ID, if the initiating gNB-CU-CP does not receive either the DATA COLLECTION RESPONSE message or the DATA COLLECTION FAILURE message, the gNB-CU-CP may reinitiate the Data Collection Reporting Initiation procedure towards the same gNB-CU-UP, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message.

If the gNB-CU-UP receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "stop" and if the gNB-CU-UP Measurement ID value received in the DATA COLLECTION REQUEST message is not used, the gNB-CU-UP shall initiate DATA COLLECTION FAILURE message with an appropriate cause value.

If in the Report Characteristics for Data Collection IE bitmap all bits are set to "0" in the DATA COLLECTION REQUEST message, then gNB-CU-UP shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the gNB-CU-UP receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "start" and the gNB-CU-CP Measurement ID IE corresponding to an existing on-going Data Collection reporting, then gNB-CU-UP shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

### 8.x.2	Data Collection Reporting

#### 8.x.2.1	General

This procedure is initiated by a gNB-CU-UP to report information accepted by the gNB-CU-UP following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.x.2.2	Successful Operation

Figure 8.x.2.1-1: Data Collection Reporting, successful operation

gNB-CU-UP shall report the accepted information in DATA COLLECTION UPDATE message. The accepted information is the information that was successfully initiated during the preceding Data Collection Reporting Initiation procedure.

#### 8.x.2.3	Unsuccessful Operation

Not applicable.

#### 8.x.2.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

### 9.2.x	Data Collection Messages

#### 9.2.x.1	DATA COLLECTION REQUEST

This message is sent by gNB-CU-CP to gNB-CU-UP to initiate the requested information reporting according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP node1 | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the GNB-DU is requested to report. First Bit = Average UE Throughput DL, Second Bit = Average UE Throughput UL, Third Bit = Average Packet Delay, Fourth Bit = Average Packet Loss DL Other bits are ignored by the GNB-DU. | YES | reject |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.3.1.x |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

#### 9.2.x.2	DATA COLLECTION RESPONSE

This message is sent by gNB-CU-UP to gNB-CU-CP to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP node1 | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the GNB-DU. First Bit = Average UE Throughput DL, Second Bit = Average UE Throughput UL, Third Bit = Average Packet Delay, Fourth Bit = Average Packet Loss DL. Other bits are ignored by the GNB-CU. | – |  |
| >>Cause | M |  | 9.3.1.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

#### 9.2.x.3	DATA COLLECTION FAILURE

This message is sent by the gNB-CU-UP to gNB-CU-CP to indicate that for all of the requested objects the reporting cannot be initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP node1 | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.4	DATA COLLECTION UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the requested information.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP node1 | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | reject |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | gNB-CU-CP UE E1AP ID 9.3.1.4 | gNB-CU-CP UE E1AP ID allocated by gNB-CU-CP. | – |  |
| >>UE Performance | O |  | 9.3.1.y |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |

<<<<<<<<<<<<<<<<<<<<<<<< NEXT CHANGE >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x	UE Performance Collection Configuration

This IE indicates the configuration for UE performance measurement collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond |

#### 9.3.1.y	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.3.1.20 |  |
| Average UE Throughput UL | O |  | Bit Rate 9.3.1.20 |  |
| Average Packet Delay | O |  | INTEGER (0.. 10000) | Defined in TS 38.314 [x]. Unit: 0.1 millisecond |
| Average Packet Loss DL | O |  | Packet Loss Rate 9.3.1.46 |  |

<<<<<<<<<<<<<<<<<<<<<<<< END OF CHANGE >>>>>>>>>>>>>>>>>>>>