3GPP TSG-RAN WG3 Meeting #127                                                                    R3-250359

Athens, Greece, Feb 17th–21st, 2025

Agenda Item:	12.3

Source:	CATT

Title:	(TP for BLCR for TS 38.300) Introduction of functional split for NR Femto

Document for:	Approval

1 Introduction

The TP for introducing function split for NR Femto is provided.

2 TP to TS 38.300

<<<<<<<<<<<<<<<<<<<< Start of TP >>>>>>>>>>>>>>>>>>>>

### 4.X.2	Functional Split

A NR Femto node hosts the same functions as a gNB as described in clause 4.1, with the following additional specifics in case of connection to the NR Femto GW:

- Discovery of a suitable Serving NR Femto GW;

- A NR Femto node shall only connect to a single NR Femto GW at one time. The NR Femto node should not simultaneously connect to another NR Femto GW, or other AMF/UPF.

- The TAC and PLMN ID used by the NR Femto node shall also be supported by NR Femto GW.

- Selection of an AMF at UE attachment is hosted by the NR Femto GW instead of the NR Femto node. Upon reception of the GUAMI from a UE, the NR Femto node shall include it in the INITIAL UE MESSAGE message.

- NR Femto nodes may be deployed without network planning. A NR Femto node may be moved from one geographical area to another and therefore it may need to connect to different NR Femto GWs depending on its location.

- Signalling the GUAMI of the Source AMF to the NR Femto GW in the PATH SWITCH REQUEST message.

- A NR Femto node may release the UE-associated logical NG-connection with NR Femto GW via UE context release procedure.

If the NR Femto GW is deployed, the NR Femto GW hosts the following functions:

- Relaying UE-associated NG application part messages between AMF serving the UE and the NR Femto node serving the UE, for example INITIAL CONTEXT SETUP REQUEST message, HANDOVER REQUEST message, HANDOVER REQUEST ACKNOWLEDGE message, PATH SWITCH REQUEST message, PATH SWITCH REQUEST ACKNOWLEDGE message.

- Terminating non-UE associated NG application part procedures towards the NR Femto node and towards the AMF, for example NG SETUP REQUEST message, PWS RESTART INDICATION message, PWS FAILURE INDICATION message, OVERLOAD START message, OVERLOAD STOP message.

- Optionally terminating NG-U interface with the NR Femto node and with the UPF.

- Supporting TAC and PLMN ID used by the NR Femto node.

- Xn interfaces shall not be established between the NR Femto GW and other nodes.

- Routing the PATH SWITCH REQUEST message towards the AMF based on the GUAMI of the source AMF received from the NR Femto node.

- Requesting AMF to release the resources related to the signalled NG UE context via UE CONTEXT RELEASE REQUEST message.

<<<<<<<<<<<<<<<<<<<< End of TP >>>>>>>>>>>>>>>>>>>>