3GPP TSG-RAN WG3 Meeting #127	R3-250400

Athens, GR, 17-21 Feb, 2025

Title:	(TP for TS 38.300 and TS 38.413) Support MBS broadcast service

Agenda Item:	14.2

Source:	Huawei

Document for:	other

# 1	Introduction

Previous RAN3 meetings agreed that the intended MBS service area provided from the AMF to the gNB can be mapped cell ID(s), TA(s), or geographical service area information, where the geographical service area is aligned with RAN2 format as captured in BLCRs in [1, 2]. It was also agreed that:

- The current MBS broadcast procedures can be reused for NR NTN without any enhancement.

Taking the above into account, this paper provides our further considerations and proposals.

# 2	Discussion

## 2.1 Overlapping issue of multiple geographical areas

In the BLCR [2], the MBS service area information could include NR CGI, TAI or the new small area as the intended MBS service area, all IEs are Optional in same level….

When supporting all the possible representation formats of MBS service area, an issue may arise that the indicated small service area may overlap with each other, or the geographical area may overlap with the mapped cell ID or TAI.

From our point of view, it would be easy that one area should complement to another area info, i.e., there is no overlapping area. For example, the small area could be separated from another small area or another mapped cell ID. But on the other hand, it should be also possible that these indicated areas could be partially overlapped, especially to indicate the MBS service for those neighbouring locations. It is suggested to discuss if there is need to restrict the indicated MBS service areas no overlapping, partially overlap or not.

Note that the previous RAN2-128 meeting made the following agreements, but does not help about the overlapping among the indicated multiple service areas.

4. A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s). 5. RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

- RAN3 to discuss if there is need to restrict the indicated MBS service areas no overlapping, or it allows at least partial overlap.

## 2.2 Stage 2 and stage 3 TPs

In the stage 3 BLCR [2], the small area is marked as FFS and pending to RAN2 as follows.

#### 9.3.1.x Small Area This IE provides information on the Small Area for MBS Broadcast Service. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Small Area; M; OCTET STRING; Pending RAN2 discussion. Editor’s Note: The IE name are pending RAN2 discussion.

The Reply LS [3] from RAN2 has described the representation formats and the candidate IE name as follows:

1.	The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle.

2.	The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon.

3.	The IntendedServiceAreaInfo is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

It can be observed that the IntendedServiceAreaInfo is agreed as the IE name.  From the contexts of the LS, the IntendedServiceAreaInfo indicates one geographical area, either an area of circle, or an area of the polygon.

It also indicated that the IE name may be updated during the CR implementation phase. Hence at this stage, it is suggested to update the stage 3 TP as follows.

- Renaming the small area as the Intended Service Area Information

- Renaming the MBS Service Area Small Area List as MBS Intended Service Area List, as well as the name of max of number.

- Update the stage3 TP as follows. And update stage 2 TP accordingly.

- Renaming the small area as the Intended Service Area Information

- Renaming the MBS Service Area Small Area List as MBS Intended Service Area List, as well as the name of max of number.

# 3	Conclusion

In this paper, we provide the following observations and proposals:

- RAN3 to discuss if there is need to restrict the indicated MBS service areas no overlapping, or it allows at least partial overlap.

- Update the stage3 TP as follows. And update stage 2 TP accordingly.

- Renaming the small area as the Intended Service Area Information

- Renaming the MBS Service Area Small Area List as MBS Intended Service Area List, as well as the name of max of number.

# 4	Reference

- R3-247934	(BL CR to 38.300) Support for Regenerative Payload and MBS broadcast in NR NTN	Ericsson, et.  al

- R3-247935	(BL CR to 38.413) Support for Regenerative Payload and MBS broadcast in NR NTN	CATT, et al.

- R2-2411196 Reply LS on supporting MBS broadcast service for NR NTN, RAN2

# Annex1	Text Proposal for TS 38.300 against R3-247934

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS;

-	The Cell Identity used for MBS broadcast service.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

To support the intended service area of a broadcast service (e.g. MBS broadcast) the AMF provides the gNB with the service area information. This information may be in the form of cell ID list, TAI list, intended service area list or any combination as defined in TS 38.413 [26]. Based on this received information, the gNB provides the intended service area information to the UE as defined in TS 38.331 [12].

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex2	Text Proposal for TS 38.413 against R3-245863

### 8.17.1	Broadcast Session Setup

#### 8.17.1.1	General

The purpose of the Broadcast Session Setup procedure is to request the NG-RAN node to setup MBS session resources for a broadcast MBS session. The procedure uses non-UE associated signalling.

#### 8.17.1.2	Successful Operation

Figure 8.17.1.2-1: Broadcast Session Setup, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION SETUP REQUEST message to the NG-RAN node. If the NG-RAN node accepts all the MBS QoS flows in the MBS session at least in one of its cells, the NG-RAN node responds with the BROADCAST SESSION SETUP RESPONSE message.

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44], and TS 38.300 [8] for NTN operation.

If the MBS Session FSA ID List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account to determine cells/frequencies within the MBS service area to broadcast MBS session data as specified in TS 23.247 [44].

If the Supported UE Type List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the Associated Session ID IE is included in the BROADCAST SESSION SETUP REQUEST message the NG-RAN node shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.300 [8].

-----------------Next Change-------------------

### 8.17.2	Broadcast Session Modification

#### 8.17.2.1	General

The purpose of the Broadcast Session Modification procedure is to request the NG-RAN node to update the MBS session resources or the area related to a previously established broadcast MBS session. The procedure uses non-UE associated signalling.

#### 8.17.2.2	Successful Operation

Figure 8.17.2.2-1: Broadcast Session Modification, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION MODIFICATION REQUEST message to the NG-RAN node.

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area and send the BROADCAST SESSION MODIFICATION RESPONSE message. For NTN operation, the NG-RAN node shall, if supported, update the intended service area information as specified in TS 38.300 [8].

If the MBS Session Modification Request Transfer IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall replace the previously provided information by the newly received one and update the MBS session resources and area as requested and send the BROADCAST SESSION MODIFICATION RESPONSE message.

If the Supported UE Type List IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the MBS NG-U Failure Indication IE is included in the BROADCAST SESSION MODIFICATION REQUEST message within the MBS Session Setup or Modification Request Transfer IE and set to "N3mb path failure", the NG-RAN node may provide new NG-U transport layer information to replace the failed one(s) or switch the data delivery to another 5GC as per the Broadcast MBS session restoration procedure for N3mb Path Failure specified in TS 23.527 [57].

-----------------Next Change-------------------

#### 9.3.1.209	MBS Service Area Information

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI | M |  | 9.3.1.7 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >TAI | M |  | 9.3.3.11 |  |
| MBS Intended Service Area List |  | 0..<maxnoofIntendedAreasforMBS> |  |  |
| >Intended Service Area Information | M |  | 9.3.1.x |  |

Editor’s Note: The IE name and encoding are pending RAN2 discussion.

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is 8192. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is 1024. |
| maxnoofIntendedAreasIforMBS | Maximum no. of intended areas allowed within one MBS Service Area. Value is 65536. |

-----------------Next Change-------------------

#### 9.3.1.x	Intended Service Area Information

This IE provides information on the Small Area for MBS Broadcast Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Intended Service Area Information | M |  | OCTET STRING | Includes the IntendedServiceAreaInfo IE, as defined in TS 38.331[8] |

Editor’s Note: The IE name are pending RAN2 discussion.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>