3GPP TSG-RAN WG3 #127	R3-250412

Athens, Greece, Feb 17th – 21st 2025

Agenda Item:	10.4

Source:	Ericsson

Title:	(TP for BL CR to 38.413 for SON) UHI for SCG activation and deactivation

Document for:	Discussion, Decision

# 1	Introduction

In the Rel-19 SON/MDT Work Item, MHI enhancement for SCG deactivation/activation is listed as one of the objectives for the Work Item. In RAN2#127, RAN2 agreed that it is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state. Two options for collecting the time with SCG state set to activated and deactivated was discussed, namely a network-based solution or a UE based solution. An LS [1] was written and sent to RAN3, asking RAN3 to analyse whether a network-based solution is sufficient enough for network optimization, or whether a UE based solution is needed. In the following discussion in RAN3 #126, RAN3 agreed to support a network-based solution for enhancing PSCell activation/deactivation. In this contribution we propose a network-based solution for logging information needed for such enhancements.

# 2	Discussion

The history information on visited cells, logged by the UE in the Mobility History Information (MHI), and by the network in the UE History Information (UHI), helps the network to understand the movement of the UE and identify ping-pong or coverage issues in the network. From Rel-17, not only time spent in the PCell, but also time spent in the PSCell is logged in the history information. In Rel-17, SCG activation and deactivation was also introduced, which enables power saving while still allowing fast access to the SCG when needed.

Currently there is no support in the history information (UHI or MHI) for tracking how often or how long the SCG state has been set to activated or deactivated. Such information could be used by the network to perform mobility optimization as it tracks the actual usage of the SCG leg. It could also play an important role for traffic and pattern analysis, and the MN could track whether a particular SCG related PSCell is configured often or is seldom used. In addition to this, the information on SCG activation/deactivation could play an important role for mobility and traffic prediction in future AI/ML applications.

To increase the understanding of the secondary cell usage, information on the accumulated time with SCG state set to activated as well as the quantity of SCG deactivations could be logged in the UE history information (UHI) for each PSCell entry. Since the time spent with the PSCell configured is already logged in the history information, the accumulated time with SCG state set to activated will give an indication of how large part of the time spent in the PSCell was spent in activated or deactivated state. The quantity of SCG deactivations will give an indication of how often the SCG state has been set to deactivated, and hence understand if the UE could have been configured with single connectivity during part of the stay in the cell or not.

Logging the accumulated time with SCG state set to activated during the stay in the PSCell in the history information will, together with the already logged time spent in the PSCell, give an indication of how large part of the time spent in the PSCell was spent in activated or deactivated state.

Logging the quantity of SCG deactivations during the stay in the PSCell in the history information will give an indication of how often the SCG state has been set to activated.

In RAN3#125-bis, it was proposed in [2] to also include UE average throughput over SCG in the UHI. But since the UE average throughput can depend on multiple factors, such as UE capability, load, and radio conditions, this is not useful information for mobility optimization or traffic and pattern analysis, which are the main use cases for MHI/UHI. It is also not clear how collection of the UE average throughput is linked to the SCG activation and deactivation; hence the collection of UE average throughput is out of scope of the objective on MHI Enhancement for SCG Deactivation/Activation, and should not be considered within this Work Item.

Collection of UE average throughput is out of scope of the Rel-19 SON/MDT Work Item.

Based on the above reasoning, we propose the following.

For each PSCell entry of the UHI, log the accumulated time with SCG state set to activated during the stay in the PSCell.

For each PSCell entry of the UHI, log the quantity of deactivations of the SCG during during the stay in the PSCell.

A Text Proposal implementing Proposal 1 and 2 is provided in Appendix A below.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Logging the accumulated time with SCG state set to activated during the stay in the PSCell in the history information will, together with the already logged time spent in the PSCell, give an indication of how large part of the time spent in the PSCell was spent in activated or deactivated state.

Observation 2	Logging the quantity of SCG deactivations during the stay in the PSCell in the history information will give an indication of how often the SCG state has been set to activated.

Observation 3	Collection of UE average throughput is out of scope of the Rel-19 SON/MDT Work Item.

Based on the discussion in the previous section we propose the following:

Proposal 1	For each PSCell entry of the UHI, log the accumulated time with SCG state set to activated during the stay in the PSCell.

Proposal 2	For each PSCell entry of the UHI, log the quantity of deactivations of the SCG during during the stay in the PSCell.

# References

R3-245009, LS on MHI enhancement solution for SCG deactivation/activation, RAN2, 3GPP TSG RAN WG2#127, Maastricht, Netherlands, 19th - 23rd August 2024.

R3-245665, (TP to BL CR for TS38.413, TS38.300, TS38.423) Rel-18 leftovers, RAN3, 3GPP TSG-RAN WG3 Meeting #125-bis, Hefei, China, 14th - 18th Oct 2024

# Appendix A - Text Proposal to 38.413

#### 9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. |
| Overall Activation Timer | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell while the SCG was in active state, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. |
| Number of SCG status changes | O |  | INTEGER (0..1000) | The number of times the SCG changes status from activated to deactivated. If the number of status changes is higher than 1000, this IE is set to 1000. |

[Unchanged parts omitted]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

[Unchanged parts omitted]

id-NR-PagingeDRXInformation,

id-NumberOfSCGStatusChanges,

id-OldAssociatedQosFlowList-ULendmarkerexpected,

id-OnboardingSupport,

id-OverallActivationTimer,

id-PagingAssisDataforCEcapabUE,

[Unchanged parts omitted]

LastVisitedPSCellList ::= SEQUENCE (SIZE(1..maxnoofPSCellsPerPrimaryCellinUEHistoryInfo)) OF LastVisitedPSCellInformation

LastVisitedPSCellInformation ::= SEQUENCE {

pSCellID			NGRAN-CGI	 			OPTIONAL,

timeStay			INTEGER (0..40950),

iE-Extensions		ProtocolExtensionContainer { {LastVisitedPSCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedPSCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-OverallActivationTimer	CRITICALITY ignore	EXTENSION OverallActivationTimer	PRESENCE optional }|

{ ID id-NumberOfSCGStatusChanges	CRITICALITY ignore	EXTENSION NumberOfSCGStatusChanges	PRESENCE optional },

...

}

[Unchanged parts omitted]

NumberOfSCGStatusChanges ::= INTEGER (0..1000)

[Unchanged parts omitted]

OverallActivationTimer ::= INTEGER (0..40950)

[End of changes]