3GPP TSG RAN WG3 Meeting #127	R3-250413

Athens, Greece, February 17 – 21, 2025

Title:	Draft Response LS on LMF-based AI/ML Positioning for Case 3b

Response to:	R3-250074 (S2-2501341)

Release:	Rel-19

Work Items:	NR_AIML_air-Core

Source:	Qualcomm Incorporated [to be RAN WG3]

To:	SA2

CC:	RAN1, RAN2

Contact Person:

#### Name:	Sven Fischer

E-mail Address:	sfischer@qti.qualcomm.com

Attachment:	-

1. Overall Description:

RAN3 thanks SA2 for their LS on "LMF-based AI/ML Positioning for Case 3b". Regarding the questions from SA2:

SA2 asks RAN3 to provide feedback on the Editor’s Notes in Clause 5.18 and Clause 6.22.3 in TS 23.273 v.19.1.0 and to provide references to the corresponding procedures in RAN TSs (or an estimation on when those procedures can be available).

RAN3 would like to provide the following feedback.

1.) Regarding Editor's Notes in Clause 5.18 in TS 23.273:

Editor's note: What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

RAN3 response: The NRPPa Measurement Information Transfer procedures defined in TS 38.455 are re-used to support requesting and reporting the positioning measurements to the LMF for Case 3b. The details of the related measurements are pending and subject to further discussion in RAN1.

Editor's note: The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs.

RAN3 Response: Please see 2.) below regarding procedures for model training.

Editor´s note: Whether user consent for UE positioning calculation is needed apart from the existing GMLC check of the LCS privacy profile is FFS.

RAN3 response: RAN3 assumes that whenever UE location information is being determined by the network, the specified LCS privacy rules and profiles apply.  However, the details are out of RAN3 scope.

2.) Regarding Editor's Notes in Clause 6.22.3 in TS 23.273:

Editor's note: The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

RAN3 response: RAN3 assumes that this Editor's Note is related to the data collection (not positioning calculation) by LMF described in Figure 6.22.3-1. RAN3 would like to provide the following feedback on the procedure defined in Figure 6.22.3-1 in TS 23.273.

- The procedure defined in Figure 6.22.3-1 in TS 23.273 for data collection by an LMF does generally match RAN3 agreements.

- For the data collection step 6 in Figure 6.22.3-1 in TS 23.273, existing NRPPa procedures related to Positioning Information Transfer (to configure an SRS in the target UE) and Measurement Information Transfer (to obtain training data from gNBs) as specified in TS 38.455 are being reused.

- For step 7 in Figure 6.22.3-1 in TS 23.273 (ground truth data collection from UE), RAN3 assumes that any of the existing UE positioning procedures defined in TS 23.273 (e.g., MT-LR, etc.) and positioning methods defined in TS 38.305 (e.g., GNSS, TDOA, etc.) can be used to obtain ground truth information for a UE.

- For the data collection step 6 in Figure 6.22.3-1 and according to (b) above, the UE-associated NRPPa Positioning Information Transfer procedures may be used. According to clause 6.11.2 of TS 23.273, a pre-requisite for UE-associated signalling is that a LCS Correlation Identifier and the AMF Identity have been passed to the LMF by the serving AMF. It is assumed that SA2 will clarify how an LCS Correlation Identifier is obtained in the procedure of Figure 6.22.3-1 in TS 23.273 (i.e., without location request from an AMF).

- For step 7 in Figure 6.22.3-1 in TS 23.273 (ground truth data collection from UE), RAN3 assumes that LPP and LCS Supplementary Services signalling may be used for obtaining UE location information. According to clause 6.11.1 of TS 23.273, a pre-requisite the LPP/LCS signalling is that a LCS Correlation Identifier and the AMF Identity have been passed to the LMF by the serving AMF. It is assumed that SA2 will clarify how an LCS Correlation Identifier is obtained in the procedure of Figure 6.22.3-1 in TS 23.273 (i.e., without location request from an AMF).

Editor's note: Whether and how the LMF takes the RAN load into account in this procedure is FFS.

RAN3 response: Whether and how the LMF takes the RAN load into account is up to implementation.

3.) The detailed steps for AI/ML positioning Case 3b will be described in TS 38.305. The completion date for the CRs is currently assumed to be RAN#109 according to the Work Item Description in RP-243244.

2. Actions:

To SA WG2

ACTION: RAN3 respectfully asks SA2 to take the above feedback into account.

3. Date of Next RAN WG3 Meetings:

TSG-RAN WG2 Meeting #127bis	April 7 to 11, 2025	Wuhan, China

TSG-RAN WG2 Meeting #128	May 19 to 23, 2025	Malta, MT