3GPP TSG RAN WG3 Meeting #127	R3-250477

Athens, Greece, 17 - 21 February, 2025

# Title:	[Draft] Reply LS on Available Data Rate notion and impacts

# Response to:	LS (S2-2412720/R3-250018) on FS_XRM Ph2 from SA2

# Release:	Release 19

# Work Item:	NR_XR_Ph3-Core

Source:	[Ericsson to be] RAN3

To:	SA2

Cc:

Contact Person:

#### Name:	Yazid Lyazidi

#### Tel. Number:

#### E-mail Address:	Yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN3 thanks SA2 for their LS reply explaining the available data rate definition. However, RAN3 is still not clear on many aspects. Namely:

- First, when SA2 mentions: “it is expected that the available data rate is properly reflecting the resources that are currently provided by the NG-RAN for the QoS Flow”

RAN3 notes there is a misconception at SA2 that “data rate” can be regarded to be “proportional” to “RAN resources” provided for a QoS flow and represents information about the “cost” of a service in terms of necessary RAN resources. Such interpretation is wrong and should not introduced.

- SA2 says: “While it is up to the application layer how to make use of it, we can however assume that the application will typically exchange data with a somewhat lower rate than what is reserved or measured at NG-RAN”.

It is RAN3’s view that the CN/AF shouldn’t consider “available data rate/resources” when it comes to admission control, e.g. by prioritising one QoS flow of the other etc., there is a clear worksplit between RAN and CN and introducing such feature will be a destabilisation of the 5G system.

- SA2 says: The “available data rate” can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without AQP.

For a GBR QoS flow, the RAN is committing to meet a bitrate requirement, thus the available data rate is supposed to remain the same. if the AF assumes the bitrate is too stringent, it can signal AQP in the QoS profile. RAN3 wonders why the addition of a new functionality in absence of AQP?

- SA2 says: Periodic reporting should not be useful for the application layer (as the “available data rate” should often be identical to the GFBR

If the Available bit rate definition according to SA2 is not expected to change often and remain the same, thus questionable if UP signaling is the right solution. It is not clear to RAN3 why to not re-use AQP and signaling over CP.

- Considering the final paragraph from SA2: Threshold based reporting is considered as useful with one or more threshold values provided by the application. When the data rate that NG-RAN is currently able to provide for a QoS Flow crosses one of the thresholds, the NG-RAN is expected to provide the reference to the respective threshold in the UL user plane packets and the application can then adjust its data rate accordingly

The introduction of “available data rate” has many impacts in RAN3 protocols and interfaces: NGAP signaling, F1AP, E1AP, XnAP, RAN capability sharing, configuration steps, some aspects are very similar to ECN. RAN3 cannot proceed with these impacts without solid motivation from the proponents and added value wrt AQP and ECN features. the unsolid stage 2 and stage 3 design will come with several mistakes.

Also, RAN3 could not help but notice that SA2 has already a long “basket” of items being agreed that will have RAN3 impacts: DL PDU Set Marking, MMSID, PDU Set AQP, AL-FEC. Supporting another feature with unclear definition and benefits is not the right decision considering RAN3 load and time left in Rel-19 XR. There is a solid concern in RAN3 on the workload. Therefore, RAN3 asks SA2 to drop this feature.

2. Actions:

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above feedback into consideration.

3. Date of Next RAN3 Meetings:

RAN3#127-bis	April 7-11, 2025				Wuhan, PRC

RAN3#128	May 19-23, 2025				Malta, MT