3GPP TSG-RAN WG3 Meeting #127	R3-250496

Athens, Greece, 17th - 21st Feb 2025

Agenda Item:	9.1

Source:	ZTE Corporation, CMCC, CATT

Title:	Discussion on coarse UE location information transfer for NB-IoT NTN

Document for:	Discussion

- Introduction

In RAN3#125bis，RAN3 has agreed the CR R3-245818 to TS36.413, to introduce the Coarse UE Location request and response between eNB and MME: The eNB requests to the MME to report the coarse UE location, if available. The request and report shall only be sent once during one connection. The INITIAL UE MESSAGE message includes the request and the INITIAL CONTEXT SETUP, DOWNLINK NAS TRANSPORT, and CONNECTION ESTABLISHMENT INDICATION messages may contain the coarse UE location.

However in RAN3 #127, RAN3 received the LS from SA2 in [1]: SA2 nevertheless observed that the CR approved by RAN3 (TS 36.413 CR1957) did not include the “Coarse UE Location Requested” IE in UL NAS Transport, Resume Request and the “Coarse UE Location IE” in Resume Response. Even though Coarse Location Information can be requested once in the Initial UE message also in these cases (and provided to the eNB in a downlink S1-AP message), this prevents the Coarse Location Information of the UE to be requested by eNB and provided from MME to eNB at a later stage during a RRC connection when user plane bearers are established. The relevant procedures in the SA2 specification are “Establishment of S1-U bearer during Data Transport in Control Plane CIoT EPS Optimisation” (see clause 5.3.4B.4 of TS 23.401), “Connection Resume procedure” (see clause 5.3.5A) and “MT-EDT procedure for User Plane CIoT EPS Optimisation” (see clause 5.3.5B). SA2 would like to understand if RAN2 and RAN3 has performed any analysis on whether Coarse Location Information is needed by the eNB in any of the above procedures. In case RAN2 and RAN3 conclude that Coarse Location Information is needed in these cases, SA2 would like to highlight that AS security is activated during these procedures which would also allow the UE to provide the Coarse Location Information to the eNB via the RRC protocol.

In this contribution, we will discuss and propose whether the “Coarse UE Location” shall be requested and responsed in more messages.

2	Discussion

It is the consensus in RAN3 that “The request and report shall only be sent once during one connection.” This is to improve the efficiency and guarantee the security of network.

According to the other messages that can be considered to contain “Coarse UE Location Requested” IE:

- UL NAS Transport may be sent from eNB to MME without after UE Initial Message, in the S1-U bearer during DT in CP CIoT Optimization procedure. However during this procedures, the UE doesn’t update the “Coarse UE Location” to MME, so the “Coarse UE location” information remains unchanged in MME. In this scenario, there is no need for eNB to request the “Coarse UE Location”

Figure 5.3.4B.4-1: Establishment of S1-U bearer during Data Transport in Control Plane CIoT EPS Optimisation [2]

And when UE moves and changes to a new “Coarse Location”, it always needs to initialize Attach Procedures, or TAU Procedures to MME, which will send NAS Initial UE message and then UL NAS Transport message. So it is enough and more efficient to only request the “Coarse Location” in Initial UE message.

Proposal 1: It is proposed not to include “Coarse UE Location Requested” in S1-AP Uplink NAS Transport message.

- UE context Resume Request and Resume Response are introduced in Connection Resume Procedures or MT for User Plan CIoT Optimization Procedures. For UE Context Resume Request message, it is sent from eNB to MME in Connection Resume Procedure. This procedure is used by the UE to resume the ECM-connection if the UE and the network support User Plane CIoT EPS Optimisation and the UE has stored the necessary information to conduct the Connection Resume procedure (see TS 36.300 [4]) otherwise the Service Request procedures are used, see clause 5.3.4 in TS 23.401 [1].

Figure 5.3.5A-1: UE initiated Connection Resume procedure [2]

Figure 5.3.5B-1: MT-EDT procedure for User Plane CIoT EPS Optimization [2]

Considering the coarse UE location transfer in above resume scenarios, there are two options as below.

Option 1: Enhance the existing UE Context Resume procedure over S1

Option 2: Enhance the existing Retrieve UE Context procedure over X2

For Option 1, if the UE redirects to a new eNB, the new eNB shall trigger the UE Context Resume procedure to the MME to resume the suspend RRC connection. To support getting Coarse UE Location, after receiving RRC Connection Resume Request from UE, the new eNB shall add the “Coarse UE Location Request” IE in the “UE Context Resume Request” message. If MME can provide the Coarse UE Location information, it will bring the “Coarse UE Location” IE in the S1 “UE Context Resume Response”message.

For Option 2,  if the UE redirects to a new eNB, the new eNB shall trigger the Retrieve UE Context procedure to the old eNB to resume the suspend RRC connection. To support getting Coarse UE Location on the new eNB, after receiving RRC Connection Resume Request or RRC Connection Re-establishment Request from UE, the new eNB shall add the “Coarse UE Location Request” IE in the “Retrieve UE Context Request” message through X2 to the old eNB. If the old eNB restores the Coarse UE Location information as part of its UE context, it will bring the “Coarse UE Location” IE in the X2 “Retrieve UE Context Response”message.

Both the options have additional impact on current specifications. For Option 2, if the eNB requests Coarse UE Location through the X2AP Retrieve UE Context procedure, it can happen after RRC Connection Resume or RRC Connection Re-establishment. Also for NB-IoT NTN, assuming that the UE is relatively static, if the new eNB already restores the Coarse Location information as part of the UE context, it is no need to request again from old eNB through the X2AP Retrieve UE Context procedure.

While, for Option 1, it may go against the principle of “The request and report shall only be sent once during one connection”, since it allows to include the “Coarse UE Location Requested” IE in two different S1AP messages. Therefore, Option 2 could be more appropriate.

Proposal 2: It is proposed to introduce “Coarse UE Location Request” IE in X2AP “Retrieve UE Context Request” message and “Coarse UE Location” IE in the X2AP “Retrieve UE Context Response” message.

3	Summary

Proposal 1: It is proposed not to include “Coarse UE Location Requested” in S1-AP Uplink NAS Transport message.

Proposal 2: It is proposed to introduce “Coarse UE Location Request” IE in X2-AP “Retrieve UE Context Request” message. And “Coarse UE Location” IE in the X2AP “Retrieve UE Context Response” message.

The corresponding CRs and reply LS are given in [3][4][5].

4	Reference

[1] R3-250017 (S2-2412665), Reply LS on UE Location Information for NB-IoT NTN

[2] TS 23.401, General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access.

[3] R3-250679, Stage 2 correction for coarse UE location information transfer for NB-IoT NTN, CMCC, CATT, ZTE

[4] R3-250418, Coarse UE location information transfer for NB-IoT NTN-X2, CATT, CMCC, ZTE

[5] R3-250497, [Draft] Reply LS on UE location information for NB-IoT NTN, ZTE, CMCC, CATT