3GPP TSG-RAN WG3 Meeting #127	             R3-250523

Athens, Greece, 17th – 21st Feb 2025

Agenda Item:	17.3

Source:	CMCC

Title:	Support On-Demand SIB1 for UEs

Document for:	Discussions & Approval

# Introduction

In the RAN3 # 125 Bis meeting, the followings were agreed [1]:

- The UL WUS configuration will be transferred over Xn via a new class1 defined procedure. - Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

In the RAN3 # 126 meeting, the followings were agreed [5]:

The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2. The procedure uses non UE-associated signaling. Cell A stores the UL WUS configuration information after it has received it. A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB. The procedure uses non UE-associated signalling. Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

# Discussion on OD-SIB1 Mode

## WUS Broadcast Capability Notification

To help the NES Cell gNB to make a decision on which neighbouring cells are suitable to be Cell A and broadcast UL WUS configuration for the NES Cell, the WUS Broadcast Capability the neighbouring cells could be notified to the NES Cell gNB.

WUS broadcast capability of neighbouring cells could be indicated from one gNB to another gNB, or from one gNB-CU to another gNB-CU, or/and from one gNB-DU to its CU, which could have Xn and F1 interface impact.

## UL WUS Configuration Broadcast Request

UL WUS Configuration Broadcast Request from the NES Cell indicates the needs from the NES Cell to request the Cell A to broadcast the UL WUS configuration on the NES Cell’s behalf. The UL WUS Configuration Broadcast Request is sent from the NES Cell gNB to the Cell A gNB, including the number of NES Cell, NES Cell List, corresponding UL WUS configuration for each NES Cell, the action associated with the NES cell, i.e., Inquiry (UL WUS Configuration Broadcast), Start (UL WUS Configuration Broadcast), Pause (UL WUS Configuration Broadcast), Cancel (UL WUS Configuration Broadcast). Figure 1 shows the possible procedures.

Figure 1 UL WUS Configuration Broadcast Request Procedures

- Inquiry UL WUS Configuration Broadcast: it means that the NES Cell gNB wants to inquiry the Cell A gNB whether it could broadcast the UL WUS configuration for a list of NES Cells. The following “Start UL WUS Configuration Broadcast” action could be contained in the same message with “Inquiry UL WUS Configuration Broadcast”, it indicates that the NES Cell gNB wants the Cell A gNB to start broadcasting the UL WUS configuration for a list of NES Cells right away. Otherwise, this action simply requests the Cell A gNB to confirm whether the Cell A gNB can accept the request of broadcasting the UL WUS configuration for a list of NES Cells upon the Cell A gNB’s “Start UL WUS Configuration Broadcast” action indication.

- Start UL WUS Configuration Broadcast: it means the NES Cell gNB wants the Cell A gNB to broadcast the UL WUS configuration for a list of NES Cells. Each NES Cell might be associated with a UL WUS configuration, time of period that the UL WUS configuration broadcast is requested by the NES Cell.

- Stop UL WUS Configuration Broadcast: it means that the NES Cell gNB wants the Cell A gNB to temporally stop the UL WUS configuration broadcast for a list of NES Cells. Such pause might be associated with a time of period (timer) for each requested NES Cell. The Cell A gNB needs to maintain the UL WUS configuration and status for the list of requested NES Cells.

- Cancel UL WUS Configuration Broadcast: it means that the NES Cell gNB wants the Cell A gNB to completely cancel the UL WUS configuration broadcast for a list of NES Cells. The cancellation will remove the configuration and state that was maintained by the Cell A gNB for the list of NES Cells.

The UL WUS Configuration Broadcast Request is sent from the NES Cell gNB to the Cell A gNB, including the number of NES Cells, NES Cell List, corresponding UL WUS configuration for each NES Cell, the action associated with the NES cell, i.e., Inquiry (UL WUS Configuration Broadcast), Start (UL WUS Configuration Broadcast), Stop (UL WUS Configuration Broadcast), Cancel (UL WUS Configuration Broadcast).

For the request that contains the action of “Inquiry UL WUS Configuration Broadcast”, the Cell A gNB might return a “Confirm” or “Reject” response:

- The “Confirm” response indicates that the CELL A gNB is willing to broadcast the UL WUS Configuration for the list of NES Cells. If the message also contains the action of “Start UL WUS Configuration Broadcast”, then it follows the same responses as “Start UL WUS Configuration Broadcast”, which is discussed below. In the meantime, the CELL A gNB might reserve the resource for the broadcasting upon the NES CELL’s “Start” request, and also use it as a factor to decide whether to accept other NES Cell’s requests in the future.

- The “Reject” response tells the NES Cell the requested Cell A refuses to broadcast the UL WUS configuration for the NES Cell. The inquiry is a failure.

The action of “Inquiry UL WUS Configuration Broadcast” inquires the Cell A whether the Cell A can broadcast the UL WUS Configuration for the requested NES Cell.

The Cell A records/updates the number of NES Cells, the NES Cell List, corresponding UL WUS configuration for each NES Cell before the “Cancel UL WUS Configuration Broadcast” is received.

For the request that contains the action of “Start UL WUS Configuration Broadcast”, the Cell A gNB might return a “Confirm” or “Reject” response:

- The “Confirm” response could be associated with timer, which indicates the time of period that the Cell A needs before the UL WUS configuration broadcast is effective. The NES Cell could use the timer to configure when to go to the OD SIB1 mode. (This is the method of “Use timer to instruct NES Cell to enter OD-SIB1 mode” as shown in Figure 1)

- The “Confirm” response could simply indicate that the Cell A is preparing itself for the broadcast of UL WUS Configuration for the NES Cell. After it is ready, the Cell A gNB could send an additional message of “UL WUS Configuration Is Broadcast” to the NES Cell gNB. Up on receiving this message, the NES Cell could go to the OD SIB1 mode safely. (This is the method of “Use followup notification to instruct NES Cell to enter OD-SIB1 mode” as shown in Figure 1)

- The “Reject” response tells the NES Cell the requested Cell A refuses to broadcast the UL WUS configuration for the NES Cell at this moment. It might be associated with a suggested time that the NES Cell could request again.

The action of “Start UL WUS Configuration Broadcast” requests the Cell A to broadcast the UL WUS Configuration for the requested NES Cell, which might be associated with a time of period that such request is effective.

The action of “Start UL WUS Configuration Broadcast” could be contained in the same message with “Inquiry UL WUS Configuration Broadcast”, which indicates that the NES Cell gNB wants the Cell A gNB to start broadcasting the UL WUS configuration for a list of NES Cells right away.

The response to “Start UL WUS Configuration Broadcast” could be a “Confirm”, which could be associated with timer, indicating the time of period that the Cell A needs before the UL WUS configuration broadcast is effective. The NES Cell could use the timer to configure when to go to the OD SIB1 mode.

The response to “Start UL WUS Configuration Broadcast” could be a “Confirm”, with a “Further Notification Indicator” to indicate an additional message follows.

The Cell A gNB could send an additional message of “UL WUS Configuration Is Broadcast” to the NES Cell gNB. Up on receiving this message, the NES Cell could go to the OD SIB1 mode safely.

If there are multiple Cell As that accept the UL WUS Configuration Broadcast Request from the NES Cell, the action of “Start UL WUS Configuration Broadcast” gives flexibility to the NES Cell gNB to decide the timing for the multiple Cell As to broadcast the UL WUS configuration for it.  The NES Cell could be the SCell for multiple UEs, depending on the PCell that active UEs connecting to, the NES Cell could switch to a different Cell A and request the Cell A to broadcast the UL WUS Configuration for it.

The Cell A as PCell could initiate the UL WUS Configuration broadcast for a NES Cell as a SCell for a UE if the Cell A accepts the UL WUS Configuration Broadcast Request from the NES Cell.

The Cell A as PCell to notifies the NES Cell as a SCell for a UE that the UE is connected to the PCell, such that the NES Cell would indicate the Cell A to broadcast UL WUS Configuration for it.

For the request that contains the action of “Stop UL WUS Configuration Broadcast”, the Cell A gNB might return a “Confirm” response:

- The “Confirm” response confirms that the UL WUS configuration broadcast is stopped temporarily at the Cell A.

For the request that contains the action of “Cancel UL WUS Configuration Broadcast”, the Cell A gNB might return a “Confirm”:

- The “Confirm” response confirms that the UL WUS configuration broadcast is completely cancelled at the Cell A. And the UL WUS Configuration previously sent from the NES Cell could be removed from the Cell A.

The action of “Stop UL WUS Configuration Broadcast” requests the Cell A to pause the UL WUS configuration broadcast for the requested NES Cell, which might be associated with a time of period that such request is effective. The Cell A needs to maintain the configuration and status for the NES Cell.

The action of “Cancel UL WUS Configuration Broadcast” requests the Cell A to completely cancel the UL WUS configuration broadcast for the requested NES Cell. The Cell A removes the configuration and status for the NES Cell.

The Cell A could initiate a UL WUS Configuration Broadcast Status Notification to the NES Cell that it needs to temporarily stop UL WUS Configuration Broadcast, or cancel UL WUS Configuration Broadcast, due to the reasons such as resource constraint or its own status change. The Cell A initiated  ULWUS Configuration Broadcast Status Notification procedures are shown in Figure 2.

- “UL WUS Configuration Broadcast Status Notification of Stop” indicates that the Cell A needs to temporarily stop UL WUS Configuration Broadcast, due to the reasons such as resource constraint or its own status change. It might be associated with a time of period that stop is effective.

- “UL WUS Configuration Broadcast Status Notification of Ready” indicates that the Cell A is ready to resume the UL WUS Configuration Broadcast for the NES Cell.

  - The response to “UL WUS Configuration Broadcast Status Notification of Ready” could be a UL WUS Configuration Broadcast Request with the action of “Start UL WUS Configuration Broadcast”.

  - The response to “UL WUS Configuration Broadcast Status Notification of Ready” could be a UL WUS Configuration Broadcast Request with the action of “Stop UL WUS Configuration Broadcast”.

  - The response to “UL WUS Configuration Broadcast Status Notification of Ready” could be a UL WUS Configuration Broadcast Request with the action of “Cancel UL WUS Configuration Broadcast”.

- “UL WUS Configuration Broadcast Status Notification of Cancel” indicates that the Cell A needs to completely cancel the UL WUS Configuration Broadcast for the NES Cell. The Cell A removes the configuration and status for the NES Cell.

The UL WUS Configuration Broadcast Status Notification could be sent from the Cell A to the NES Cell, which could be “UL WUS Configuration Broadcast Status Notification of Stop”, “UL WUS Configuration Broadcast Status Notification of Cancel”, “UL WUS Configuration Broadcast Status Notification of Ready”.

“UL WUS Configuration Broadcast Status Notification of Stop” indicates that the Cell A needs to temporarily stop UL WUS Configuration Broadcast.

“UL WUS Configuration Broadcast Status Notification of Ready” indicates that the Cell A is ready to resume the UL WUS Configuration Broadcast for the NES Cell.

“UL WUS Configuration Broadcast Status Notification of Cancel” indicates that the Cell A needs to completely cancel the UL WUS Configuration Broadcast for the NES Cell.

Figure 2  Cell A Initiated UL WUS Configuration Broadcast  Status Notification Procedures

## Split Architecture

If the CU/DU split architecture is adopted in gNB, a NES Cell and Cell A could be in different DU(s). In order to transit the UL WUS Configuration, both F1 interface and Xn interface could be impacted.

- The NES Cell and Cell A are on the same gNB, but on different DU(s). gNB-CU is in the middle to forward the UL WUS configuration between two gNB-DUs on the same gNB.

- The NES Cell and Cell A are on different gNB(s). Two gNB-CU(s) are in the middle to forward the UL WUS Configuration between two gNB-DUs on different gNB(s).

The procedures and messages that are discussed above are also applicable to F1 interface.

All procedures and messages are applicable to F1 interface when the NES Cell and Cell A are on the same gNB, but on different DU(s); or the NES Cell and Cell A are on different gNB(s).

## UL WUS Configuration Details

For discussion on specific content for on-demand SIB1, the following agreements are agreed during RAN1#118bis meeting [3] and RAN1#117 meeting [2]:

Agreement in RAN1#118bis: For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. To which cell does the configuration applies; NES-CellId; PhysCellId. ARFCN-ValueNR. Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell. Agreement in RAN1#118bis: For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration: - rsrp-ThresholdSSB - prach-RootSequenceIndex - msg1-SubcarrierSpacing - restrictedSetConfig Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon Agreement in RAN1#118bis: For the purpose of “WUS transmission”, at least the following parameters to indicate “frequency information for UL-WUS transmission” are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz (working assumption)ULSubCarrierSpacing. Agreement in RAN1#118bis: For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. SSB-positionInBurst. tdd-UL-DL-ConfigurationCommon. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResources; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. Note: - In legacy spec, ss-PBCH-BlockPower/SSB-positionInBurst/tdd-UL-DL-ConfigurationCommon are under the IE ServingCellConfigCommon/ServingCellConfigCommonSIB - Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA Agreement in RAN1#117: For further study of on-demand SIB1 in idle/inactive mode, use the following Table I (from R1-2405106, Ericsson) as a starting point to discuss the required parameters/contents inside the UL WUS configuration. Table I. Purpose; Parameters. To which cell does the config applies; NES-CellId; PhysCellId. ARFCN-ValueNR. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResource; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. rsrp-ThresholdSSB. prach-RootSequenceIndex. msg1-SubcarrierSpacing. restrictedSetConfig. tdd-UL-DL-ConfigurationCommon. frequencyInfoUL; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz. SIB1 reception; pdcch-ConfigSIB1; ssb-SubcarrierOffset. controlResourceSetZero. searchSpaceZero. RAR Reception; pdcch-ConfigOD-SIB1-RAR; controlResourceSet. monitoringSlotPeriodicityAndOffset. Duration. monitoringSymbolsWithinSlot. aggregationLevels.

From RAN3’s perspective, a new IE to contain the UL WUS configuration is needed. Based on the above agreements, the following sub-elements shall be included in the new IE:

- NES-CellId: it indicates to which cell does the WUS configuration apply to.

- WUS transmission: it contains the configuration of WUS transmission of the NES Cell.

- SIB1 reception: it is the configuration of SIB1 reception, which is used to instruct the UEs where to receive the SIB1 from the Nes Cell.

- RAR Reception: the configuration of RAR reception, which is used to instruct UEs where and how to receive the RAR from the NES Cell. RAR reception allows the UE to synchronize its timing with the network and proceed with the connection setup or re-establishment process.

A new IE that contains UL WUS configuration of the NES Cell needs to be defined, which includes NES-CellId, WUS Transmission, SIB1 Reception, RAR Reception. The details are still upon RAN1’s progress.

## Decision of NES Cell

This subsection mainly focuses on the split architecture.  The CU manages signalling between the network and the UE, including connection setup, release, mobility management, and security functions. And it has more global picture of the neighbouring cell deployment, inference level, signal strength and quality, energy consumption, power status, etc.  The CU can be helpful by providing information before the DU initiates the on-demand SIB1 operation for some NES cells. For instance, the CU could supply a list of cells where the on-demand SIB1 operation should be enabled.

The CU of the NES Cell could provide some assistance information (a list of cells where the on-demand SIB1 operation should be enabled, and a list of corresponding cells could be requested as Cell A) before a DU initiates the OD SIB1 operation.

# Conclusions

In this submission, we discussed the procedures of the OD-SIB1. The following proposals are presented in the paper:

# Reference

- draft_MeetingReport_RAN3#125-bis_vOct25

- Draft_Minutes_report_RAN1#117_v030

- Draft_Minutes_report_RAN1#118b_v010

- Draft-R3-245737_V2

- draft_MeetingReport_RAN3#126_vDec05