3GPP TSG-RAN WG3 Meeting #127	R3-250537

Athens, Greece,  17 – 21 February, 2025

Agenda item:	19.2

Source:	Nokia

Title:	Further discussion on network support for SBFD operation and CLI management

Document for:	Discussion and Decision

# 1	Introduction

In this paper we provide discussion relative to the following agreement and open point captured at RAN3#126:

- The SBFD time and frequency location configuration with cell granularity in Served Cell Information NR IE shall be introduced in XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

- Exchange reference signalling configuration(SSB and NZP CSI-RS) in interface management procedures: Xn Setup and NG-RAN node Configuration Update procedure?

# 2	Discussion

## 2.1	Signalling of SBFD configuration

The agreement (1) cited in introduction will ensure that a gNB will inform its neighbour gNBs about SBFD operation in one or more of its cells.

The following open point from RAN3#125bis remains open:

- Semi-static cell-specific SBFD time and frequency location configuration with cell granularity (FFS on container or explicit IEs over XnAP and F1AP)

A natural solution would be to request RAN2 to define an RRC-encoded IE (inter-node message) that RAN3 can include for signalling on its interfaces.

Proposal 1: RAN3 requests RAN2 to define an RRC-encoded IE (inter-node message) for SBFD configuration that RAN3 can include for signalling on its interfaces.

Furthermore, for support of split architecture the SBFD configuration needs to be transferred from the gNB-DU to the gNB-CU. By following a similar approach on F1 as already agreed for Xn, i.e. to include the SBFD configuration in the F1AP Served Cell Information IE, this information will be included in the F1 SETUP REQUEST and GNB-DU CONFIGURATION UPDATE messages.

We therefore propose to capture a similar agreement for F1AP as already done for XnAP:

Proposal 2: The SBFD time and frequency location configuration with cell granularity shall be introduced in the F1AP Served Cell Information IE in the F1 SETUP REQUEST message and GNB-DU CONFIGURATION UPDATE message.

Taking into account that the gNB-DU is also a consumer of the SBFD configuration information, it would also need to be transferred in the direction from the gNB-CU to the gNB-DU so that it will be available for gNB-DUs serving neighbour cells. This could be done in a similar way as for the CLI (Cross-Link Interference) mitigation feature that was introduced in Rel-16, by using the Neighbour Cell Information List IE in the GNB-CU CONFIGURATION UPDATE message:

| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| >>SBFD Configuration | O |  | 9.3.1.xx |  | - |  |

Proposal 3: The SBFD time and frequency location configuration with cell granularity shall be introduced in the  Neighbour Cell Information List IE in the GNB-CU CONFIGURATION UPDATE message.

## 2.2	Signalling of CLI-RS configuration

The open point (2) from last meeting relates to signalling of CLI-RS configuration:

(2) Exchange reference signalling configuration(SSB and NZP CSI-RS) in interface management procedures: Xn Setup and NG-RAN node Configuration Update procedure?

The CLI-RS (Cross-Link Interference Reference Signal) may be the SSB or the NZP CSI-RS. A starting point in analysis of this open point is that SSB information is already transferred as part of Served Cell Information over F1 and Xn interfaces, and periodically transmitted over the radio in all active (pre-Rel-19) cells, and this reference signal serves in many scenarios (connected mode, idle/inactive mode, intra-cell, inter-cell…).

The NZP CSI-RS signal has a different purpose and is today not mandatorily made available for inter-cell use. The question raised by (2) is therefore whether SBFD operation changes this situation and needs to mandate systematic transmission of NZP CSI-RS for the purpose of CLI-RS in the concerned resources. But first it seems useful to analyze whether SBFD operation in a cell necessarily also requires that this cell performs inter-cell interference management.

Observation 1: The open point captured by RAN3 leads to the question whether SBFD operation in a cell necessarily also requires that this cell performs inter-cell interference management.

The answer to this question would in our view depend on the gNB implementation and on deployments. We believe there are two main cases:

- Case 1: In some deployments inter-gNB CLI for SBFD could be very low due to e.g. antenna orientations, long inter-site distance and simply traffic load. Some gNBs may also be able to perform noise measurements in SBFD resources using IRC receiver, and when noise level increase is detected the gNB may start interference management. Interference management could also be triggered upon need e.g. based on observed throughput degradation in SBFD resources.

- Case 2: In some other deployments e.g. dense deployments with short inter-site distance and/or high traffic load, inter-gNB CLI may be “guaranteed”. It could also happen that noise measurements don’t give a reliable result, and any observed throughput degradation could have multiple causes.

At this point in the analysis, one would need to take into consideration that for what concerns inter-cell CLI for SBFD, the victim cell is always operating SBFD. However, the aggressor cell may either operate with 1) legacy TDD, in which the cross-link interference is mainly generated from the overlapping resources of the TDD carrier with the UL subband; or 2) SBFD in which the inter-cell CLI interference generated from its DL transmissions in SBFD resources, i.e., interference from non-overlapping resources.

Observation 2: For inter-cell CLI for SBFD, the victim cell always operates SBFD while the aggressor cell may or may not operate SBFD.

For the beam nulling interference mitigation technique the victim cell will transmit the CLI-RS, while for beam pairing mitigation technique the aggressor cell transmits the CLI-RS. However, the interference mitigation action will always need to be taken by the aggressor cell.

When it comes to the analysis of case 1 and 2 described above, we believe case 1 corresponds to a common scenario and therefore motivates the possibility to enable SBFD operation without at the same time enable CLI mitigation techniques. This means that the standard should not mandate wasteful transmission of NZP CSI-RS for the purpose of interference mitigation for SBFD in periods where this is not needed, also because this could unnecessarily impact gNBs that have not enabled SBFD. Case 1 therefore seems to motivate introduction of specific signalling in order to transmit the information required to trigger inter-cell interference mitigation, including the NZP CSI-RS information. At this point our intention is not to restrict any solution to using existing or new signalling procedures, but rather that the signalling framework should be able to enable and disable inter-cell interference mitigation, including request of start and stop of transmission of NZP CSI-RS in case of the beam pairing mitigation technique.

Furthermore, we believe that also case 2 should be considered as a relevant scenario. In this case we think that the existence of a signalling framework as described above will still be suitable. A node 1 starting SBFD operation will then, without any initial phase based on noise detection or similar, activate interference mitigation based on CLI-RS (e.g. NZP CSI-RS) towards relevant neighbours. The outcome of this process may determine that some neighbours create inter-cell interference in SBFD resources and some not, and the node 1 can then deactivate inter-cell interference mitigation towards cells that don’t contribute to the inter-cell interference. This means that the described framework is suitable for both case 1 and case 2.

Proposal 4: Introduce a signalling framework to enable and disable inter-cell interference mitigation, including request of start and stop of transmission of NZP CSI-RS in neighbour cells in case of the beam pairing mitigation technique.

Below we illustrate such signalling framework for beam nulling and beam pairing. In both figures, step 2 has been made optional in order to take into account Case 2 described above.

Figure 1: Flow chart of the inter-gNB signalling used in beam nulling. Both gNB1 and gNB2 have activated SBFD. In this example a cell controlled by the gNB 1 is victim of gNB-to-gNB CLI originated in a cell (aggressor cell) controlled by gNB 2.

Step 1: Exchange of SBFD time and frequency configuration information.

Step 2 (optional): gNB 1 detects CLI based on UL performance degradation. No specific information exchange is needed to support this step. gNB1 uses locally available information to detect CLI based on UL performance degradation.

Step 3: Signalling of the CLI mitigation request. Upon the determination of presence of gNB-to-gNB CLI, gNB 1 sends its CLI-RS configuration to gNB 2, which corresponds to the CLI-RS transmitted by the victim cell in gNB 1 at step 4. In other words, we think that the CLI Mitigation Request content should include the measurement resource configuration that should be used by the aggressor gNB to determine/estimate the victim-to-aggressor channel matrix.

Proposal 5: The CLI Mitigation Request for beam nulling includes the CLI-RS (measurement resource) configuration corresponding to the transmitted CLI-RS.

Step 4: gNB 1 transmits the periodic CLI-RS configured in the prior step.

Step 5: gNB 2 estimates the victim-to-aggressor channel by measuring the CLI-RS of the victim gNB

In this step the gNB 2 uses the CLI-RS measurement configuration provided by gNB1 in step 3.

Step 6: gNB 2 may adjust the DL precoder if DL transmission is scheduled on the identified aggressor beam.

The aggressor beam is identified by gNB2’s own measurement, hence no additional information exchange is required to support this step.

***

For beam pairing, we propose the following signalling flow (Figure 2).

Figure 2: Flow chart of the inter-gNB signalling used in beam pairing. Example of inter-gNB signalling in support of beam pairing. Both gNB1 and gNB2 have activated SBFD. In this example a cell controlled by the gNB 1 is victim of gNB-to-gNB CLI originating in a cell (aggressor cell) controlled by gNB 2.

Step 1: Exchange of: SBFD time and frequency configuration information; SSB information for CLI measurements.

Step 2 (optional): gNB 1 detects UL performance degradation, and performs SSB-RSRP measurements to identify one or more aggressor cells / SSB beams.

Step 3: Signalling of the CLI mitigation request: gNB 1 indicates the cell ID and optionally SSB indices with highest CLI to gNB 2 (potential aggressor gNB). We believe that the CLI Mitigation Request can be used for this purpose, however the conveyed information will be different for beam nulling and beam pairing.

Proposal 6: The CLI Mitigation Request is structured so that it can convey different information for beam nulling and beam pairing.

Proposal 7: The CLI Mitigation Request for beam pairing can convey one or more SSB indices.

Step 4: gNB 2 sends CSI-RS configuration to gNB1 in response to the CLI Mitigation Request in step 3.

A new procedure might be needed for this step, however also Resource Status Reporting could be considered.

Proposal 8: RAN3 to discuss the message or signalling procedure (new?) to convey the CSI-RS configuration information from aggressor node to victim node for beam pairing.

Step 5: gNB 2 starts transmitting periodic CSI-RS, corresponding to the CSI-RS configuration sent in step 4.

Step 6: gNB1 performs per beam CLI measurements according to the aggressor CSI-RS

Step 7: CLI mitigation request: gNB2 indicates the one or more CRIs with highest measured gNB-to-gNB CLI to the gNB1

Proposal 9: The CLI Mitigation Request for beam pairing can convey one or more CSI Resource Indicators (CRI).

Step 8: gNB2 may avoid DL scheduling in the identified CSI-RS beam during all SBFD slots

# 3	Conclusion

We have made the following proposals:

Proposal 1: RAN3 requests RAN2 to define an RRC-encoded IE (inter-node message) for SBFD configuration that RAN3 can include for signalling on its interfaces.

Proposal 2: The SBFD time and frequency location configuration with cell granularity shall be introduced in the F1AP Served Cell Information IE in the F1 SETUP REQUEST message and GNB-DU CONFIGURATION UPDATE message.

Proposal 3: The SBFD time and frequency location configuration with cell granularity shall be introduced in the  Neighbour Cell Information List IE in the GNB-CU CONFIGURATION UPDATE message.

Observation 1: The open point captured by RAN3 leads to the question whether SBFD operation in a cell necessarily also requires that this cell performs inter-cell interference management.

Observation 2: For inter-cell CLI for SBFD, the victim cell always operates SBFD while the aggressor cell may or may not operate SBFD.

Proposal 4: Introduce a signalling framework to enable and disable inter-cell interference mitigation, including request of start and stop of transmission of NZP CSI-RS in neighbour cells in case of the beam pairing mitigation technique.

Proposal 5: The CLI Mitigation Request for beam nulling includes the CLI-RS (measurement resource) configuration corresponding to the transmitted CLI-RS.

Proposal 6: The CLI Mitigation Request is structured so that it can convey different information for beam nulling and beam pairing.

Proposal 7: The CLI Mitigation Request for beam pairing can convey one or more SSB indices.

Proposal 8: RAN3 to discuss the message or signalling procedure (new?) to convey the CSI-RS configuration information from aggressor node to victim node for beam pairing.

Proposal 9: The CLI Mitigation Request for beam pairing can convey one or more CSI Resource Indicators (CRI).