3GPP TSG-RAN WG3 Meeting #127	R3-250575

Athens, Greece, 17 – 21 February, 2025

Agenda item:	11.4

Source:	Nokia

Title:	Discussion on Rel-18 UE Performance measurements

Document for:	Other

# 1	Introduction

In RAN3 #125 we made the following agreement:

The details of signaling measured UE performance metrics from DU to CU-CP and/or from CU-UP To CU-CP needs further discussion during normative phase. The above overrides the agreement “Transferring Measured UE performance from CU-UP to CU-CP.” from RAN3#124.

In this contribution we provide our views regarding the definitions of the introduced UE Performance measurements in Rel-18. We think that the introduced UE Performance reporting should be consistent across use cases, including AI/ML Mobility Optimization for NR-DC, enabling a common solution with respect to the entity that measures the different UE Performance measurements. UE Performance measurements measured in the gNB-CU-UP meet this objective and to this end, we propose an LS to SA5 in the annex to ask SA5 feasibility of a throughput measurement in UL/DL measured in gNB-CU-UP for UE Performance reporting.

# Discussion

## 2.1	UE Performance Measurements

### 2.1.1	Packet Delay

According to TS 38.314, the Average Packet Delay measurement comprises different components when measured in UL and DL directions. Here, we only consider the RAN part of the Average Packet Delay in UL and DL.

Specifically, the Average Packet Delay in DL is the sum of D1+ D2+ D3+ D4, which are defined (in TS 28.558) as follows:

  - D1 (DL delay in over-the-air interface): This is the Average delay in the DL, providing the average time it takes for packet transmission over the air interface. It is measured at the gNB-DU. It is described in TS 28.558, clause 6.3.1.1.1.

  - D2 (DL delay on gNB-DU): This is the average RLC SDU delay in the DL within the gNB-DU, for initial transmission of all RLC packets. It is measured at the gNB-DU. It is described in TS 28.558, clause 6.3.1.1.2.

  - D3 (DL delay on F1-U): This is the average GTP packet delay DL on the F1-U interface. It is measured at the gNB-CU-UP. It is described in TS 28.558, clause 6.3.1.1.3.

  - D4 (DL delay in CU-UP): This is the average PDCP SDU delay on the downlink within the gNB-CU-UP, for all PDCP packets. It is measured at the gNB-CU-UP. It is described in TS 28.558, clause 6.3.1.1.4.

D1, D2, D3 and D4 measurements should be measured per DRB per UE.

Similarly, the Average Packet Delay in UL is the sum of D1+ D2.1+ D2.2+ D2.3+ D2.4, which are defined as follows:

  - D1 (UL PDCP packet average delay): This is the average PDCP delay in the UL. It is defined in TS 38.314, clause 4.3.1.1. It is a measurement performed by the UE to measure UL buffering delay in the PDCP layer, i.e. the average time a packet is buffered in the UE PDCP layer before it is transmitted. This measurement is obtained according to the definition in clause 4.3.1.1 of TS 38.314, named "UL PDCP Packet Average Delay per DRB per UE".

  - D2.1 (average over-the-air interface packet delay): This is the air interface UL delay. It is defined in TS 38.314, clause 4.2.1.2.2 (and in TS 28.558, clause 6.3.1.1.6). It is measured at the gNB-DU.

  - D2.2 (average RLC packet delay): It measures RLC packet delay in the UL. It is defined in TS 38.314, clause 4.2.1.2.3 (and in TS 28.558, clause 6.3.1.1.7). It is measured at the gNB-DU.

  - D2.3 (average delay UL on F1-U): This is the average GTP packet delay UL on the F1-U interface. It is measured using the same metric as the average delay DL on F1-U defined in TS 28.558, clause 6.3.1.1.3. It is measured by the gNB-CU-UP.

  - D2.4 (average PDCP re-ordering delay in the UL): It measures PDCP re-ordering delay in the UL. It is defined in TS 38.314, clause 4.2.1.2.4 (and in TS 28.558, clause 6.3.1.1.8). It is measured at the gNB-CU-UP.

In a split-architecture, some of the above delay components are measured in gNB-DU while some others are measured in gNB-CU-UP.  For example, for DL Packet Delay, D1 and D2 are measured at the gNB-DU while D3 and D4 are measured at gNB-CU-UP. In case of  UL Packet  Delay, D2.1 and D2.2 are measured in gNB-DU while D2.3, D2.4 are measured in gNB-CU-UP. It should also be noted that D1 is measured in the UE and gNB-CU-UP also needs this measurement to calculate the total UL packet delay.

Observation 1: Some delay components are measured at the gNB-CU-UP while some others in the gNB-DU.

In RAN3 #125, companies proposed two approaches to enable gNB-CU-CP to receive the overall Average Packet Delay measurements:

- gNB-CU-UP centric approach: gNB-DU sends the delay components measured by itself to gNB-CU-UP (over F1-U) and gNB-CU-UP forwards all the delay components (also measured by itself) to gNB-CU-CP (over E1). In this case, gNB-CU-UP aggregates all the delay components and sends them to gNB-CU-CP.

- gNB-CU-CP centric approach: gNB-DU sends the delay components measured by itself to gNB-CU-CP (over F1) and gNB-CU-UP sends the delay components measured by itself to gNB-CU-CP (over E1). In this case, gNB-CU-CP needs to aggregate the received delay components.

In our view, the gNB-CU-UP centric approach is the simplest since even though Average Packet Delay comprises different individual components, some of which are being measured in gNB-CU-UP and some in the gNB-DU, the overall delay is eventually known by the gNB-CU-UP. As can be seen in TS 38.415 [1], an NG-RAN node (gNB-CU-UP) as part of the QoS monitoring can send to UPF in the UL PDU SESSION INFORMATION (PDU Type 1) frame the DL Delay Result and UL Delay Result IEs:

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  | QMP | DL Delay Ind. | UL Delay Ind. | SNP | 1 |
| N3/N9 Delay Ind. | New IE Flag | QoS Flow Identifier |  |  |  |  |  | 1 |
| DL Sending Time Stamp Repeated |  |  |  |  |  |  |  | 0 or 8 |
| DL Received Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| UL Sending Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| DL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL QFI Sequence Number |  |  |  |  |  |  |  | 0 or 3 |
| N3/N9 Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| New IE flag 7(E) | New IE Flag 6 | New IE Flag 5 | New IE Flag 4 | New IE Flag 3 | New IE Flag 2 | New IE Flag 1 | New IE Flag 0 | 0 or 1 New IE Flags Octet |
| Spare |  |  |  |  |  |  | D1 UL PDCPDelay Result Ind | 0 or 1 |
| UL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| DL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| Padding |  |  |  |  |  |  |  | 0-3 |

Figure 2-1 UL PDU SESSION INFORMATION (PDU Type 1) Format (TS 38.415)

DL Delay Result IE indicates the downlink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and in the gNB-DU) and the delay over Uu interface in milliseconds for the involved QoS flow. Similarly, the UL Delay Result IE indicates the uplink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface and the delay in the UE in milliseconds for the involved QoS flow. Even though the purpose of sending the UL PDU SESSION INFORMATION is to send control information elements related to the PDU session from NG-RAN to UPF, this delay information is available to NG-RAN.

Observation 2: The overall uplink delay measurement result, which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU), the delay over Uu interface and the delay in the UE in milliseconds for the involved QoS flow, is available in the NG-RAN.

Proposal 1: We propose to consider gNB-CU-UP centric approach, where gNB-CU-UP aggregates the overall delay result in the DL and UL and sends it to gNB-CU-CP, similarly to DL Delay Result IE or UL Delay Result IE.

### 2.1.2 Average Throughput UL/DL

The total throughput of a UE in the target node after a handover can be impacted by multiple factors, including congestion, transport delay and radio-related impacts within the target node. In a split architecture, these different types of impacts are visible at different nodes, such as in the gNB-CU-UP or in the gNB-DU. Hence, a throughput measurement in the gNB-CU-UP would be able to measure throughput performance due to congestion while a throughput measurement in the gNB-DU would measure UE throughput performance due to radio or scheduling issues.

We proceed to discuss the different notions of throughput measurements available.

Average UE Throughput UL: One way to measure Average UE throughput in the UL is described in TS 28.558, clause 6.3.1.4.2 where throughput represents the average UE throughput in uplink for data bursts that are large enough to require transmissions to be split across multiple slots. This measurement is measured by the gNB-DU.

Average UE Throughput DL: One way to measure Average UE Throughput in the DL is through clause 6.3.1.4.1 in TS 28.558 where throughput measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. This measurement is measured by the gNB-DU.

Average buffered UE throughput DL: An alternative way to measure throughput is through clause 5.1.1.3.7 of TS 28.552 which provides the average downlink buffered UE throughput per DRB on NRCellCU. For standalone operation, the DRBs are mapped with the same 5QI while for EN-DC with the same QCI. It measures per UE and per bearer throughput independent of traffic patterns and packet size. It supports monitoring in Dual Connectivity scenario and in NSA option3a and 3x. This measurement is measured by the gNB-CU-UP.

In case of split architecture, there are two possible options to measure throughput, namely a) by measuring throughput in the gNB-DU (RLC level throughput) or b) in the gNB-CU-UP (PDCP level throughput). Below, we list a number of observations on these different options of throughput measurements:

- Throughput measurement at the gNB-DU is not end-to-end. It only reflects the effect of the lower layers in the measured UE Performance, e.g., throughput after handover. As part of the handover decision, UE measures the different neighbours’ radio quality and a specific target cell is selected for handover considering the radio quality and other resource utilization aspects.  Hence, measuring the RLC level throughput after the handover provides somewhat redundant information as the handover decision itself is made based on the radio quality. On the contrary, measuring the overall throughput at PDCP layer will be more useful since it reflects the overall performance which includes gNB-CU-UP aspects, e.g., related to congestion, F1-U transport issues together with radio level issues. If the throughput UE Performance is then sent to the source node, the latter will be able to identify if a handover decision that was based on radio quality only, also was good in terms of end-to-end throughput.

Using gNB-DU level measurement alone as feedback can result in incorrect evaluation at source node. This is because the PDCP level throughput may be quite different from the RLC-related measurements. For example, RLC level UE Performance can be good if the radio conditions are good, but at the same time the PDCP level UE Performance may be bad, e.g. due to congestion in gNB-CU-UP.  In another case, RLC level UE Performance may be poor if the radio conditions are poor, but PDCP level UE Performance may be good, that is there is no congestion in gNB-CU-UP. Hence, an RLC level measurement alone is not sufficient at the source node for evaluating the impact of its AI/ML based action.

- In NR-DC split-bearer, RLC level measurement is per-leg and can’t be aggregated to derive per-DRB measurement. To illustrate this with an example we consider the scenario where a UE is configured with one or more NR-DC split bearers after the handover to target node. If the throughout measurements are collected at RLC level, even though it is possible to collect the individual MCG and SCG leg level performance these measurements cannot be aggregated per-DRB since there is no common RLC SDU point for MCG and SCG legs. The RLC SDU point for MCG is located in MN while for SCG in SN node. Therefore, for MN, the information on how long and exactly in which time intervals the data is transmitted or kept in the buffer for SCG is hidden. The same holds for SN for MCG leg. Therefore, as neither MN nor SN knows the transmission/buffer status of data in the other leg, it is not possible to aggregate the MCG and SCG throughput per DRB. With PDCP level measurements, it is possible to collect per-bearer (QoS Flow) in all scenarios with or without NR-DC split bearers. A question that arises is whether then it is possible to send the per-leg (MCG/SCG) measurements directly to the source node without aggregation. However, also the source node will be missing required information to interpret the received measurements similarly as the target node (MN, SN). Therefore, such approach will not solve the problem.

Hence, it seems that PDCP level measurement of Average Throughput at gNB-CU-UP is the preferred option for measuring UL/DL Average Throughput.

Proposal 2: Measure Average UE Throughput UL/DL for UE Performance measurement at the gNB-CU-UP.

Currently, TS 28.552 describes Average DL UE Throughput  measurement per DRB in clause 5.1.1.3.7 as follows:

##### 5.1.1.3.7	Average DL UE buffered Throughput per DRB

a)	This measurement provides the average down link buffered UE throughput per DRB on NRCellCU. The DRBs are mapped with the same 5QI for NR SA or mapped with the same QCI for EN-DC. This measurement is intended for throughput per UE and bearer independent of traffic patterns and packet size. The measurement is based on Desired buffer size communicated within DDDS from DU to CU UP and is intended for services with burst duration spanning over the time interval of a couple of consequent DDDSs. For very bursty traffic with burst duration within the interval of one DDDS the measured values can be unprecise and cannot be compared across vendors. Initial buffering time in CU and on F1, meant as time interval the first PDCP SDU of the new burst is received in CU until this first part is received in DU, is excluded. The monitoring is supported also in DC scenario and in NSA option3a and 3x.

b)	DER(N=1);

c)	This measurement is obtained by the following formula for a measurement period:

where each ThroughputVolume and ThroughputTime is intended to represent one DL burst as explained in the Fig. 5.1.1.3.7-1, Fig. 5.1.1.3.7-3, Fig. 5.1.1.3.7-5 and Table 5.1.1.3.7-2, Table 5.1.1.3.7-4, Table 5.1.1.3.7-6 for DRB (SA, NSA option 3a), split DRB (DC), split DRB (NSA option 3x), respectively. Separate counters are maintained for each mapped 5QI (or QCI for EN-DC).

Figure 5.1.1.3.7-1 Average DL buffered UE throughput per DRB (SA, NSA option 3a)

Table 5.1.1.3.7-2 DRB (SA, NSA option 3a)

| T0’ | First PDCP SDU of the new burst arrived to CU and there are not any other PDCP SDUs in CU UP waiting for transmission to DU nor the ones mapped to PDCP PDUs and sent to DU are in the DU buffer. |
| --- | --- |
| T1’ | First PDCP PDU has been received in DU after T0’ (can be obtained as point in time when PDCP PDU sent from CU to DU plus F1 delay). |
| T2’ | The buffer in DU gets empty after T0’. |
| ThroughputTime | T2’ – T1’ [ms] See NOTE 1. The Achievable DRB throughput is obtained as the “Desired buffer size for data radio bearer” as part of last DDDS feedback [TS 38.425[56]] divided with the DDDS reporting period time interval. In case the desired buffer size is 0 and an PDCP PDU sent to DU it is considered it will be spent the whole time interval in the buffer of DU until desired buffer size >0 is reported in the next DDDS feedback. See NOTE 2. |
| ThroughptVolume | The PDCP SDU volume in bits successfully transmitted (acknowledged by DDDS) in DL to UE per bearer and one burst (consisting of PDCP SDU 1, 2 and 3 in example in Fig. 5.1.1.3.7-1). |
| NOTE 1: Contribution of the given PDCP SDUi to ThroughputTime, i.e. the time period the PDCP PDUi related to the PDCP SDUi will spend in the buffer of DU enitity, is obtained as volume of the PDCP PDUi divided with Achievable DRB throughput of the UE.In case in the point of time the PDCP PDUi is sent to DU while the previous one still kept in the DU buffer the time period these two PDCP SDUs will spend in DU buffer is cumulated, i.e. obtained as sum of volume of the PDCP PDUi and PDCP PDUi-1 divided with Achievable DRB throughput. NOTE 2: The precision of the measured ThroughputTime may be impacted with the precision of the measured F1 delay in case of not time synchronized CU and DU when it is obtained as F1 RTT/2. |  |

Figure 5.1.1.3.7-3: Average DL buffered UE throughput per split DRB (DC)

Table 5.1.1.3.7-4 Split DRB (DC)

| T0 | First PDCP SDU of the new burst arrived to CU and there are not any other PDCP SDUs in CU UP waiting for transmission to DU nor the ones mapped to PDCP PDUs and sent to DU are in the DU buffer for any of the legs relevant to the bearer. |
| --- | --- |
| T1 | First PDCP PDU has been received in DU1 of the first leg after T0 (can be obtained as point in time when PDCP PDU sent from CU to DU1 plus F1 delay). |
| T2 | First PDCP PDU has been received in DU2 of the second leg after T0 (can be obtained as point in time when PDCP PDU sent from CU to DU2 plus F1 delay). |
| T3 | The buffer in DU1 of the first leg gets empty after T0. |
| T4 | The buffer in DU2 of the second leg gets empty after T0 |
| ThroughputTime | T4 – T1 [ms] See NOTE 1. The Achievable DRB throughput is obtained as the “Desired buffer size for data radio bearer” as part of last DDDS feedback [TS 38.425 [56]] divided with the DDDS reporting period time interval. In case the desired buffer size is 0 and an PDCP PDU sent to DU it is considered it will be spent the whole time interval in the buffer of DU until desired buffer size >0 is reported in the next DDDS feedback. See NOTE 2. |
| ThroughputVolume | The PDCP SDU volume in bits successfully transmitted (acknowledged by DDDS) in DL to UE per bearer and one burst (consisting of PDCP SDU 1, 2, 3 and 4 in example in Fig. 5.1.1.3.7-3). |
| NOTE 1: Contribution of the given PDCP SDUi to ThroughputTime, i.e. the time period the PDCP PDUi related to the PDCP SDUi will spend in the buffer of DU related leg is obtained as volume of the PDCP PDUi divided with Achievable DRB throughput. In case in the point of time the PDCP PDUi is sent to DU while the previous one still kept in the DU buffer of the same leg the time period these two PDCP SDUs will spend in DU buffer is cumulated, i.e. obtained as sum of volume of the PDCP PDUi and PDCP PDUi-1 divided with Achievable DRB throughput. NOTE 2: The precision of the measured ThroughputTime may be impacted with the precision of the measured F1 delay in case of not time synchronized CU and DU when it is obtained as F1 RTT/2. |  |

Figure 5.1.1.3.7-5: Average DL buffered UE throughput per split DRB (NSA option 3x)

Table 5.1.1.3.7-6 Split DRB (NSA option 3x)

| T0 | First PDCP SDU of the new burst arrived to CU and there are not any other PDCP SDUs in CU UP waiting for transmission to DU/MeNB nor the ones mapped to PDCP PDUs and sent to DU/MeNB are in the DU/MeNB buffer for any of the legs relevant to the bearer. |
| --- | --- |
| T1 | First PDCP PDU has been received in DU of the first leg after T0 (can be obtained as point in time when PDCP PDU sent from CU to DU plus F1 delay). |
| T2 | First PDCP PDU has been received in MeNB of the second leg after T0 (can be obtained as point in time when PDCP PDU sent from CU to MeNB plus X2 delay). |
| T5 | The buffer in DU of the first leg gets empty after T0. |
| T6 | The buffer in MeNB of the second leg gets empty after T0 |
| ThroughputTime | T6 – T1 [ms] The Achievable DRB throughput is obtained as the “Desired buffer size for data radio bearer” as part of last DDDS feedback [TS 38.425[56]] divided with the DDDS reporting period time interval. In case the desired buffer size is 0 and an PDCP PDU sent to DU it is considered it will be spent the whole time interval in the buffer of DU until desired buffer size >0 is reported in the next DDDS feedback. In case of split DRB NSA option 3x to calculate the Achievable DRB throughput for LTE leg the “Desired buffer size for E-RAB” as part of last DDDS feedback [TS 36.425 [57]] reported from MeNB to CU UP via X2 interface is considered. Note; The precision of the measured ThroughputTime may be impacted with the precision of the measured F1/X2 delay in case of not time synchronized CU and DU when it is obtained as F1/X2 RTT/2. |
| ThroughputVolume | The PDCP SDU volume in bits successfully transmitted (acknowledged by DDDS) in DL to UE per bearer and one burst (consisting of PDCP SDU 1, 2, 3 and 4 in example in Fig. 5.1.1.3.7-5). |
| NOTE 1: Contribution of the given PDCP SDUi to ThroughputTime, i.e. the time period the PDCP PDUi related to the PDCP SDUi will spend in the buffer of DU related leg is obtained as volume of the PDCP PDUi divided with Achievable DRB throughput. In case in the point of time the PDCP PDUi is sent to DU while the previous one still kept in the DU buffer of the same leg the time period these two PDCP SDUs will spend in DU buffer is cumulated, i.e. obtained as sum of volume of the PDCP PDUi and PDCP PDUi-1 divided with Achievable DRB throughput. |  |

d)	Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level subcounter is performed, the number of measurements is equal to the number of mapped 5Q and QCIs for EN-DC.

e)	The measurement name has the form 
DRB.PDCP.UEThpDl, or optionally DRB.PDCP.UEThpDl.QOS, where QOS identifies the target quality of service class.

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

This measurement is measured at the gNB-CU-UP. However, the equivalent measurement in UL is not specified.

Therefore, we propose RAN3 to send an LS to SA5 to ask SA5 whether the already defined Average DL UE Throughput measurement per DRB is suitable to be used for the reporting of throughput measurement in the UE Performance reported from gNB-CU-UP to gNB-CU-CP. In addition, in the LS to SA5, RAN3 shall ask SA5 to define a new Average UL UE Throughput measurement per DRB measured at the gNB-CU-UP. Finally, since in an earlier LS exchange between SA5 and RAN3, SA5 had asked RAN3 to refer to TS 28.558 as opposed to TS 28.552 in their specifications, RAN3 shall ask SA5 to add these PDCP measured throughput per UE measurements in TS 28.558.

Proposal 3: Send an LS to SA5 to ask SA5 whether:

- The already defined Average DL UE Throughput measurement per DRB is suitable to be used for the reporting of throughput UE Performance from gNB-CU-UP to gNB-CU-CP.

- It is feasible to define an Average new UL UE Throughput measurement per DRB measured at the gNB-CU-UP.

- SA5 can add both Average DL UE Throughput measurement per DRB and Average UL UE Throughput measurement per DRB in TS 28.558.

We provide a draft LS to SA5 in the annex of this paper.

### 2.1.3 	DL Packet Loss

DL Packets may not be delivered to the UE due to two reasons. One reason is due to packet losses. Those include packets that are transmitted from the sending entity, e.g., gNB-CU-UP in case of DL PDCP SDUs, but are lost on F1 (split architecture) or Xn e.g., NR-DC split bearer or are not acknowledged from the receiving entity, e.g UE. Another reason is due to packets that are being discarded. Those include packets that could not be transmitted from the sending entity e.g., gNB-CU-UP in case of DL PDCP SDUs.

Observation 3: RAN3 agreement on Average Packet Loss DL considers only the packet losses, and not the dropped packets.

For Average Packet Loss DL, the following measurements are defined in TS 28.558/ TS 38.314.

- DL F1-U Packet Loss Rate (measured at DU of the DL packet loss on the F1-U interface) defined in TS 28.558, Clause 6.3.1.3.3: Packet Loss Rate measures the fraction of the PDCP SDU packets which are not successfully received at the gNB-DU. It is measured at gNB-DU.

- Packet Uu Loss Rate in the DL per DRB per UE defined in TS 38.314, Clause 4.2.1.5.1. This measurement measures packets that are lost at Uu transmission.

However, similarly to the case of throughput measurement, Packet Uu Loss Rate measurement at RLC level cannot be aggregated in case of NR-DC split bearer. This is because when the UE has an NR-DC split-bearer, there will be two independent Packet Uu loss rates in MCG and SCG legs. These individual measurements can’t be aggregated per-DRB level.

Different approaches could be considered to address this issue:

- Consider Packet Loss Rate measurement at gNB-DU level and preclude split bearer from NR-DC discussions

- Introduce Packet Loss Rate measurement at PDCP level

A measurement at gNB-CU-UP is preferable because it is end-to-end.

Proposal 4: RAN3 to discuss whether to introduce a new Packet Loss Rate measurement at PDCP level.

## 2.2. UE Performance Measurement Aspects in NR-DC

In NR-DC, when Dual Connectivity is established, the UE is served by two legs, the Master Leg and the Secondary Leg. With MN-Terminated Dual Connectivity, the Core Network Termination and PDCP layer is hosted in MN and data is transmitted over both Master and Secondary legs. Accordingly, the measured UE Performance (e.g, Average Throughput DL) varies depending on whether UE is configured with Dual Connectivity or not.

In case of NR-DC for split-bearer, only for non-split bearers (MCG or SCG bearers), the measurements have to be collected from a single node ie. either from Master Node or Secondary Node only. In case of split-bearers, RLC layer measurements have to be collected for both MCG-leg and SCG-leg from Master Node and Secondary Node respectively. In case of PDCP level measurements, the measurements can always be obtained from the node hosting PDCP entity independent of the bearer type (MCG or SCG or split). In case of MN-Terminated MCG Bearer, SCG bearer or split bearer, measurements are always obtained from PDCP layer hosted in Master Node. In case of SN-Terminated MCG Bearer, SCG Bearer or split bearer, measurements are always obtained from the PDCP layer hosted in Secondary Node.

One specific aspect of NR-DC is split bearers with data transmission on two legs (MCG and SCG) from RLC to physical layer and their common handling on PDCP layer in MN or SN node for MN and SN terminated bearers, respectively, as is depicted in Fig. 2a and 2b.

Figure 2a Possible options (green colour) for UE performance feedback collection in the case of MN-terminated split bearer (blue highlighted path). Left hand side: UE performance measurements are collected at the RLC layer. Right hand side: UE performance measurements are collected at the PDCP layer in the MN.

Figure 2b Possible options (green colour) for UE performance feedback collection in the case of SN-terminated split bearer (blue highlighted path). Left hand side: UE performance measurements are collected at the RLC layer. Right hand side: UE performance measurements are collected at the PDCP layer in the SN.

A similar situation exists for the cases of MN-terminated SCG bearer and SN-terminated MCG bearers, where, for the first case, RLC, MAC and physical layer is in SN, whereas PDCP layer in MN, while, for the second case, RLC, MAC and physical layer is in MN, whereas PDCP layer in SN, as depicted in Fig. 3a and 3b, respectively.

Figure 3a: Possible options (green colour) for UE performance feedback collection in the case of MN-terminated SCG bearer (blue highlighted path). Left hand side: UE performance measurements are collected at the RLC layer in the SN. Right hand side: UE performance measurements are collected at the PDCP layer in the MN.

Figure 3b: Possible option (green colour) for UE performance feedback collection in the case of SN-terminated MCG bearer (blue highlighted path), collected at the PDCP layer in the SN.

Observation 4: The processing layers for split bearer, MN terminated SCG bearer, and SN terminated MCG bearer cases are not within the same node.

In Fig. 4, SN terminated SCG bearer case is shown. As it may be seen all processing layers are within the SN node.

Figure 4: Possible options (green colour) for UE performance feedback collection in the case of SN-terminated SCG bearer (blue highlighted path). Left hand side: UE performance measurements are collected at the RLC layer in the SN. Right hand side: UE performance measurements are collected at the PDCP layer in the SN.

If the measurements are collected at PDCP layer, then the measurements can always be collected from a single layer and single node i.e., from PDCP Layer in Master Node in case of MN-Terminated MCG, SCG or Split bearer and from PDCP Layer in Secondary Node in case of SN-Terminated MCG, SCG or Split bearer. Master Node configures Data Collection initiation and reporting to the Secondary node only if there are one or more SN-Terminated bearers. Otherwise, MN can measure UE Performance measurements from its own PDCP layer.

Based on the above, we propose the following:

Proposal 5: Enable the reporting of UE performance measurement focusing on service quality, i.e., per-QoS Flow List as defined in TS 28.552 irrespective of whether UE is in single connectivity or dual connectivity.

# 3	Conclusion

In this paper we provide the following observations and proposals:

Observation 1: Some delay components are measured at the gNB-CU-UP while some others in the gNB-DU.

Observation 2: The overall uplink delay measurement result, which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU), the delay over Uu interface and the delay in the UE in milliseconds for the involved QoS flow, is available in the NG-RAN.

Proposal 1: We propose to consider gNB-CU-UP centric approach, where gNB-CU-UP aggregates the overall delay result in the DL and UL and sends it to gNB-CU-CP, similarly to DL Delay Result IE or UL Delay Result IE.

Proposal 2: Measure Average UE Throughput UL/DL for UE Performance measurement at the gNB-CU-UP.

Proposal 3: Send an LS to SA5 to ask SA5 whether:

- The already defined Average DL UE Throughput measurement per DRB is suitable to be used for the reporting of throughput UE Performance from gNB-CU-UP to gNB-CU-CP.

- It is feasible to define an Average new UL UE Throughput measurement per DRB measured at the gNB-CU-UP.

- SA5 can add both Average DL UE Throughput measurement per DRB and Average UL UE Throughput measurement per DRB in TS 28.558.

Observation 3: RAN3 agreement on Average Packet Loss DL considers only the packet losses, and not the dropped packets.

Proposal 4: RAN3 to discuss whether to introduce a new Packet Loss Rate measurement at PDCP level.

Observation 4: The processing layers for split bearer, MN terminated SCG bearer, and SN terminated MCG bearer cases are not within the same node.

Proposal 5: Enable the reporting of UE performance measurement focusing on service quality, i.e., per-QoS Flow List as defined in TS 28.552 irrespective of whether UE is in single connectivity or dual connectivity.

# References

[1]	3GPP TS 38.415	“NG-RAN; PDU Session User Plane Protocol”

# Annex – Draft LS to SA5

3GPP TSG-RAN WG3 Meeting #127	R3-25xxxx

Athens, Greece, 17 – 21 February, 2025

Title:	[Draft] LS on New UL Throughput measurement in TS 28.558

Response to:

Release:	Release 19

Work Item:	NR_AIML_NGRAN_enh-Core

Source:	Nokia [to be RAN3]

To:	SA5

CC :

Contact Person:

#### Name:	Anna Pantelidou

E-mail Address:	anna.pantelidou@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 has agreed to exchange Average UE Throughput UL/DL between NG-RAN nodes as part of UE Performance Feedback to evaluate the performance of a Handover. Even though Average UE can be measured Throughput measurements can be measured at the RLC layer, RAN3 has agreed that PDCP level measurement of Average Throughput measured at gNB-CU-UP is the preferred option for measuring UL/DL Average Throughput if feasible from SA5’s point of view. RAN3 has the following questions to SA5:

- Is the already defined Average DL UE Throughput measurement per DRB suitable to be used for the reporting of throughput UE Performance from gNB-CU-UP to gNB-CU-CP?

- Is it feasible to define a new Average UL UE Throughput measurement per DRB measured at the gNB-CU-UP?

- Is it possible for SA5 to add both Average DL UE Throughput measurement per DRB and Average UL UE Throughput measurement per DRB in TS 28.558?

2. Actions:

To SA5 : 	RAN3 would like to ask SA5 to implement the above in their specifications, if possible, and provide feedback to RAN3 if needed.

3. Dates of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting #127bis			  	7 - 11 		April 2025								TBD, China

TSG-RAN WG3 Meeting #128			  		19 - 23 	May 2025								Malta, Malta

TSG-RAN WG3 Meeting #129			  		25 - 29 	August 2025								TBD, India