3GPP TSG-RAN WG3 Meeting #127	R3-250576

Athens, Greece, 17 – 21 February, 2025

Agenda item:	11.4

Source:	Nokia, Deutsche Telekom, Jio Platforms (JPL)

Title:	Continuous Management-based MDT Requirements

Document for:	Discussion and Decision

# 1	Introduction

The topic of Continuous MDT has been discussed in RAN3 over several meetings, with two types of solutions being proposed by companies, namely solutions that impact the UE and solutions that are network-based and do not have UE impacts. In RAN3 #126 we made an important agreement with respect to Continuous Management-based MDT, which limits the solution space to solutions that do not impact the UE:

In R19, introducing the Continuous MDT functionality with a solution being applicable to legacy UEs and not only to Rel-19 UEs.

In the last meeting, we also agreed that since Continuous MDT is configured by OAM, the solution needs to be specified by SA5. However, since the problem of Continuous MDT is defined by RAN3, we need to provide SA5 with relevant information regarding the problem description, solution requirements and also potential solutions if those can be agreed in RAN3:

Agree to send an LS to SA5 capturing e.g. problem description, solutions requirements, potential solutions (if agreeable). When to send the LS depends on convergence on the content of the LS.

In this paper, we provide a list of requirements that need to be fulfilled for the Continuous MDT solution. We also provide a draft LS to SA5 in [1], including the problem description and this list of requirements.

# 2	Discussion

The problem of continuous Management-based is described in TR 38.743 as follows:

“Problem Description:

The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection.

The Data Collection continuity in this scenario can be split into two tasks as below:

-	Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

-	Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.”

We believe that even though this problem description is accurate, it is important to further clarify what are the implications of Continuous MDT on the collected measurements.

When continuous management-based MDT is configured to a gNB, the gNB will need to select the UE(s) that better match the received measurement configuration from OAM. In legacy management-based MDT a gNB selects UEs for immediate MDT and for logged MDT separately, after checking gNB capabilities and UE capabilities. In particular, a gNB receiving management-based Trace Activation from OAM verifies whether it supports the Trace function and configuration (all or part of) parameters, and accordingly, if supported, initiates suitable device(s) selection, based on their capabilities, by e.g., determining support of the requested measurements Mx. Selecting the UEs in this way allows OAM to receive “enough” measurements from different UEs, e.g., from UEs that have logging capability (which can provide logged MDT reports) and from UEs that don’t (and which may provide immediate MDT).

When it comes to continuous MDT the requirement is that the “same” UE provides both logged and immediate MDT measurements which means that continuous MDT is only applicable to UEs that have logging capability. In addition, if those UEs must provide all the measurements indicated in the MDT configuration this will possibly lead to a very limited set of measurement-providing UEs, which would subsequently be a source of bias in the data collection and limit usefulness of the continuous MDT feature. Therefore, it seems that in order to maintain a sufficient number of UEs providing measurements we do not need to mandate a UE to provide all measurements indicated in the Trace Activation.

Based on the above, we believe that for an accurate problem description towards SA5, the description in TR 38.743 may serve as a starting point on top of which RAN3 may include additional clarifications as discussed above and  continuity requirements.

Proposal 1: Describe the problem of Continuous MDT, taking the use case description in TR 38.743 as a starting point. RAN3 to further discuss whether UE is allowed not to provide some of the measurements indicated in the Trace Reference to enable a more wide applicability of Continuous MDT feature.

Next, we proceed to provide a number of requirements that the solution for Continuous Management-based MDT should satisfy.

## 2.1	Requirements

In RAN3 #126, we reached an agreement that in Rel-19 the solution to Continuous MDT is applicable to legacy UEs and not only to Rel-19 UEs. This is an important requirement that would affect the space of Continuous MDT solutions and needs to be communicated to SA5.

Requirement 1: It should be possible to select a legacy (Rel-16) UE for continuous MDT session.

Even though we agreed that Continuous MDT is configured to legacy UEs meaning that the continuous MDT solutions do not introduce UE standardization impacts, we think that another important requirement is that among different solutions providing Continuous MDT preference should be given to solutions that have minimum impacts in normal UE operation in terms e.g., of the amount of reporting or needed configuration/reconfiguration.

Requirement 2: Among various possible Continuous MDT solutions provide preference to solutions that minimally impact normal UE operation.

Also, in RAN3 #125bis we agreed that according to RAN3 Continuous Management-based MDT operation is decided by OAM and is configured by OAM to the NG-RAN:

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

How this is indicated is not agreed, though different solutions have been proposed in the previous meeting, specifically to:

- Reserve a range of Trace References to indicate Continuous MDT.

- Introduce a new Job Type through a flag e.g., “Immediate MDT and Logged MDT”.

Introducing a new Job Type for “Logged and Immediate MDT” requires SA5 changes to introduce a new Job Type. Furthermore, according to TS 32.422 Job Type clause 5.9a, e.g., to support anonymization is described as follows:

***************************************************

## 5.9a	Job Type (M)

The Job Type is a mandatory parameter. It defines if a single trace job, a single MDT job, a combined MDT and trace job,  a single RLF report collection job, a single RCEF report collection job, a single 5GC UE level measurements job, a combined trace and 5GC UE level measurements job, a combined MDT and 5GC UE level measurements job, a combined trace, MDT and 5GC UE level measurements job or a single RRC report collection job is activated. This parameter also defines the MDT mode.

[…]

***************************************************

According to the above observation creating a new Job Type combining Logged MDT and Immediate MDT goes against the current use of Job Type where it defines the MDT mode. So, it is our understanding that Job Type can characterize a single MDT mode only.

Observation 1: Job Type can characterize a single MDT mode only, i.e., either Logged MDT or Immediate MDT.

Hence, we think that the alternative to indicate Continuous MDT through a reserved range of Trace References is simpler and doesn’t require standardization changes.

Proposal 2: Ask SA5 feasibility of OAM indicating Continuous Management-based MDT to RAN through a reserved range of Trace References.

In legacy management-based MDT, OAM may configure different management-based MDT configurations in an area of gNBs. Thus, a gNB may receive different management-based immediate MDT and logged MDT configurations. Similarly, in continuous MDT in an area comprising multiple gNBs, there can exist multiple different management-based Continuous MDT configurations similarly to management-based MDT (i.e. multiple management-based immediate MDT configurations and/or multiple management-based logged MDT configurations).

Requirement 3: In an area comprising multiple gNBs, there can exist multiple different management-based immediate MDT configurations and/or multiple management-based logged MDT configurations corresponding to Continuous MDT.

A gNB that serves at a point in time a UE that has an active Continuous MDT session should be able to provide the UE with the same configuration that UE was previously configured with. This means that for a UE that transitioned back to RRC Connected state from RRC Idle state, the gNB to which UE connects needs to apply the same configuration that was chosen by the previous gNB. For a UE that is handed over from a source to a target node this means that the target node needs to apply the same configuration that the source node had configured the UE with.

Requirement 4: A gNB needs to be able to apply the same MDT Configuration  that was previously configured to a given UE that was selected for Continuous MDT.

Another aspect pertains to trace correlation for Continuous MDT since the current mechanisms do not allow correlation of immediate and logged MDT records from the same UE at the TCE, collected across RRC states and across different gNBs. For Continuous MDT such correlation must be enabled.

There are two ways that such correlation identifier may be introduced. One alternative, corresponds to one or more identifiers provided from the RAN to the TCE. However, there is no permanent UE identifier in the RAN. We believe this principle should not change for the purpose of Trace Record correlation.

Requirement 5: RAN3 shall not introduce a permanent UE Identifier in the RAN for the purpose of Trace Record Correlation.

An alternative approach is to introduce an identifier from the AMF to the TCE, that could be sent similarly to the existing IMEI-TAC identifier.

Logged MDT does not continue in the UE for more than the logging and retrieval duration which is 48 hours. After this, a UE should stop an ongoing trace session for logged MDT. Therefore, logged MDT and immediate MDT records do not need to be correlated over a longer period of time than 48 hours.

Requirement 6: The identifier that may be provided by the RAN or by the Core Network needs to identify a session during the logging and retrieval duration of logged MDT (48 hours).

Therefore, we make the following proposal:

Proposal 3: The Continuous MDT solution should meet the following requirements:

- It should be possible to select a legacy (Rel-16) UE for continuous MDT session.

- Among various possible Continuous MDT solutions provide preference to solutions that minimally impact normal UE operation.

- In an area comprising multiple gNBs, there can exist multiple different management-based immediate MDT configurations and/or multiple management-based logged MDT configurations corresponding to Continuous MDT.

- A gNB needs to be able to apply the same MDT Configuration  that was previously configured to a given UE that was selected for Continuous MDT.

- RAN3 shall not introduce a permanent UE Identifier in the RAN for the purpose of Trace Record Correlation.

- The identifier that may be provided by the RAN or by the Core Network needs to identify a session during the logging and retrieval duration of logged MDT (48 hours).

# 3	Conclusion

In this paper we make the following observations and proposals:

Proposal 1: Describe the problem of Continuous MDT, taking the use case description in TR 38.743 as a starting point. RAN3 to further discuss whether UE is allowed not to provide some of the measurements indicated in the Trace Reference to enable a more wide applicability of Continuous MDT feature.

Observation 1: Job Type can characterize a single MDT mode only, i.e., either Logged MDT or Immediate MDT.

Proposal 2: Ask SA5 feasibility of OAM indicating Continuous Management-based MDT to RAN through a reserved range of Trace References.

Proposal 3: The Continuous MDT solution should meet the following requirements:

- It should be possible to select a legacy (Rel-16) UE for continuous MDT session.

- Among various possible Continuous MDT solutions provide preference to solutions that minimally impact normal UE operation.

- In an area comprising multiple gNBs, there can exist multiple different management-based immediate MDT configurations and/or multiple management-based logged MDT configurations corresponding to Continuous MDT.

- A gNB needs to be able to apply the same MDT Configuration  that was previously configured to a given UE that was selected for Continuous MDT.

- RAN3 shall not introduce a permanent UE Identifier in the RAN for the purpose of Trace Record Correlation.

- The identifier that may be provided by the RAN or by the Core Network needs to identify a session during the logging and retrieval duration of logged MDT (48 hours).

# References

[1]		R3-250577, “[Draft] LS on Continuous MDT Requirements”, Nokia