3GPP TSG-RAN WG3 Meeting #127	R3-250577

Athens, Greece, 17 – 21 February, 2025

Title:	[Draft] LS on Continuous MDT Requirements

Response to:	-

Release:	Release 19

Work Item:	NR_AIML_NGRAN_enh-Core

Source:	Nokia [to be RAN3]

To:	SA5

CC :

Contact Person:

#### Name:	Anna Pantelidou

E-mail Address:	anna.pantelidou@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 introduced the problem of Continuous management-based MDT, where MDT measurements can be provided by the same UE across RRC states and across handover events. Specifically, the problem description of continuous MDT is described in TR 38.743, quotes as follows:

The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection.

The Data Collection continuity in this scenario can be split into two tasks as below:

-	Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

-	Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

RAN3 has identified the following requirements for the Continuous MDT solution:

- It should be possible to select a legacy (Rel-16) UE for continuous MDT session.

- Among various possible Continuous MDT solutions provide preference to solutions that minimally impact normal UE operation.

- In an area comprising multiple gNBs, there can exist multiple different management-based immediate MDT configurations and/or multiple management-based logged MDT configurations corresponding to Continuous MDT.

- A gNB needs to be able to apply the same MDT Configuration that was previously configured to a given UE that was selected for Continuous MDT.

- RAN3 shall not introduce a permanent UE Identifier in the RAN for the purpose of Trace Record Correlation.

- The identifier that may be provided by the RAN or by the Core Network needs to identify a session during the logging and retrieval duration of logged MDT (48 hours).

RAN3 would also like to ask SA5 regarding the feasibility of indicating Continuous Management-based MDT from OAM to the NG-RAN through a reserved range of Trace References.

2. Actions:

To SA5 : 	RAN3 would like to ask SA5 to take the above requirements into account and define a Continuous Management-based MDT solution. RAN3 would also like to ask SA5 to answer the feasibility question.

3. Dates of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting #127bis			  	7 - 11 		April 2025								TBD, China

TSG-RAN WG3 Meeting #128			  		19 - 23 	May 2025								Malta, Malta

TSG-RAN WG3 Meeting #129			  		25 - 29 	August 2025								TBD, India