3GPP TSG-RAN WG3 Meeting #127	R3-250600

Athens, GR, 17 – 21 February, 2025

Agenda item:	8.1

Source: 	Huawei

Title: 	Discussion on multi-modality awareness

Document for:	Discussion and Decision

# 1. Introduction

During SA#106 meeting, an LS (SP-241921/R3-250032) on multi-modality awareness was sent from SA to the other WGs. According to the LS, SA has reached consensus that the multi-modality awareness at NG-RAN can be achieved by the CN providing the MMSID. Both SA and RAN WGs are asked to consider the specification impacts for the solution.

In this contribution, we will analyse the impacts on RAN3 specifications to transfer MMSID from CN to NG-RAN to support the multi-modality awareness.

# 2. Discussion

The incoming LS in SP-241921 [1] is excerpted below. According to the LS, SA understood it is useful and feasible to support multi-modality awareness at NG-RAN by transferring the MMSID from CN to RAN.

- Overall description SA has discussed the RAN2(R2-2409272), RAN3(R3-245682) and SA4(S4-242223) LSs on Multimodality awareness. Based on the understanding that in R2-2409272, RAN2 agreed that multi-modality information (e.g., MMSID), can be used for joint admission control and QoS flow to DRB mapping and that RAN3 agreed, as reported in R3-245682, that it is technically feasible to enhance NGAP to provide the MMSID to NG-RAN node, SA has reached consensus that the multimodality awareness can be achieved in the RAN by the CN providing multimodality information (i.e., the MMSID) to the NG-RAN by N2 SM information. SA expects SA2 to handle the above LSs and provide related specification updates based on the principle described above. - Actions To RAN, RAN2, RAN3, SA2: ACTION: SA kindly asks RAN, RAN2, RAN3 and SA2 to take the above guidance into account in order to achieve a 3GPP-wide coordinated approach to the support of multimodality.

To enable the solution, RAN3 should consider the specific impacts on NG, Xn, F1 and E1 interfaces.

- Impacts on NGAP

First of all, to obtain the multi-modality information from CN, N2 signaling should be enhanced to support the CN providing MMSID during the PDU Session Resource Setup and PDU Session Resource Modify procedures. Besides, in case of NG-based handover, the MMSID may be provided by the CN to the target network, thus the Handover Resource Allocation procedure should also be considered.

Since the MMSID is associated with each data flow, it is suitable to include it in the QoS Flow Level QoS Parameters IE, which is sent from the CN to the RAN during the above mentioned procedures. A new IE for MMSID should be introduced, its definition and format can refer to the R18 MMSID defined by SA, as specified in TS 29.514 [2]. Corresponding CR for NGAP can be found in R3-250602 [3].

- Impacts on XnAP

Considering the Xn interface, the cases of Xn-based handover and DC should be taken into account. During handover, the source gNB may provide the MMSID to the target gNB so that the target gNB can perform joint admission control, which means the MMSID should be included in the HANDOVER REQUEST message. For DC case, the MN may need to provide the MMSID to the SN during the SN addition procedure and SN modification procedure to allow the SN to take the information into account.

Similar to NGAP, the MMSID can be added into the QoS Flow Level QoS Parameters IE so that it can be transferred associated with the QoS flow within the handover and DC related massages. Corresponding TP for XnAP is provided in R3-250605 [4].

- Impacts on F1AP

For the scenario of CU-DU split, the MMSID can be further transferred to the DU so that the DU can utilize the information when performing scheduling or resource allocation. For example, the DU may reserve sufficient resource for the QoS flows with the same MMSID so that the service can be satisfied.

The MMSID can be carried in the QoS Flow Level QoS Parameters IE within the UE context setup and modification messages. Corresponding TP is provided in R3-250604 [5].

- Impacts on E1AP

If the NG-RAN node using disaggregated architecture with CU-CP and CU-UP split, it would be needed to transfer the MMSID over the E1 interface so that the CU-UP can take the information into account. Similarly, the CU-CP can provide the MMSID during bearer context setup and bearer context modification procedures by including MMSID in the QoS Flow Level QoS Parameters IE, as shown in the CR in R3-250603 [6].

Proposal 1: Add a new IE for MMSID in the QoS Flow Level QoS Parameters IE in NGAP, XnAP, E1AP and F1AP specifications. RAN3 to endorse the attached CR/TPs in R3-250602, R3-250603, R3-250604 and R3-250605.

Based on the above analysis, a draft reply LS is provided in [7].

# 3. Conclusion

In this contribution, we analyzed the specification impacts to support multi-modality awareness. The following proposal was made:

Proposal 1: Add a new IE for MMSID in the QoS Flow Level QoS Parameters IE in NGAP, XnAP, E1AP and F1AP specifications. RAN3 to endorse the attached CR/TPs in R3-250602, R3-250603, R3-250604 and R3-250605.

# 4. Reference

[1]. SP-241921, TSG SA, LS on multi-modality awareness.

[2]. 3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[3]. R3-250602, Huawei, (CR to 38.413) Correction to support multi-modality awareness.

[4]. R3-250605, Huawei, (TP for BL CR for TS 38.423) Support for multi-modality awareness.

[5]. R3-250604, Huawei, (TP for BL CR for TS 38.473) Support for multi-modality awareness.

[6]. R3-250603, Huawei, (CR to 37.483) Correction to support multi-modality awareness.

[7]. R3-250601, Huawei, [draft] Reply LS on multi-modality awareness.